3GPP TSG RAN WG1 #120 		                                    		   R1-2500708

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	5

Source:	InterDigital, Inc.

Title:	Discussion for SA2 LS on LMF-based AI/ML Positioning for Case 2b

Document for:	Discussion and Decision

# Introduction

In RAN1#120, RAN1 received an LS [1] from SA2 with the following question.

SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

In this contribution, we propose a reply for the received LS.

# Status of discussion related to Case 2b in RAN1

In RAN1 the following agreements related to Case 2b are made. To summarize, high-level agreements related to measurements are made but the details related to actual measurements have not been agreed in RAN1 yet. This is mainly due higher priority assigned to first-priority topics, namely Case 1, Case 3a and Case 3b [2].

Proposal 1 : Send SA2 the following list of the agreements relevant to Case 2b made in RAN1#116, 116b, 117, 118 and 118b.

RAN1#116 Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. RAN1#116b Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. RAN1#117 Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed RAN1#118 Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. RAN1#118b Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

Given that RAN1 has only 3 meetings (RAN1#120, RAN1#120b, RAN#121) left to finalize details of the first-priority items, our proposal is to invest the remaining time in RAN1 meetings to complete the specification work of the first-priority topics.

Proposal 2 : Respond to SA2 indicating that, during Release 19, RAN1 has no intention on working on Case 2b which is indicated as the second priority topic in the WID.

# Conclusion

Proposal 1 : Send SA2 the following list of the agreements relevant to Case 2b made in RAN1#116, 116b, 117, 118 and 118b.

Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

Proposal 2 : Respond to SA2 indicating that, during Release 19, RAN1 has no intention on working on Case 2b which is indicated as the second priority topic in the WID.

# Reference

[1] R1-2500388, “LS on LMF-based AI/ML Positioning for Case 2b, ” vivo, RAN1#120, Athens, Greece, Feb., 2025.

[2] RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, ” Qualcomm, RAN#106, Madrid, Spain, Dec., 2024.