3GPP TSG RAN WG1 #120                                                                                                  R1-2500712

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.1.3

Source:	            		Xiaomi

Title:	Discussion on AI/ML model based CSI prediction

Document for:	Discussion and Decision

Introduction

In the RAN#105 meeting, the following WID on UE-sided AI/ML based CSI prediction was discussed and revised achieved [1]. In this contribution, the consistency of training and inference and specification aspects on AI model based CSI prediction are discussed.

Provide specification support for the following aspects: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

Discussion on consistency of training and inference

In [2], the following agreements on consistency between training and inference were achieved.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

It was agreed which NW side additional conditions impacting on performance CSI prediction needs to further study. Potential NW-side additional conditions, e.g., tilt angle and TXRU mapping were provided to identify these conditions impact on UE assumption for CSI prediction. We provide the evaluation generalization of TXRU mapping in this section. The following two TXRU mapping methods are adopted. The main simulation assumption is given in Table 1.

- TXRU mapping 1：32 port CSI-RS, [N1, N2, P] = [2, 8, 2] with (8x8x2) antenna elements, (dH,dV) = (0.5λ, 0.8λ)

- TXRU mapping 2：32 port CSI-RS, [N1, N2, P] = [2, 8, 2] with (12x8x2) antenna elements, (dH,dV) = (0.5λ, 0.8λ)

Table1: Main simulation assumption for CSI prediction

| Assumption | Value |
| --- | --- |
| Scenario | UMa |
| Channel model | Uma 38.901 |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Subcarrier spacing | 15 KHz |
| Data type | raw channel |
| Time domain part | 2000 consecutive slots |
| UE speed | 30 km/h or 60 km/h |
| UE distribution | 100% outdoor |
| CSI feedback periodicity | 5ms |
| Observation window (number/distance) | 5/5ms |
| Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 1/-/5ms |
| Whether/how to adopt spatial consistency | None |
| Whether/how channel estimation error is modelled | None |
| Whether/how phase discontinuity is modelled | None |
| Number of UE receiver antenna | 2 |
| Site number | 1 |
| Sector number | 3 |
| User number per cell | 1050 |
| Drop number | 10 |

In the simulation, the model used for CSI prediction is shown in Figure 1. The key values of the model structure are given in Table 2. The training data are respectively generated by using TXRU mapping 1 and TXRU mapping 2.

Figure 1: Model structure of AI model for CSI prediction

Table 2: The key values of model structure

| Model structure | Value |
| --- | --- |
| LSTM_1 or LSTM_2 | 256 neurons |
| Dense | 128 neurons |
| Activation | ‘Linear’ |
| Loss function | NMSE |

In Table 3, the simulation results are provided for 30 Km/h. In the simulation, three AI models are trained by using generated dataset according to TXRU mapping 1, TXRU mapping 2 and TXRU mapping 1 & TXRU mapping 2, respectively. TXRU mapping 1 & TXRU mapping 2 denotes that dataset generated by using TXRU mapping 1 and dataset generated by using TXRU mapping 2 are mixed according to the ratio 1:1.

Table 3: The simulation results for different training models inference on different test data for 30 Km/h

| Test dataset Model | Test on TXRU mapping 1 |  | Test on TXRU mapping 2 |  |
| --- | --- | --- | --- | --- |
|  | SGCS | NMSE (dB) | SGCS | NMSE (dB) |
| TXRU mapping 1 | 0.9707 | -14.43 | 0.9715 (-0.021%) | -14.57 (-0.137%) |
| TXRU mapping 2 | 0.9704 (-0.031%) | -14.39 (-0.277%) | 0.9717 | -14.59 |
| Mixed TXRU mapping 1 & TXRU mapping 2 | 0.9701 (-0.061%) | -14.30 (-0.901%) | 0.9714 (-0.01%) | -14.56 (-0.206%) |

Observation 1: When UE’s velocity is configured 30 Km/h, for the AI/ML CSI model trained on a dataset with one TXRU mapping and doing inference on a dataset with different TXRU mapping, there is no performance degradation compared with the training dataset and test dataset generated with the same TXRU mapping.

Observation2：When UE’s velocity is configured 30 Km/h, for the AI/ML CSI model trained on a mixed dataset with different TXRU mapping and doing inference on those datasets with different TXRU mapping, there is no performance degradation compared with the training dataset and test dataset generated with the same TXRU mapping.

Similarly, the simulation results are provided for 60 Km/h in Table 4.

Table 4: The simulation results for different training models inference on different test data for 60 Km/h

| Test dataset Model | Test on TXRU mapping 1 |  | Test on TXRU mapping 2 |  |
| --- | --- | --- | --- | --- |
|  | SGCS | NMSE (dB) | SGCS | NMSE (dB) |
| TXRU mapping 1 | 0.7676 | -5.6219 | 0.7801 (-6.41%) | -5.8712 (-20.87%) |
| TXRU mapping 2 | 0.7358 (-4.14%) | -4.7830 (-14.92%) | 0.8335 | -7.4201 |
| Mixed TXRU mapping 1 & TXRU mapping 2 | 0.7674 (-0.026%) | -5.6149 (-0.125%) | 0.8165 (-2.04%) | -6.8173 (-8.12%) |

Observation 3: When UE’s velocity is configured 60 Km/h, for the AI/ML CSI model trained on a dataset with one TXRU mapping and doing inference on a dataset with different TXRU mapping, some performance loss (SGCS loss is about 4.14%~6.41%, NMSE loss is about 14.92%~20.87%) could be observed compared with the training dataset and test dataset generated with the same TXRU mapping.

Observation 4：When UE’s velocity is configured 60 Km/h, for the AI/ML CSI model trained on a mixed dataset with different TXRU mapping and doing inference on those datasets with different TXRU mapping, the performance loss could be reduced (SGCS loss is about 0.026%~2.04%, NMSE loss is about 0.125%~8.12%) compared with the training dataset and test dataset generated with the same TXRU mapping.

According to above observations, NW’s additional condition can have impact on the inference performance of AI model f. Although the performance loss could be reduced by using model trained via mixed dataset, there still be performance gap with the best performance. In order to obtain the best performance, the NW’s additional conditions should be indicated to UE.

Proposal 1: NW’s additional condition is suggested to indicate to UE.

For UE-sided model for BM-Case 1 and BM Case 2, associated ID is used to ensure consistency between training and inference. The procedure of functionality identification or ensuring consistency of training and inference for UE-side AI/ML model-based CSI prediction and UE-side AI/ML model-based beam prediction are similar. Both use cases need to collect training dataset based on NW configurations or conditions, and the model is trained according to the collected dataset. The configurations and its related associated IDs are signaled by NW. Hence, we suggest that the associated ID which is used to ensure consistency of training and inference for AI based CSI prediction could be studied as a starting point.

Proposal 2: For UE-side AI/ML model-based CSI prediction, the associated ID which is used to ensure consistency of training and inference for AI based CSI prediction could be studied as a starting point.

Discussion on specification impact of AI model based CSI prediction

Performance monitoring

Regarding on performance monitoring, the following agreements or observations have been identified during RAN1#114, RAN1#116-bis and RAN1#117 meetings [3]-[5].

Agreement For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculate the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculate the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW. Agreement For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects. - To clarify the boundary between type 1 and type 3 - To clarify definition of monitoring output and performance metric Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

For Type 1, the performance threshold needs to discuss and specify for calculating monitoring. The performance threshold is related with the number of This will increase specification efforts. However, Type 3 does not define or configure threshold criterion. For Type 3, the average of SGCS or NMSE of predicted N slots needs to report i order to make the monitoring result be robust. The values of N could be further studied. NW makes decision of functionality fallback or model switch operation according to the received performance metric.

For Type 2, the performance metric is calculated by NW which utilizes the predicted CSI and/or the corresponding ground truth CSI reported by UE. This overhead of Type 2 is obvious larger than that of Type 1 and Type 3 due to predicted CSI and ground truth CSI reporting.

Proposal 3: Considering specification efforts and reporting overhead, at least type 3 (UE calculates the performance metrics and reports the metrics for NW making decision) is adopted to monitor model performance.

For performance monitoring type 3, UE needs to report performance metric. The remaining issue is how to report the values of metric. Since the overhead of reporting performance metric is limited, L1 signalling can be considered to report monitoring performance. Aperiodic, periodic or semi-persistent reporting can be reused to report performance matric. UE-event report can be considered as well since the model is deployed at UE side and UE knows when to report monitoring results according to UE side measurement.

Proposal 4:  The following specification impact on monitoring performance reporting could be studied as a starting point:

- L1 signalling for reporting monitoring performance.

- Aperiodic, periodic, semi-persistent or UE-event reporting monitoring performance.

In order to obtain ground truth CSI, channel measurement resource (CMR) should be configured to UE. Before discussing the CMR configuration for performance monitoring, it is necessary to study the relationship between CMR configuration for performance monitoring and CMR configuration for model inference. In our view, the CMR could be same for them if one periodic CMR is configured, since both historic and predicted CSI could be measured by the CMR. Otherwise, different CMRs are respectively configured for model monitoring and model inference. Assume dedicated CMR for performance monitoring is configured. It is necessary to study whether/how to enhance CMR configuration/transmission, since more CMR configurations or transmissions will consume a lot of signalling overhead. Especially, the monitoring information is measured based on multiple CMR transmission occasions.

Proposal 5: For obtaining ground truth CSI to monitor performance, whether/how to enhance CSI-RS configuration/transmission enhancement needs to further study.

It has agreed that CSI processing criteria and timeline for inference need to study [4]. Since monitoring result or the values of performance metric needs to measure and report, it is also necessary to study the CSI processing criteria and timeline for model monitoring. The monitoring information could be jointly or separately reported with model inference information. If they are jointly reported, CSI processing criteria and timeline could be studied for model inference and monitoring together.

Proposal 6: For performance monitoring, the necessity and potential specification impacts including at least following aspects need to study:

- CSI processing criteria and timeline

Data collection

The following observations on data collection for CSI prediction using UE sided model were provided in [3] and [4].

Observation In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including: - Signaling and procedures for the data collection - data collection indicated by NW - Requested from UE for data collection - CSI-RS configuration - Assistance information for categorizing the data, if needed - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. Agreement For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement.

Two options on the signalling and procedures for the data collection were proposed by companies. One option is data collection indicated by NW. The other one is data collection requested from UE. Note that the model of CSI prediction is UE sided model. According to description in TR 38.843 [2], the training data for the UE sided model could be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server. In our view, this implies the UE sided model could be trained at UE or UE-side OTT server. UE clearly knows what kind of training data are requested. Compared with data collection indicated by NW, it is more flexible that data collection is requested by UE for model training.

Proposal 7: At least requested from UE for training data collection model should be supported.

For UE-side CSI prediction AI model, the training dataset is generated at UE side based on received CSI-RS signal measurement. If there are too many antenna ports at gNB side, e.g., no less than 32 ports, it will consume a lot of CSI-RS signalling overhead for channel measurement. It is necessary to study how to reduce CSI-RS signalling overhead. The channel information of frequency domain and spatial domain is correlated. Such correlation could be utilized to reduce signalling overhead. For example, lower density of CSI-RS resource is configuration. The number of the number of CSI-RS ports is less than the number of data transmission antenna ports. The remained channel information in frequency domain and spatial domain is inferred by AI model prediction or legacy interpolation algorithm.

Proposal 8: For training dataset collection, CSI-RS resource configuration with lower frequency density or less antenna ports could be supported to reduce CSI-RS signalling overhead.

Model inference

For model inference, it has agreed that legacy CSI-RS configuration can be a starting point and further study on whether there is a need for specification enhancement. According to SID given in RAN#102 meeting, it needs to further study performance gain over Rel-18 non-AI/ML based approach, i.e., Rel-18 Type II Doppler codebook. Multiple aperiodic CSI-RS (A-CSI-RS) resources and one periodic/semi-persistent CSI-RS could be configured for Rel-18 Type II Doppler codebook. The interval of adjacent A-CSI-RS resources is 1 sot or m slots, where m = 1 or 2.  For fair comparison, the overhead of CSI-RS resource should be same for UE sided AI/ML model based CSI prediction and Rel-18 Type II Doppler codebook. Hence, the legacy CSI-RS configuration for Rel-18 Type II Doppler codebook could be reused for model inference.

Proposal 9: For model inference, CSI-RS resource configuration for Rel-18 Type II Doppler codebook based CSI feedback could be reused.

It was agreed that potential specification impacts on CSI processing criteria and timeline needs to further study [4]. In current specification [6], when CSI report reportQuantity is configured as 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the value of OCPU is related with the time domain behaviour type of CSI-RS resources for channel measurement and the number of CSI-RS resources. If the CSI-RS resource is aperiodic,  OCPU=8 for K=12 and OCPU=Y1⋅K for K<12, where Y1∈{1, 2, 3} is reported by UE capability indication and K is the number of CSI-RS resources in a CSI-RS resource set. If the CSI-RS resource is periodic or semi-persistent, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where N4 is length of Doppler domain basis vector, and Y2∈{1, 2, 3} is reported by UE capability indication. Note that only one CSI-RS resource is configured for periodic or semi-persistent CSI-RS resource. According to above agreement, legacy CSI-RS configuration and Rel-18 Type II Doppler codebook used for CSI feedback are considered as a starting point of discussion. Hence, the value of OCPU for Rel-18 Type II Doppler codebook could be studied as a starting point as well.  In order to reuse the legacy CPU pool, the CPU of  AI based CSI prediction and CSI reporting could be set to wOcpu for AI based CSI predication, where w is indicated by UE capability reporting.

In [6], if a CSI-RS resource is referred N times by one or more CSI Reporting Settings not configured with higher layer parameter csi-ReportSubConfigList, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times. For Rel-18 Type II Doppler codebook-based CSI feedback, different time domain behaviour of CSI-RS resources leads to different active CSI-RS resource counting. For a periodic or semi-persistent CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted KP times, where the value of KP∈{1,2,4} is indicated by UE capability. For AI-based CSI prediction, such legacy CSI processing criteria could be used.

Proposal 10: For AI/ML based CSI prediction, the following CSI processing criteria could be considered:

- The CPU occupied by AI based CSI prediction and CSI reporting is set to wOcpu such that legacy CPU pool is reused, where Ocpu denotes the occupied CPU of no-AI based CSI prediction and CSI reporting and the values of w is indicated by UE capability reporting.

- Legacy active CSI-RS resource counting is reused.

Conclusions

In this contribution, the proposals and observation on UE side AI/ML model based CSI prediction are summarised as follows:

Observation 1: When UE’s velocity is configured 30 Km/h, for the AI/ML CSI model trained on a dataset with one TXRU mapping and doing inference on a dataset with different TXRU mapping, there is no performance degradation compared with the training dataset and test dataset generated with the same TXRU mapping.

Observation2：When UE’s velocity is configured 30 Km/h, for the AI/ML CSI model trained on a mixed dataset with different TXRU mapping and doing inference on those datasets with different TXRU mapping, there is no performance degradation compared with the training dataset and test dataset generated with the same TXRU mapping.

Observation 3: When UE’s velocity is configured 60 Km/h, for the AI/ML CSI model trained on a dataset with one TXRU mapping and doing inference on a dataset with different TXRU mapping, some performance loss (SGCS loss is about 4.14%~6.41%, NMSE loss is about 14.92%~20.87%) could be observed compared with the training dataset and test dataset generated with the same TXRU mapping.

Observation 4：When UE’s velocity is configured 60 Km/h, for the AI/ML CSI model trained on a mixed dataset with different TXRU mapping and doing inference on those datasets with different TXRU mapping, the performance loss could be reduced (SGCS loss is about 0.026%~2.04%, NMSE loss is about 0.125%~8.12%) compared with the training dataset and test dataset generated with the same TXRU mapping.

Proposal 1: NW’s additional condition is suggested to indicate to UE.

Proposal 2: For UE-side AI/ML model-based CSI prediction, the associated ID which is used to ensure consistency of training and inference for AI based CSI prediction could be studied as a starting point.

Proposal 3: Considering specification efforts and reporting overhead, at least type 3 (UE calculates the performance metrics and reports the metrics for NW making decision) is adopted to monitor model performance.

Proposal 4:  The following specification impact on monitoring performance reporting could be studied as a starting point:

- L1 signalling for reporting monitoring performance.

- Aperiodic, periodic, semi-persistent or UE-event reporting monitoring performance.

Proposal 5: For obtaining ground truth CSI to monitor performance, whether/how to enhance CSI-RS configuration/transmission enhancement needs to further study.

Proposal 6: For performance monitoring, the necessity and potential specification impacts including at least following aspects need to study:

- CSI processing criteria and timeline

Proposal 7: At least requested from UE for training data collection model should be supported.

Proposal 8: For training dataset collection, CSI-RS resource configuration with lower frequency density or less antenna ports could be supported to reduce CSI-RS signalling overhead.

Proposal 9: For model inference, CSI-RS resource configuration for Rel-18 Type II Doppler codebook based CSI feedback could be reused.

Proposal 10: For AI/ML based CSI prediction, the following CSI processing criteria could be considered:

- The CPU occupied by AI based CSI prediction and CSI reporting is set to wOcpu such that legacy CPU pool is reused, where Ocpu denotes the occupied CPU of no-AI based CSI prediction and CSI reporting and the values of w is indicated by UE capability reporting.

- Legacy active CSI-RS resource counting is reused.

References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, September. 9-12, 2024.

- 3GPP RAN1#118bis meeting, Chairman’s notes, October, 2024.

- 3GPP RAN1#114, Chairman’s notes, August, 2023.

- 3GPP RAN1#116-bis meeting, Chairman’s notes, April, 2024.

- 3GPP RAN1#117 meeting, Chairman’s notes, May, 2024.

- 3GPP TS38.214, June, 2024.