3GPP TSG-RAN WG1 Meeting #120	R1-2500751

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.3.3

Source:	Ericsson

Title:	SBFD CLI Handling

Document for:	Discussion, Decision

# 1	Introduction

The WID for SBFD [1] contains objectives for CLI handling in the following 3 areas:

- Information exchange among gNBs

- UL resource muting for PUSCH

- L1-based UE-to-UE CLI measurement and reporting

where the first 2 items relate to gNB-gNB CLI and the 3rd one to UE-UE CLI. In this contribution we discuss the RAN1 aspects of the 2nd and 3rd items. We don’t discuss the 1st item since that is already covered in an LS that was sent to RAN3 in RAN1#118.

# 2	UL Resource Muting

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - …

    - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

      - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

      - PUSCH resource mapping, i.e., rate-matching around the muted REs

      - UCI resource determination in symbols with muted REs

    - …

    - Note: Without dedicated optimization for dynamic/flexible TDD.

- …

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

- …

- Notes: Following are assumed for UL resource muting

  - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting

  - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212

  - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH

  - UL resource muting applies only for UEs in RRC_CONNECTED mode

  - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol

  - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols

  - No changes on TBS determination for PUSCH

  - Check impact on transmit signal quality/MPR requirement, if any, with RAN4

  - This feature is subject to UE capability

The WID contains the following objective and related notes corresponding to UL resource muting for handling gNB-gNB CLI:

## 2.1 	Configuration of ZP-SRS for UL Resource Muting

As stated in the WID extract shown above, UL resource muting for PUSCH shall be specified assuming comb-2 pattern in each allocated RB of PUSCH and in up to 2 symbols in the time domain. In this section we discuss the configuration of UL resource muting. As a starting point, we use the following agreement from RAN#116bis that states “The existing reference signal time-frequency resource pattern, e.g., comb-2 SRS, is the candidate for UL resource muting pattern.” We point out that PT-RS is not a candidate anymore since comb-2 pattern was agreed. We think that making use of legacy SRS configuration is attractive in order to minimize implementation burden and spec impact.

Agreement (RAN1#116bis)

If non-transparent UL resource muting is supported for interference covariance matrix measurement for gNB-to-gNB CLI handling, the following are recommended to be specified

- Definition and indication of UL resource muting pattern

- Collision with DMRS/PTRS

- PUSCH resource mapping, i.e., rate-matching around the muted REs

- UCI resource determination

- Power allocation in symbols with muted REs considering potential impact to phase continuity

- TB size determination

Note: The existing reference signal time-frequency resource pattern, e.g., PT-RS, comb-2 SRS, are the candidates for the UL resource muting pattern.

Note: Consider pattern without adverse impact on PAPR

Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4.

Note: The above does not apply for PUSCH transmission during random access procedures.

#### 7.4.1.5	CSI reference signals

##### 7.4.1.5.1	General

Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined

-	for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3

-	for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.

Since UL resource muting is equivalent to UL rate matching for PUSCH, we believe it makes sense to follow a similar approach as defined in current specifications for DL rate matching for PDSCH. For the latter, a zero-power (ZP) CSI-RS is defined around which PDSCH is rate matched. A ZP-CSI-RS resource is configured using the IE ZP-CSI-RS-Resource in 38.331, and the time/frequency mapping based on this configuration is described in the following section of 38.211:

Definition of a ZP-SRS resource to enable UL resource muting (equivalent to UL rate matching for PUSCH) should follow a similar principle as definition of a ZP-CSI-RS resource used for the purposes of DL rate matching for PDSCH.

For the configuration of ZP-SRS resource, we propose to define a new IE in 38.331, e.g., called ZP-SRS-Resource, following a similar principle as for definition of the ZP-CSI-RS-Resource. Following this principle, the new IE should contain only those parameters needed to configure the time and frequency location of the REs of the ZP-SRS resource. These parameters should define a comb-2 RE pattern in the frequency domain with configurable comb offset (0 or 1), and occupy one OFDM symbol. If muting is needed in two OFDM symbols, then two single-symbol ZP-SRS resources can be configured.

Regarding the configurability of the comb-offset, this was agreed in RAN1#119 as follows:

Agreement

For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

These agreements imply that if muting is configured in two OFDM symbols, that the comb-offset is the same for both symbols. This restriction can be handled easily by configuring the same value in each of the two ZP-SRS resources. In our view, it is preferrable to have consistent behavior / configuration regardless of whether or not CP-OFDM or DFT-s-OFDM is configured, hence we think the working assumption on the latter should be confirmed.

Confirm the Working Assumption from RAN1#119 on the configuration of a comb offset {0,1} for UL resource muting for DFT-s-OFDM.

Most importantly, within the new IE defined for ZP-SRS, the needed time/frequency location parameters should reuse the same names as the corresponding ones in a “normal” (legacy) SRS resource (relevant IE in 38.331 is SRS-Resource). The same approach is used for definition of ZP-CSI-RS. The reason to do this is so that the “Mapping to physical resources” section in 38.211 for a legacy SRS (38.211 Section 6.4.1.4.3) can be reused almost completely “as is” for defining the muting patterns. We believe that this minimizes the implementation burden for both gNBs and UEs since legacy SRS is already well-supported.

In RAN1#118bis, the following agreement was made regarding the muting bandwidth:

Agreement

If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol.

For a ZP-SRS resource, the above agreement can be achieved by configuring the bandwidth of the ZP-SRS resource to always occupy the the full BWP. Then muting is guaranteed to apply to all of the allocated PRBs of PUSCH, regardless of the actual PRB allocation. Analogous to ZP-CSI-RS which is only used for rate matching PDSCH (not other signals/channels), it can be specified that ZP-SRS is only used for muting PUSCH, i.e., thus any RBs outside the PUSCH allocation do not affect any other signals or channels.

At a high level, we propose the following:

For configuration of UL resource muting, support a new IE ZP-SRS-Resource in 38.331 for defining a zero-power SRS. A ZP-SRS resource occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP.

    - Note 1: If muting is to be applied in two OFDM symbols in a slot, two single-symbol ZP-SRS resources are configured/indicated for the slot, each with the same comb offset

    - Note 2: The The same parameter names for the needed time and frequency parameters of a ZP-SRS-Resource are reused from a legacy SRS resource configuration (SRS-Resource) which allows the procedure for mapping to physical resources defined in 38.211 Section 6.4.1.4.3 to be resused with minimal spec change.

Here we illustrate this proposal with some example ASN.1 encoding for a new IE for a zero-power SRS, e.g., called ZP-SRS-Resource. We point out that even if UL resource muting would be configured in some other way, e.g., based on some other new IE, parameters similar to all of the ones shown here would be needed to define the muting pattern anyway. Such an approach would have the drawback that new spec text would be needed in 38.211 to define the precise RE mapping of the muting pattern. This would be an unnecessary duplication since spec text for RE mapping for an SRS resource already exists and can be reused (see 38.211 Section 6.4.1.4.3).

ZP-SRS-Resource information element

ZP-SRS-Resource ::=                     	SEQUENCE {

transmissionComb                        	CHOICE {

n2                                      	SEQUENCE {

combOffset-n2                           	INTEGER (0..1)

},

...

},

resourceMapping                     		SEQUENCE {

startPosition                       		INTEGER (0..13),

nrofSymbols                         		ENUMERATED {n1, ...},

...

},

...

}

We emphasize that the parameters in the ZP-SRS-Resource IE reuse the same names as the corresponding ones in a legacy SRS resource in the SRS-Resource IE. This is key to enabling reuse of the legacy spec text in 38.211 Section 6.4.1.4.3. In the following list, the parameters marked in green are the reused ones in the ZP-SRS-Resource IE. The parameters marked in red are notably absent from the ZP-SRS-Resource IE.

- Time domain configuration:

  - Symbol position in slot (startPosition): configurable 0 .. 13

  - Number of symbols (nrofSymbols): fixed at 1 per ZP-SRS resource

    - Note: One can configure two ZP-SRS resources to achieve muting in 2 symbols

  - Since a ZP-SRS Resource is used for PUSCH rate matching, it is not necessary to define a resource type (aperiodic / semi-persistant / periodic) independently from PUSCH. Hence the parameter resourceType should be absent from the ZP-SRS-Resource IE

- Frequency domain configuration:

  - Comb configuration:

    - Comb-value (n2): fixed at 2

    - Comb-offset (combOffset-n2): configurable 0 or 1

  - Since the muting bandwidth does not need explicit configuration (as per above RAN1 agreement), the following two legacy parameters should be absent from the ZP-SRS-Resource IE:

    - Start PRB (freqDomainShift)

    - Number of PRBs (c-SRS)

    - Note: since neither c-SRS nor frequencyDomainShift are configured, 38.211 Section 6.4.1.4.3 should be amended to state that if these parameters are absent, the ZP-SRS resource occupies all PRBs of the BWP

  - Since frequency hopping is not relevant for UL resource muting, the following two legacy parameters should be absent from the ZP-SRS-Resource IE:

    - b-hop

    - b-SRS

    - Note: as per existing 38.211 Secton 6.4.1.4.3, if these parameters are absent, they default to 0, and since b-hop = b-SRS = 0, frequency hopping is disabled

### 2.1.1	Changes to 38.211 corresponding to ZP-SRS Configuration

In Appendix A, we illustrate the corresponding changes to 38.211 that are needed to define a ZP-SRS resource to achieve UL resource muting, and then to define PUSCH rate matching around an ZP-SRS. We find that the required changes are quite minimal, thus motivating using this approach rather than creating a new kind of resource which would anyway require similar parameters as for ZP-SRS, but would require additional spec text to describe configuration + procedures. We prefer to use existing spec as much as possible and have a parallel solution to ZP-CSI-RS.

Appendix A shows that Section 6.4.1.4 “Sounding reference signal” can reused almost “as is” for definition of a ZP-SRS resource. The only changes that are needed is to provide various references to the new ZP-SRS-Resource IE and provide default values for parameters that are not configured by this IE. Furthermore, the entire sub-section on sequence generation can be skipped, since the sequence values are zero because the ZP-SRS REs are muted.

With the definition of ZP-SRS in hand, Appendix C shows that all that is needed in 38.211 Section 6.3.1.6 “Mapping to virtual resource blocks” is to add some text to state the following:

        - REs of a ZP-SRS are not available for PUSCH transmission, i.e., are muted.

        - For the non-muted REs, additional power scaling is needed to ensure all OFDM symbols have the same transmission power in accordance with the WID that states “Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols” (see Section 2.5 for a discussion on EPRE determination).

We emphasize that this approach closely mirrors that for DL rate matching for PDSCH with ZP-CSI-RS as discussed earlier in this section.

## 2.2 	Determination of Time Location of UL Muting Symbol(s) in a Slot

In RAN1#118bis, the following agreement was made on the time location of UL muting symbols in a slot. The agreement contains 3 options.

Agreement (RAN1#118bis)

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH

- Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

- Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH.

FFS: details of semi-static configuration and dynamic indication (if supported)

FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI

In RAN1#119, Option 2 was down-selected amongst the above options:

Agreement (RAN1#119)

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

A benefit of Option 2 is that it avoids discussion and eventual hard coding in specifications of rules/conditions for muting. Several such conditions were discussed in a previous meeting:

Proposal 1-7 (LP)

Study the necessity of following conditions to apply UL resource muting for a PUSCH

  - Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

  - Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

  - Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

  - Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

  - Condition 5: UL resource muting is not applied according to the PHY priority of PUSCH

  - Condition 6: For PUSCH that is a retransmission, UL resource muting is not applied

  - Condition 7: For PUSCH that is scheduled by DCI format 0_0, UL resource muting is not applied

  - Condition 8: for PUSCH with repetition type B, UL resource muting is not applied

  - Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

  - Other conditions are not precluded.

With Option 2, we believe that none of these conditions need to be specified, since the gNB by implementation can be dynamically turned on/off. Based on the above discussion, we observe the following

With the agreement on Option 2, it is not necessary to further discuss conditions on UL resource muting as in Proposal 1-7 from RAN1#118bis.

### 2.2.1	Details of semi-static configuration and dynamic indication

The RAN1#118bis agreement shown in the previous section contains an FFS on details of semi-static configuration and dynamic indication for the time location of UL resource muting. First we observe that the RAN1#119 agreement contains the following wording: “the time location of each of one or two UL muting symbols is semi-statically configured.” As we discussed in the previous section, it is natural to define UL muting for a given OFDM symbol in a slot through semi-static configuration of a ZP-SRS resource. The ZP-SRS resource contains the time location in a slot through the legacy RRC parameter resourceMapping which contains the start symbol of the ZP-SRS resource (startPosition), and the number of symbols of the ZP-SRS resource (nrofSymbols) which is hard coded to 1. If it is desired to achieve muting in 2 OFDM symbols in a slot, then two ZP-SRS resources can be configured/indicated.

Here we show how Option 2 can be realized. First a list of one or two ZP-SRS resources is configured in PUSCH-Config, where each ZP-SRS resource corresponds to a separate OFDM symbol location. Then, a “Muting Indicator” parameter with value “enabled” is optionally configured in each row of the the semi-statically configured TDRA table (also configured in PUSCH-Config). With this approach, when the TDRA field in DCI dynamically indicates a row from the configured TDRA table, it automatically indicates whether or not muting in the slot shall occur. If the muting indiator parameter is enabled, muting is applied. If the muting indicator parameter is absent, no muting is applied.

Figure 1 shows an example in which a list of 2 ZP-SRS resources is configured and the TDRA table includes two rows for a given SLIV, one with muting enabled, and one with muting disabled. We observe that it is not strictly necessary to duplicate all rows of the TDRA table like this. For example, for short SLIVs, it may not be desirable to enable muting due to an increase in the effective code rate from rate matching, thus potentially causing a performance issue.

We note that exactly the same approach as illustrated here applies to Type-1 CG-PUSCH.

Configuration of list of ZP-SRS Resource(s) in PUSCH-Config

| List of ZP-SRS Resource(s) | Muting Symbol Location in Slot (startPosition in resourceMapping) |
| --- | --- |
| 1st ZP-SRS resource in list | 2 |
| 2nd ZP-SRS resource in list (if configured) | 10 |

(a)

TDRA table for Scheduling of Single PUSCH in PUSCH-Config

| Row index | PUSCH mapping type | K2 | S | L | Rate Matching (Muting) Indicator |
| --- | --- | --- | --- | --- | --- |
| 1 | Type A | 1 | 0 | 14 | Enabled |
| 2 | Type A | 1 | 0 | 14 | Absent |
| ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ |

(b)

Figure 1: Example configuration according to Option 2 for DG-PUSCH and both Type-1 and Type-2 CG-PUSCH.

Based on the above, discussion, we make the following pair of proposals:

For DG-PUSCH and both Type-1 and Type-2 CG-PUSCH, support semi-static configuration of a list of one or two ZP-SRS resources in PUSCH-Config, each corresponding to a different OFDM symbol location in a slot (parameter startPosition).

For single PUSCH scheduling without repetition, support optional configuration of a rate matching (muting) indicator parameter with value ‘enabled’ in any row of the configured TDRA table. If the rate matching indicator is present for the row, muting is enabled according to the configured list of one or two ZP-SRS resources. If the rate matching indicator is absent, muting is disabled for the row.

### 2.2.2	PUSCH Repetition and Multi-PUSCH Scheduling with Single DCI

In the RAN1#118bis agreement shown in Section 2.2, there is an FFS for how to handle PUSCH repetition and scheduling of multiple PUSCHs with a single DCI. Furthermore, in RAN1#119, the following agreement was made for PUSCH repetition (Type-A and Type-B) for Configuration 1:

Agreement

For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions

We think it makes sense that the same agreement should apply to multi-PUSCH scheduling with single DCI, hence we make the following aligned proposal for multi-PUSCH scheduling with single DCI:

Support the following proposal for multi-PUSCH scheduling with single DCI aligned with the Agreement from RAN1#119 on PUSCH repetition Type A:

For multi-PUSCH scheduling with single DCI in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCHs

For Configuration 2, where PUSCH(s) / PUSCH repetitions can occur in both SBFD and non-SBFD symbols, we think it makes sense to apply muting only in SBFD symbols. Hence we make the following aligned proposal for Configuration 2:

Support the following proposal for Configuration 2 aligned with the Agreement from RAN1#119 on Configuration 1:

For multi-PUSCH scheduling with single DCI and PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH(s)/PUSCH repetitions that overlap SBFD symbols. No muting is applied for PUSCH(s)/PUSCH repetitions that overlap non-SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions that overlap SBFD symbols. No muting is applied for actual repetitions that overlap non-SBFD symbols.

Based on the above proposals and agreement, it is clear that a very similar framework as discussed in Section 2.2.1 (see Proposal 4) can be used for enabling disabling rate matching (muting) for different PUSCH(s)/PUSCH repetitions. Specifically, a rate matching (muting) indicator parameter is optionally configured for each row of the TDRA table, and it can be specified that it applies only for all PUSCH(s) / PUSCH repetitions that overlap SBFD symbols.

For multi-PUSCH scheduling with single DCI and for PUSCH repetition Type-A and Type-B, support optional configuration of a rate matching (muting) indicator parameter with value ‘enabled’ in any row of the configured TDRA table. If the rate matching indicator is present for a row, muting is enabled for the row for all PUSCH(s) / PUSCH repetitions / PUSCH actual repetitions that overlap SBFD symbols. Muting is not applied for the row for any PUSCH / PUSCH repetition / PUSCH actual repetition that does not overlap SBFD symbols. If the rate matching indicator is absent, muting is disabled for the row.

## 2.3	Transform precoding for symbols with muting

In previous meetings, options for muting in conjunction with DFT-s-OFDM (transform precoding) were discussed, and the following agreement was made:

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

In the following we study the two options including two variants of Option 2, and discuss some pros and cons for each. We first describe our understanding of Option 1:

- Option 1 (Half-size DFT): The MscPUSCH/2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH/2, and the result is mapped to the MscPUSCH/2 unmuted scheduled subcarriers. The unmuted subcarriers may be even-indexed or odd-indexed subcarriers depending on the configured comb offset. A detailed example of how this can be described in the specification was provided in [2] and is included in Appendices B and C. We observe that the specification changes, particularly in Appendix C, are minor indeed.

For Option 2, we identify at least two different potential variants:

- Option 2a (Repetition): The block of MscPUSCH/2 modulated symbols is repeated, thus forming a length-MscPUSCH sequence of modulated symbols which is transformed to the frequency domain using a size-MscPUSCH DFT. Due to the repetition, the odd-indexed carriers at the output of the DFT are zero. The MscPUSCH/2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. The unmuted subcarriers are either the even-indexed or odd-indexed subcarriers depending on the configured comb offset. Option 2a is mathematically equivalent to Option 1 (see discussion later in this section).

- Option 2b (Block spreading): The MscPUSCH/2 modulated symbols are block spread by OCC [+1,+1] or [+1,-1] into a length-MscPUSCH sequence of modulated symbols, depending on whether the odd-indexed or even-indexed subcarriers should be muted, respectively, and then transformed to frequency domain using a size-MscPUSCH DFT. The result is then mapped to the MscPUSCH scheduled subcarriers.

There are at least two possible variants of Option 2 for transform precoding using DFT size MscPUSCH: 
 Option 2a: Repetition before the DFT, followed by extraction of even-indexed subcarriers and subsequent mapping to non-muted subcarriers.
 Option 2b: Block spreading by [+1,+1] or [+1,-1] before the DFT, depending on whether odd or even subcarriers should be muted, and the resulting modulated symbols then mapped to all scheduled subcarriers.

All options (Options 1, 2a, and 2b) maintain the same low cubic metric (CM) and peak-to-average power ratio (PAPR) as without muting. Given that Options 1 and 2a are mathematically equivalent (see discussion below), and seem more straight forward/natural than Option 2b, we think discussion should be focused on Options 1 and 2a.

For transform precoding for muting, discuss at least Options 1 and 2a and their respective merits.

Some companies expressed a concern about Option 1 since it uses a size MscPUSCH/2 DFT. While such a DFT should already be supported by the UE and gNB hardware for transmissions over a scheduled bandwidth of MscPUSCH/2, it might still be a complication to switch between DFTs of size MscPUSCH (in non-muted OFDM symbols) and MscPUSCH/2 (in muted OFDM symbols) within the same slot.

In contrast, Options 2a and 2b use a size MscPUSCH in both muted and non-muted symbols; however, the tradeoff is that in muted OFDM symbols Option 2a requires the extra step of repetition of the time domain sequence, and Option 2b requires the extra step of block spreading with OCCs.

Despite the differences in processing between Option 1 and Option 2a; it is important to note that these two options are mathematically equivalent (assuming the overall power scaling is adjusted correctly). This follows from the well-known mathematical fact that block repetition + DFT gives the same result as just a simple DFT, except that between each pair of elements, a zero element is interspersed (again, assuming appropriate overall scaling).

Given this equivalence, one potential way forward could be to write the specification such that is is consistent with both Option 1 and Option 2a, and let the implementing company choose which one to use depending on which is more convenient from a hardware perspective.

Options 1 and 2a are mathematically equivalent, and hence, the specifications can be written to be consistent with both. Then the implementing company may choose between DFT size MscPUSCH/2 (Option 1) and DFT size MscPUSCH (Option 2a) depending on which approach is better aligned with their existing hardware chains.

In Appendix B and C, we give an example of specification changes are consistent with Options 1 and 2a due to the mathematical equiavalence. We note that while Appendix B more closely follows the description of Option 1 above, it would be a simple matter to add some additional wording to make it more explicit that Option 2a is an equivalent implementation option.

- Appendix B (Example changes to 38.211 Clause 6.3.1.4)

  - The changes exmplified in Appendix B are specific to transform precoding (DFT-s-OFDM). The main change is related to the fact that with UL resource muting in one or more OFDM symbols, not all OFDM symbols carry the same number of PUSCH resources. However, such variation occurs also due to Rel-18 PT-RS, and one may therefore largely just duplicate part of the existing PT-RS-related paragraph to the non-PT-RS paragraph (Note that PT-RS is not included in the input sequence x00,…,x0Msymblayer-1 to this section). More specifically, if UL resource muting is performed in an OFDM symbol, a DFT is performed on the complex-valued symbols of each PUSCH OFDM symbol in a similar manner to legacy specifications, except that in the case of an OFDM symbol with muting, the DFT size may vary by a factor of two from OFDM symbol to OFDM symbol depending on whether or not UL resource muting is performed. The so processed complex-valued symbols are then distributed to non-muted REs according to the changes exemplified in Appendix C.

- Appendix C (Example changes to 38.211 Cluase 6.3.1.6)

  - The changes exemplified in Appendix C are the same irrespective of whether or not transform precoding is used. The two main changes are (i) an addition to the list of resources to skip in the mapping of PUSCH to resource elements., and (ii) additional power scaling to ensure all OFDM symbols have the same transmission power in accordance with the WID that states “Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols” (see Section 2.5 for a discussion on EPRE determination).

## 2.4	UL resource muting in DFT-s-OFDM symbols containing PTRS

In RAN1#119, agreements regarding symbols with DMRS and CP-OFDM symbols with PTRS were reached:

Agreement

If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Agreement

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

It is still open for discussion on UL resource muting in DFT-s-OFDM symbols with PTRS. We note the following proposed conclusion on this topic was discussed in a previous meeting:

Proposal 1-4-2 (conclusion)

If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and when transform precoding is enabled for the PUSCH, there is never collision between PTRS and UL muting resources since PTRS is inserted prior to DFT, and the output of DFT is mapped only to unmuted subcarriers.

In our contribution to RAN1#119, we raised a concern about potentially forbidding the configuration of PTRS time domain density L = 1 (i.e., PTRS in every OFDM symbol) in conjuction with RE muting. This concern applies especially to the DFT-s-OFDM PUSCH since we identify FR2 deployments as the more suitable targets for SBFD technologies from our extensive feasibility studies. For FR2 deployments, it is important to be able to configure PTRS in every symbol to achieve good phase tracking/compensation performance. It is also important to use DFT-s-OFDM PUSCH to improve UL coverage. Furthermore, as the conclusion in Proposal 1-4-2 shows, there is in fact not even a real collision between RE muting and PTRS. Therefore, we propose

Support RE muting with a symbol containing PT-RS for a PUSCH with transform precoding enabled.

For a DFT-s-OFDM PUSCH, the number of PTRS groups, NgroupPT-RS, and the number of samples per group, Nsampgroup, depend on the number of allocated RBs, NRB, per Table 6.2.3.2-1 in TS 38.214 shown below. The design allows for more groups and/or more samples per group for larger allocated bandwidth. The exact locations of the PTRS samples also depend on the allocated bandwidth, via the number of subcarriers, MscPUSCH=12⋅NRB, per Table 6.4.1.2.2.2-1 in TS 38.211.

Table 6.2.3.2-1: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

It is obvious that the actual carrying capacity of a symbol with RE muting is reduced by half in comparison to ones without RE muting. As a result, the number of PTRS groups, NgroupPT-RS, and the number of samples per group, Nsampgroup, can be determined first using NRB/2 and existing specifications. The locations of the PTRS samples can be similarly determined using MscPUSCH/2 and existing specifications. Specifically, the input to  Table 6.4.1.2.2.2-1 in TS 38.211 should be MscPUSCH/2 instead of MscPUSCH. There is no need for any other spec changes than applying the acutal frequency domain resource size with existing procedures for a PUSCH symbol with RE muting. We can conclude that the impact to the specs and UE implementation is minimal.

Note that, because of the implicit/explicit repetition of the transform coding for a DFT-s-OFDM symbol with RE muting as discussed in Section 2.3, the actual transmitted PTRS samples will be inline with the number of allocated RBs, NRB. That is, while fewer groups and/or fewer samples per group are first determined based on NRB/2, these PTRS samples located within the length-(MscPUSCH/2) samples will be implicitly/explicitly repeated in the full length-MscPUSCH samples for transmission. Therefore, the density of the PTRS samples intended by the existing specs will be maintained.

For RE muting in a DFT-s-OFDM PUSCH symbol, the UE determines the number of PTRS groups and the number of samples per group using NRB/2 and the locations of the PTRS samples using MscPUSCH/2 according to existing specs. There is no need for any other specs changes than applying the acutal frequency domain resource size with existing procedures.

## 2.5	PUSCH and PTRS EPRE Determination with UL Resource Muting for CP-OFDM

In RAN1#119 the following agreement was made regarding EPRE determination in a symbol with UL resource muting but without PTRS. 3 dB power boosting is applied to all REs that are not muted.

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

This agreement has an FFS on PUSCH and PTRS EPRE determination in a symbol with PTRS for the case of CP-OFDM (transform precoding not enabled). Here we discuss such EPRE determination. We start by noting that while the NR spec supports configuration of Qp PTRS ports, where Qp can be 1 or 2, in the analysis below we only consider the case of Qp=1 ports. This is because if muting is applied to a symbol containing a 2-port PTRS, one of the two ports will always collide with a muted RE, and this is not supported according to the following agreement from RAN1#119:

Agreement

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

If any UL muting symbol overlaps a symbol containing PTRS and transform precoding is not enabled for the PUSCH, configuration of only a single PTRS port (Qp=1) is supported. Configuration of two PTRS ports (Qp=2) is not supported  since it would result in a collision of one of the two ports with a muted RE which is not expected by the UE.

TS 38.214 Section 6.2.3.1 contains the below table which gives the PUSCH-to-PTRS power ratios for the legacy case where UL resource muting is not supported. The PUSCH-to-PTRS power ratio is defined per RE per PUSCH layer as

ρPTRSPUSCH=EPUSCH,lEPTRS=10-αPTRSPUSCH10

where EPTRS is the EPRE for PTRS, EPUSCH,l=EPUSCHL is the PUSCH EPRE for the l-th PUSCH layer, EPUSCH is the aggregate PUSCH EPRE over all L layers, and αPTRSPUSCH is the PTRS boosting factor (in dB) in the table.

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE  other than 8TX PUSCH transmission and other than SDM PUSCH

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

Let NRB be the number of RBs allocated to PUSCH and N=12∙NRB be the number of allocated REs. Furthermore, let M=NRBK be the number of subcarriers occupied by PTRS where K∈2,4 is the configured PTRS density in the frequency domain. This results in the PTRS overhead ratio as MN=112∙K.

### 2.5.1	Analysis for symbols without muting

The total energy across the PUSCH bandwidth in a symbol without PTRS is simply

EtotWithout PTRS, No Muting=N∙EPUSCH

The total energy across the PUSCH bandwidth in a symbol with PTRS is

EtotWith PTRS, No Muting=M∙EPTRS+l=1LN-MEPUSCH,l
=ML10αPTRSPUSCH10+N-MEPUSCH

This equation accounts for the fact that for the case of Qp=1 PTRS port, only one layer carries non-zero power PTRS REs. The remaining L – 1 layers have zero power at the PTRS RE locations.

In the following analysis, we assume that the aggregate PUSCH EPRE across layers EPUSCH is invariant across symbols of the PUSCH allocation, regardless of the presence/absence of PTRS in a symbol. With this assumption, it follows that the total energy across the allocated PUSCH bandwith may vary between symbols depending on the presence/absence of PTRS in a symbol. Taking the ratio of the above two expressions results in the following:

EtotWith PTRS, No MutingEtotWithout PTRS, No Muting=1-112K1-1L10αPTRSPUSCH10

Observing 38.211 Table 6.2.3.1-3 shown in the previous section, one can see that if the 2nd row in the table is configured (UL-PTRS-power = ‘01’), then 10αPTRSPUSCH10=L, and the total energy ratio becomes unity regardless of the number of layers L. In other words there is no difference in the total energy between a symbol with PTRS and a symbol without PTRS when the PTRS is boosted. This is captured in the last two rows of Table 1 where the power difference is shown as 0 dB.

In contrast, if the first row is configured (UL-PTRS-power = ‘00’) for which PTRS is not boosted, 10αPTRSPUSCH10=1 and then the total energy ratio is non-unity. Table 1 below shows that the total energy in a symbol with PTRS is slightly less than in a symbol without PTRS, and the difference depends on the PTRS frequency domain density K and number of layers L. The maximum difference (-0.14 dB) occurs for the case of higher PTRS density (K = 2) and L = 4 PUSCH layers.

Table 1: Difference (in dB) between the total energy in a symbol with PTRS vs. a symbol without PTRS assuming Qp=1 PTRS port and non-coherent UEs in 38.211 Table 6.2.3.1-3

| UL-PTRS-power | PTRS Frequency Domain Density (K) | Difference (in dB) between the total energy in a symbol with PTRS vs. a symbol without PTRS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | L = 1 | L = 2 | L = 3 | L = 4 |
| 00 | 2 | 0 | -0.091 | -0.12 | -0.14 |
|  | 4 | 0 | -0.046 | -0.061 | -0.068 |
| 01 | 2 | 0 | 0 | 0 | 0 |
|  | 4 | 0 | 0 | 0 | 0 |

In legacy NR (no UL resource muting), the total energy across the allocated PUSCH bandwidth may be less by a fraction of a dB in a symbol with PTRS compared to a symbol without PTRS depending on the configured PTRS boosting factor (UL-PTRS-power = ’00’ or ‘01’). For UL-PTRS-power = ’00’, the energy difference depends on the PTRS frequency domain density K and the number of layers L.

### 2.5.2	Analysis for symbols with muting

Now turning to the case of UL muting, the total energy across the PUSCH bandwidth in a symbol without PTRS is simply

EtotWithout PTRS + Muting=N2∙ρ1∙EPUSCH
= N∙EPUSCH

where ρ1=2 is a boosting factor applied in a symbol without PTRS to boost the PUSCH EPRE by 3 dB to account for the fact that half of the REs are muted. This keeps the total PUSCH power equivalent to the case without muting as required by the WID.

For symbol with PTRS, the total energy acoss the PUSCH bandwidth is

EtotWith PTRS + Muting=M∙ρ2∙EPTRS+l=1LN2-M∙ρ2∙EPUSCH,l
=ρ2ML10αPTRSPUSCH10+N2-MEPUSCH

where ρ2 is a common boosting factor applied to both PUSCH and PTRS REs in a symbol with PTRS.

Like in the previous section, here we assume that the aggregate PUSCH EPRE across layers EPUSCH is the same in a muted symbol regardless of the presence/absence of PTRS. Taking the ratio of the above two expressions results in the following:

EtotWith PTRS + MutingEtotWithout PTRS + Muting=ρ221-16K1-1L10αPTRSPUSCH10

If one chooses the boosting factor ρ2=ρ1=2 (i.e., 3 dB boosting) Table 2 below shows the power ratio (in dB) for both the case of 10αPTRSPUSCH10=1 (UL-PTRS-power = ‘00’) and 10αPTRSPUSCH10=L (UL-PTRS-power = ‘01’). As can be seen, for the case of UL-PTRS-power = ‘01’, there is no difference (0 dB) in the total energy between symbols with and without PTRS due to the fact that PTRS is boosted. However, for the case of UL-PTRS-power = ’01,’ the the total energy in a symbol with PTRS is slightly less than in a symbol without PTRS, and the difference depends on the PTRS frequency domain density K and number of layers L. The maximum difference (-0.28 dB) occurs for the case of higher PTRS density (K = 2) and L = 4 PUSCH layers. This is compared to a maximum difference of -0.14 dB for legacy NR (no UL muting). Despite the larger difference compared to legacy NR, the energy difference it is still small. The benefit is that a common bossting factor of 3 dB is used for all settings of UL-PTRS-power, K, and L.

Table 2: Difference (in dB) between the total energy in a symbol with muting + PTRS vs. a symbol with muting and no PTRS when applying 3 dB boosting of PTRS and PUSCH REs (ρ2=2) assuming Qp=1 PTRS port and non-coherent UEs in 38.211 Table 6.2.3.1-3.

| UL-PTRS-power | PTRS Frequency Domain Density (K) | Difference (in dB) between the total energy in a symbol with muting + PTRS vs. a symbol with muting and no PTRS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | L = 1 | L = 2 | L = 3 | L = 4 |
| 00 | 2 | 0 | -0.19 | -0.25 | -0.28 |
|  | 4 | 0 | -0.091 | -0.12 | -0.14 |
| 01 | 2 | 0 | 0 | 0 | 0 |
|  | 4 | 0 | 0 | 0 | 0 |

If 3 dB boosting of both PTRS and PUSCH REs is used in an symbol with UL resource muting, there can be a slightly larger difference in the total energy across the PUSCH bandwidth between a symbol with PTRS and a symbol without PTRS compared to the difference observed for legacy NR with no UL muting. The maximum difference is -0.28 dB compared ot legacy NR where the maximum is -0.14 dB.

If the slight increase in total energy variation for the case of muting is unacceptable (we don’t think it is), then this power variation can be reduced to the same level as for legacy NR (no UL muting) by equating the above expression for the total energy ratio to the corresponding one in the previous section and solving for ρ2. The result is

ρ2=2∙1-112K1-1L10αPTRSPUSCH101-16K1-1L10αPTRSPUSCH10

The values of ρ2 are tabulated in Table 3. As expected if UL-PTRS-power = ‘01’, the boosting factor remains as ρ2=2 (3.01 dB) regardless of K and L. However, if UL-PTRS-power = ‘01’, the boosting factor becomes slight larger than 2 (by at most 0.14 dB).

Table 3: Adjusted boosting factor ρ2 (in dB) to reduce the energy difference between a symbol with and without PTRS to the same level as for legancy NR (no UL muting) shown in Table 1.

| UL-PTRS-power | PTRS Frequency Domain Density (K) | Boosting factor ρ2 (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | L = 1 | L = 2 | L = 3 | L = 4 |
| 00 | 2 | 3.01 | 3.10 | 3.14 | 3.15 |
|  | 4 | 3.01 | 3.06 | 3.07 | 3.08 |
| 01 | 2 | 3.01 | 3.01 | 3.01 | 3.01 |
|  | 4 | 3.01 | 3.01 | 3.01 | 3.01 |

With the above adjusted boosting factor ρ2, the total energy variaton between symbols with muting with and without PTRS becomes the same as shown in Table 1, i.e., same as legacy NR (no UL muting). The tradeoff, however, is that a different boosting factor is needed for each combination of PTRS density K and number of layers L. It is simpler to just use 3 dB boosting in all cases, and as we observed previously the slight increase in energy difference between symbols is insignificant indeed.

RAN1 to down-select to one of the following boosting approaches for REs in symbol a symbol with muting:

  - Alt-1: 3 dB boosting for all unmuted REs (both PUSCH and PTRS, if present), regardless of the PTRS density K and number of PUSCH layers L

  - Alt-2: 3 dB boosting for all unmuted REs in a symbol without PTRS and X dB boosting for all unmuted REs in a symbol with PTRS, where X ≥ 3 dB and depends on the configured value of UL-PTRS-power, PTRS density K, and the used number of PUSCH layers L.

## 2.6	UCI Resource Determination

In the most recent two meetings, the following agreements were made related to UCI resource determination:

Agreement (RAN1#119)

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement (RAN1#118bis)

For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

- Option 2: The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on MscUCIl after excluding the UL muting resource.

In section 38.212 Section 6.3.2.4.1 on rate matching for Polar-code encoded UCI bits multiplexed on PUSCH, calculations of the number of REs that are available for UCI transmission in an OFDM symbol are specified for various UCI types in various application scenarios. The essence of the updates required in calculations for UL resource muting is to reduce the number of REs available for UCI transmission when UL resource muting is present in the same OFDM symbol as UCI. The detailed discussion presented below is based on the specification for HARQ-ACK in Section 6.3.2.4.1.1, and can be generalized and applied to the other UCI types and the other UCI scenarios in Section 6.3.2.4.1 (i.e., CSI part 1, CSI part 2, CG-UCI, etc.).

First of all, UCI can not be transmitted in a OFDM symbol that carries DMRS. In a PUSCH symbol that doesn’t carry DMRS, the number of REs available for UCI transmission is calculated as below.

For HARQ-ACK transmission on a basic PUSCH, assuming

- MscPUSCH is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

- MscPT-RSl is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PUSCH transmission;

- MscUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l.

For any OFDM symbol that doesn’t carry DMRS, the number of REs available for UCI transmission is calculated as

MscUCIl= MscPUSCH-MscPT-RSl

Based on the above agreement from RAN1#118bis, if UL resource muting occurs in the OFDM symbol (e.g., via presence of ZP-SRS), then calculation for the number of REs available for UCI transmission should be updated as

MscUCIl= MscPUSCH-MscPT-RSl-MscZP-SRSl

where MscZP-SRSl is the number of resource elements in OFDM symbol l allocated to UL resource muting (e.g., via ZP-SRS).

The existing calculations of number of REs available for UCI transmission in a PUSCH symbol is specified in section 6.3.2.4.1 in TS 38.212. The following update is needed to align with the agreement in RAN1#118bis: MscUCIl= MscPUSCH-MscPT-RSl-MscZP-SRSl

In TS38.212 Section 6.2.7, the following relevant text still holds (no modification needed) with the understanding that “the set of resource elements … available for transmission of data” and “the set of resource elements … available for transmission of UCI” implicitly excludes muted REs (e.g., REs of a ZP-SRS).

- Denote  as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for .

- Denote MscUL-SCHl=ΦlUL-SCH as the number of elements in set . Denote  as the -th element in .

- Denote  as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote MscUCIl=ΦlUCI as the number of elements in set . Denote  as the -th element in . For any OFDM symbol that carriers DMRS of the PUSCH, . For any OFDM symbol that does not carry DMRS of the PUSCH, .

We observe that this is consistent with the Note in the above quoted RAN1#119 agreement.

No specification changes are needed to Section 6.2.7 of TS 38.212 in order to support UL resource muting in a PUSCH symbol that carries UCI. This is consistent with the RAN1#119 agreement.

# 3	L1-Based UE-UE CLI Measurement and Reporting

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - …

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

      - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

      - At least aperiodic reporting

      - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - UCI bits generation

      - Priority rules for multiple CSI reporting

      - CLI measurement accuracy requirement

    - Note: Without dedicated optimization for dynamic/flexible TDD.

- …

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

- …

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

  - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting

  - For the new reporting quantities, at least wideband reporting is supported

  - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

The WID contains the following objective and related notes corresponding to L1-based UE-UE CLI measurement are reporting:

## 3.1	Measurement Configuration

In RAN1#119, the following agreements were made regarding configuration of CLI measurement resources for both SRS-RSRSP and CLI-RSSI measurement:

Agreement

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

In these agreements there is an FFS on “Other parameters.” The discussion was on whether or not it is necessary to configure a reference BWP, reference serving cell, or reference SCS, similar to the measurement resources for L3-based CLI measurement defined in Rel-16, i.e., SRS-ResourceConfigCLI-r16 and RSSI-ResourceConfigCLI-r16. In our view, it is not necessary to define this additional parameters since the generally the L1-based measurement correspond to the UEs serving cell, active BWP and corresponding SCS.

Conclude that it is not necessary to define a reference BWP, reference serving cell, or reference SCS in the measurement resource configurations for L1-based SRS-RSRP and CLI-RSSI measurement.

## 3.2	Measurement Reporting

### 3.2.1	Periodic and semi-persistent CLI reporting

In RAN1#118 [1] the following proposal was discussed in detail (but not agreed):

Proposal 2-3

For L1 UE-to-UE CLI measurement and reporting, at least the following are supported

- Aperiodic CLI reporting on PUSCH

- Periodic CLI reporting on PUCCH

- FFS: Semi-persistent CLI reporting over PUCCH or PUSCH.

- FFS: Aperiodic CLI reporting on PUCCH.

Regarding the 1st sub-bullet, aperiodic CLI reporting on PUSCH was already been agreed in RAN1#118bis and can be based on aperiodic, semi-persistent, and periodic CLI measurement resources. Since UE-UE CLI is typically bursty and further is not a dominant issue for operation of SBFD, in our view support for aperiodic reporting is sufficient. It is not necessary to have frequent and regular reporting on the UE/UE CLI as would occur with periodic and/or semi-persistent reporting. If the gNB still desires some form of regular report, the gNB can infer from periodic/semi-persistent CSI reports if there is a CLI problem, e.g., due to low CQI. It can then trigger an aperiodic CLI report to receive an interference only measurement. Hence we propose that neither the 2nd sub-bullet nor the 3rd sub-bullet is supported.

For measurement reporting for UE-UE CLI mitigation, neither of the following is supported: semi-persistent reporting (on PUSCH or on PUCCH) and periodic reporting on PUCCH.

Regarding the 4th sub-bullet, we note that it was concluded in RAN1#119 that ther is no consensus to support this:

Conclusion

There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

### 3.2.2	Number of measurement reports

In RAN1#119, the following was agreed regarding number of reported measurements:

Agreement

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported

- The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4}

- The M CLI measurement resource ID(s) are reported

- The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported

- When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference

- FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

- FFS: whether/how to report differential value in case the largest measurement is out of range

There are two FFSs that need to be addressed in this agreement. Regarding the first one on “other reporting criteria,” in our view we think it is sufficient to support reporting only the M largest (most interfered) CLI measurements. We don’t think being able to configure reporting of M the smallest (least interfered) measruements is particularly useful, as these measurements may be “in the noise” and thus not have particular value to the gNB.

Conclude that besides the agreed reporting of the M most-interfered CLI measurements, “other reporting criteria” are not supported.

Regarding the second FFS, we understand that the concern is that if the largest measurement is out of range, then it is not possible for the UE to calculate the differential values for the other report. We question how likely this is given that the range may be defined sufficiently large enough to avoid this problem. By analogy, we note that there is no special handling for this case in legacy L1-RSRP and L1-SINR measurement reporting. Hence, we don’t think it is necessary to introduce special handling mechanisms, perhaps other than to define an “out-of-range” value for the largest report in which case the UE can choose dummy values for the differential reports and the gNB can just ignore those dummy values.

If the largest CLI measurement is out of range, the UE reports “dummy” differential values for the other measurements.

### 3.2.3	Range and resolution of measurement reports

In RAN1#118bis, the following proposal was discussed but not agreed:

Proposal 2-9

The range and resolution of L3 based CLI measurement report is reused for L1 based CLI measurement

  - CLI-RSSI: -100 dBm to -25 dBm with 1 dB step size

  - SRS-RSRP: -140 dBm to -44 dBm with 1 dB step size

As an analogy, we note that all of the details of L3-based CLI measurement defined in Rel-16 are contained in RAN4 specs (38.133) including range and resolution of the reports. Hence it is for RAN4 to decide on the same for L1-based CLI measurement. We note that RAN4 is better positioned since they are in the process of running simulations for establishing CLI accuracy requirements already which can influence the range and resolution of the reports.

RAN4 to discuss and decide on range and resolution of CLI measurement reports.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Definition of a ZP-SRS resource to enable UL resource muting (equivalent to UL rate matching for PUSCH) should follow a similar principle as definition of a ZP-CSI-RS resource used for the purposes of DL rate matching for PDSCH.

Observation 2	With the agreement on Option 2, it is not necessary to further discuss conditions on UL resource muting as in Proposal 1-7 from RAN1#118bis.

Observation 3	There are at least two possible variants of Option 2 for transform precoding using DFT size MscPUSCH:   Option 2a: Repetition before the DFT, followed by extraction of even-indexed subcarriers and subsequent mapping to non-muted subcarriers.  Option 2b: Block spreading by [+1,+1] or [+1,-1] before the DFT, depending on whether odd or even subcarriers should be muted, and the resulting modulated symbols then mapped to all scheduled subcarriers.

Observation 4	Options 1 and 2a are mathematically equivalent, and hence, the specifications can be written to be consistent with both. Then the implementing company may choose between DFT size MscPUSCH/2 (Option 1) and DFT size MscPUSCH (Option 2a) depending on which approach is better aligned with their existing hardware chains.

Observation 5	If any UL muting symbol overlaps a symbol containing PTRS and transform precoding is not enabled for the PUSCH, configuration of only a single PTRS port (Qp=1) is supported. Configuration of two PTRS ports (Qp=2) is not supported  since it would result in a collision of one of the two ports with a muted RE which is not expected by the UE.

Observation 6	In legacy NR (no UL resource muting), the total energy across the allocated PUSCH bandwidth may be less by a fraction of a dB in a symbol with PTRS compared to a symbol without PTRS depending on the configured PTRS boosting factor (UL-PTRS-power = ’00’ or ‘01’). For UL-PTRS-power = ’00’, the energy difference depends on the PTRS frequency domain density K and the number of layers L.

Observation 7	If 3 dB boosting of both PTRS and PUSCH REs is used in an symbol with UL resource muting, there can be a slightly larger difference in the total energy across the PUSCH bandwidth between a symbol with PTRS and a symbol without PTRS compared to the difference observed for legacy NR with no UL muting. The maximum difference is -0.28 dB compared ot legacy NR where the maximum is -0.14 dB.

Observation 8	The existing calculations of number of REs available for UCI transmission in a PUSCH symbol is specified in section 6.3.2.4.1 in TS 38.212. The following update is needed to align with the agreement in RAN1#118bis: MscUCIl= MscPUSCH-MscPT-RSl-MscZP-SRSl

Observation 9	No specification changes are needed to Section 6.2.7 of TS 38.212 in order to support UL resource muting in a PUSCH symbol that carries UCI. This is consistent with the RAN1#119 agreement.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm the Working Assumption from RAN1#119 on the configuration of a comb offset {0,1} for UL resource muting for DFT-s-OFDM.

Proposal 2	For configuration of UL resource muting, support a new IE ZP-SRS-Resource in 38.331 for defining a zero-power SRS. A ZP-SRS resource occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP.

	Note 1: If muting is to be applied in two OFDM symbols in a slot, two single-symbol ZP-SRS resources are configured/indicated for the slot, each with the same comb offset

	Note 2: The The same parameter names for the needed time and frequency parameters of a ZP-SRS-Resource are reused from a legacy SRS resource configuration (SRS-Resource) which allows the procedure for mapping to physical resources defined in 38.211 Section 6.4.1.4.3 to be resused with minimal spec change.

Proposal 3	For DG-PUSCH and both Type-1 and Type-2 CG-PUSCH, support semi-static configuration of a list of one or two ZP-SRS resources in PUSCH-Config, each corresponding to a different OFDM symbol location in a slot (parameter startPosition).

Proposal 4	For single PUSCH scheduling without repetition, support optional configuration of a rate matching (muting) indicator parameter with value ‘enabled’ in any row of the configured TDRA table. If the rate matching indicator is present for the row, muting is enabled according to the configured list of one or two ZP-SRS resources. If the rate matching indicator is absent, muting is disabled for the row.

Proposal 5	Support the following proposal for multi-PUSCH scheduling with single DCI aligned with the Agreement from RAN1#119 on PUSCH repetition Type A:

Proposal 6	Support the following proposal for Configuration 2 aligned with the Agreement from RAN1#119 on Configuration 1:

Proposal 7	For multi-PUSCH scheduling with single DCI and for PUSCH repetition Type-A and Type-B, support optional configuration of a rate matching (muting) indicator parameter with value ‘enabled’ in any row of the configured TDRA table. If the rate matching indicator is present for a row, muting is enabled for the row for all PUSCH(s) / PUSCH repetitions / PUSCH actual repetitions that overlap SBFD symbols. Muting is not applied for the row for any PUSCH / PUSCH repetition / PUSCH actual repetition that does not overlap SBFD symbols. If the rate matching indicator is absent, muting is disabled for the row.

Proposal 8	For transform precoding for muting, discuss at least Options 1 and 2a and their respective merits.

Proposal 9	Support RE muting with a symbol containing PT-RS for a PUSCH with transform precoding enabled.

Proposal 10	For RE muting in a DFT-s-OFDM PUSCH symbol, the UE determines the number of PTRS groups and the number of samples per group using NRB/2 and the locations of the PTRS samples using MscPUSCH/2 according to existing specs. There is no need for any other specs changes than applying the acutal frequency domain resource size with existing procedures.

Proposal 11	RAN1 to down-select to one of the following boosting approaches for REs in symbol a symbol with muting:

	Alt-1: 3 dB boosting for all unmuted REs (both PUSCH and PTRS, if present), regardless of the PTRS density K and number of PUSCH layers L

	Alt-2: 3 dB boosting for all unmuted REs in a symbol without PTRS and X dB boosting for all unmuted REs in a symbol with PTRS, where X ≥ 3 dB and depends on the configured value of UL-PTRS-power, PTRS density K, and the used number of PUSCH layers L.

Proposal 12	Conclude that it is not necessary to define a reference BWP, reference serving cell, or reference SCS in the measurement resource configurations for L1-based SRS-RSRP and CLI-RSSI measurement.

Proposal 13	For measurement reporting for UE-UE CLI mitigation, neither of the following is supported: semi-persistent reporting (on PUSCH or on PUCCH) and periodic reporting on PUCCH.

Proposal 14	Conclude that besides the agreed reporting of the M most-interfered CLI measurements, “other reporting criteria” are not supported.

Proposal 15	If the largest CLI measurement is out of range, the UE reports “dummy” differential values for the other measurements.

Proposal 16	RAN4 to discuss and decide on range and resolution of CLI measurement reports.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Rapporteur (Huawei), June 2024.

R1-2408527, “SBFD CLI Handling,” Ericsson, RAN1 #118bis, Hefei, October 2024.

# Appendix A:	Example changes to 38.211 to support Zero-Power SRS

*** Begin changes to 38.211 Section 6.4.1.4 ***

6.4.1.4	Sounding reference signal

6.4.1.4.1	SRS resource

An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and a zero-power (ZP) SRS resource is configured by the ZP-SRS-Resource IE and consists of

-	NapSRS∈1,2,4,8 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook'.

-	Nhop, the number of hops for SRS Tx hopping for an SRS resource configured by SRS-PosResource and given by the higher layer parameter numberOfHops if configured, otherwise Nhop=1.

-	NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbol per hop.

-	l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1. If Nhop>1 l0 is the starting position of each hop in the time domain, determined by the field startPosition for each SRS transmission hop.

-	k0, the frequency-domain starting position of the sounding reference signal.

6.4.1.4.2	Sequence generation

For a ZP-SRS resource configured by the ZP-SRS-Resource IE, sequence generation in this clause is not relevant.

*** Unchanged text omitted ***

6.4.1.4.3	Mapping to physical resources

Throughout this clause, when the higher layer parameter numberOfHops is provided for SRS-PosResource, the sounding reference signal sequence definitions applies to a given hop.

When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with  to resource elements  in a slot for each of the antenna ports  according to

aKTCk'+k0pi,  l'+l0(pi)=1NapβSRSrpi(k',l')if k'=0, 1, …, Msc,bSRS-1 and l'=0,1,…,NsymbSRS-10otherwise

For a ZP-SRS resource, the values of the sounding reference signal sequence rpi(n,l') are all zero. The length of the sounding reference signal sequence is given by

Msc,bSRS=mSRS,bNscRBKTCPF

where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with  where  is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index  given by the field c-SRS contained in the higher-layer parameter freqHopping, if configured. If neither c-SRS nor b-SRS are configured, b = 0 and mSRS,0 is given by the number of RBs of the BWP. The quantity PF ∈2, 4 is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6.

*** Unchanged text omitted ***

The frequency domain shift value nshift adjusts the SRS allocation with respect to the reference point grid and is contained in the higher-layer parameter freqDomainShift in the SRS-Resource IE or the SRS-PosResource IE. If the freqDomainShift is not configured, nshift=0. The transmission comb offset kTC∈0,1,…,KTC-1 is contained in the higher-layer parameter transmissionComb in the SRS-Resource IE or the SRS-PosResource IE or the ZP-SRS-Resource IE and nb is a frequency position index.

Frequency hopping of the sounding reference signal is configured by the parameter bhop∈0,1,2,3, given by the field b-hop contained in the higher-layer parameter freqHopping if configured, otherwise bhop=0.

If bhop≥BSRS, frequency hopping is disabled and the frequency position index nb remains constant (unless re-configured) and is defined by

for all NsymbSRS OFDM symbols of the SRS resource. The quantity  is given by the higher-layer parameter freqDomainPosition if configured, otherwise nRRC=0, and the values of mSRS,b and Nb for b=BSRS are given by the selected row of Table 6.4.1.4.3-1 corresponding to the configured value of , if provided. If neither c-SRS nor b-SRS are configured, b = 0 and N0=1.

*** End changes ***

# Appendix B: Example changes to 38.211 Clause 6.3.1.4 “Transform Precoding”

*** Begin changes to 38.211 Section 6.3.1.4 ***

If transform precoding is not enabled according to 6.1.3 of [6, TS38.214],  for each layer .

If transform precoding is enabled according to 6.1.3 of [6, TS38.214], υ=1 and  depends on the configuration of phase-tracking reference signals.

If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are not being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 for the single layer λ=0 shall be divided into MsymblayerMscPUSCH sets, each set corresponding to one OFDM symbol, andx(0)(i)=x(0)(i). where set l contains Msc,lPUSCH symbols and is mapped to the complex-valued symbols xl0i', corresponding to OFDM symbol l prior to transform precoding, with i'∈0,1,…,Msc,lPUSCH -1.

If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 shall be divided into sets, each set corresponding to one OFDM symbol, and where set  contains Msc,lPUSCH-εlNsampgroupNgroupPTRS symbols and is mapped to the complex-valued symbols xl(0)(lMscPUSCH+i') corresponding to OFDM symbol  prior to transform precoding, with i'∈0,1,…,Msc,lPUSCH-1 and . The index m of PT-RS samples in set , the number of samples per PT-RS group Nsampgroup, and the number of PT-RS groups NgroupPT-RS are defined in clause 6.4.1.2.2.2. The quantity  when OFDM symbol  contains one or more PT-RS samples, otherwise .

Transform precoding shall be applied according to

y(0)yl(0)l∙MscPUSCH+k=1Msc,lPUSCHi=0Msc,lPUSCH-1xl(0)l∙MscPUSCH+ie-j2πikMsc,lPUSCH

k=0,…, Msc,lPUSCH-1

l=0,…,MsymblayerMscPUSCH-1

resulting in a set of blocks of complex-valued symbols yl(0) that shall be concatenated in order of increasing l to form y00,…,y0Msymblayer-1. The variable, where  represents the bandwidth of the PUSCH in terms of resource blocks, and shall fulfil

where  is a set of non-negative integers. The variable Msc,lPUSCH equals MscPUSCH/2 when OFDM symbol l is occupied by a ZP-SRS resource, otherwise Msc,lPUSCH=MscPUSCH.

*** End changes ***

# Appendix C: Example changes to 38.211 Clause 6.3.1.6 “Mapping to virtual resource blocks”

Note: in the following example, we assume Alt-1 in Proposal 11, i.e., 3 dB boosting of all unmuted REs in a symbol with UL resource muting. This translates to the scale factor 2 of shown below.

*** Begin changes to 38.211 Section 6.3.1.6 ***

For each of the antenna ports used for transmission of the PUSCH, each symbol in the block of complex-valued symbols  shall be multiplied with 2 if the symbol corresponds to an OFDM symbol l occupied by a ZP-SRS resource, and by 1 otherwise, and further be multiplied with the amplitude scaling factor βPUSCH in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with  to resource elements (k',l)p,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria:

-	they are in the virtual resource blocks assigned for transmission, and

-	the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, or DM-RS intended for other co-scheduled UEs as described in clause 6.4.1.1.3, and are not mapped to a ZP-SRS resource.

The mapping to resource elements (k',l)p,μ allocated for PUSCH according to [6, TS 38.214] shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index , with the starting position given by [6, TS 38.214].

*** End changes ***