3GPP TSG RAN WG1 #120	  																		R1-2500802

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.15.2

Source: 	Apple Inc.

Title: 	Views on UE features for NR MIMO Phase 5

Document for:	Discussion/Decision

- Views on UE features for Rel-19 NR MIMO Phase 5

In this contribution, we present our high level views on UE features for Rel-19 NR MIMO Phase 5 based on the endorsed session note in RAN1#119 with track change on. At high level, the FGs are organized in the following way to handle different objectives/agendas in NR Rel-19 MIMO Phase 5.

- FG59-1x covers the enhancement for UE-initiated/event driven beam management

- FG59-2x covers the CSI enhancement

  - FG59-2-1x covers Rel-15 Type-I SP (single panel) codebook enhancement, Rel-15 Type-I MP (multi panel) codebook enhancement, Rel-16 eType-II codebook enhancement, Rel-17 FeType-II PS (port selection) codebook enhancement and Rel-18 Type-II Doppler codebook enhancement for up to 128 ports

  - FG59-2-2x covers CRI enhancement for Rel-15 Type-I SP codebook and  Rel-16 eType-II codebook for up to 128 ports

  - FG59-2-3x covers delay/frequency/phase offset report

- FG59-3x covers the 3Tx UL operation

- FG59-4x covers the enhancement for asymmetric DL sTRP/UL mTRP scenarios

## Enhancements for UE-initiated/event-driven beam management

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for event-2 Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] |  | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | Per band | n/a | n/a | n/a | FFS: Further partitioning of this FG based one existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-1a | UE-initiated/event-driven beam management for event -1 Mode A | Support UE-initiated/event-driven beam management for event -1 Mode A |  | yes | n/a | UE-initiated/event-driven beam management for event -1 Mode A is not supported | Per band | n/a | n/a | n/a | Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-1b | UE-initiated/event-driven beam management for event -7 Mode A | Support UE-initiated/event-driven beam management for event -7 Mode A |  | yes | n/a | UE-initiated/event-driven beam management for event -7 Mode A is not supported | Per band | n/a | n/a | n/a | Note: the event definition Event 7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1.Support of UE-initiated/event-driven beam report [based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X] |  | yes | n/a | UEI/ED beam report Mode-B is not supported for Event | FFS | Per band | n/a | n/a | FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-2a | UE-initiated/event-driven beam management for event -1 Mode B | Support UE-initiated/event-driven beam management for event -1 Mode B |  | yes | n/a | UE-initiated/event-driven beam management for event -1 Mode B is not supported | Per band | n/a | n/a | n/a | Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-2b | UE-initiated/event-driven beam management for event -7 Mode B | Support UE-initiated/event-driven beam management for event -7 Mode B |  | yes | n/a | UE-initiated/event-driven beam management for event -7 Mode B is not supported | Per band | n/a | n/a | n/a | Note: the event definition Event 7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-4 | Measurement resource for UE initiated beam report | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report Memory limitations: - The max number of CSI-RS resources across all CCs configured for UE initiated beam report - The max number of aperiodic CSI-RS resources across all CCs configured for UE initiated beam report - The max number of CSI-RS resources in a CSI-RS resource set configured for new beam for UE initiated beam report Other limitations: - Supported density of CSI-RS (CMR) |  | Yes | n/a | UE initiated beam report is not supported | Per band | n/a | n/a | n/a | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {0, 4, 8, 16, 32, 64} Component 3: Candidate values {8, 16, 32, 64, 128} Component 4: Candidate values {0, 2, 4, 8, 16, 32, 64} Component 5: Candidate values {2, 4, 8, 16, 32, 64} Component 6: Candidate values {'1 only', '3 only', '1 and 3'} Note: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note: For component 3, 4, 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note: For components 1, 2, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances within a time window. | Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam] |  | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | Per band | n/a | n/a | n/a | FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-5 | Maximum N for UE initiated beam report | Support of UE initiated beam report with maximum N L1-RSRP values |  | Yes | n/a | Maximum N=1 | Per band | n/a | n/a | n/a | Candidate values {1, 2, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-6 | Configuration that current beam is always reported for UE initiated beam report | Support RRC to configure that current beam is always reported for UE initiated beam report |  | Yes | n/a | RRC to configure that current beam is always reported for UE initiated beam report is not supported | Per band | n/a | n/a | n/a | Note: The reported current beam, if configured, is not counted in the N reported beams | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-7 | Beam reporting timing for Mode B UE initiated beam report | For Mode B UE initiated beam report, the minimum number of symbols, X, between the last symbol of report notification on the first PUCCH channel and the first symbol of the second UL channel carrying the UE initiated beam report |  | Yes | n/a |  | Per band | n/a | n/a | n/a | Candidate values: different X in symbols can be reported for different SCS 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-8 | Q-th best quality for UE initiated beam report event 7 | For event 7, quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. |  | Yes | n/a |  | Per band | n/a | n/a | n/a | Candidate values: {best, worst, both} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-9 | Different PUCCH group for the first PUCCH and second PUSCH | Support the first PUCCH and the second PUSCH from different PUCCH group for both Mode A and Mode B |  | Yes | n/a | The first PUCCH and the second PUSCH must be configured from the same PUCCH group for both Mode A and Mode B | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |

## CSI enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | FFS 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 64 Tx ports | Per band and Per BC | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1c | Enhanced Type-I SP codebook for 64 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1d | Enhanced Type-I SP codebook for 48 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1e | Enhanced Type-I SP codebook for 128 ports – Scheme-b | 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-1f | CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports | Support CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports |  | Yes | n/a | CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1g | Spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports | - Support spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports by extending the bitmap (i.e., port-subsetIndicator) - Processing timeline |  | Yes | n/a | Spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate value {Capability 1, Capability 2} Note: For component of processing timeline Capability 1 timeline: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1h | CSI-RS port mapping for Type-I and Type II codebook enhancement for up to 128 ports | - Supported CSI-RS port mapping schemes for Type-I and Type II codebook enhancement for up to 128 ports |  | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate value {Mapping method 1, Mapping method 2, Mapping method 3} Note: The CSI-RS port mapping applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5 | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1i | CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports | - Maximum number of slots the K CSI-RS resources shall be configured within for Type-I and Type II codebook enhancement for up to 128 ports |  | Yes | n/a | No time domain restriction for K CSI-RS resources | Per band and Per BC | n/a | n/a | n/a | Candidate value {1 slot, 2 slots} Note: The supported CSI-RS resource time domain restriction applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5 | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-2 | Enhanced Type-I MP codebook | - Support 64 ports - Additionally supported number of aggregated ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported maximum number of panels - Max # of CSI-RS resource in a resource set - Supported processing capability |  | yes | n/a | Enhanced Type-I MP codebook is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values bitmap {48 ports, 128 ports} Component 3 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 4 candidate value {2, 4} Component 5 candidate value {1:8} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-3 | Extended Rel-16 eType-II codebook for 64 Tx ports | - Support of extended Rel-16 eType-II codebook] for 64 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability | FFS 16-3a | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 64 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-3a | Extended Rel-16 eType-II codebook for 48 Tx ports | - Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 48 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-3b | Extended Rel-16 eType-II codebook for 128 Tx ports | 1. Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 128 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3c | PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports | - Support of PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 |  | Yes | n/a | PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3d | Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports | - Support of parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports |  | Yes | n/a | Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3e | Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports | - Support of Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports - Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability |  | Yes | n/a | Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-4 | Extended Rel-17 FeType-II codebook with 64 Tx ports | 1. Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources] - Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability | FFS 23-9-1 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-4a | Extended Rel-17 FeType-II codebook with 48 Tx ports | 1. Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources] 2. Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability | 59-2-1-4 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4b | M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | - Support M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports - Support of parameter combinations with M=2 A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=1 | 59-2-4 | Yes | n/a | M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4c | R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | - Support R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2 | 59-2-4 | Yes | n/a | R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4d | Rank 3,4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | Support rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | 59-2-4 | Yes | n/a | Rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-5 | Extended Rel-18 eType-II Doppler codebook for 64 Tx ports | - Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups] - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp | FFS 40-3-2-1 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-5a | Extended Rel-18 eType-II Doppler codebook for 48 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-5b | Extended Rel-18 eType-II Doppler codebook for 128 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports | Per band and Per BC | n/a | n/a | n/a | Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-5c | N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | - Support for the size of DD-basis, N4>1 - A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} for one CSI report setting Value of d=m for the DD unit size when A-CSI-RS is configured for CMR | 59-2-5 | Yes | n/a | N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {2,3,4 … 64} d. {64, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {48, 64,128} c. {4,8,12} d.{64, …, 256} |  |
| 59. NR_MIMO_Ph5 | 59-2-5d | DD unit size d=1 when A-CSI-RS is configured for CMR N4>1 | Support value of d=1 for the DD unit size when A-CSI-RS is configured for CMR for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR N4>1 is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5e | Maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values: {4, 8, 12} |  |
| 59. NR_MIMO_Ph5 | 59-2-5f | PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | - Support PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports - A list of supported combinations, each combination is {Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-5 | Yes | n/a | PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64,128} b. {2,3,4 … 64} c. {64, …, 256} |  |
| 59. NR_MIMO_Ph5 | 59-2-5g | X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | X=1 based on first and last slot of WCSI, for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5h | X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5i | l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | l = (n – nCSI,ref) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5j | L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support L=6 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5k | Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support Rank 3 and 4 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5ls | Processing timeline for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | - Aperiodic CSI report timing relaxation, w, Rel-18 Type-II Doppler codebook enhancement for up to 128 ports Aperiodic CSI report timing relaxation Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols: {14*(KP–1)*d, 14*KP*d} Note: Kp is according to Component 12 of FG59-2-5 Note: d=4 (minimum periodicity of periodic CSI-RS) Component 2 candidate values: {CAP1, CAP2} For N4 = 1 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP1 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP2 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m + Z'2, 2Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w + Z'2, 2Z'2) Z2/Z'2 are defined in Table 5.4-2 in TS38.214 KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots |  |
| 59. NR_MIMO_Ph5 | 59-2-5m | Maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values (in slots): {4, 5, 8, 10, 20} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-6 | CSI-RS resource slot offset configuration for aperiodic CSI-RS | [Support of 2-slot CSI-RS resource aggregation] | FFS | yes | n/a | 2-slot resource aggregation is not supported | FFS | FFS | FFS | FFS | [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-7 | Group-specific 3-bit scaling factors for up to 128 ports | Support of group-specific 3-bit scaling factors |  | yes | n/a | Group-specific 3-bit scaling factors is not supported | Per band and Per BC | n/a | n/a | n/a | [Candidate values: {’rank-1’, ‘rank-1 and rank-2’}] Note: 3-bit scaling applies only to the Type-I SP codebook FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate FGs for RI=1 and RI=2 | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-8 | SRS Port Grouping | Support of SRS port grouping |  | yes | n/a | SRS Port Grouping is not supported | Per FSPC | n/a | n/a | n/a | Candidate value, bitmap { xT6R, xT8R } FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-1 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | - Maximum number of Ks configured CSI-RS resources for channel measurement - Maximum number of M reported CRI |  | yes | n/a | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-1a | Configuration of MR CSI-RS resource for Rel-15 Type-I SP (single panel) codebook enhancement for CRI-based CSI refinement | - Support the NW configuration of MR (<M) of KS CSI-RS resources to be selected as part of reporting the M CRI(s), for A-CSI only, for Rel-15 Type-I SP (single panel) codebook enhancement for CRI-based CSI refinement - Maximum number of MR |  | Yes | n/a | NW configuration of MR (<M) of KS CSI-RS resources to be selected as part of reporting the M CRI(s) is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-2 | Hybrid BF (CRI-based) with Rel-16 eType-II codebook | - Maximum number of Ks configured CSI-RS resources for channel measurement - Maximum number of M reported CRI |  | yes | n/a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2a | Configuration of MR =1 CSI-RS resource for Rel-16 eType-II codebook enhancement for CRI-based CSI refinement | - Support the NW configuration of MR =1 of KS CSI-RS resources to be selected as part of reporting the M CRI(s), for A-CSI only, for Rel-16 eType-II codebook enhancement for CRI-based CSI refinement |  | Yes | n/a | NW configuration of MR =1 of KS CSI-RS resources to be selected as part of reporting the M CRI(s) is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report | - Support delay offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for delay offset report - Maximum number of configured TRS resource sets for delay offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for delay offset report per CC - Maximum number of simultaneously active CSI-RS resources for delay offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - 6. Value of X for CPU occupation (OCPU=XNTRP) |  | yes | n/a | CJTC Dd report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-1a | Maximum number of TRS resource sets in a report configuration for delay offset report | - Maximum number of TRS resource sets in a report configuration for delay offset report |  | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {2, 3, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-1b | Maximum number of delay offset report settings per BWP | - Maximum number of delay offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID |  | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {1, 2, 3, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-1c | Quantization for delay offset report | - Maximum number of quantization points MD for delay offset report - Quantization range AD for delay offset report |  | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {32, 64, 128, 256} Component 2 candidate values: bitmap {0.5CP, CP} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report | - Support frequency offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for frequency offset report - Maximum number of configured TRS resource sets for frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for frequency offset report per CC - Maximum number of simultaneously active CSI-RS resources for frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC 6. Value of X for CPU occupation (OCPU=XNTRP) |  | yes | n/a | CJTC FO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-2a | Maximum number of TRS resource sets in a report configuration for frequency offset report | - Maximum number of TRS resource sets in a report configuration for frequency offset report |  | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {2, 3, 4} |  |
| 59. NR_MIMO_Ph5 | 59-2-3-2b | Maximum number of frequency offset report settings per BWP | - Maximum number of frequency offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID |  | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {1, 2, 3, 4} |  |
| 59. NR_MIMO_Ph5 | 59-2-3-2c | Quantization for frequency offset report | - Maximum number of quantization points MFO for frequency offset report - Quantization range AFO for frequency offset report |  | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {16, 32, 256} Component 2 candidate values: bitmap {0.1ppm, 0.2ppm} |  |
| 59. NR_MIMO_Ph5 | 59-2-3-3 | CJTC wideband PO report | - Support wideband phase offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured CSI-RS resources for phase offset report - Maximum number of configured CSI-RS resources for phase offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for phase offset report per CC - Maximum number of simultaneously active CSI-RS resources for phase offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Value of X for CPU occupation (OCPU=XNTRP) |  | yes | n/a | CJTC PO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-4 | CJTC subband PO report | - Support subband phase offset report for aperiodic standalone CJT calibration reporting - Maximum number of reported subbands - Minimum subband size | FFS | yes | n/a | CJTC subband PO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {2, 4, 6, 8, 10, 12, 14, 16} Component 3 candidate values: {1, 2, 4, 8,6} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3a | Maximum number of CSI-RS resources in a report configuration for phase offset report | - Maximum number of CSI-RS resources in a report configuration for phase offset report | 59-2-10 | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {2, 3, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3b | Maximum number of phase offset report settings per BWP | - Maximum number of phase offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID | 59-2-10 | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {1, 2, 3, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3c | Maximum number of quantization points M for phase offset report | - Maximum number of quantization points M for phase offset report | 59-2-10 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values: {16, 32} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-5 | CJTC Dd+FO report | - Support joint delay and frequency offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for joint delay and frequency offset report - Maximum number of configured TRS resource sets for joint delay and frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for joint delay and frequency offset report per CC - Maximum number of simultaneously active CSI-RS resources for joint delay and frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC 6. Value of X for CPU occupation (OCPU=2XNTRP) | FFS | yes | n/a | CJTC Dd+FO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-6 | New CJT QCL assumptions for PDSCH pre-compensation | Supported QCL assumption for PDSCH with two indicated TCI states |  | yes | n/a |  | Per band | n/a | n/a | n/a | Candidate values: bitmap {Scheme C, Scheme D, Scheme E} Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay} Scheme E: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7 | Linkage of CJTC Dd and Rel-18 eType-II CJT | 1. Support of RRC configuration a UE to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset - 2. Supported mode(s) |  | yes | n/a | Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values: {Joint trigger, separate trigger, both} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-8 | Separate triggering with configuration of 1-bit indicator per CSI trigger state | Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting |  | yes | n/a | Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported | Per band and Per BC | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7a | Processing time relaxation for joint triggering of CJTC Dd and Rel-18 eType-II CJT | Processing time relaxation for joint triggering of CJTC Dd and Rel-18 eType-II CJT |  | yes | n/a | No relaxation | Per band and Per BC | n/a | n/a | n/a | Component candidate values: 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-9 | RRC configuration of 1 SRS port of antenna switching associated with phase offset report | Support of RRC configuration of 1 SRS port of antenna switching associate with phased offset report | 59-2-10 | Yes | n/a | RRC configuration of 1 SRS port of antenna switching associated with phase offset report is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | - Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously |  | yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port is {48, 64, 128} The candidate value set of the max # of resources is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256} | Optional with capability signalling |

## 3Tx UL operation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | Support of maximum number of 3 PUSCH MIMO layers for non-codebook-based PUSCH |  | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH | - Support association between NZP-CSI-RS and SRS resource set for non-codebook-based 3Tx PUSCH - A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously | 59-3-2 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based PUSCH is not supported | Per band | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port is {2, 4, 8, 12, 16, 24, 32, 48, 64, 128} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256} Note: For max # of Tx ports, 1 CSI-RS resource is used for <= 32 ports, and more than 1, up to 4 CSI-RS resources, are used for >32 ports | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of 3 PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 3. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE |  | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values {1, 2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL |  | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL |  | yes | n/a | 3TX 3T3R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3b | Maximum 2 semi-persistent and 1 periodic SRS sets for 3 port SRS antenna switching | Support of maximum 2 semi-persistent SRS resource sets and maximum 1 periodic SRS resource set for 3 port SRS antenna switching | 59-3-3, 59-3-3a | Yes | n/a | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported | Per FS | n/a | n/a | n/a | Note: The two semi-persistent SRS resource sets are not activated at the same time | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set |  | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set |  | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3.4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set |  | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set |  | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | Per FSPC | n/a | n/a | n/a | Component candidate values {1, 2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-7 | Support of full-power Mode 0 for codebook-based for 3 Tx operation | UL full power transmission mode of fullpower | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported for 3 Tx operation | Per FSPC | n/a | n/a | n/a | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

## Enhancement for asymmetric DL sTRP/UL mTRP scenarios

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-1 | Support of association of a UL TCI state with a PL offset | Support association of a UL TCI state with a PL offset |  | Yes | n/a | Association of a UL TCI state with a PL offset is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-1a | Support of association of a joint TCI state with a PL offset | Support association of a joint TCI state with a PL offset |  | Yes | n/a | Association of a UL TCI state with a PL offset is not supported | Per FS | n/a | FR1 only | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-2 | Path Loss offset on PDCCH-order PRACH | Support of applying path loss offset on PDCCH-order PRACH | 59-4-1 or 59-4-1a | yes | n/a | Applying path loss offset on PDCCH-order PRACH is not supported | Per FS | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH SRS closed loop indices | Support of two separate SRS closed loop indices separate from PUSCH |  | yes | n/a | Two separate SRS closed loop indexes is not supported | Per FS | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4 | Support two TAs enhancement | Support of two TAs without the restriction of multi-DCI based multi-TRP operation |  | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported | Per FS | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements (see below for further partition) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4 | Support of Single-DCI based intra-cell Multi-TRP operation with two TA | Support single-DCI based inter-cell Multi-TRP operation with two TA |  | Yes | n/a | Single-DCI based inter-cell Multi-TRP operation with two TA is not supported | Per FS | n/a | n/a | n/a | Candidate value: bitmap {PUSCH TDM repetition Type A, PUSCH TDM repetition Type B, PUCCH TDM repetition, PUSCH SDM, PUSCH SFN, PUCCH SFN} Note: Single-DCI based Multi-TRP is the operation without the restriction of “the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs” Note: FG40-2-8, if reported, or supportedNumberTAG” in 38.306 is applied for the maximum number of TAGs | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4a | Support of Single-DCI based inter-cell Multi-TRP operation with two TA | Support single-DCI based inter-cell Multi-TRP operation with two TA |  | Yes | n/a | Single-DCI based inter-cell Multi-TRP operation with two TA is not supported | Per FS | n/a | n/a | n/a | Candidate value: bitmap {PUSCH TDM repetition Type A, PUSCH TDM repetition Type B, PUCCH TDM repetition, PUSCH SDM, PUSCH SFN, PUCCH SFN} Note: Single-DCI based Multi-TRP is the operation without the restriction of “the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs” Note: FG40-2-8, if reported, or supportedNumberTAG” in 38.306 is applied or the maximum number of TAGs | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | 59-4-4 | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | Per band | n/a | n/a | n/a | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | 59-4-1 or 59-4-1a | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | Per FS | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-7 | Extended value range of starting bit of block in DCI format 2_3 | Support of the extended value range of starting bit of DCI format 2_3 |  | yes | n/a | The extended value range of starting bit of DCI format 2_3 is not supported | Per FS | n/a | n/a | n/a | Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate 2-bit TPC for separate SRS closed loop index(es) | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) |  | yes | n/a | DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported | Per FS | n/a | n/a | n/a | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-8a | Support of DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH | Support DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH |  | Yes | n/a | DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9 | Support of including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH | Support including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH |  | Yes | n/a | PL offset in the calculation of Type 1 PHR is not supported | Per FS | n/a |  | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-10 | Support of one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH | Support one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH |  | Yes | n/a | One 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |