3GPP TSG RAN WG1 #120			R1-2500806

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.11.4

Source:	Apple

Title:	Discussion on IoT-NTN Uplink Capacity Enhancement

Document for:	Discussion/Decision

# Introduction

In the RAN #102 meeting, the WID of Release 19 IoT-NTN was agreed [1]. One objective of the WID is uplink capacity/throughput enhancement for IoT-NTN. In addition, the following agreements were made in the last RAN1 meeting [2].

Agreement For NPUSCH Format 1 single-tone 15kHz SCS, RAN1 studies at least the following options for CDM DMRS with legacy pattern for down-selection: - Option 1: DMRS symbols are spread before the OCC is applied, e.g. according to the formula: Where: M is the OCC length, q is the assigned OCC codeword for the UE and is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 - Option 2: DMRS symbols are not spread before the OCC is applied. - Option 2_1: OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2, e.g. according to the formula: - Option 2_2: OCC is applied to the complex-valued DMRS symbol used in slot 1 and slot 2. Depending on the OCC codeword, different DMRS sequence is used. - Option 3: DMRS symbols are not spread and OCC is not applied. - Legacy complex-valued DMRS symbol is used in slots corresponding to an OCC codeword of NPUSCH. Different DMRS sequences are used for multiplexed UEs. Agreement For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot. - Note: whether RU length is extended or not after applying OCC is a separate discussion. Agreement For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are: - OCC sequence index - Enabling of OCC feature - FFS: whether signaling is explicit or implicit

In this contribution, we provide our views on the study of IoT-NTN uplink capacity enhancement.

# Discussion

## NPUSCH capacity enhancement

### OCC spreading with frequency domain scheduling

The current specification allows for scheduled NPUSCH transmission with either a single subcarrier or multi-subcarrier, depending on the user's channel condition and throughput requirements. When employing time domain OCC, the OCC sequence is multiplied on the repeated OFDM symbols or slots. OCC spreading is supported for single subcarrier NPUSCH transmission, and the repetition pattern will be updated accordingly.

For multi-subcarrier transmission, the existing repetition pattern, with at least more than two repetitions, is suitable for OCC spreading. If additional repetitions are already necessary for multi-subcarrier PUSCH transmissions, then multiplexing with other user’s NPUSCH via OCC can enhance the uplink capacity with minimal impact on the standard. It allows for more efficient use of the uplink resources by enabling multiple users to share the same time-frequency resources without causing significant interference, thus increasing the overall capacity of the uplink transmission.

Proposal 1: Support OCC for multi-subcarrier NPUSCH format 1 transmission with the repetitions.

### Signaling aspects of NPUSCH with OCC

When multiple UEs transmit NPUSCH with OCC spreading using different OCC sequences on the same time and frequency resources, it's important to ensure that UEs have the necessary information about the OCC sequence index or OCC size to be used. This information can be delivered through various methods such as RRC signaling, scheduling DCI format N0, MAC CE, or implicitly derived from C-RNTI. Considering the standard impacts and scheduling flexibility, indication the OCC codeword index by MAC CE is preferred.

Proposal 2: Support indication of OCC sequence index by MAC CE.

In addition, the OCC scheme specified for dynamic scheduled NPUSCH can be applied to Msg3 NPUSCH to increase the IoT-NTN random access capacity. To achieve this target, some of the NPRACH sub-carriers will be reserved for the purpose of early indication of UE Msg3 NPUSCH OCC capability to the network. The OCC codeword index could be indicated by the reserved bit in the MAC RAR.

Proposal 3: To support Msg3 NPUSCH with OCC spreading, RAN1 to study the NPRACH resource reservation and OCC sequence index indication.

### UL transmission gap

In previous RAN1 meeting, several UL transmission gaps were agreed to study further.

- UL gaps for synchronization

According to current specification, the UL transmission gap is 40ms for every 256 subframe transmission. If two UEs are paired for transmissions with OCC spreading, and UE1 NPUSCH transmissions is overlapped with UL transmission gap of UE2, the OCC sequences of two UEs are still orthogonal before and after the UL transmission gap as shown in Figure 1. OCC performance is not impacted by the UL transmission gap.

Figure 1: Example of UL transmission gap

- TDM DMRS that are muted

For TDMed DMRS pattern, to keep the DMRS are orthogonal among paired UEs, some DMRS are muted or blanked. After muting the DMRS, the distance between DMRSs is 14symbols or 1ms for 15kHz SCS, and 14 symbols or 4ms for 3.75kHz SCS. The phase rotation impacts on channel estimation needs to be evaluated further according to the agreed assumptions in last meeting.

- Guard periods for 3.75kHz UL transmissions

The guard period in one slot is 2304Ts or 0.075ms in a 3.75kHz SCS slot. As RAN4 indicated in Rel-17 coverage enhancement work item that phase continuity is assumed if gap between two PUSCH/PUCCH transmissions does not exceeds 13 symbols. This 2304Ts gap will not impact the phase continuity in OCC design.

Observation: The following gaps do not impact the OCC design for IoT-NTN.

- UL gaps for synchronization

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

- Gaps around NPRACH occasions

As indicated by the specification 36.211, the NPUSCH transmission will be delayed in the event of a collision with NPRACH.

If a mapping to slots or a repetition of the mapping contains a resource element which overlaps with - any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or - …… then, - for the NPUSCH transmission in overlapped slots is postponed until the next slots not overlapping with any configured NPRACH resource. - for the NPUSCH transmission in overlapped slots is postponed until the next slots starting with the first slot satisfying ns mod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

For 15kHz SCS NPUSCH transmission, if the transmission overlaps with NPRACH resource, it is postponed, and restart form the first slot of a subframe after the gap. Since we have agreed that the OCC length of 2 is supported and OCC spreading is applied per slot. Therefore, NPRACH gap has no impacts on OCC orthogonality.

For 3.75kHz SCS NPUSCH transmission, the overlapped transmission is postponed on a per-slot basis. However, the two slots with the same OCC sequence index may be separated by the NPRACH gap, which can impact OCC performance.

In case where the OCC covering the NPUSCH transmission collides with NPRACH, the entire NPUSCH transmissions covered by an OCC sequence shall be postponed to avoid the impacts of the NPRACH gap.

- UL timing adjustment gaps and segmentation

The UL segment and compensation gap is used for TA pre-compensation, the TA pre-compensation gap could be 1symbol, 1slot, or 2 slots. Each NPUSCH segment could transmit with duration of 2ms, 4ms, up to 256ms. The UL timing adjustment gap will impact both slot-level OCC and symbol-level OCC which depends on the TA pre-compensation gap is symbol level or slot level.

In principle, if the performance of NPUSCH with OCC is impacted by TA pre-compensation gap, the whole PUSCH transmissions covering by an OCC sequence shall be dropped.

Proposal 4: RAN1 to study the standard impacts of gaps around NPRACH occasions and UL timing adjustment gaps on supporting OCC for NPUSCH.

## NPRACH capacity enhancement

### OCC spreading scheme for NPRACH

According to current specification 36.211, FDD Narrowband IoT supports three NPRACH formats. These formats include format 0 and format 1 with a subcarrier spacing of 3.75 kHz, as well as format 2 with SCS of 1.25 kHz.

Table 10.1.6.1-1: Random access preamble parameters for frame structure type 1

| Preamble format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 4 | 5 |  |  |
| 1 | 4 | 4 | 5 |  |  |
| 2 | 6 | 6 | 3 |  | 3∙24576Ts |

For NPRACH format 0 and 1, the identical symbols repeat 5 times, and one CP is inserted before the first symbol to form a symbol group. Each NPRACH transmission consists of four symbol groups with frequency hopping.

Intra-symbol group OCC

If the OCC sequence directly covers each symbol of a symbol group. After OCC spreading, 5 symbols are no longer identical for format 0/1, a single CP could not avoid the inter-symbol interference among the symbols in the symbol group. To mitigate this issue, a CP can be inserted between the symbols. For example, Figure 2 illustrates a preamble symbol group in a new NPRACH format. In this format, if the number of symbols in a symbol group remains 5 or 3, only DFT-based OCC can be used for NPRACH spreading. Additionally, without introducing CP, it could cause the inter carrier interference among the NPRACH subcarriers, and the missed detection probability increase.

Figure 2: example of preamble symbol group for OCC spreading

### Signaling aspects of NPRACH with OCC

If the network supports NPRACH with OCC spreading, Rel-19 capable UEs would select the PRACH subcarrier and an OCC sequence to perform the random access. To avoid the impacts on legacy UE to perform the random access, the dedicated PRACH resources need to be reserved including the subcarrier block for PRACH with OCC. Additional NPRACH start time and periodicity may be configured separately if a new NPRACH format is introduced. From signaling perspective, a set of parameters can be configured in a new NPRACH-Parameters-NB information element.

Proposal 5: RAN1 to study the signaling to support NPRACH with OCC spreading, including the resource allocation for NPRACH with OCC.

# Conclusion

In this contribution, we provided our views on IoT-NTN uplink capacity enhancements. Our observation and proposals are as follows:

Proposal 1: Support OCC for multi-subcarrier NPUSCH format 1 transmission with the repetitions.

Proposal 2: Support indication of OCC sequence index by MAC CE.

Proposal 3: To support Msg3 NPUSCH with OCC spreading, RAN1 to study the NPRACH resource reservation and OCC sequence index indication.

Observation: The following gaps do not impact the OCC design for IoT-NTN.

- UL gaps for synchronization

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Proposal 4: RAN1 to study the standard impacts of gaps around NPRACH occasions and UL timing adjustment gaps on supporting OCC for NPUSCH.

Proposal 5: RAN1 to study the signaling to support NPRACH with OCC spreading, including the resource allocation for NPRACH with OCC.

# References

- RP-234077, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3,” Dec. 2023.

- R1-2500003, “Report of RAN1#119 meeting”, Feb. 2025