3GPP TSG RAN WG1 #120		R1-2500830

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.1

Source:	Samsung

Title:	Discussion on two candidate starting symbols for sidelink unlicensed

Document for:	Discussion and Decision

# Introduction

In RAN1#119 [1], the following conclusion regarding the two starting symbols in sidelink unlicensed was made:

Conclusion

Continue to discuss the following issues in RAN1#120:

- Issue 1: misalignment between 38.213 and 38.214 on UE behavior on using sl-StartingSymbolFirst and/or sl-StartingSymbolSecond

- Issue 2: UE behavior at least when only sl-StartingSymbolSecond is (pre-)configured and sl-StartingSymbolFirst is not (pre-)configured

This contribution discusses our view on the two remaining issues.

# Discussion

For Issue 1, the key point is whether legacy RRC parameter sl-StartSymbol can be used for unlicensed spectrum (the other possibility that sl-StartingSymbolFirst and sl-StartingSymbolSecond are used for licensed band anyway is not feasible based on current RAN2 specification). Based on current specification TS 38.331, the condition “need M” applies for both sl-StartingSymbolFirst and sl-StartingSymbolSecond, which implies when SL-Unlicensed-r18 is configured, the value of sl-StartingSymbolFirst and sl-StartingSymbolSecond will anyway be known to UE – either provided at the first time configuration, or maintained from previous configuration. In this sense, the legacy RRC parameter sl-StartSymbol may not be used for unlicensed spectrum.

On the other hand, the misalignment of the condition on using sl-StartingSymbolFirst and sl-StartingSymbolSecond is still there, and it is preferable to adopt an alignment CR (i.e., Option 1 in R1-2410783) to fix this issue.

Proposal 1: For Issue 1:

- Adopt Option 1 in R1-2410783 as part of the Rel-18 alignment CR for TS 38.213.

- Conclusion:

  - sl-StartSymbol is only used for operation without shared spectrum channel access.

  - sl-StartingSymbolFirst and/or sl-StartingSymbolSecond are only used for operation with shared spectrum channel access.

For Issue 2, it should be a common understanding that the interpretation of “when RRC parameter is provided” in RAN1 specifications includes the case that such RRC parameter is not explicitly configured but using its default value. Based on this understanding, Issue 2 may not exist and current specifications are complete (i.e., UE behavior for that at least when only sl-StartingSymbolSecond is (pre-)configured and sl-StartingSymbolFirst is not (pre-)configured is already specified).

Proposal 2: For Issue 2:

- Conclusion:

  - It is RAN1 common understanding that in RAN1 specifications, when a RRC parameter is provided, it includes the case that the RRC parameter is not configured and a default value (if specified in TS 38.331) of the RRC parameter is used.

# Conclusion

The proposals from this contribution are summarized below:

Proposal 1: For Issue 1:

- Adopt Option 1 in R1-2410783 as part of the Rel-18 alignment CR for TS 38.213.

- Conclusion:

  - sl-StartSymbol is only used for operation without shared spectrum channel access.

  - sl-StartingSymbolFirst and/or sl-StartingSymbolSecond are only used for operation with shared spectrum channel access.

Proposal 2: For Issue 2:

- Conclusion:

  - It is RAN1 common understanding that in RAN1 specifications, when a RRC parameter is provided, it includes the case that the RRC parameter is not configured and a default value (if specified in TS 38.331) of the RRC parameter is used.

# Reference

[1] 3GPP RAN1#119 chairman notes, November, 2024.