3GPP TSG-RAN WG1 Meeting #120														R1-2500896

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.2.1

Source:	Nokia

Title:	Enhancements to facilitate UE-initiated/event-driven beam management

Document for:		Discussion and Decision

# Introduction

The UE-initiated beam management (UEIBM) was agreed upon for the Rel. 19 MIMO WI in RP-234007 [1], and the agreed work item description was as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In RAN1# 119, the following agreements, working assumptions and conclusions were made:

[119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted [119] Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability [119] Conclusion On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the case that the first PUCCH and the second PUSCH are from the different CG is NOT supported. [119] Agreement On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. [119] Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. [119] Agreement Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. [119] Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A. [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain. [119] Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded [119] Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. [119] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4. Final LS in R1-2410914.

This paper discusses the possible enhancements that would facilitate UE-initiate/event-driven beam management in the context of the unified TCI framework, i.e., for UL and DL beam management.

Such enhancements should result in reducing the overhead and/or latency of FR2 beam management. The preferred approach is leveraging the legacy CSI measurement and reporting configuration framework. The use case focuses on sTRP while including intra-cell and inter-cell beam management.

The discussion is structured in six parts. First, the UL signaling medium containers are discussed, including left-over for first PUCCH. Second, trigger event detection is discussed, including left-over for Event-2 and needed enhancements for Event-1 and Event-7. Third, the UL signaling contents are described and compared for all the agreed events. Fourth, cross-CC measurements and reports are discussed. Fifth, solutions for beam application latency reduction are compared. Finally, simultaneous configuration of multiple events are discussed.

The paper closes with the conclusions on this objective and Nokia’s observations and proposals.

# Discussion

## UL signaling medium/container

In the last RAN1 meeting, an important decision has been taken with respect to the signalling of the first PUCCH, and we agreed that the first PUCCH channel is a new UCI-type (Alt 2). There are however few issues still open for the first PUCCH for both Mode-A and Mode-B:

#1 - Whether/how to (re-)transmit the first PUCCH channel.

#2 - Whether/how to support 1-bit PUCCH for multi-CC & multi-event report configurations; if not, whether/how to associate a 1-bit PUCCH resource with multiple CSI report configurations.

#3 - Multiplexing/dropping rule(s) of 1-bit first PUCCH.

Besides first PUCCH design, for the UL signaling medium there are also some other issues that need to be solved on the following aspects:

4# Left-over on Step-2&3 in Mode-A, e.g., whether CSI trigger state is dedicated to UE-initiated/event-driven beam reporting.

5# Left-over on Step-2 in Mode-B, e.g., whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

#1 Whether/how to (re-)transmit the first PUCCH channel

Few candidates are on the table regarding first PUCCH retransmissions controlled by a prohibit timer:

- Option-1: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

- Option-2: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

In our view, Option-2 is about retransmission of first channel/ PUCCH (carrying request for UL resources for beam/CSI reporting) for mode A for the situation when the UE does not receive response to the first PUCCH from the gNB, where this response is nothing but UL DCI triggering beam/CSI reporting corresponding to an event. Specifically, the UE would need to monitor for a reception of the beam/CSI report triggering indication from the gNB, and if the UE doesn’t receive or correctly decode the indication from the gNB within a configured period of time, the UE can be configured to retransmit the first PUCCH. That could be controlled by introducing such prohibit timer that starts after the first PUCCH has been sent. On the other hand, a prohibit timer is simply gaining time though a time window, is not a solution as such to a potential problem of PUCCH transmission failure, the solution lying in fact in the PUCCH re-transmissions. The utilization of the prohibit timer may be avoided by implementation where reasonable amount of time is allowed before a PUCCH re-transmission.

For Mode A, do not support Option-2, i.e., a prohibit timer started after the first PUCCH, for first PUCCH re-transmissions.

There is another issue related to first PUCCH (re-)transmission that deserves attention, and that relates to Option-1. In Mode A and in Mode B, after the UE has successfully sent the beam report (in step 3 for Mode A and in step 2 for Mode B), it may happen that the network does not take any action, e.g., no beam switching, but the UE cannot know if that happened because the beam report was lost or just because of a network decision. Anyhow, it may happen that the conditions triggering the same event and beam report may persist over a period of time (the same Event-2, i.e., for same new beam(s) and for same conditions, e.g., same RSRP value(s) of the new beam(s) and of the current beam, keep repeating over time). In the current framework this may cause the transmission of multiple consecutive beam reports with same content, i.e., same top-N beams with same RSRP values, which bring no additional information to the network and just cause additional overhead. That could be controlled by a prohibit timer, such that the UE may send a first PUCCH for same event (with same conditions) only after such timer has expired. On the other hand, the utilization of such timer may not be really needed, as UEIBM already strongly reduced the overhead when compared for example to periodic reporting, and solution without any prohibit timer would be the leanest.

For both Mode A and Mode B, do not support Option-1, i.e., a prohibit timer started after the PUSCH, for first PUCCH re-transmissions.

#2 Whether/how to support 1-bit PUCCH for multi-CC & multi-event report configurations

Few candidates are on the table on this issue:

- Option-1: Regarding both mode-A and Mode-B, support multi-bit indication in the first PUCCH for the multi-CC(s) or multi-event(s) case.

  - FFS: PUCCH format in such case.

- Option-2: Regarding both mode-A and Mode-B, support one-bit indication in the first PUCCH for the multi-CC(s) or multi-event(s) case.

  - Note: Multi-bit indication in the first PUCCH is NOT supported

  - FFS: whether/how to associate a 1-bit PUCCH resource with multiple CSI report configurations.

If the UE would be configured with multiple events, it needs to be clarified that each event is associated to its own CSI report configuration.

Do not consider simultaneous events for a single CSI report configuration.

Although a multi-bit indication in the first PUCCH has the advantage of using a common first PUCCH for all events when multiple events are configured, it also requires a new design, therefore a simpler solution can be achieved by using a one-bit indication in the first PUCCH.

With 1-bit first PUCCH, the UE can inform about which event(s) triggered the report either with a 1-bit first PUCCH per event or on the PUSCH when a 1-bit first PUCCH is shared among all the events. Some techniques can be used to associate a 1-bit PUCCH resource with multiple CSI report configurations, for example considering the maximum payload size and zero padding when smaller reports are sent.

Regarding both Mode-A and Mode-B, support one-bit indication in the first PUCCH for the multi-CC(s) or multi-event(s) case.

#3 Multiplexing/dropping rule(s) of 1-bit first PUCCH.

A new UCI type is used to represent the information carried by the first channel/ PUCCH, so for multiplexing involving the first channel, the new UCI type can be essentially treated as SR, meaning that same/similar multiplexing and prioritization rules as for SR could be used for the new UCI type. Hence similar handling/multiplexing rules as legacy for SR vs. HARQ-ACK/CSI can be adopted for new UCI type vs. HARQ-ACK/CSI.

For handling overlapping UL channels involving the first channel, the new UCI type can be treated as SR, i.e., similar multiplexing/dropping rules as for SR can be used for the new UCI type.

- - Note: this applies at least to PUCCH(s) carrying HARQ-ACK and/or CSI and overlapping with the first channel.

(Case 1):

The cases where a PUCCH carrying ‘normal’ SR/ LRR overlaps with the PUCCH carrying the new UCI type should be discussed. One possibility would be to avoid such cases, i.e., treating those as an error case. However, this would add scheduling restriction on the gNB. Another possibility would be to allow the overlap to occur but not allow multiplexing, meaning that dropping either the normal SR/LRR or the new UCI type; which one to drop could either be based on rules and/or based on gNB configuration, or even could be left up to UE implementation.

Regarding the case where the 1-bit first PUCCH overlaps with a PUCCH carrying normal SR or normal LRR, adopt the following approach: Allow overlap of the first PUCCH with another PUCCH for SR or for LRR, where only dropping PUCCH(s) is supported (i.e., no multiplexing).

- - For the purpose of determining which PUCCH to drop/prioritize, downselect one of the following options:

- - Option a: based on rules

- - Option b: based on gNB configuration

- - Option c: left up to UE implementation

(Case 2):

Similarly as for Case 1 above, we should avoid defining multiplexing for cases where the new UCI type overlaps with PUSCH. That is especially to avoid complicated maintenance discussions related to multiplexing/dropping.

Regarding the case where the 1-bit first PUCCH overlap with a PUSCH, adopt the following approach: Allow overlap of the first PUCCH with a PUSCH, where only dropping one of the PUCCH and PUSCH is supported (i.e., no multiplexing).

- - For the purpose of determining which UL channel to drop/prioritize, downselect one of the following options:

- - Option a: based on rules

- - Option b: based on gNB configuration

- - Option c: left up to UE implementation

(Case 3):

For the cases where 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain, multiplexing should not be supported as otherwise this becomes similar to agreeing to support multi-bit indication for the first channel/ PUCCH; also to avoid complicated maintenance discussions related to multiplexing/dropping.

Regarding the case where 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting overlap in the time domain, adopt the following approach: Allow overlap where only dropping is supported (i.e., no multiplexing).

- - For the purpose of determining which PUCCH to drop/prioritize, downselect one of the following options:

- - Option a: based on rules

- - Option b: based on gNB configuration

- - Option c: left up to UE implementation

4# Left-over on Step-2&3 in Mode-A, e.g., whether CSI trigger state is dedicated to UE-initiated/event-driven beam reporting

Regarding the CSI trigger state in step-2 of Mode A, we have the following issue:

[119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

Both options are fine, but as we do not see any issue in having a shared CSI trigger state between UEIBM and AP-CSI reporting, we prefer support Option 2.

For Mode A, support having a non-dedicated CSI trigger state in Step-2.

5# Left-over on Step-2 in Mode-B, e.g., whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms)

In previous meeting it was agreed that for Mode B there is one-to-one mapping between first PUCCH and pre-configured resource for second UL channel. That means that only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with a CSI report configuration for UEIBM. Regarding a) the periodicities of first PUCCH and second UL channel and b) when the UE shall transmit the UEI report after the first PUCCH, we have the following issue still:

[118] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. [118b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, - The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel. - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms)

After sending the first PUCCH, it is uniquely defined on which specific resource occasion the UE will transmit the UEI report, i.e., it will transmit it on the first available transmission occasion of the second UL channel X symbols after the first PUCCH. Therefore, we do not see any benefit in allowing different periodicities of first PUCCH and pre-configured resources for second UL channel, as a proper implementation will use same periodicity, with a proper relative delay.

For Mode B, do not support different periodicities of first PUCCH resource and pre-configured resource for second UL channel.

## Trigger-event detection

Left-over on trigger-event detection for Event-2

We have the following open issues w.r.t trigger event detection for Event-2

[117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. • Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. [118b] Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

Several candidate solutions are on the table on resetting the counting:

-	Candidate #1: RS reconfiguration/update for new beam is received;

-	Candidate #2: The measured current beam based on the indicated TCI state is updated;

-	Candidate #3: UEI beam report is transmitted;

-	Candidate #4: NW response (e.g., DCI in step-2 of Mode-A) is detected;

- Candidate #5: The time window expires;

- Candidate #6: The threshold for event evaluation is re-configured by RRC signaling.

First of all, we want to reiterate the agreement that the counting is done per new beam.

We see an added values in Candidates #1, #2 and #6 as they contain a form of implicit acknowledgement both w.r.t the fact that a previous UEIBM report reached the gNB and more importantly the fact that the gNB took a decision regarding the future operations. On other hand, Candidates #1 and #6 should be considered jointly, as such re-configuration of new beam set and threshold for event evaluation are both performed with an RRC re-configuration of the CSI report configuration associated to Event-2.

Then, Candidate #3 does not guarantee anything w.r.t the gNB further operation neither to the fact that the UEI report has been received by the gNB or not. Candidate #4 alleviates the previously mentioned shortcomings of Candidate #3 yet does not say anything on the actual beam configurations. Candidate #5 is not needed, as we have already agreement that the counting should be done within a time window, i.e., when the time window expires, the counting must be reset. Based on this discussion, we see a benefit in Candidate #2 which contains a ready-to-be-used form of signaling, yet if this is not sufficient, Candidate #1 and Candidate#6 could be considered in addition.

Regarding the triggering event determination within a time window for Event-2, for resetting the counting support Candidate #2 (update of the indicated TCI state):

- If deemed necessary, Candidate #1 (reconfiguration of new beams) and Candidate #6 (reconfiguration of threshold for event evaluation) can also be supported as a joint Candidate on RRC re-configuration of the CSI report configuration associated to Event-2.

It is still open if this proposed counter within a configurable time should be captured in RAN1 or RAN2 specifications. Although timer/counter based mechanisms are common in RAN2, where the MAC is responsible to handle such mechanisms, we think that for UEIBM the agreed solution, in particular if prohibit timers to handle first PUCCH retransmission are not supported, is rather simple and it would be beneficial to capture that in the PHY specifications for better clarity.

Regarding the triggering event determination within a time window for Event-2, capture the counter-based mechanisms in RAN1 specifications.

Trigger-event detection for Event-1 and Event-7

Regarding trigger-event detection for Event-1, we have the following issue:

[119] Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

As it has been also agreed that the design for Event-1 and Event-7 shall re-use as much as possible the Event-2 design, we think a new beam RS resource set should be configured for Event-1, and then we have an implicit manner to enable either scheme-1 or scheme-2.

Regarding the triggering event determination for Event-1, support having a RS resource set for new beam configured, and then using the implicit manner for enabling either scheme-1 or scheme-2 (Option-2).

For Event-7, the UE evaluates whether the quality of a non-activated TCI state/new beam becomes a threshold value better than the quality of Q-th strongest activated TCI state/beam. The quality of the beam is measured with e.g. L1-RSRP. Parameters as Q and threshold are configured by gNB. Furthermore, N represents the number of beams to be reported, e.g., 1, 2, 3 or 4, and is also configured by RRC (like for Event-2).

For example, if gNB configures Q=3, then UE triggers a beam report when the quality of a new beam is better than the 3rd best active TCI state.

If Q=1, the UE is triggering the UEIBM report when a beam is a threshold better than the best active TCI state. This operation is resulting in very similar operation than for Event-2, so values of Q greater than 1 should be supported.

As the aim of Event-7 is to update the active TCI state list, we think it is worth to further discuss how many beams should be reported for certain configured Q. For example, in case 3 TCI states are in the active TCI list and Q is set to 3, the network wants to update the 3rd/worst active beam with a non-activated beam. On the other hand, in case the RS resource set for new beam includes also activated TCI states, for example the 1st and 2nd best active beam, the network should configure at least N=3, to make sure that the UE reports among the three beams also the non-activated one.

Regarding the triggering event determination for Event-7, specify N≥Q.

## UL signaling content

Left-over on report format for Event-2

Regarding the UL signaling content, we have the following RAN1#118 agreement:

[118] Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID). [119] Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

On the indication of which CRI/SSBRI(s) satisfy the condition of Event-2 we have the following observations. gNB has full knowledge of mapping between the current beam and the new beam, and without any further indication, it can distinguish which beam fulfills the event condition.

There is one argument on the validity of RSRP due to triggering condition of fulfilling of M consecutive measurements/samples, where M=1 when higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 when it is not configured. For example, in case M=3, which is likely used to ensure some L1 filtering/averaging in case the UEIBM time window and counter are not configured (note that in RAN1 we defined there another parameter M, which however has nothing to do with this further M controlled by timeRestrictionForChannelMeasurement), when one beam is fulfilling the condition less than M times in the reporting instance, then the gNB cannot exactly identify what beams are the beams to fulfilling the event trigger. However, the interpretation of the reported RSRP can be up to network implementation.

On report format for Event-2, do not support reporting of additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

There is a further discussion point on the assumption for the current beam. If there is no indication of current beam assumption provided by the UE, when the event occurs, there could be ambiguity at the gNB regarding which current beam/ TCI state the UE has used to determine the event occurrence. Such ambiguity would, for example, specifically happen when the UE is configured with a time window for the determination of event occurrence, and when a new TCI state, i.e., a new current beam, becomes applicable within that time window. Further, ambiguity may also occur when the DCI indicating a (new) TCI state is mis-detected. In fact, in RAN1#118, there was an FFS point on the indication of the ‘current beam assumption’:

- FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

Figure 1 illustrates two examples/cases where ambiguity regarding current beam assumption can happen.

Figure 1: Illustration of examples with ambiguity regarding current beam assumption.

- Case 1 (in the Figure 1): M=1, i.e., event triggered if (at least) one instance of measurement satisfies the event. It can be noticed that, in this case, unless the UE provides an indication regarding the current beam assumption, the gNB would not be able to know whether TCI state 1 or TCI state 2 has been used by the UE to trigger the event, and thus for which TCI state/beam the reported RSRP would correspond to.

- Case 2 (in the Figure 1): M=2, i.e., event triggered if (at least) two instances of measurements satisfy the event. Similar ambiguity as in Case 1 can also be observed in this case.

Thus, there could be some ambiguity on the ‘current beam’ between UE and gNB when beam switching happens.

If there is no indication of ‘current beam’ assumption, when the event occurs, there could be ambiguity at the gNB regarding which current beam/ TCI state the UE has used to determine the event occurrence. Such ambiguity would essentially happen due to TCI state switching.

This then justifies the need for either reporting indication regarding the current beam assumption (e.g., corresponding TCI state identifier, RS index, or simply ‘current beam’ change, etc.) or some predefined rule that would clear out such ambiguity, for example assuming that the current beam is the one associated to the indicated TCI state at the time of the UE transmission of the first PUCCH.

On report format for Event-2, support the network to configure the UE to indicate/report the ‘current beam’ assumption in the UE-initiated beam report.

Report format for Event-1 and Event-7

Beam report for Event-1 and Event-7 should re-use as much as possible the design for Event-2, on the other hand we think that few modifications are needed.

Event-1 is triggered only because of the current beam, so it should be that the current beam is always reported.

On report format for Event-1, support that current beam is always reported.

For Event-7, the “current beam” against which new beams are compared should be interpreted differently as already discussed for scheme-1 and scheme-2, see also agreement here below.

[119] Agreement On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Therefore, the current beam that is reported when Event-7 is triggered should not be the RS associated to the indicated TCI state, but it should be the RS associated to the activated TCI state with the Q-th best quality.

On report format for Event-7, specify that the current beam is the RS associated to the activated TCI state with the Q-th best quality.

## Cross-CC measurement/report

On cross-CC beam report transmission procedure, we agreed in the last RAN1#119 meeting that the first PUCCH and the second PUSCH are from the same PUCCH group.

Furthermore, there was the discussion, with related Working Assumption, on if the first PUCCH and the second PUSCH can be from different PUCCH groups. We think however that such option would increase complexity, and therefore should not be supported.

On cross-CC beam report transmission procedure, revert the WA, and do not support that the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups.

Then, another open issue is about allowing or not current beam (or indicated TCI state) and new beams to be on different CC, with the following candidates on the table:

- The current beam RS and new beam RS(s) should be:

  - Option 1-1: the same CC.

  - Option 1-2: same/difference CC, i.e., no restriction.

- The indicated TCI state and new beam RS(s) should be:

  - Option 2-1: the same CC.

  - Option 2-2: same/difference CC, i.e., no restriction.

We think that Option 1-1, i.e., restricting current beam RS and new beam RS(s) to be on the same CC, is the most straightforward option.

On cross-CC beam report transmission procedure, support that current beam RS and new beam RS(s) should be on the same CC (Option 1-1).

## Beam application latency reduction

Regarding beam application latency reduction, we have the following issue:

[119] Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

The three alternatives which have been proposed for further discussion in RAN1#119 target any event, i.e., Event-1/2/7, and have the objective to reduce the TCI state activation delay.

-	Alt-1 proposes to automatically activate reported new beams. This alternative can be supported, although some further discussion is needed on some aspects. For example, in case a UE supports just one active TCI state, that is also the indicated TCI state, updating the active TCI state list in such context would mean that a UE decides a network beam switching, and of course that cannot be supported. A further questions arises here: what happens with the current of higher layer signaling? Hence with the MAC-CE operation which would have the authority to indicate such information! There can be in fact signaling conflicts as even as the information contained in a UEIBMR may not be in fact reflected in the higher layer signaling update coming after the UEIBMR reception.

-	Alt-3 proposes that, after the MAC CE, a very fast activation delay is required by the UE without additional SSB reception for T/F sync for the reported new beams. This seems more a RAN4 solution, and should be discussed within that WG.

-	Alt-2 is in our view similar to Alt-3, and just restricts this very fast activation delay after the MAC CE to a subset of the reported new beams, with this subset indicated for example by the UE. Similarly as for Alt-3, this seems as well a RAN4 solution, which should be discussed within that WG.

On beam application latency reduction, further study Alt-1 on automatic update of the active TCI state list with reported new beams, considering for example if new beams are just added to the active TCI state list or if we should allow replacing some actual beams in the active TCI state list. For Alt-2 and Alt-3, send an LS to RAN4 informing of potential latency reduction.

In the context of the enhancements that can be introduced after the UE has sent the beam report on PUSCH, besides beam application latency reduction solutions discussed above, we think some further aspects could be considered. The UE expectation after reporting certain beams is that these beams are for example activated via MAC-CE, to allow a potential fast beam switching. However, there may be a mismatch between beams reported by the UE on PUSCH and the MAC-CE command to update the active TCI state list, for example because a new beam reported by the UE may temporarily not be available for network scheduling for that UE. In such scenario, it makes sense that the UE stops monitoring, even just temporarily such reported but not activated beams for UE-initiated reporting.

After sending a UE-initiated beam report, the UE stops monitoring the new beams included in the UE-initiated beam report but not included in the subsequent TCI states Activation/Deactivation MAC-CE.

## Simultaneous multiple events configuration

If the UE is configured with multiple events, it should be discussed whether the events may be simultaneously configured. Because the agreed events target different use cases, e.g., Event-2 to facilitate beam switching, Event-1 to avoid Beam Failure Recovery and Event-7 to update the active TCI state list, we think that we should support that multiple events can be configured simultaneously.

Support that multiple events can be configured simultaneously, with one CSI report configuration per event.

In event-based reporting, the UE may be configured with multiple event-triggered reporting configurations that may have same reporting resource (reporting occasions) or the same resource for requesting uplink resources for reporting. Such resource allocation may create a conflict on the reporting (i.e. network may not be able to identify which of the events triggered a given report) or on the reporting resource request (i.e. network may not be able to know for which event UE has triggered the resource request).

Alternatively, each event may have independent reporting occasions (at the cost of restricting resources allocation flexibility and increasing scheduler complexity).

When the events are simultaneously configured, prioritization of the resources may be important to avoid missing triggered reports.

For the first PUCCH, when a UE is configured with multiple events, we have different options that can be studied for letting the network know which event(s) has been triggered. In particular, we consider the following ones:

- The first PUCCH is multi-bit, and the bits are used by the UE to indicate to the network which event has been triggered, for example 00 for Event-2, 01 for Event-1, 10 for Event-7 or similar. This option requires a new first PUCCH design, but has the advantage of using a common first PUCCH for all the events. This solution has been however discussed and precluded.

- We have a different PUCCH resource ID for each event, to let the network know which event has been triggered. The advantage of this option is that the first PUCCH single-bit design can be reused, but the disadvantage is a potential waste of resources as each event has its own first PUCCH.

- The first PUCCH is single-bit and may be triggered by any event (when the condition is met), and in the second UL channel the UE informs the network about which event(s) has been triggered. This option has the advantage of reusing completely the design of the single event case, but the drawback of letting the network know which event triggered the report only in the second UL channel.

Considering that single-bit first PUCCH has already been agreed, we propose to use it also for the case that multiple events are simultaneously configured.

When multiple events are simultaneously configured at the UE, support different single-bit first PUCCH resource(s) associated to the events.

For the second UL channel, there is then the issue of what to report in case multiple events are triggered simultaneously. This issue is relevant also considering that potentially different beam report formats can be associated to different events. For example, with Event-2 the UE may report N=2 beams and the beam associated to the indicated TCI state, whereas with Event-7 the UE may report N=4 beams (the network may configure different N for different events) and the beam associated to the activated TCI state with the Q-th best quality. In such context, we need to discuss which beam report, for example some prioritization may be needed based on Event ID (for example with a rule or with a network configuration such that Event-2 is reported over Event-7) or based on timing (for example by reporting the event that has been triggered/detected last) or even introducing some rule to determine a common beam report for all the triggered events.

When multiple events are simultaneously configured at the UE, discuss a prioritization/rule of the beam reports associated to the different triggered events.

# 3.	Conclusions

Based on the discussion above we made the following observations and proposals:

Proposal 1:	For Mode A, do not support Option-2, i.e., a prohibit timer started after the first PUCCH, for first PUCCH re-transmissions.

Proposal 2:	For both Mode A and Mode B, do not support Option-1, i.e., a prohibit timer started after the PUSCH, for first PUCCH re-transmissions.

Proposal 3:	Do not consider simultaneous events for a single CSI report configuration.

Proposal 4:	Regarding both Mode-A and Mode-B, support one-bit indication in the first PUCCH for the multi-CC(s) or multi-event(s) case.

Proposal 5:	For handling overlapping UL channels involving the first channel, the new UCI type can be treated as SR, i.e., similar multiplexing/dropping rules as for SR can be used for the new UCI type.

- Note: this applies at least to PUCCH(s) carrying HARQ-ACK and/or CSI and overlapping with the first channel.

Proposal 6:	Regarding the case where the 1-bit first PUCCH overlaps with a PUCCH carrying normal SR or normal LRR, adopt the following approach: Allow overlap of the first PUCCH with another PUCCH for SR or for LRR, where only dropping PUCCH(s) is supported (i.e., no multiplexing).

- For the purpose of determining which PUCCH to drop/prioritize, downselect one of the following options:

- Option a: based on rules

- Option b: based on gNB configuration

- Option c: left up to UE implementation

Proposal 7:	Regarding the case where the 1-bit first PUCCH overlap with a PUSCH, adopt the following approach: Allow overlap of the first PUCCH with a PUSCH, where only dropping one of the PUCCH and PUSCH is supported (i.e., no multiplexing).

- For the purpose of determining which UL channel to drop/prioritize, downselect one of the following options:

- Option a: based on rules

- Option b: based on gNB configuration

- Option c: left up to UE implementation

Proposal 8:	Regarding the case where 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting overlap in the time domain, adopt the following approach: Allow overlap where only dropping is supported (i.e., no multiplexing).

- For the purpose of determining which PUCCH to drop/prioritize, downselect one of the following options:

- Option a: based on rules

- Option b: based on gNB configuration

- Option c: left up to UE implementation

Proposal 9:	For Mode A, support having a non-dedicated CSI trigger state in Step-2.

Proposal 10:	For Mode B, do not support different periodicities of first PUCCH resource and pre-configured resource for second UL channel.

Proposal 11:	Regarding the triggering event determination within a time window for Event-2, for resetting the counting support Candidate #2 (update of the indicated TCI state):

-	If deemed necessary, Candidate #1 (reconfiguration of new beams) and Candidate #6 (reconfiguration of threshold for event evaluation) can also be supported as a joint Candidate on RRC re-configuration of the CSI report configuration associated to Event-2.

Proposal 12:	Regarding the triggering event determination within a time window for Event-2, capture the counter-based mechanisms in RAN1 specifications.

Proposal 13:	Regarding the triggering event determination for Event-1, support having a RS resource set for new beam configured, and then using the implicit manner for enabling either scheme-1 or scheme-2 (Option-2).

Proposal 14:	Regarding the triggering event determination for Event-7, specify N≥Q.

Proposal 15:	On report format for Event-2, do not support reporting of additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

Observation 1:	If there is no indication of ‘current beam’ assumption, when the event occurs, there could be ambiguity at the gNB regarding which current beam/ TCI state the UE has used to determine the event occurrence. Such ambiguity would essentially happen due to TCI state switching.

Proposal 16:	On report format for Event-2, support the network to configure the UE to indicate/report the ‘current beam’ assumption in the UE-initiated beam report.

Proposal 17:	On report format for Event-1, support that current beam is always reported.

Proposal 18:	On report format for Event-7, specify that the current beam is the RS associated to the activated TCI state with the Q-th best quality.

Proposal 19:	On cross-CC beam report transmission procedure, revert the WA, and do not support that the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups.

Proposal 20:	On cross-CC beam report transmission procedure, support that current beam RS and new beam RS(s) should be on the same CC (Option 1-1).

Proposal 21:	On beam application latency reduction, further study Alt-1 on automatic update of the active TCI state list with reported new beams, considering for example if new beams are just added to the active TCI state list or if we should allow replacing some actual beams in the active TCI state list. For Alt-2 and Alt-3, send an LS to RAN4.

Proposal 22:	After sending a UE-initiated beam report, the UE stops monitoring the new beams included in the UE-initiated beam report but not included in the subsequent TCI states Activation/Deactivation MAC-CE.

Proposal 23:	Support that multiple events can be configured simultaneously, with one CSI report configuration per event.

Observation 2:	When the events are simultaneously configured, prioritization of the resources may be important to avoid missing triggered reports.

Proposal 24:	When multiple events are simultaneously configured at the UE, support different single-bit first PUCCH resource(s) associated to the events.

Proposal 25:	When multiple events are simultaneously configured at the UE, discuss a prioritization/rule of the beam reports associated to the different triggered events.

# References

[1] RP-234007 New WID: NR MIMO Phase 5