3GPP TSG RAN WG1 #120	R1-2500908

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.1

Source: 	ETRI

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on remaining details for SBFD TX/RX/measurement procedures.

Discussion

Table 1 captures the agreements on DL receptions and UL transmissions in SBFD subbands.

Table 1. Agreements on DL receptions and UL transmissions in SBFD subbands [3].

Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot., except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition if it is mapped to both SBFD and non-SBFD symbols is dropped. Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

According to the first agreement in Table 1, UE behaviour for PUSCH repetition type B while a nominal repetition is across boundary of SBFD symbols and non-SBFD symbols. Among the available two options, we think option 1 is aligned with the original use case / motivation of PUSCH repetition type B, which is to ensure both time constraint and reliability, simultaneously. Given that this new type of actual repetitions from option 1 can be implemented by software update (i.e., no hardware impacts are foreseen), we think option 2 has no benefits but removes potential uplink transmission occasions.

Proposal 1. For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols (option 1).

Figure 1 shows an example for UL resource allocations across SBFD/UL symbols. In some sense, the SBFD symbols and the UL symbol may have different pre/post-processing and those might cause the event to break the channel coherence for the UL signal at either/both UE or gNB, in which case the antenna port assumption does not hold. However, such events may not be induced from some of network implementations, e.g., allocating the same frequency resource across SBFD symbols and non-SBFD symbols. In this regard, although separate power control will be supported for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots in Rel-19 as captured in Table 1, we think it needs to be clarified that a single power control still can be used for both SBFD and non-SBFD symbols (i.e., separate power control for SBFD and non-SBFD symbols is not a mandatory feature).

Figure 1. Uplink power consistency / phase continuity between SBFD resources and TDD resources.

Observation 1. Per the network implementation, a UE may maintain power coherence across SBFD/non-SBFD symbol boundary for uplink transmissions.

Proposal 2. RAN1 to clarify that a single power control can be used for both SBFD and non-SBFD symbols (i.e., separate power control for SBFD and non-SBFD symbols is not a mandatory feature).

Table 2 shows the agreements on CSI measurement and report in SBFD subbands.

Table 2. Agreements on CSI report and TBS determination [1], [2], [3].

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols. Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation. - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured). - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands. Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only. Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Conclusion Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy. Nominal CSI reporting subband size in SBFD symbols is determined based on the size of DL BWP as legacy. Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

RAN1 has defined specification supports on TBS determination for various types of PDSCH receptions and PUSCH transmissions on SBFD symbols. Since TBS determination for CSI derivation is also one of the important aspects for accurate CSI report, we think a clear agreement on TBS determination for SBFD CSI report is required as well as those for the other PDSCH receptions and PUSCH transmissions. Based on the previous agreements on how to define time and frequency resources for SBFD TBS determination, we suggest the following proposal to clarify TBS determination for SBFD CSI report and non-SBFD CSI report.

Proposal 3. For a CSI report associated with periodic/semi-persistent CSI-RS,

- if the valid symbol type configured by the CSI report is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

  - CSI reference resource for a CSI report with valid symbol type configured as SBFD symbol should include one or more SBFD symbols and one or more DL usable PRBs

- if the valid symbol type configured by the CSI report is non-SBFD symbols, only the REs on non-SBFD symbols in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

Regarding a CSI report associated with aperiodic CSI-RS, we propose to start the discussion with followings:

Proposal 4. For a CSI report associated with aperiodic CSI-RS,

- if the aperiodic CSI-RS is measured in SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

  - CSI reference resource for a CSI report with aperiodic CSI-RS measured in SBFD symbol should include one or more SBFD symbols and one or more DL usable PRBs

- if the aperiodic CSI-RS is measured in non-SBFD symbols, only the REs on non-SBFD symbols in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

With the clarifications in proposal 3 and 4, we think the specification will ensure that the UE will report a CSI, only if all of the corresponding CSI reference resource, CMR, and IMR is in the same type of symbols, i.e., either of SBFD symbols or non-SBFD symbols. Otherwise, there will be no need to report such CSI, which should be contaminated by mixed measurements/assumptions across SBFD and non-SBFD symbol types.

Proposal 5. A SBFD-aware UE reports a CSI, only if all of the corresponding CSI reference resource, CMR, and IMR is in the same type of symbols, i.e., either of SBFD symbols or non-SBFD symbols.

- Otherwise, the CSI report is dropped.

As shown by one of the agreements in Table 2, CSI computation delay requirements were relaxed for SBFD subbands CSI report. Figure 2 summarizes our analysis on the remaining issues to finalize specification supports on SBFD CSI report. Per the previous agreements, we think that CPU occupation (first bullet in Figure 2), CSI processing timeline (second bullet in Figure 2), and subband size for subband CQIs (forth bullet in Figure 2) had been well clarified already. However, at least two remaining issues, i.e., 1) number of wideband CQI(s) for 2 DL subbands (1 for both DL subbands or 2), and 2) CSI dropping rule (currently, the same value of PriiCSIy,k,c,s can be assumed for 2 DL subbands) should be further discussed.

Figure 2. Remaining issues on SBFD CSI report.

Proposal 6. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the number of wideband CQI is one for a given CSI reference resource

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: the number of wideband CQI can be up to two for a given CSI reference resource. (One wideband CQI per DL subband)

Proposal 7. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the same value of PriiCSIy,k,c,s is applied for all DL subbands.

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: different values of PriiCSIy,k,c,s are applied across different DL subbands.

Conclusion

In this contribution, ETRI’s views on SBFD were shown and the following observation and proposals were made:

Observation 1. Per the network implementation, a UE may maintain power coherence across SBFD/non-SBFD symbol boundary for uplink transmissions.

Proposal 1. For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols (option 1).

Proposal 2. RAN1 to clarify that a single power control can be used for both SBFD and non-SBFD symbols (i.e., separate power control for SBFD and non-SBFD symbols is not a mandatory feature).

Proposal 3. For a CSI report associated with periodic/semi-persistent CSI-RS,

- if the valid symbol type configured by the CSI report is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

  - CSI reference resource for a CSI report with valid symbol type configured as SBFD symbol should include one or more SBFD symbols and one or more DL usable PRBs

- if the valid symbol type configured by the CSI report is non-SBFD symbols, only the REs on non-SBFD symbols in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

Proposal 4. For a CSI report associated with aperiodic CSI-RS,

- if the aperiodic CSI-RS is measured in SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

  - CSI reference resource for a CSI report with aperiodic CSI-RS measured in SBFD symbol should include one or more SBFD symbols and one or more DL usable PRBs

- if the aperiodic CSI-RS is measured in non-SBFD symbols, only the REs on non-SBFD symbols in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

Proposal 5. A SBFD-aware UE reports a CSI, only if all of the corresponding CSI reference resource, CMR, and IMR is in the same type of symbols, i.e., either of SBFD symbols or non-SBFD symbols.

- Otherwise, the CSI report is dropped.

Proposal 6. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the number of wideband CQI is one for a given CSI reference resource

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: the number of wideband CQI can be up to two for a given CSI reference resource. (One wideband CQI per DL subband)

Proposal 7. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the same value of PriiCSIy,k,c,s is applied for all DL subbands.

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: different values of PriiCSIy,k,c,s are applied across different DL subbands.

References

- 3GPP RAN1#117 Chair notes

- 3GPP RAN1#118-bis Chair notes

- 3GPP RAN1#119 Chair notes