3GPP TSG-RAN WG1 Meeting #120	R1-2500977

Athens Greece, February 17 – February 21, 2025

Source:	Panasonic

Title: 	Enhancements for UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion

# Introduction

During RAN#102, a new WID for Rel-19 MIMO evolution for DL and UL was agreed in RP-234007. One of the objectives agreed is as follows:

Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching.

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting.

In this contribution, we discuss several design criteria for enhancing the UE-initiated/event-driven beam management procedure.

# Discussion

## UE-initiated Beam Switching and Latency Reduction

In the UE-initiated beam reporting, the UE might report a beam that is not in the activated TCI state pool, and this will induce some latency since according to TS38.133, the UE, after receiving the NW activation command, is required to receive the SSB corresponding to the TCI state for synchronization to be complete.

RAN1 should address reducing this latency, since unlike the case of NW-initiated beam reporting, the UE-initiated reporting should be designed and configured so that most beam reports lead to an updated TCI state, and therefore once the UE transmits such a beam report, it is quite likely that a TCI state activation will follow shortly after.

However, there is no need to send synchronization information in the UE-initiated report. For example, after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. Moreover, we think that some input from RAN4 could be needed on this issue.

In the previous meeting, the following agreement was made:

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

  - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

    - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

    - update TCI state(s) with the reported new beam(s)

    - activate new beam(s) without additional SSB reception

  - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

    - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

    - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

  - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

  - Other alternatives are not precluded

Alt-1 is unclear since the NW might not send a confirmation/acknowledgement. Also, it suggests a UE-initiated beam switching procedure which is a separate discussion.

On the other hand, Alt-2 requires reporting which of the new beam(s) that are included in the UEI/ED beam report are synchronized. After receiving TCI state activation, the TCI state(s) associated with the synchronized beams becomes applicable for DL reception without additional SSB reception.

Alt-3 is the simplest because it does not require reporting that the new beam(s) associated with the TCI state(s) is synchronized in the UEI/ED beam report.

Support Alt-3 where after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

Regarding beam switching, this can be discussed separately and can be considered as a UE capability.

Study the feasibility, as a UE capability, of UE-initiated Beam switching after the UE identifies a favorable beam and it is reported successfully to the gNB.

## Event 2 Signalling Content

### L1-RSRP Based Triggering

L1-RSRP levels can fluctuate a lot and would require further processing for it to be useful, otherwise the event is triggered very frequently causing unnecessary overhead.

Therefore, in RAN1#117, the following was agreed:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs.

FFS: Whether the above is captured in RAN1 or RAN2 specification.

More details need to be specified regarding the triggering mechanism above. It is important to specify the time window's starting point with respect to beam of index j. The time window's starting point should be when the UE detects that the L1-RSRP of the new beam of index j becomes a threshold value better than the current beam for the first time (i.e. Event 2 instance w.r.t. beam of index j). It is explained in Figure 1. A different time window is used for each of the new beams detected that reach a threshold level higher than the current beam. Some companies suggested using a backward window, but whether backward or forward window is used, the detection mechanism would be the same.

Regarding the triggering for Event 2, a time window’s starting point with respect to beam of index j is when the UE detects that the L1-RSRP of the new beam of index j becomes a threshold value better than the current beam for the first time (i.e. Event 2 instance w.r.t. beam of index j).

Figure 1

Finally, in RAN1#118b, there was a discussion on whether a resetting mechanism for the counter and window is needed, and if so, what is the condition for resetting it. Following candidates were suggested:

- Candidate#1: RS reconfiguration for new beam is received;

- Candidate#2: The indicated TCI state is updated;

- Candidate#3: The number of UEI beam report transmission is larger than or equal to a threshold.

- Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected.

However, the counter and window are defined with respect to a specific new beam and specific current beam. When either of those beams change, a new counter and window are defined. In the case that Event 2 was triggered with respect to a new beam and current beam, then the UE needs to report the event to the NW and reset the counter/timer with respect to those specific current beam and new beam.

The counter and window for event 2 are defined with respect to a specific new beam and specific current beam. When either of those beams change, a new counter and window are defined.

In the case, that Event 2 was triggered with respect to a new beam and current beam, then the UE needs to report the event to the NW and reset the counter/timer with respect to those specific current beam and new beam.

### RS measurement for the new beam

Regarding RS measurement for the new beam for Event-2, RAN1 agreed that the RS(s) for new beam(s) are explicitly configured.

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

Beam failure recovery is a more critical procedure than UE-initiated beam reporting. For beam failure recovery procedure in legacy, a MAC-CE is used to update the RSs for beam failure detection (failureDetectionSet) and not the candidate list that is explicitly configured (candidateBeamRS).

Moroever, the new beam set configuration will be configured inside CSI-ReportConfig within the CSI framework (no MAC-CE update functionality there either). Using the CSI framework, it should be possible to configure different new beam sets in different CSI-ReportConfig. The UE does measurements using a different CSI-ReportConfig, rather than updating dynamically the beam set. Therefore, we support following legacy CSI framework, and we do not support using a MAC-CE to update the RSs.

Do not support MAC-CE update of the RS(s) for new beam measurement.

### UL Signalling Content

In legacy beam management, the UE can report a panel index by reporting a capability index. Therefore, for UE-initiated beam management, it is important that panel switching information could be also reported if the UE needs to switch panels and if the UE supports multiple panel architecture.

Panel switching information should be also reported for UEs with multiple panels.

Moreover, in RAN1#116b, the following options were discussed regarding UL signaling content(s) of L1-RSRP report depending on Event-2:

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

In RAN1#117, Option 3 was agreed as follows:

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

As indicated in a previous agreement in section 2.2.1, there are two flavors of Event 2 triggering with the basic feature being:

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs.

Therefore, for Option 3 reporting, it should be clarified what is meant by “At least one of N reported beam(s) should satisfy the condition of Event-2.” Triggering of the event is different that beam reporting. For beam reporting, a new beam that satisfises condition of Event 2, is simply a beam whose L1-RSRP becomes a threshold value better than the current beam.

For Option 3 beam reporting for Event 2, a new beam that satisfies condition of Event 2, is a beam whose L1-RSRP becomes a threshold value better than the current beam.

In RAN1#118bis, it was discussed whether additional indication of which CRI/SSBRI(s) satisfies the condition of Event-2 is reported per CRI/SSBRI. We think that this is not necessary since the NW can estimate using the reported beams L1-RSRP and the threshold, which beams likely satisfy the condition of Event-2. Additional reporting for beams that are below the threshold causes additional unnecessary reporting overhead. No further enhancement is needed.

For Option 3 beam reporting for Event 2, do not support additional indication of which reported CRI/SSBRI(s) satisfies the condition of Event-2.

Moreover Option-1/1a/1b/2 for beam reporting are still FFS. In option 1, the network configures the gNB with the maximum number it can report. Since all the beams reported are above the threshold, then in low congestion scenarios, reporting the RSRP of the corresponding beams can be omitted, and therefore some overhead could be saved by not reporting the RSRP of the reported beams. The number of beams reported needs to be indicated as variable size option.

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, support the following:

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

- The N beam(s) should satisfy the condition of Event-2

- Nmax is configured by gNB

- FFS: Including RSRP information of reported beams is configurable.

- FFS: Whether the indication of payload size should be provided additionally.

## UL Signalling Medium of the UE-initiated BM Reporting

The UE initiated reporting follows the configuration inside CSI-ReportConfig. The type of the event triggering the report can also be included in the CSI-ReportConfig. Therefore, multiple events can be configured at the same time by configuring a different CSI-ReportConfig with different event configuration.

Study whether/how to support the configuration of multiple events simultaneously and if needed determining the priority order of the events reporting.

Multiple CSI-ReportConfig can be associated with the same PUCCH resource for Modes A or Mode B respectively. Within each Mode A or Mode B, the UE report can be used to distinguish which CSI-ReportConfig the report belongs to from the list of configured CSI-ReportConfigs. This would reduce overhead by eliminating the need to configure different PUCCH resources. The PUCCH resource for Mode A and Mode B should be separated (if both are configured is supported) as the action of the gNB after the reception of step 1 is different between Mode A and Mode B. Note that our preference/assumption is Mode A and Mode B are not configured together for simplicity.

Multiple CSI-ReportConfig can be associated with the same PUCCH resource. Mode A and Mode B are not configured together. If simultaneous configuration together is supported, then multiple CSI-ReportConfig can be associated with the same PUCCH resource but distinct for Modes A and Mode B.

### Mode A Design

After receiving the scheduling request, the NW sends a scheduling DCI. The NW indicates a CSI request field that would match the CSI-ReportConfig that the UE is using for reporting. In legacy, a CSI request field (configured to be between 0 and 6 bits wide) points to an aperiodic trigger state. The trigger state is defined as a list of up to 16 aperiodic CSI-ReportConfigs (typically a much lower number).

In RAN1#118b/119, the following agreements were made regarding reusing the CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

- Option-2: A CSI trigger state can be associated with

  - only UE-initiated/event-driven beam reporting

  - or only legacy AP-CSI configuration

  - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

First, the UE would be configured with a group of CSI-ReportConfig that corresponds to legacy CSI reporting and it would be configured with another group of CSI-ReportConfig that corresponds to UE-initiated beam reporting. The network maps a trigger state to group of CSI-ReportConfig. How this mapping is done is up to NW implementation.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, a CSI trigger state can be associated with UE-initiated/event-driven beam reporting and legacy AP-CSI configuration.

When the UE has a triggering event on a certain CSI-ReportConfig, the UE only needs to determine the PUCCH resource and transmit the request bit. In mode A, the UE expects a scheduling DCI and it has to know from the CSI request field that this DCI is a response for the request bit. When the UE receives the DCI with a CSI request that matches the CSI-ReportConfig which the UE is using for reporting, the UE can use this as an Ack for the success of step 1. Otherwise, the UE can retransmit step 1.

When the UE receives the DCI with a CSI request that matches the CSI-ReportConfig which the UE is using for reporting, the UE can use this as an Ack for the success of step 1. Otherwise, the UE can retransmit step 1.

If the CSI trigger state of the CSI request field triggers only one ReportConfig, then this is a straightforward case. If the CSI trigger state triggers many CSI_ReportConfig, then the UE needs to be specified what to do.

- Option 1: transmit only one UE-initiated beam report

- Option 2: transmit multiple UE-initiated beam report

- Option 3: transmit multiple UE-initiated beam report alongside legacy CSI reports.

It makes sense that the UE always transmits the legacy CSI reports requested by the NW and are included in the trigger state. The UE can append one or multiple UE-initiated beam reports that have triggered by one or multiple events.

Upon receiving a CSI request in the UL scheduling DCI, UE always transmits the legacy CSI reports requested by the NW which are included in the trigger state. The UE can append one or multiple UE-initiated beam reports that have been triggered by one or multiple events.

- FFS: the number of UE-initiated beam reports, report multiplexing rules.

Moreover, for Mode A reporting, the UE should be configured with a first timer that it starts after sending the request bit in step 1. If the UE does not receive a scheduling DCI for UE-initiated beam reporting before the timer expires, the UE retransmits step 1. Combining the two above criteria, we support the following,

For Mode A reporting, the UE should be configured with a first timer that it starts after sending the request bit in step 1 on PUCCH. If the UE does not receive a scheduling DCI for the requested UE-initiated beam reporting in Step 1 before the timer expires, the UE retransmits step 1.

Moreover, after the UE transmits the report in step 3, if it is not received by the NW, the NW can ask the UE to retransmit the report again by sending another DCI. However, given the time sensitivity of these reports and the delay in reporting, the UE should be configured with a second timer.  When the UE transmits step 1, the UE starts the second timer. If the UE sends the report in step 3, and did not receive a CSI retransmission request from the NW by the time this timer expires, the UE is not required to store further the report contents and does not report the triggering event anymore.

For Mode A reporting, when the UE transmits step 1, the UE starts the second timer, which was configured before. If the UE sends the report in step 3, and did not receive a CSI retransmission request from the NW by the time this timer expires, the UE is not required to store the report contents and does not report the triggering event anymore.

### Mode B Design

- In RAN1#118b, the following agreement was made.

- Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

- Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI.

FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

- A remaining issue is whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report. In legacy, the NW can schedule dynamic grant UL transmission on CG PUSCH. We think legacy procedures should be followed here as well.

Regarding reporting mode B when the pre-configured Type-1 CG PUSCH does not carry the beam report, legacy procedure is followed where the NW can schedule dynamic grant UL transmission on the pre-configured Type-1 CG PUSCH for UE-initiated beam reporting.

UE is not expected to multiplex UE-initiated beam report with the dynamic grant UL transmission for reporting mode B.

Unlike Mode A, the motivation is not clear if the UE should start a timer after the transmission of the request bit on PUCCH. If no retransmission of the report is possible, then no timer is needed. We suggest discussing the need of any retransmission schemes for Mode B first.

Study whether to allow the retransmission of the report for Mode B and whether a timer for mode B is needed after the transmission of the request bit on PUCCH.

## Event 1 Signalling

RAN1 agreed to support Event-1 (Quality of the current beam is worse than a certain threshold). Event 1 and Event 2 would share the same design when applicable, to simplify design and minimize spec change.

Below we mention a few proposals that are derived from agreements for Event 2:

At least L1-RSRP is supported as quality metrics used for Event-1

‘Current beam’ for Event 1 is a beam corresponding to the indicated TCI state. RS for current beam is the QCL RS in the indicated TCI state

It would also be useful to configure a set of new beams that the UE can report after Event 1 is triggered. The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration. Moreover N ≥ 0 beam(s) are reported in the report instance where the candidate value of ‘N’ at least comprises {0,1, 2, 3, 4}. Here the new beams reported are beams that are above a certain threshold. When N=0, the UE only transmits step 1 in Modes A and B.

Support Option 2: A set of new beams are configured that the UE can report after Event 1 is triggered. The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration. and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

N ≥ 0 beam(s) are reported in the report instance where the candidate value of ‘N’ at least comprises {0,1, 2, 3, 4}. The new beams reported are beams that are above a certain threshold.

The triggering mechanism would be slightly different since only the current beam is used to trigger the reporting.

Regarding the triggering event determination for Event 1:

- If within a time window (which is configurable), the number of Event-1 instance(s) is greater than or equal to a configurable number M, UE initiated beam report occurs.

Both signalling modes A and B are supported for all supported events according to the agreement below in RAN1#116b (agreement not tied to an event type):

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

If Event 1 is supported, then both signalling modes A and B are supported for Event 1

# Conclusions

Based on the discussion above, we propose the following:

- Support Alt-3 where after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

Study the feasibility, as a UE capability, of UE-initiated Beam switching after the UE identifies a favorable beam and it is reported successfully to the gNB.

Regarding the triggering for Event 2, a time window’s starting point with respect to beam of index j is when the UE detects that the L1-RSRP of the new beam of index j becomes a threshold value better than the current beam for the first time (i.e. Event 2 instance w.r.t. beam of index j).

- The counter and window for event 2 are defined with respect to a specific new beam and specific current beam. When either of those beams change, a new counter and window are defined.

In the case, that Event 2 was triggered with respect to a new beam and current beam, then the UE needs to report the event to the NW and reset the counter/timer with respect to those specific current beam and new beam.

Do not support MAC-CE update of the RS(s) for new beam measurement.

Panel switching information should be also reported for UEs with multiple panels.

For Option 3 beam reporting for Event 2, a new beam that satisfies condition of Event 2, is a beam whose L1-RSRP becomes a threshold value better than the current beam.

For Option 3 beam reporting for Event 2, do not support additional indication of which reported CRI/SSBRI(s) satisfies the condition of Event-2.

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, support the following:

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

- The N beam(s) should satisfy the condition of Event-2

- Nmax is configured by gNB

- FFS: Including RSRP information of reported beams is configurable.

- FFS: Whether the indication of payload size should be provided additionally.

Study whether/how to support the configuration of multiple events simultaneously and if needed determining the priority order of the events reporting.

Multiple CSI-ReportConfig can be associated with the same PUCCH resource. Mode A and Mode B are not configured together. If simultaneous configuration together is supported, then multiple CSI-ReportConfig can be associated with the same PUCCH resource but distinct for Modes A and Mode B.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, a CSI trigger state can be associated with UE-initiated/event-driven beam reporting and legacy AP-CSI configuration.

When the UE receives the DCI with a CSI request that matches the CSI-ReportConfig which the UE is using for reporting, the UE can use this as an Ack for the success of step 1. Otherwise, the UE can retransmit step 1.

Upon receiving a CSI request in the UL scheduling DCI, UE always transmits the legacy CSI reports requested by the NW which are included in the trigger state. The UE can append one or multiple UE-initiated beam reports that have been triggered by one or multiple events.

- FFS: the number of UE-initiated beam reports, report multiplexing rules.

For Mode A reporting, the UE should be configured with a first timer that it starts after sending the request bit in step 1 on PUCCH. If the UE does not receive a scheduling DCI for the requested UE-initiated beam reporting in Step 1 before the timer expires, the UE retransmits step 1.

For Mode A reporting, when the UE transmits step 1, the UE starts the second timer, which was configured before. If the UE sends the report in step 3, and did not receive a CSI retransmission request from the NW by the time this timer expires, the UE is not required to store the report contents and does not report the triggering event anymore.

Regarding reporting mode B when the pre-configured Type-1 CG PUSCH does not carry the beam report, legacy procedure is followed where the NW can schedule dynamic grant UL transmission on the pre-configured Type-1 CG PUSCH for UE-initiated beam reporting.

UE is not expected to multiplex UE-initiated beam report with the dynamic grant UL transmission for reporting mode B.

Study whether to allow the retransmission of the report for Mode B and whether a timer for mode B is needed after the transmission of the request bit on PUCCH.

At least L1-RSRP is supported as quality metrics used for Event-1

‘Current beam’ for Event 1 is a beam corresponding to the indicated TCI state. RS for current beam is the QCL RS in the indicated TCI state

Support Option 2: A set of new beams are configured that the UE can report after Event 1 is triggered. The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration. and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

N ≥ 0 beam(s) are reported in the report instance where the candidate value of ‘N’ at least comprises {0,1, 2, 3, 4}. The new beams reported are beams that are above a certain threshold.

Regarding the triggering event determination for Event 1:

- If within a time window (which is configurable), the number of Event-1 instance(s) is greater than or equal to a configurable number M, UE initiated beam report occurs.

If Event 1 is supported, then both signalling modes A and B are supported for Event 1

# References

- [1]		RP-234007, New WID: NR MIMO Phase 5, RAN#102, Dec 2023