3GPP TSG RAN WG1 Meeting #120	                           R1- 2500982

Athens, Greece, February 17th – February 21th, 2025

Agenda Item: 9.11.5

Source:	TCL

Title: Discussion on the IoT-NTN TDD mode

Document for: Discussion and Decision

- Introduction

In the RAN1#119 meeting, the following agreement on IoT-NTN TDD mode had been achieved[1]:

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9. Agreement RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-U NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of U NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Impact on segmented pre-compensation. Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications Agreement The downlink subframes within D=8 are to be down-selected among the following: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

In this contribution, we provide our views on the study of IoT-NTN TDD mode.

- The impact of the TDD mode

In the last meeting, the three cases for IoT-NTN TDD mode meet the link budget requirements and are feasible for RAN1:

-Case #1: N=9, D=8. There are 8 DL subframes in every 90ms.

-Case #2: N=9, D=20. There are two consecutive radio frames(20 subframes) in every 90ms.

-Case #3: N=9, D=30. There are three consecutive radio frames(30 subframes) in every 90ms.

  - NPSS/NSSS detection

According to the legacy NB-IoT system, NPSS and NSSS are used to achieve DL synchronization and obtain cell ID. For NPSS detection, UE can detect NPSS in every radio frame and the TDD pattern will not have an impact. In case of NSSS, NSSS is transmitted in subframe#9 of every even frame with a period of 80ms and four cyclic shift values are applied across different frames. UE will assume NPSS in every radio frame and NSSS every 20ms. In TDD pattern, UE may not detect the available radio frame containing NSSS when N=9 and the usable DL radio frame within the N radio frames is one radio frame with the current transmission scheme. To solve this, the NSSS can be transmitted on every radio frame.

For case 1 and case 2, the formula for the cyclic shift of NSSS needs to be modified due to the period difference with the legacy scheme. For case 3, there will be three continuous radio frames for DL and the condition will be complex based on the current mapping rule of NSSS. If transmitting three consecutive radio frames starting from an odd frame within the first 90ms, the number of radio frames NSSS mapped on the first, second and third 90ms will be two, one, and two respectively. Conversely, when transmitting three consecutive subframes starting from an even frame within the first 90ms, the number of radio frames NSSS mapped on the first, second, and third 90ms will be one, two, and one respectively.

Observation 1: The formula of the cyclic shift of NSSS needs to be modified according to the pattern of the frame structure.

Proposal 1: RAN1 should take the impact of frame structure on the formula of NSSS into consideration.

  - NPBCH decoding

According to TS 36.211, the NPBCH of a cell will occupy 64 radio frams and transmit 1600 bits with QPSK for the normal cyclic prefix. 1600 bits will be divided into 8 NPBCH blocks and each block will carry 100 modulation symbols. The first transmission of the NPBCH block is scheduled on subframe #0 of radio frames fulfilling the condition that SFN mod 64=0 and the transmission of each NPBCH block will repeat 8 times. The different scrambling masks for each repetition transmission of the same NPBCH block are applied.

The system frame number ranges from 0 to 1023 and requires 10 bits to indicate. UE can obtain the 4 MSB of the system frame number information by decoding the MIB-NB in the NPBCH, while the remaining 6 bits of frame information are implicitly indicated through the decoding of the NPBCH. Specifically, the cell's NPBCH is transmitted through 8 independently decodable NPBCH blocks, indicating 3 bits of frame information. Each NPBCH block is repeatedly transmitted and applies 8 different scrambling sequences, indicating the remaining 3 bits. At the last meeting, the observations that at least N=9 is feasible and three cases of N=9 are feasible from the link budget requirements were approved. The transmission period of NPBCH will expand by 9 times with the current NPBCH transmission mechanism if N=9 and the available DL frame within the N radio frames is one radio frame. The delay of MIB-NB reception will be very large. Some methods to reduce the delay of the MIB-NB reception should be considered. For example, the repetition mode or number of NPBCH blocks can be modified. The repetition number of NPBCH blocks is associated with the bits for indicating the SFN numbers. The MSB decoded directly from the MIB-NB can be modified to decrease the repetition number of each NPBCH block. Also, the repetition of NPBCH block on the other available subframes in a radio frame can also be considered to shorten the transmission period of NPBCH.

Observation 2: For IoT NTN TDD mode, the delay of MIB-NB reception will be extended.

Proposal 2: For IoT NTN TDD mode, RAN1 can consider the following options to reduce the delay of MIB-NB reception:

  - Option 1: Modify the MSB bits of MIB-NB to decrease the repetition of NPBCH block

  - Option 2: Repetition of NPBCH block on the other available subframe in a radio frame

  - Discussion on the periodic pattern

The downlink subframe within D=9 when N=9 has been discussed in the last meeting and several options have been approved. These options can be considered as a period that starts from one subframe in the first radio frame and ends in the following radio frame. To ensure the downlink synchronization, the pattern should cover at least the subframes carrying NPSS(subframe#5), NSSS(subframe#9), and NPBCH(subframe#0). The reference time for the start of the periodic pattern can align with the radio frame SFN#0, which is an easy method to determine the start of the periodic pattern.

Proposal 3: The reference time of TDD pattern can align with the radio frame e.g. SFN#0

In order to implement TDD pattern, some information need to be determined or configured, such as the position information of the downlink, the position information of the gap between the downlink and uplink, and so on. Considering the flexibility and scalability in configuration, the duration of downlink and uplink can also be configured. The offset between the reference time can determine the start of the TDD pattern. The position information of the gap can be determined by several methods. For example, the offset between the start of the gap and the end of the downlink can be configured to determine the start of the gap. The duration of the gap can be configured to determine the end of the gap. Also, the offset between the start of the gap and the end of the downlink and the offset between the end of the uplink and the end of the downlink. Considering the overhead of signaling, the configuration method to implement the TDD pattern should be based on the least overhead.

Proposal 4: To implement TDD pattern, the method of parameter configuration should be based on the least signaling overhead.

In NB-IoT, the SFN index will be wrap around after 1024 radio frames. In IoT-NTN TDD mode, at least N=9 will be supported. When the start of TDD pattern aligns with SFN0, a hyper frame will not carry an integer number of periodic patterns and there will be 7 remaining radio frames. The direct solution is to skip the remaining radio frames before SFN wrap around.

Proposal 5: The orphan radio frames can be skipped directly before SFN wrap around.

  - Impact on random access procedure

In NB-IoT, the periodicity of NPACH is in the range of {40, 80, 160, 240, 320, 640, 1280, 2560}ms. The periodicities cannot match the integer number of 90ms so the chance of NPRACH transmission will greatly reduce. The preferred solution is to postpone the transmission of NPRACH to the next available uplink resource when it overlaps with non-Uplink subframes.

Proposal 6: Postpone the transmission of NPRACH to the next available uplink resource when it overlaps with non-Uplink subframes.

- Conclusion

In this contribution, we provided our views on IoT-NTN TDD mode. Our observations and proposals are as follows:

Observation 1: The formula of the cyclic shift of NSSS needs to be modified according to the pattern of the frame structure.

Observation 2: For IoT NTN TDD mode, the delay of MIB-NB reception will be extended.

Proposal 1: RAN1 should take the impact of frame structure on the formula of NSSS into consideration.

Proposal 2: For IoT NTN TDD mode, RAN1 can consider the following options to reduce the delay of MIB-NB reception:

  - Option 1: Modify the MSB bits of MIB-NB to decrease the repetition of NPBCH block

  - Option 2: Repetition of NPBCH block on the other available subframe in a radio frame

Proposal 3: The reference time of TDD pattern can align with the radio frame e.g. SFN#0.

Proposal 4: To implement TDD pattern, the method of parameter configuration should be based on the least signaling overhead.

Proposal 5: The orphan radio frames can be skipped directly before SFN wrap around.

Proposal 6: Postpone the transmission of NPRACH to the next available uplink resource when it overlaps with non-Uplink subframes.

- References

- Chair’s notes for RAN1# 119.