3GPP TSG RAN WG1 #120                                                                                                    R1-2500985

Athens, Greece, February 17th – 21st, 2025

Source:               TCL

Title:                    Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda item:      9.12.1

Document for:    Discussion and Decision

- Introduction

In the RAN #106 meeting, an revised WID on Rel-19 multi-cell PUSCH/PDSCH scheduling with a single DCI was approved [1], the objectives of the WID are shown below:

Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

In RAN1 #119 meeting, the following agreements have been achieved [2], as shown below:

Agreement

- Time-domain HARQ-ACK bundling is configured per cell as Rel-17.

Agreement

- Specification supports the maximum number of PUSCHs/PDSCHs for a scheduled cell by a DCI format 0_3/1_3 is 8.

- Payload size of a DCI format 0_3/1_3 exceeding 140 is not supported in Rel-19.

Agreement

- Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook.

- The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception.

- The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1.

- Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18.

- Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18.

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options.

  - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

  - Option 2: M is explicitly configured by RRC parameter for the PUCCH group.

  - Other options are not precluded.

Agreement

- For NDI indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18.

Agreement

- For RV indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

In this contribution, further enhancements for multi-cell PUSCH/PDSCH scheduling with a single DCI are discussed.

- Discussion

Reference PDSCH for HARQ-ACK reporting

For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs, it can guarantee enough processing time for PDSCH and preparation time for HARQ-ACK. In the current specification, procedure of a UE for reporting HARQ-ACK as shown below:

If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception.

For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.

If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

However, when there are multiple PDSCHs among the co-scheduled PDSCHs than share the same ending symbol, ambiguity may arise between the gNB and UE. This is due to the potential differences in the SCS/carrier type for the multiple PDSCHs, which could lead to varying reference points for determining the PUCCH for HARQ-ACK. In our understanding, the Rel-18 design principle can be followed, the reference PDSCH is the PDSCH with the smallest SCS and ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs.

Proposal 1: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs with different SCS/carrier type by a DCI format 1_3, the reference PDSCH is the PDSCH with the smallest SCS and ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs.

Type-2 HARQ-ACK codebook

As mentioned in the WID [1], the maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling, thus, 2 sub-codebooks for type-2 HARQ-ACK codebook in Rel-19 can be used.

In Rel-18 MC, for Type-2 HARQ-ACK codebook, two sub-codebook are generated with the first sub-codebook comprising HARQ-ACK information bits for PDSCH scheduled by DCI(s) with each scheduling a single cell, and the second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell, similar way can be re-used in Rel-19. If at least one cell of a set of cells which can be co-scheduled by DCI format 1_3 is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_3 that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_3 in the PUCCH group for the UE. Thus, In the second sub-codebook, each DCI corresponds to  HARQ-ACK bits, where  is the maximum total number of TBs in PDSCH receptions that can be scheduled by a DCI format 1_3 over more than one serving cells in a set of serving cells across the number of sets of serving cells. When more than one PDSCHs scheduled per cell is used, the similar way can also be re-used, with this way, separate DAI counting for DCI(s) with each scheduling a single PDSCH and DCI(s) with each scheduling more than one PDSCHs can be used.

Furthermore, the information bits can be ordered as, the first, in ascending order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair, the second, in ascending order of serving cell index, and the third, in ascending order of PDCCH monitoring occasion index.

Proposal 2: For type-2 HARQ-ACK information bits, can be ordered as, the first, in ascending order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair, the second, in ascending order of serving cell index, and the third, in ascending order of PDCCH monitoring occasion index.

NDI field size

The size of Option 1 for each block of NDI field is determined by the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell. Option 2a has been suggested as a method to reduce DCI overhead. However, compared to Option 1, it introduces more complexity into the UE implementation for DCI decoding. The UE needs to check the scheduled cell in the DCI to ascertain the number of scheduled PUSCHs/PDSCHs based on the TDRA-tables to finally being able to determine the number of the NDI bits. Therefore, option 1 is prefer.

- Proposal 3: For NDI indication in DCI format 0_3/1_3 for TB1, Option 1 should be supported. For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

RV field size

Similar to the NDI size determination, the size of Option 1 for each block of RV field is determined by the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell. Option 2a has been suggested as a method to reduce DCI overhead. However, compared to Option 1, it introduces more complexity into the UE implementation for DCI decoding. The UE needs to check the scheduled cell in the DCI to ascertain the number of scheduled PUSCHs/PDSCHs based on the TDRA-tables to finally being able to determine the number of the NDI bits. Therefore, option 1 is prefer.

In addition to the size of the RV field discussed above, there is another issue that needs to be determined, which 1-bit RV table for multiple PUSCHs/PDSCHs scheduling via single DCI in Rel-19 will be used. In the current specification, for multiple PUSCHs/PDSCHs scheduling via DCI format 0_1/1_1, the usage of RV table is shown below:

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 2 |

For multiple PUSCHs/PDSCHs scheduling via DCI format 0_3/1_3, the usage of RV table is shown below:

Table 7.3.1.2.3-1: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 3 |

In our views,  align with Rel-18 MC-DCI operation is suitable.

- Proposal 4: For RV indication in DCI format 0_3/1_3 for TB1, Option 1 should be supported. For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

  - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

Proposal 5: The RV sequence corresponding to a scheduled PUSCH/PDSCH of multi-PUSCHs/PDSCHs scheduling using DCI format 0_3/1_3 is align with Rel-18 MC-DCI operation (RV 0 and RV 3).

- Conclusion

In this contribution, the following proposals have been made:

Proposal 1: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs with different SCS/carrier type by a DCI format 1_3, the reference PDSCH is the PDSCH with the smallest SCS and ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs.

Proposal 2: For type-2 HARQ-ACK information bits, can be ordered as, the first, in ascending order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair, the second, in ascending order of serving cell index, and the third, in ascending order of PDCCH monitoring occasion index.

- Proposal 3: For NDI indication in DCI format 0_3/1_3 for TB1, Option 1 should be supported.  For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

- Proposal 4: For RV indication in DCI format 0_3/1_3 for TB1, Option 1 should be supported. For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

  - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

Proposal 5: The RV sequence corresponding to a scheduled PUSCH/PDSCH of multi-PUSCHs/PDSCHs scheduling using DCI format 0_3/1_3 is align with Rel-18 MC-DCI operation (RV 0 and RV 3).

4. References

[1] RP-242904, Revised WID: Multi-carrier enhancements for NR Phase 3.

[2] 3GPP RAN1 #119 meeting, Chairman notes.