3GPP TSG-RAN WG1 Meeting #120	R1-2501510

Athens, Greece, February 17-21 2025

Agenda Item:	7

Source:	Moderator (Huawei)

Title:	Summary #1 of discussion on skipping uplink transmission in case of BWP switching

Document for:	Discussion and Decision

Introduction

This document is created to collect company views on the proposals in [1] and [2].

Problem description

In Rel-15 and Rel-16, if no data is available for uplink transmission and gNB configures the skipUplinkTxDynamic or enhancedSkipUplinkTxDynamic-r16 as true, UE can skip UL transmission for an uplink grant. In the case of an uplink grant schedules an uplink transmission and it simultaneously indicates an active UL BWP change, a UE may skip UL transmission for this UL grant if no data is available. However, for gNB, it is not sure whether the BWP switch is successful or not due to no uplink transmission from the UE, and misunderstanding on actual BWP may happen between gNB and UE.

Based on the discussion in RAN #119, majority companies agree the above issue existing and controversial part is what kind of solutions should be adopted. In [1], proposed solutions are categorized in two categories:

- Category 1: Resolve the issue by gNB implementation without spec changing.

- Category 2: Resolve the issue with spec changing.

For each category, [1] also list multiple possible solutions.

1st round Discussion

Companies’ view

In moderator’s opinion, I think we can start based on the output from last meeting, and further decide which solution is applied in the end.

Proposal 1:

Conclude that when UL BWP change is triggered (e.g., by DCI format 0_1) and skipUplinkTxDynamic or enhancedSkipUplinkTxDynamic-r16 is set as true, following ambiguity exists:

- If the UE has no data to transmit, the gNB cannot know if the UE has successfully received the triggering DCI and understood the BWP change.

Q1: Do you agree the proposal 1 to conclude that the issue exists? If not, please elaborate the reason.

| Company | Yes/No | Comment |
| --- | --- | --- |
| HWHiSi | Yes | Based on our contribution [1], the issue happens in actual network when UL BWP switching is triggered by a UL grant and meanwhile UL BWP is UL skipping is enabled. Therefore, to make companies at same understanding, we believe a concrete conclusion will be helpful for further discussion. We support to have this conclusion. |
| ZTE |  | The proposal is more like an observation. In CR phase, it seems no need to agree on observations. [Moderator]: This is not an observation. This is to clarify the common understanding of RAN1 where an issue exists in current spec. although no dedicated solution is provided in the proposal, but it helps to make further progress in future. |
| NTT DOCOMO | Yes |  |
| Xiaomi |  | We share the same view with ZTE. [Moderator]: Please refer my response to ZTE. |
| Samsung |  | Similar view with ZTE/Xiaomi [Moderator]: Please refer my response to ZTE. |

Q2: If your answer for Q1 is yes, which category of solution do you prefer?

- Category 1: Resolve the issue without spec changing.

- Category 2: Resolve the issue with spec changing.

| Company | Which category? | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Category 2 | We prefer to clarify this issue in the spec clearly. In this way, it can guarantee that UE vendor and network vendor align the understanding correctly. |
| ZTE | Category 1 |  |
| NTT DOCOMO | Category 2 | Our preference is to make the specification clearer about how to resolve the issue. |
| Xiaomi | Category 1 | Solutions without spec change is always preferred, especially at this stage. |
| QC | Category 1 | Given this is a issue exist already in Rel-15, we prefer to leave the solution up to UE/NW implementation without spec impact. |
| Samsung | Category 1 |  |

Q3: If your answer for Q2 is category 1, which following option(s) do you prefer, and why?

- Option 1-1: When skipping UL transmission is enabled, UL grant-based BWP switching is avoided

- Option 1-2: gNB triggers a BWP switching only when it knows UE has data to transmit, i.e., based on SR or BSR.

- Option 1-3: If gNB is not sure about UL data availability at the UE side, it can indicate BWP switch on DL (and consequently UE switches UL BWP as well).

- Others are not precluded

| Company | Which option(s)? | Comment |
| --- | --- | --- |
| ZTE |  | All options could be applied by implementation. Other solutions could be also considered, e.g., NW uses AL =16 for DCI indicating BWP switch and assumes that the UE would not miss the DCI, i.e., NW always assumes the UE would change the BWP even no PUSCH is transmitted. [Moderator]: Yes, this could be another solution. |
| NTT DOCOMO | Comments | DCI 0_1 based BWP switching is used for UL BWP switching, not DL BWP, in our understanding. Given that, it seems to us that Option 1-1 and Option 1-3 simply loose the way to switch UL BWP when UL skipping is configured, which doesn’t seem to be a solution. Option 1-2 might work but limits UL BWP switching possibility. Not sure if it is really workable in real field. [Moderator]: I share the similar understanding that Option 1-1 and Option 1-3 seems UL skipping feature mutually exclusive with another feature, which is not preferred. For Option 1-2, when gNB deploy proactive scheduling, UE might not need to report SR or BSR. |
| Xiaomi |  | Given that all the options are implementation-based solutions, there is no need to do down-selection. All the options are applicable. [Moderator]: Agree. If the final decision is up to gNB implementation. All implementations are possible. During the discussion phase, I prefer to list options and help group understand how gNB operates. |
| QC |  | UE can send dummy PUSCH by implementation. Or NW can use DL grant to trigger BWP switching. [Moderator]: Yes, these as also solutions. |
| Samsung |  | It is gNB implementation issue. [Moderator]: Please refer my response to Xiaomi. |

Q4: If your answer for Q2 is category 2, which following option(s) do you prefer, and why?

- Option 2-1: Skipping UL transmission is temporarily disabled when active BWP is changed by UL grant (An example of spec change is listed in Appendix).

- Option 2-2: MAC layer generates a MAC PDU for the PUSCH transmission scheduling by UL grant to trigger a UL BWP switching.

- Others are not precluded

| Company | Which option(s)? | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2-1 | We prefer a PHY-layer-based solution, since BWP switching is a PHY layer issue. We are also open for other solution like modifying MAC PDU spec a little bit, but it will rely on RAN2 discussion. |
| NTT DOCOMO | Comments | While we sympathize with Huawei that PHY-based solution might be easier because it is related to BWP switching, we think there may be more or different specification impacts than Appendix. Our understanding is that MAC generates MAC PDU and then send it to PHY layer called as a TB. In this situation, without MAC specification impact, PHY cannot get the valid bits for the TB. Here, we understand that Option 2-1 still performs the transmission of the TB from PHY layer. In this case, what bits are to be transmitted? Can it be completely up to UE implementation? A potential problem in Appendix CR from Huawei is that even if PHY behaves such that UL skipping is not configured, there is anyway no MAC PDU (as MAC still sees UL skipping configured), then PUSCH content will be quite unclear. With that we think, if we go with PHY-based solution, some clarification is necessary for PUSCH content. [Moderator]: This is a good question. One possible way is let UE padding the bits in PHY, like control signaling padding. Another way is let MAC generate a padding MAC PDU. All solutions are open to be discussed. |
| Xiaomi |  | We think category 2 should be discussed in TEI, not maintenance. [Moderator]: Kind of disagree. In Rel-16, RAN1 also considered skipping PUSCH and PUCCH overlapping issue and specified related procedure in the spec. Thus, from my side, it is fine to discuss in CR phase to make spec more complete. |
| QC |  | Option 2-1 can be a UE implementation. |

Summary of 1st round discussion

For Q1: 2 companies support to conclude the issue exist, and other companies do not object.

For Q2: 4 companies support category 1 solution, i.e. without spec impact, and 2 companies support category 2 solution which requires spec change.

For Q3: 2 companies think that it is not necessary to mention implementation details; 2 companies list other solutions. 1 company doubts whether suggested options listed in proposal can work well or not.

For Q4: 1 company support Option 2-1 but 1 company has concern. 1 company suggest to discuss the issue in TEI agenda and 1 company think Option 2-1 can be also up to UE implementation.

Proposal for Tuesday online

Proposed conclusion:

Conclude that when UL BWP change is triggered (e.g., by DCI format 0_1) and skipUplinkTxDynamic or enhancedSkipUplinkTxDynamic-r16 is set as true, following ambiguity exists:

- If the UE has no data to transmit, the gNB cannot know if the UE has successfully received the triggering DCI and understood the BWP change.

Conclusions

To be updated.

- References

R1-2501321, Discussion on skipping uplink transmission in case of BWP switching, Huawei, HiSilicon

R1-2501322, Correction on skipping uplink transmission in case of BWP switching, Huawei, HiSilicon

- Appendix. TS 38.213 v17.11.0, Clause 12, Draft TP

- 12 Bandwidth part operation < Unchanged parts are omitted > A UE does not expect to detect a DCI format with a BWP indicator field that indicates an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change, respectively [10, TS 38.133]. If a UE detects a DCI format 0_1 indicating an active UL BWP change for a cell, and if a UE is configured with skipUplinkTxDynamic or enhancedskipUplinkTxDynamic -r16 with value true, the UE transmits a PUSCH indicated by that DCI with the assumption of skipUplinkTxDynamic value as false or enhancedskipUplinkTxDynamic -r16 as not being configured. If a UE detects a DCI format with a BWP indicator field that indicates an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format or determined by the slot offset value corresponding to the first PDSCH of the more than one PDSCH scheduled by the DCI format for the cell. < Unchanged parts are omitted >