3GPP TSG RAN WG1 #120		                                   R1-2501387

Athens, Greece, February 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.2

Agenda Item:		8.2

Document for: 	Endorsement

- 8.2	Rel-18 UE Features

The maximum number of contributions per company/organization/university is limited to 1 per agenda item unless stated otherwise.

[120-R18-UE_features] Email discussion on Rel-18 UE features – Hiroki (DOCOMO)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2501385	Summary of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

Agreement:

Introduce following FG for UE 8Tx PUSCH processing capability for codebook.

- FG name

  - UE 8Tx PUSCH processing capability for codebook

- Component

  - 1. Require additional timeline to process two TBs for 8Tx PUSCH

- Prerequisite

  - 40-7-1 or 40-7-2

- Consequence if not supported

  - UE should process two TBs within legacy timeline

- Type

  - Per FS

- Need of FR1/FR2 differentiation

  - n/a

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64, 128}

  - For 960kHz SCS: {32,64,128, 256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH

  - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8}

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG for UE 8Tx PUSCH processing capability for non-codebook.

- FG name

  - UE 8Tx PUSCH processing capability for non-codebook

- Component

  - 1. Require additional timeline to process two TBs for 8Tx PUSCH

- Prerequisite

  - 40-7-1 or 40-7-2

- Consequence if not supported

  - UE should process two TBs within legacy timeline

- Type

  - Per FS

- Need of FR1/FR2 differentiation

  - n/a

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64, 128}

  - For 960kHz SCS: {32,64,128, 256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH

  - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8}

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Reply to RAN2 that following per-band capabilities refer to the source band.

- ue-TA-Measurement-r18

- ltm-BeamIndicationJointTCI-r18

- ltm-BeamIndicationSeparateTCI-r18

- rach-EarlyTA-Measurement-r18

- ltm-MAC-CE-JointTCI-r18

- ltm-MAC-CE-SeparateTCI-r18

- ta-IndicationCellSwitch-r18

R1-2501386	Summary#2 of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

Agreement:

Introduce following FG for simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3.

- FG name

  - Simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3

- Component

  - Indicates whether the UE can be configured with both Rel-18 DL DMRS and DCI format 1_3

- Prerequisite

  - one of {49-1, 49-1b} and 40-4-1

- Consequence if not supported

  - Rel-18 DL DMRS and DCI format 1_3 should not be configured simultaneously regardless of on scheduling cell or on the scheduled cell(s).

- Type

  - Per band

- Note

  - None

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Reply to RAN2 with following RAN1 answers.

- Answer to Q1: No, the FGs are only applicable to FR2. RAN1 has not agreed to support cross-carrier scheduling of MDCI based STxMP PUSCH.

- Answer to Q2: the capability value does not need to be reported for each of FR2 SCS in a given CC. UE reports candidate value for the applicable SCS in unit of symbols.

- Consequently, the FGs are updated as below.

| 40-6-3a-1 | 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for the applicable each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |
| --- | --- | --- | --- | --- |
| 40-6-3b-2 | 1. Require additional timeline to process multiple TBs for non-codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for the applicable each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |

R1-2500833	UE features for Rel-18 work items	Samsung

R1-2501142	UE features for Rel-18	Qualcomm Incorporated

Withdrawn

R1-2501325	Rel-18 UE Features	Huawei, HiSilicon