3GPP TSG RAN WG1 #120		                                   						R1-2501394

Athens, Greece, February 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.5

Agenda Item:		9.15.5

Document for: 	Endorsement

- 9.15.5 UE features for LP-WUS/WUR for NR

For RAN1#120, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Proposal:

Support Alt-2 below:

- Alt-2: Introduce the following FG(s), where the supported LP-WUS/WUR waveform and operation are reported in a FG, with FFSs on whether/which to define as a basic FG for LP-WUS/WUR (and potential prerequisite relationship in between), Type and further coupling/decoupling of the FG(s)

| X-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | [Per Band] | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported | [Per Band] | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |

Alternative proposal:

Support Alt-1 below:

- Alt-1: Introduce separate FGs for LP-WUS/WUR waveform and operation as follows, with FFSs on whether/which to define as a basic FG for LP-WUS/WUR (and potential prerequisite relationship in between), Type and further coupling/decoupling of the FG(s)

| X-1 | OOK-based LP-WUS and LP-SS | 1. Support of LP-WUS based on OOK-1 or OOK-4 2. Support of LP-SS based on OOK-1 or OOK-4 FFS: Other components and/or further FG separation |  | YES | n/a | OOK-based LP-WUS is not supported | [Per Band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-2 | OFDM-sequence based LP-WUS | 1. Support of LP-WUS based on overlaid OFDM sequence(s) over OOK symbol FFS: Other components and/or further FG separation |  | YES | n/a | OFDM-sequence based LP-WUS is not supported | [Per Band] | n/a | n/a | n/a |  | Optional with capability signalling |
| X-3 | Further RRM relaxation of UE MR in IDLE/INACTIVE modes | 1. Support of further RRM relaxation of UE MR for both serving and neighbour cell measurements in IDLE/INACTIVE modes, including necessary conditions | X-1 or X-2 | NO | N/A | Further RRM relaxation of UE MR is not supported in IDLE/INACTIVE modes | N/A | N/A | N/A | N/A |  | Optional without capability signalling |
| X-4 | UE serving cell RRM measurement offloaded from MR to LP-WUR in IDLE/INACTIVE modes | 1. Support of UE serving cell RRM measurement offloaded from MR to LP-WUR in IDLE/INACTIVE modes, including necessary conditions | X-1 or X-2 | NO | N/A | UE serving cell RRM measurement offloaded from MR to LP-WUR is not supported in IDLE/INACTIVE modes | N/A | N/A | N/A | N/A |  | Optional without capability signalling |
| X-5 | Paging monitoring triggered by LP-WUS in IDLE/INACTIVE modes LP-WUS operation in IDLE/INACTIVE | 1. Support of LP-WUS monitoring and paging monitoring triggered by LP-WUS[, including at least configuration, sub-grouping and entry/exit condition for LP-WUS] 2. The supported wake-up delay for IDLE/INACTIVE modes 3. Support of further RRM relaxation of UE MR for both serving and neighbour cell measurements in IDLE/INACTIVE modes, including necessary conditions 4. Support of UE serving cell RRM measurement offloaded from MR to LP-WUR in IDLE/INACTIVE modes, including necessary conditions FFS: Other components and/or further FG separation |  | YES | N/A | Paging monitoring triggered by LP-WUS is not supported in IDLE/INACTIVE modes LP-WUS operation in IDLE/INACTIVE is not supported | [Per Band or Per UE] | N/A | N/A | N/A | Component 2 candidate value: [two values for ultra-deep sleep state and one value for deep sleep state] Note: Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. | Optional with capability signalling |
| X-6 | LP-WUS operation in CONNECTED mode | 1. The supported LP-WUS operation in CONNECTED mode [2. Time gap between LP-WUS reception and MR to start PDCCH monitoring in CONNECTED mode] FFS: Other components and/or further FG separation |  | YES | N/A | LP-WUS operation in CONNECTED mode is not supported | [Per-band or Per UE] | N/A | N/A | N/A | Component 1 candidate value: {Option 1-1, Option 1-2, both} | Optional with capability signaling |

Component candidates for IDLE/INACTIVE mode operation:

- Support LP-WUS in IDLE/INACTIVE mode and getting LP-WUS information from OOK-4 modulation with M=1, 2 and 4.

- Support of T/F tracking based on the OOK sequence of LP-SS with M=1,2 and 4.

- Support of serving cell RRM measurement based on the OOK symbols of LP-SS

- Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs.

- Support of exit operation based on configured channel condition threshold

- Minimum value of LO offset from reference PO/PF with candidate values [{40, 400, 800}] ms

- Support of up to 32 number of subgroups per PO

Component candidates for CONNECTED mode operation:

- Support LP-WUS in CONNECTED mode and getting LP-WUS information from OOK-4 modulation with M=1, 2 and 4, when C-DRX is configured

- Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS is 11 PRBs.

- Minimum Offset between LP-WUS occasion and the starting location of drx-OnDurationTimer

R1-2500076	UE features for Rel-19 LP-WUS	Huawei, HiSilicon

R1-2500113	Views on LP-WUS/WUR UE features	Nokia

R1-2500136	Discussion on WUR features	ZTE Corporation, Sanechips

R1-2500215	LP-WUS/WUR UE feature	CATT

R1-2500311	Discussion on UE features for low-power wake-up signal and receiver for NR	CMCC

R1-2500375	Rapporteur input on UE features for Rel-19 LP-WUS/WUR	vivo, DOCOMO

R1-2500724	UE features for NR LP-WUS/WUR	Xiaomi

R1-2500876	UE features for LP-WUS/LP-WUR	Samsung

R1-2501039	UE features for LP-WUS/WUR for NR	MediaTek Inc.

R1-2501050	UE Features for LP-WUS	Vodafone

R1-2501184	UE features for LP-WUS/WUR for NR	Qualcomm Incorporated

R1-2501258	Initial view on UE features for Rel-19 LP-WUS/WUR WI	Ericsson