3GPP TSG-RAN WG1 Meeting #120bis	R1-2501722

Wuhan, China, April 7th – April 11th, 2025

Agenda Item:	9.4.4

Source:	Ericsson

Title:	Other aspects for Ambient IoT

Document for:	Discussion, Decision

# 1	Introduction

The Rel-19 work item “Solutions for Ambient IoT (Internet of Things) in NR” [1] concerns standardization of RAN aspects for Ambient IoT (A-IoT), a new 3GPP IoT technology suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. Definitions and decisions from preceding study items are captured in [2] and [3]. The final feature lead summary for this agenda item in the previous RAN1 meeting can be found in [4].

In this contribution we present our views on ‘other aspects’ for A-IoT including multiplexing/multiple access, scheduling information, timing relationships, L1 control information, and other L1 procedural aspects.

# 2	Multiplexing/multiple access

## 2.1	Scheduling FDMA resources

The following agreement regarding small frequency shift (SFS) was made in RAN1#120.

Agreement For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation. - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

This agreement covers the following options discussed in SI.

- Option 1: Each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length).

- Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency shift, the time duration Tb corresponding to each information bit includes R number of square wave periods, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). The multiplication operation is performed as either an XOR or XNOR operation between a Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

Note that in the above assumption Tb becomes the coded bit duration when FEC and/or bit-repetition type 1 is applied.

The above options indicate the following relationship between frequency scheduling and repetitions.

SFSHz=RTb=12 × D2R chip length=R×Data rate (bps)		Equation (1)

Data rate bps=1Tb=12 × D2R chip length × R			Equation (2)

R being the line code repetition factor or number of square wave periods (depending on whether option 1 or 2 is chosen).

Equations (1) implies the following observation.

- Choosing different D2R chip lengths and/or R allow A-IoT devices to shift their transmission frequencies.

In relation with the above, RAN1#120 has made the following agreement.

Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values

However, as can be observed from Equation (2), choosing a certain D2R chip length also impacts the coded bit duration Tb, and thereby the data rate of the devices. Since the transmission bandwidth of backscattered transmission is calculated as BW=4/Tb  (2/Tb  per mirror image), the transmission bandwidth of the A-IoT devices also get impacted based on the repetition chosen by the A-IoT device.

- Transmission bandwidth and the data rate of the A-IoT devices get impacted based on the D2R chip length and repetition factor R, where R is the chip-level repetition in case of line coding and number of square wave periods when SFS is implemented without line coding.

The above relationship implies that while assigning frequency resources, the reader can follow two approaches.

Approach 1: Keeping the data rate among the devices constant- i.e., 1/Tb being constant among the devices which are FDMA’ed simultaneously. This implies that the bandwidth of the scheduled frequency resources BW=4/Tb , can be equal between the FDMA’ed resources.

Approach 2: Support for different data rates between the devices. This implies that, for any two devices, the coded bit durations Tb1¹ Tb2, which implies that their bandwidths need to be different. This approach is more flexible in terms of configuration, but the scheduling requires frequency resources to support variable BWs.

- While scheduling FDMA resources, one of the following approaches can be supported:

Approach 1: Keeping the data rate the same between simultaneously FDMA’ed devices thereby resulting in scheduling frequency resources with the same transmission bandwidth

Approach 2: Support variable data rates as well where FDMA scheduling requires supporting resources with variable BWs

Out of the two approaches, Option 1 of TR 38.769 of implementing SFS by implementing chip level repetition within the Manchester line code seems to support Approach 1 from Observation 3.

Below we note different D2R message types along with where the variable BW support might be beneficial.

- Different D2R messages along with whether variable BWs support might be beneficial for devices participating within the same access round is noted below.

| D2R Message | Same/variable TBS for devices | Can variable BWs be beneficial? |
| --- | --- | --- |
| Msg1 | Same TBS | Not beneficial. Support fixed BW |
| Msg3 | Variable sizes possible based on SA2 agreements – manufacturer ID or device ID can be sent here | Might be beneficial if a longer ID is to be sent by the device |
| Msg5, .. | Since these are command type messages variable sizes might be required here | Might be beneficial based on the contents to be sent here. |

Based on the above observation, we make the following proposals.

- For Msg1, it is assumed that all devices transmit with same TBS. Therefore, Msg1 resources need to support resources with the same bandwidth/Tb.

- For Msg3 and further command messages, TBS of different sizes might be supported. Therefore, RAN1 to discuss whether or not to support variable BWs (different Tb) in these cases.

## 2.2	Resource allocation for Msg1

RAN1 #120 made the following agreements relevant to the below discussion.

Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4.

### FDMA for Msg1

RAN2#129 has made the following agreement on FDMA-

RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

For the devices to make an unbiased choice from among the resources provided, Approach 1 from Observation 3 might be better.

- FDMA resources scheduled in paging message for a set of devices should include resources supporting the same bandwidth and same data rate.

Scheduling FDMA resources with the same bandwidth however needs to ensure that Equation (2) above is met for all FDMA’ed devices. That is, for any two devices with same bandwidth BW1=BW2 implies Tb1= Tb2, which from Equation (2) essentially means that the product of their D2R chip duration and repetition is to be kept constant, i.e., the product D2R chip length x R needs to be kept constant for all the devices.

- For scheduled FDMA resources to support the same bandwidth, the devices implementing SFS need to keep the product of D2R chip length x R to be constant

The above observation implies that FDMA scheduling from the reader involves providing the devices with combinations of D2R chip length and R such that their product remains constant for a certain transmission bandwidth and data rate. In other words, for two devices that are to be scheduled at different frequencies they should meet the following criterion.

D2R chip length1×R1=D2R chip length2×R2=…= D2R chip lengthY×RY  	Equation (3)

Therefore, we make the following proposal.

- Scheduling FDMA for Msg1 involves the reader providing implicitly or explicitly a set of tuples containing (D2R chip duration, R) such that their product  D2R chip length x R remains constant for all the scheduled resources.

### TDMA for Msg1

Support for X>1 in terms of feasibility

Figure 1 depicts an example for random access procedure with X>1 versus X=1. The feasibility of supporting the former involves some design challenges as listed below.

- Devices have to maintain a counter to track its corresponding time occasion by relying on its clock. A workaround here would be for the reader to have additional R2D trigger signals (similar to QueryRep) within an access occasion, which adds to the signalling overhead and hence not desirable.

- With SFO of 10%, a guard time needs to be accounted between the TDMA occasions for each message to prevent any timing overlaps, which prolongs access occasion duration. For instance, in the inventory procedure depicted in Figure 1, an additional guard time of 
2*(X-1)*(TMsg1+TMsg2+TMsg3)*SFO has to be considered in the design so as to prevent timing overlaps.

Figure 1: Random access procedure with X>1 (top) versus X=1 (bottom)

- Supporting X>1 needs to consider two additional design factors.

- Devices having to keep track of a counter within the access occasion

- Guard bands required between messages to prevent timing overlaps

Support for X>1 in terms of performance

Latency related benefits for different X values is tested with parameters depicted from Table 1.

Table 1: Parameters for testing inventory completion time

| Parameter | Value |
| --- | --- |
| Paging message | 22 bits |
| Msg1 | 16 bits |
| Msg2 | 16 bits |
| Msg3 | 96 bits |
| Msg0 | 4 bits |
| TD2R | 87.5 µs |
| TR2D | 75 µs |
| D2R preamble | 66 chips |
| R2D start indicator+clock acquistion | 6 chips |
| SFO | 10^5 ppm |
| Data rates (R2D, D2R) | 1 kbps |

Substituting these parameters for different message sizes as depicted in Table 1, with OOK-1 for both R2D and D2R transmissions with Manchester encoding and no FEC in D2R, the overall inventory times for the cases where there are X TDMA resources within a single access occasion (the case with A-IoT denoted as T1) versus if X TDMA occasions are spread across different access occasions (as with RFID denoted as T2) is compared in Figure 2 below.

Figure 2: Overall inventory times for a case with X TDMA resources within one access occasion vs X access occasions.

We make the following observation.

- Overall benefits from X>1 seems limited considering the additional design factors that need to be considered. The inventory time seems to be worser with higher X values when the SFO is 10^5 ppm.

Therefore, we propose the following.

- Resource allocation for Msg1 needs to consider only X=1.

## 2.3	Msg2 related aspects

RAN1 #120 made the following agreements relevant to the below discussion.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

### Number of Msg1 responses within a PRDCH for Msg2

Number of Msg1 responses to be carried within a PRDCH depends on the deployment factors like coverage, required BLER target, as well on the maximum TBS supported for R2D. Since this is deployment specific, we propose the following.

- Number of Msg1 responses within a PRDCH for Msg2 should be reader-configurable but within the maximum supported TBS for R2D.

- From RAN1 perspective, there is no need to specify maximum number of Msg1 responses that can be carried within a PRDCH. It is enough to specify the maximum supported TBS for R2D.

### Start time monitoring for Msg2

Regarding the start time for monitoring Msg2, the FL summary discussions seem to indicate defining a TMsg2_start in the standard after which the devices are able to receive their Msg2 transmission. However, since there might be multiple PRDCH transmissions for Msg2 based on the current agreements, a more useful approach might be to define a start time to monitor an L1 R2D control information containing where and when to start Msg2 monitoring. L1 R2D control can be defined from the perspective of the end of the last of X time domain resource(s) determined for Msg1 transmission(s). Reader can then configure the start time for monitoring Msg2 transmissions corresponding to each Msg1 in a much more flexible way.

- Support separate start time for Msg2 monitoring, at least for the case where there are multiple-Msg2 PRDCH transmissions corresponding to multiple Msg1 responses.

- To enable separate start times for each Msg2 PRDCH transmission, support R2D L1 control information before Msg2 PRDCH transmissions. The L1 control information can contain the individual start times (or offsets) for Msg2 transmissions corresponding to each Msg1.

### Feasibility of defining a Msg2 monitoring window

While defining a minimum time after which Msg2 can be expected might be feasible, defining a maximum time in the standard does not seem practical since the time duration for monitoring Msg2 response corresponding to multiple Msg1 transmissions depends on the following.

- How many Msg1 time-frequency occasions are provided

- Variable sized Msg2s- Msg2 size can be variable based on the modulation format, bit-level repetitions, support for MAC CEs, etc. Msg2 size can also be different based on how many Msg2s are fitted into one PRDCH transmission by the reader.

Considering these aspects, predefining a monitoring time duration for Msg2 does not seem practical. If the time duration to monitor is provided as part of R2D triggering random access, e.g., paging message, then the Msg2 TB size will have to be fixed, irrespective of number of Msg1 responses. If needed, the option that provides best configurability in such a case is to indicate this as part of L1 control information in R2D preceding Msg2 transmissions.

Therefore, we propose the following.

- The time duration for monitoring Msg2 in response to multiple Msg1 transmissions can be indicated as part of L1 control information preceding the Msg2 transmissions.

- A time monitoring window for monitoring L1 control information preceding Msg2 transmissions may be specified instead of a Msg2 monitoring window

## 2.4	Resource allocation for Msg3

### Scheduling of Msg3

RAN1#120 made the following agreements related to scheduling the frequency resource for Msg3.

Agreement

Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

Agreement

Support the following option(s) for frequency domain resource for Msg3 transmission:

- Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices.

- FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule

  - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

Based on this Option 2 seems to suggest a RAR like scheduling with Msg2 containing the scheduling information for Msg3. As we state in proposal 3, for the scheduling of FDMA with a reader configured bandwidth, data rate and a specific frequency shift, the reader needs to implicitly or explicitly indicate to the device the required D2R chip level duration as well as the chip-level repetition rate R.

- FDMA resource scheduling for Msg3 within the PRDCH for Msg2 transmission contains at least the chip-level duration and repetition combination for one or multiple devices.

Supporting option1 there are following challenges.

- Msg3 using the same frequency resource as Msg1 (Option 1) might incur less scheduling overhead but is extremely resource inefficient. Msg1 frequency resources which encountered collision with no successful transmissions will then be wasted.

- FDMA scheduling as mentioned in proposal 3 involves configuring a D2R chip duration and repetition to a device. The bandwidth of the FDMA resource is directly related to this configuration (as explained in Equations 1-3). Since Msg1 and Msg3 transmissions are different in terms of size (Msg1 is assumed to be ~16 bits, and Msg3 is assumed to be ~96bits), therefore it doesn’t make sense to have the same bandwidth/data rate constraints for Msg3 as was provided for Msg1.

- For supporting frequency domain resource for Msg3 transmission, do not support Option 1 where the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

### Interleaving Msg3 transmissions

The FLS [4] from RAN1#120 discusses two ways in which Msg3 can be transmitted.

- Way 1: Non-interleaving Msg3 transmissions (e.g., Msg2-Msg2-Msg3-Msg3), where Msg3 transmissions are FDMed and follows all PRDCHs for Msg2 transmissions.

- Way 2: interleaving Msg3 transmissions (e.g., Msg2-Msg3-Msg2-Msg3), where the PRDCH for Msg2 transmission and the corresponding Msg3 are sent one by one.

In our view, interleaving Msg3 transmissions comes with the following challenges.

- Way 1 provides better D2R spectral efficiency compared to Way 2 (since many D2R transmissions can be FDMA’ed simultaneously).

- Each PRDCH for Msg2 followed by corresponding Msg3 responses imply some of the devices having to keep monitoring for its own Msg2 transmissions potentially for a very long duration. Devices might not have enough stored energy to support this behaviour.

- The above issue becomes worse when there are retransmissions

- The above issue is not ideal specifically if each Msg2 transmission is a separate PRDCH transmission.

- The above issue is even more problematic in cases where there are continued command-based transmissions as well- Msg4 and so on.

Altogether supporting Way2 brings in more challenges. Therefore, non-interleaving as mentioned in Way1 seems like a better fit.

- Support only Way1 of non-interleaving Msg3 transmissions (e.g., Msg2-Msg2-Msg3-Msg3).

# 3	Timing relations

FL summary from RAN1#120 discusses the following about the timing relationships.

For the following timing relations, companies are encouraged to provide the justifications on the necessity, function/purpose. (i.e., whether 3GPP need to define these parameter(s)) Timings; Description in TR 38.769; Necessity/Function/Purpose (Example) Whether 3GPP need to define these parameter(s)?. TR2D_min; Minimum Time between a R2D transmission and the corresponding D2R transmission following it; Requirement on device E.g., Time required for device processing from the end of R2D Rx to the earliest start of the corresponding D2R Tx from device perspective. TR2D_max; Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].; Requirement on device Time required for device processing from the end of R2D Rx to the latest start of the corresponding D2R Tx from device perspective. TD2R_min; Minimum Time between a D2R transmission and the corresponding R2D transmission following it.; E.g., Requirement on device Time required for device processing from the end of D2R Tx to the earliest start of the corresponding R2D Rx from device perspective. TD2R_max; When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]..

Below, we study each time value, discuss their intended functions, purpose and necessity along with the challenges that need to be taken into consideration while specifying the timers.

## 3.1	Time between an R2D transmission and corresponding D2R response

| TR2D_min : Minimum Time between a R2D transmission and the corresponding D2R transmission following it |  |
| --- | --- |
| Intended Function | Requirement on device This can be the minimum time required for the device to process R2D message and generate its corresponding D2R response (similar to PUSCH preparation time in NR) |
| Intended Purpose (example) |  |
| Necessity | Defining in the standards provides a minimal processing capability that the A-IoT device needs to support. Reader can adjust the scheduling time to ensure that the D2R scheduling maintains at least a gap of TR2D_min between the end of R2D transmission and the start of the corresponding D2R transmission. Such time can be used also for the CWT node prepare the CW transmission in D1T1-B. D2R transmission cannot occur before CW is present at device antenna. |
| Additional considerations? | From device’s perspective This seems to work even if a common reference time is not considered as shown in the example below. The device may need to switch between DL reception to UL transmission, in a case the R2D signal and CW used for energy harvesting, this time may diminish the capacity to hold up the power. From reader’s perspective CW cancellation at the reader- If the CW power is high and the reader needs to make some training to cancel CW from the backscattered transmission, then some additional time might be required. In TR 38.769 , RAN4 conclude the D2R reception should be avoided during the CW cancellation training time. |

- Having a TR2D_min time allows defining a minimum time that the device might take for processing an R2D reception and generating its own D2R response to it. Defining a TR2D_min value within the standard allows setting a minimal processing capability that the A-IoT devices need to support.

| TR2D_max :Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. |  |
| --- | --- |
| Intended Function | Requirement on the device- Time required for device processing from the end of R2D Rx to the latest start of the corresponding D2R Tx from device perspective, potentially considering SFO. |
| Intended purpose (example) |  |
| Necessity | May not be necessary since the reader, while scheduling, can always assume guard spaces in time to compensate for SFO. Also defining this time comes with its challenges as detailed below |
| Challenges | Defining a fixed TR2D_max in the standard reduces the flexibility of configuration of resources within paging. For example, in the above figure, if X TDMA occasions are scheduled in paging, the current definition of TR2D_max requires the Xth time occasion to also be scheduled within [TR2D_min, TR2D_max], which reduces the configurability. A similar challenge can also be seen for Msg2-Msg3 transmissions. For instance, if we assume non-interleaving of Msg2 and Msg3 (proposal 13), a device may be scheduled to transmit its Msg3 after all PRDCHs containing Msg2 transmissions are done. Since number of PRDCH transmissions containing Msg2 cannot be preconfigured, pre-defining TR2D_max is not possible. |

- Defining TR2D_max in the standard is not necessary since the reader, while scheduling, can always assume guard spaces in time to compensate for SFO. From RAN1 perspective, defining this value in the standard might reduce the resource configuration flexibility for the reader.

## 3.2	Time between a D2R transmission and corresponding R2D transmission following it

| TD2R_min : Minimum Time between a D2R transmission and the corresponding R2D transmission following it |  |
| --- | --- |
| Intended Function | Requirement on device This can be the switching time required for device processing from the end of D2R Tx to the earliest start of the corresponding R2D Rx |
| Intended Purpose (example) |  |
| Necessity | A large enough switching time does not seem to be very necessary since A-IoT devices work with an envelope detector architecture. Therefore, no switching between frequency bands is required to go from transmit to reception mode. The necessity otherwise seems to be for the device to know after how long it needs to switch to receive monitoring mode. Therefore, defining this time can be optionally considered. |
| Additional considerations? | - Devices automatically switching to receive mode after TD2R_min might also not be ideal since this means the device might keep receiving and monitoring other D2R transmissions. Therefore, the actual time for monitoring R2D transmission after a D2R transmission should be reader configurable. - May work even without a common reference time. - To clarify whether corresponding R2D transmission can also be a L1 R2D control information scheduling Msg2 |

- Defining TD2R_min in the standard might allow defining a switching time for the device to go from transmit to receive mode. While this can be optionally considered based on the requirement, the following additional considerations will have to be discussed.

- Though a minimum time after which the device can switch to receive mode is defined, the actual time for monitoring receptions after a D2R transmission should be reader configurable.

- Clarify whether the ‘corresponding R2D transmission’ in the definition of TD2R_min can include L1 R2D control information as well.

| TD2R_max: When a R2D transmission in response to a D2R transmission is expected, for eg: A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max ] |  |
| --- | --- |
| Intended Function | Requirement on device Having a maximum time might help devices monitor response within a certain time duration thereby handling error in transmissions using a timeout |
| Intended Purpose (example) |  |
| Necessity | Defining a maximum time until which a device should wait for its response might be required to handle timeouts. However, it is infeasible to have this time pre-defined in the standard if there are multiple PRDCH Msg2 transmissions since Msg1. |
| Additional considerations? | Defining a fixed TD2R_max in the standard is not practical since number of PRDCH containing Msg2s and the duration of each Msg2 depends on successful Msg1 transmissions, reader’s configuration and coverage consideration. For timeout purposes it is better to configure a time as part of a L1 control information that precedes the PRDCH containing data. |

- Defining TD2R_max might enable the devices to monitor responses within a certain duration, thereby handling error cases using timeouts. However, pre-defining this value in the standard is infeasible since number of PRDCH transmissions as part of Msg2 is completely dependent on the success/failure within Msg1 occasions, reader’s configurability and other considerations like coverage. Handling timeouts can instead rely on dynamic configuration of TD2R_max using L1 control signalling.

Based on the above observation we propose the following.

- The timing relations should be supported with the following considerations.

| Timings | Description in TR 38.769 | Defined in 3GPP standards/configured dynamically | Additional considerations if any? |
| --- | --- | --- | --- |
| TR2D_min | Minimum Time between a R2D transmission and the corresponding D2R transmission following it | Defined in 3GPP standards. Defining TR2D_min allows setting a minimal processing capability that the A-IoT devices need to support | From device’s perspective The device may need to switch between DL reception to UL transmission, in a case the R2D signal and CW is used for energy harvesting. From reader’s perspective CW cancellation at the reader- If the CW power is high and the reader needs to make some training to cancel CW from the backscattered transmission, then some additional time might be required. In TR 38.769 , RAN4 conclude the D2R reception should be avoided during the CW cancellation training time. |
| TR2D_max | Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. | Not necessary. If present configured dynamically. | Defining a fixed TR2D_max in the standard reduces the flexibility of configuration of resources within paging. |
| TD2R_min | Minimum Time between a D2R transmission and the corresponding R2D transmission following it. | Might not be necessary. If defined in the standards, this time should not mean that the devices automatically switch to reception state after TD2R_min expires | Devices automatically switching to receive mode after TD2R_min is not ideal since this means the device might keep receiving and monitoring other D2R transmissions. Therefore, the actual time for monitoring R2D transmission after a D2R transmission should be reader configurable. To clarify whether corresponding Msg2 can also be a L1 R2D control information scheduling Msg2 |
| TD2R_max | When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. | Has to be reader-configurable to handle issues like timeouts. Cannot be predefined in the standards. | Dynamic configuration of TD2R_max using L1 control signalling is preferred. |

# 4	Scheduling information

## 4.1	For R2D reception

### Field indicating PRDCH function (e.g., Message Type/Command Code)

Supporting a field indicating PRDCH function is intended for the following purposes.

    - To identify the functionality- paging, Msg2,ACK/NACK response for Msg3 or command type instructions

    - TBS determination if message type implicitly indicates message/command with predefined length or size

Considering that paging triggers the inventory procedure, and the length of paging message need not be fixed, expecting that the message type comes with a predefined length or size may not be applicable for paging.

Regarding Msg2 transmissions, the current RAN1 agreements support one or multiple Msg2 transmissions, the variable TB argument is applicable here as well.

In addition, the following is already agreed in RAN2#129

- Agreement

- At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s).

Besides, since inventory or command is a TDMA’ed procedure, meaning the order of transmissions might be in sequence rather than completely random. Therefore, the device might already be aware of which stage in the procedure it is in; and therefore, might expect which R2D message it is receiving.

Therefore, we make the following proposal

- Message type/command code is already agreed to be part of R2D message within the MAC header. This can be part of R2D control information, but from higher layer perspective than L1 perspective.

### Field for Device ID (for both R2D and D2R)

TR 38.769 states the following:

For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH:

-	ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported)

Based on this, the following observation is made in the RAN1 FLS [4].

From RAN1 perspective, the R2D control information carrying the message type and/or device ID that is/are placed at the front part of the PRDCH is beneficial for a device to early identify the function of the PRDCH, e.g., it is A-IoT paging, A-IoT Msg2 for inventory or specific command (e.g., read/write/disable) and determine whether it is the target device

It is important to note that the early identification function of PRDCH is possible for the device only if the R2D control information carrying the message type and/or device ID is self-decodable, i.e, comes with a CRC on its own.

- Early identification/termination function of PRDCH is possible for the device only if the R2D control information carrying the message type and/or device ID is self-decodable and is transmitted as a separate PRDCH

Since paging is a broadcast message and Msg2 MAC contents already includes message type and device ID, the early detection argument is not valid here.

Instead, there can be a separate PRDCH containing L1 control information that can enable early detection with reduced blind decoding.

### End identification of PRDCH

For end identification of PRDCH the following options were proposed as part of FL summary.

For a PRDCH transmission with variable size, down-select from following two options

- Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH.

- Option 2: Based on R2D control information.

We discuss the two options below.

Option 1: R2D postamble :

Even if the size of postamble is only a few chips, the idea of postamble is for it to act as an end delimiter for the PRDCH transmissions. However, there can still be multiple PRDCH transmissions, each containing one or more Msg2s. Having postambles without any L1 control signalling indication then implies a device will have to perform blind decoding across the entire sequence of PRDCH until its own Msg2 is found. This is the case irrespective of whether the PRDCH has fixed or variable size.

- Having postamble is not particularly beneficial in reducing blind decodes of R2D messages

Option 2: Based on R2D control information

The alternate on using a control information that precedes the R2D transmission offers more flexibility for decoding certain PRDCH transmissions as part of Msg2 or even parts of PRDCH containing multiple Msg2s (assuming each are self-decodable). This alternate can reduce the number of blind decodes. In addition, a control information might also be required to specify a receive monitoring window as we see from Observation 11.

- R2D control information is beneficial not only to handle the reception monitoring window as stated in Observation 11, but is also useful to reduce the number of blind decodes.

- End of identification for a PRDCH can be handled using Option (2) based on R2D control information.

Having said that the only use-case where postamble maybe applicable even when an R2D control information is present is as an end delimiter when SFO is high. For instance, assume R2D control information contains a monitoring window that specifies where to monitor for the R2D message. Within the window, R2D message is detected based on the preamble. Having an end delimiter like postamble can then mark the end of the message.

- In addition to R2D control information, discuss whether postamble is beneficial to indicate the end of transmission when SFO is high

### PRDCH chip length

The following was discussed as part of FLS [4]

The clock-acquisition part of the R2D time acquisition signal preceding the PRDCH is used to determine the OOK chip duration used for the following PRDCH transmission.

One of the primary considerations against this proposal is to see whether having different M values for supporting data and control can be supported. If R2D control and R2D data are transmitted as separate PRDCH transmissions, then supporting different M values is indeed feasible with the above agreement. The only case where it doesn’t agree is the case where different M values need to be supported within the same PRDCH. This design seems overly complicated. Therefore, the proposal above seems agreeable.

- The clock-acquisition part of the R2D time acquisition signal preceding the PRDCH is used to determine the OOK chip duration used for the following PRDCH transmission

## 4.2	For D2R transmission

RAN1#120 made the following agreements regarding D2R transmission

Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

### Indicating R value

In addition to the above agreements the following discussion was part of FLS [4].

Consider at least following options for indicating the parameter R related to Small Frequency Shift(s) for D2R transmission(s) with and without FDMA:

- Option 1: Bitmap based, where each bit corresponding to one R value

- Option 2: Codepoint based, where each codepoint corresponding to one R value

- Option 3: Codepoint based, where each codepoint corresponding to one or more R value(s)

As we discuss in proposal 4, providing different SFS for D2R transmissions at constant information bit duration require providing a combination of <D2R chip duration, parameter R>. Option 3 above involves a codepoint based behaviour with the possibility of one or more R values which might be matched with the same D2R chip duration, thereby creating variable data rates.

- Supporting option 3 where a codepoint based mapping corresponds to one or more R values for FDMA leads to a possibility of creating FDMA resources which have different transmission capabilities as explained in observation 5.

- Choose from among option1 (bit map based corresponding to one R value) or option2 (codepoint based corresponding to one R value) at least for Msg1 FDMA scheduling

- Alternately, a codepoint specifying <D2R chip duration, parameter R> can be specified to support SFS.

### Indicating D2R chip duration or D2R information bit duration

Regarding the indication of chip duration and information bit duration, the following options were considered in the FLS [4]

Consider the following options for indicating [D2R chip duration] or [D2R information bit duration]: - Option 1: the [D2R chip duration]/[D2R information bit duration] is indicated in R2D control information from predefined a set of [D2R chip duration]/[D2R information bit duration] values - Option 2: the scaling factor between the [D2R chip duration]/ [D2R information bit duration] and R2D chip duration is indicated in R2D control information

For frequency scheduling especially in Msg1, the reader needs to consider a set of feasible frequency occasions taking into consideration the required guard bands in between so as to prevent transmission overlaps from among FDMA’ed resources. This means predefining a set of values in the standard for the reader to choose from might be the most viable option.

- Support option 1 where the [D2R chip duration]/[D2R information bit duration] is indicated in R2D control information from predefined a set of [D2R chip duration]/[D2R information bit duration] values

- [D2R chip duration]/[D2R information bit duration] can be indicated as an integer multiple/scaled versions of a reference unit time where this unit time can be defined as for example 1/15kHz, where 15kHz is the D2R transmission BW

## 4.3	D2R control information

Regarding the D2R control information the following was concluded in the FLS [4].

RAN1 assumes whether and how the confirmation from a device to reader to indicate the successful or unsuccessful reception of an A-IoT R2D data transmission should be discussed and decided by RAN2. RAN1 will not discuss ACK/NACK as L1 D2R control information in Rel-19 A-IoT unless requested by RAN2.

We agree with this conclusion.

- RAN1 will not discuss ACK/NACK as L1 D2R control information in Rel-19 A-IoT unless requested by RAN2

## 4.4	R2D control information

### Joint vs Separate CRC

According to the FLS [4], R2D control information includes parameters that enable scheduling of PRDCH reception and PDRCH transmission. The benefit of supporting separate CRC for R2D control includes self-decoding capability but comes at the cost of increased overhead of adding the overall number of bits that are transmitted.

However, as we note from Observation 12, benefit of early detection and termination requires R2D control to be transmitted as a separate PRDCH transmission altogether. For R2D control that might be included as part of higher layer (for example message type) the device still needs to receive the entire PRDCH; meaning that the argument on early detection and termination is no longer valid here.

Therefore, we have the following proposal.

- Joint CRC is supported for higher layer control as well as data in a PRDCH transmission. Separate CRC is applicable only for L1 control information which is sent as a separate PRDCH transmission.

### L1-control vs. Higher layer signalling

As noted from the FLS [4] discussions, the ID/identification or R2D reception scheduling information should be in L1 to allow early determine whether to continue the following R2D reception, avoid blind decoding for R2D reception, enable more efficient process pipelining and reduce PRDCH reception latency.

However, some parameters like message type, length for upper layer SDU etc., are already agreed to be part of MAC header- probably as part of higher layer control field according to RAN2#129 discussions. This need not be repeated as part of L1-control information again as we state in Proposal 13.

- R2D control includes both higher layer control as well as L1 control parameters.

Having said that it is indeed beneficial to have L1 control information indicating Msg1 transmission occasion and probably parameters like monitoring window which enable the devices to reduce the blind decodes; particularly for R2D receptions like Msg2s. To optimize the L1 control information further there can be optimizations performed on the contents of L1 control information to not explicitly include the entire device ID but parameters like Msg1 occasions instead.

As an example, assume X time occasions and F frequency occasions that the devices can use for transmitting Msg1s. An L1 control information as shown in Figure 3 containing log2(XF) bits can be used to indicate to the device whether its Msg2 is to be expected.

If X=1 and F=4, XF occasions can be indicated by 2 bits.

This means a device that doesn’t find its frequency-time information in the control information need not decode and process Msg2s further.

Figure 3: Receiving a common control block enable the device to find if its Msg2 is present or not, thereby reducing the blind decodes.

Additionally, L1-control information can also contain details about the monitoring window- from where until where the device should keep on monitoring for its Msg2 reception as we conclude from proposal 12.

- A common L1 control signalling should be supported for scheduling R2D reception, at least for Msg2.

- L1 common control information should be send as a separate PRDCH transmission that precedes Msg2.

- The contents of L1 control signal should contain at least an indicator (for eg:  index to Msg1 transmission occasion) for the devices to see if its R2D response is present or not, and if present a monitoring window specifying where to monitor the R2D response.

### Contents of R2D control information

Relevant to the contents of R2D control, the following was discussed in FLS [4]

At least for device-specific PDRCH scheduling, except for ID associated with the device, at least the following R2D control information if needed is carried by higher layer signalling.

- Time domain resource/TBS

- Frequency domain resource

- MCS-like information

- Chip duration

- Repetitions

- Midamble

The above discussion refers to device-specific PDRCH scheduling like Msg3 scheduling. Since Msg3 is scheduled only for successful Msg2 receptions, therefore a RAR-like scheduling is the best case possibility here. This also implies that the scheduling details for Msg3 comes as part of Msg3 payload- which automatically implies that unless there is a strong reasoning otherwise, the parameters for scheduling Msg3 like TDMA-FDMA resources, MCS-like information, chip duration, repetitions and midamble can be part of HL R2D control information.

- The following information may be carried as part of R2D control signalling for device-specific PDRCH scheduling like Msg3.

    - Time domain resources

    - MCS-like information- this includes FEC code rate

    - <Chip duration, Chip level Repetition R> for SFS

    - Bit-level Repetition

    - Midamble- Number of midambles, Location of midamble (s) (if present)

- If Msg3 scheduling follows a RAR-like scheduling, above R2D control signalling parameters are carried in higher layer payload.

- FFS to discuss the behaviour of device-specific scheduling for messages after Msg3.

# 5	Other aspects

## 5.1	Device (un)availability via Direction 1

Our understanding of Directions 1 is that the device manages without any information like paging occasions from the reader. Hence, we believe this is fully autonomous in the device and is solely implementation dependent.

- Device (un)availability via Direction 1 does not require any specification update.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	Choosing different D2R chip lengths and/or R allow A-IoT devices to shift their transmission frequencies.

Observation 2	Transmission bandwidth and the data rate of the A-IoT devices get impacted based on the D2R chip length and repetition factor R, where R is the chip-level repetition in case of line coding and number of square wave periods when SFS is implemented without line coding.

Observation 3	While scheduling FDMA resources, one of the following approaches can be supported:

Approach 1: Keeping the data rate the same between simultaneously FDMA’ed devices thereby resulting in scheduling frequency resources with the same transmission bandwidth

Approach 2: Support variable data rates as well where FDMA scheduling requires supporting resources with variable BWs

Observation 4	Different D2R messages along with whether variable BWs support might be beneficial for devices participating within the same access round is noted below.

| D2R Message | Same/variable TBS for devices | Can variable BWs be beneficial? |
| --- | --- | --- |
| Msg1 | Same TBS | Not beneficial. Support fixed BW |
| Msg3 | Variable sizes possible based on SA2 agreements – manufacturer ID or device ID can be sent here | Might be beneficial if a longer ID is to be sent by the device |
| Msg5, .. | Since these are command type messages variable sizes might be required here | Might be beneficial based on the contents to be sent here. |

Observation 5	For scheduled FDMA resources to support the same bandwidth, the devices implementing SFS need to keep the product of D2R chip length x R to be constant

Observation 6	Supporting X>1 needs to consider two additional design factors.

1.	Devices having to keep track of a counter within the access occasion

2.	Guard bands required between messages to prevent timing overlaps

Observation 7	Overall benefits from X>1 seems limited considering the additional design factors that need to be considered. The inventory time seems to be worser with higher X values when the SFO is 10^5 ppm.

Observation 8	Having a TR2D_min time allows defining a minimum time that the device might take for processing an R2D reception and generating its own D2R response to it. Defining a TR2D_min value within the standard allows setting a minimal processing capability that the A-IoT devices need to support.

Observation 9	Defining TR2D_max in the standard is not necessary since the reader, while scheduling, can always assume guard spaces in time to compensate for SFO. From RAN1 perspective, defining this value in the standard might reduce the resource configuration flexibility for the reader.

Observation 10	Defining TD2R_min in the standard might allow defining a switching time for the device to go from transmit to receive mode. While this can be optionally considered based on the requirement, the following additional considerations will have to be discussed.

(1)	Though a minimum time after which the device can switch to receive mode is defined, the actual time for monitoring receptions after a D2R transmission should be reader configurable.

(2)	Clarify whether the ‘corresponding R2D transmission’ in the definition of TD2R_min can include L1 R2D control information as well.

Observation 11	Defining TD2R_max might enable the devices to monitor responses within a certain duration, thereby handling error cases using timeouts. However, pre-defining this value in the standard is infeasible since number of PRDCH transmissions as part of Msg2 is completely dependent on the success/failure within Msg1 occasions, reader’s configurability and other considerations like coverage. Handling timeouts can instead rely on dynamic configuration of TD2R_max using L1 control signalling.

Observation 12	Early identification/termination function of PRDCH is possible for the device only if the R2D control information carrying the message type and/or device ID is self-decodable and is transmitted as a separate PRDCH

Observation 13	Having postamble is not particularly beneficial in reducing blind decodes of R2D messages

Observation 14	R2D control information is beneficial not only to handle the reception monitoring window as stated in Observation 11, but is also useful to reduce the number of blind decodes.

Observation 15	Supporting option 3 where a codepoint based mapping corresponds to one or more R values for FDMA leads to a possibility of creating FDMA resources which have different transmission capabilities as explained in observation 5.

Observation 16	R2D control includes both higher layer control as well as L1 control parameters.

Observation 17	Device (un)availability via Direction 1 does not require any specification update.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For Msg1, it is assumed that all devices transmit with same TBS. Therefore, Msg1 resources need to support resources with the same bandwidth/Tb.

Proposal 2	For Msg3 and further command messages, TBS of different sizes might be supported. Therefore, RAN1 to discuss whether or not to support variable BWs (different Tb) in these cases.

Proposal 3	FDMA resources scheduled in paging message for a set of devices should include resources supporting the same bandwidth and same data rate.

Proposal 4	Scheduling FDMA for Msg1 involves the reader providing implicitly or explicitly a set of tuples containing (D2R chip duration, R) such that their product  D2R chip length x R remains constant for all the scheduled resources.

Proposal 5	Resource allocation for Msg1 needs to consider only X=1.

Proposal 6	Number of Msg1 responses within a PRDCH for Msg2 should be reader-configurable but within the maximum supported TBS for R2D.

Proposal 7	From RAN1 perspective, there is no need to specify maximum number of Msg1 responses that can be carried within a PRDCH. It is enough to specify the maximum supported TBS for R2D.

Proposal 8	Support separate start time for Msg2 monitoring, at least for the case where there are multiple-Msg2 PRDCH transmissions corresponding to multiple Msg1 responses.

Proposal 9	To enable separate start times for each Msg2 PRDCH transmission, support R2D L1 control information before Msg2 PRDCH transmissions. The L1 control information can contain the individual start times (or offsets) for Msg2 transmissions corresponding to each Msg1.

Proposal 10	The time duration for monitoring Msg2 in response to multiple Msg1 transmissions can be indicated as part of L1 control information preceding the Msg2 transmissions.

Proposal 11	A time monitoring window for monitoring L1 control information preceding Msg2 transmissions may be specified instead of a Msg2 monitoring window

Proposal 12	FDMA resource scheduling for Msg3 within the PRDCH for Msg2 transmission contains at least the chip-level duration and repetition combination for one or multiple devices.

Proposal 13	For supporting frequency domain resource for Msg3 transmission, do not support Option 1 where the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

Proposal 14	Support only Way1 of non-interleaving Msg3 transmissions (e.g., Msg2-Msg2-Msg3-Msg3).

Proposal 15	The timing relations should be supported with the following considerations.

| Timings | Description in TR 38.769 | Defined in 3GPP standards/configured dynamically | Additional considerations if any? |
| --- | --- | --- | --- |
| TR2D_min | Minimum Time between a R2D transmission and the corresponding D2R transmission following it | Defined in 3GPP standards. Defining TR2D_min allows setting a minimal processing capability that the A-IoT devices need to support | From device’s perspective The device may need to switch between DL reception to UL transmission, in a case the R2D signal and CW is used for energy harvesting. From reader’s perspective CW cancellation at the reader- If the CW power is high and the reader needs to make some training to cancel CW from the backscattered transmission, then some additional time might be required. In TR 38.769 , RAN4 conclude the D2R reception should be avoided during the CW cancellation training time. |
| TR2D_max | Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. | Not necessary. If present configured dynamically. | Defining a fixed TR2D_max in the standard reduces the flexibility of configuration of resources within paging. |
| TD2R_min | Minimum Time between a D2R transmission and the corresponding R2D transmission following it. | Might not be necessary. If defined in the standards, this time should not mean that the devices automatically switch to reception state after TD2R_min expires | Devices automatically switching to receive mode after TD2R_min is not ideal since this means the device might keep receiving and monitoring other D2R transmissions. Therefore, the actual time for monitoring R2D transmission after a D2R transmission should be reader configurable. To clarify whether corresponding Msg2 can also be a L1 R2D control information scheduling Msg2 |
| TD2R_max | When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. | Has to be reader-configurable to handle issues like timeouts. Cannot be predefined in the standards. | Dynamic configuration of TD2R_max using L1 control signalling is preferred. |

Proposal 16	Message type/command code is already agreed to be part of R2D message within the MAC header. This can be part of R2D control information, but from higher layer perspective than L1 perspective.

Proposal 17	End of identification for a PRDCH can be handled using Option (2) based on R2D control information.

Proposal 18	In addition to R2D control information, discuss whether postamble is beneficial to indicate the end of transmission when SFO is high

Proposal 19	The clock-acquisition part of the R2D time acquisition signal preceding the PRDCH is used to determine the OOK chip duration used for the following PRDCH transmission

Proposal 20	Choose from among option1 (bit map based corresponding to one R value) or option2 (codepoint based corresponding to one R value) at least for Msg1 FDMA scheduling

Proposal 21	Alternately, a codepoint specifying <D2R chip duration, parameter R> can be specified to support SFS.

Proposal 22	Support option 1 where the [D2R chip duration]/[D2R information bit duration] is indicated in R2D control information from predefined a set of [D2R chip duration]/[D2R information bit duration] values

Proposal 23	[D2R chip duration]/[D2R information bit duration] can be indicated as an integer multiple/scaled versions of a reference unit time where this unit time can be defined as for example 1/15kHz, where 15kHz is the D2R transmission BW

Proposal 24	RAN1 will not discuss ACK/NACK as L1 D2R control information in Rel-19 A-IoT unless requested by RAN2

Proposal 25	Joint CRC is supported for higher layer control as well as data in a PRDCH transmission. Separate CRC is applicable only for L1 control information which is sent as a separate PRDCH transmission.

Proposal 26	A common L1 control signalling should be supported for scheduling R2D reception, at least for Msg2.

Proposal 27	L1 common control information should be send as a separate PRDCH transmission that precedes Msg2.

Proposal 28	The contents of L1 control signal should contain at least an indicator (for eg:  index to Msg1 transmission occasion) for the devices to see if its R2D response is present or not, and if present a monitoring window specifying where to monitor the R2D response.

Proposal 29	The following information may be carried as part of R2D control signalling for device-specific PDRCH scheduling like Msg3.

	Time domain resources

	MCS-like information- this includes FEC code rate

	<Chip duration, Chip level Repetition R> for SFS

	Bit-level Repetition

	Midamble- Number of midambles, Location of midamble (s) (if present)

Proposal 30	If Msg3 scheduling follows a RAR-like scheduling, above R2D control signalling parameters are carried in higher layer payload.

Proposal 31	FFS to discuss the behaviour of device-specific scheduling for messages after Msg3.

# References

RP-250796, “Revised Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc., RAN#107, March 2025.

TR 38.769 V19.0.0, “Study on Solutions for Ambient IoT (Internet of Things)”, 3GPP.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, 3GPP.

R1-2501625, “Final FL summary on other aspects for Rel-19 Ambient IoT”, Moderator (Vivo), RAN1#120, February 2025.