3GPP TSG RAN WG1 #120bis	R1-2501907

Wuhan, China, April 7th – 11st, 2025

Title: 	Discussion on transmission, reception and measurement procedures for SBFD operation

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.3.1

Document for:	Discussion/Decision

Introduction

In RAN#104, a revised WID on NR duplex evolution is endorsed in [1]. Based on the WID scope and the agreements reached in previous RAN1 meetings[2][3][4][5][6][7], we provide our analysis in this contribution on SBFD aware UEs in RRC_CONNECTED mode for the aspects including subband configuration and UE transmission/reception/measurement behavior and procedures.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. ….

Discussion

SBFD configuration

Guard period

In RAN4, the following agreements were reached for determination of the guard period between SBFD and non-SBFD symbols.

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

Given the existing TDD BS transmitter transient period is reused, RAN1 can decide that no explicit indication is needed as legacy.

Proposal 1: Explicit indication is NOT supported for the guard periods between SBFD and non-SBFD symbols.

UE behavior and procedures in SBFD symbols and/or non-SBFD symbols

Transmission and reception behaviors

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

Regarding a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, the following agreement has been reached in previous RAN1 meeting.

Agreement (RAN1#119) For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.

For a physical channel/signal with transmission/reception occasion mapped to SBFD and non-SBFD symbols within a slot, it’s preferable to define simplified solutions with minimized specification impacts. For most cases, it’s sufficient to not allow UE to transmit or receive the physical channel/signal across SBFD symbols and non-SBFD symbols within a slot. However, given a nominal repetition of PUSCH repetition type B can split into different actual repetitions if different Tx parameters are used. PUSCH repetition type B can be naturally fit into the transmission in different symbol types within a slot. Therefore, we prefer Option 1 above.

If Option 1 is supported, same rule as defined for PUSCH repetition Type A with Configuration 2 can be applied to each actual repetition within a slot or across different slots for PUSCH repetition Type B. For instance, the starting RB of actual repetition in SBFD symols can be determined based on the following equation.

- Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

  - If RBoffsetSBFD is not configured, it is zero

In addition, if Option 1 is supported for PUSCH repetition type B, there is no need to additionally support neither Configuration 1 or Configuration 2 for PUSCH repetition type B for simplicity.

Furthermore, given there is no need to indicate the transition period as discussed in Section 2.1.1, we don’t think it is relevant to the discussion here.

Based on the above analysis, we have the following proposal.

Proposal 2: For PUSCH repetition type B within a slot, Option 1 is supported.

  - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

    - The transmission mechanism of PUSCH repetition type A with Configuration 2 on different symbol types across different slots is applied to each actual repetition of PUSCH repetition type B on different symbol types within a slot.

    - Configuration 1 and Configuration 2 is not applicable to PUSCH repetition type B.

Enhancement on resource allocation in the frequency domain

FDRA RA type 1 for PDSCH

In RAN1#117 the following agreement was reached for frequency domain resource allocation Type 1 for PDSCH.

Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols.

Remaining details for partial PRG

In the RAN1#118b meeting, the maximum number of partial PRGs supported in SBFD symbols was discussed and a UE capability was agreed on the maximum number of partial PRGs that the UE can support. However, there was still no consensus on the relationship between the maximum number of partial PRGs and the corresponding value of the UE capability signalling. The following analysis is provided to address this issue.

If one DL subband is configured, it may result in at most 2 partial PRGs. If two DL subbands are configured, at most 4 partial PRGs can be generated. Therefore, we recommend that the UE can report one from {2, 4}.

If the value 2 is reported when two DL subbands are configured, the question is which 2 partial PRGs are allowed for a PDSCH transmission. From our point of view, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband. If the value 2 is reported when one DL subband is configured or if the value 4 is reported, there is no any NW scheduling restriction due to partial PRG.

For the above analysis, it is assumed that the number of scheduled partial PRGs cannot exceed the value reported by the UE. Obviously, it may limit the scheduling of the base station for some cases. Another way is that the base station can schedule a PDSCH including 1~4 partial PRGs no matter which value is reported by the UE. But, only some of the partial PRGs is considered as valid and used for transmission subject to the capability value reported by the UE.  This is similar as that the PDSCH resource outside the DL usable PRB is not used for actual transmission. From our point of view, this issue can be further discussed.

Proposal 3: In SBFD symbols, the maximum number of partial PRGs reported by a UE can be 2 or 4.

- If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband.

- Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains.

- FFS whether the number of scheduled partial PRGs can exceed the capability value reported by the UE.

PDSCH scheduled by a DCI format in CSS

For a DCI format in CSS, it may schedule a PDSCH carrying SIB1, other SIB, Msg2/MsgB, Msg4 or Paging according to existing specification. FDRA Type 1 is supported for these broadcast PDSCHs. For a PDSCH scheduled by a DCI format in CSS, which is namely DCI format 1_0, the UE shall assume that PRG size is equal to 2 PRBs.

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets - a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell - a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, then downlink resource allocation type 1 is used.

If a UE is scheduled a PDSCH with DCI format 1_0 or 4_0 for broadcast or 4_1 for multicast, the UE shall assume that is equal to 2 PRBs.

Observation 1: For PDSCH scheduled by a DCI format in CSS, FDRA Type 1 and PRG size of 2 PRBs are used.

Given RA type 1 is supported for PDSCH scheduled by a DCI format in CSS, it will be almost impossible to confine the allocated PRBs within DL subbands if VRB-to-PRB mapping is enabled. An example is shown in Figure 1. If Option 1-1 agreed for PDSCH scheduled by DCI format in USS is reused, the legacy UEs cannot be able to successfully receive the PDSCH as the TBS determination is changed.

Figure 1 Resource allocation in frequency domain for PDSCH scheduled by a DCI format in CSS

Observation 2: For PDSCH scheduled by a DCI format in CSS, if the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only, legacy UEs will not be able to successfully receive the broadcast PDSCH.

To minimize the impact of legacy UEs, one way is to apply puncturing of the invalid PRBs. Given the payload of PDSCH scheduled by a DCI format in CSS is typically small, it expects the punctured PRBs, if any, may be very few so that legacy UEs may still be able to successfully receive the broadcast PDSCH. Another way is to always disable VRB-to-PRB mapping for these broadcast PDSCH and restrict the transmission only in DL subbands. Apparently, the latter way is not optimal since it imposes scheduling restrictions for both legacy UEs and new UEs.

Proposal 4: For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled by DCI format in CSS, support the following:

- Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid.

  - Existing RB indexing and VRB-to-PRB mapping are reused.

  - Down-select one of the following options.

    - Option 1: The number of PRBs for TBS determination is based on the assigned PRBs as legacy. The invalid PRBs are punctured.

    - Option 2: The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only. VRB-to-PRB mapping is always disabled and a UE does not expect that the PDSCH is scheduled out of the DL usable PRBs

- No impact on DMRS sequence generation.

- DMRS sequence mapped to the RBs outside the DL usable PRBs are punctured.

- When an assigned PRG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception.

FDRA RA type 1 for PDSCH and PUSCH with indication granularity larger than 1

According to TS 38.214, the indication granularity of RIV for PDSCH and PUSCH with RA type 1 could be larger than 1 for the following two cases.

- When the DCI size for DCI format 0_0/1_0 in USS is derived from the initial UL BWP with size  but applied to another active BWP with size of , and if , the indication granularity K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

- When the scheduling grant is received with DCI format 0_2/1_2, the indication granularity P is defined by resourceAllocationType1GranularityDCI-0-2 or resourceAllocationType1GranularityDCI-1-2 if provided, and P=1 otherwise.

Therefore, it is possible that a group of PRBs with indication granularity K or P may overlap with subband boundary. Similar to RA type 0, we have the following proposal.

Proposal 5: For frequency resource allocation Type 1 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS),

- When an assigned RB group overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with an RB group for PDSCH which is fully outside DL usable PRBs or an RB group for PUSCH which is fully outside UL usable PRBs.

- The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Note: the RB group is the indication granularity K or P of RA type 1 for PDSCH or PUSCH in TS 38.214.

FDRA RA type 1 for PUSCH in UL usable PRBs

For PUSCH transmission in a single slot or in multiple slots with configuration 1 (including at least DG PUSCH, type 2 CG PUSCH), if it is scheduled on SBFD symbols, there may be the following two interpretations on how to understand the FDRA field in the DCI format:

- Option 1: The number of bits of the FDRA field is determined based on the size of the activated UL BWP, and the frequency resources for a PUSCH are also determined based on the active UL BWP.

- Option 2: The number of bits of the FDRA field is determined based on the size of the activated UL BWP, while the frequency resources for a PUSCH are determined based on the UL usable PRB. In other words, the number of bits in the FDRA field actually utilized for PUSCH resource indication are determined based on the size of UL usable PRBs, e.g., the LSBs of the FDRA field.

For Option 1, all the bits of the FDRA are used for frequency resource indication. Frequency domain resource allocation of PUSCH will be limited since the UL usable PRBs are only a part of the UL BWP and the indicated frequency resource may be out of the UL usable PRBs. An example shown in Figure 2.

Figure 2: Example of existing FDRA field structure in DCI format

A detailed example is shown in Table A1 of the Appendix. Assuming the size of the active UL BWP is 51 PRBs, 11 bits are required for FDRA field. If 2 MSB bits are used for indication of frequency offset for the second hop, the RIV value will be limited to 0~511, and about 59.9% of the RIV values (red highlighted values in the table) cannot be used. The situation may get much worse for SBFD operation since the UL usable PRBs occupy part of RBs in UL BWP.

Observation 3: The MSB(s) of the FDRA RA type 1 is used to indicate the frequency offset for the second hop, which will seriously affect the scheduling flexibility of PUSCH with Configuration 1 in SBFD symbols.

For Option 2, the above issue will be alleviated. As an example shown in Figure 3, the log2NSBsize∙NSBsize+12 LSBs of the FDRA field are actually utilized for frequency domain resource indication since the UL usable PRBs is less than the active UL BWP. That is, the number of bits can be determined based on the size of the UL usable PRBs (i.e., NSBsize). The NUL,hop MSBs of the FDRA field can be employed for indicating the frequency offset for the 2nd hop. This will reduce the influence on scheduling flexibility. In the example mentioned above, if the size of the UL usable PRBs equals to or less than 31 PRBs, the number of bits required for frequency domain resource allocation will not exceed 9 bits, and 2 MSBs in the FDRA field can be used to indicate the frequency offset without any restrictions on frequency domain resource indication, i.e., all the possible frequency allocation can be indicated.

Figure 3: Example of enhanced FDRA field structure in the DCI format

For PUSCH with Configuration 2, as agreed in RAN1#119, the PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. This represents that the FDRA field (including, number of bits of FDRA field and number of bits utilized to indicate frequency domain resource allocation) for PUSCH transmissions in non-SBFD symbols should be interpreted based on the size of the active UL BWP. Since PUSCH transmissions with Configuration 2 on both types of symbols shares the same FDRA field, it is difficult to implement the similar enhancement for PUSCH transmissions in SBFD symbols as proposed above for Configuration 1.

Proposal 6: For PUSCH transmission in a single slot or multiple slots with Configuration 1, if SBFD symbol is valid symbol type for PUSCH (including at least DG PUSCH, type 2 CG PUSCH), the following method should be supported for RA type 1,

- The number of bits of the FDRA field is determined based on the size of the active UL BWP.

- The MSB(s) of the FDRA field is employed for indicating the frequency offset for the 2nd hop.

- The LSBs of the FDRA field utilized for frequency domain resource indication are determined based on the size of UL usable PRBs.

- The remaining bits of the FDRA field are set to 0 if any.

CSI-RS resource

In RAN1#117, the following WA and agreement were reached for CSI-RS resource allocation.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

We propose to confirm the WA as no technical concern is identified.

Proposal 7: Confirm the following WA.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

PDCCH

For PDCCH, it was discussed in SI whether it can be configured to be across subband boundary. In our view, at least for PDCCH transmitted in CORESET 0, it is too restrictive for NW to avoid its collision with subband boundary. Because interleaving is always used for PDCCH transmission in CORESET 0, and the size of initial DL BWP is equal to the size of CORESET 0 if the size of initial DL BWP is not explicitly configured yet. If PDCCH in CORESET 0 cannot be across subband boundary, it means the initial DL BWP cannot be across subband boundary. Then, given the center frequency of DL BWP and UL BWP shall be aligned, which may cause no usable UL PRBs if the bandwidth of the initial UL BWP is no larger than the bandwidth of the initial DL BWP, which is a typical configuration. As a consequence, UL transmission cannot be performed in SBFD symbols in such case. Figure 4 illustrates the issue as an example.

Figure 4. Whether PDCCH in CORESET 0 can across subband boundary or not

If CORESET 0 is allowed to be across subband boundary, RAN1 needs to further discuss the corresponding UE behaviour, e.g., puncturing or rate matching the resources outside of DL subbands.

In RAN1#116bis, it was argued by some companies that NW can reconfigure the initial BWP size to be larger than CORESET 0 in SIB1. However, if this approach is adopted, the legacy NW and UE implementation would be impacted. Alternatively, similar as Redcap, a separate initial BWP can be configured to SBFD UEs to ensure isolated impacts to SBFD UEs only.

Proposal 8: For PDCCH transmitted at least in CORESET 0, RAN1 further discuss the following two options.

  - Option 1: CORESET 0 is allowed to be across subband boundary. FFS details.

  - Option 2: A separate initial BWP is configured for SBFD aware UEs, and no enhancement to CORESET 0 is pursued, i.e., CORESET 0 shall be contained within the DL useable PRBs.

Enhancements on physical channels/signals across SBFD symbols and non-SBFD symbols in different slots

In the WID [1], the following objectives for physical channels/signals across SBFD symbols and non-SBFD symbols in different slots are approved. In the following, we will discuss the two sub-bullets in separate subsections.

- Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

Detailed configuration design

In RAN1#118 and RAN1#119, the following agreements were reached.

Agreement (RAN1#118) For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation. Agreement(RAN1#119) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable. Agreement (RAN1#119) For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - For type 2 CG PUSCH, SPS PDSCH, down-select from the following options: - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. Agreement(RAN1#119) It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted> Agreement(RAN1#119) For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

Valid symbol type for PUCCH transmission carrying HARQ-ACK corresponding to an SPS PDSCH configuration

For each of SPS PDSCH, there may be a corresponding PUCCH carrying its HARQ-ACK. Then, a follow-up question is that, whether the different PUCCH occasions associated with different SPS PDSCHs are considered as independent PUCCH occasions or as one periodic PUCCH configuration. For the former, it means the PUCCH occasions across different symbol types does NOT belong to the case that ‘one transmission across SBFD symbols and non-SBFD symbols in different slots’. Then, no further discussion on how to determine the valid symbol type is needed for these PUCCH occasions (assuming no PUCCH repetition). On the other hand, given all PUCCH occasions are associated with one PUCCH resource for one SPS PDSCH configuration, it could be argued that it is the latter case. If so, further discussion is need to determine the valid symbol type of each PUCCH occasion carrying HARQ-ACK corresponding to SPS PDSCHs.

Proposal 9: For Configuration 1, further clarify whether/how to determine the valid symbol type of each PUCCH occasion carrying HARQ-ACK corresponding to SPS PDSCHs.

SRS

In RAN1#119, the following agreement was reached.

Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

For separate SRS configurations for SBFD and non-SBFD symbols, no issue is identified for the above working assumption and therefore we propose to confirm it.

Proposal 10: Confirm the above working assumption for separate SRS configurations for SBFD and non-SBFD symbols.

FFS usage set to  'beamManagement'

For SRS resource sets configured with usage set to 'beammanagement', considering that different beams for UL transmission in different symbol types should be supported, gNB should separately measure SRS resources in SBFD symbol and non-SBFD symbol to get different best beams for SBFD symbols and non-SBFD symbols, hence separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols can be supported for SRS resource sets configured with usage set to 'beammanagement'.

Proposal 11: The applicable usages of separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols include 'beammanagement'.

PUSCH

In RAN1#118b and RAN1#119, the following agreements were reached for PUSCH transmission across different symbol types.

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded. Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

Resource allocation type 0

In RAN1#119 meeting, the following equation 1-C2 was agreed to determine the allocated PRBs in SBFD symbols for PUSCH repetitions/TBoMS/CG PUSCH configuration in Configuration 2. It can be applied with RA type 1, but FFS for RA type 0.

- Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

  - If RBoffsetSBFD is not configured, it is zero

RA type 0 can allocate non-contiguous PRBs in non-SBFD symbols by adopting RBG-based bitmap signaling. However, it is difficult to get non-contiguous PRBs in SBFD symbols based on Equation 1-C2. It cannot be applied to the non-contiguous resource allocation directly. To resolve this issue, one of the following methods can be considered.

- Option 1: Equation 1-C2 is reused to determine the minimum PRB index allocated in SBFD symbols based on the minimum PRB index among the non-contiguous PRBs allocated in non-SBFD symbols. The PUSCH occupies a number of contiguous PRBs starting from the obtained minimum PRB index and the number of PRBs of the PUSCH does not change. In Option 1, only the contiguous PRBs are obtained in SBFD symbols, but it is simple.

- Option 2: If the bandwidth of UL usable PRBs is the same as the bandwidth of UL BWP, RA type 0 can be used and PRBs allocated based on FDRA are applicable to both SBFD symbols and non-SBFD symbols. Otherwise, RA type 0 is prohibited.

Obviously, it is difficult to obtain a simple and efficient solution to obtain non-contiguous PRBs based on Equation 1-C2. Option 1 is the simplest. However, only the contiguous frequency resource can be obtained. Therefore, we slightly prefer Option 2 or a combination of Option 1 and Option 2.

Proposal 12: For PUSCH repetitions/TBoMS/CG PUSCH configuration in Configuration 2, select one or multiple the following options for determination of frequency resource in SBFD symbols when RA type 0 is used.

- Option 1:  Equation 1-C2 is reused to determine the minimum PRB index allocated in SBFD symbols based on the minimum PRB index among the non-contiguous PRBs allocated in non-SBFD symbols. The PUSCH occupies a number of contiguous PRBs starting from the minimum PRB index and the number of PRBs of the PUSCH does not change.

- Option 2:  If the bandwidth of UL usable PRBs is the same as the bandwidth of UL BWP, RA type 0 can be used and PRBs allocated based on FDRA are applicable to both SBFD symbols and non-SBFD symbols. Otherwise, RA type 0 is prohibited.

Type 1 HARQ-ACK codebook

Type 1 HARQ-ACK codebook is constructed based on candidate PDSCHs within the slots determined by HARQ-ACK timing k1 set. However, some of the candidate PDSCHs may not be valid anymore due to SBFD operation. For example, for DG PDSCH with repetition, if Configuration 1 is provided (i.e., the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only) to SBFD aware UEs, the PDSCH repetition in one symbol type would be invalid. The invalid PDSCH should not be included in PDSCH candidates for determination of Type 1 HARQ-ACK codebook. Similar situation exits for SPS PDSCH which may also to be restricted to one symbol type.

Proposal 13: For PDSCH with repetition or SPS PDSCH, the invalid PDSCH should not be considered in in PDSCH candidates for determination of Type 1 HARQ-ACK codebook.

- The invalid PDSCH includes at least PDSCH scheduled on invalid symbol type for Configuration 1.

Configuration for SRS/PUCCH/PUSCH on SBFD symbols and non-SBFD symbols

Separate frequency configurations for PUCCH

The following agreement was reached in RAN1#118, but some FFSs are still pending.

Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

Regarding whether other parameters are configured separately or not

From our perspective, the parameter maxCodeRate in PUCCH-FormatConfig should be configured separately for SBFD symbols and non-SBFD symbols. This is because there is a significant difference in interference between UL subband and UL BWP, and independent maxCodeRate can ensure the performance and efficiency of PUCCH transmission in different types of symbols.

Proposal 14: The parameter maxCodeRate in PUCCH-FormatConfig should be configured separately for SBFD symbols and non-SBFD symbols.

Collision handling between DL reception in DL subband(s) and UL transmission in UL subband

Intra-UE collision handling in SBFD subbands

In previous RAN1 meetings, the following agreements were reached for collision handling.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded. Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included. Agreement For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition

Case 5: SSB vs. dynamically scheduled or configured UL transmission

In RAN1#117 and RAN1#118b, the following agreement and Working Assumption on Case 5 were reached.

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

For the WA, we would like to clarify that the SSB refers to the actual SSB transmission configured by RRC signaling “ssb-PositionsInBurst” in SIB1.

Proposal 15: RAN1 clarifies that the SSB refers to the actual SSB transmission configured by RRC signaling “ssb-PositionsInBurst” in SIB1 in the following WA.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Collision handling order

In the RAN1#120 meeting, the collision handling order in SBFD symbols was discussed, and the proposed agreement is as follows in [8].

Proposed Agreement: For collision handling in SBFD symbols for SBFD aware UEs, down select one from the following options - Option 1 (From Round 1): - Step 1: Resolving SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) , if any. - Step 2: Resolving the collision as in legacy, if any. - Option 2 (From Samsung): - Step 1: Resolving SBFD specific collision between PUCCH with repetitions/PUSCH and unusable resources, if any. - Step 2: Resolving the collision as in legacy, if any. - Step 3: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any. - Option 3 (New): - Step 1: Resolving the collision as in legacy, if any. - Step 2: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

From our point of view, option 3 is preferred because it complies with most of the collision handling rules in the current specification, such as resolving collisions between PUCCHs or between PUCCH and PUSCH, before deleting PUCCHs that collide with the frame structure or with high priority transmissions.

Based on the above analysis, we have the following proposal.

Proposal 16: For collision handling in SBFD symbols for SBFD aware UEs, Option 3 is supported.

- Step 1: Resolving the collision as in legacy, if any.

- Step 2: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

UL inter-UE collision handling in SBFD subbands

During Rel-16 URLLC, UL cancelation mechanism was specified for UL inter-UE multiplexing.

More specifically, a DCI format 2_4 is defined for indicating the cancelation resource. And the time domain range for the indicated resources is obtained by excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon. However, the UL subband can be configured in DL symbol, then there may be UL transmission in DL symbol. With the legacy rule, UL transmission in UL subband within a DL symbol cannot be cancelled by the DCI format 2_4. Therefore, how to apply the UL cancelation mechanism in the UL subband needs to be further studied.

Observation 4: UL transmission in UL subband within a DL symbol cannot be cancelled by the DCI format 2_4.

Interaction between SBFD operation and Rel-16~19 features

For SBFD aware UEs, it may also support optional features that have been already specified in Rel-16~ Rel-18 or is being specified in Rel-19. For instance, as raised by many companies, a SBFD aware UEs may also support Rel-17 coverage enhancement features, e.g., available slot counting, TBoMS or DMRS bundling. Then, optimization may be needed to support the combination of such features and SBFD operation. In our view, RAN1 needs to further discuss whether any optimization is needed for each of the Rel-16~Rel-19 features.

Proposal 17: RAN1 needs to further discuss whether any optimization is needed for each of the Rel-16~Rel-19 features.

RRC parameters

In the RAN1#120 meeting, the following new agreements involving RRC parameters were reached, but the corresponding RRC parameters were discussed.

Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for Type 1 CG PUSCH is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - The valid symbol type for PUCCH configured for SR is configured in SchedulingRequestResourceConfig. - The valid symbol type for PUCCH carrying P-CSI is configured in periodic in CSI-ReportConfig. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for PUSCH carrying SP-CSI is determined based on the symbol type of the first PUSCH occasion associated with activation DCI. - For SP-CSI on PUCCH: - Option 2: the valid symbol type for PUCCH carrying SP-CSI is configured in semiPersistentOnPUCCH in CSI-ReportConfig. Agreement If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId.

Therefore, we have the following proposal about RRC parameters per above agreements.

Proposal 18: Introduce and modify the following RRC parameters.

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| rrc-ConfiguredUplinkGrant | symbolType | New | Configure the valid symbol type for Type 1 CG PUSCH with Configuration 1 | {sbfd, non-sbfd} | Per BWP In rrc-ConfiguredUplinkGrant | UE-specific |
| SchedulingRequestResourceConfig | symbolType | New | Configure the valid symbol type for SR with Configuration 1 | {sbfd, non-sbfd} | Per BWP In SchedulingRequestResourceConfig | UE-specific |
| periodic | symbolType | New | Configure the valid symbol type for PUCCH carrying P-CSI with Configuration 1 | Per DL BWP In CSI-ReportConfig | UE-specific | UE-specific |
| semiPersistentOnPUCCH | symbolType | New | Configure the symbol type for PUCCH carrying SP-CSI with Configuration 1 | Per DL BWP In CSI-ReportConfig | UE-specific | UE-specific |
| PUCCH-Resource | PUCCH-StartingPRB-SBFD | New | The starting PRB of the PUCCH resource in SBFD symbols. If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId. | PRB-Id | Per UL BWP In PUCCH-Resource | UE-specific |

Conclusion

In this contribution, we provide our analysis for subband non-overlapping full duplex for possible solutions with the following proposals and observations.

SBFD configuration

Proposal 1: Explicit indication is NOT supported for the guard periods between SBFD and non-SBFD symbols.

UE behavior and procedures in SBFD symbols and/or non-SBFD symbols

Proposal 2: For PUSCH repetition type B within a slot, Option 1 is supported.

  - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

    - The transmission mechanism of PUSCH repetition type A with Configuration 2 on different symbol types across different slots is applied to each actual repetition of PUSCH repetition type B on different symbol types within a slot.

    - Configuration 1 and Configuration 2 is not applicable to PUSCH repetition type B.

Proposal 3: In SBFD symbols, the maximum number of partial PRGs reported by a UE can be 2 or 4.

- If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband.

- Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains.

- FFS whether the number of scheduled partial PRGs can exceed the capability value reported by the UE.

Observation 1: For PDSCH scheduled by a DCI format in CSS, FDRA Type 1 and PRG size of 2 PRBs are used.

Observation 2: For PDSCH scheduled by a DCI format in CSS, if the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only, legacy UEs will not be able to successfully receive the broadcast PDSCH.

Proposal 4: For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled by DCI format in CSS, support the following:

- Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid.

  - Existing RB indexing and VRB-to-PRB mapping are reused.

  - Down-select one of the following options.

    - Option 1: The number of PRBs for TBS determination is based on the assigned PRBs as legacy. The invalid PRBs are punctured.

    - Option 2: The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only. VRB-to-PRB mapping is always disabled and a UE does not expect that the PDSCH is scheduled out of the DL usable PRBs

- No impact on DMRS sequence generation.

- DMRS sequence mapped to the RBs outside the DL usable PRBs are punctured.

- When an assigned PRG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception.

Proposal 5: For frequency resource allocation Type 1 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS),

- When an assigned RB group overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with an RB group for PDSCH which is fully outside DL usable PRBs or an RB group for PUSCH which is fully outside UL usable PRBs.

- The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Note: the RB group is the indication granularity K or P of RA type 1 for PDSCH or PUSCH in TS 38.214.

Observation 3: The MSB(s) of the FDRA RA type 1 is used to indicate the frequency offset for 2nd hop, which will seriously affect the scheduling flexibility of PUSCH with Configuration 1 in SBFD symbols.

Proposal 6: For PUSCH transmission in a single slot or multiple slots with Configuration 1, if SBFD symbol is valid symbol type for PUSCH (including at least DG PUSCH, type 2 CG PUSCH), the following method should be supported for RA type 1,

- The number of bits of the FDRA field is determined based on the size of the active UL BWP.

- The MSB(s) of the FDRA field is employed for indicating the frequency offset for the 2nd hop.

- The LSBs of the FDRA field utilized for frequency domain resource indication are determined based on the size of UL usable PRBs.

- The remaining bits of the FDRA field are set to 0 if any.

Proposal 7: Confirm the following WA.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

Proposal 8: For PDCCH transmitted at least in CORESET 0, RAN1 further discuss the following two options.

  - Option 1: CORESET 0 is allowed to be across subband boundary. FFS details.

  - Option 2: A separate initial BWP is configured for SBFD aware UEs, and no enhancement to CORESET 0 is pursued, i.e., CORESET 0 shall be contained within the DL useable PRBs.

Proposal 9: For Configuration 1, further clarify whether/how to determine the valid symbol type of each PUCCH occasion carrying HARQ-ACK corresponding to SPS PDSCHs.

Proposal 10: Confirm the above working assumption for separate SRS configurations for SBFD and non-SBFD symbols.

Proposal 11: The applicable usages of separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols include 'beammanagement'.

Proposal 12: For PUSCH repetitions/TBoMS/CG PUSCH configuration in Configuration 2, select one or multiple the following options for determination of frequency resource in SBFD symbols when RA type 0 is used.

- Option 1:  Equation 1-C2 is reused to determine the minimum PRB index allocated in SBFD symbols based on the minimum PRB index among the non-contiguous PRBs allocated in non-SBFD symbols. The PUSCH occupies a number of contiguous PRBs starting from the minimum PRB index and the number of PRBs of the PUSCH does not change.

- Option 2:  If the bandwidth of UL usable PRBs is the same as the bandwidth of UL BWP, RA type 0 can be used and PRBs allocated based on FDRA are applicable to both SBFD symbols and non-SBFD symbols. Otherwise, RA type 0 is prohibited.

Proposal 13: For PDSCH with repetition or SPS PDSCH, the invalid PDSCH should not be considered in in PDSCH candidates for determination of Type 1 HARQ-ACK codebook.

- The invalid PDSCH includes at least PDSCH scheduled on invalid symbol type for Configuration 1.

Proposal 14: The parameter maxCodeRate in PUCCH-FormatConfig should be configured separately for SBFD symbols and non-SBFD symbols.

Proposal 15: RAN1 clarifies that the SSB refers to the actual SSB transmission configured by RRC signaling “ssb-PositionsInBurst” in SIB1 in the following WA.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Proposal 16: For collision handling in SBFD symbols for SBFD aware UEs, Option 3 is supported.

- Step 1: Resolving the collision as in legacy, if any.

- Step 2: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

Observation 4: UL transmission in UL subband within a DL symbol cannot be cancelled by the DCI format 2_4.

Interaction between SBFD operation and Rel-16~19 features

Proposal 17: RAN1 needs to further discuss whether any optimization is needed for each of the Rel-16~Rel-19 features.

RRC parameters

Proposal 18: Introduce and modify the following RRC parameters.

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| rrc-ConfiguredUplinkGrant | symbolType | New | Configure the valid symbol type for Type 1 CG PUSCH with Configuration 1 | {sbfd, non-sbfd} | Per BWP In rrc-ConfiguredUplinkGrant | UE-specific |
| SchedulingRequestResourceConfig | symbolType | New | Configure the valid symbol type for SR with Configuration 1 | {sbfd, non-sbfd} | Per BWP In SchedulingRequestResourceConfig | UE-specific |
| periodic | symbolType | New | Configure the valid symbol type for PUCCH carrying P-CSI with Configuration 1 | Per DL BWP In CSI-ReportConfig | UE-specific | UE-specific |
| semiPersistentOnPUCCH | symbolType | New | Configure the symbol type for PUCCH carrying SP-CSI with Configuration 1 | Per DL BWP In CSI-ReportConfig | UE-specific | UE-specific |
| PUCCH-Resource | PUCCH-StartingPRB-SBFD | New | The starting PRB of the PUCCH resource in SBFD symbols. If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId. | PRB-Id | Per UL BWP In PUCCH-Resource | UE-specific |

Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- Draft_Minutes_report_RAN1#116_v010, RAN1 chairman.

- Draft_Minutes_report_RAN1#116b_v030, RAN1 chairman.

- Draft_Minutes_report_RAN1#117_v020, RAN1 chairman.

- Draft_Minutes_report_RAN1#118_v020, RAN1 chairman.

- Draft_Minutes_report_RAN1#118b_v020, RAN1 chairman.

- Draft_Minutes_report_RAN1#119_v020, RAN1 chairman.

- Draft_Minutes_report_RAN1#120_v020, RAN1 chairman.

Appendix

Table A1: Example of scheduling limitations due to indicating frequency offset

| N=51 RBs, FDRA size=11bits, NUL,hop=2bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 2 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 |  |
| 3 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 | 150 |  |  |
| 4 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 | 200 |  |  |  |
| 5 | 204 | 205 | 206 | 207 | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 |  |  |  |  |
| 6 | 255 | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 | 300 |  |  |  |  |  |
| 7 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 | 315 | 316 | 317 | 318 | 319 | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 | 330 | 331 | 332 | 333 | 334 | 335 | 336 | 337 | 338 | 339 | 340 | 341 | 342 | 343 | 344 | 345 | 346 | 347 | 348 | 349 | 350 |  |  |  |  |  |  |
| 8 | 357 | 358 | 359 | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 | 368 | 369 | 370 | 371 | 372 | 373 | 374 | 375 | 376 | 377 | 378 | 379 | 380 | 381 | 382 | 383 | 384 | 385 | 386 | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 | 396 | 397 | 398 | 399 | 400 |  |  |  |  |  |  |  |
| 9 | 408 | 409 | 410 | 411 | 412 | 413 | 414 | 415 | 416 | 417 | 418 | 419 | 420 | 421 | 422 | 423 | 424 | 425 | 426 | 427 | 428 | 429 | 430 | 431 | 432 | 433 | 434 | 435 | 436 | 437 | 438 | 439 | 440 | 441 | 442 | 443 | 444 | 445 | 446 | 447 | 448 | 449 | 450 |  |  |  |  |  |  |  |  |
| 10 | 459 | 460 | 461 | 462 | 463 | 464 | 465 | 466 | 467 | 468 | 469 | 470 | 471 | 472 | 473 | 474 | 475 | 476 | 477 | 478 | 479 | 480 | 481 | 482 | 483 | 484 | 485 | 486 | 487 | 488 | 489 | 490 | 491 | 492 | 493 | 494 | 495 | 496 | 497 | 498 | 499 | 500 |  |  |  |  |  |  |  |  |  |
| 11 | 510 | 511 | 512 | 513 | 514 | 515 | 516 | 517 | 518 | 519 | 520 | 521 | 522 | 523 | 524 | 525 | 526 | 527 | 528 | 529 | 530 | 531 | 532 | 533 | 534 | 535 | 536 | 537 | 538 | 539 | 540 | 541 | 542 | 543 | 544 | 545 | 546 | 547 | 548 | 549 | 550 |  |  |  |  |  |  |  |  |  |  |
| 12 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 570 | 571 | 572 | 573 | 574 | 575 | 576 | 577 | 578 | 579 | 580 | 581 | 582 | 583 | 584 | 585 | 586 | 587 | 588 | 589 | 590 | 591 | 592 | 593 | 594 | 595 | 596 | 597 | 598 | 599 | 600 |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 612 | 613 | 614 | 615 | 616 | 617 | 618 | 619 | 620 | 621 | 622 | 623 | 624 | 625 | 626 | 627 | 628 | 629 | 630 | 631 | 632 | 633 | 634 | 635 | 636 | 637 | 638 | 639 | 640 | 641 | 642 | 643 | 644 | 645 | 646 | 647 | 648 | 649 | 650 |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 663 | 664 | 665 | 666 | 667 | 668 | 669 | 670 | 671 | 672 | 673 | 674 | 675 | 676 | 677 | 678 | 679 | 680 | 681 | 682 | 683 | 684 | 685 | 686 | 687 | 688 | 689 | 690 | 691 | 692 | 693 | 694 | 695 | 696 | 697 | 698 | 699 | 700 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 714 | 715 | 716 | 717 | 718 | 719 | 720 | 721 | 722 | 723 | 724 | 725 | 726 | 727 | 728 | 729 | 730 | 731 | 732 | 733 | 734 | 735 | 736 | 737 | 738 | 739 | 740 | 741 | 742 | 743 | 744 | 745 | 746 | 747 | 748 | 749 | 750 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 765 | 766 | 767 | 768 | 769 | 770 | 771 | 772 | 773 | 774 | 775 | 776 | 777 | 778 | 779 | 780 | 781 | 782 | 783 | 784 | 785 | 786 | 787 | 788 | 789 | 790 | 791 | 792 | 793 | 794 | 795 | 796 | 797 | 798 | 799 | 800 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 816 | 817 | 818 | 819 | 820 | 821 | 822 | 823 | 824 | 825 | 826 | 827 | 828 | 829 | 830 | 831 | 832 | 833 | 834 | 835 | 836 | 837 | 838 | 839 | 840 | 841 | 842 | 843 | 844 | 845 | 846 | 847 | 848 | 849 | 850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 867 | 868 | 869 | 870 | 871 | 872 | 873 | 874 | 875 | 876 | 877 | 878 | 879 | 880 | 881 | 882 | 883 | 884 | 885 | 886 | 887 | 888 | 889 | 890 | 891 | 892 | 893 | 894 | 895 | 896 | 897 | 898 | 899 | 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 918 | 919 | 920 | 921 | 922 | 923 | 924 | 925 | 926 | 927 | 928 | 929 | 930 | 931 | 932 | 933 | 934 | 935 | 936 | 937 | 938 | 939 | 940 | 941 | 942 | 943 | 944 | 945 | 946 | 947 | 948 | 949 | 950 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | 969 | 970 | 971 | 972 | 973 | 974 | 975 | 976 | 977 | 978 | 979 | 980 | 981 | 982 | 983 | 984 | 985 | 986 | 987 | 988 | 989 | 990 | 991 | 992 | 993 | 994 | 995 | 996 | 997 | 998 | 999 | 1000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | 1020 | 1021 | 1022 | 1023 | 1024 | 1025 | 1026 | 1027 | 1028 | 1029 | 1030 | 1031 | 1032 | 1033 | 1034 | 1035 | 1036 | 1037 | 1038 | 1039 | 1040 | 1041 | 1042 | 1043 | 1044 | 1045 | 1046 | 1047 | 1048 | 1049 | 1050 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | 1071 | 1072 | 1073 | 1074 | 1075 | 1076 | 1077 | 1078 | 1079 | 1080 | 1081 | 1082 | 1083 | 1084 | 1085 | 1086 | 1087 | 1088 | 1089 | 1090 | 1091 | 1092 | 1093 | 1094 | 1095 | 1096 | 1097 | 1098 | 1099 | 1100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | 1122 | 1123 | 1124 | 1125 | 1126 | 1127 | 1128 | 1129 | 1130 | 1131 | 1132 | 1133 | 1134 | 1135 | 1136 | 1137 | 1138 | 1139 | 1140 | 1141 | 1142 | 1143 | 1144 | 1145 | 1146 | 1147 | 1148 | 1149 | 1150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | 1173 | 1174 | 1175 | 1176 | 1177 | 1178 | 1179 | 1180 | 1181 | 1182 | 1183 | 1184 | 1185 | 1186 | 1187 | 1188 | 1189 | 1190 | 1191 | 1192 | 1193 | 1194 | 1195 | 1196 | 1197 | 1198 | 1199 | 1200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | 1224 | 1225 | 1226 | 1227 | 1228 | 1229 | 1230 | 1231 | 1232 | 1233 | 1234 | 1235 | 1236 | 1237 | 1238 | 1239 | 1240 | 1241 | 1242 | 1243 | 1244 | 1245 | 1246 | 1247 | 1248 | 1249 | 1250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 1275 | 1276 | 1277 | 1278 | 1279 | 1280 | 1281 | 1282 | 1283 | 1284 | 1285 | 1286 | 1287 | 1288 | 1289 | 1290 | 1291 | 1292 | 1293 | 1294 | 1295 | 1296 | 1297 | 1298 | 1299 | 1300 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | 1325 | 1324 | 1323 | 1322 | 1321 | 1320 | 1319 | 1318 | 1317 | 1316 | 1315 | 1314 | 1313 | 1312 | 1311 | 1310 | 1309 | 1308 | 1307 | 1306 | 1305 | 1304 | 1303 | 1302 | 1301 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | 1274 | 1273 | 1272 | 1271 | 1270 | 1269 | 1268 | 1267 | 1266 | 1265 | 1264 | 1263 | 1262 | 1261 | 1260 | 1259 | 1258 | 1257 | 1256 | 1255 | 1254 | 1253 | 1252 | 1251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | 1223 | 1222 | 1221 | 1220 | 1219 | 1218 | 1217 | 1216 | 1215 | 1214 | 1213 | 1212 | 1211 | 1210 | 1209 | 1208 | 1207 | 1206 | 1205 | 1204 | 1203 | 1202 | 1201 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | 1172 | 1171 | 1170 | 1169 | 1168 | 1167 | 1166 | 1165 | 1164 | 1163 | 1162 | 1161 | 1160 | 1159 | 1158 | 1157 | 1156 | 1155 | 1154 | 1153 | 1152 | 1151 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 1121 | 1120 | 1119 | 1118 | 1117 | 1116 | 1115 | 1114 | 1113 | 1112 | 1111 | 1110 | 1109 | 1108 | 1107 | 1106 | 1105 | 1104 | 1103 | 1102 | 1101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | 1070 | 1069 | 1068 | 1067 | 1066 | 1065 | 1064 | 1063 | 1062 | 1061 | 1060 | 1059 | 1058 | 1057 | 1056 | 1055 | 1054 | 1053 | 1052 | 1051 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 1019 | 1018 | 1017 | 1016 | 1015 | 1014 | 1013 | 1012 | 1011 | 1010 | 1009 | 1008 | 1007 | 1006 | 1005 | 1004 | 1003 | 1002 | 1001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | 968 | 967 | 966 | 965 | 964 | 963 | 962 | 961 | 960 | 959 | 958 | 957 | 956 | 955 | 954 | 953 | 952 | 951 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 | 917 | 916 | 915 | 914 | 913 | 912 | 911 | 910 | 909 | 908 | 907 | 906 | 905 | 904 | 903 | 902 | 901 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 | 866 | 865 | 864 | 863 | 862 | 861 | 860 | 859 | 858 | 857 | 856 | 855 | 854 | 853 | 852 | 851 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 | 815 | 814 | 813 | 812 | 811 | 810 | 809 | 808 | 807 | 806 | 805 | 804 | 803 | 802 | 801 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | 764 | 763 | 762 | 761 | 760 | 759 | 758 | 757 | 756 | 755 | 754 | 753 | 752 | 751 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 | 713 | 712 | 711 | 710 | 709 | 708 | 707 | 706 | 705 | 704 | 703 | 702 | 701 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | 662 | 661 | 660 | 659 | 658 | 657 | 656 | 655 | 654 | 653 | 652 | 651 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 | 611 | 610 | 609 | 608 | 607 | 606 | 605 | 604 | 603 | 602 | 601 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 | 560 | 559 | 558 | 557 | 556 | 555 | 554 | 553 | 552 | 551 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 | 509 | 508 | 507 | 506 | 505 | 504 | 503 | 502 | 501 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 | 458 | 457 | 456 | 455 | 454 | 453 | 452 | 451 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 45 | 407 | 406 | 405 | 404 | 403 | 402 | 401 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 | 356 | 355 | 354 | 353 | 352 | 351 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 | 305 | 304 | 303 | 302 | 301 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 | 254 | 253 | 252 | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 49 | 203 | 202 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 | 152 | 151 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |