3GPP TSG RAN WG1 #120bis			R1-2501928

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.3.1

Source:	InterDigital, Inc.

Title:	On SBFD TX_RX_measurement procedures

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item are as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC-CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]: - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Related agreements made in this agenda from RAN1#120 meeting [2] were outlined:

Agreement For aperiodic SRS with available slot counting, - For SRS-ResourceSet configured for SBFD symbol, an available slot is a slot satisfying there are SBFD symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - For SRS-ResourceSet configured for non-SBFD symbol, an available slot is a slot satisfying there are UL or flexible symbol(s) not configured as SBFD symbols for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. Agreement The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for PUSCH carrying SP-CSI is determined based on the symbol type of the first PUSCH occasion associated with activation DCI. - For SP-CSI on PUCCH: - Option 2: the valid symbol type for PUCCH carrying SP-CSI is configured in semiPersistentOnPUCCH in CSI-ReportConfig. Conclusion - There is no RAN1 consensus to support semi-static link direction indication for SBFD aware UEs. - For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, TDD-UL-DL-ConfigDedicated is only applicable to non-SBFD symbols but not applicable to SBFD symbols Agreement For determining starting PRB for PUSCH transmissions in SBFD symbols for Configuration 2, - For type 1 CG PUSCH, a RBoffsetSBFD is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - For type 2 CG PUSCH and DG PUSCH, a RBoffsetSBFD is configured per UL BWP in PUSCH-Config. - The RBoffsetSBFD is commonly applicable to all DCI formats which schedule PUSCH - If RBoffsetSBFD is not configured, it is assumed to be zero Agreement When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports should be the same for SRS resources with the same corresponding SRI values in the two SRS resource sets associated with SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for Type 1 CG PUSCH is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - The valid symbol type for PUCCH configured for SR is configured in SchedulingRequestResourceConfig. - The valid symbol type for PUCCH carrying P-CSI is configured in periodic in CSI-ReportConfig. Agreement For a type 1 CG PUSCH with Configuration 2, if FH offset for SBFD symbols is not configured for the type 1 CG PUSCH, frequency hopping is disabled for the type 1 CG PUSCH transmissions in SBFD symbols. For a type 2 CG PUSCH/PUSCH scheduled by DCI, if FH offset lists for SBFD symbols are not configured for the type 2 CG PUSCH/PUSCH scheduled by DCI, frequency hopping is disabled for the transmission of the type 2 CG PUSCH/PUSCH scheduled by DCI in SBFD symbols. Agreement For SPS PDSCH with Configuration 2, for TBS determination, - For SPS PDSCH without repetition, - The number of PRBs for TBS determination for an SPS PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. - The number of PRBs for TBS determination for an SPS PDSCH in non-SBFD symbols is based on assigned PRBs. - For SPS PDSCH with repetition, TBS is determined based on the first repetition occasion. - If the first repetition occasion of a SPS PDSCH is in SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs within DL usable PRBs only. - If the first repetition occasion of a SPS PDSCH is in non-SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs. Agreement If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId. Agreement The one configuration of intraSlotFrequencyHopping for a PUCCH-Resource is applied to both PUCCH transmissions in SBFD symbols and in non-SBFD symbols associated with this pucch-ResourceId. Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

In this contribution, we discuss issues on SBFD Tx, Rx, measurement procedures, including SBFD operation with CA, implicit guard periods between different symbol types, SBFD operation in SSB symbols, and extending SBFD beyond Rel-19 for flexibility and scalability.

# Discussion on SBFD TX, RX, measurement procedures

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-18, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-19, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the work item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## SBFD operation with CA

In RAN1#120 meeting, it was agreed to support SBFD operation on one TDD carrier in multi-carrier scenario. In such case of carrier aggregation (CA), it is noted that, if the TDD carrier for SBFD operation is an Scell, a UE receives UE-dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. The agreement implies that the Rel-19 SBFD feature is limited to one carrier among multiple carriers if the UE is configured with CA, although in the future releases it should be considered to relax the limitation for further improved flexibility and performance. For example, when at least one carrier operates with SBFD in CA, UE behavior on control channel monitoring may be fallback to a different carrier not operating with SBFD in case a type of collisions is detected such as link direction collisions, etc., for further investigations beyond Rel-19.

In consideration of such limited scope and timeframe in Rel-19, the FFS point on whether/how to handle half-duplex CA case seems not desired to be optimized but should rely on the network implementation such as proper DL and UL scheduling considering the reported UE capability. For example, the UE may expect to be scheduled with either a DL reception or a UL transmission at a time across cells, and the UE may not expect to be scheduled simultaneously with SBFD operation which violates the half-duplex CA constraint. With such network implementation, it is not justified to further optimize UE behaviors in combination with the half-duplex CA case in Rel-19.

Observation 1. The agreement on supporting SBFD operation on one TDD carrier implies that the Rel-19 SBFD feature is limited to one carrier among multiple carriers if the UE is configured with CA.

Proposal 1. In consideration of such limited scope and timeframe in Rel-19, it is not justified to further optimize UE behaviors in combination with the half-duplex CA case, where the UE does not expect to be scheduled simultaneously with SBFD operation which violates the half-duplex CA constraint.

## On implicit guard periods between SBFD and non-SBFD symbols for one UE perspective

In conventional TDD systems, the UL and DL slots are considered separately in time domain. Therefore, subsequent slots are split in DL-only, UL-only, and flexible slots. The symbols in flexible slots can be scheduled to be used as DL or UL based on received configurations. The flexible symbols can also be used as guard period for timing alignment requirements (e.g., DL/UL switching, UL timing advance (TA)).

In SBFD schemes with one or more subbands allocated for UL transmission in DL slots, a non-zero timing advance or switching time in SBFD could result in slot collision cases. As an example, shown in Figure 3, where the UL signals in UL SBs and DL signals in DL SBs/slots are collided by each other due to timing advance. Also, another example on slot-boundary collision case is shown in Figure 4 that is affecting the SBFD slots due to required switching time for UE between DL and UL that is also a factor for the timing alignment requirements.

For example, especially for a semi-statically scheduled resource (e.g., configured grant PUSCH), every time when UE receives a TA command which affects all related UL transmission including the CG-PUSCH, the UE has to check whether the adjusted/accumulated TA and the switching time requirement result in overlap with DL symbol in a prior DL slot, if scheduled, for SBFD operation. And if any slot-boundary collision happens, the UE does not know which selected one of a DL reception and a UL transmission the UE should perform as scheduled and how to perform the other scheduled one that is currently overlapped in symbols, since the UE performs either DL only or UL only on a given set of symbols as a half-duplex UE. This does not happen in legacy TDD system as there is the flexible slot for this reason at least, while in SBFD operation, it is always possible to face a back-to-back scheduling between DL and UL for one UE perspective.

Figure 3. Slot collision issue due to timing advance for one UE perspective

Figure 4. Slot collision issue due to switching time between DL and UL

While the gNB might address the required timing advance/switching time based on resource allocations and scheduling (e.g., as part of gNB implementation), this requires significantly increased scheduling complexity at the gNB side and also increases the signaling overhead, especially for taking such semi-statically scheduled resources for both DL and UL into account. In another aspect, in case a scheduled (dynamic) grant for UL or DL with high priority is received and if the timing alignment requirement including required Rx/Tx or Tx/Rx switching time is not met, e.g., resulting in some DL/UL overlapped symbols being faced at the UE, this could again result in a severe performance degradation or dropping corresponding slots.

Observation 2. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

In case a UL transmission in SBFD UL subband overrides the symbols from a previous slot (e.g., a legacy ‘D’ slot) due to the non-zero timing advance, the DL scheduled in those symbols results in the slot collision case and may experience inter-subband CLI as well as inter-slot interference. The slot collision issue due to non-zero TA becomes critical, as SBFD framework in Rel-19 has no ‘Special’ slot in-between the legacy D slot and the SBFD slot back-to-back. The slot collision case with the inter-slot interference is more severe for the legacy UEs scheduled in the previous slot for DL reception, as such a timing misalignment results in intra-subband (directly colliding) and inter-slot interference on the symbols close to the end of the DL slot. The legacy UEs may be scheduled to receive SSB, CORESET#0, or DMRS in PDSCH in the symbols that are overridden by the misaligned UL transmission in the UL subband in the right next slot. As such, the performance of legacy UEs may be affected due to the non-zero TA.

Therefore, there should be a means for UE to detect if the allocated UL/DL timing alignment is accurate and enough. In case UE has detected such timing discrepancies, the UE could report the issues along with requesting for adjustment or at least should be able to determine which overlapped portion of either DL symbol(s) or UL symbol(s) can be dropped or punctured. For example, the UE could also determine to include the time required for the timing alignment inside respective UL/DL SB in SBFD slot, as implicit guard periods, where the scheduled UL transmission on the SBFD slot can have a rate-matched or punctured symbol(s) in the front symbol positions of the UL transmission, or alternatively DL symbol(s) can be punctured, which should depend on gNB’s flexible configurations for such UE behaviours.

Observation 3. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 4. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 2. Support implicit guard periods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

## SBFD operation in SSB symbols

During the study phase, the support for the SBFD operation in SSB symbols was discussed. Based on the discussions, it was clear that not supporting the SBFD operation in SSB symbols may degrade the performance and objectives of the SBFD operation.

Contiguous UL subband resources in time domain

Considering the UL repetition, the contiguous UL subband resources should be provided in time domain. However, in case the SBFD operation is not allowed on SSB symbols, this requires switching to DL-only symbols each time there is an SSB transmission. As such, switching between SBFD symbols and DL-only symbols will affect the resources for UL repetition in SBFD symbols and will degrade the coverage enhancement purposes.

Latency in UL transmission

In addition to coverage enhancement, one of the objectives in SBFD operation is to reduce latency in UL transmissions. However, in case the SSB symbols are not transmitted in SBFD resources, this will change the TDD model back to DDDSU each time there is an SSB burst transmission. On the other hand, considering SSB transmission in SBFD symbols may improve the latency even in initial access or cell reselection procedures due to availability of resources for sending PRACH preamble and random-access procedures.

Observation 5. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

In RAN1 #117, following agreement was made:

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Based on this agreement, in case an SSB transmission overlaps with an SBFD symbol, the SBFD symbol is converted to a full DL slot. Alternatively, the UE expects to only receive DL within DL usable PRBs and no UL is allowed for the SBFD-aware UE in the corresponding SBFD symbol. One of the main reasons for not transmitting UL in the overlapping SBFD symbols is handling the collision between the UL transmission and SSB reception, as the potential CLI caused by UEs transmitting UL could degrade SSB reception for the other UEs.

The CLI due to UL transmission in UL subbands of SBFD symbols that overlap in time with SSB transmission could also be caused by UL transmission by SBFD-aware UEs in an adjacent cell with SBFD configuration in the same symbol (although the entire network has a common “DXXXU” pattern for all the cells for example). That is, while the SBFD-aware UEs in the serving cell are refrained from UL transmission in the SBFD symbol overlapping with SSB transmission, the UL transmission from SBFD-aware UEs in an adjacent cell could affect SSB reception performance. As an example, see Figure 5, where the UEs in both Cell 1 and Cell 2 are configured commonly with DXXXU SBFD configuration. In Figure 5, Cell1 is configured with SSB transmission in SBFD Slot2, Cell2 is configured with SSB transmission in SBFD Slot 3, and so the UEs in Cell2 are refrained from UL transmission in Slot 3. On the other hand, the UEs in Cell1 may perform UL transmissions in Slot 3, as there are no SSB transmission from their serving Cell1 in Slot3, which may cause CLI and degradation in SSB reception for UEs in Cell2.

Figure 5. Issue on neighbor-cell’s SSB slot to be considered for the ‘D’ slot fallback on UEs in serving-cell

Observation 6. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

To mitigate the CLI and potential degradation in SSB reception in a first cell, caused by UL transmission from the SBFD-aware UEs in an adjacent second cell, the respective UEs can explicitly receive information on additional neighbor-cell(s) with their SSB transmission slot information to additionally apply the ‘D’ slot fallback, or could verify some conditions before UL transmission in the SBFD symbols that overlap in time with SSBs from another cell. For example, in Figure 5, UE1 that is closer to the center of Cell1 and far from Cell2 can still transmit UL in the SBFD symbols that overlap in time with Slot3, where SSB is transmitted in Cell2. On the other hand, in Figure 5, UE2 that is far from the center of Cell1 and close to Cell2 cannot transmit UL in Slot3, where SSB is transmitted in Cell2.

Proposal 3. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

## On extending SBFD beyond Rel-19 for flexibility and scalability

SBFD operation in Rel-19 is based on a strict assumption on semi-statically aligned time/frequency locations of DL/UL subbands in the network, mainly due to challenges of CLI handling [3]. However, in reality, traffic conditions are varying among cells in the network, and semi-statically fixed DL/UL resource partitioning may result in significantly reduced resource utilization efficiency. For example, even in a low-load scenario, the SBFD in Rel-19 does not support a dynamic fallback to a legacy downlink slot opportunistically which results in a DL performance degradation.

In this section, we provide an initial analysis on extending SBFD beyond Rel-19 for flexibility and scalability. For the system-level simulations we focus on an outdoor UMa scenario that utilizes a clustered dropping of UEs. The simulation assumptions are based on those provided in section 7 of [4]. We considered three  different FD schemes:

- Scheme 1: Semi-static SBFD (Rel-19) with 20% UL subband resource in ‘X’ slot, with frame structure (XXXXU)

- Scheme 2: Flexible/Dynamic FD operation with adaptation where ‘X’ slot in (XXXXU) may fallback to a full DL slot

- Scheme 3: Flexible/Dynamic FD operation with adaptation where ‘X’ slot in (XXXXU) may fallback to a full DL slot, full UL slot, or ‘X’ slot with 60% UL subband (as opposed to the default 20% UL subband).

We compare the relative DL and UL performance of the schemes based on mean and 5%-ile UPT and latency. Figure 6 provides the relative performance gain/loss of mean and 5%-ile DL and UL UPT for Scheme 2 relative to Scheme 1. Figure 7 provides the relative performance gain/loss of mean and 5%-ile DL latency and UL latency for the same two schemes. From the figures we observe that allowing for a fall back to full DL slot, results in considerable performance improvement in both mean and 5%-ile DL UPT (Figure 6) and latency (Figure 7) at low and medium loads – mean UPT gain is 19% (low load) and ~15% at medium load, whereas 5%-ile UPT gain is 12% at low load and ~10% at medium load; mean latency gain is 13% at low load and ~5% at medium load, whereas 5%-ile latency improves by 15% at both low and medium loads. However, Scheme 2 doesn’t perform as well as Scheme 1 for UL UPT (Figure 6) and latency (Figure 7) – mean UL UPT for Scheme 2 is comparable to Scheme 1 for low and medium loads, with a 4% loss at high load. For 5%-ile UL UPT Scheme 2 shows losses of between 5-10% depending on load. Losses are also observed for mean and 5%-ile latency, with losses of up to 8% for mean UL latency, and losses of between 3.5-7% for 5%-ile latency depending on load. We observe that utilizing a dynamic/flexible FD scheme with fallback to DL only slot can improve DL performance at the cost of some losses for UL performance.

Figure 8 provides the relative performance gain/loss of mean and 5%-ile DL and UL UPT for Scheme 3 relative to Scheme 1. Figure 9 provides the relative performance gain/loss of mean and 5%-ile DL latency and UL latency for the same two schemes. From Figure 8 we observe improved DL UPT performance for Scheme 3 over Scheme 1 for low (14% for mean UPT, 10% for 5%-ile) and medium load (6% for mean, 4% for 5%-ile), with losses for high load. From Figure 9 we observe improved DL latency for both low and medium loads with higher gains for lower loads and for 5%-ile latency vs. mean latency. For UL UPT (in Figure 8) we observe significant improvement in UL mean and 5%-ile UPT with gains ranging from 175% to 30% for mean UPT, and gains ranging from 38% to 28% when going from low to high loads. We also observe gains for UL mean latency with gains of 39% for low load and ~5% for high loads, whereas gains for 5%-ile UL latency ranges from 50% to 34% for low and high loads respectively. Flexible/Dynamic FD (Scheme 3) offers much more significant gains for UL UPT and latency (when compared to Scheme 2), while still maintaining gains for DL UPT and latency for low and medium loads (albeit slightly lower than those observed for Scheme 2).

Figure 6: DL and UL mean and 5%-ile UPT gains for different traffic loads (Scheme 2 vs Scheme 1)

Figure 7: DL and UL mean and 5%-ile latency gains for different traffic loads (Scheme 2 vs Scheme 1)

Figure 8: DL and UL mean and 5%-ile UPT gains for different traffic loads (Scheme 3 vs Scheme 1)

Figure 9: DL and UL mean and 5%-ile latency gains for different traffic loads (Scheme 3 vs Scheme 1)

Observation 7. Flexible adaptation (for e.g., dynamic fallback of SBFD slots) for SBFD can result in improved performance over semi-statically fixed DL/UL resource configuration.

Proposal 4. Further study on how to enhance SBFD performance beyond Rel-19 for improved flexibility and scalability, in consideration of dynamic and future-proof full duplex design approach.

# Conclusion

In this contribution, we discussed issues on SBFD Tx, Rx, measurement procedures, including SBFD operation with CA, implicit guard periods between different symbol types, SBFD operation in SSB symbols, and extending SBFD beyond Rel-19 for flexibility and scalability. From the discussions, we made following observations and proposals:

[2.1	SBFD operation with CA]

Observation 1. The agreement on supporting SBFD operation on one TDD carrier implies that the Rel-19 SBFD feature is limited to one carrier among multiple carriers if the UE is configured with CA.

Proposal 1. In consideration of such limited scope and timeframe in Rel-19, it is not justified to further optimize UE behaviors in combination with the half-duplex CA case, where the UE does not expect to be scheduled simultaneously with SBFD operation which violates the half-duplex CA constraint.

[2.2	On implicit quard periods between different symbol types]

Observation 2. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Observation 3. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 4. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 2. Support implicit guard periods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

[2.3	SBFD operation in SSB symbols]

Observation 5. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

Observation 6. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

Proposal 3. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

[2.4	On extending SBFD beyond Rel-19 for flexibility and scalability]

Observation 7. Flexible adaptation (for e.g., dynamic fallback of SBFD slots) for SBFD can result in improved performance over semi-statically fixed DL/UL resource configuration.

Proposal 4. Further study on how to enhance SBFD performance beyond Rel-19 for improved flexibility and scalability, in consideration of dynamic and future-proof full duplex design approach.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

“Report of 3GPP TSG RAN WG1 #119”, 3GPP RAN1 Meeting #119, November 2024.

TR 38.858, “Study on Evolution of NR Duplex Operation”, V18.0.0.

R1-2212993, “Final Summary on evaluation on NR duplex evolution”, Moderator (CMCC).

# Appendix – Simulation assumptions

Table 1. SLS simulation assumptions, based on section 7 of [4]

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor. 20 UEs per cell. UE clustering distribution with 2 UE clusters and 8 UEs per cluster. |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 53 dBm, UE: 23 dBm |
| BS antenna configuration | SBFD: Separate antenna array for Tx/Rx Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): SBFD Alt. 2 - Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration Semi-static (Rel-19) SBFD (Scheme 1): SBFD operation with 20% UL subband resource in ‘X’ slot with (XXXXU). Flexible/Dynamic SBFD (Scheme 2): Flexible/Dynamic SBFD operation where ‘X’ slot may fallback to full ‘DL’ slot. Flexible/Dynamic SBFD (Scheme 3): Flexible/Dynamic SBFD operation where ‘X’ slot may fallback to full ‘DL’ slot, full ‘UL’ slot, or ‘X’ slot with 60% UL subband resource in ‘X’ slot. |
| gNB self-interference | 1 dB UL desense |
| CO-site inter sector inter SB CLI | Option 2: 93 dB (spatial isolation), 0 dB digital isolation |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB for DL, 0.125 MB for UL) Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered Load scenario Low load Type-2 RU < 10%) Medium load Type-2 RU 20%-35% High load Type-2 RU >50% |