3GPP TSG RAN WG1 #120bis		R1-2502002

Wuhan, China, April 7th – 11th, 2025

Source:	CATT, CICTCI

Title:	Discussion on ISAC deployment scenarios

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

Integrated sensing and communication (ISAC) is widely known as a key capability for 5G-A and 6G communication system to facilitate the commercial use in vertical industries. Recently, a list of promising ISAC use cases was provided by SA1 [1]. Therefore, it is important to establish a solid channel modelling framework to enable evaluation of sensing techniques. In [2], a Rel-19 study item on channel modelling for ISAC was approved and then revised, focusing on object detection and/or tracking. The following sensing objects are selected as the example target use cases:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

A total number of six sensing modes (TRP-to-TRP bi-static, TRP monostatic, TRP-to-UE bi-static, UE-to-TRP bi-static, UE-to-UE bi-static, UE monostatic) are to be studied. The primary frequency range is 0.5 to 52.6 GHz, with the assumption that the modelling approach should scale to 100 GHz if feasible.

In this contribution, we discuss the details of ISAC deployment scenarios corresponding to the above use cases based on the progress until RAN1#120.

Discussion

Simulation assumptions for large scale calibration

UAV

The following agreements were reached in RAN1#120 [15].

Agreement(RAN1#120) For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for large scale calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. Component A of the RCS for each scattering point; a fixed value of A. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Metrics; Coupling loss (based on LOS pathloss) - FFS: how to select sensing Tx and Rx FFS: additional metrics, wideband SIR and SINR based on RSRP if interference is modelled..

For UAV sensing scenario, some additional necessary parameters need to be declared. The following parameters are proposed by FL below in Table 1.

Table 1: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m (horizontal) |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Definition of Coupling loss | power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N=[1], 4 Tx-Rx pairs to be selected for the target. NOTE1: TRP mono-static: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. NOTE2: TRP-TRP bistatic: Total: C(19,2) =19*18/2 Tx-Rx pairs Based on Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss (based on LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - For the combined channel, the ratio of the target channel energy to the environment/clutter energy can be used. - The metric SIR can be defined as the power received via the sensing target channel to the power received from the environment/background channel |

For the metric coupling loss, if the CDFs have been separately generated and calibrated for target channel and background channel, the CDF for combined channel can be omitted. In this way, it is not necessary to unify the ratio of the target channel energy to the environment/clutter energy. The CDF for combined channel can be predicted from the CDFs for target channel and background channel.

For the metric SIR, the issue is what the interference contains. The possible sources of interference are as follows:

- I1: background channel of the same Tx-Rx pair

- I2: background channels of other Tx-Rx pairs with same Rx node

- I2A: target channels of other Tx-Rx pairs with same Rx node

- I3: background channels from other Tx nodes in Tx-Rx pairs with different Rx nodes

- I3A: target channels from other Tx nodes in Tx-Rx pairs with different Rx nodes

The sources of interference are closely related to resource allocation, and it is also assumed that simultaneous reception and transmission, so it is recommended that SIR can be ignored for calibration.

The proposed modifications for large scale calibration for UAV sensing targets are shown in Table 2 and the update is marked in red.

Table 2: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m (horizontal) |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Definition of Coupling loss | power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N=[1], 4 Tx-Rx pairs to be selected for the target. NOTE1: TRP mono-static: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. NOTE2: TRP-TRP bistatic: Total: C(19,2) =19*18/2 Tx-Rx pairs Based on Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss (based on LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - For the combined channel, the ratio of the target channel energy to the environment/clutter energy can be used. - The metric SIR can be defined as the power received via the sensing target channel to the power received from the environment/background channel |

Proposal 1: For UAV sensing scenario, support large scale calibration parameters in Table 2.

Automotive vehicles

The following agreement was reached in RAN1#119 for automotive sensing scenario [14].

Agreement For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Automotive Details on ISAC-Automotive scenarios are listed in Table x. Table x. Evaluation parameters for Automotive sensing scenarios Parameters; Values. Applicable communication scenarios; Highway, Urban Grid. NOTE1. Sensing transmitters and receivers properties; Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m. Sensing target; LOS/NLOS; LOS and NLOS (including NLOSv). Outdoor/indoor; Outdoor. Mobility (horizontal plane only); Based on TR37.885 mobility for urban grid or highway scenario. Distribution (horizontal); Based on dropping in TR37.885 per urban grid or highway communication scenario. Orientation; Lane direction in horizontal plane. Physical characteristics (e.g., size); Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc.. NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

In the agreed table, the only FFS is about environment objects:

- Number of buildings in Urban grid

For evaluation, the number of buildings should follow the road configuration and warp around rule, as shown in Figure 1 and Figure 2, respectively. Therefore, for a 7-cell layout with ISD=500m, 14 buildings may be modeled.

Figure 1: Road configuration for urban grid (Figure A-1 in TR 37.885)

Figure 2: Wrap around model for urban case (Figure A.1.3-1 in TR 36.885)

For calibration, we can consider reducing the number of buildings for simplicity. For example, we can consider that only 9 buildings are modeled in a 7-cell layout, i.e., 9 road grids as configured in TR 37.885. Regarding the sensing target distribution, one target uniformly distributed (across multiple drops) within the center road grid is sufficient for calibration, since all the cases can be evaluated by considering the impact of all the nine buildings as EO.

- Shifting BS or road grid

As shown in Figure 2, in conventional layout, BSs are placed on the top central of the buildings. When walls (EO type-2) are modeled, the LOS probability between a BS and a vehicle may be quite low (depending on the progress on LOS condition determination in Agenda 9.7.2). A more reasonable and realistic assumption is that the BSs are placed at the corner of the top of buildings. Therefore, the road grids shall be shifted by (ISD3-10,32*ISD3-10) m in horizontal plain, or equivalently, the BSs are shifted by (-ISD3+10,-32*ISD3+10) m in horizontal plain. Figure 3 illustrates how the road grids are shifted for the suggested deployment.

Figure 3: Shifting BSs onto the corner of the top of buildings

- Number of walls to be modelled.

It is straightforward that 4 walls should be modeled per building.

Note that, for a specific sensing target, not all the walls of a building will have impact on sensing. Only if the specular point between sensing target and Tx/Rx falls in a wall’s surface (a finite area, i.e. 413m x 20m, or 230 x 20m, with certain location and orientation), the wall will impact sensing in target channel. Otherwise, from simulation point of view, this wall can be ignored at least for target channel.

- Additional building size.

Currently we do not see strong need to consider model additional building size. A small size building will not cause significant impact in sensing since the specular reflection condition at a small wall is difficult to meet. To sensing, impact from small size building can be considered at part of clutters.

Proposal 2: For Automotive sensing scenario, for the EO type-2 in Urban Grid, when modelled,

- The number of buildings follows the road configuration and warp around rule in TR 37.885/36.885; for calibration, the number of buildings can be reduced for simplicity, e.g. to 9 (for a 7-cell layout with ISD=500m);

- Consider shifting the road grids by (ISD3-10,32*ISD3-10)  m and thus BSs are placed at the corner of the top of buildings;

- 4 walls per building are modelled. The impact of a wall is ignored if specular reflection condition is not met at the surface of the wall;

- No need to introduce additional building size.

For the purposes of large scale calibration for Automotive sensing targets, the following calibration parameters are proposed by FL below in Table 3.

Table 3: Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | Urban Grid (ISD=250m, BS height=25m) Highway (ISD=1732m, BS height=35m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic |
| Target type | Vehicle type 2 [37.885] |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | Per 37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. |
| Component A of the RCS for each scattering point | The mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10m |
| Wrapping Method | As defined in urban grid/highway scenario |
| Definition of Coupling loss | power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 1 NOTE1: TRP mono-static: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. NOTE2: TRP-TRP bistatic: Total: Tx-Rx pairs dependent on scenario. Based on Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss (based on LOS pathloss) - CDFs can be separately generated for target channel, background channel, CDFs can be separately generated for target channel, background channel, and combined channel - For the combined channel, the ratio of the target channel energy to the environment/clutter energy can be used. - The metric SIR can be defined as the power received via the sensing target channel to the power received from the environment/background channel |

For Automotive sensing scenarios, there are some proposed modifications:

- For Urban Grid scenario, ISD=250m is an additional option, so it is not recommended to use for calibration. We suggest to use ISD=500m (in TR 37.885) for calibration instead.

- For Highway scenario, ISD=500m can be an additional option, which is also supported in TR 37.885.

- For sensing mode, the TRP-UE bi-static and UE-UE bi-static can be added and RSU-type UE can be used for Tx/Rx point. The deployment of RSU-type UE follows the description in clause A.1.3 in TR 36.885, i.e., deploying at the centre of intersection in urban grid scenario and uniformly deploying in the middle of the freeway with 100m spacing in highway scenario.

- For sensing target distribution, dropping one target is sufficient for calibration as already determined for UAV scenario. For highway scenario, one target can be uniformly distributed (across multiple drops) within the simulation region (i.e., two cells). For urban grid scenario, one target is uniformly distributed (across multiple drops) within the centre road grid. Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally is for Highway scenario. For Urban Grid, the vehicle speed is 60 km/h in all the lanes as baseline and 15 km/h in all the lanes optionally.

- For metrics, the modification is consistent with that of the UAV which is shown in 2.1.1.

The proposed modifications for large scale calibration for Automotive sensing targets are shown in Table 4 and the update is marked in red.

Table 4: Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | Urban Grid (ISD=250mISD=500m, BS height=25m) Highway (ISD=1732m and 500m, BS height=35m) |
| Sensing mode | TRP monostatic, TRP-TRP bi-static Urban Grid (TRP monostatic, TRP-TRP bi-static, TRP-UE bi-static, UE-UE bi-static, RSU-type UE can be used for Tx/Rx point) Highway (TRP monostatic, TRP-TRP bi-static, TRP-UE bi-static, UE-UE bi-static, RSU-type UE can be used for Tx/Rx point) |
| Target type | Vehicle type 2 [37.885] |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | Per 37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline and 15 km/h in all the lanes optionally. |
| Component A of the RCS for each scattering point | The mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10m |
| Wrapping Method | As defined in urban grid/highway scenario |
| Definition of Coupling loss | power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 1 NOTE1: TRP mono-static: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. NOTE2: TRP-TRP bistatic: Total: Tx-Rx pairs dependent on scenario. Based on Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss (based on LOS pathloss) - CDFs can be separately generated for target channel, background channel, CDFs can be separately generated for target channel, background channel, and combined channel - For the combined channel, the ratio of the target channel energy to the environment/clutter energy can be used. - The metric SIR can be defined as the power received via the sensing target channel to the power received from the environment/background channel |

Proposal 3: For Automotive sensing scenarios support large scale calibration parameters in Table 4.

Simulation assumptions for full scale calibration

UAV

For the purposes of full scale calibration for UAV targets, the following calibration parameters are proposed by FL below in Table 5.

Table 5: Simulation assumptions for full calibration for UAV sensing targets

| Parameters |  | Values |
| --- | --- | --- |
| Scenario |  | UMa-AV |
| Sensing mode |  | TRP monostatic, TRP-TRP bistatic |
| Target type |  | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization |  | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency |  | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations |  | Single dual-pol isotropic antenna |
| BS Tx power |  | FR1: 56dBm FR2: 41dBm |
| Bandwidth |  | FR1: 100MHz FR2: 400MHz |
| BS noise figure |  | FR1: 5dB FR2: 7dB |
| UT antenna configurations |  | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure |  | FR1: 9dB FR2: 10dB |
| Sensing target distribution |  | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Fast fading model |  | 36.777 Annex B.1.3 |
| (u, std) for XPR of target |  | (17, 3.4) |
| Component B1/B2 of bistatic RCS |  | Same values as monostatic (i.e., we temporally assume bistatic RCS is isotropic). FFS: Values that are not re-used from monostatic sensing |
| The power threshold for path dropping after concatenation for target channel |  | [-40dB] FFS: Other power thresholds. |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel |  | [-25dB] FFS: Other power thresholds. |
| Definition of Coupling loss |  | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Tx-Rx pairing |  | Select [1, 4] Tx-Rx pairs with smallest power scaling factor. |
| Absolute delay |  | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref]. |
| Metrics | CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) | Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996 |
|  | Curve of CDF | Coupling loss (based on LOS+LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - CDF of per-cluster coupling loss for the background channel |

For the metric coupling loss, as with large-scale calibration, if the CDFs have been separately generated and calibrated for target channel and background channel, the CDF for combined channel can be omitted. In this way, it is not necessary to unify the ratio of the target channel energy to the environment/clutter energy. The CDF for combined channel can be predicted from the CDFs for target channel and background channel.

In addition, considering the definition of per-cluster coupling loss is unclear and it isn’t calibrated in legacy communication, the CDF of per-cluster coupling loss is suggested to be removed.

The proposed modifications for full calibration for UAV sensing targets are shown in Table 6 and the update is marked in red.

Table 6: Simulation assumptions for full calibration for UAV sensing targets

| Parameters |  | Values |
| --- | --- | --- |
| Scenario |  | UMa-AV |
| Sensing mode |  | TRP monostatic, TRP-TRP bistatic |
| Target type |  | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization |  | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency |  | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations |  | Single dual-pol isotropic antenna |
| BS Tx power |  | FR1: 56dBm FR2: 41dBm |
| Bandwidth |  | FR1: 100MHz FR2: 400MHz |
| BS noise figure |  | FR1: 5dB FR2: 7dB |
| UT antenna configurations |  | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure |  | FR1: 9dB FR2: 10dB |
| Sensing target distribution |  | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Fast fading model |  | 36.777 Annex B.1.3 |
| (u, std) for XPR of target |  | (17, 3.4) |
| Component B1/B2 of bistatic RCS |  | Same values as monostatic (i.e., we temporally assume bistatic RCS is isotropic). FFS: Values that are not re-used from monostatic sensing |
| The power threshold for path dropping after concatenation for target channel |  | [-40dB] FFS: Other power thresholds. |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel |  | [-25dB] FFS: Other power thresholds. |
| Definition of Coupling loss |  | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Tx-Rx pairing |  | Select [1, 4] Tx-Rx pairs with smallest power scaling factor. |
| Absolute delay |  | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref]. |
| Metrics | CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) | Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996 |
|  | Curve of CDF | Coupling loss (based on LOS+LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - CDF of per-cluster coupling loss for the background channel |

Proposal 4: For UAV sensing scenario, support full scale calibration parameters in Table 6.

Automotive vehicles

For the purposes of full scale calibration for Automotive sensing targets, the following calibration parameters are proposed by FL below in Table 7.

Table 7: Simulation assumptions for full calibration for Automotive sensing targets

| Parameters |  | Values |
| --- | --- | --- |
| Scenario |  | Urban Grid (ISD=250m, BS height=25m) Highway (ISD=1732m, BS height=35m) |
| Sensing mode |  | TRP monostatic, TRP-TRP bistatic |
| Target type |  | Vehicle type 2 [37.885] |
| Sectorization |  | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency |  | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations |  | Single dual-pol isotropic antenna |
| BS Tx power |  | FR1: 56dBm FR2: 41dBm |
| Bandwidth |  | FR1: 100MHz FR2: 400MHz |
| BS noise figure |  | FR1: 5dB FR2: 7dB |
| UT antenna configurations |  | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure |  | FR1: 9dB FR2: 10dB |
| Sensing target distribution |  | 37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. |
| Fast fading model |  | Procedures based on 37.885 |
| (u, std) for XPR of target |  | [FFS] |
| Component B1/B2 of bistatic RCS |  | Same values as monostatic (i.e., we temporally assume bistatic RCS is isotropic). FFS: Values that are not re-used from monostatic sensing |
| The power threshold for path dropping after concatenation for target channel |  | [-40dB] FFS: Other power thresholds. |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel |  | [-25dB] FFS: Other power thresholds. |
| Definition of Coupling loss |  | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Tx-Rx pairing |  | Select [1, 4] Tx-Rx pairs with smallest power scaling factor. |
| Absolute delay |  | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref]. |
| Metrics | CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) | Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996 |
|  | Curve of CDF | Coupling loss (based on LOS+LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - CDF of per-cluster coupling loss for the background channel |

For the Automotive sensing scenario, our view is the same as in Section 2.1.2. In short:

- For Urban Grid scenario, ISD=250m is an additional option, so it is not recommended to use for calibration. We suggest to use ISD=500m (in TR 37.885) for calibration instead.

- For Highway scenario, ISD=500m can be an additional option, which is also supported in TR 37.885.

- For sensing mode, the TRP-UE bi-static and UE-UE bi-static can be added and RSU-type UE can be used for Tx/Rx point. The deployment of RSU-type UE follows the description in clause A.1.3 in TR 36.885, i.e., deploying at the centre of intersection in urban grid scenario and uniformly deploying in the middle of the freeway with 100m spacing in highway scenario.

- For sensing target distribution, dropping one target is sufficient for calibration as already determined for UAV scenario. For highway scenario, one target can be uniformly distributed (across multiple drops) within the simulation region (i.e., two cells). For urban grid scenario, one target is uniformly distributed (across multiple drops) within the centre road grid. Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally is for Highway scenario. For Urban Grid, the vehicle speed is 60 km/h in all the lanes as baseline and 15 km/h in all the lanes optionally.

- For metrics, the modification is consistent with that of the UAV which is shown in 2.2.1.

The proposed modifications for full calibration for Automotive sensing targets are shown in Table 8 and the update is marked in red.

Table 8: Simulation assumptions for full calibration for Automotive sensing targets

| Parameters |  | Values |
| --- | --- | --- |
| Scenario |  | Urban Grid (ISD=250mISD=500m, BS height=25m) Highway (ISD=1732m and 500m,, BS height=35m) |
| Sensing mode |  | TRP monostatic, TRP-TRP bi-static Urban Grid (TRP monostatic, TRP-TRP bi-static, TRP-UE bi-static, UE-UE bi-static, RSU-type UE can be used for Tx/Rx point) Highway (TRP monostatic, TRP-TRP bi-static, TRP-UE bi-static, UE-UE bi-static, RSU-type UE can be used for Tx/Rx point) |
| Target type |  | Vehicle type 2 [37.885] |
| Sectorization |  | 3 sectors per cell site: 30, 150 and 270 degrees For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency |  | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations |  | Single dual-pol isotropic antenna |
| BS Tx power |  | FR1: 56dBm FR2: 41dBm |
| Bandwidth |  | FR1: 100MHz FR2: 400MHz |
| BS noise figure |  | FR1: 5dB FR2: 7dB |
| UT antenna configurations |  | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure |  | FR1: 9dB FR2: 10dB |
| Sensing target distribution |  | 37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline and 15 km/h in all the lanes optionally. |
| Fast fading model |  | Procedures based on 37.885 |
| (u, std) for XPR of target |  | [FFS] |
| Component B1/B2 of bistatic RCS |  | Same values as monostatic (i.e., we temporally assume bistatic RCS is isotropic). FFS: Values that are not re-used from monostatic sensing |
| The power threshold for path dropping after concatenation for target channel |  | [-40dB] FFS: Other power thresholds. |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel |  | [-25dB] FFS: Other power thresholds. |
| Definition of Coupling loss |  | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Tx-Rx pairing |  | Select [1, 4] Tx-Rx pairs with smallest power scaling factor. |
| Absolute delay |  | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref]. |
| Metrics | CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) | Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996 |
|  | Curve of CDF | Coupling loss (based on LOS+LOS pathloss) - CDFs can be separately generated for target channel, background channel, and combined channel - CDF of per-cluster coupling loss for the background channel |

Proposal 4: For Automotive sensing scenario, support full scale calibration parameters in Table 8.

Simulation results for Large scale Calibration

UAV of TRP monostatic

For large scale calibration for UAV of TRP mono-static, the CDF curves of Coupling Loss for FR1/FR2 are shown in Figure 4 and.

Figure 4: CDF curves of Coupling Loss for TRP mono-static (a) FR1 (left), (b) FR2 (right)

UAV of TRP-TRP bi-static

For large scale calibration for UAV of TRP-TRP bi-static, the CDF curves of Coupling Loss for FR1/FR2 are shown in Figure 5.

Figure 5: CDF curves of Coupling Loss for TRP-TRP bi-static (a) FR1 (left), (b) FR2 (right)

Comparison of different cell sectorization methods

During the email discussion, two cell sectorization methods are considered for calibration. One is to deploy 3 sectors per cell site (30, 150 and 270 degrees), as shown in Figure 6 (a). The other is to assume a single 360-degree sector, illustrated in Figure 6 (b). In this section, the impact of different cell sectorization methods is evaluated.

Figure 6: (a) Option 1: 3 sectors per cell site (left), (b) Option 2: a single 360-degree sector (right)

The CDF curves of Coupling Loss for FR1 with different cell sectorization methods are shown in Figure 9. It can be observed that no matter which option is adopted, the distribution of coupling loss is consistent. Different cell sectorization methods have no impact on the result of large scale calibration.

Figure 7: CDF curves of Coupling Loss for FR1 with different cell sectorization methods (Option 1: 3 sectors per cell site, Option 2: a single 360-degree sector)

Observation 1: Different cell sectorization methods have no impact on the result of large scale calibration.

Simulation results for Full Calibration

For full calibration for UAV of TRP mono-static and TRP-TRP bi-static, the CDF curves of Coupling Loss for FR1 of target channel are shown in Figure 8.

Figure 8: CDF curves of Coupling Loss for FR1 of target channel

Conclusion

In this contribution, we discuss the detailed scenarios of each use case for ISAC. The observation and proposals are summarized as follows:

Observation 1: Different cell sectorization methods have no impact on the result of large scale calibration.

Proposal 1: For UAV sensing scenarios, support large scale calibration parameters in Table 2.

Proposal 2: For Automotive sensing scenarios, for the EO type-2 in Urban Grid, when modelled,

- The number of buildings follows the road configuration and warp around rule in TR 37.885/36.885; for calibration, the number of buildings can be reduced for simplicity, e.g. to 9 (for a 7-cell layout with ISD=500m);

- Consider shifting the road grids by (ISD3-10,32*ISD3-10)  m and thus BSs are placed at the corner of the top of buildings;

- 4 walls per building are modelled. The impact of a wall is ignored if specular reflection condition is not met at the surface of the wall;

- No need to introduce additional building size.

Proposal 3: For Automotive sensing scenarios, support large scale calibration parameters in Table 4.

Proposal 4: For UAV sensing scenarios, support full scale calibration parameters in Table 6.

Proposal 5: For Automotive sensing scenarios, support full scale calibration parameters in Table 8.

References

- TR 22.837, Feasibility Study on Integrated Sensing and Communication, v19.0.0.

- RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, RAN#105, Melbourne, Australia, September 9th – 12th, 2024.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, v18.0.0.

- TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles, v15.0.0.

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR, v15.3.0.

- TR 38.802, Study on New Radio Access Technology Physical Layer Aspects, v14.2.0.

- TR 36.885, Study on LTE-based V2X services, v14.0.0.

- TR 38.859, Study on expanded and improved NR positioning, v18.0.0.

- Chairman’s Notes RAN1#116, Athens, Greece, February 26th – March 1st, 2024.

- Chairman’s Notes RAN1#116bis, Changsha, China, April 15th – 19th, 2024.

- Chairman’s Notes RAN1#117, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman’s Notes RAN1#118, Maastricht, NL, August 18th – 23rd, 2024.

- Chairman’s Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- Chairman’s Notes RAN1#119, Orlando, the USA, November 18th – 22nd, 2024.

- Chairman’s Notes RAN1#120, Greece, February 17th – 21st, 2025