3GPP TSG RAN WG1 #120bis		                                                      R1-2502004

Wuhan, China, April 7th – 11st, 2025

Source:	CATT

Title:	On channel model validation for 7-24GHz

Agenda Item:	9.8.1

Document for:	Discussion and Decision

- Introduction

In RAN1#119 meeting and RAN1#120 meeting, several details on the channel model validation have been discussed [1] [2].

In this contribution, our views on ISD assumption for suburban scenario, the number of clusters, and intra-cluster power profile models are provided.

- Discussion

  - Suburban scenario

In RAN1#119 meeting, the following agreement on ISD assumption for suburban scenario was achieved [1].

Agreement For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes: - Option 1) ISD = 1299 m - Option 2) ISD = 1732 m Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

One of the remaining issues for suburban scenario is the determination of ISD assumption. According to the agreement, two options are listed for ISD, and further down-selection is needed. Similar to other parameters in suburban scenario (e.g., BS height and minimum BS to UT distance in 2D), the assumption defined in ITU-R M.2135 [3] can be used as reference. Therefore, 1299m (i.e., Option 1) can be adopted for suburban scenario for calibration purposes.

Proposal 1: For Suburban scenario, the following option is supported for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m.

  - The number of clusters

In RAN1#120 meeting, the following agreement on the number of clusters was achieved [2].

Agreement - Conclude in RAN1 #120bis among the following: - Alt 1) - RAN1 has identified that number of clusters spread for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] scenario is smaller compared to delay spread in the TR at least for 6-24 GHz frequency range. Further discuss, necessary changes for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS]. - RAN1 to determine which scenarios among [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] requires necessary changes. - Data samples for discussion on necessary changes Scenario; InH @10 GHz; Umi @10 GHz; UMa @6.5 GHz; UMa @13 GHz; UMa @15GHz. TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Number of clusters; 15; 19; 10; 11; 12; 19; 5; 7; 12; 20; 13; 17; 12; 20; 11; 15; 12; 20; 7; 13. - Proponent companies to provide detailed information on potentially necessary changes to number of clusters for scenarios in question. - Alt 2) - For the following scenarios, while sources have observed smaller number of clusters for various deployment scenario from measurement taken for 6 – 24 GHz frequency range, due to impact on frequency continuity outside 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update number of clusters for all existing scenarios.

According to the measurement results in [4], the number of clusters for LOS and NLOS indoor scenario at 10GHz is 10 and 11, respectively, which is smaller than the model in TR38.901 (15 for LOS, and 19 for NLOS) [5].  In [6], the number of clusters for Shopping Mall scenario at 15GHz is further reduced (6.33 for LOS, and 6.98 for NLOS). Furthermore, [7] and [8] also show that the measured cluster numbers is smaller than the 3GPP model in 13GHz and 10.25GHz, respectively. Therefore, the number of clusters can be updated for 7-24 GHz.

Proposal 2: Update the number of clusters for 7-24 GHz.

  - Intra-cluster power profile

In previous meetings, whether to introduce intra-cluster power profile was discussed and three examples of ray power ratio were provided. According to the measurement results in [9], there is a dominant ray within each cluster in multiple scenarios, and the dominant ray within the cluster contains most of the power (e.g., 3 times the power of the remaining paths). However, measurement results in [10] show that the median intra-cluster power ratios are -1.1 dB and -2.5 dB for UMi LOS and NLOS scenarios, which means the power of the strongest ray in a cluster is not as dominant as the LOS path. Besides, all the ray power ratio models in these three examples are not verified sufficiently. Considering that the intra-cluster power profile has a large impact on both frequency channel characteristics and system performance, and the necessity cannot be proven by measurement results, we propose not to introduce intra-cluster power profile for 7-24 GHz.

Proposal 3: Do not introduce intra-cluster power profile for 7-24 GHz.

- Conclusions

In this contribution, remaining aspects of channel model involved in the validation for 7-24GHz are discussed. Based on the discussion, the following proposals are made:

Proposal 1: For Suburban scenario, the following option is supported for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m.

Proposal 2: Update the number of clusters for 7-24 GHz.

Proposal 3: Do not introduce intra-cluster power profile for 7-24 GHz.

- References

- 3GPP TSG RAN WG1 Meeting RAN1 #119, Chairman's Notes, November 18th – 22nd, 2024.

- 3GPP TSG RAN WG1 Meeting RAN1 #120, Chairman's Notes, February 17th – 21st, 2025.

- Report ITU-R M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”, 2009.

- R1-2402009, Considerations on the 7-24GHz channel model validation, Huawei, HiSilicon, Tongji University, RAN1#116bis, April 15th – 19th, 2024.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz V17.1.0”, 2023-12.

- Aalto University et al., 5G Channel Model for bands up to 100 GHz, Globecom 2015, Dec. 2015 (http://www.5gworkshops.com/5GCM.html).

- R1-2406744, “Discussion on channel model validation of TR38.901 for 7-24 GHz”, BUPT, Spark NZ Ltd, RAN1#118, August 19th – 23rd, 2024.

- R1-2406393, “New measurement results for TR38.901 channel model validation and adaptation/extension consideration”, Keysight Technologies, RAN1#118, August 19th – 23rd, 2024.

- R1-2410213, “Discussion on channel model validation of TR38.901 for 7-24 GHz”, BUPT, RAN1#119, November 18th – 22nd, 2024.

- R1-2409402, “Considerations on the 7-24GHz channel model validation”, Huawei, HiSilicon, RAN1#119, November 18th – 22nd, 2024.