3GPP TSG RAN WG1 #120bis	R1- 2502126

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.4.4

Source:	Fujitsu

Title:	Discussion on other aspects of Ambient IoT

Document for:	Discussion

Introduction

In Rel-19, as a promising new IoT type device/service, Ambient IoT was studied as a RAN SI. The study conclusions and observations on devices architectures, potential solutions for ambient IoT, etc., were captured in TR38.769 [1]. In RAN#106 meeting, the Ambient IoT was successfully approved as Rel-19 WI [2].

The scope for this sub-agenda is about multiplexing/multiple access, scheduling information and timing relationships. In this contribution, we provide our views about these aspects.

Discussion

Multiplexing/multiple access

Msg.1 related aspects

In RAN-120 meeting, the FDMA with Y≥1 for Msg.1 transmission from multiple devices has been agreed. But the TDMA with X>1 for Msg.1 transmission is still under discussion.

Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. FL3 High Priority Proposal 2.1.1-1b For following according to TR 38.769 for Msg1, if X>1 is supported. - The maximum value of X is 2. - RAN1 targets to decide on the support of X>1 at RAN1#120bis

TDMA with X>1 for Msg.1 transmission should be supported for higher spectrum efficiency. However, the maximal value of X should be limited considering the timing error caused by residual SFO embedded on the device. The maximal value of X is analyzed as follows from the perspective of spectrum efficiency.

It’s assumed that M Msg.1s need to be transmitted from devices. For X=1, M Msg.0s need to be transmitted by the reader to trigger the Msg.1 transmission due to a Msg.0 corresponding to one time domain resource used for a single Msg1 transmission. For X>1, one Msg.0 is required to trigger Msg.1 transmission, and the X-1 guard periods between X time domain resources are required due to the timing error. In our analysis, it’s assumed X=M for TDMA with X>1. Figure 1 and Figure 2 show examples.

Figure 1 A R2D transmission triggers Msg.1 transmission with X=1.

Figure 2 A R2D transmission triggers Msg.1 transmission with X>1.

The length of guard period splitting X>1 Msg.1 transmissions depends on the duration of Msg.1 and X value and is highly impacted by the timing error caused by residual SFO on the devices. Assuming 10% residual SFO, the length of guard period can be derived as following equations.

ti=0.1i*Tmsg1+ti-1+0.1i*Tmsg1+ti-1+ti

ti=0.22i*Tmsg1+ti-1

The time domain resource required for the scheme supporting X>1 and X=1 is compared in the following table. For fair comparison, for X=1, the time intervals of the TR2D and TD2R should include the margin for timing error caused by residual SFO on device.

Table 1: Comparison of time domain resources required for the scheme supporting X>1 and X=1

| Parameters | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Data rate for R2D and D2R | 5kbps |  |  |  |
| TBS for Msg.0 | 4bits |  |  |  |
| TBS for Msg.1 | 16bits |  |  |  |
| TR2D | 240us |  |  |  |
| TD2R | 780 us |  |  |  |
|  | M=2 | M=3 | M=4 | M=5 |
| Required time domain resource for X=1 | 9.3ms | 14.3ms | 19.3ms | 24.5ms |
| Required time domain resources for X>1 assuming X=M | 8.1ms | 13ms | 18.6ms | 25.1ms |

Note:

TR2D is assumed to be 200us Tprocessing_delay and 0.2* Tprocessing_dealy for margin of time error.

TD2R is assumed to be 100us Tprocessing_delay and 0.2*(TR2D +Tmsg.1) for margin of time error.

From the above table, the time domain resource required by X/M=2,3 is about 10% less than that required by X=1. In that case, the spectrum efficiency for X/M=2,3 is higher than that of X=1. For X/M = 4, the required time domain resource is comparable with that required X=1. However, the time domain resource required by X/M=5 is even larger than that required by X=1.

Observation 1:

Assuming that M Msg.1s need to be transmitted,

- Required transmissions/resources:

  - In the case of X=1, M Msg.0s are required to trigger M Msg.1 transmissions.

  - In the TDMA case of X=M>1, 1 Msg.0 is required to trigger M Msg.1s and M-1 guard periods are required to split the M Msg.1 transmissions.

- The spectrum efficiency of the scheme supporting X=2,3 is higher than that of X=1.

  - The total time domain resource required by X/M=2 or 3 is about 10% less than that required by X=1.

  - The total time domain resource required by X/M=4 is comparable with that required by X=1.

  - The total time domain resource required by X/M=5 is even larger than that required by X=1.

Proposal 1:

Support TDMA-based transmission of Msg.1, X is no larger than 2 or 3.

To support the TDMA based Msg.1 transmission, the Msg.1 time domain resource should be allocated/indicated by the reader via the R2D control information. The information on time domain resources allocation may include the start of the time domain resource and the payload size of Msg.1. To indicate the start of time domain resource, the reference timing can be the end of R2D transmission, e.g., the end of Msg.0. The payload size for Msg.1 transmission may be always fixed, e.g., 16-bits random ID generated by devices. The payload size for Msg.1 should be predefined in specification.

Proposal 2:

To support Msg.1 transmission in TDMA manner, the Msg.1 time domain resource should be allocated/indicated by the reader.

- The start of the Msg.1 time domain resource should be indicated in the control information.

  - The reference timing can be the end of Msg.0 when the start of time domain resource is indicated.

- The payload size of Msg.1 should be fixed and predefined in the specification.

Msg.2 related aspects

In the RAN1-120 meeting, the agreement on Msg.2 transmission was reached as the following agreement.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

According to the above agreement, both separate Msg.2 transmission (option 1) and common Msg.2 transmission (option 2) are supported. How a device monitors the separate Msg.2 and common Msg.2 is an issue to be addressed.

In the case that one Msg.0 triggers more than one Msg.1 transmission, the starting time for monitoring Msg.2 should be common among devices which have transmitted Msg.1 regardless of the common or separate Msg.2 transmission. Regarding the common Msg.2 transmission, it’s straightforward to have a common starting time due to only one Msg.2 is transmitted by the reader. For the separate Msg.2 transmission, if the separate starting time is supported, a device needs to be provided the information about multiple starting times for monitoring Msg.2 which leads to signaling overhead. And additional efforts for standardization on the related signaling design are needed.

Proposal 3:

In the case that one Msg.0 triggers more than one Msg.1 transmission, the starting time for monitoring Msg.2 should be common among devices which have transmitted Msg.1 for both the separate Msg.2 transmission (option 1) and the common Msg.2 transmission (option 2).

In [3], FL proposed a question on whether and how to define the Msg.2 monitoring window for device 1 as the following two figures show examples for the cases of TDMA with X=1 and X>1.

Figure 3 An example of Msg.2 monitoring window for the case of TDMA with X=1.

Figure 4 An example of Msg.2 monitoring window for the case of TDMA with X>1.

Firstly, to accelerate the discussion on Msg.2 monitoring, it needs to clarify the parameters of TD2R_min and TD2R_max are defined from a reader’s perspective or from a device’s perspective. According to Figure 3 and Figure 4, the TD2R_min and TD2R_max seem to be defined from a device’s perspective. From our understanding, the definition of the parameters TD2R_min and TD2R_max should be defined from a reader’s perspective as the discussion in section 2.2.

Observation 2:

To accelerate the discussion on Msg.2 monitoring, clarification on whether the definitions of TD2R_min and TD2R_max are from a reader’s perspective or a device’s perspective is necessary.

Proposal 4:

Clarify/update the definitions of TD2R_min and TD2R_max are from a reader’s perspective.

A monitoring window should be a basic assumption for Msg.2 reception at device side. The benefit of the monitor window is saving the energy for the reception of Msg.2 by a device. For example, if a device transmits a Msg.1 but fails to be detected by the reader, without the definition of Msg.2 monitoring window, the device will monitor the time domain resource until the next Msg.0. The device needs to monitor the additional time domain resources for Msg.3 transmission from other devices which leads to more energy consumption on the reception of Msg.2.

Proposal 5:

A monitoring window should be a basic assumption for Msg.2 reception at the device side which can save energy for device 1 to receive Msg.2.

A Msg.2 monitoring window should be defined with the starting time and time duration. A device starts to monitor Msg.2 at the beginning of the window and continues to monitor Msg.2 within the time duration of the window.

Proposal 6:

A monitoring window for Msg.2 reception should be defined with the starting time and the time duration.

Starting time for Msg.2 monitor window

The potential starting time of Msg.2 monitoring window is analyzed as follows with consideration of the TDMA/FDMA based Msg.1 transmission and the separate or common Msg.2 transmission.

- Regarding the common Msg.2 transmission, only one Msg.2 is transmitted by the reader. Figure 5 shows an example.

  - In the case of TDMA with X=1 or FDMA, the reader guarantees the start of the one Msg.2 in the range of [TD2R_min, TD2R_max] after the end of the time domain resource for Msg.1 transmission. A device can start to monitor Msg.2 no earlier than TD2R_min after the end of the time domain resource for Msg. 1 transmission.

  - In the case of TDMA with X>1, the reader guarantees the start of the one Msg.2 in the range of [TD2R_min, TD2R_max] after the end of the last time domain resource for Msg.1 transmission. A device can start to monitor Msg.2 no earlier than TD2R_min after the end of the last time domain resource for Msg. 1 transmission.

- Regarding the separate Msg.2 transmission, multiple Msg.2s are transmitted by the reader. Figure 6 shows an example.

  - In the case of TDMA with X=1, the reader guarantees the start of the one Msg.2 in the range of [TD2R_min, TD2R_max] after the end of the time domain resource for Msg.1 transmission. A device can start to monitor Msg.2 no earlier than TD2R_min after the end of the time domain resource for Msg. 1 transmission.

  - In the case of TDMA with X>1, the reader guarantees the start of the first Msg.2 in the range of [TD2R_min, TD2R_max] after the end of the last time domain resource for Msg.1 transmission. And the multiple Msg.2s are transmitted by the reader with the time interval TR2D_R2D. A device can start to monitor Msg.2 no earlier than TD2R_min after the end of the last time domain resource for Msg. 1 transmission.

  - In the case of FDMA, the reader guarantees the start of the first Msg.2 in the range of [TD2R_min, TD2R_max] after the end of the time domain resource for Msg.1 transmission. A device can start to monitor Msg.2 no earlier than TD2R_min after the end of the time domain resource for Msg. 1 transmission.

Figure 5 An example of Msg.2 monitoring window for the case of common Msg.2 transmission.

Figure 6 An example of Msg.2 monitoring window for the case of separate Msg.2 transmission.

Proposal 7:

From a device perspective, the window for Msg.2 monitoring starts no earlier than TD2R_min.

- The reference time for determining the start of the window can be the end of time domain resource where Msg. 1 is transmitted for TDMA with X=1 or FDMA.

- The reference time for determining the start of the window can be the end of the last Msg.1 time domain resource among the X Msg.1 time domain resources for TDMA with X>1.

Time duration for Msg.2 monitor window

For the common Msg.2 transmission, the duration of Msg.2 monitoring window needs to take into account the factors: the duration of Msg.2 transmission, the potential uncertainty of the start of Msg.2 transmission. If the size of Msg.2 is variable, the predefined maximal size of Msg.2 can be considered.

For the separate Msg.2 transmission, the duration of the monitoring window depends on the number of TDMA and FDMA resource for Msg.1 transmission. The duration of the monitoring window needs to take into account the factors: the number of TDMA/FDMA resource for Msg.1 transmission, the duration of Msg.2 transmission, the potential uncertainty of the starting time of the first Msg.2 transmission, and the gap between consecutive R2D transmissions, e.g., TR2D_R2D.

A device can determine the duration of Msg.2 monitoring window by either the pre-defined rule or indication in R2D control information. With the consideration of the factors on the duration of Msg.2 monitoring window, a rule can be predefined for devices to derive the duration of Msg.2 monitoring window or the duration of Msg.2 monitoring window is directly indicated in R2D control information.

Proposal 8:

An indication carried by control information or a rule predefined in the specification is necessary for devices to determine the duration of the window for monitoring Msg.2.

- In the case of common Msg.2 transmission, the following factors need to be taken into account on duration of the window:

  - The duration for Msg.2 transmission, and

  - The potential uncertainty of the start of the Msg.2 transmission.

- In the case of separate Msg.2 transmissions, the following factors need to be taken into account on duration of the window:

  - The number of TDMA/FDMA resource for Msg.1 transmissions,

  - The duration for Msg.2 transmission,

  - The potential uncertainty of the starting time of the first Msg.2 transmission, and

  - The gap between two consecutive Msg.2 transmissions, e.g., TR2D_R2D.

Msg.3 related aspects

FDMA Msg.3 transmission from multiple devices has been agreed in RAN-120 meeting as follows. TDMA Msg.3 transmission is still an open issue.

Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

For Msg.3 transmission, the FDMA is considered and TDMA is not supported. Devices transmit permanent IDs, e.g., EPC IDs in response to Msg.2 transmission from the reader. The size of Msg.3 is obviously larger than the size of Msg.1. An obvious accumulated timing error occurs due to the long Msg.3 transmission. In that case, a larger guard period between the adjacent time domain resources for Msg.3 transmission is required to handle timing errors caused by residual SFO if TDMA is supported. The Msg.3 transmission in TDMA manner (X>1) cannot provide the better spectrum efficiency than the scheme of X=1.

Observation 3:

TDMA Msg.3 transmission schemes (X>1) cannot provide better spectrum efficiency than the scheme of X=1.

- The size of Msg.3 is obviously larger than Msg.1. Long Msg.3 transmission can cause more obvious accumulated timing error.

- The more obvious accumulated timing error requires a larger guard period between the adjacent time domain resources for Msg.3 transmission to handle timing errors caused by residual SFO.

Proposal 9:

For Msg.3 transmission,

- FDMA can be considered.

- TDMA is not supported.

Another open issue is about Msg.3 transmission if the separate Msg.2 transmission is supported. In the case of separate Msg.2s transmitted by the reader, two alternatives can be considered for Msg.3 transmissions. The one alternative is the non-interleaving Msg.3 transmissions given in FL summary [3]. The Msg.3s can be transmitted after all Msg.2 transmissions are completed. The other alternative is the interleaving Msg.3 transmission given in FL summary [3]. Each Msg.3 is transmitted right after its corresponding Msg.2. The following figure shows an example.

Figure 7 An example of non-interleaving and interleaving Msg.3 transmission.

Proposal 10:

If separate Msg2 transmission is supported, Msg.3 transmissions could be:

- Alternative 1: Msg.3s can be transmitted after all Msg.2 transmissions are completed. (Non-interleaving Msg.3 transmission given in the FL summary).

- Alternative 2: Each Msg.3 is transmitted right after its corresponding Msg.2. The Msg.2 and Msg.3 of one device are after the Msg.2 and Msg.3 of another device. (Interleaving Msg.3 transmission given in the FL summary.)

The non-interleaving Msg.3 transmission should be adopted if the separated Msg.2 transmission is supported. If the interleaving Msg.3 transmission scheme is supported, the Msg.2 monitoring window could be enlarged because both the Msg.2 transmissions and the Msg.3 transmissions should be counted in the window. A device has to monitor the time sources for Msg.3 transmission from other devices before reaching its own Msg2 response. In this case, the energy consumption of devices may increase due to a longer monitoring period.

Observation 4:

If the interleaving Msg.3 transmission scheme given in the FL summary is supported, the Msg.2 monitoring window could be enlarged because both the Msg.2 transmissions and the Msg.3 transmissions should be counted in the window. In this case, the energy consumption of devices may increase due to a longer monitoring period.

Proposal 11:

Adopt the non-interleaving Msg.3 transmission if the separate Msg.2 transmission scheme is supported.

Timing relationships

The AIoT processing time aspects have been studied in [1] as follows.

TR38.769 6.1.3 timing relationship A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device.

The necessity, function/purpose of the above time intervals on processing time aspects are still open to discuss. In this section, we will provide our views on the AIoT processing time aspects.

Firstly, it needs to be clarified that the time intervals given in TR38.769 section 6.1.3 are defined from a reader’s perspective or a device’s perspective. An AIoT device uses its internal clock to receive R2D signal and transmit D2R signal. A device cannot perceive the timing error due to its low-quality clock. On the contrary, a reader always has high-quality clock and powerful processing capability. The reader can detect and handle the timing error when it receives the D2R signal and schedules R2D transmission. The impact of the timing error should be considered when the time intervals on processing timing are defined, otherwise the potential collision between transmissions from different devices cannot be avoided. In that case, the definition of time intervals should be from a reader’s perspective.

Observation 5:

The definitions of the timing intervals given in TR38.769 section 6.1.3 should also be further clarified/updated, i.e., whether the time intervals are defined from a reader’s perspective or a device’s perspective.

Proposal 12:

The definition of the time intervals on AIoT processing timing should be from a reader’s perspective.

- A device cannot perceive the timing error due to its internal low-quality clock. On the contrary, a reader can detect and handle the timing error.

- The impacts of timing error must be considered when the time intervals are defined, otherwise the potential collision between transmissions from different devices cannot be avoided.

Timing for D2R response to R2D

From a reader’s perspective, the TR2D_min /TR2D_max can be defined as the earliest/latest start of D2R signal arriving at the reader from the end of R2D signal transmitted by reader. The TR2D_min /TR2D_max should take into account the following factors: the processing delay of the device on reception of R2D signal and preparation of D2R transmission, the margin for timing error caused by residual SFO on devices. The value of TR2D_min /TR2D_max can be up to RAN4 discussion. A device can start to transmit D2R signal no earlier than TR2D_min. The reader monitors the start of D2R signal within [TR2D_min, TR2D_max]. If the reader does not observe the start of a D2R signal within [TR2D_min, TR2D_max], it means the device does not receive or execute the command successfully and the reader can transmit the subsequent R2D signal.

Observation 6:

Regarding the necessity and purpose for the time interval between R2D transmission and the subsequent D2R transmission,

- The TR2D_min and TR2D_max are necessary, which can be defined as the earliest and latest start of D2R signal arriving at the reader from the end of R2D signal transmitted by reader.

- The TR2D_min /TR2D_max should take into account the following factors: the processing delay of the device on reception of R2D signal and preparation of D2R transmission, the margin for timing error caused by residual SFO on devices.

- A device can start to transmit D2R signal no earlier than TR2D_min. The reader monitors the start of D2R signal within [TR2D_min, TR2D_max].

Timing for R2D response to D2R

From a reader’s perspective, the TD2R_min /TD2R_max can be defined as the earliest/latest start of R2D signal transmitted by the reader from the end of D2R signal arriving at the reader. The TD2R_min /TD2R_max should take into account the following factors: the processing delay of the reader on reception of D2R signal and preparation of R2D transmission, the margin for timing error. The value of TD2R_min /TD2R_max can be up to RAN4 discussion. The reader guarantees the start of R2D signal in the range of [TD2R_min, TD2R_max]. A device can start to monitor the R2D signal no earlier than TD2R_min.

Observation 7:

Regarding the necessity and purpose for the time interval between R2D transmission and the subsequent D2R transmission,

- The TD2R_min and TD2R_max are necessary, which can be defined as the earliest and latest start of R2D signal transmitted by the reader from the end of D2R signal arriving at the reader.

- The TD2R_min /TD2R_max should take into account the following factors: the processing delay of reader on reception of D2R signal and preparation of R2D transmission, the margin for timing error caused by residual SFO on devices.

- The reader guarantees the start of R2D signal in the range of [TD2R_min, TD2R_max]. A device can start to monitor the R2D signal no earlier than TD2R_min.

Scheduling information

Scheduling information for R2D reception

The scheduling information for the R2D reception has been discussed in RAN-120 meeting as the following proposal from FL.

FL2/FL3 High Priority Observation 4.1-1a: From RAN1 perspective, the R2D control information carrying the message type and/or device ID that is/are placed at the front part of the PRDCH is beneficial for a device to early identify the function of the PRDCH, e.g., it is A-IoT paging, A-IoT Msg2 for inventory or specific command (e.g., read/write/disable) and determine whether it is the target device.

Generally, we agree with the proposal from FL. The message type and/or device ID should be carried by the R2D control information which is placed at the front part of PRDCH. The message type and/or device ID enables a device to early identify whether the subsequent R2D transmission is for itself or not. And the energy is saved for the non-target device by allowing it to skip the reception of subsequent R2D.

Proposal 13:

For R2D reception, at least the following information should be indicated in the control information:

- ID associated with device(s).

- Message/command type.

A gap in time domain between the control information and the subsequent R2D transmission relevant to the control information should be reserved. This can guarantee that devices have enough time to successfully decode the control information and prepare to receive the subsequent R2D transmission according to the control information. Otherwise, additional memory is required to buffer the subsequent R2D transmission before the device completes the decoding of the control information.

Proposal 14:

A gap in time domain between the control information and the subsequent R2D transmission relevant to the control information should be reserved.

- This can guarantee that devices have enough time to successfully decode the control information and prepare to receive the subsequent R2D transmission according to the control information.

Scheduling information for D2R transmission

The control information for D2R scheduling has been discussed in RAN1-120 meeting. And the following agreements have been reached.

Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

Apart from the above agreements, there are still some issues to be addressed regarding the scheduling information for D2R transmission.

Frequency domain resources

FDMA is supported for D2R transmission as the description in [2]. The FDMA for D2R transmission is based on small frequency shift. In [1], the amount of small frequency shift in Hz is R/Tb, where the time duration Tb corresponds to an information bit, R is a codeword repetition number for small frequency shift with Manchester line code Option1 or R is the number of square wave periods for small frequency shift with Manchester line code Option2 or no line codes. The frequency domain resource for D2R transmission can be derived by the indication of Tb or D2R chip duration and R for small frequency shift. If the chip duration before small frequency shift has been indicated in the control information as the agreement in RAN-120 meeting, the Tb can be derived by the devices. For example, if the Manchester line code is used, the Tb is two times the chip duration. So, the value of R should be included in the control information for frequency domain resource allocation.

Proposal 15:

The value of R should be indicated in the control information for frequency domain resource allocation.

Time domain resources

If TDMA is supported for D2R transmission, the time domain resources should be allocated via the control information. The information on time domain resources allocation may include the start of the time domain resource, TBS for D2R transmission with variable size.

Proposal 16:

The allocation of D2R time domain resource in the control information at least includes

- The start of the allocated time domain resource

- TBS for D2R transmission with variable size

MCS-like information

For MSC-like information, the following agreements were reached in RAN1-120 meeting.

Agreement For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round. Agreement RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and coding rate of 1/3 from TS36.212 are reused as the mother code for Ambient IoT.

For D2R modulation, it is up to device implementation to use OOK or BPSK modulation. The reader is not necessary to indicate the modulation scheme for D2R transmission via the control information.

LTE convolutional code (CC) with constraint length of 7 and mother coding rate of 1/3. Whether other coding rates are supported or not and whether no FEC is used for D2R transmission is still under discussion in AI 9.4.2. The discussion on MCS-like information indicated by the control information can be postponed until the conclusions are reached in AI 9.4.2.

Repetition

For D2R repetition, the following agreement was reached in AI 9.4.2 in RAN1-120 meeting.

Agreement For D2R repetition, block level repetition from TR38.769 is supported.

From our understanding, the block level repetition could be transparent to the devices. The block level repetition can rely on the implementation of the reader. For example, when block level repetition is necessary, the reader can transmit a command to the device to generate the information again and the device can simply follow the command. In that case, no dedicated information for block level repetition is required for D2R scheduling.

Proposal 17:

No dedicated information for block level repetition is needed.

- The block level repetition can be realized by the reader’s implementation and completely transparent to the device.

Midamble (if supported) related information

The D2R midamble has been discussed in AI 9.4.3 and the following agreement was reached in RAN1-120 meeting.

Agreement - For D2R preamble design, the functionalities of timing acquisition, SFO estimation/time tracking and channel estimation should be supported - For D2R midamble design, the functionalities of SFO estimation/time tracking and channel estimation should be supported - D2R midamble can be transmitted at the end of the PDRCH transmission. If it is at the end, it is not designed for being used for indicating the end of PDRCH transmission - FFS: condition(s) and/or indication where the D2R midamble is present or not

The condition and/or indication where the D2R midamble is present or not is FFS. A device needs to determine the position and the number of midamble within a D2R transmission either by a predefined rule or an explicit indication in the control information. The discussion in AI 9.4.4 on whether and how the midamble related information is indicated within the control information can be postponed until the related conclusion is reached in AI 9.4.3.

R2D control information

When the scheduling information for R2D reception and D2R transmission is indicated by the R2D control information, an issue on the R2D control information is carried by L1 control signaling or higher layer signaling needs to be addressed.

The scheduling information for R2D reception should be carried the L1 control signaling. The scheduling information for R2D reception carried by L1 signaling has an advantage as the device can early detect whether the subsequent R2D transmission is addressed to the device or not. If the subsequent R2D transmission is not for the device, the device can skip receiving and processing the subsequent R2D transmission. The energy is saved for R2D reception by avoiding unnecessary processing of the R2D payload.

Proposal 18:

Support the scheduling information for R2D reception carried by the L1 control signaling.

Separate CRC should be attached to L1 control information and R2D data respectively. If separate CRC is attached to L1 control information and the R2D data, a device first receives and processes the control information and, if the device identifies that the message is intended to the device, then the device receives and processes the following R2D data. Given that device 1 operates using limited energy, this option can extend the device’s sustainable operation time by avoiding unnecessary processing.

Proposal 19:

Support the separate CRC attached to the L1 control signaling and subsequent R2D data respectively.

Furthermore, only PRDCH is supported for R2D transmission on which the L1 control signaling and/or high layer data are carried. The device may have different behaviors to process the received R2D signal when L1 control signaling is or isn’t involved in it. If a R2D signal does not involve the L1 control signaling, the device decodes the R2D signal and delivers all the decoded bits to the higher layer for further processing. In the case that a R2D signal involves the L1 control signaling, after decoding the R2D signal, the device should resolve the decoded bits about L1 control signaling in L1. In that case, the device should be able to identify whether a R2D transmission includes L1 control signaling or not.

To let a device be able to identify whether a R2D transmission includes L1 control signaling or not, the following options can be considered:

Option 1: A R2D transmission always includes a predefined zone reserved for L1 control signaling. The device can identify whether there is L1 control signaling by checking the predefined zone. If the control information for reception of high layer data does not need to be updated, the fields of predefined zone can be filled with padding bits.

Option 2: An explicit indicator is adopted to inform the device whether a R2D transmission includes L1 control signaling or not. The indicator can be carried by this R2D transmission, but it should precede the L1 control signaling and/or high layer data in time domain. Or the indicator can be carried by a previous R2D transmission. A device can identify whether L1 control signaling is contained in the R2D transmission via the detection of the indicator.

Observation 8:

- The device may have different behaviors to process the received R2D signal when L1 control signaling is or isn’t involved in it.

- The device should be able to identify whether a R2D transmission includes L1 control signaling or not.

Proposal 20:

To let a device be able to identify whether a R2D transmission includes L1 control signaling or not, the following options can be considered:

- Option 1: A R2D transmission always contains a predefined zone reserved for the L1 control signaling.

  - The device can identify whether there is L1 control signaling by checking this predefined zone.

- Option 2: An explicit indicator is adopted to inform the device whether L1 control signaling is contained in the R2D transmission.

  - The explicit indicator can be carried by this R2D transmission or a previous R2D transmission.

The scheduling information for D2R transmission is carried by high layer signaling. For DT and DO-DTT service of AIoT, the D2R transmission is always triggered by a R2D signal. Devices only transmit the D2R signal once they have successfully received the R2D signal. If the R2D signal is incorrectly decoded, the device will be unable to provide D2R transmission. Consequently, there are no obvious benefits for the scheduling information for D2R transmission carried by L1 signaling. Furthermore, using higher layer signaling to carry the scheduling information for D2R transmission avoids standardization effort on physic layer design.

Proposal 21:

Support the scheduling information for D2R transmission carried by the high layer signaling.

- Conclusion

In this contribution, we have the following observations and proposals considering the other aspects of AIoT:

Observation 1:

Assuming that M Msg.1s need to be transmitted,

- Required transmissions/resources:

  - In the case of X=1, M Msg.0s are required to trigger M Msg.1 transmissions.

  - In the TDMA case of X=M>1, 1 Msg.0 is required to trigger M Msg.1s and M-1 guard periods are required to split the M Msg.1 transmissions.

- The spectrum efficiency of the scheme supporting X=2,3 is higher than that of X=1.

  - The total time domain resource required by X/M=2 or 3 is about 10% less than that required by X=1.

  - The total time domain resource required by X/M=4 is comparable with that required by X=1.

  - The total time domain resource required by X/M=5 is even larger than that required by X=1.

Proposal 1:

Support TDMA-based transmission of Msg.1, X is no larger than 2 or 3.

Proposal 2:

To support Msg.1 transmission in TDMA manner, the Msg.1 time domain resource should be allocated/indicated by the reader.

- The start of the Msg.1 time domain resource should be indicated in the control information.

  - The reference timing can be the end of Msg.0 when the start of time domain resource is indicated.

- The payload size of Msg.1 should be fixed and predefined in the specification.

Proposal 3:

In the case that one Msg.0 triggers more than one Msg.1 transmission, the starting time for monitoring Msg.2 should be common among devices which have transmitted Msg.1 for both the separate Msg.2 transmission (option 1) and the common Msg.2 transmission (option 2).

Observation 2:

To accelerate the discussion on Msg.2 monitoring, clarification on whether the definitions of TD2R_min and TD2R_max are from a reader’s perspective or a device’s perspective is necessary.

Proposal 4:

Clarify/update the definitions of TD2R_min and TD2R_max are from a reader’s perspective.

Proposal 5:

A monitoring window should be a basic assumption for Msg.2 reception at the device side which can save energy for device 1 to receive Msg.2.

Proposal 6:

A monitoring window for Msg.2 reception should be defined with the starting time and the time duration.

Proposal 7:

From a device perspective, the window for Msg.2 monitoring starts no earlier than TD2R_min.

- The reference time for determining the start of the window can be the end of time domain resource where Msg. 1 is transmitted for TDMA with X=1 or FDMA.

- The reference time for determining the start of the window can be the end of the last Msg.1 time domain resource among the X Msg.1 time domain resources for TDMA with X>1.

Proposal 8:

An indication carried by control information or a rule predefined in the specification is necessary for devices to determine the duration of the window for monitoring Msg.2.

- In the case of common Msg.2 transmission, the following factors need to be taken into account on duration of the window:

  - The duration for Msg.2 transmission, and

  - The potential uncertainty of the start of the Msg.2 transmission.

- In the case of separate Msg.2 transmissions, the following factors need to be taken into account on duration of the window:

  - The number of TDMA/FDMA resource for Msg.1 transmissions,

  - The duration for Msg.2 transmission,

  - The potential uncertainty of the starting time of the first Msg.2 transmission, and

  - The gap between two consecutive Msg.2 transmissions, e.g., TR2D_R2D.

Observation 3:

TDMA Msg.3 transmission schemes (X>1) cannot provide better spectrum efficiency than the scheme of X=1.

- The size of Msg.3 is obviously larger than Msg.1. Long Msg.3 transmission can cause more obvious accumulated timing error.

- The more obvious accumulated timing error requires a larger guard period between the adjacent time domain resources for Msg.3 transmission to handle timing errors caused by residual SFO.

Proposal 9:

For Msg.3 transmission,

- FDMA can be considered.

- TDMA is not supported.

Proposal 10:

If separate Msg2 transmission is supported, Msg.3 transmissions could be:

- Alternative 1: Msg.3s can be transmitted after all Msg.2 transmissions are completed. (Non-interleaving Msg.3 transmission given in the FL summary).

- Alternative 2: Each Msg.3 is transmitted right after its corresponding Msg.2. The Msg.2 and Msg.3 of one device are after the Msg.2 and Msg.3 of another device. (Interleaving Msg.3 transmission given in the FL summary.)

Observation 4:

If the interleaving Msg.3 transmission scheme given in the FL summary is supported, the Msg.2 monitoring window could be enlarged because both the Msg.2 transmissions and the Msg.3 transmissions should be counted in the window. In this case, the energy consumption of devices may increase due to a longer monitoring period.

Proposal 11:

Adopt the non-interleaving Msg.3 transmission if the separate Msg.2 transmission scheme is supported.

Observation 5:

The definitions of the timing intervals given in TR38.769 section 6.1.3 should also be further clarified/updated, i.e., whether the time intervals are defined from a reader’s perspective or a device’s perspective.

Proposal 12:

The definition of the time intervals on AIoT processing timing should be from a reader’s perspective.

- A device cannot perceive the timing error due to its internal low-quality clock. On the contrary, a reader can detect and handle the timing error.

- The impacts of timing error must be considered when the time intervals are defined, otherwise the potential collision between transmissions from different devices cannot be avoided.

Observation 6:

Regarding the necessity and purpose for the time interval between R2D transmission and the subsequent D2R transmission,

- The TR2D_min and TR2D_max are necessary, which can be defined as the earliest and latest start of D2R signal arriving at the reader from the end of R2D signal transmitted by reader.

- The TR2D_min /TR2D_max should take into account the following factors: the processing delay of the device on reception of R2D signal and preparation of D2R transmission, the margin for timing error caused by residual SFO on devices.

- A device can start to transmit D2R signal no earlier than TR2D_min. The reader monitors the start of D2R signal within [TR2D_min, TR2D_max].

Observation 7:

Regarding the necessity and purpose for the time interval between R2D transmission and the subsequent D2R transmission,

- The TD2R_min and TD2R_max are necessary, which can be defined as the earliest and latest start of R2D signal transmitted by the reader from the end of D2R signal arriving at the reader.

- The TD2R_min /TD2R_max should take into account the following factors: the processing delay of reader on reception of D2R signal and preparation of R2D transmission, the margin for timing error caused by residual SFO on devices.

- The reader guarantees the start of R2D signal in the range of [TD2R_min, TD2R_max]. A device can start to monitor the R2D signal no earlier than TD2R_min.

Proposal 13:

For R2D reception, at least the following information should be indicated in the control information:

- ID associated with device(s).

- Message/command type.

Proposal 14:

A gap in time domain between the control information and the subsequent R2D transmission relevant to the control information should be reserved.

- This can guarantee that devices have enough time to successfully decode the control information and prepare to receive the subsequent R2D transmission according to the control information.

Proposal 15:

The value of R should be indicated in the control information for frequency domain resource allocation.

Proposal 16:

The allocation of D2R time domain resource in the control information at least includes

- The start of the allocated time domain resource

- TBS for D2R transmission with variable size

Proposal 17:

No dedicated information for block level repetition is needed.

- The block level repetition can be realized by the reader’s implementation and completely transparent to the device.

Proposal 18:

Support the scheduling information for R2D reception carried by the L1 control signaling.

Proposal 19:

Support the separate CRC attached to the L1 control signaling and subsequent R2D data respectively.

Observation 8:

- The device may have different behaviors to process the received R2D signal when L1 control signaling is or isn’t involved in it.

- The device should be able to identify whether a R2D transmission includes L1 control signaling or not.

Proposal 20:

To let a device be able to identify whether a R2D transmission includes L1 control signaling or not, the following options can be considered:

- Option 1: A R2D transmission always contains a predefined zone reserved for the L1 control signaling.

  - The device can identify whether there is L1 control signaling by checking this predefined zone.

- Option 2: An explicit indicator is adopted to inform the device whether L1 control signaling is contained in the R2D transmission.

  - The explicit indicator can be carried by this R2D transmission or a previous R2D transmission.

Proposal 21:

Support the scheduling information for D2R transmission carried by the high layer signaling.

- Reference

[1] TR 38.769 V2.0.0, Study on solutions for ambient IoT (Internet of Things), 2024-12.

[2] RP-243326, New WID: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[3] R1-2501625, Final FL summary on other aspects for Rel-19 Ambient IoT, Athens, Greece, February 17th – 21st, 2025