3GPP TSG RAN WG1 #120bis                                                         R1-2502309

Wuhan, China, April 7th – 11th, 2025

Agenda Item:					    9.1.1

Source:	Sony

Title:	Discussion on specification support for beam management

Document for:	Discussion/ Decision

Introduction

Referring to the findings presented during the RAN1 meeting's discussion on AI for physical layer use cases in Rel18 [1], it is evident that leveraging AI for predicting candidate beams yields significant advantages, as indicated by the evaluation results from companies’ contributions. The following is the core part of the further normative work for AI-based beam management.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this paper, we will explore the potential specification impact of AI-based beam management, focusing on two key aspects: model inference, and model monitoring.

AI/ML model inference

Regarding the following proposals about model inference and monitoring, which is provided by FL [2] and expected to be discussed at the RAN1#120bis meeting, we would like to share some of our perspectives on it.

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability related information of predicted Top K beam(s) among a set of beams - Where the probability information is the probability of the beam to be the Top 1 or Top K beam [4] bits are used to equally quantize the probability information with a range of [0, 1]

We spent quite a long time discussing the content of the inference results. The criteria for the model's output can be based on the highest probability information, identifying the top 1 or K beams. Providing the probability information of the predicted top K beams among a set of beams can significantly assist the NW in selecting the optimal beam(s) based on UE reporting. Therefore, for the UE-side model, we strongly advocate including the probability information of the predicted top K beams in the reports. This will enhance the efficiency of beam selection.

- : For the UE-side model, regarding the reporting of inference content, support beam information for the predicted Top K beams among a set of beams, along with probability-related information for these beams.

Model monitoring

The following proposal concerns the performance metrics for model monitoring, as provided by the FL summary for discussion at the RAN1#120bis meeting.

- Introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1). - the size of CSI field associated with the BAI is log2(N+1) - Where N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s) - Where Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, - at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) - Where K is the number of predicted beam(s) in the corresponding inference report - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, and M is up to 4

We agree to define the BAI to evaluate beam prediction accuracy. However, there is a question regarding how to calculate Np. Specifically, using the inclusion of one of the Top M beams from the monitoring resource set in the Top K predicted beams as a criterion for prediction correctness might not be rigorous. At a least, we should consider whether the RSRP values of the measured beams and the predicted beams are within an acceptable threshold. For example, suppose the Top M measured beams are beam #1 and beam #2 with measured RSRP values of 6 dBm and 7 dBm, respectively. The Top K predicted beams are beam #2 and beam #3 with predicted RSRP values of 12 dBm and 11 dBm, respectively. Although beam #2 is correctly identified as the best beam, the RSRP difference between the measured and predicted values is 4 dBm, which may exceed the acceptable threshold and therefore the model prediction failure should be considered.

we propose that the definition of a correct prediction instance should include not only the identification of the correct beam ID but also ensure that the RSRP difference between the measured and predicted values falls within a predefined threshold.

- : For model monitoring, If the beam ID of one of the Top M beams is included in the Top K predicted beams, and the corresponding RSRP difference between the measured and predicted values should fall within a predefined acceptable threshold, then the prediction can be considered correct.

This proposal focuses on the configuration of resources for monitoring purposes and is also provided by the FL summary.

- The resource set(s) for monitoring may or may not cover the full resources of Set A - The mapping between the resources for Set A and resources for monitoring can be configured in RRC. - FFS on whether to use TCI-StateID or a new parameter - If the mapping is not configured, the mapping between the resources for monitoring and resources of Set A is based on the NZP-CSI-RS-ResourceId/SSB-Index.

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring, - for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to [the CSI reference resource or the transmission occasion of the resource] - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource with is “closest” to the time instance for prediction FFS on definition of “closest” and any other condition for the linking

For monitoring, the NW does not always need to indicate the full set to the UE, especially considering the potential for larger-scale beams in the future. Therefore, we fully support the use of subsets for monitoring. We propose defining the mapping between the resources for Set A and the resources for monitoring. This can be achieved by using the TCI-StateID or by defining a new parameter to indicate a group of beams. The advantage of this approach is that when the NW indicates the TCI-StateID or the new parameter to the UE, it can carry additional transmission configuration information, offering greater flexibility.

- : Support defining the mapping between the resources for Set A and the resources for monitoring, which can be configured in RRC.

Regarding the time domain linkage between monitoring instances and report instances, considering the time-varying nature of the channel, the transmission occasion for each monitoring instance should be close to the transmission occasion for the resource in Set B in BM-Case 1, or correspond to the prediction time in BM-Case 2. This ensures the accuracy of monitoring. The transmission timing for each monitoring instance can also be synchronized closely with the inference results derived from a CSI reference resource, once the NW receives the inference result report.

- : For UE-side model monitoring of Type 1 Option 2 (when applicable), the configuration for monitoring:

  - For BM-Case 1, the transmission occasion of monitoring resources should be close to the transmission occasion of the resource for Set B.

  - For BM-Case 2, the transmission occasion of monitoring resources should correspond to or be close to the associated prediction time.

Whether the subset used for monitoring measurements should include the Top K predicted beams depends on the performance metric being used. If the metric focuses on the RSRP difference between the predicted and measured RSRP of the corresponding beams, the subset has to involve the Top K predicted beams. However, if the performance metric is based on Top 1 or Top K beam prediction accuracy, the subset does not necessarily need to cover the Top 1 or Top K predicted beams. For example, if the highest RSRP in the measured set exceeds the predicted highest RSRP in the set of predicted beams, the subset can still be considered valid even if it does not include the Top 1 or Top K predicted beam. In this case, such an event can be counted as a prediction failure, leading to two statistical methods: one is when the number of such failures exceeds a predefined threshold, the model can be considered to have failed; another is that the failure probability can be calculated based on these failures, and the beam prediction accuracy equals 1 minus the failure probability. The advantage of this approach is that the NW does not need to wait for the UE to report its prediction results. Instead, after the monitoring mechanism is activated, the NW can directly allocate a monitoring set to the UE for measurement, thereby reducing latency. Additionally, the NW does not need to reconfigure/update the monitoring set due to the absence of predicted beams in the initial configuration.

Certainly, if a subset does not contain the Top 1 or Top K predicted beams and the measured RSRP values are lower than those of the predicted beams, this subset can be considered invalid. Reconfiguration of the subset in such case is worthy of further research and analysis.

- : Support further investigation into the scenario where the resource set(s) configured for monitoring does not include the Top K predicted beams.

For the UE-side model, further study is needed to determine how to trigger reporting based on events for Option 1 and/or Option 2 during monitoring. Initially, certain events should be clearly defined.

Regarding option 2, UE-assisted performance monitoring, it is necessary to define specific events that trigger reporting for model monitoring. Considering the scenario, in BM-Case 1, when predicted candidate beams are also measured in Set C or Set B, a direct comparison between the predicted and measured results can be performed to determine whether the inference is successful or not. This involves checking if the predicted RSRP differs from the measured RSRP. When such a failure is detected, the UE can report it to the NW to initiate further monitoring procedures. This process can be defined as an event.

For BM-Case 2, performing model monitoring needs to measure the full resource set configured in data collection window by NW which results in significant measurement overhead and latency. Especially, to avoid the "ping-pong" effect, the performance metric must be calculated across multiple time instances rather than relying on a single measurement. Hence, to reduce this overhead, for BM-Case 2, we propose reusing part of the input data from the previous inference instance for each new inference instance. This approach can be illustrated as following picture: for each inference instance, the model's input is based on measurements from five consecutive slots and predict for future four consecutive slots. In the first inference instance, the input data is derived from the measurements of slots one through five. In the second inference instance, the input data is collected from two parts: the new measurement from slot six and the reused measurement data from slots two through five, which were measured in the first inference instance.

This approach doesn’t need NW to configure the full set of resources in data collection window according to the model's input requirements. Instead, only a portion of the resources needs to be configured, while the other part of the input data is derived from the measurements obtained in the previous instance. This approach can significantly reduce the measurement overhead.

Figure1. schematic diagram of multiple inferences instances with data reuse in BM-Cases 2

In this figure, the candidate beam in slot eight has been predicted in multiple inference instances. If the predicted results in each instance are the same or similar, this indicates that the model's performance is stable and suitable for the current scenario. Conversely, if the predicted results vary significantly, it may suggest that the model is failing to infer accurately. This can also be defined as an event to trigger reporting for monitoring.

- : Support for defining event(s) to trigger reporting for monitoring,

  - when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

  - when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

- : For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

Regard whether/how to configure or define a window for calculation, we propose introducing a model failure detection mechanism similar to the existing beam failure detection mechanism. For example, during the process of model switching, the phenomenon known as the "ping-pong effect" may also occur. To guarantee robustness and account for external influences, the initiation of model failure should take into consideration multiple detection results.

When the performance metric of a model drops below a pre-defined threshold, it can be marked as one failure and then, the UE/gNB will start a failure count with a specific timer activated. Upon reaching the maximum allowable failure count within the timer's duration, model failure can then be triggered. This ensures a comprehensive approach to handling model failures aligned with the principles established for beam failure mechanisms.

- : Model failure is determined by the performance metrics that are calculated multiple times over a specific time period.

Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : For the UE-side model, regarding the reporting of inference content, support beam information for the predicted Top K beams among a set of beams, along with probability-related information for these beams.

- : For model monitoring, If the beam ID of one of the Top M beams is included in the Top K predicted beams, and the corresponding RSRP difference between the measured and predicted values falls within a predefined acceptable threshold, then the prediction can be considered correct.

- : Support defining the mapping between the resources for Set A and the resources for monitoring, which can be configured in RRC.

- : For UE-side model monitoring of Type 1 Option 2 (when applicable), the configuration for monitoring:

  - For BM-Case 1, the transmission occasion of monitoring resources should be close to the transmission occasion of the resource for Set B.

  - For BM-Case 2, the transmission occasion of monitoring resources should correspond to or be close to the associated prediction time.

- : Support further investigation into the scenario where the resource set(s) configured for monitoring does not include the Top K predicted beams.

- : Support for defining event(s) to trigger reporting for monitoring,

  - when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

  - when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

- : For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

- : Model failure is determined by the performance metrics that are calculated multiple times over a specific time period.

Reference

- 3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- R1-2501595   FL summary #6 for AI/ML in beam management	(Samsung).