3GPP TSG-RAN WG1 Meeting #120bis	 R1-2502797

Wuhan, China, April 7-11, 2025

Source:	KDDI Corporation

Title:	Discussion on common signal adaptation

Tdoc Type:	Discussion

Agenda Item:	9.5.3

Document for:  Discussion

Release:       Rel-19

1	Introduction

In this contribution, we will provide our further consideration on common signal adaptation.

2	Discussion

2.1 adaptation of SSB

Agreement @RAN1#120 For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Regarding value x, since the current SSB periodicity allows for the configuration of 6 types of SSBs, we think it is reasonable to set the x to a maximum of 6.

Additionally, currently the active SSB index can only be changed via bitmap, which is configured in RRC. It would be beneficial to allow not only the periodicity adaptation, but also the dynamic adaptation of the SSB index

Proposal 1: up to 6 additional SSB burst periodicity for Scell can be configured to the UE.

Proposal 2:RAN1 to discuss dynamic adaptation of SSB index

2.2 adaptation of PRACH

Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

When the NES UE is indicated the additional PRACH resources is available, it is straightforward to apply the PRACH mask index to the additional PRACH resource. In this case, we think the legacy PRACH mask index can be used without enhancement.

Regarding how to indicate to the UE, one option is to use a dedicated Random access preamble index, like PDCCH order, another option is to use the 1 reserved bit.

Proposal 3: The legacy PRACH mask index can be applied to the additional PRACH resources.

Proposal 4: RAN1 to discuss to use either a dedicated random access preamble index, or a reserved bit to indicate the availability of the additional PRACH resources.

Agreement For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE, - Option 1: SFN of the first PF from the next I-DRX cycle - Option 2: SFN of the first PF from the current I-DRX cycle - Option 3: From the first frame of the first PRACH association period after UE receives the DCI - Option 4: From the first frame of the current SI modification period - Option 5: From the first frame of the next SI modification period

Regarding to the reference point, similar to the timing of SI modification, we consider option 1 and option 5 to be reasonable. Other options will have more spec impact.

Regarding whether DCI-based deactivation is needed in addition to the validity time duration approach, we consider it necessary, as the DCI-based approach offers greater flexibility and is particularly important in areas with significant traffic fluctuations.

Proposal 5: Reference point application approach can be considered using the SI modification approach, new method is not needed unless there is significant specification impact.

Proposal 6: DCI-based PRACH adaptation deactivation is supported.

Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

When msg1-FDM for additional PRACH resources is not configured, we believe it is unnecessary to define a predefined msg1-FDM value for the UE. Since msg1-FDM is a mandatory configuration for legacy PRACH resources, the UE can rely on the legacy PRACH resource configuration when msg1-FDM for additional PRACH resources is not provided.

Proposal 7: UE rely on the msg1-FMD for legacy PRACH resource configuration when it is not configured for the additional PRACH resource.

Agreement For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources - The mask is applicable at unit of - Alt 1: PRACH association period - Alt 2: PRACH association pattern period - Alt 3: SFN level - The PRACH association period is determined based on valid additional ROs only. - The mask is applied after valid RO determination and SSB-RO mapping. - Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask. - This is applicable at least for adaptation for DCI 1_0 with P-RNTI - The DCI does not indicate PRACH mask selection - FFS: how the mask is identified - Option 1: The PRACH mask is from a PRACH mask table - Pre-defined table with N=[4 or 8 or 16] rows - The semi-static signalling indicates a PRACH mask index - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Regarding the PRACH mask table, we believe the legacy PRACH mask table can be reused. If a company finds the current table insufficient, the reserved bits can be used to extend it. A separate PRACH mask table is not necessary.

Proposal 8: Reuse the legacy PRACH mask table for the additional PRACH resources.

3	Conclusion

We make the following observations and proposals:

Proposal 1: up to 6 additional SSB burst periodicity for Scell can be configured to the UE.

Proposal 2:RAN1 to discuss dynamic adaptation of SSB index

Proposal 3: The legacy PRACH mask index can be applied to the additional PRACH resources.

Proposal 4: RAN1 to discuss to use either a dedicated random access preamble index, or a reserved bit to indicate the availability of the additional PRACH resources.

Proposal 5: Reference point application approach can be considered using the SI modification approach, new method is not needed unless there is significant specification impact.

Proposal 6: DCI-based PRACH adaptation deactivation is supported.

Proposal 7: UE rely on the msg1-FMD for legacy PRACH resource configuration when it is not configured for the additional PRACH resource.

Proposal 8: Reuse the legacy PRACH mask table for the additional PRACH resources.