3GPP TSG-RAN WG1 Meeting #120bis	R1-2503097

Wuhan, China, April 7th-11th, 2025

Source:	Moderator (vivo)

Title:	Summary #5 of discussion on LP-WUS and LP-SS design

Agenda Item:	9.6.1

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the discussions on LP-WUS and LP-SS design in RAN1# 120bis.

The issues in this document are tagged and color coded with [H] or [M].

- Proposals for Online Sessions

  - Proposals for Monday online session

#### [H][FL1] Proposal 3.1-5r1: For both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints indicated by a LP-WUS from system perspective.

#### [H][FL1] Proposal 3.2-2: Support LP-WUS duration can be larger than one slot.

#### [H][FL1] Proposal 3.3-1r1: For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell,

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119)

- For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value.

- FFS: For FR2, UE capability reporting on the supported number of candidates overlaid sequences within the range given by the maximum numbers

#### [H][FL1] Proposal 3.3-2: For M=1, the overlaid OFDM sequence length LZC=12∗X, X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

#### [H][FL1] Proposal 3.3-4r1: Support maximum 4 root(s) to be used for overlaid OFDM sequences for M=1/2/4.

- 2 pairs of complementary roots, i.e., root q and root (Bzc-q) are supported.

#### [H][FL1] Proposal 3.3-7: For WUS information carried by the overlaid OFDM sequence(s), support Alt 1.

- Alt 1: Raw information bits are mapped to sequence(s)

#### [H][FL1] Proposal 3.3-8: For WUS information carried by the overlaid OFDM sequence(s), support repetition of overlaid OFDM sequence

- gNB configures repetition number of overlaid OFDM sequence.

#### [H][FL1] Proposal 5.2-1: Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1 and M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support LP-SS periodicity of 160ms

- FFS:  LP-WUS with M=4

#### [H][FL1] Proposal 3.1-3: For RRC idle/inactive, support Alt 1:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP.

Note: The legacy initial DL BWP is not the Redcap-specific initial DL BWP, if configured.

  - Proposals for Tuesday online session

#### [H][FL2] Proposal 5.2-1: Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1 and M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support LP-SS periodicity of 160ms

- FFS:  LP-WUS with M=4

#### [H][FL 2] Proposal 3.3-7r1: For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from LSB to MSB, where K= ceil (N/log2L) , L is the number of overlaid OFDM sequences for one OOK ON chip.

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

- Alt 2: Raw information bits are mapped to sequence(s) after channel coding

  - Same channel coding scheme(s) as OOK is applied.

    - FFS same or different rate matching and/or repetition factor as OOK

#### [H][FL2] Proposal 3.3-4r1: Support maximum 4 roots to be used for overlaid OFDM sequences for M=1/2/4.

- 2 pairs of complementary roots, i.e., root q and root (Bzc-q) are supported for 4 roots.

#### [H][FL2] Proposal 3.1-3: For RRC idle/inactive, support Alt 1:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP.

Note: The legacy initial DL BWP is not the Redcap-specific initial DL BWP, if configured.

#### [M][FL 2] Proposal 3.4-1: Confirm the following working assumption for RRC connected mode.

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

    - Depending on UE capability, a UE may support less than 8

  - Proposals for 1st Thursday online session

#### [H][FL2] Proposal 3.2-3r1: For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block (coded bits) length after rate matching < 32

- Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

- Alt 1a: Information bits are repeated before RM coding, and rate matching after RM coding is applied, as section 5.4.3 in TS 38.212. (Nokia)

- Alt 2: Rate matching after RM coding is applied, wherein the rate matching is performed by truncating from the heading bits or tailing bits according to a single code block(coded bits) length threshold. The threshold is 16. (Huawei)

- Alt 3: Rate matching after RM coding is performed, where the rate matching is performed according to an interleaving pattern, and the interleaving pattern is applicable to all code block length. Interleaving pattern follows the following table: (Huawei)

| i | 0…4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21…24 | 25…31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| πi | 0…4 | 14 | 10 | 8 | 7 | 9 | 6 | 13 | 5 | 16 | 19 | 24 | 11 | 12 | 15 | 17 | 18 | 20…23 | 25…31 |

- Alt 4: Rate matching after RM coding is applied, wherein Rate matching is performed according to on pre-defined pattern for bit collection for each code block length L, i.e., multiple patterns are to be specified.

  - Predefined pattern 1:CATT

|  | detail |
| --- | --- |
| OOK-4 with M=4 | Puncture bit index set = {1~4} |
| OOK-4 with M=2 | Puncture bit index set = {15~32} |
| OOK-4 with M=1 | Puncture bit index set = {8~32} |

  - Predefined pattern 2:Qualcomm

| Sequence Length L | Pattern for retained bits |
| --- | --- |
| 2 | [1 4] |
| 3 | [1 2 4] |
| 4 | [1 2 5 9] |
| 5 | [1 2 4 6 7] |
| 6 | [1 3 4 5 7 9] |
| 7 | [1 2 3 4 5 7 9] |
| 8 | [1 2 5 6 9 14 17 25] |
| 9 | [1 2 4 5 6 7 9 10 12] |
| 10 | [1 2 3 4 5 6 7 8 9 11] |
| 11 | [1 2 3 4 5 6 7 8 9 11 25] |
| 12 | [1 3 4 5 6 7 9 10 11 14 15 17] |
| 13 | [1 2 3 4 5 6 7 8 9 10 11 14 25] |
| 14 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17] |
| 15 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17 25] |
| 16 | [1 2 5 6 9 12 14 16 17 19 22 24 25 26 28 31] |
| 17 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25] |
| 18 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 28] |
| 19 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 27 28] |
| 20 | [1 2 3 4 5 6 7 8 9 10 11 14 15 16 17 22 23 24 25 31] |
| 21 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 19 21 25 27 28] |
| 22 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 29] |
| 23 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 28 29] |
| 24 | [2 3 4 5 7 8 9 10 11 12 13 14 17 18 19 21 22 24 25 27 28 29 30 31] |
| 25 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 18 19 21 25 26 27 28 29 31] |
| 26 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 22 23 24 25 26 27 28 29 30] |
| 27 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 21 22 23 24 25 26 27 28 29 30] |
| 28 | [2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 27 28 29 30 31] |
| 29 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30] |
| 30 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30 31] |
| 31 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31] |

#### [H][FL 3]Proposal 4.2-1r2  For the length L of LP-SS binary sequence, supports

- M=1, L= {6, 8}

- M=2, L= {12,16}

- M=4, L= {16,32}

- Note: No additional UE capability on supporting length L for each M

#### [H][FL 3]Proposal 4.2-1  For LP-SS with M=4, supports set1 (balanced).

#### [H][FL2] Proposal 3.3-7r1: For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L) , L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

    - FFS whether padding bit(s) is added to N raw information bits

    - FFS the case N=5, L=16

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

- Alt 2: Raw information bits are mapped to sequence(s) after channel coding

  - Same channel coding scheme(s) as OOK is applied.

    - FFS same or different rate matching and/or repetition factor as OOK

#### [H][FL 2] Proposal 3.1-3: For RRC idle/inactive, support Alt 1:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP.

Note: The legacy initial DL BWP is not the Redcap-specific initial DL BWP, if configured.

#### [M][FL 2] Proposal 3.4-1: Confirm the following working assumption for RRC connected mode.

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

[H][FL 4] Proposal 4.1-2: For the M value for LP-WUS and LP-SS, supports:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

- [M][FL 2] Proposal 3.2-4r1: For > 2 information bits with RM coding, support one of the alternatives for candidate code block length(coded bits) after rate matching

- Alt 1: support any code block (coded bits) length no less than [3].

- Alt 2: support up to X candidate code block (coded bits) length, at least including code block (coded bits) length=32. FFS X, and other values.

#### [H][FL2] Proposal 3.3-3: For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC,LZC is the overlaid OFDM sequence length.

  - Proposals for 2nd Thursday online session

#### Proposal 3.3-3: For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC,LZC is the overlaid OFDM sequence length.

#### Proposal 3.3-7r1: For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1 and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L) , L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

    - FFS whether padding bit(s) is added to N raw information bits

    - FFS the case N=5, L=16

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

- Alt 2: Raw information bits are mapped to sequence(s) after channel coding

  - Same channel coding scheme(s) as OOK is applied.

    - FFS same or different rate matching and/or repetition factor as OOK

- Proposal 3.2-4r1: For > 2 information bits with RM coding, support candidate code block (coded bits) length after rate matching within the below range:

- Alt 1a: code block (coded bits) length: [3,4]~[32,64]

  - FFS whether a subset of lengths are selected.

  - FFS whether same range applies to all M values

  - Note: above lengths do not consider repetition (if supported) after rate matching

#### Proposal 3.2-1r1: Support 1 & 2 information bits for LP-WUS.

#### Proposal 3.1-3: For RRC idle/inactive, support Alt 1:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP.

Note: The legacy initial DL BWP is not the Redcap-specific initial DL BWP, if configured.

#### Proposal 3.4-1: Confirm the following working assumption for RRC connected mode.

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

[H][FL 4] Proposal 4.1-2: For the M value for LP-WUS and LP-SS, supports:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

  - Proposals for Friday online session

[M][FL 2] Proposal 3.2-6:  Do not support scrambling for LP-WUS.

#### [M][FL2] Proposal 3.1-4: For LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB, with range of 0~ 263.

#### [M][FL 2] Proposal 3.2-7:  For WUS information carried by OOK, do not support further repetition after coding and rate matching .

- LP-WUS design

  - OOK-1/OOK-4 waveform

M values for OOK-4

RAN1 made following agreement for candidate M value

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

For working assumption of M for OOK-4 for 30kHz SCS,

- [2][3][4][5][12][14][16][18][19][21][23] proposed to support M=4, to reduce LP-WUS overhead/increase data rate,

- [2] support M=4 for RRC connected state, but do not support M=4 for 30kHz SCS for RRC idle/inactive state, considering no performance gain compared with over OOK-1/OOK-4 M=1 or M=2 at Msg 3 PUSCH coverage, and potentially larger LP-SS overhead to meet tighter synchronization requirements. [18] support M=4 at least for RRC connected state.

- [17] does not support M=4 for 30kHz SCS, with concern on LP-WUR complexity.

FL suggests to go with majority view, to avoid complication of design for different RRC state or SCS for FR1.

#### [M][FL 2] Proposal 3.1-1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW | Y |  |
| Nokia.1 | Y |  |
| OPPO | Y | Support the proposal. |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |
| docomo | Y |  |
| Qualcomm | Y |  |

[2][7][17][23] also discuss M value for FR2. Considering the UE detection complexity, memory size and reusing the designed overlaid OFDM sequences as much as possible, FL suggests the following :

#### [M][FL 2] Proposal 3.1-2: For FR2, support at least M=1 for SCS=60kHz and 120kHz. FFS other M values.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW | Y |  |
| Nokia.1 | Y |  |
| OPPO |  | Support reusing the design of FR1. |
| ZTE, Sanechips | Y | Support reusing the design of FR1 and no additional efforts as much as possible |
| docomo | Y |  |
| Qualcomm | Y |  |

Frequency resource location for LP-WUS/LP-SS

RAN1 made following agreement for LP-WUS frequency resource for RRC connected mode. For idle and inactive modes, RAN1 is to down-select between the two options that LP-WUS is either within or can be outside of the initial DL BWP.

Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP

The benefit of confining LP-WUSLP-SS within initial DL BWP allows reusing existing BWP framework and procedures and reduces the need for UE RF retuning and facilitates offloading of RRM measurements to WUR.

The benefit of allowing LP-WUS outside of initial DL BWP includes more flexibility for placing WUS/LP-SS in the carrier and less congestion in the initial DL BWP. In addition, it can avoid separate LP-WUS transmission for Redcap UE and regular UE, if Redcap initial DL BWP is non-overlapped with regular initial DL BWP.

If LP-WUS/LP-SS frequency resource can be out of initial DL BWP, [9][17] suggests the separation between LP-WUS/LP-SS and SSB in frequency domain must be restricted within X MHz, to ensure that all three physical channels experience similar fade.

Company views are summarized as below:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within initial DL BWP.

[5][6] [7] [11] [25][17][23][10] for OFDM-based LP-WUR.

- Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP.

[2][9][15][20][24][10] for OOK-based LP-WUR.

For Redcap UE, in addition to legacy initial DL BWP (initial DL BWP for normal UE), Redcap-specific initial DL BWP can be configured. For Alt 1, to avoid separate LP-WUS for Redcap and non-Redcap UE, [8] proposes LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP, and Redcap UE can retune to the legacy initial DL BWP for LP-WUS/LP-SS reception.

Considering additional effort for Alt 2, e.g., further discussion on the separation between LP-WUS/LP-SS and SSB in frequency domain, FL suggests to go with Alt 1 for simplicity.

#### [H][FL 2] Proposal 3.1-3: For RRC idle/inactive, support Alt 1:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP.

Note: The legacy initial DL BWP is not the Redcap-specific initial DL BWP, if configured.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW |  | We prefer the support of frequency resource outside the legacy initial DL BWP for scheduling flexibility and to avoid overloading the resources used for initial DL BWP. |
| Nokia.1 | N | For IQ receiver, SSB based measurements may not guarantee the detectability of LP-WUS if SSB and LP-WUS are far apart in frequency spectrum even if both are within initial BWP. |
| OPPO | N | Considering the resource capacity and flexibility. Support LP-WUS/LP-SS can be configured out of initial DL BWP. |
| Samsung | Y |  |
| CATT | Y |  |
| Huawei, HiSilicon | N | We don’t support Alt1. We support Alt 2 for the following reasons: - Due to the time domain detection, the frequency centre of LP-WUR should be aligned exactly with the centre of LP-WUS/LP-SS. With even a single RE offset between LP-WUS/LP-SS and SSB, retuning cannot be avoided. On the other hand, initial DL BWP can be configured with up to 100MHz BW. Thus, confining LP-WUS/LP-SS within initial DL BWP cannot restrict the frequency distance between SSB and LP-WUS/LP-SS. - We can go with vivo’s proposal, i.e., put restriction to the maximum frequency distance between SSB and LP-WUS/LP-SS. Specifically, the restriction can be up to 100MHz, corresponding to the same carrier. - It is beneficial if LP-WUS can be flexibly configured. |
| vivo | No | We think it is important to allow offloading of LP-WUS/LP-SS to a frequency region out of initial DL BWP, to alleviate congestion and avoid collision between LP-WUS/LP-SS and NR channels, especially when a LP-WUS can occupy more than one slot. Regarding retuning issue, since LP-SS with overlaid OFDM sequence is supported, as long as LP-SS and LP-WUS is in same frequency region (we think it is common understanding they are in same frequency resource), LP-WUR can perform measurement using LP-SS without need of retune to receive SSB. So, we support Alt 2. |
| LGE | Y in principle | Support the proposal in principle In addition, we have similar view with Nokia. Especially, for a large number of PRBs are configured for initial DL BWP, a moderate gap from SSB is preferred. |
| ZTE, Sanechips | N | It is too challenging for the NW to deploy LP-WUS in the same initial DL BWP. Regarding outside initial DL BP, if the concern is the WUR retuning between SSB and LP-WUS, there is no difference if the NW configure initial DL BWP same bandwidth as the carrier. Otherwise, the LP-WUS should be within the SSB bandwidth. |
| docomo | Y |  |
| Panasonic | N | The crucial factor is the frequency gap between SSB and LP-WUS/LP-SS for any possible RF reusing. So frequency gap with a maximum value should be used to limit the LP-WUS/LP-SS location and in the meanwhile to allow for allocation flexibility. |
| Qualcomm | Y |  |

Another issue to be discussed is, how to configure the frequency resource for LP-WUS/LP-SS. LP-WUS/LP-SS bandwidth is fixed as 11 PRBs at least for channel BW no smaller than 5MHz per RAN1 agreement, so only offset to determine LP-WUS/LP-SS starting PRB needs to be configured. Considering LP-WUS can be out of active BWP for RRC connected mode, and better to have unified configuration for all RRC states, FL suggests to configure the offset with reference to carrier boundary.

#### [M][FL2] Proposal 3.1-4: For LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB, with range of 0~ 263.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW |  | OK |
| Nokia.1 | [Y] | Does this include guard RBs/SCs as well? |
| OPPO |  | It depends on whether the LP-WUS/LP-SS resource can be configured out of initial DL BWP. |
| Huawei, HiSilicon | Y | Suggest to use the carrier boundary or pointA as the reference, since our preference is Alt 2 for Proposal 3.1-3. |
| LGE |  | If Proposal 3.1-3 is agreed to support Alt 1, no need to configure the wide range of 0~263. Instead, PRB offset from the lowest RE for initial DL BWP would be sufficient. We can discuss this proposal after Proposal 3.1-3. |
| ZTE, Sanechips | Y |  |
| docomo | Y |  |
| Qualcomm | Y |  |

(closed)Number of information bits for LP-WUS/LP-SS

RAN1 agreed to support up to 5 bits information for RRC idle/inactive state, while the upper bound for RRC connected mode is still open. Company views are summarized as below.

- Up to 5 bits: [2][3][5][8][6][15][27][7][10][16][20][23]

- Up to 6 bits: [11][7]

- Up to 7 bits: [18]

On one hand, for RRC connected mode, additional codepoints can be useful to reduce the false alarm probability that can be caused by frequent data traffic types. On the other hand, more codewords can increase FAR, and it may complicate design, e.g., the proper rate matching pattern and code block length as discussed in section 3.2. FL suggests to go with unified design for both RRC idle/inactive and connected state, i.e., 5 bits.  Correspondingly, the maximum number of codepoints per LP-WUS is 32.

#### [H][FL 2] Proposal 3.1-5: For both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints indicated by a LP-WUS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW |  | We prefer the support to be for at least 6 information bits to enable the distinguishing of at least 2 POs when one LO to multiple PO mapping is considered to avoid reducing the number of subgroups per PO which can have negative impact on power saving gain. |
| Nokia.1 | Y |  |
| OPPO | N | We have made following agreement in RAN1#118 meeting. Agreement [RAN1#118] For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.. For example, a LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for 4 POs, and the number of subgroups per PO is 31. How to use 5 information bits to indicate one or more subgroup(s) in the 4 POs? If the information bits are ‘01010’, and each PO has the subgroup corresponding to codepoint value ‘01010’. In this case, which PO’s subgroup is the target subgroup would be woke-up? If considering the similar way as specified for PEI, i.e., multiple fields in one LP-WUS, each field is associated with one PO. It can not work directly. We support introducing one PO indication field to distinguish the POs associated with one LO. |
| Samsung | Y |  |
| CATT | Y |  |
| Huawei, HiSilicon | Y | Just to clarify, it is from spec perspective, instead of from UE perspective. |
| vivo | Y |  |
| LGE | Y |  |
| ZTE, Sanechips |  | For idle mode, Yes. For connected mode, we think more information bits can be supported since 5bits can not support UE specific wake-up if the NW has more than 32UEs configured with LP-WUS |
| mod |  | @oppo, we have also agreed that for one LP to multiple PO case, the total number of codepoints indicated by a LP-WUS is not larger than one to one mapping case #### [H][FL 2] Proposal 3.1-5r1: For both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints indicated by a LP-WUS from system perspective. |
| docomo | Y |  |

  - The schemes for OOK to meet target requirements for LP-WUS

RAN1 agreed coding with rate matching for LP-WUS information carried by OOK.

Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding

Whether support < 3 information bits and coding details for such case, if supported

In last meeting, RAN1 agreed RM coding for < 2 information bits case, while it is still open whether 1 and 2 information bit is supported. Company views are summarized as below:

- Yes : [2][7][8][10][12][23]

- No : [3][4][5][27]

Motivation to support 1& 2 information bits is mainly for RRC connected mode, where only 1 or 2 UEs monitoring the same LP-WUS MOs.

Argument for not support 1 & 2 information bits provided by companies include,

- (1) Effective paging rate is relatively high, e.g., > 3%, resulting in limited power saving gain [4]

- (2) Coding in section 5.3.3.1 and 5.3.3.2 of 38.212 is simple repetition which can not efficiently improve coverage [4].

- (3) Resource overhead would be high due to less capacity of less than 2 bits [5]

- (4) Larger effort to discuss the coding length and repetition number for 2 information bits and 1 information bit to meet the same detection performance as that of RM coding [5]

- (5) Higher complexity because the OOK-based LP-WUR needs to store 3 decoding methods [5][3]

If 1 &2 information bits is supported, which row in table 5.3.3.1-1 and table 5.3.3.1-2 in section 5.3.3.1 and 5.3.3.2 of 38.212 is used for OOK needs further discussion. [2][7] propose to use 1st row, i.e., Qm=1 for OOK. [12] proposes to reuse the table by replacing Qm order with scaling factor to support different code block length.

Considering limited use case, potential higher UE complexity and additional efforts to support 1 &2 information bits, FL suggests  following,

#### [M][FL 2] Proposal 3.2-1:  Do not support 1 & 2 information bits for LP-WUS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW | Y |  |
| Nokia.1 | Y |  |
| OPPO | Y | Support the proposal. |
| Huawei, HiSilicon | Y | With 1~2 info bits, the power saving gain is impacted. Thus, no need to support these cases. |
| ZTE, Sanechips | Y | If supported, it should be indicated by some reserved states of the 5-bit information instead of using separate encoding scheme. |
| Qualcomm | N | For idle/inactive, it is fine given that there is no stringent delay requirement for paging in the level of ms. We discussed short MO LP-WUS for connected mode for short delay introduced for data scheduling. For connected mode, the flexibility of small number of information bits should be maintained. |

Code block length and Rate matching

For more than 2 information bits with RM coding, several companies provide evaluation results:

- [2][6][3][10] show code block length = 32 (no rate matching) is needed to meet performance requirement @ SNR=-3dB, for 5 information bits.

- [8] shows code block length after rate matching = 11~17 to meet performance requirement @ SNR=-3dB for 5 information bits.

- [18] shows code block length after rate matching =12 to meet performance requirement @ SNR = 1dB ~3dB for 5~7 information bits, additional repetition is needed to achieve SNR =-3dB.

Based on input from companies, it can be seen that, maximum LP-WUS duration is more than a slot, e.g., 64 chips (32 chip after RM coding + Manchester coding @ SNR=-3dB).

[37][38][39][40] also propose a LP-WUS occasion can be across multiple contiguous slots to meet performanc requirement.

#### (closed) [H][FL 2] Proposal 3.2-2: Support LP-WUS duration can be larger than one slot.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW | Y |  |
| OPPO | N | For LP-SS, the duration can be smaller than one slot. The baseline duration of LP-WUS should also be smaller than one slot, thus it can reduce the impact of TDD frame structure. In the case of coverage enhancement, we can further consider repetition of LP-WUS, it could transmit the LP-WUS via multiple slots. |
| Samsung | Y |  |
| CATT | N | It would have high complexity for OOK-based LP-WUR to decode the RM code for OOK over one slot. |
| Huawei, HiSilicon | Y |  |
| vivo | Y |  |
| LGE |  | We are confusing if the intension of the proposal is to exclude LP-WUS with the less than one slot length. We think it could be a Question so that the future discussion on more detailed LP-WUS duration. |
| ZTE, Sanechips | Y |  |
| FL |  | @OPPO, since LP-WUS carry [5] bit information bits, it is different from LP-SS to achieve the performance target. @CATT, not sure why higher complexity to decode the RM for OOK over one slot. @LG, the intention is not to preclude the smaller than one slot case, but to support both smaller and more than one slot case depending on the SNR. |
| docomo | Y |  |
| Panaosnic | Y |  |

- (closed)Issue 1: Rate matching after RM coding

As provided by companies above, up to 5 information bits with code block length =32 can meet @ SNR=-3dB. For better coverage case, e.g., the cell size is small so that SNR  is above -3dB, or UEs in good coverage  receiving UE specific LP-WUS, code block length can be smaller than 32.

When the code block length < 32, [4] [5] [6] observe that the Hamming distance is not optimal. NR RM coding is designed with minimum Hamming distance 16, for 32 bits output of RM coding. However, with shorter length of the codewords after rate-matching as defined in Section of 5.4.3 of 38.212 where the first E bits are selected to generate the codeword (f0,…,fE−1), the performance is not optimized as the minimum Hamming distance is not maximized. [6] provides a table (copied bleow for easy reading) showing minimum hamming distance after rate matching based on existing RM and rate matching in TS 38.212. K is number of bits before coding and L is code block length after rate matching. The black step line across the table represents the optimal linear code that has the minimum Hamming distance HD >= L/2. The larger K, the smaller the minimum Hamming distance can be achieved by the optimal linear code for a given L. Any combination of L and K above the step line in the table has a minimum Hamming distance >= L/2. Colour coding in the table indicates whether the NR RM code + rate matching can meet this goal. Green means the minimum Hamming distance goal is met so that a correlation based OOK detector would work, yellow means the goal is not met and a ML OOK detector is needed, and red means a set of codewords for such L and K combination does not exist. In addition, [6] evaluates performance of several code block length L=8/16/32 with minimum Hamming distance less than L/2, showing poor performance of FAR from others much higher than 10%. Therefore, enhancement is needed to improve performance for RM coding with rate matching.

Table 1 Minimum Hamming distance for RM code with rate matching from [6]

But according to [8], code block length L=10 ~17, can also achieve 1% MDR & 1% or 0.1% FAR @ SNR = -3dB, for 5 information bits. [3] shows code block length L=16 can achieve 1% MDR &FAR @SNR= -1.5dB for 5 information bits. [18] shows code block length L =12 can achieve 1% MDR &FAR @ SNR = 1dB for 5 information bits. Thus, it seems further enhancement on rate matching may not be needed at least for such code block lengths.

Mechanism to improve performance for code block length L <32 proposed by companies can be summarized as two categories:

- Category 1: No change of RM coding, enhancement for rate matching is applied

  - [4] proposes an interleaving before truncation to improve minimum Hamming distance for code block length <32, especially for code block length =8 and 16 to achieve minimum Hamming distance = 4 & 8. The proposed interleaving table and corresponding minimum Hamming distance after interleaved rate matching is shown below.

[4] also proposes rate-matching by truncating from the heading bits or tailing bits based on code block length, e.g., truncating from the heading bits for code block length <16 and truncating from tailing bits for code block length ≥16.

| - (a) 3-bit info | (b) 4-bit info | (c) 5-bit info |
| --- | --- | --- |

Figure 10  The minimum Hamming distance after rate-matching from [4]

Table 2 Interleaving for rate-matching to shorter 1st-order RM codes from [4]

| i | 0…4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21…24 | 25…31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| πi | 0…4 | 14 | 10 | 8 | 7 | 9 | 6 | 13 | 5 | 16 | 19 | 24 | 11 | 12 | 15 | 17 | 18 | 20…23 | 25…31 |

| (a) 3-bit info | (b) 4-bit info | (c) 5-bit info |
| --- | --- | --- |

Figure 11  The minimum Hamming distance after interleaved rate-matching guaranteeing length E=8 and 16 first-order RM codes [4]

  - [5] proposes uses a rate matching pattern (pattern for bit selection) for each code block length L after rate matching, to achieve maximum minimum hamming distance after rate matching under constraint that the output bits are collected consecutively. The optimal rate matching pattern is searched by compute (Method 3 for Alt 2 in the tdoc).

  - [6] proposes a new rate matching pattern (pattern for bit selection) for each code block length L after rate matching with maximum hamming distance≥ L/2. The optimal rate matching pattern are searched by compute (Alt 1 in the tdoc).

Table 2 Retained bits after 32-bit RM encoding from [6] for Alt 1

| Sequence Length L | Pattern for retained bits |
| --- | --- |
| 2 | [1 4] |
| 3 | [1 2 4] |
| 4 | [1 2 5 9] |
| 5 | [1 2 4 6 7] |
| 6 | [1 3 4 5 7 9] |
| 7 | [1 2 3 4 5 7 9] |
| 8 | [1 2 5 6 9 14 17 25] |
| 9 | [1 2 4 5 6 7 9 10 12] |
| 10 | [1 2 3 4 5 6 7 8 9 11] |
| 11 | [1 2 3 4 5 6 7 8 9 11 25] |
| 12 | [1 3 4 5 6 7 9 10 11 14 15 17] |
| 13 | [1 2 3 4 5 6 7 8 9 10 11 14 25] |
| 14 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17] |
| 15 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17 25] |
| 16 | [1 2 5 6 9 12 14 16 17 19 22 24 25 26 28 31] |
| 17 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25] |
| 18 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 28] |
| 19 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 27 28] |
| 20 | [1 2 3 4 5 6 7 8 9 10 11 14 15 16 17 22 23 24 25 31] |
| 21 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 19 21 25 27 28] |
| 22 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 29] |
| 23 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 28 29] |
| 24 | [2 3 4 5 7 8 9 10 11 12 13 14 17 18 19 21 22 24 25 27 28 29 30 31] |
| 25 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 18 19 21 25 26 27 28 29 31] |
| 26 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 22 23 24 25 26 27 28 29 30] |
| 27 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 21 22 23 24 25 26 27 28 29 30] |
| 28 | [2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 27 28 29 30 31] |
| 29 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30] |
| 30 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30 31] |
| 31 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31] |

- Category 2: RM coding is enhanced by new generator matrix G

The new generator matrix G consists of a subset of rows and columns from existing RM coding matrix M in TS 38.212. The matrix G is different for different code block length L and information bits length K before RM coding.

  - [5] proposes a subset matrix Gx,y from the RM mother code matrix Mi,k. The subset matrix Gx,y  is searched by compute (Method 3 for Alt 1 in the tdoc).

Table 1: The mapping relationship of (x, y) and (i_x, j_y) in Alt 1 and equivalent puncture bit index set in Alt 2 (Case 1) based on Method 1 ~ Method 3 from [5]

|  |  | detail |
| --- | --- | --- |
| OOK-4 with M=4 | Method 1 | Alt 1: (x=0~19, y=0~4) = (i_x =0~19; j_y =0~4) Alt 2: Puncture bit index set = {21~32} |
|  | Method 2 | Alt 1: (x=0~19, y=0~4) = (i_x =12~31; j_y =0~4) Alt 2: Puncture bit index set = {1~12} |
|  | Method 3 | Alt 1: (x=0~19, y=0~4) = (i_x =0~19; j_y =1~5) Alt 2: Puncture bit index set = {1~12} |
| OOK-4 with M=2 | Method 1 | Alt 1: (x=0~9, y=0~4) = (i_x =0~19; j_y =0~4) Alt 2: Puncture bit index set = {11~32} |
|  | Method 2 | Alt 1: (x=0~9, y=0~4) = (i_x =22~31; j_y =0~4) Alt 2: Puncture bit index set = {1~22} |
|  | Method 3 | Alt 1: (x=0~9, y=0~4) = (i_x =0~9; j_y =1~5) Alt 2: Puncture bit index set = {11~32} |
| Note: There are three methods to select the Gx,y (x=0…L-1; y=0..K-1) from the RM mother code matrix Mi,j (i=0…31; j=0..10) for Alt 1. - Method 1: The target mapping relationship of (x, y) and (i_x, j_y) are sequentially selected from the beginning of the matrix. - Method 2: The target mapping relationship of (x, y) and (i_x, j_y) are sequentially selected from the end of the matrix. - Method 3: Searched by computer based on the principle of maximizing the minimum Hamming distance. Accordingly, the puncture bit index set in Alt 2 can be derived based on Method 1 ~ Method 3. The different is that the selected matrix in Alt1 is from the beginning of the matrix and the selected bit index is from the beginning of the RM code block in Alt2. |  |  |

  - [6] proposes to use a new generator matrix Gx for each information bits length, where Gx is chosen from a subset of RM mother code matrix Mi,k. The generator matrix Gx are searched by compute to achieve minimum hamming distance ≥ L/2.

Proposed generation matrix G from [6] for Alt 2

| K=2 | G2=100111 for L in [2, 3, 5, 9] |
| --- | --- |
| K=3 | G3=100101100111010101110 for L in [6, 7, 10, 11, 13, 17, 18, 19, 21, 25] |
| K=4 | G4=100100011010110111110100100001100011110010111110001001010111 for L = [12, 14, 15, 20, 26, 27, 29] and G'4=101000101101011000011001010111100111100010110100110000111111 for L = [12, 14, 15, 22, 23, 26, 27, 29] |
| K=5 | G5=11010111001010001111111011011110101011011011001011010011100101100111101100001010111111101101000001010011101110001100010000011100110001010010100001000100001 for L in [24, 28, 30, 31] |
| Note: K is information bit length before RM coding |  |

In FL’s understanding, Category 2 with new generator matrix G selected from RM coding matrix Mi,k  is kind of a new coding scheme different from RM coding in 38.212 as agreed in last meeting, thus is not further considered.

FL invites companies to discuss whether existing rate matching in TS 38.212 is sufficient or enhancement is needed. If enhancement is needed, which alternative in Category 2 is preferred, with consideration of performance, complexity and standard effort.

#### [H][FL 2] Proposal 3.2-3: For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block length after rate matching < 32

- Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

- Alt 2: Rate matching after RM coding is applied, wherein the rate matching is performed by truncating from the heading bits or tailing bits according to a single code block length threshold. FFS the threshold value.

- Alt 3: Interleaving after RM coding is performed, and then the rate matching is performed as section 5.4.3 in TS 38.212. The single interleaving pattern is applicable to all code block length. FFS interleaver design.

- Alt 4: Rate matching after RM coding is applied, wherein Rate matching is performed according to on pre-defined pattern for bit collection for each code block length L, i.e., multiple patterns are to be specified. FFS the pattern for bit collection for each code block length.

#### [H][FL2] Proposal 3.2-3r1: For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block (coded bits) length after rate matching < 32

- Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

- Alt 1a: Information bits are repeated before RM coding, and rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

- Alt 2: Rate matching after RM coding is applied, wherein the rate matching is performed by truncating from the heading bits or tailing bits according to a single code block(coded bits) length threshold. The threshold is 16.

- Alt 3: Rate matching after RM coding is performed, where the rate matching is performed according to an interleaving pattern, and the interleaving pattern is applicable to all code block length. Interleaving pattern follows the following table:

| i | 0…4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21…24 | 25…31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| πi | 0…4 | 14 | 10 | 8 | 7 | 9 | 6 | 13 | 5 | 16 | 19 | 24 | 11 | 12 | 15 | 17 | 18 | 20…23 | 25…31 |

- Alt 4: Rate matching after RM coding is applied, wherein Rate matching is performed according to on pre-defined pattern for bit collection for each code block length L, i.e., multiple patterns are to be specified.

  - Predefined pattern 1:

|  | detail |
| --- | --- |
| OOK-4 with M=4 | Puncture bit index set = {1~4} |
| OOK-4 with M=2 | Puncture bit index set = {15~32} |
| OOK-4 with M=1 | Puncture bit index set = {8~32} |

  - Predefined pattern 2:

| Sequence Length L | Pattern for retained bits |
| --- | --- |
| 2 | [1 4] |
| 3 | [1 2 4] |
| 4 | [1 2 5 9] |
| 5 | [1 2 4 6 7] |
| 6 | [1 3 4 5 7 9] |
| 7 | [1 2 3 4 5 7 9] |
| 8 | [1 2 5 6 9 14 17 25] |
| 9 | [1 2 4 5 6 7 9 10 12] |
| 10 | [1 2 3 4 5 6 7 8 9 11] |
| 11 | [1 2 3 4 5 6 7 8 9 11 25] |
| 12 | [1 3 4 5 6 7 9 10 11 14 15 17] |
| 13 | [1 2 3 4 5 6 7 8 9 10 11 14 25] |
| 14 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17] |
| 15 | [1 2 3 4 5 6 7 8 9 10 11 14 15 17 25] |
| 16 | [1 2 5 6 9 12 14 16 17 19 22 24 25 26 28 31] |
| 17 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25] |
| 18 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 28] |
| 19 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 21 25 27 28] |
| 20 | [1 2 3 4 5 6 7 8 9 10 11 14 15 16 17 22 23 24 25 31] |
| 21 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 19 21 25 27 28] |
| 22 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 29] |
| 23 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 22 23 24 25 27 28 29] |
| 24 | [2 3 4 5 7 8 9 10 11 12 13 14 17 18 19 21 22 24 25 27 28 29 30 31] |
| 25 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 16 17 18 19 21 25 26 27 28 29 31] |
| 26 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 22 23 24 25 26 27 28 29 30] |
| 27 | [1 2 3 4 5 6 7 8 9 10 11 13 14 15 16 17 19 21 22 23 24 25 26 27 28 29 30] |
| 28 | [2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 27 28 29 30 31] |
| 29 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30] |
| 30 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 21 22 23 24 25 26 27 28 29 30 31] |
| 31 | [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31] |

Alt 1<  alt 1a, alt 2, alt 3, alt4 (CATT)  < alt 4 (QC) in terms of detection complexity and performance

Next plan: make decision this meeting or align the evaluation assumptions for further comparison?

Supported coded bits length L:

SNR:

Information bits:

Detection algorithm：

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y | Generally we are OK with the spirit of the proposal. The output 32 bits is too much, and rate matching is needed. |
| FW |  | OK |
| Nokia.1 |  | Alt.1 is preferred with information bits repeated by a factor X before encoding or by appending CRC bits to utilize more columns of Generator matrix used in TS 38.212 |
| OPPO |  | Support Alt1 One question, the rate matching is before or after Manchester coding? |
| Samsung |  | We prefer Alt 1, because the performance depends on the signal detection method (e.g., one sequence detection, ML-like, how to perform Manchester decoding,…) as well as the hamming distance of codeword. We think that it is enough to discuss the minimum code block length after rate matching to achieve the performance based on the legacy rate matching. |
| CATT |  | The Alt4 can bring the maximum minimum Hamming distance since the pre-defined pattern for bit collection comes from the computer search. For simplicity, the LP-WUS information bits can be fixed at 5 bits to corresponding to 32 codepoint values. If the information bits of LP-WUS are 3 bit o 4 bits, the 3 bit information or 4 bits information can be considered as the subset of 5 bits raw information using the number of selected codepoints with a mapping rule for full RM code. The mapping rule can be repetition or selecting 2 or 4 codepoint values out of 32 codepoint values to carry the wake-up information. The maximum minimum Hamming distance can still be guaranteed. Thus, multiple patterns may is not needed with considering fixed 5 information bit. |
| Huawei, HiSilicon |  | We are generally OK with this proposal. But clarification of Alt 3 is needed. The intension is to use another method to determine how the rate-matching is done, instead of introducing another operation before rate-matching. Thus we suggest to following updates to Alt 3 - Alt 3: InterleavingRate matching after RM coding is performed, and then the rate matching is performed as section 5.4.3 in TS 38.212. The single where the rate matching is performed according to an interleaving pattern, and the interleaving pattern is applicable to all code block length. FFS interleaver design. |
| vivo |  | We are fine with the spirit of the proposal. To check whether enhancement is needed, it would be helpful to clarify the factors which may result in different observations. If enhancement is needed, standard effort and complexity for both gNB and UE should be kept in mind, which is manageable within 2 meetings. |
| ZTE, Sanechips |  | Alt1 is preferred. We fail to see clear benefits from other options. Since the code block length >= 32 is needed to meet performance requirement @ SNR=-3dB, for 5 information bits, when the code block length is larger(e.g., larger than 16) the issue on Minimum Hamming Distance does not exist. |
| Qualcomm |  | The bottom line is at least for L=8 and 16, the minimum Hamming distance with simple Hadamard code should be achieved. |
| FL |  | Update as below to reflect companies’ comments: #### [H][FL 2] Proposal 3.2-3r1: For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block length after rate matching < 32 - Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212. - Alt 2: Rate matching after RM coding is applied, wherein the rate matching is performed by truncating from the heading bits or tailing bits according to a single code block length threshold. FFS the threshold value. - Alt 3: InterleavingRate matching after RM coding is performed, and then the rate matching is performed as section 5.4.3 in TS 38.212. The single where the rate matching is performed according to an interleaving pattern, and the interleaving pattern is applicable to all code block length. FFS interleaver design - Alt 4: Rate matching after RM coding is applied, wherein Rate matching is performed according to on pre-defined pattern for bit collection for each code block length L, i.e., multiple patterns are to be specified. FFS the pattern for bit collection for each code block length. |
| Huawei, HiSilicon |  | Since we are going to make the down-selection among the alternatives, we suggest make the alternatives clearer for further down-selection. - Alt 2: Rate matching after RM coding is applied, wherein the rate matching is performed by truncating from the heading bits or tailing bits according to a single code block length threshold. FFSAnd the threshold value is 16. - Alt 3: InterleavingRate matching after RM coding is performed, and then the rate matching is performed as section 5.4.3 in TS 38.212. The single where the rate matching is performed according to an interleaving pattern, and the interleaving pattern is applicable to all code block length. FFS The interleaver design follows the following table. i; 0…4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19; 20; 21…24; 25…31. πi; 0…4; 14; 10; 8; 7; 9; 6; 13; 5; 16; 19; 24; 11; 12; 15; 17; 18; 20…23; 25…31. |

- Issue 2: Candidate code block length

Candidate code block length after rate matching can be different, depending on several factors, such as (a) SNR/coverage, e.g., the cell size is small so that SNR in the entire cell is above -3dB, or UEs in good coverage  receiving UE specific LP-WUS, (b) length of information bits before coding, e.g., 3~ 5 information bits before RM coding, (c) Reception algorithm, e.g., single sequence detection or ML detection, (d) target FAR from noise and FAR from other codepoints.

- [8] proposes to support any code block length after channel coding, which is determined by min (Y, (F*M)/2), where F is the number of available WUS OFDM symbols with Manchester encoding, M is the number of OOK symbols per OFDM symbol, and Y=1, 3, and 32 for 1-bit, 2-bit, and ≥2 bits information, respectively. Then, the code block length can be any value between 1 ~ 32. [2] also supports code block length can be any value between 1 ~ 32.

- [3][4][5][6][18][27] propose to support limited number of code block length L.

- [4] proposes to restrict candidate length is to avoid complicated MO design. The number of candidates codeword lengths is limited to, e.g., up to 4 lengths from {32, 28~26, 20~16, 12~8}, based on minimum Hamming distances (e.g., {16, 12, 8, 4}).

- [3] [5][18] propose to support code block length for LP-WUS within a slot. [5][3] consider integer multiples of 7, e.g., 7, 14 and 28 for M=1, 2, 4. In addition, [5] considers code block length 5 bits, 10 bits and 20 bits for M=1, 2, 4 to support LP-WUS with preamble within a slot. [18] considers code block length=12 for LP-WUS within a slot assuming up to 2 symbols reserved for NR signal transmission.

- [6] propose a set of candidate code block length L < 32 in addition to L=32 for each number of information bits K before coding, to achieve optimal hamming distance (≥L/2). L=[4, 6, 7, 10, 11, 13, 17, 18, 19, 21, 25] for K=3, L=[8, 12, 14, 15, 20, 22, 23, 26, 27, 29] for K=4, and [16, 24, 28, 30, 31] for K=5.

- [27] proposes to support always 32 bits for 3~5 information bits to ensure performance.

#### [M][FL 2] Proposal 3.2-4: For > 2 information bits with RM coding, support one of the alternatives for candidate code block length after rate matching

- Alt 1: support any code block length from 1 ~ 32.

- Alt 2: support up to X candidate code block length, at least including code block length=32. FFS X, and other values.

#### [M][FL 2] Proposal 3.2-4r1: For > 2 information bits with RM coding, support one of the alternatives for candidate code block length after rate matching

- Alt 1: support any code block (coded bits) length from [3] ~ 32.

- Alt 2: support up to X=4? candidate code block (coded bits) length, at least including code block length=32. FFS X, and other values.

- Alt 3: support up to 1 candidate code block length,FFS the sequence length.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Can be discussed after Issue 1. |
| FW |  | OK |
| Nokia.1 | Y |  |
| OPPO |  | How to understand the code block length, it means the length of output bit sequence after rate matching? In our views, the number of output bit sequences after rate matching is calculated based the resource allocation. If the code block length means the length of output bit sequence after rate matching, we think it depends on the resource allocation. No need to limit the code block length. After rate matching, we can get the output bit sequences mapping to the resource. |
| Huawei, HiSilicon |  | Prefer Alt 2, since it can reduce the complexity of gNB implementation. Also, allowing all possible lengths may be not feasible for LP-WUS design (e.g., odd value for M=4). |
| ZTE, Sanechips |  | Larger code block length after rate matching should be larger than 32 to achieve msg3 coverage. |
| Qualcomm | Y |  |

As discussed in proposal 3.2-2, maximum OOK WUS duration can be more than one slot. [2][8][39] discuss, when mapping LP-WUS to multiple slots, some of the symbols in the slots must be reserved for other NR channels/signals, e.g., for PDCCH or SSB, or for TDD UL symbols, so only a subset of OFDM symbols is available for LP-WUS mapping. [12] also considers collision handling between LP-WUS/LP-SS and NR signals. [2][8][39] propose the available or unavailable symbols are provided to avoid collision, e.g., [8] proposes to configure number of slots (A) used for LP-WUS (A=1,2 or 3 slots) and an additional parameter indicating the available OFDM symbols in each slot (e.g., by configuring a 14bit bitmap or by using a SLIV value as done for PDSCH/PUSCH). Ideally, if gNB can ensure the available symbols in each MO does not collide with SSB or UL symbols, same code block length in each MO can be guaranteed by configuring same number of available symbols in each MO. However, considering SSB and UL symbols only appear in some slots, it would be difficult to configure a pattern for available symbols commonly applied to all MOs without collision with SSB and UL symbols. [2][8] propose that LP-WUS maps around certain symbols based on pre-defined rule, e.g., [2] proposes LP-WUS maps around SSB/UL symbols in addition to configured unavailable symbols. Then, available symbols excluding SSB/UL symbols in a MO would be different in different MOs.

If actual number of symbols for LP-WUS transmission in different MOs may be different as discussed above, one problem is, the actual code block length in different MOs varies, by assuming the actual code block length = (actual available number of chips in a MO)/2 (Manchester coding is applied after rate matching). If the actual code block length varies, the performance in different MO is un-uniform, and it also increases LP-WUS reception complexity. [2] proposes, gNB configures MO resource and code block length. Within configured MO duration, the code block length of LP-WUS is fixed, e.g., configured by gNB, and LP-WUS maps to a number of avaible symbols within the MO according to the code block length, and remaining symbols in the MO is not used for LP-WUS.

#### [H][FL 2] Proposal 3.2-5:  Down-select between Alt 1 and Alt 2 for code block length determination in a MO

- Alt 1: The code block length can vary in different MOs, e.g., the code block length is determined by the total number of available symbols within the MO, which may vary in different MO.

- Alt 2: The code block length is same for all MOs. FFS how to derive/configure the code block length.

  - FFS what if the available symbols are not sufficient for the target code block length

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Alt 2 is preferred to reduce UE receiving complexity |
| FW |  | OK |
| Nokia.1 |  | Alt.2 as LR cannot adapt the codeword size dynamically to perform ML detection over all possible codepoints depending on the MO used for scheduling. |
| OPPO |  | Support Alt2. Worry about the performance of different MO for Alt1. If different MOs can have different code block length, and different MOs transmit different codepoint, it may cause the different performance within different MOs. |
| Samsung |  | We are fine to discuss it for down-selection. For Alt1, we are not sure how UE can determine the available symbols. Not only SSB, TDD UL symbol, the other channels can be overlapped with LP-WUS MO resources, especially for RRC CONNECTED mode. Therefore, it should be further discussed if Alt 1 is adopted. |
| CATT |  | This can be discussed later after we have progress on proposal 3.2-2 |
| Huawei, HiSilicon |  | This issue is related to the discussion on how to configure the resources of a MO, which will be discussed in 9.6.2. If by a mechanism the number of symbols in every MO is the same, then this code block length issue disappears. BTW, this is also the reason we propose to limit the X candidate code block length, since the symbols in a MO may be not arbitrarily configured. |
| vivo | Y | We support Alt 2, for uniform performance as well as simpler LP-WUR processing. |
| ZTE, Sanechips | Alt2 | The gNB configure the same resources for LP-WUS across MOs. It seems Alt1 allows the gNB to configure different number of available symbols for different MO |
| Qualcomm | Y | Support Alt 2. |

Whether support scrambling

One FFS point for coding is, whether scrambling in the same section of coding is applied or not. For >2 bit, there is no scrambling in the same section of RM coding. For 1 or 2 information bit, if Qm=1 in table 5.3.3.1-1 and table 5.3.3.1-2 is used for coding, there is no scrambling, otherwise, scrambling by x or y is needed.

Besides, some companies [5][7][8] discuss another kind of scrambling, i.e., scrambling after coding and before Modulation, similar as scrambling operation in TS 38.211, e.g., for inter-cell interference randomization [7][8], or for security between gNB and UE [5]. [8] supports scrambling, while [5] [7] object the scrambling considering additional complexity and may not actually improve coverage.

#### [M][FL 2] Proposal 3.2-6:  Do not support scrambling for LP-WUS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nokia.1 | N | If MC encoding is used and LP-WUS coverage is limited, no need for scrambling. |
| OPPO |  | Support the proposal. |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |
| Qualcomm | N |  |

Whether repetition is needed in addition to rate matching

For FFS point in the agreement, whether further repetition before or after Manchester coding is needed, company views are summarized as below:

- Yes: [5] [8] [24]

- No: [2][3][4][10][16][23][26]

The argument to not support repetition includes (1) RM coding with code block length=32 is sufficient to meet performance requirement @ SNR=-3dB, no need of additional repetition, (2) if code block length is multiple integers of 32, repetition can be achieved by rate matching, i.e., cyclic extension of the buffer,  (3) If code block length is not multiple integers of 32, [2][3] provide evaluation results showing rate matching to a longer code block length is better than the case with shorter code block length + repetition.

#### [M][FL 2] Proposal 3.2-7:  Do not support further repetition after coding and rate matching .

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Can be discussed after issue 1 in 3.2.2. It depends on the length of the output sequence. |
| Nokia.1 | Y |  |
| OPPO |  | Support the proposal. |
| Huawei, HiSilicon | Y | The Msg3 coverage is already reached via 32 coded bits. |
| ZTE, Sanechips | Y | Repetition can be achieved by rate matching with more flexibility. |
| Qualcomm | N | Rate matching itself can achieve repetition effect. |

Others

[15][26] propose CRC, and [9] proposes to add known bits to the information bits (instead of CRC) to improve the detectability and coverage of LP-WUS. In FL’s understanding, CRC is already excluded, and then, appended known bits similar as CRC is not further considered.

  - Overlaid OFDM sequence for LP-WUS

Overlaid Sequence Design

(closed)3.3.1.1: Number of overlaid OFDM sequences per OOK ON chip for RRC connected mode

Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

The maximum number of overlaid OFDM sequences per OOK ON chip for RRC connected mode is still open.

- The maximum number of overlaid OFDM sequences per OOK ON chip is same as RRC idle state without UE capability: [2][10][5][23]

- [2][10] think, the same consideration on the LR complexity applies equally to connected mode and idle/inactive. If supporting different UE capability, it complicats UE grouping.

- The maximum number of overlaid OFDM sequences per OOK ON chip is per UE capability: [6][7][15]

FL suggests to go with unified design for RRC idle and connected mode without UE capability.

#### [H][FL 2] Proposal 3.3-1: For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell,

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Generally OK. |
| FW | Y |  |
| Nokia.1 | Y |  |
| OPPO |  | Support the proposal. |
| Samsung |  | Because the maximum number of candidates overlaid sequences depends on the length of ON pulse duration which is related to SCS as well as M value, we prefer to have the additional UE capability reporting within the supported maximum values considering the case for 60kHz, 120kHz SCS (which was already agreed to support). |
| CATT | Y |  |
| Huawei, HiSilicon | Y |  |
| vivo | Y |  |
| LGE | Y |  |
| ZTE, Sanechips | Y | The number of candidates overlaid sequence should be configurable |
| FL |  | Update as below to reflect companies’ comments： #### [H][FL 2] Proposal 3.3-1r1: For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: For FR2, UE capability reporting on the supported number of candidates overlaid sequences within the range given by the maximum numbers |

(closed)3.3.1.2: Sequence length Lzc for M=1

RAN1 made following working assumption and agreement for sequence length Lzc. The sequence length Lzc for M>1 is crystal clear based on the agreement below. For M=1 case, different companies have different understanding. [4] [23] [27] think Lzc=128 based on DFT size 2^n per working assumption, [2][3][8][16][26] support Lzc =132.

According to FL’s understanding, the working assumption in RAN1 118bis has not been confirmed due to different views on DFT size 2^n.  RAN1 agreed overlaid sequence length Lzc = 12*X/M for at least M>1 case in RAN1 120, implying no restriction of DFT size 2^n. For M=1 case, it is reasonable to support unified design for all M values.

Working Assumption in RAN1 118 bis For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

Agreement in RAN1 119 At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:LZC=2kM, with 2k≤12∗X - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2:LZC=12∗XM - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Agreement in RAN1 120 At least for M>1, for the overlaid OFDM sequence length, support - Alt2:LZC=12∗XM - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

#### [H][FL 2] Proposal 3.3-2: For M=1, the overlaid OFDM sequence length LZC=12∗X, X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW | Y |  |
| Nokia.1 | Y |  |
| OPPO | Y | Support the same design of overlaid OFDM sequence length |
| Samsung | Y |  |
| vivo | Y |  |
| LGE |  | To respect WA on DFT size of 2^n, we prefer the overlaid sequence length for M=1 is the same as DFT size. If we are the only company not to support the proposal, we would be fine with it, but in this case, we suggest to add “FFS how to map the overlaid OFDM sequence to DFT”. |
| ZTE, Sanechips |  | If the DFT size is 132, it limit the gNB to implement this only based on software, e.g., prestore. Based on the chip implementation, the DFT size should be some fixed values and some typical values are 128,256,384. |

3.3.1.3: How to select root and CS for overlaid OFDM sequences

- Issue 1: Optimal set of sequence roots

Companies discuss whether any root between 1~ (Bzc -1) or only some particular root is proper for overlaid OFDM sequence.

[2][3] [4] discuss the impact of frequency offset on different root. [2] shows similar auto-correlation for all roots for frequency offset 0~ 5ppm, and similar timing offset, e.g., no larger than one sample with 7.68MHz sample rate, for all roots. [2] also provides sequence detection performance for different root, showing similar MDR & FAR performance for different roots assuming 2ppm frequency offset, SNR gap is no larger than 0.7dB for best and worst roots, for each M value. So [2] thinks no need to specify a specific set of roots in the standard, it can be up to gNB to configure any root between 1 ~ Bzc-1. [3] says, maximum number of supported ZC sequences with CS in a root index can be different for different root index. The maximum number is also related with the CS separation.

Using similar design principle of NR preamble restricted set type A, which is designed for frequency error up to one subcarrier SCS, a set of optimized ZC sequence root index which can support at least 2 ZC sequences with CS for a root index even for larger value of CS separation >4 can be found. These root sets can be specified for LP-WUS. [4] provides auto-correlation of different root for frequency offset 0~ 6ppm, e.g., root q=1, 2,3 and 32, observing some particular root suffers significant performance degradation. [4] also provides timing offset of different root for 30kHz frequency offset (>10ppm), showing significant timing offset for particular root. So [4] thinks necessary to specify a specific set of roots.

[8] discusses the optimal set of sequence roots from another perspective other than impact of frequency offset, i.e., minimum cross correlation to reduce FAR between sequences with different roots. [8] assumes quite small frequency offset, 0.5ppm, assuming OFDM WUR has superior capability over the OOK WUR.

[18] provides MDR for different roots for 4 overlaid OFDM sequence, showing ~ 1dB performance difference between sequence root q∈{1,13,18,26,27,35,40,52} without any frequency offset.

[5] proposes to specify 16, 8, 4 root for M=1, 2 and 4 respectively, with value from 1 ~8, or 1~4 or 1~2 for M= 1, 2 and 4 respectively, together with complementary roots, based on PRACH root list logic.

Besides, [2][5][18] proposes to use complementary roots, e.g., root q and root (Bzc -q) to reduce sequence correlation complexity, similar as NR PRACH or LTE PSS logic.

In summary, [2][27] prefer no restriction on any root by specification, while [3][4][5][8][18] prefer to specify a set of roots.

Preferred roots provided by companies are summarized in the table below. It can be seen that, the preferred root by different companies is different.

| Proposed roots by [3] |  |
| --- | --- |
| The length of ZC | Optimal ZC root sequences |
| L = 131 | q = [3 4 6 7 8 15 16 23 25 27 30 31 32 36 37 39 40 41 45 46 47 50 51 53 54 57 59 60 62 64 67 69 71 72 74 77 78 80 81 84 85 86 90 91 92 94 95 99 100 101 104 106 108 115 116 123 124 125 127 128] |
| L = 61 | q = 3,6,7, 11, 16, 17,18,19,23,25,27,29,32,34,36,38,42,43,44,45,50,54,55,58 |
| L = 31 | q = 3,4,6,7,8,12,14,17,19,23,24,25,27, 28 |
| Proposed roots by [4] |  |
| L = 127 | q = 1, 32, 42, 63, 64, 85, 95, 126 |
| L = 61 | q = 1, 15,20, 30, 31, 41, 46, 60 |
| L = 59 | q = 1, 15, 20, 29, 30, 39, 44, 58 |
| L = 53 | q = 1, 13, 18, 26, 27, 35, 40, 52 |
| L = 31 | q = 1, 8, 10, 15, 16, 21,23, 30 |
| L = 29 | q = 1, 7, 10, 14 ,15, 19, 22, 28 |
| Proposed roots by [8] |  |
| L=131 | q=2, 65, 68, 88 |
| L=61 | q=11,35,42, 44 |
| L=31 | q=5,12,23,25 |
| Proposed roots by [5] |  |
| L=131 | q=1, 130, 2, 129, 3, 128, 4, 127, 5, 126, 6, 125, 7, 124, 8, 123 |
| L=61 | q=1, 60, 2, 59, 3, 58, 4, 57 |
| L=31 | q=1, 30, 2, 29 |
| Proposed roots by [18] |  |
| L = 53 | q = 1, 13, 18, 26, 27, 35, 52, for overlaid OFDM sequence case |

Considering diverged views on the necessity of specify certain roots, and quite different candidate root values proposed by companies supporting to specify certain roots, FL suggests to not specify any root in the specification and it’s up to gNB to configure a proper root value by implementation.

#### [H][FL 2] Proposal 3.3-3: For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC,LZC is the overlaid OFDM sequence length.

#### [H][FL 2] Proposal 3.3-3r3: For the root(s) used for overlaid OFDM sequences, gNB can configure any value from the roots which are robust to frequency error, where the roots for M=1, and 2 are provided by

| L = 61 | q = 1, 15,20, 30, 31, 41, 46, 60 |
| --- | --- |
| L = 31 | q = 1, 8, 10, 15, 16, 21,23, 30 |
| L=131 | FFS |

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y | Generally OK with the proposal. gNB can configure the best roots by implementation. |
| FW |  | OK |
| Xiaomi | Y | Generally OK with the proposal. gNB can configure the best roots by implementation. |
| OPPO |  | OK with the proposal. |
| Samsung | Y |  |
| CATT |  | A simple way using complementary roots, e.g., root q and root (Bzc -q) to reduce sequence correlation complexity, similar as NR PRACH or LTE PSS logic should be considered. |
| Huawei, HiSilicon | N | Root values should be selected to be robust to frequency error. |
| vivo | Y | Support the proposal. It’s up to gNB to configure best roots by implementation. |
| ZTE, Sanechips | Y |  |

- (closed)Issue 2: Number of roots and number of CS for multiple overlaid OFDM sequences in a cell

The purpose of defining a limited number of overlaid candidate sequences is to limit the maximum memory usage by a LP-WUR for storing the candidate sequences. [6] thinks, ZC sequences with the same root and different cyclic shifts (CSs) are essentially the same sequence from memory usage perspective. UE only needs to store the sequence once, e.g., the sequence for the zero CS. Hence, only the number of roots matters for storing the candidate overlaid sequences in memory. For one extreme case, single root is supported, and multiple overlaid OFDM sequences is generated by different CS [4][11]. [2] [5] [18] propose supporting a root pair of root q and root (Bzc-q). Required memory is same as single root case, i.e., only sequence with root q needs to be stored, and the sequence with root (Bzc-q) can be generated by flipping the sign of imaginary part, and single correlator can be used to detect both.

Required UE memory is not the single factor to determine maximum number of roots Nmax-r. Maximum number of CS to support maximum number of overlaid OFDM sequences with desirable performance is also critical to determine the maximum number of roots. In other words, CS separation for 16/ Nmax-r, 8/ Nmax-r, 4/ Nmax-r CS for M=1, 2, 4 should provide sufficient room for timing error. [2] [4][7] analysis that minimum CS separation should be no smaller than 2*maximum timing error. In addition, some margin is needed to account for channel effects or non-ideal OOK waveform [2][7]. [2] observes 16/8/4 CS would be challenging with limited margin thus 8/4/2 CS is preferred, i.e., with CS separation ≥ 16, assuming 0.9us timing error for M values.  [7] observes up to 4/2/1 CS is feasible, assuming same maximum timing error for OOK and OFDM-based LP-WUR, 3.599µs, 2.208µs, and 1.581µs for M = 1, 2, and 4 respectively. [8] provides FAR performance for different CS separation, showing CS separation≥ 10 is needed, assuming up to 0.16us timing error. [12] observes 16/8/4 CS is workable for M = 1, 2, and 4, assuming delay spread (without consideration of timing error) is less than 1 us. [27] proposes CS separation =16, thus 8/4/2 CS for M = 1, 2, and 4.

Company views on Maximum number of roots and corresponding maximum number of roots are summarized as bleow.

Maximum number of roots Nmax-r:

- 16/8/4 for M=1/2/4: [5]

- 4/2/1 for M=4/2/1: [6]

- 4 roots for all M values: [8][7][16]

- 2 roots for all M values: [2][27] [12]

- 1 root for all M values: [4][21][11]

Maximum number of CS Nmax-cs:

- 16/8/4 for M=1/2/4: [4][11][12]

- 8/4/2 for M=1/2/4: [2][27]

- 4/2/1 for M=1/2/4: [7][16]

- 1 for all M values: [5]

FL suggests to go with the median number 4 roots, with trade-off between detection complexity, memory and robustness to timing error. Complementary roots, i.e., root q and root (Bzc-q) is supported to reduce memory and reduce detection complexity

#### [H][FL 2] Proposal 3.3-4: For maximum number of root(s) and maximum CS used for overlaid OFDM sequences, support 4 roots and 4/2/1 CS for M=1/2/4 respectively.

- Complementary roots of root q and root (Bzc-q) is supported.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Sequences generated by different roots have worse orthogonality performance than sequences generated by the same root. For M=2/4, we expect fewer roots number is better. For example, 2/1 roots for 4 CS for M=2/4 respectively. |
| FW |  | Just a minor clarification/suggestion, if a maximum number of roots of 4 is considered, the corresponding number of CSs will be the minimum. Proposal 3.3-4: For maximum number of root(s) and maximum minimum number of CSs used for overlaid OFDM sequences, support 4 roots and 4/2/1 CSs for M=1/2/4 respectively. |
| Nokia.1 |  | Complementary roots of q, which is (Bzc – q), is a complex conjugate of original sequence with q. Thus, may not be needed. |
| OPPO |  | For the number of candidate overlaid OFDM sequence per OOK ON, is any number except for the maximum value can be supported? If we also support 8 candidate overlaid OFDM sequences for M=1, no need to limit the number of CS for each M values. |
| Samsung | Y |  |
| CATT |  | We are confused with Proposal 3.3-4 and Proposal 3.3-3, seems that they are contradictory. |
| Huawei, HiSilicon | N | We prefer to use different cyclic shifts for the same cell, since different cyclic shifts can provide better orthogonality between sequences. Also, with same root and different cyclic shifts, the correlation operation is easier to be done. More than 1 roots can be further discussed if it is justified that the number of cyclic shifts are not enough. |
| Samsung | Y |  |
| vivo |  | Just to clarify the meaning of sub-bullet. Based on proposal 3.3-3, it means, gNB can configure any value for root q1 and root q2. With sub-bullet in proposal 3.3-4, it means, in case of 4 roots, it’s up to gNB to configured any value for root q1 and root q2, but gNB ensures another two roots are root (Bzc-q1) and root (Bzc-q2) ? |
| ZTE, Sanechips |  | It is recommended that root index and CS used for overlaid OFDM sequence can be configured by eNB by the same way of NR PRACH preamble configuration, such as using a starting root index, supported sequence number and Ncs. |
| FL |  | Update the proposal to reflect companies’ comments: #### [H][FL2] Proposal 3.3-4r1: Support maximum 4 roots to be used for overlaid OFDM sequences for M=1/2/4. - 2 pairs of complementary roots, i.e., root q and root (Bzc-q) are supported for 4 roots. |

3.3.1.4 How to configure multiple overlaid OFDM sequence

Regarding the configuration signaling, [14][18][21] proposes the root is derived by cell identifier or paging group ID, [2][4][7][8] propose it to be configurable by the network to have more flexibility.

[2][7][14][18] propose the number of CS and corresponding cyclic shift value is derived by the number of sequences and number of roots. [2][3][19] propose the overlaid ZC sequences is enumerated in increasing order of first increasing cyclic shift of a root ZC sequence and then in increasing order of the root index.

#### [M][FL 2] Proposal 3.3-5: For LP-WUS overlaid OFDM sequence configuration in a cell,

- gNB configures number of sequences N1 (up to 16/8/4 for M=1/2/4), number of roots N2 and root index(s).

  - The root can be any value between 1 ~ Bzc-1.

  - FFS how to configure root value.

- The number of CS is derived by N1/N2

- N1 sequences are enumerated in increasing order of first increasing cyclic shift of first root sequence, and then in increasing order of the root sequence index in case of N2>1.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Generally OK with the spirit of this proposal. |
| Nokia.1 |  | Cross correlation of q with q’ (complementary ones) have matched real part. Thus, should be avoided. Limiting the number of roots to prime numbers less Bzc-1 is better. |
| OPPO |  | Generally OK with the proposal. N1/N2 is better. |
| Huawei, HiSilicon |  | Generally OK, but this can be discussed after Proposal 3.3-4. |

3.3.1.5 Overlaid OFDM sequences for different cells

[2] [4] [7] [8] [14][23] propose to use different overlaid OFDM sequence for different cells to minimize inter-cell interference. [4] analyze that, ZC sequences generated by the same root but difference cyclic shifts may not reduce inter-cell interference, since the timing difference between different cells may be larger than the minimum cyclic shift which results in strong interference, while different root helps because the aperiodic cross-correlation between sequences with different root can be kept small even with large timing difference among cells. [8] thinks it can be up to gNB implementation to configure same or different sets of sequences for different cells.

#### [M][FL 2] Proposal 3.3-6: Different overlaid OFDM sequences based on different roots are used for different cells.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | We think it is more of a implementation issue, dependent on gNB configuration. |
| FW |  | OK |
| Nokia.1 | Y |  |
| OPPO |  | It depends on gNB implementation. If the previous proposal 3.3-3 is agreed, gNB can configure different roots for different cells. |
| Huawei, HiSilicon | Y | OK with the principle, but it is difficult to specify this. Since there will anyway be two far-away cells using the same root(s). |
| ZTE, Sanechips |  | It is up to gNB configuration. |
| Panasonic |  | It can be up to implementation of gNB configuration, unless the intention of the proposal is to use Cell ID to determine the root sequence? |
| Qualcomm |  | No need to mandate different sequences for different cells. It can be upto network configuration. |

How to carry information by OFDM sequences

Agreement For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives: - Alt 1: Raw information bits are mapped to sequence(s) - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK - Alt 3: Codepoint/Subgroup is mapped to sequence(s)

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

- Issue 1: down-selection of 3 alternatives for WUS information carried by the overlaid OFDM sequence

RAN1 agreed to down-select from 3 alternatives for WUS information carried by the overlaid OFDM sequence(s).  Companies’ preference are summarized as below

- Alt 1: [2][6] [7] [8] [11][12] [13][14][15] [17][23]

- Alt 2: [3][4][16][18]

- Alt 3: [5][8][19][22]

Argument to support each alternative are summarized in the table below.

|  | Reason to support corresponding Alt |
| --- | --- |
| Alt 1 | - Simplest design with low detection complexity [2][6] - Early termination in case of repetition [6][11][7] - Similar performance as Alt 2 &3 [2] |
| Alt 2 | - Better performance than Alt 1 [4] - Joint detection of overlaid sequence and OOK ON/OFF pattern to support early termination [4]. - Easier to ensure code block length an integer multiple of a bit size that can be carried by the ZC sequences in accordance with M value [3][9] |
| Alt 3 | - Better performance by well-designed combination of overlaid OFDM sequences in multiple OOK ON chips [8], or by detection in only target OFDM symbols instead of all OFDM symbols [5] - Lower power consumption and complexity because UE only detects few numbers of symbols carrying subgroup index for this UE [5]. |

[2][4] [11] provide simulation results for comparison.

- [2] compares Alt 1, Alt 2 and Alt 3, showing similar performance for three alternatives, with same overhead.

- [2][6] explains Alt 1 with repetition can obtain coherent processing gain by soft combining. For Alt 2, the RM coding gain cannot be fully achieved when more than one bit is carried by overlaid sequences in an OOK ON chip, due to hard decoding for these multiple bits carried by overlaid sequences, and the hard decoding performance is determined by the correlation property of overlaid sequence [2].

- [4] compares Alt 1 and Alt 2, showing ~2dB gain by Alt 2 for MDR, and slightly lower FAR from noise by Alt 2, with same overhead of 4 OFDM symbols for M=4.

- [11] compares Alt 1 and Alt 3, showing equal or less number of OFDM symbols to achieve 1% MDR and 1% FAR at both -0.5 dB and -3 dB SNR values by Alt 1.

Several companies mentioned that, Alt 3 is unclear. Besides, companies who discuss Alt 3 show different understanding of Alt 3. To help understanding of Alt 3, FL copies description of Alt 3 from proponent companies [8][5].

[8] proposes each codepoint index can be mapped to a sequence and UE detects its sequence to find the codepoint/subgroup. The sequence can be composed of multiple overlaid sequences in multiple OOK ON chips. A lookup table can provide mapping of each of 32 codepoints to a sequence based on the number of overlaid sequence candidates. An example is provided in Table 6 for a case that there are 4 OFDM sequence candidates per OOK ON chip/symbol and the entire sequence spans over 3 OOK ON symbols.

Figure 13: Decoding codepoint value via sequence detection in [8]

Table 6: Example of codepoints/subgroup index mapping to OFDM sequences (3 OOK ON symbols, 4 OFDM sequence candidates) for Alt1 and Alt3 from [8]

| Codepoint (subgroup index) | Alt1: OFDM sequence pattern in (OOK ON symbol 1, OOK ON symbol 2, OOK ON symbol 3), {s1: 00, s2: 01, s3:10, s4:11} | Alt3: OFDM sequence pattern in (OOK ON symbol 1, OOK ON symbol 2, OOK ON symbol 3) with optimized ordering |
| --- | --- | --- |
| Z1 | s1 s1 s1 | s1 s1 s1 |
| Z2 | s1 s1 s3 | s2 s2 s2 |
| Z3 | s1 s2 s1 | s3 s3 s3 |
| Z4 | s1 s2 s3 | s4 s4 s4 |
| Z5 | s1 s3 s1 | s3 s2 s1 |
| Z6 | s1 s3 s3 | s4 s1 s2 |
| Z7 | s1 s4 s1 | s1 s4 s3 |
| Z8 | s1 s4 s3 | s2 s3 s4 |
| Z9 | s2 s1 s1 | s4 s3 s1 |
| Z10 | s2 s1 s3 | s3 s4 s2 |
| Z11 | s2 s2 s1 | s2 s1 s3 |
| Z12 | s2 s2 s3 | s1 s2 s4 |
| Z13 | s2 s3 s1 | s2 s4 s1 |
| Z14 | s2 s3 s3 | s1 s3 s2 |
| Z15 | s2 s4 s1 | s4 s2 s3 |
| Z16 | s2 s4 s3 | s3 s1 s4 |
| Z17 | s3 s1 s1 | s2 s1 s1 |
| Z18 | s3 s1 s3 | s1 s2 s2 |
| Z19 | s3 s2 s1 | s4 s3 s3 |
| Z20 | s3 s2 s3 | s3 s4 s4 |
| Z21 | s3 s3 s1 | s3 s1 s1 |
| Z22 | s3 s3 s3 | s4 s2 s2 |
| Z23 | s3 s4 s1 | s1 s3 s3 |
| Z24 | s3 s4 s3 | s2 s4 s4 |
| Z25 | s4 s1 s1 | s4 s1 s1 |
| Z26 | s4 s1 s3 | s3 s2 s2 |
| Z27 | s4 s2 s1 | s2 s3 s3 |
| Z28 | s4 s2 s3 | s1 s4 s4 |
| Z29 | s4 s3 s1 | s1 s2 s1 |
| Z30 | s4 s3 s3 | s2 s1 s2 |
| Z31 | s4 s4 s1 | s3 s4 s3 |
| Z32 | s4 s4 s3 | s4 s3 s4 |

[5] proposes scheme 1 for Alt 3.  By scheme 1, one overlaid sequence can carry part of subgroup specific indication or the common indication for all subgroups independently in one PO. Table below provides example of OOK-4 with M=4. It can be seen that, OFDM-based LP-WUR only needs to detect the overlaid OFDM sequences on one or two OOK ON chips which is associated with its own subgroup index and common indication

Table 9: The mapping between candidate sequence indexes in each OOK ON chip and wakeup information for 32 subgroup UEs (Scheme 1) from [5]

| OOK ON symbol index | Candidate Sequence index #1 | Candidate Sequence index #2 | Candidate Sequence index #3 | Candidate Sequence index #4 |
| --- | --- | --- | --- | --- |
| 1 | Subgroup #1 | Subgroup #2 | Subgroup #3 | No wake up |
| 2 | Subgroup #4 | Subgroup #5 | Subgroup #6 | No wake up |
| i | Subgroup #(i-1)*3+1 | Subgroup #(i-1)*3+2 | Subgroup #(i-1)*3+3 | No wake up |
| 11 | Subgroup #31 | Common indication | No wake up | None |

Based on majority view, FL suggests to go with Alt 1.

#### [H][FL 2] Proposal 3.3-7: For WUS information carried by the overlaid OFDM sequence(s), support Alt 1.

- Alt 1: Raw information bits are mapped to sequence(s)

#### [H][FL 2] Proposal 3.3-7r1: For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from LSB to MSB, where K= ceil (N/log2L) , L is the number of overlaid OFDM sequences for one OOK ON chip.

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

- Alt 2: Raw information bits are mapped to sequence(s) after channel coding

  - Same channel coding scheme(s) as OOK is applied.

    - FFS same or different rate matching and/or repetition factor as OOK

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW |  | We prefer Alt2 to be able to combine information detected from OOK symbols and from overlaid sequences for lower detection time. |
| Nokia.1 | N | We prefer Alt.2 as the LR can benefit from underlying encoding, which is same as OOK. Simple repetition of Alt.1 vs Alt.2 of same length shows that Alt.2 is superior even though multiple bits are carried by a overlay sequence. |
| OPPO |  | Raw information bits cannot be mapped to sequence(s) directly, it needs rate matching before the mapping. e.g. The number of raw information bits is 7. The number of OOK ON symbol is 7, and the number of candidate overlaid OFDM sequences per OOK ON is 4. It could transmit 14 bits via overlaid OFDM sequences. After rate mating for raw information, the number of output bit sequence is 14, it can be mapped to sequences. Support ‘Raw information bits after rate matching are mapped to sequence(s)’ |
| Samsung | Y |  |
| CATT |  | To make clear, our illustration is shown below for Alt3. For Alt3, there are two schemes can be further considered as shown in our Tdoc: Scheme1: One overlaid sequence carries the subgroup specific indication or the common indication. UE monitors its associated overlaid sequences in the associated OOK ON symbol index. As shown in above Table 9, one OOK ON symbol can be associated with 3 subgroup information. The total number of OOK ON symbols is 11 for indicating 32 subgroup information. For example, UEs belongs to Subgroup #5, will monitor the 2nd and the 11th OOK ON symbol. Scheme2: Multiple- sequence combination for indicating the subgroup information as shown in above Table 6 with Alt3. |
| Huawei, HiSilicon | N | Based on our evaluation, Alt 2 should be supported for the following reasons: - Some companies support Alt1 because of simplicity, since most of them think Alt2 needs complexed decoding. However, decoding is not needed. UE can compare the received signal with the codeword after channel coding to determine whether or not wake up. - The coverage performance of Alt2 is better than that of Alt1 due to the coding gain, allowing for earlier termination of reception. - Alt 2 can be combined with different order mapping, which can be used for early termination. |
| vivo | Y |  |
| LGE | Y |  |
| ZTE, Sanechips | N | Alt2 is preferred. - Alt3 is too complicated since for a given number of information bits, a table is needed. - Alt1 has a problem that the performance on different OOK-ON symbols is different. For example, 5bits in total can be carried by first OOK-ON symbol with 3bits and second OOK-ON symbol with 2bits, which is clear that these two OOK-ON symbols has different performance. - Moreover, No company show that Alt1 has better performance than Alt2, but some companies show the gain of Alt2. |
| docomo | N | I have a similar view as FW and Nokia. Alt.2 can be used for early termination in combination with OOK binary. Alt.1 cannot be used in combination because a different bit sequence is sent between OOK binary and overlaid OFDM. |
| Panasonic | Y |  |
| FL |  | #### [[H][FL 2] Proposal 3.3-7r1: For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1and Alt2: - Alt 1: Raw information bits are mapped to sequence(s) - N raw information bits are divided into K segments from LSB to MSB, where K= ceil (N/log2L) , L is the number of overlaid OFDM sequences for one OOK ON chip. - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2. - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK #### [H][FL2] Proposal 3.3-4r1: Support maximum 4 roots to be used for overlaid OFDM sequences for M=1/2/4. |

For Alt 1 & Alt 3, to improve coverage, some companies [2][8][10][12][25] propose to support repetition for overlaid OFDM sequences.

#### [H][FL 2] Proposal 3.3-8: For WUS information carried by the overlaid OFDM sequence(s), support repetition of overlaid OFDM sequence

- gNB configures repetition number of overlaid OFDM sequence.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| Nokia.1 | Y |  |
| OPPO | N | No need to configure the repetition number of overlaid OFDM sequence. The number of OOK ON symbol for OOK based WUS can support the repetition of overlaid OFDM sequence. If gNB configures repetition number of overlaid OFDM sequence, how to generate the OOK symbol if the number of OOK symbols for overlaid OFDM sequences repetition is not integer multiple of the number of OOK symbols for OOK based WUS? How to understand the repetition of overlaid OFDM sequence? Alt1: seq1 seq1 seq2 seq2 seq3 seq3 seq4 seq4 Alt2: seq1 seq 2 seq 3 seq 4 seq1 seq 2 seq 3 seq 4 |
| Samsung | Y | Assuming that Alt 1 in proposal 3.3-7 is adopted, we are Ok to support repetition. |
| CATT |  | Ok with the proposal. |
| Huawei, HiSilicon |  | This should be discussed after Proposal 3.3-7. If Alt 2 is used, rate matching can be used. |
| vivo | Y |  |
| LGE | Y |  |
| ZTE, Sanechips |  | The repetition can be predetermined to fulfill all the OOK-ON symbols. Not sure about the motivation why some OOK-ON symbols are not used if the configured repetition number is small |
| Panasonic |  | We are okay with the design direction, but the wording or formulation would be decided by the design of the overlaid sequence mapping to OOK ON chips. |
| Qualcomm | Y |  |

- Issue 2: information bits order carried by the overlaid OFDM sequence

Companies discussed whether the information bits are carried by	 the overlaid OFDM sequence in ascending order or descending order.

- Option 1: in ascending order [2][6][10][13][25]

- Option 2: in descending order [4][18][23][25]

Motivation to support information bits in descending order by option 2 is to facilitate early termination for OFDM-based LP-WUR, if the receiver can obtain first part of information bits by first several OOK ON chip location and remaining part of information bits by overlaid OFDM sequence within these OOK ON chips. Whether early termination can be achieved highly depends on which alternatives to be chosen in issue 1. For example, if Alt 1 or Alt 3 is chosen, the bits carried by overlaid OFDM sequences and OOK are raw information bits before channel coding and the bits after channel coding, there is no obvious way to combine the soft bit information from the OOK symbol and the overlaid sequence, so descending order does not help early termination. If Alt 2 is chosen, early termination may be achieved, depending on how rate matching is performed as discussed in section 3.2.

In addition to early termination, [4] thinks option 2 is more interference-resilience, because, for option 1, the same overlaid sequence shall be also used on the overlapped-ON chip if the same ON/OFF pattern is used, while for option 2, the overlaid sequence can be different. In FL’s understanding, inter-cell interference can be handled by overlaid sequences with different roots for different cells, thus, option 1 is also interference-resilience.

Besides, [4] thinks option 2 can avoid increase of UE storing memory because same coded bit for both OOK and overlaid OFDM sequence. In FL’s understanding, OFDM-based LP-WUR does not need to store coded bits carried by OOK symbols, so whether same or different bits (coded or uncoded) carried by OOK and overlaid sequence does not impact storage.

FL suggests to discuss this issue after progress of issue 1 above, and issues in section 3.2.

- Issue 3: Whether transmission duration of a LP-WUS targeting to wake up OFDM-based receiver can be shorter than the transmission duration required for OOK-based receiver

[4] mentions, if a LP-WUS is targeted to wake up OFDM-based receiver, gNB can transmit only the first four OFDM symbols, since OFDM-based receiver can get the whole LP_WUS information from both OOK modulation and overlaid OFDM sequences. Therefore, to reduce the resource overhead of LP-WUS, transmission duration of a LP-WUS targeting to wake up OFDM-based receiver should be allowed to be shorter than the transmission duration required for envelope-detection based receiver.

In FL’s understanding, for RRC connected mode, different transmission duration of a LP-WUS for OFDM-based receiver and OOK-based receiver can be easily supported, because gNB is aware of UE LP-WUR type and gNB can configure UEs with same LP-WUR type to monitor same LP-WUS while configure UEs with different LP-WUR type to monitor different LP-WUS configuration. For RRC idle/inactive state, gNB provides single MO configuration. Whether/how to enable different transmission of a LP-WUS for OFDM-based receiver and OOK-based receiver needs further discussion. FL invites companies to show your view.

#### [M][FL 2] Question 3.3-1: Do you agree,  is it beneficial to allow gNB to transmit a LP-WUS targeting only OFDM-based receiver with shorter length than a LP-WUS targeting OOK-based receiver, assuming same size of information with same coverage target?

- If yes, how to support it for RRC idle state

- If yes, how to support it for RRC connected state

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | We support the intention that the OFDM-based LP-WUS can end earlier than the OOK-based LP-WUS. But in this Proposal, it seems the OOK-based LP-WUS is not trnsmitted completely? If so, we are not sure how OOK-based LR will work. |
| Nokia.1 |  | Does that mean no OOK LRs in the system. If so, do we still use MC encoding to serve only OFDM receivers? |
| OPPO | N | Support fully using OOK symbols to transmit the overlaid OFDM sequence. No need to limit the time duration of OFDM based WUS. It depends on UE implementation whether to receive the short duration. |
| Huawei, HiSilicon | Y | We think it is beneficial to reduce the overhead. For IDLE state, this can be done by gNB implementation, i.e., gNB transmit partial of the LP-WUS. For CONNECTED, this can be done by configuring the MO length, since in CONNECTED mode the LP-WUS can be targeted to a particular UE, and the coverage can be controlled by gNB configuration. |
| Panasonic | N | For CONNECTED mode, it can be up to gNB configuration to do so, e.g., only to transmit part of the LP-WUS as a shorter version. But a question is it needs UE capability to support? |
| Qualcomm | Y | At least for RRC connected mode, this is preferred. |

- Others

[9] discusses whether the information bits carried by overlaid OFDM sequence is codepoint or bitmap indication. [9] proposes the overlay sequences based on bitmap scheme for both RRC idle/inactive and RRC connected mode, for easier sequence detection by OFDM detector, without trying several MOs for sequence detection.

However, based on FL’s understanding, the agreement to use codepoint for RRC idle/inactive state is applicable to both OOK and overlaid OFDM sequence.

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

  - Remaining issue for OOK LP-WUS for RRC connected mode

RAN1 made working assumption to support codepoint for RRC connected mode.

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8

[2][7][8][9][10][12][15][22] propose to confirm the working assumption. [9] compares cases where a UE only monitors one codepoint (one-to-one mapping), or two codepoint (one-to-one and one-to-all), or > two codepoint (e.g., including combination of two UEs), showing > two codepoint case better when traffic arrival is higher for RRC connected mode. [8] propose to confirm the working assumption but deleting ‘Depending on UE capability, a UE may support less than 8’, because the complexity is unlikely to increase significantly for UE to support up to 8 codepoints, and introducing different UE capabilities may complicate the process for the gNB when mapping the waking UEs to their respective codepoints.[10] proposes to revise the working assumption to support only up to 2 code points to be checked per MO by a UE, without UE capability, considering up to 8 codepoints increases UE complexity and it is challenging for the gNB to make best use of these capabilities.

[2][8][22] point out, different from RRC idle state where the codepoint using a predefined rule to determine the mapping relationship between codepoints and waking UE, the codepoint to be checked by UE should be explicitly configured by gNB, i.e., gNB configures a list of code point values. [25] suggests to first discuss whether one codepoint corresponds to [one or more] UEs before discussion of detailed signaling parameter.

Considering quite opposite view from [7] and [10], it seems keep up to 8 codepoint to be checked by UE and UE capability in working assumption is a good compromise. Therefore, FL suggests to confirm the working assumption without any revision.

#### [M][FL 2] Proposal 3.4-1: Confirm the following working assumption for RRC connected mode.

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

    - Depending on UE capability, a UE may support less than 8

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| Nokia.1 | Y |  |
| OPPO | Y |  |
| Huawei, HiSilicon |  | We suggest to directly discuss the basic UE capability, where we support 2. Regarding the max number of codepoints, we are also OK to reduce it fro 8 to a lower number. |

[6] proposes short MO LP-WUS as shown in the lower part of the figure below. The basic idea is that in each MO, only one UE’s LP-WUS is configured. The short MO LP-WUS is similar to bitmap-based design (upper part of Figure 6) in the sense that LP-WUSs for different UEs is configured in separate resources. If UE is not be waken-up, gNB does not transmit LP-WUS. According to FL’s understanding, this can be achieved by configuring different MOs for different UEs by UE specific LP-WUS configuration and single codepoint configured for a UE. Another way is, if UE is not to be waken-up, gNB still transmits LP-WUS but with a different codepoint. Considering RAN1 never discussed the scheme that a codepoint to indicate not to wake-up, and such scheme requires always transmission of LP-WUS which resulting in large overhead, [16] has similar proposal, to support multiple fields to indicate different codepoints in one LP-WUS.

In FL’s understanding, it is in fact bitmap not codepoint, which is already excluded.

Figure 6: Short MO LP-WUS

- LP-SS

  - M value for LP-SS

    - Supported M values for LP-SS

[5] proposes to confirm the working assumption on the M value for LP-SS and M value does not depend on the SCS. Further, considering it has been agreed that OOK-1 is considered as specific case of OOK-4 with M=1. FL suggests to confirm the working assumption with the following updates in red:

#### [M][FL 2] Proposal 4.1-1: Confirm the working assumption:

Working Assumption

Support the following options for LP-SS

- Option 1: OOK-1

- Option 2: OOK-4 with M=2,4, FFS:1,8,16

  - FFS whether value of M depends on SCS

  - Value of M doesn’t depend on SCS

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

- Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi | Y |  |
| FW |  | Given the difference in performance between M=1,2 and M=4 using the same sampling frequency and while expecting the same complexity at the LR, we prefer to support only OOK-4 with M=4 for LP-SS. |
| Nokia.1 | Y |  |
| LGE | N | As captured in TR 38.869, tolerance to timing error is dependent of SCS. In this manner, the value of M should be dependent on SCS especially in FR2 due to shorter OFDM symbol duration. |
| OPPO | Y |  |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |

    - Same or different M value for LP-WUS and LP-SS

RAN1#119 Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Companies view are summarized as below:

- Alt 1: supported by [8], [6], [5], [15], [20],[21], [17], [25]:

  - For LP-WUS M=1, sufficient timing accuracy can be achieved by LP-SS with M=1. [8]

  - The same M value for LP-WUS and LP-SS is more UE implementation friendly, e.g., no extra memory is needed for storing overlaid OFDM sequence for LP-SS. [6]

  - It would be simpler in UE implementation for LP-WUR detection of both LP-WUS and LP-SS by same sampling rate and chip duration for OOK symbol. Moreover,The sync performance of LP-SS is not related to the M value but only depends on the sampling rate of 15.36 MHz. [5]

- Alt 2: supported by [4], [10], [2], [9], [11],[13],[16]with the following reason:

  - With same time resource, larger M provides better synchronization time accuracy

  - Using same M value for both LP-WUS and LP-SS may not be efficient as the requirement for timing offset itself will be the accuracy of estimation.

  - it ensures that LP-SS provides better accuracy to detect LP-WUS if the choice of M used by LP-SS is greater than LP-WUS

- Alt 3: supported by [3], [7], [17],  [8] (if UE supports all M values {1,2,4}):

  - WUR has the stronger capability than Ambient-IoT UE, and there is no issue to support multiple M values. Moreover, different M value does not impact implementation much. The WUR side still can use similar or same decoding algorithm. [3]

Based on companies’ inputs, there are more support of LP-WUS and LP-SS using difererent M value than same M value. Regarding selection of alt 2 or alt 3, it depends on the progress on section 4.2 and section 5, and thus, this part can be discussed later.

[H][FL 4] Proposal 4.1-2: For the M value for LP-WUS and LP-SS, supports:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

  - Binary sequence design for LP-SS

    - Determination on sequence length : M=1

Agreement

For M=1, L=4 (if supported), the set of LP-SS binary sequences is:

[0 1 0 1]

[0 1 1 0]

[1 0 0 1]

[1 0 1 0]

Agreement

For M=1, L=6 (if supported), the set of LP-SS sequence is:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

Agreement

For M=1, L=8 (if supported), the set of LP-SS sequence is:

[1 0 1 0 0 1 0 1]

[1 0 1 0 1 0 0 1]

[1 0 0 1 0 1 0 1]

[0 1 0 1 0 1 0 1]

- Down-selection on L=4, 6, or 8

- L=4: [8], [5], [24],[16],[23],[27]

- L=6: [8], [9], [3]

- L=8: [4], [3], [2],[16],[20],[19], [25], [27]

- not supporting L=4 and 6: [4], [6], [18] (and 8 either)

Reasons for not supporting L=4 and 6:

- [4]: For L=4, L=6, the sync accuracy is 13.93 and 5.98 us, respectively with cross-correlation equal to 1

- [6]: For {M=1, L=4} and for {M=1, L=6}, there are two sequences that are shifted versions of each other by one bit. Without Manchester coding, this corresponds to a shift by one OFDM symbol. LP-WUR cannot distinguish the two sequences if the random timing error exceeds half the OFDM symbol duration.

- [18](not supporting 4, 6, 8): The performance of the LP-SS sequence sets for M=1 and L∈{4,6,8} does not meet the requirements to support the at least OOK-1 LP-WUS waveform without a LP-SS period <320ms or an additional synchronization signal.

Reasons for supporting L=4:

- [8]: LP-SS duration of 4 or 6 OFDM symbols can provide sufficient timing accuracy at least for OOK1 and OOK4 (M=2), and allows having four distinct sequences, 2.2 us and 2.8 us for OOK1.

Reasons for supporting L= 6:

- [8]: LP-SS duration of 4 or 6 OFDM symbols can provide sufficient timing accuracy at least for OOK1 and OOK4 (M=2), and allows having four distinct sequences, 2.2 us and 2.8 us for OOK1.

- [3]: compared with 6-length binary sequence, additional resource overhead of 33.3% for 8-length binary sequence can only bring additional TO estimation gain of 5.73%

    - Determination on sequence length : M=2

Agreement

For M=2, L=8 (if supported), the set of LP-SS sequence is:

[0 1 0 1 1 0 0 1]

[0 1 1 0 0 1 0 1]

[0 1 1 0 1 0 0 1]

[1 0 0 1 0 1 1 0]

Agreement

For M=2, L=12 (if supported), the set of LP-SS sequence is:

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

Agreement

For M=2, L=16 (if supported), the set of LP-SS sequence is:

[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

[1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

[1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1]

[1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

- Down-selection on L=8, 12, or 16

- L=8: [8],[5], [24],[16], [23],[27]

- L=12: [8], [9], [3], [7]

- L=16: [4],[2],[16],[20],[19],[25],[27]

Reasons for supporting L=8:

- Sync accuracy is sufficient, less overhead compared with L=12,16, same resource overhead as a 4-length sequence with M=1

Reasons for supporting L=12:

- [3]: From LP-SS cross-checking results, it can be seen that 12-length binary sequences can meet the requirement of TO correction accuracy <=2us.12-length binary sequences balances overhead and performance better compared to 8 binary sequences and 16 binary sequences, and has the same overhead as a 6-length sequence with M=1

Reasons for supporting L=16:

- [3]: From LP-SS cross-checking results, it can be seen that 12-length binary sequences can meet the requirement of TO correction accuracy <=2us.12-length binary sequences balances overhead and performance better compared to 8 binary sequences and 16 binary sequences, and has the same overhead as a 6-length sequence with M=1

    - (closed)Determination on sequence length : M=4

Agreement

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

- Set 1

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

- Set 2

[1 0 0 0 1 0 0 0 0 0 0 1 0 0 1 0]

[1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0]

[1 0 0 0 0 1 0 0 0 1 0 0 1 0 0 0]

[1 0 0 0 0 1 0 0 0 0 0 1 0 0 0 1]

Agreement

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

- Set 1:

[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

[0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

[0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

[0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]

- Set 2

[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

[0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

[0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0]

[0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

- Down-selection on L=16 or 32

- L=16: [5], [9], [3], [18], [24],[16],[23],[27]

- L=32: [8], [4], [18], [7], [11],[16],[20],[19],[25],[27]

Reasons for supporting L=16:

- [9]: sufficient sync accuracy and better RSRP

| LP-SS sequences and length | SEQ1 |  | SEQ2 |  | SEQ3 |  | SEQ4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TE (μs) | RSRP (dB) | TE (μs) | RSRP (dB) | TE (μs) | RSRP (dB) | TE (μs) | RSRP (dB) |
| 16 (SET1) | 0.91 | 2.62 | 0.98 | 2.59 | 0.94 | 2.65 | 0.85 | 2.61 |
| 16 (SET2) | 0.52 | 2.77 | 0.65 | 2.63 | 0.59 | 2.67 | 0.39 | 2.75 |
| 32 (SET1) | 0.46 | 2.74 | 0.46 | 2.66 | 0.52 | 2.69 | 0.52 | 2.66 |
| 32 (SET2) | 0.26 | 2.76 | 0.33 | 2.74 | 0.33 | 2.73 | 0.33 | 2.7 |

- [3]: additional resource overhead of 100% for 32-length binary sequence can bring additional TO estimation gain of 32.44%

Reasons for supporting L=32:

- [7]: The main challenge arises when M=4. Even with the best-performing sequence set at L=32 (set2), achieving the required tolerance remains difficult, best-performing sequence set while also considering additional methods to mitigate the impact of timing errors exceeding the tolerance

- Set 1(balanced) or set 2 (unbalanced) sequence

- Set 1(balanced) is supported by [8], [4], [9], [5], [2] with the following reason

  - Set 1 should be supported as it has lower PAPR while it provides similar performance as set 2 [8]

  - Sequences with balanced ‘0’ and ‘1’ facilitates threshold setting based on mean value, AGC adjustment, and easier noise/interference estimation and cancellation for RRM measurement.[2]

Set 2 (unbalanced): [18], [7], [11]

Reason for supporting set 2:

- lower number of required measurement samples, and improved timing synchronization performance. [18]

- Unbalanced LP-SS achieves better absolute accuracy and less bias in the RSRP measurements. [11]

- Unbalanced LP-SS has PAPR of 6 dB, which is lower than PAPR of NR SSB (>9 dB). [11]

Reason not support set 2:

- Set 2 impacts the frequency error estimation (for OFDM based receiver) , AGC and RRM measurements: [4]

- The LP-SS sequence length would be 64 bits after 1/2 Manchester coding, it cannot fit within one slot: [5]

#### [H][FL 3]Proposal 4.2-1  For the length L of LP-SS binary sequence, down-select among the following:

Alt 1:

- M=1, L= {6, 8}

- M=2, L= {12,16}

- M=4, L= {32}

Alt 2:

- M=1, L= {8}

- M=2, L= {16}

- M=4, L= {32}

Alt 3:

- M=1, L= {6}

- M=2, L= {12}

- M=4, L= {32}

| Company | Y/N | Comments |
| --- | --- | --- |
| FW |  | Based on our evaluation, LP-SS with M=2 and L=16 can achieve the sync requirements for OOK-4/M=2 at 160ms periodicity assuming residual frequency error <=5ppm. LP-SS with M=4 and L=16 (SET 2) can achieve the same requirements at improved resource overhead and RRM measurement accuracy. Additionally, only LP-SS with M=4 and L=32 (SET 2) may achieve the sync requirements for OOK-4/M=2 at 320ms periodicity and OOK-4/M=4 at 160ms periodicity assuming residual frequency error <=5ppm. Therefore, we prefer to support M=4, L={16,32}. |
| Nokia.1 |  | Alt.3 with M=4 using L=16 ensures LP-SS can fit within a slot. |
| LGE | Alt 1 | We prefer to have multiple L values corresponding to each M value to obtain gNB flexibility depending on channel conditions. |
| OPPO |  | Support Alt2. It has same time duration for different M values. |
| Huawei, HiSilicon |  | We prefer to go with Alt 2 directly. Sequences for different M values with the same overhead is preferred. And based on our evaluation, to achieve the target coverage, L={8, 16, 32} are needed. |
| ZTE, Sanechips |  | For OOK-4 with M=4, the four agreed 16-length binary sequences can meet the requirement of TO correction accuracy <=1us and thus it should be considered in the alternatives. For Alt1, L=16 for M=4 also should be included. Additionally, Alt4 should be considered to minimize the NW overhead with all the sync and measurement target satisfied. Alt 4: - M=1, L= {6} - M=2, L= {12} - M=4, L= {16} |
| Qualcomm |  | Do not support M=1, L=6 due to the timing ambiguity issue. |

Currently, only a few companies provide the analysis and preferences on set 1 and set 2,  companies are encouraged to provide your views on the selection of set 1 and set2.

#### [M][FL 2]Question 4.2-2 For LP-SS binary sequence with M=4 L=16 (if supported) or L=32(if supported), can you provide your preferences on set 1 (balanced 0 and 1 within one OFDM symbol)or set 2(unbalanced 0 and 1 within one OFDM symbol)?

| Company | Comments |
| --- | --- |
| FW | As discussed in our previous comment, we prefer to support set 2 as they provide better time sync accuracy and improved RRM measurement accuracy. |
| Nokia.1 | Set 1 (Even though set 2 provides better timing accuracy, benefit is not significant. Furthermore, PAPR is higher to set 1 by a margin). |
| LGE | Prefer set 1. Balanced set may be beneficial for threshold determination and obtaining AGC gain. |
| OPPO | Support set2 for the better sync performance. |
| Huawei, HiSilicon | We prefer set 1, since the balanced sequence is more friendly for RRM measurement, AGC. Also, to improve the frequency error estimation range for OFDM receiver, balance sequences are also preferred. |
| ZTE, Sanechips | Balanced 0 and 1 is preferred which is more friendly to gNB power allocation. |
| Qualcomm | Set 1 is preferred. |

Based on the feedback so far, there are more support of set 1, and thus FL suggests the following:

#### [H][FL 3]Proposal 4.2-1r1  For the length L of LP-SS binary sequence, down-select among the following:

Direction 1: More than 1 lengths supported for each M

Alt 1: up to 2

- M=1, L= {6, 8}

- M=2, L= {12,16}

- M=4, L= {16,32}

Alt 6: support all lengths

Note: No additional UE capability on supporting the length for each M

Direction 2: Single length supported for each M

Alt 2:

- M=1, L= {8}

- M=2, L= {16}

- M=4, L= {32}

Alt 3:

- M=1, L= {6}

- M=2, L= {12}

- M=4, L= {32}

Alt 4:

- M=1, L= {6}

- M=2, L= {12}

- M=4, L= {16}

Alt 5:

- M=1, L= {4}

- M=2, L= {8}

- M=4, L= {16}

#### [H][FL 3]Proposal 4.2-1  For LP-SS with M=4, supports set1 (balanced).

  - LP-SS with overlaid OFDM sequences

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

    - How to specify OOK waveform if overlaid OFDM sequence is not configured

Since transmitting LP-SS by using a specified overlaid OFDM sequence is configurable, how to specify OOK waveform if overlaid OFDM sequence is not configured shall be considered. [2] [20] discuss two approaches:

- Approach 1: the input of DFT is simply specified by a virtual sequence with constant envelope to guarantee the OOK detection performance [2].

It’s up to gNB to generate any sequence with constant envelop, e.g., CSI-RS sequence (bipolar Gold sequence) or PDCCH (QPSK modulation).

- Approach 2: the input of DFT is unspecified in RAN1, and it is left to RAN4 requirements to guarantee the OOK detection performance [2] [20].

It’s up to gNB to generate any sequence as long as it meets RAN4 requirements.

#### [M][FL 2] Question 4.3-1: What’s your consideration on how to specify OOK waveform if overlaid OFDM sequence is not configured?

| Company | Comments |
| --- | --- |
| Nokia.1 | If this is only for M=1, only EPRE should be the same as LP-WUS to ensure detectability of LP-WUS upon receiving LP-SS. |
| LGE | Since overlaid OFDM sequence of LP-SS does not convey any information, default sequence index from candidate set of overlaid OFDM sequence for LP-WUS can be used according to the cell ID (e.g. sequence index = mod(PCI,4)). In this manner, inter-cell interference can be mitigated for OFDM based LP-WUR. |
| OPPO | It can depend on gNB implementation. |
| Huawei, HiSilicon | It is difficult to specify OOK waveform if overlaid OFDM sequence is not configured. One possible way is to revisit the previous agreement and agree to specify overlaid sequence also for M=1. |
| Qualcomm | Agree with Huawei. |

    - How LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS

In RAN1#118bis meeting, it has been agreed that LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. Regarding how to reuse such specified overlaid OFDM sequence(s), [4] proposes using different roots of ZC sequences on different OOK "on" symbols to reduce aliasing caused by the frequency offset and a time offset. [2] proposes that for LP-SS with specified overlaid OFDM sequence, gNB configures a root value with default CS=0.  FL notices that in RAN1#118bis meeting, it has been agreed that for the case of overlaid OFDM sequence not carrying information, the solution of using different overlaid sequences across different OOK "on" symbols is precluded and single overlaid sequence on each OOK ‘ON’ symbol has been agreed.

RAN1 #118bis Agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

#### [M][FL 2] Question 4.3-2: Do you support using different roots of ZC sequences on different OOK "on" symbols for LP-SS?

| Company | Comments |
| --- | --- |
| Nokia.1 | Y |
| LGE | As shown in above agreement, single overlaid OFDM sequence is used when overlaid OFDM sequence does not carry any information. For LP-SS, which does not carry any information, single OFDM sequence should be used for each OOK ‘ON’ symbol. |
| OPPO | Support same overlaid OFDM sequence. For LP-SS, no need to carry information via the OFDM sequence, single overlaid OFDM sequence is enough. |
| Huawei, HiSilicon | Yes, we found that to improve the performance of frequency error correction, the complimentary roots can be used for adjacent ON symbols. |
| Panasonic | Yes for improving coverage. Also agree with Huawei regarding the potential frequency error sensitivity to certain roots. |

Further, to reduce UE complexity, [6] proposes to maintain the same overlaid sequence length for LP-SS and LP-WUS when they have different M values, e.g., when the M value for LP-SS is larger than that for LP-WUS, length of the overlaid sequence is determined by M for LP-SS. From FL’s understanding, the existing agreement doesn’t restrict that the length of the overlaid sequence used by LP-SS shall be the same with the one used by LP-WUS. What’s your consideration on the overlaid OFDM sequence length for LP-SS and LP-WUS when they have different M values?

#### [M][FL 2] Question 4.3-3: What’s your consideration on the overlaid OFDM sequence length for LP-SS and LP-WUS when they have different M values?

Option 1: Same overlaid OFDM sequence length for LP-SS and LP-WUS

Option 2: Different overlaid OFDM sequence lengths corresponding to its respective M value

| Company | Option | Comments |
| --- | --- | --- |
| Nokia.1 | Option 2 | Timing is determined from LP-SS while satisfying RSRP requirements. Better accuracy in timing is beneficial for LP-WUS detection. |
| OPPO | Option 2 | For LP-WUS, we generate the overlaid OFDM sequence based on the M values. In our views, reusing the overlaid OFDM sequence specified for LP-WUS means the same generation of overlaid OFDM sequence, it should generate the overlaid OFDM sequence based on the respective M value of LP-SS. |
| Huawei, HiSilicon | Option 2 | More clarification should be provided for Option 1. |
| Panasonic | Option 2 | Without any agreements on the M value relation between LP-WUS and LP-SS, Option 2 should be starting point. |
| Qualcomm |  | We do not support different M values. |

- Necessity of additional synchronization signal

  - Summary on companies’ views

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

Companies’ views are summarized as below:

- Option 1A supported by [4], [2], [7],[16],[26], [25], [27]:

  - [4], [7]: additionally support 160ms

  - [2]: additionally support 80ms for at least M=4, 160ms for at least M=1,320ms is sufficient for M=2

  - [16], [25]: additionally support 80ms or 160ms

  - [18]: only support under the condition target residual time error is updated as T=0.5 μs and the residual frequency error is ≤ 5 ppm, additionally support160ms

  - [7]: it can be up to UE implementation for how to receive LP-WUS successfully under the given LP-SS periodicity, e.g., via sliding window detection

  - [27]: Additionally support at least one of [80ms,160ms] for M=4

- Option 1B supported by [8], [9], [11], [20]:

  - [8]: for OOK-1, providing evaluation results shows that WUS+ preamble provides similar performance without preamble but increasing the WUS length by assuming the same overhead

  - [9]: RAN1 should refrain from defining/using additional synchronization signal like preamble sequence before LP-WUS, since the codeword itself can be used as one.

- Option 2 supported by [8], [10],[6],[3], [5], [18], [13],[21], [23] :

  - [8]: for OOK-4 with M=2,4

  - [10]: additionally support LP-SS periodicity of 160ms and additional synchronization signal is always present at least for M=4 for both 320ms and 160ms periodicity

  - [5]: Support additional sync signal for LP-SS period longer than 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

  - [23]: Additional sync signal needs to be supported to accommodate M=4 for LP-WUS

  - Time error tolerance vs Total timing error due to residual timing error by LP-SS and residual frequency error

- In this section, total timing error is calculated based on the median residual timing error provided by companies for the selected set of LP-SS (the sequence with maximum residual timing error is used ) for M=1, 2, 4 in terms of sync accuracy from the cross-checking results in RAN1#120.

  - M=1

| LP-SS periodicity | Fr=5 ppm/10ppm |  |  |
| --- | --- | --- | --- |
| 320 ms | L=4, Tr=9.26 us | L=6, Tr=3.66 us | L=8, Tr=3.71 us |
|  | 10.86 /12.46us | 5.26/6.86 us | 5.31/6.91 us |
| 160ms | 10.06/10.86us | 4.46/5.26us | 4.51/5.31us |

  - M=2

| LP-SS periodicity | Fr=5 ppm/10ppm |  |  |
| --- | --- | --- | --- |
| 320 ms | L=8, Tr=1.43 us | L=12, Tr=1.1 us | L=16, Tr=0.83 us |
|  | 3.03/4.63 us | 2.7/4.3 us | 2.43/4.03 us |
| 160ms | 2.23/3.03us | 1.9/2.7us | 1.63/2.43us |

  - M=4

| LP-SS periodicity | Fr=5 ppm/10ppm |  |  |  |
| --- | --- | --- | --- | --- |
| 320 ms | L=16, Tr=0.98 us (balanced 0 and 1 within an OFDM symbol) | L=16, Tr=0.79us (unbalanced 0 and 1 within an OFDM symbol) | L=32, Tr=0.65 us (balanced 0 and 1 within an OFDM symbol) | L=32, Tr=0.65 us (unbalanced 0 and 1 within an OFDM symbol) |
|  | 2.58/4.18 us | 2.39/3.99 us | 2.25 /3.85us | 2.25/3.85 us |
| 160ms | 1.78/2.58us | 1.59/2.39us | 1.45/2.25us | 1.45/2.25us |
| 80ms | 1.38/1.78us | 1.19/1.59us | 1.05/1.45us | 1.05/1.45us |

Compared with time error tolerance in TR 38.869, i.e., 5,3,1 us for LP-WUS with M=1,2,4,  the total time error above based on 5ppm, 10ppm residual frequency error and LP-SS with M=1,2,4 are summarized as below, the case that sufficient sync accuracy from LP-SS is highlight in cyan.

| Total time error | 320ms periodicity |  |  | 160ms periodicity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LP-SS with M=1 (L=4,6,8) | LP-SS with M=2 (L=8,12,16) | LP-SS with M=4 (L=16,32) | LP-SS with M=1 (L=4,6,8) | LP-SS with M=2 (L=8,12,16) | LP-SS with M=4 (L=16,32) |
| LP-WUS with M=1 (5us tolerance) | 10.86 /12.46us; 5.26/6.86 us; 5.31/6.91 us | 3.03/4.63 us; 2.7/4.3 us; 2.43/4.03 us | 2.58/4.18 us 2.25 /3.85us; | 10.06/10.86us; 4.46/5.26us; 4.51/5.31us | 2.23/3.03us; 1.9/2.7us; 1.63/2.43us | 1.78/2.58us; 1.45/2.25us |
| LP-WUS with M=2 (3us tolerance) | 10.86 /12.46us; 5.26/6.86 us; 5.31/6.91 us | 3.03/4.63 us; 2.7/4.3 us; 2.43/4.03 us | 2.58/4.18 us 2.25 /3.85us; | 10.06/10.86us; 4.46/5.26us; 4.51/5.31us | 2.23/3.03us; 1.9/2.7us; 1.63/2.43us | 1.78/2.58us; 1.45/2.25us |
| LP-WUS with M=4 (1us tolerance) | 10.86 /12.46us; 5.26/6.86 us; 5.31/6.91 us | 3.03/4.63 us; 2.7/4.3 us; 2.43/4.03 us | 2.58/4.18 us 2.25 /3.85us; | 10.06/10.86us; 4.46/5.26us; 4.51/5.31us | 2.23/3.03us; 1.9/2.7us; 1.63/2.43us | 1.78/2.58us; 1.45/2.25us |

  - LP-WUS with M=1:

    - For LP-SS with M=1 and L=4, 6, 8 under 320 ms periodicity, the total timing error is always larger than the time error tolerance.160ms is needed and L=6,8 provides total timing error smaller than time error tolerance for 5ppm residual frequency error.

    - For LP-SS with M=2 and L= 8,12,16 or M=4 and L=16, 32 under 320 ms periodicity, the total timing error is smaller than the time error tolerance for both 5ppm and 10 ppm residual frequency error.

  - For LP-WUS with M=2:

    - For LP-SS with M=2 and L=12,16 under 320 ms periodicity, the total timing error is smaller than the time error tolerance for 5ppm residual frequency error. 160ms is needed and L=12,16 provides total timing error smaller than time error tolerance for 10 ppm residual frequency error.

    - For LP-SS with M=4 and L=16, 32 under 320 ms periodicity, the total timing error is smaller than the time error tolerance for 5ppm residual frequency error. Under 160ms periodicity, L=16,32 provides total timing error smaller than time error tolerance

For LP-WUS with M=4 and LP-SS M=4 with L=16, 32, with 80ms periodicity, the total time error 1.05 is slightly larger than time error tolerance 1 us.

Based on the analysis above, FL suggests the following

#### [H][FL 2] Proposal 5.2-1: Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1 and M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support LP-SS periodicity of 160ms

- FFS:  LP-WUS with M=4

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Based on the observation of the evaluations, we are OK with the proposal. |
| FW |  | We are OK, but would like to clarify that 160ms periodicity is sufficient for LP-WUS with M=2 only if residual frequency error is limited to 5ppm or if LP-SS with M=4/L=32 (set 2) is considered. Further, for LP-WUS with M=4, it may be supported by 160ms periodicity if residual frequency error is limited to 5ppm and if LP-SS with M=4/L=32 (set 2) is considered. |
| Nokia.1 |  | During SI phase, we evaluated the timing offset for uncoded LP-WUS, which required the timing offset to be 1us for M=4. However, with the current assumption of codeword, the impact of timing error reduces to ~0.5dB with the code rate < 0.33. Thus, we do not prefer increasing the LP-SS periodicity rather increasing the codeword length ensures better reliability and tolerance to LR impairments. |
| LGE |  | Okay with the proposal. However, in the case of LP-WUS with M=4 which is left as FFS needs to always support additional sync signal. As summarized in the above table, LP-WUS with M=4 cannot be supported to any periodicities. Additional sync signal must be supported for in this case. |
| OPPO |  | Support. |
| Samsung |  | Our preference is having the common option regardless of M value(Option 1A with 160ms LP-SS or Option 1B is also OK). For M =4, the gNB can improve the performance by other schemes (e.g., longer code block length after rate-matching, power boosting) rather than transmitting the always-on LP-SS signal. |
| Huawei, HiSilicon | Y | We also support no additional sync for M=4, where 160ms/80ms periodicity is supported in addition. But we can accept to agree on M=1/2 first. |
| vivo | Y | For FFS point, in our view, M=4 with 15kHz can work with 160ms or even 320ms periodicity (same as M=2 with 30khZ). M=4 with 30kHz is more efficient for RRC connected mode as we commented in proposal 3.1-1, where we can expect better sync by MR. Therefore, we’re fine with no enhancement for LP-WUS M=4 case. |
| ZTE, Sanechips | N | We do not think this proposal is kind of solution to solve the sync issue and reduce overhead. For example, For LP-WUS with M=4 and LP-SS with M=4, 320ms periodicity can not be supported. Moreover, if 10ppms is considered, we do not think 1us can be achieved after drifting. Instead, longer LP-SS and smaller LP-SS periodicity is needed, which cause more system overhead and still can not guarantee the sync performance for 320ms. Additionally, if additional sync sync is needed for M=4, we do not need to conclude based on M=1 and M=2 since also this additional sync signal can be used for M=1 and M=2. |
| docomo |  | Based on our evaluation, this proposal is OK for OOK based receiver. However, OOK-4 M=4 should always need additional sync. Signals. So,OOK-4 M=4 is separated operation from OOK-1,OOK-4 M=2. |

- The time error tolerance considered by companies:

|  | Time error tolerance |  |  |
| --- | --- | --- | --- |
|  | M=1 | M=2 | M=4 |
| TR 38.869 (SNR degradation <= 2dB) | 5us | 3us | 1us |
| [4] | 8us | 4us | 2us |
| [8] (SNR degradation <1dB) | 6us | 2us | 1us |

- Total timing error derived by companies

  - [8]: periodicity of 320ms, Fr=5 or 10 ppm, LP-SS length of 4 or 6 OFDM symbols for M=1,2,4

| LP-SS periodicity and duration | Fr=5 ppm for OOK-WUR |  |  | Fr=10 ppm for OOK-WUR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 320 ms, 4 OFDM symbols | Tr=2.8 us [OOK-1] | Tr=1.5 us [OOK4, M=2] | Tr=1.4 us [OOK4, M=4] | Tr=2.8 us [OOK-1] | Tr=1.5 us [OOK4, M=2] | Tr=1.4 us [OOK4, M=4] |
|  | 4.4 us | 3.1 us | 3 us | 6 us | 4.7 us | 4.6 us |
| 320 ms, 6 OFDM symbols | Tr=2.2 us [OOK-1] | Tr=1.1 us [OOK4, M=2] | Tr=1 us [OOK4, M=4] | Tr=2.2 us [OOK-1] | Tr=1.1 us [OOK4, M=2] | Tr=1 us [OOK4, M=4] |
|  | 3.8 us | 2.7 us | 2.6 us | 5.4 us | 4.3 us | 4.2 us |

  - [2]: periodicity of 320,160, 80ms, Fr=5 ppm, LP-SS length of {6,8},{4,6,8},{4,8} OFDM symbols for M=1,2,4, respectively

|  | - Tr=4.3 and 3.5us and based on evaluation in section 5 | - Tr=1.04, 0.78 and 0.52us based on evaluation in section 5 | - Tr=0.78 and 0.52 us based on evaluation in section 5 |
| --- | --- | --- | --- |
| - Time error tolerance in TR 38.869 | - 5 (OOK-1) | - 3 (OOK4, M=2) | - 1(OOK4, M=4) |
| - LP-SS periodicity 320 ms | - 5.9us, 5.1us | - 2.6us, 2.4us, 2.1us | - 2.4us, 2.1us, |
| - LP-SS periodicity 160 ms | - 5.1us, 4.3us | 1.84us, 1.58us, 1.32us | - 1.6us, 1.3us, |
| - LP-SS periodicity 80 ms | - 4.7us, 3.9us | - 1.44us, 1.18us, 0.92us | - 1.2us, 0.9us |
| Note 1: Fe=5ppm is assumed. Note 2: As evaluated in section 5, - For M=1, assuming no power pooling across OFDM symbols. LP-SS length = 6 bits to achieve Tr=4.3us, LP-SS length = 8 bits to achieve Tr=3.5us. - For M=2, LP-SS length = 8 bits to achieve Tr=1.04us, LP-SS length = 12 bits to achieve Tr =0.78us and LP-SS length = 16 bits to achieve Tr =0.52us. - For M=4 & 30kHz SCS, LP-SS length =16 to achieve Tr=0.78us, and LP-SS length = 32 bits to achieve Tr=0.52us. |  |  |  |

  - [3]: periodicity of 320,160, 80ms, Fr=5 or 10 ppm

- A rough calculation of TO value before LP-WUS detection assuming residual frequency error 10ppm

|  | Residual frequency error=10ppm |  |  |
| --- | --- | --- | --- |
| LP-SS periodicity (ms) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) |
| 80 | 0.8 | 5/3.04/2.99 | 5.8/3.84/3.79 |
|  |  | 2/1.676/0.98 | 2.8/2.476/1.78 |
|  |  | 1/1.01/0.68 | 1.8/1.81/1.48 |
| 160 | 1.6 | 5/3.04/2.99 | 6.6/4.94/4.59 |
|  |  | 2/1.676/0.98 | 3.6/3.276/2.58 |
|  |  | 1/1.01/0.68 | 2.6/2.61/2.28 |
| 320 | 3.2 | 5/3.04/2.99 | 8.2/6.24/6.19 |
|  |  | 2/1.676/0.98 | 5.2/4.876/4.18 |
|  |  | 1/1.01/0.68 | 4.2/4.21/3.88 |

- A rough calculation of TO value before LP-WUS detection assuming residual frequency error 5ppm

|  | Residual frequency error=5ppm |  |  |
| --- | --- | --- | --- |
| LP-SS periodicity (ms) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) |
| 80 | 0.4 | 5/3.04/2.99 | 5.4/3.44/3.39 |
|  |  | 2/1.676/0.98 | 2.4/2.076/1.38 |
|  |  | 1/1.01/0.68 | 1.4/1.41/1.08 |
| 160 | 0.8 | 5/3.04/2.99 | 5.8/4.14/3.79 |
|  |  | 2/1.676/0.98 | 2.8/2.476/1.78 |
|  |  | 1/1.01/0.68 | 1.8/1.81/1.48 |
| 320 | 1.6 | 5/3.04/2.99 | 6.6/4.64/4.59 |
|  |  | 2/1.676/0.98 | 5.2/3.276/2.58 |
|  |  | 1/1.01/0.68 | 4.2/2.61/2.28 |

  - Overhead comparison

- Overhead of LP-SS only vs LP-SS and additional sync compared by companies are summarized as below:

According to the table, FL observes overhead results depend on the following aspects:

  - The length of LP-SS

  - The length of additional sync signal

  - Subgroup paging rate

  - Number of Pos within one I-DRX cycle

Under different assumptions, which scheme provides less overhead is different.

Before going through the overhead comparison details, it’s more important to figure out whether the sync accuracy provided by LP-SS only without additional sync signal can satisfy the time error tolerance with certain periodicities in 5.2.  Then the overhead comparison can be further discussed.

[2]:

|  | Options | LP-SS periodicity T and LP-SS length | Additional sync signal length | Overhead for sync | Overhead ratio of additional aperiodic sync signal/(LP-WUS without additional aperiodic sync signal) |
| --- | --- | --- | --- | --- | --- |
| M=1 | Option 1A | 320ms Length 8 | N/A | 0.18% | N/A |
|  | Option 1A | 160ms Length 6 | N/A | 0.27% | N/A |
|  | Option 1A | 160ms Length 8 | N/A | 0.36% | N/A |
|  | Option 2-1 | 320ms Length 4 | Length 6 | Set1: 1.25% Set2: 0.92% | Set1: 32% Set2: 37% |
|  | Option 2-2 MOs within 160ms share one additional sync signal | 320ms Length 4 | Length 6 | Set1: 0.36% Set2: 0.35% | Set1: 9.2% Set2: 14.1% |
| M=2 | Option 1A | 320ms Length 8 | N/A | 0.09% | N/A |
|  | Option 1A | 320ms Length 12 | N/A | 0.14% | N/A |
|  | Option 1A | 320ms Length 16 | N/A | 0.18% | N/A |
|  | Option 2-1 | 320ms Length 8 | Length 8 | Set1: 0.87% Set2:0.64% | Set1: 22% Set2:26% |
| M=4 | Option 1A | 80ms Length 32 | N/A | 0.71% | N/A |
|  | Option 2-1 | 320ms Length 16 | Length 16 | Set1: 0.87% Set2: 0.64% | Set1: 22% Set2:26% |
|  | Option 2-2 MOs with 40ms shared one additional sync signal | 320ms Length 16 | Length 16 | Set1: 0.51% Set2: 0.44% | Set1: 13% Set2:18% |
| Note: UE settings as provided in appendix 9.4 (also same as the setting in [3]) is reused for additional sync signal overhead calculation. |  |  |  |  |  |

  - [4]:

| Total overhead (OFDM symbols) |  | periodicity of LP-SS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 160 ms | 320 ms | 640 ms | 1280 ms |
| no preamble |  | 256 | 128 | 64 | 32 |
| with preamble= 8 OFDM symbols # of LP-WUS per 1.28s | 30 | 1232 (=30*4*8+256) | 1104 (=30*4*8+128) | 1024 (=30*4*8+64) | 992 (=30*4*8+32) |
|  | 50 | 1856 | 1728 | 1664 | 1632 |
|  | 100 | 3456 | 3328 | 3264 | 3232 |
| with preamble= 4 OFDM symbols # of LP-WUS per 1.28s | 30 | 736 | 608 | 544 | 512 |
|  | 50 | 1056 | 928 | 864 | 832 |
|  | 100 | 1856 | 1728 | 1664 | 1632 |

[8]:

|  | 0.01% per UE paging rate | 0.1% per UE paging rate | 0.4% per UE paging rate | 1% per UE paging rate |
| --- | --- | --- | --- | --- |
| Preamble overhead | 0.02 | 0.2 | 0.7 | 1.7 |
| LP-SS overhead (320 ms periodicity) | 1 | 1 | 1 | 1 |
| LP-SS overhead (160 ms periodicity) | 2 | 2 | 2 | 2 |
| LP-SS overhead (80 ms periodicity) | 4 | 4 | 4 | 4 |
| Preamble+LP-SS (320 ms periodicity) | 1.02 | 1.2 | 1.7 | 2.7 |
| Note: according to the SI (TR 38.869), the following per-UE paging rates can be considered: RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s Note: for 0.4% per UE paging rate, the PO group paging rate is over 10%. Note: 4-symbol preamble and 6-symbol LP-SS are assumed. |  |  |  |  |

[3]:

Evaluated case 1: Target 4000 UEs in a cell, number of subgroups is 31, per UE paging rate 0.001, per group paging rate=3.17%

Table 14. Overhead comparison for the evaluated case 1

|  | LP-SS periodicity and length | additional sync signal M and Length | overhead for sync | NOTE |
| --- | --- | --- | --- | --- |
| M=1 baseline | 320ms, L=6 | / | 0.135% |  |
| M=1 | Option 1A: 160ms, L=8 | / | 0.36% |  |
| M=1 | Option 2-1: 320ms, L=6 (one preamble for one LP-WUS) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.268% 0.223% 0.223% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=1 | Option 2-2: 320ms, L=6 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.169%~0.27% 0.158%~0.225% 0.158%~0.225% (it depends on whether all the LP-WUS is paged in one LO or not) |  |
| M=2 | 320ms, L=8 | / | 0.09% |  |
| M=2 | Option 1A: 160ms, L=16 | / | 0.36% |  |
| M=2 | Option 2-1: 320ms, L=8 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.222% 0.179% 0.179% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=2 | Option 2-2: 320ms, L=8 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |
| M=4 | 320ms, L=16 | / | 0.09% | same as M=2 |
| M=4 | Option 1A: 160ms, L=32 | / | 0.36% |  |
| M=4 | Option 2-1: 320ms, L=16 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.222% 0.179% 0.179% |  |
| M=4 | Option 2-2: 320ms, L=16 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |

Evaluated case 2: Number of subgroups is 31, per group paging rate=3%, number of POs is 8

Table 15. Overhead comparison for case 2

|  | LP-SS periodicity and length | additional sync signal M and Length | overhead for sync | NOTE |
| --- | --- | --- | --- | --- |
| M=1 baseline | 320ms, L=6 | / | 0.135% |  |
| M=1 | Option 1A: 160ms, L=8 | / | 0.36% |  |
| M=1 | Option 2-1: 320ms, L=6 (one preamble for one LP-WUS) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.3861% 0.3024% 0.3024% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=1 | Option 2-2: 320ms, L=6 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.169%~0.27% 0.158%~0.225% 0.158%~0.225% (it depends on whether all the LP-WUS is paged in one LO or not) |  |
| M=2 | 320ms, L=8 | / | 0.09% |  |
| M=2 | Option 1A: 160ms, L=16 | / | 0.36% |  |
| M=2 | Option 2-1: 320ms, L=8 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.3411% 0.2574% 0.2574% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=2 | Option 2-2: 320ms, L=8 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |
| M=4 | 320ms, L=16 | / | 0.09% | same as M=2 |
| M=4 | Option 1A: 160ms, L=32 | / | 0.36% |  |
| M=4 | Option 2-1: 320ms, L=16 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.3411% 0.2574% 0.2574% |  |
| M=4 | Option 2-2: 320ms, L=16 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |

- Reply LS to RAN4

In RAN4 LS on LP-WUS UE RF to RAN1 , there are two parts : in 1st part, RAN4’s agreed two sets of LR IM+NF are provided ; in 2nd part, RAN4 ask feedback on the information to facilitate deriving SNR. The two parts are discussed in the following seperately for easy follow :

## Part I

In RAN4#114 meeting, the following agreements for LP-WUS/WUR have been reached for deriving REFSENS: - Set 1: LP-WUR IM+NF = 18.0 dB - Set 2: LP-WUR IM+NF = 13.5 dB - Note: RAN4 assumes the MR NF is 9.0 dB. RAN4 adds implementation margin (IM) on top of NF for deriving REFSENS requirements. - Note: the mapping between different sets and different receiver types are still under RAN4 discussion.

Regarding LR NF, RAN1 has agreed the following in RAN1#117meeting:

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

[30][32][35][41] analyze the LR NF+IM difference between RAN1 and RAN4. For IM, even it is not explicitly reflected in the RAN1 agreements, it is observed that companies assume IM=2dB in link budget analysis in the spreadsheet included in TR 38.869 [29]. Therefore, the total values of LR IM and NF from RAN1 assumption are calculated as below, based on NF of MR=7dB.

- LP-WUR IM+NF = NF of MR+ 8dB+2dB=17dB

- LP-WUR IM+NF = NF of MR+ 5dB+2dB=14dB

- LP-WUR IM+NF =NF of MR+ 2dB+2dB=11dB

If we ignore the MR NF assumption difference between RAN1 and RAN4, and directly compare the total value of LR IM and NF from RAN4 and RAN1, set 2 from RAN 4 is within RAN1’range and set 1 from RAN4 is 1dB larger than the maximum value 17dB from RAN1.  In RAN1, SNR=-3dB is assumed for the maximum NF to evaluate LP-WUS and LP-SS design. To accommodate the 1 dB difference, [41] consider some design margin for LP-WUS and LP-SS in RAN1. [35] considers RAN1 has only two WI meetings left for Rel-19, it is highly unlikely that RAN1 is able to update the SNR requirement assumptions, evaluate the LP-WUS based on the new assumptions. Further, the entry/exit condition for LP-WUS monitoring in idle/inactive mode and UE specific signal allow network to control the actual coverage of LP-WUS irrespective of the targeted coverage. Hence, it is fine that RAN1 and RAN2 have different assumptions on NF+IM and RAN1 does not need to consider new SNR for LP-WUS and LP-SS design.

#### [M][FL 2] Question 6-1: Do you agree that RAN1 does not need to consider new SNR for LP-WUS and LP-SS design caused by the 1dB difference on LR IM+NF between RAN1and RAN 4? Some design margin can be further considered for the LP-WUS and LP-SS design in RAN1, e.g., longer code block length, potential repetition.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | Agree not to consider new assumptions. |
| Qualcomm | Y | The entry/exit condition can handle the mismatch between RAN1 and RAN4 assumptions. |

## Part II

Further details on signal configuration of LP-WUS and LP-SS for both idle/inactive and connected modes to facilitate deriving SNR with sufficient coverage are helpful, e.g., - Chip rate and length of information bits (the number of ON/OFF symbols for the single LP-WUS transmission) of OOK sequence - Coding, if any - Whether CRC will be configured? If CRC is configured, what length of CRC is used? - Any other information relevant to detection sensitivity.

In part II, RAN 4 is expecting RAN1 to provide feedback on the information which could be used to derive SNR for LP-WUR Rx requirements. The potentially related parameters are listed below, some of which rely on RAN1further progress, and hopefully we can achieve consensus this meeting:

- LP-WUS for both idle/inactive and connected modes: OOK

- Chip rate: M=1, 2, 4 (working assumption)

- Number of information bits: up to [5] bits

- Coding:

- For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not)

- 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not)

- 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not)

- FFS: Code block length

- Further, for the code block after above listed coding schemes, Manchester coding is further applied.

- CRC: not supported.

- LP-WUS for both idle/inactive and connected modes:  overlaid OFDM sequence

- Number of information bits: up to [5] bits

- Number of overlaid OFDM sequences for RRC idle:

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4

- FFS RRC connected

- The length of overlaid OFDM sequences:

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included), and M=1, 2, 4 (working assumption)

- Roots and CS(s) of ZC sequence(s) to generate overlaid OFDM sequence(s): FFS

- How to carry information by overlaid OFDM sequence: FFS

- LP-SS : OOK and overlaid OFDM sequence

- Chip rate: M=1, 2, 4

- For the length of binary sequences, limit the selection to the following

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

- Set of binary sequences for LP-SS: FFS

- Coding: there is no coding scheme agreed for LP-SS so far

- CRC: not supported

- Roots and CS(s) of ZC sequence(s) to generate overlaid OFDM sequence(s): FFS

For the highlight part in cyan, further RAN1 progress is needed. Thus, it’s better to rely LS for part II when RAN1 achieve consensus.

#### [M][FL 2] Question 6-2: For the information listed above, do you have any concern to include them in the reply LS when RAN1 resolve the FFS parts? Besides, is there any other information to be included in reply LS?

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We can go back to this after further discussion in this week. |

- References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

- R1-2501814, Discussion on LP-WUS and LP-SS design, vivo

- R1-2501767, Discussion on LP-WUS design, ZTE Corporation, Sanechips

- R1-2502237, Signal Design of LP-WUS and LP-SS, Huawei, HiSilicon

- R1-2501999, Design of LP-WUS and LP-SS, CATT

- R1-2502846, LP-WUS and LP-SS Design, Qualcomm Incorporated

- R1-2502375, Discussion on LP-WUS and LP-SS design, Samsung

- R1-2502805, LP-WUS and LP-SS design, Ericsson

- R1-2502007, LP-WUS and LP-SS design, Nokia

- R1-2502615, LP-WUS and LP-SS design, Apple

- R1-2502711, LP-WUS and LP-SS Design, MediaTek Inc.

- R1-2502448, Discussion on LP-WUS and LP-SS design, Xiaomi

- R1-2502167 Discussion on LP-WUS and LP-SS design, CMCC

- R1-2501737, Discussion on LP-WUS and LP-SS Design, EURECOM

- R1-2501875, Discussion on LP-WUS and LP-SS design, Spreadtrum, UNISOC

- R1-2502257, Signal design for LP-WUS and LP-SS, OPPO

- R1-2502580, Discussion on the LP-WUS and LP-SS design, Panasonic

- R1-2501710, Discussion on LP-WUS and LP-SS Design, FUTUREWEI

- R1-2502694, Discussion on LP-WUS and LP-SS design, Honor

- R1-2502302, Discussion on LP-WUS and LP-SS design, InterDigital, Inc.

- R1-2502322, LP-WUS and LP-SS design, Sony

- R1-2501845, Discussion on LP-WUS and LP-SS Design, TCL

- R1-2502481, Discussion on LP-WUS and LP-SS design, LG Electronics

- R1-2502075, Discussion on LP-WUS and LP-SS design, NEC

- R1-2502773, Discussion on LP-WUS and LP-SS design, NTT DOCOMO, INC

- R1-2502675, Discussion on LP-WUS and LP-SS design, Sharp

- R1-2502880, On LP-WUS and LP-SS design, Nordic Semiconductor ASA

- R1-2502908, Discussion on LP-WUS and LP-SS design, Lenovo

- TR 38.869, Study on low-power wake up signal and receiver for NR

- R1-2501758, Discussion on LP-WUS UE RF, ZTE Corporation, Sanechips

- R1-2502348, Discussion on RAN4 reply LS on signal configuration LP-WUS and LP-SS, Samsung

- R1-2502590, Discussion on the RAN4 reply LS on LP-WUS UE RF, Apple

- R1-2502591, Reply LS on LP-WUS UE RF, Apple

- R1-2502800, Discussion on RAN4 LS on LP-WUS UE RF, Ericsson

- R1-2502826, Discussion on response to RAN4 LS on LP-WUS UE RF, Qualcomm Incorporated

- R1-2502945, Draft Reply LS on LP-WUS UE RF, Huawei

- R1-2502168, Discussion on LP-WUS operation in IDLE/INACTIVE modes, CMCC

- R1-2502376, Discussion on LP-WUS operation in IDLE/INACTIVE modes, Samsung

- R1-2502238, Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode, Huawei

- R1-2502616, LP-WUS operation in IDLE/INACTIVE modes, Apple

- R1-2501794, Draft reply LS on LP-WUS UE RF, vivo

- R1-2501703 (R4-2503003), LS on LP-WUS UE RF, vivo

- Appendix : Proposals from contributions

## R1-2501814 vivo

Proposal 1: For RRC connected mode, LP-WUS is configured per BWP. The starting PRB of the LP-WUS is determined by offsetToCarrier corresponding to SCS of the active BWP and the configured offset with range of 0~ 264.

Proposal 2: For RRC idle/inactive state, LP-WUS/LP-SS frequency resource can be out of initial DL BWP. The LP-WUS/LP-SS starting PRB is determined by offsetToCarrier corresponding to SCS of the DL initial BWP and the configured offset with range of 0~ 264.

Proposal 3: For candidate M values,

- For FR1: Do not support M=4 OOK-4 for 30kHz SCS for RRC idle mode, FFS for RRC connected mode.

- For FR2: Support only M=1 for LP-WUS and LP-SS.

Proposal 4: Support Alt 2 for LP-WUS/LP-SS M value, i.e., the M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. gNB separately configures M value for LP-WUS and LP-SS.

Proposal 5: For RRC connected mode, gNB configures maximum number of codepoints which can be indicated by LP-WUS and  a list of codepionts to be checked by the UE.

- Maximum number of codepoints which can be indicated by  LP-WUS is in the range of 1 ~ 32.

- The number of elements in the codepoint list to be checked by the UE is up to 8, and the value of each element is in the range of 1~ maximum number of codepoints per LP-WUS.

Proposal 6: For LP-WUS information carried by OOK,

- For larger than 2 information bits, RM coding with rate matching as section 5.4.3 in 38.212 is supported.

- For 2 information bits, coding as in section 5.3.3.2 of 38.212 with Qm =1 is applied. Rate matching as section 5.4.3 in 38.212 can be further applied.

- For 1 information bits (if supported), coding as in section 5.3.3.1 of 38.212 with Qm =1 is applied. Rate matching as section 5.4.3 in 38.212 can be further applied.

- The code block length after rate matching can be from 1 to 32 bits.

- Note: no scrambling operation.

Proposal 7: The overlaid OFDM sequence carries raw information bits (Alt 1) in order.

- Repetition can be applied to improve performance. Maximum repetition number, e.g., up to [4], can be provided by gNB.

Proposal 8: For RRC connected mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

Proposal 9: RAN1 clarifies the maximum number of overlaid OFDM sequences is 16/8/4 for M=1/2/4 for FR1, and 4/2/1 for M=1/2/4 for FR2, if corresponding M value for FR2 is supported.

Proposal 10: For candidate root and cycic shift to support multiple overlaid OFDM sequences for LP-WUS in a cell,

- Support up to 2 roots

- Support up to 8/4/2 CS for M=1/2/4 for FR1, 2/1/1 CS for FR2 (if supported).

Proposal 11: For LP-WUS overlaid OFDM sequence configuration in a cell,

- gNB configures number of sequences N1 (up to 16/8/4 for M=1/2/4), number of roots N2 (1 or 2) and a root index.

  - The root can be any value between 1 ~ Bzc-1.

  - In case of N2=2, second root value is (Bzc-q), where q is indicated for 1st root sequence.

- The number of CS is derived by N1/N2.

- N1 sequences are enumerated in increasing order of first increasing cyclic shift of first root sequence, and then in increasing order of the root sequence index in case of N2=2.

Proposal 12: For LP-WUS transmission in a MO,

- MO duration is configured by gNB, in the range of one symbol to 5 or 6 slots.

- An MO is a valid MO, if the avaible symbols within the MO is no smaller than the configured LP-WUS length L, wherein the avaible symbols exclude TDD UL symbols, SSB and rate matching pattern provide by gNB.

- In the valid MO, LP-WUS maps to L OOK chips within avaible symbols.

Proposal 13: For M=4 case, LP-SS binary sequence with  balanced ‘0’ and ‘1’ within one OFDM symbol is supported, i.e., set 1.

Proposal 14: Support LP-SS binary sequences for M=1 & Length 8, M=2 & Length 16, and M=4 & Length 32. gNB configures the LP-SS binary sequence index for a cell.

Proposal 15: For LP-SS with specified overlaid OFDM sequence, gNB configures a root value with default CS=0.

Proposal 16: RAN1 to discuss how to specify OOK generation, when LP-SS is not configured with specified overlaid OFDM sequence.

Proposal 17: For LP-SS occasion,

- gNB configures a number of norminal occasions for LP-SS, wherein the number is no smaller than number of beams for LP-SS/LP-WUS. UE assumes LP-SS transmission with beam sweeping in valid occasions within the configured norminal LP-SS occasions, wherein  the valid occasion does not collide with UL symbols.

- gNB configures periodicty and slot-level offset for the first LP-SS occasion, e.g.,

- sl80, INTEGER {0..79}

- sl160, INTEGER {0..159}

- sl320, INTEGER {0..319}

- sl640, INTEGER {0..639}

- gNB configures symbol-level offset for the first LP-SS occasion, or the symbol-level offset is pre-defined.

- Remaining LP-SS occasions within a period is in consecutive slots with same symbol-level offset as the first LP-SS occasion.

Proposal 18: Support option 1A with LP-SS periodicty of 80ms for at least M=4, 160ms for at least M=1, and 320ms or smaller periodicity for M=2, without addtional sync signal.

## R1-2501767 ZTE Corporation, Sanechips

Proposal 1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

Proposal 2: For the M value for LP-WUS and LP-SS, it should be configured by gNB respectively.

Proposal 3: Confirm the working assumption on overlaid OFDM sequence(s) of LP-WUS.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

- Proposal 4:Candidate LP-SS binary sequences should satisfy that

- For OOK-4 with M>1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols of one OFDM symbol for OOK-4 with M>1.

- For OOK-4 with M=1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within the binary sequence.

Proposal 5: For OOK-4 with M=1, the following four 6-length binary sequences are supported for LP-SS.

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

Proposal 6: For OOK-4 with M=2, the following four 12-length binary sequences are supported for LP-SS.

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

Proposal 7: For OOK-4 with M=4, the following four 16-length binary sequences are supported for LP-SS.

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

Proposal 8: Option 1B can be precluded and down-select between option 1A and option 2.

Proposal 9: Support additional sync signal

- reuse LP-SS design

- it is configurable

- support larger and smaller LP-SS periodicity

Proposal 10: For LP-WUS information carried by OOK, channel coding schemes in section 5.3.3.2 of 38.212 for 2 information bits or 1 information is not supported.

Proposal 11: For LP-WUS information carried by OOK, RM coding as in section 5.3.3.3 of 38.212 with rate matching is supported before Manchester coding

- Rate matching to a longer output binary sequence lager than 32 can be used as repetition.

- Output sequence length after rate matching is preferred to integer multiples of 7 or 14.

- Additional repetition after rate matching is not supported.

- LP-WUS information bits size is no larger than 5 and detailed value can be configured through signaling.

Proposal 12: For LP-WUS information carried by the overlaid OFDM sequence(s), Alt2 is supported.

- Rate matching after RM code is supported and the output sequence length should be an integer multiple of a bit size that can be carried by the ZC sequences in accordance with M value.

Proposal 13: For the overlaid ZC sequences, values of LZC and BZC for OOK-4 with M=1,2,4 are adopted as shown in the following Table.

| M value | LZC | BZC |
| --- | --- | --- |
| 1 | 132 | 131 |
| 2 | 66 | 61 |
| 4 | 33 | 31 |

Proposal 14: For LP-WUS overlaid ZC sequence generation, it is recommended to reuse NR preamble generation method of restricted type A.

- ZC root index is prioritized to the root index with supporting larger number of ZC sequences with CS in the root index

## R1-2502237 Huawei, HiSilicon

- For OOK-4, confirm the working assumption of M=4 is confirmed for 30kHz SCS.

- For WUS information carried by the overlaid OFDM sequence(s), Alt 2 is supported, i.e., Raw information bits are mapped to sequence(s) after channel coding, where the same channel coding scheme(s) as OOK is applied.

- Note: Rate-matching is still FFS

- It is supported to let OFDM-based LP-WUR to get different coded bits from overlaid OFDM sequences and from OOK ON/OFF pattern by one of following option.

  - Option 1: The same coding and rate-matching order for OOK and overlaid OFDM sequences, and different mapping order from rate-matched bits to the OOK symbols/overlaid OFDM sequences.

  - Option 2: the same coding but different rate-matching order for OOK and overlaid OFDM sequences, then the same mapping order from rate-matched bits to the OOK symbols/overlaid OFDM sequences.

- For the mapping from the coded bits to overlaid sequences, support a mapping order different from the mapping order of OOK sequences-based modulation, to enable OFDM-based receiver to terminate LP-WUS reception as early as possible.

- The coded bits after rate matching are to be circularly repeated when the number of bits that can be carried by overlaid sequences are more than 32 bits or to be truncated to match the number of bits that can be carried by the OFDM symbols of a LP-WUS, which is similar as the operation in section 5.4.3 in TS 38.212.

- In order to reduce resource overhead, transmission duration of a LP-WUS targeting to wake up OFDM-based receiver can be shorter than the transmission duration required for ED based receiver.

- In order to make ZC sequences robust to timing/frequency error, optimized roots of ZC sequences are selected.

- Different overlaid ZC sequences can be used to minimize the interference between different cells.

  - Prefer to use different roots for different cells to minimize the interference.

  - Prefer to use different cyclic shifts to carry information.

- For M=1, the frequency sequence after DFT is mapped to the center of 132 REs with 2 guard REs at each side.

- Support enhanced rate-matching scheme after block codes defined in section 5.3.3.3 of 38.212, where the enhancement includes:

  - Rate-matching is performed by truncating from the heading bits or tailing bits based on the length of output codeword.

  - Before rate-matching, the codewords are interleaved  di=dπi as

| i | 0…4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21…24 | 25…31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| πi | 0…4 | 14 | 10 | 8 | 7 | 9 | 6 | 13 | 5 | 16 | 19 | 24 | 11 | 12 | 15 | 17 | 18 | 20…23 | 25…31 |

- The number of candidate codeword lengths is limited to (e.g. 4), where the candidate lengths can be selected based on minimum Hamming distances (e.g., {16, 12, 8, 4}).

- For LP-WUS information carried by OOK, do not support cases with 1 or 2 information bits.

- The design of LP-SS should follow the guidelines:

  - Good auto-correlation and cross-correlation property taking into account modulation with CP

  - Balanced '0's and '1's in the whole sequence

  - Balanced ‘0’ and ‘1’ within OFDM symbols for M>1.

- For the M value relationship between LP-WUS and LP-SS, support Alt2, i.e.,

  - The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- If the LP-SS sequences need further down-selection, the choices are as follows:

  - M=1, L= 8

  - M=2, L= 16

  - M=4, L= 32 , Set 1

- For both timing and frequency error evaluation purpose, the residual frequency error (Fr) can be <= 5ppm after assistance from MR.

- For LP-SS periodicity, to support the functionality of RRM measurement by LP-WUR and the functionality of fallback operation, values larger than 320ms are NOT supported.

- For LP-SS periodicity and preamble, support Option 1A, i.e., additionally support 160ms and does not support preamble.

## R1-2501999 CATT

Proposal 1: OOK-4 with M=1 is the DFT-S-OFDM waveform with low PAPR and should be supported.

Proposal 2: The OOK waveform of LP-WUS can be specified by a configurable variable M with value range {1, 2, 4}.

Proposal 3: The following two alternatives should be supported for RM code block generation for LP-WUS:

- Alt 1: K bits information block [c1,…,ck] is encoded by subset matrix Gx,y (x=0,…,L-1; y=0,…,K-1) from the RM mother code matrix Mi,j (i=0,…,31; j=0,...,10) specified in TR 38.212 to generate L bits coded block [d1,…,dL]

  - Gx,y = Mi_x, j_y, ((x=0~L-1, y=0~4) = (i_x =0~(L-1); j_y =1~5).

- Alt 2: K bits information block [c1…ck] is encoded by RM mother matrix specified in TR 38.212 to generate 32 coded bits, di=( )mod2, i=0,…,31,j_k=1~5. The 32 bits coded block [d1…d32] is truncated to L bits code block [y1,…,yL] by puncturing (32-L) bits based on a puncture bit index set {L+1~32}.

Proposal 4: LP-WUS with the 1 or 2 information bit for wakeup indication should not be supported.

Proposal 5: The information bit size should be 5 to carry wake-up information.

Proposal 6: If preamble is not allocated within MO, the code block length can be 28bits, 14bits and 7bits for OOK-4 with M=4, OOK-4 with M=2 and OOK-4 with M=1/OOK-1, respectively.

Proposal 7: If preamble is allocated within MO, the code block length can be 20 bits, 10 bits and 5bits for OOK-4 with M=4, OOK-4 with M=2 and OOK-4 with M=1/OOK-1, respectively.

Proposal 8: The scrambling over the LP-WUS should not be supported.

Proposal 9: Repetition over RM coding before Manchester coding should not be supported.

Proposal 10: Block-level repetition after Manchester coding could be supported for coverage enhancement.

Proposal 11: It is needed to be clarified that the OFDM sequences are overlaid after Manchester coding at each ON chip.

Proposal 12: Support same maximum number of candidates overlaid sequences for both RRC_CONNECTED mod and RRC_IDLE/RRC_INACTIVE mods.

Proposal 13: The CS of overlaid OFDM sequence should be fixed according to the bandwidth index of LP-WUS.

Proposal 14: The set of root values for overlaid ZC sequence can reuse the principle of the root values specified for RACH Preamble in TS38.211 as shown in Table 6.

Table 9: The set of root value for different M

| M value | Set of root value |
| --- | --- |
| 1 | {1, 130, 2, 129, 3, 128, 4, 127, 5, 126, 6, 125, 7, 124, 8, 123} |
| 2 | {1, 60, 2, 59, 3, 58 4, 57} |
| 4 | {1, 30, 2, 29} |

Proposal 15: The mapping between wakeup information and the root values should be discussed after the principle of the ZC root sequences selection are agreed.

Proposal 16: Support scheme 1: One overlaid OFDM sequence carries subset of subgroups specific indication or the common indication in one PO.

Proposal 17: Alt 3 should be supported for low power consumption, better detection performance and low complexity.

Proposal 18: Alt 1: LP-WUS/LP-SS frequency resource is confined within initial DL BWP should be supported for RRC_IDLE/INACTIVE modes.

Proposal 19: Both OOK-1 and OOK -4 with M=1/2/4 can be supported for LP-SS waveform.

Proposal 20: Unified OOK waveform for LP-SS and LP-WUS can be supported.

Proposal 21: Support Alt1: The M values for LP-WUS and LP-SS are always same.

Proposal 22: The Manchester coding can be the supported for LP-SS as coverage enhancement same with LP-WUS.

Proposal 23: The following sequence sets of LP-SS should be supported:

- For M=1, L=4, the set of LP-SS binary sequences is:

[0 1 0 1]

[0 1 1 0]

[1 0 0 1]

[1 0 1 0]

- For M=2, L=8, the set of LP-SS sequence is:

[0 1 0 1 1 0 0 1]

[0 1 1 0 0 1 0 1]

[0 1 1 0 1 0 0 1]

[1 0 0 1 0 1 1 0]

- For M=4, L=16, the set of LP-SS sequence is:

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

Proposal 24: The unbalanced sequence set 2 for LP-SS should not be supported for M=4.

Proposal 25: The sequence length L=4/8/16 can be supported for M=1/2/4 , respectively.

Proposal 26: Support additional sync signal for LP-SS period longer than 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Proposal 27: Alt 3: The resource of preamble is allocated outside of MO, each preamble resource associated with one or more MOs could be supported and can be specified by a configured offset between preamble resource and associated MO or an MO within a MO group.

Proposal 28: The sequence design for LP-SS can be reused for the Preamble.

Proposal 29: The M value of OOK-4 waveform or OOK-1 waveform for LP-SS, LP-WUS and Preamble should be configured to be the same in a cell to minimize the implementation complexity.

## R1-2502846, Qualcomm Incorporated

Proposal 1: Clarify in the RAN1 #118bis agreement that in the case that overlaid OFDM sequence carries no information, the single overlaid sequence is configured by gNB as a known sequence to the UE.

Proposal 2: For each M=1, 2 or 4, the same set of candidate overlaid OFDM sequences is configured for all OOK ON chips for the LP-WUS.

Proposal 3: For idle/inactive mode, to keep the memory usage for storing overlaid sequences constant for different M values, the maximum number of roots configured in OOK On chips for M=1 and 2 is 1 and 2, respectively.

Proposal 4: For connected mode, the maximum number of sequences and roots over OOK On chips are UE capabilities.

Proposal 5: The maximum number of overlaid sequences applicable for a UE is 2 per OOK On chip if the common codepoint indicating all UE subgroups is configured in the LP-WUS MO. Otherwise, the maximum number of overlaid sequences applicable for a UE is 1 per OOK On chip.

Proposal 6: Support Alt 1: Raw information bits are mapped to sequence(s) for WUS information carried by the overlaid OFDM sequence(s).

Proposal 7: For OOK LP-WUS design, the minimum Hamming distance should be not smaller than half the sequence length.

Proposal 8: To generate the OOK LP-WUS from RM code, replace the NR rate matching by Table 2. The sequence generation procedure includes the following steps:

- For a sequence length L (1<L<32), determine the maximum number of information bits K that satisfies the minimum Hamming distance requirement HD>= L/2 according to [5]

- Add a zero prior to the first bit of the input information bit vector i.e., convert b0, b1…bK−1 to 0, b0, b1…bK−1

- Encode the bits according to section 5.3.3.3 from TS 38.212

- Select the encoded bits from the 32-bit output according to Table 2 in section 2.2.3.1 where each row contains positions of bits retained from the 32-bit RM encoded bits.

Proposal 9: Sequences for OOK LP-WUS can be generated by NR RM code in the following steps

- Choose the number of information bits K depending on the desired sequence length L according to Table 1 in section 2.2.3

- Define the following bit selection vectors. The nth value in the vector is associated with the nth row of the corresponding G matrix defined in section 2.2.3.2 and it is the row index for a row of the matrix spanned by Mi,1 to Mi,K in Table 5.3.3.3-1 that contains the same binary combination as the nth row of the G matrix

  - π2 = [0 3 1]

  - π3 = [0 2 6 4 3 8 1]

  - π4 = [5 14 13 16 24 10 0 3 7 1 8 4 2 6 9]

  - π4' = [13 2 16 3 14 15 6 4 9 0 8 10 1 7 24]

  - π5 = [15 30 14 12 10 6 24 16 21 26 9 19 11 0 5 18 7 27 29 20 2 13 1 4 22 8 23 25 28 3 17]

- If L is not a power of 2, add a zero prior to b0 of the input (from b0,b1…bK−1 to 0,b0,b1…bK−1) and select the vector πK

- If L is a power of 2, select vector πK−1, and append 31 at its end

- Encode the bits according to section 5.3.3.3 from TS 38.212

- Select and reorder bits according to the indices contained in selected π vector

  - E.g., if c0,c1…c31 is the output of the RM encoding process and we selected π2, then the output of this step is c0, c3, c1

- If necessary (L ≠ 2K−1), apply rate matching as described in section 5.4.3 from TS 38.212

Proposal 10: Support LP-WUS with short duration, with the number of codepoints per MO equal to 1. Network can configure one of the following two modes

- Mode 1 with one sequence: a sequence is transmitted in the MO to indicate the associated UE to monitor PDCCH, no signal is transmitted in the MO if the associated UE does not to monitor PDCCH

- Mode 2 with two sequences: a first sequence is transmitted in the MO to indicate the associated UE to monitor PDCCH, a second sequence (! first sequence) is transmitted in the MO to indicate the associated UE to not monitor PDCCH

Proposal 11: Support preamble in the LP-WUS.

Proposal 12: Support the M values for LP-WUS and LP-SS are always same.

Proposal 13: Do not specify LP-SS binary sequences for {M=1, L=4} and {M=1, L=6}.

Proposal 14: For RRC idle/inactive, support Alt 1: LP-WUS/LP-SS frequency resource is confined within initial DL BWP.

## R1-2502375 Samsung

Proposal 1: Confirm the working assumption related to codepoint-based LP-WUS for RRC CONNECTED mode with the following modification:

- Delete “Depending on UE capability, a UE may support less than 8.”

- Add “Codepoint value set to be checked by a UE can be configured by the gNB.”

Proposal 2: For the maximum information bit size before coding, 5 or 6 bits can be considered as the candidates.

- FFS: whether the same maximum information bit size is applied for different RRC mode, which maximum information bit size is used among 5 bits and 6bits.

Proposal 3: Regarding the coding scheme for OOK-based LP-WUS, support reusing the current specification as much as possible with the following details:

- For 1 and 2bit information transmission, the section 5.3.3.1 and 5.3.3.2 in 38.212 can be applied respectively for code block generation using modulation order (Qm) equal to 1.

- For more than 2bit information, the section 5.3.3.3 in 38.212 can be applied for length 32 code block generation.

- After applying the coding, code block can be repeated or punctured based on rate matching.

- No need to support scrambling and repetition on top of rate matching.

Proposal 4: Codepoint value without any coding schemes (including Manchester coding and rate matching) is carried by corresponding overlaid OFDM sequence via OOK ON symbol (i.e., Alt 1).

- Unlike UE with OOK-based LP-WUR, UE with OFDM-based LP-WUR can meet the target FAR by setting the proper threshold for overlaid OFDM sequence detection.

- FFS: repetition for the remaining ON symbols.

Proposal 5: For RRC CONNECTED, support UE capability reporting for the maximum number of candidates overlaid OFDM sequence per ON symbol.

Proposal 6: Regarding the determination on the number of cyclic shift values for overlaid OFDM sequences for RRC IDLE/INACTIVE, at least 4 different root values should be used to transmit information by the overlaid OFDM sequence (X=4).

Proposal 7: Root values of overlaid OFDM sequence per a cell can be configured by the gNB for both RRC IDLE/INACTIVE states.

- The number of used cyclic shifts can be implicitly derived based on the number of overlaid OFDM sequence per ON symbol and the number of root values configured by the gNB, or explicitly configured by the gNB pairing with each root value.

Proposal 8: The gNB can configure the information bit size carried by overlaid OFDM sequences per ON symbol.

- Explicitly or implicitly configure the monitoring window of OFDM-based LP-WUR within an LP-WUS monitoring occasion based on the number of overlaid OFDM sequence.

Proposal 9: For RRC idle/inactive, LP-WUS/LP-SS frequency resource is confined within initial DL BWP.

Proposal 10: Support configuring M values for LP-WUS and LP-SS to be same or different (Alt3).

Proposal 11: Further consideration on M values for 60kHz SCS and 120kHz SCS is necessary.

- Consider limiting M value for 60kHz/120kHz to have the same maximum OOK chip rate with 15kHz/30kHz SCSs case (e.g., 112kcps).

Proposal 12: For the LP-SS binary sequence used in a cell, additional support of sequence determination by predefined rule is not needed.

Proposal 13. Regarding LP-SS binary sequence set for each M value, the sets with the following L value should be supported.

- L=8 for M=1,

- L=12 for M=2,

- L=32 (set 2) for M=4.

Proposal 14: Support Option 1A with additional support for 160ms LP-SS periodicity.

## R1-2502805 Ericsson

a.	It should be possible to generate LP-WUS/LP-SS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

b.	It should be possible to multiplex the LP-WUS/LP-SS with other NR transmissions in time or frequency domain without causing interference.

c.	It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Proposal 2	Paging misdetection performance of the UE should not be impacted when LP-WUS is used by the UE for power savings.

Proposal 3	Regarding M value for LP-WUS and LP-SS, Alt 1 “the M values for LP-WUS and LP-SS are always same” is supported.

Proposal 4	LP-WUS should support any number of codepoints from 1 to 32 (targeting up to 31 subgroups and one common codepoint).

Proposal 5	Regarding LP-WUS information bits, all cases of 1 information bit, 2 information bits and larger than 2 information bits should be supported.

Proposal 6	The OFDM symbols used for LP-WUS should be indicated to the UE by configuring number of slots (A) used for LP-WUS (A=1,2 or [3] slots) an additional parameter indicating the available OFDM symbols in each slot (e.g., by configuring a 14-bit bitmap or by using a SLIV value).

Proposal 7	If repetition is supported for OOK-based LP-WUS, the OOK symbols after Manchester coding are repeated N_rep times. The N_rep repetitions occur in same MO.

Proposal 8	Maximum N_rep=[5] repetitions can be supported in the specifications considering different deployment scenarios, gNB flexibility to reserve some symbols in each slot for other transmissions, UE implementation margins etc., resulting in maximum MO duration for LP-WUS (i.e., after maximum repetitions) of [15] slots.

Proposal 9	The code-block length after channel encoding (i.e., N in 38.212 in section 5.3.3.3) is min (Y, (F*M)/2), where F is the number of WUS OFDM symbols with Manchester encoding, M is the number of OOK symbols per OFDM symbol, and Y=1, 3, and 32 for 1-bit, 2-bit, and ≥2 bits information, respectively. When (F*M)/2 > Y, the code-block is cyclically extended to fit the available OOK symbols.

Proposal 10	It should be possible to support or indicate 32 UE subgroup with one WUS, no segmentation.

Proposal 11	For regular UEs, the LP-WUS/LP-SS should be located within the initial DL BWP (as in Alt1). For RedCap, the LP-WUS/LP-SS can be located outside the separate RedCap initial DL BWP (as in Alt2) but within an initial DL BWP that includes CORESET#0 used for paging.

Proposal 12	In the working assumption from RAN1#118bis regarding OOK-1 and OOK-4 [M=1], the following note should be removed: “DFT size is 2^n”.

Proposal 13	LP-WUS design should allow OFDM-based LP-WUR to detect the information sent using OFDM sequences using a smaller monitoring duration compared to that of OOK-based LP-WUR (which detects information sent via OOK). i.e., low detection complexity and early termination should be ensured for OFDM WUR.

Proposal 14	The cyclic shifts of the OFDM sequences should be configured by the network.

Proposal 15	The number of roots for OFDM sequences (in the specification) for each M value should be at least 4.

Proposal 16	Optimal set of sequence roots can be determined based on a minimum cross-correlation metric. For four sequences roots, consider sets {2, 65, 68, 88}, {11, 35, 42, 44}, {5, 12, 23, 25} for M=1, 2, and 4, respectively.

Proposal 17	To carry OFDM information, further consider “Alt3: Codepoint/Subgroup is mapped to sequence” or “Alt1: Raw information bits are mapped to sequence(s)”.  “Alt 2: Raw information bits are mapped to sequence(s) after channel coding” should not be considered.

Proposal 18	Confirm the Working Assumption from RAN1#120 regarding codepoint scheme in connected mode.

Proposal 19	It should be possible for NW to flexibly configure the placement of LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact.

Proposal 20	LP-SS periodicities smaller than 320 ms should not be supported. Consider following values for configuring LP-SS periodicity: 320ms, 640ms, 1280ms, 2560ms.

Proposal 21	To ensure network flexibility for LP-SS transmissions and balance the tradeoff between overhead/network energy efficiency and timing accuracy in different SNR conditions, multiple LP-SS sequence lengths should be supported for each M value.

Proposal 22	LP-SS duration should be 4 or 6 OFDM symbols, corresponding to the following lengths for different M values: L= {4,6} for M=1, L= {8,12} for M=2, and L=32 for M=4.

Proposal 23	For LP-SS sequences of M=4, set 1 should be supported as it has lower PAPR while it provides similar performance as set 2.

Proposal 24	Including a preamble part before the data part of LP-WUS transmissions should be considered for OOK-based LP-WUS. The presence of preamble should be configurable.

Proposal 25	LP-SS periodicity smaller than 320 ms should not be supported.

Proposal 26	Regarding additional synchronization signal (i.e., preamble), among options identified in RAN1#119, Option 1B should be supported at least for OOK-1. For OOK-4 (M>1), Option 2 can be considered further.

## R1-2502007 Nokia

Proposal 1:	The separation between LP-WUS/LP-SS and SSB in frequency domain must be restricted to ensure that all three physical channels experience similar fade.

Proposal 2:	As unified signal design is preferred to minimize the specification impact, pulse shaping for M={2,4} shall not be considered, since in RAN1#119 it was agreed that M=1 does not.

Proposal 3:	RAN1 should refrain from defining/using additional synchronization signal like preamble sequence before LP-WUS, since the codeword itself can be used as one.

Proposal 4:	To carry WUS information in overlay sequence, RAN1 shall consider Alt-2, which is same as the mapping used for underlying OOK transmission.

Proposal 5:	RAN1 should consider appending a known bits to the information bits (instead of CRC) to improve the detectability and coverage of LP-WUS.

Proposal 6:	RAN1 shall consider encoding of subgroup information using channel coding to extend the LP-WUS coverage.

Proposal 7:	As the expected traffic arrival rate is higher for connected mode, option 2C with known padded bits, is preferred for LP-WUS design.

Proposal 8:	We prefer Alt-2 as a suitable option for choosing M values between LP-SS and LP-WUS.

Proposal 9:	RAN1 shall consider the set of length 6 sequences for M=1, which provides sufficient accuracy for time and RSRP.

Proposal 10:	RAN1 shall consider length 12 sequences for M=2 to achieve desired level of estimation accuracy from both time and RSRP measurements.

Proposal 11:	RAN1 shall consider SET-1 with 16 length sequences for LP-SS using M=4.

Proposal 12:	We favour option 1B because there is no additional requirement to increase the periodicity of LP-SS to enhance either timing or RRM measurements.

## R1-2502615 Apple

Proposal 1: For LP-WUS information carried by OOK with 1 or 2 information bits, the information bit(s) are encoded using simple repetitions.

Proposal 2: For LP-WUS information carried by OOK with more than 2 information bits, no repetition is supported after RM coding.

Proposal 3: For both idle/inactive and connected mode, the maximum number of codepoints supported by an LP-WUS is 32, i.e., 5 information bits.

Proposal 4: For candidate overlaid sequences across all OOK ON chips of LP-WUS in both idle/inactive and connected mode, the number of roots configured is up to 2.

Proposal 5: For WUS information carried by the overlaid OFDM sequence(s), support Alt 1, where raw information bits, with padding if needed, are mapped to one or more overlaid OFDM sequences, and the overlaid sequences are applied to OOK ON symbols sequentially and repeated until the end of LP-WUS.

Proposal 6: For the M value for LP-WUS and LP-SS, support Alt 2 (The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS).

Proposal 7: For idle/inactive mode,

- For OOK-based receiver, LP-WUS/LP-SS frequency resource can be out of initial DL BWP.

- For OFDM-based receiver, the baseline capability is that LP-WUS/LP-SS frequency resource is confined within initial DL BWP.

  - FFS: UE capability on the support of LP-WUS/LP-SS frequency resource out of initial DL BWP

Proposal 8: Confirm the working assumption with the following modifications:

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 2 8

    - Depending on UE capability, a UE may support less than 8

Proposal 9: Support the following combination of M and LP-SS sequence length L: {M = 1, L = 8}, {M = 2, L = 16}, and {M = 4, L = 16}.

Proposal 10: For additional synchronization signal, support Option 2 (With additional sync signal, LP-SS periodicity =320ms).

- Support LP-SS periodicity of 160ms.

- Additional synchronization signal is always present at least for M=4 for both 320ms and 160ms periodicity. FFS other conditions.

Proposal 11: For M=4, L=16 and L=32 (if supported), use Set 1 (balanced).

## R1-2502711 MediaTek Inc

Proposal 1: LP-WUS/LP-SS is located within the initial DL BWP and frequency-aligned with SSB.

Proposal 2: For the M value for LP-WUS and LP-SS, Alt 2 is selected, i.e., requiring M value for LP-WUS to be no larger than that of LP-SS.

Proposal 3: Adopt a 6-bit information payload for LP-WUS (allowing 64 codepoints).

Proposal 4: Use a single ZC root sequence for LP-WUS overlaid sequences, with cyclic shifts mapping to the needed sequence variations for M = 1, 2, or 4.

Proposal 5: Specifically, 16 circular shifts of a ZC sequence of a given root are utilized, and j-th subset of circular shifts are utilized for the j-th possible on-chip occasion within a OFDM symbol, where j = 1, …, M and M = 1, 2, 4.

Proposal 6: Adopt Alt 1 (Raw information bits are mapped to sequence(s)) for carrying information by the overlaid OFDM sequence(s). Note: Alt 1 embraces hierarchical subgroup mapping over multiple on-chips.

Proposal 7: Adopt LP-SS with M = 4 and L = 32 to ensure LP-WUS performance for both OOK and OFDM WURs.

Proposal 8: Support LP-SS design of M = 4, L = 32, and single on-chip within each OFDM symbol as this configuration provides the best synchronization and measurement performance.

Proposal 9: With LP-SS sequences of good timing accuracy, Option 1B is adopted since LP-WUR can work with no additional synchronization and the baseline LP-SS periodicity of 320 ms.

## R1-2502448 Xiaomi

Proposal 1：Support to confirm the working assumption that OOK-4 with M=4 for 30kHz SCS is supported for LP-WUS.

Proposal 2：For the M values for LP-WUS and LP-SS, adopt Alt3

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Observation 1：For RRC idle/inactive state, it is sufficient to support maximum number of ZC sequences with N=16/8/4 for M=1/2/4 with a single root.

Proposal 3：For RRC idle/inactive state, the number of roots (in specification) is up to 2.

Proposal 4：For RRC idle/inactive state, the number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip

Observation 2 ：The OFDM-based LP-WUS signal ends earlier than the OOK-based LP-WUS signal if maximum candidates sequence number value is larger than 2.

Proposal 5：Multiple repetitions can be supported for OFDM-based LP-WUS signal.

Proposal 6: Support the following working assumption:

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

- The maximum number of codepoints checked per MO by a UE is up to 8

- Depending on UE capability, a UE may support less than 8

Proposal 7：For WUS information carried by the overlaid OFDM sequence(s), support

-	Alt 1: Raw information bits are mapped to sequence(s).

Proposal 8: For the coding of LP-WUS information bits with rate matching, the modulation order can be replaced by scaling factor for section 5.3.3.1/5.3.3.2 of 38.212.

Proposal 9: For the coding of LP-WUS information bits with rate matching, truncation can be considered for the encoded bits for more than 2 information bits.

Proposal 10: For the coding of LP-WUS information bits with rate matching, scrambling bits to maximize the Euclidean distance of the modulation symbols carrying the information bits are still needed.

Proposal 11: Define multiple candidates overlaid OFDM sequences for LP-SS, and one sequence can be configured to apply on LP-SS.

Proposal 12: Regarding whether to support additional sync signal to LP-SS, Option 2 is supported and the additional synchronization signal can be the preamble prepending LP-WUS.

Proposal 13: The presence of preamble for LP-WUS is configurable.

Proposal 14: LP-SS/LP-WUS beam sweeping pattern is left for gNB configuration.

Proposal 15: Collisions handling between LP-WUS/LP-SS and legacy NR channels needs to be considered.

## R1-2502167 CMCC

Proposal 1: For WUS information carried by the overlaid OFDM sequence(s), support Alt 1:

- Alt 1: Raw information bits are mapped to sequence(s)

Proposal 2: The overlaid OFDM sequence carries WUS information bits in ascending order.

Proposal 3: Support to specify time domain signal before DFT/LS/IFFT processing for LP-WUS/LP-SS waveform generation for both OOK-4 and OOK-1.

Proposal 4: The multiplexing between legacy NR signal and LP-WUS/LP-SS should be before IFFT.

Proposal 5: Confirm the working assumption:

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

    - Depending on UE capability, a UE may support less than 8

Proposal 6: For LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, both 1-to-1 and 1-to-all mapping from codepoint to UEs are supported.

Proposal 7: For the M value for LP-WUS and LP-SS, support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

Proposal 8: Support additional sync signal with LP-SS periodicity =320ms.

- Without additionally support other LP-SS periodicities.

- The LP-SS sequence can be reused as the additional sync signal.

## R1-2501737 EURECOM

Proposal 1: Consider if pulse-shaping is required after sequence design and potential preamble are agreed.

Proposal 2: The DFT-shift is compensated at the LR.

Proposal 3: Do not consider mapping/quantizing WUS in frequency-domain.

Proposal 4: Specify Manchester Coding as 0 [1 0] and 1[0 1].

Proposal 5: For M=4, consider jointly encoding multiple bits into ON pulse position to increase SNR by 3dB.

Proposal 6: Allow configuration of Pulse Position Coding for M=4.

Proposal 7: Consider 4 different roots for inter-cell interference mitigation.

Proposal 8: CS(s) and/or roots(s) are derived from the WUS configuration without need for explicit signaling.

Proposal 9: Reuse ON-Sequences specified from sequences available if overlaid OFDM sequences carry information.

Proposal 10: Support Alternative 1, mapping raw information bits to sequence(s)

Observation 6:

•	COR-WUR performs better than COR-WUR-OOK due to the processing gain of carrying out longer correlations.

•	Transmitting the same payload as the OOK waveform with the overlaid OFDM sequences but in a different bit sequence yields a significant performance gain.

•	Using Pulse Position Coding and increasing the number of sequences results in a significant performance gain

Proposal 11: For multiple ON-Sequences, jointly encode the payload with OOK and sequence encoding.

Proposal 13: For M=4 and L=32, consider Set 3 for superior detection performance.

## R1-2501875 Spreadtrum Communications

Proposal 1: The following working assumption should be confirmed.

Proposal 2: In RRC idle/inactive mode, the maximum information bit size per MO is 5.

Proposal 3: For connected mode, the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell depends on UE capability.

Proposal 4: For WUS information carried by the overlaid OFDM sequence(s), raw information bits are mapped to sequence(s).

Proposal 5: If CRC length is small enough, OOK payload (channel coding + CRC) can be supported.

Proposal 6: OOK-1 can be supported for R19 LP-SS similar to LP-WUS.

Proposal 7: OOK-4 with M=2 or 4 can be supported for R19 LP-SS.

Proposal 8: LP-SS and LP-WUS has the same waveform in each combination.

Proposal 9: For LP-SS, multiple OFDM sequences overlaid on an OOK symbol may have low priority currently, but cell ID can be considered in OFDM sequence generation.

Proposal 10: LP-WUS/LP-SS frequency resource is configured within a carrier, which can be out of initial DL BWP.

## R1-2502257 OPPO

- LP-WUS

- Support the same design of overlaid OFDM sequence length, i.e. LZC = 12*X/M for M=1.

- Confirm the working assumption for M=1 with removing the ‘DFT size is 2^n’.

- Confirm the working assumption. Support M=4 for 30KHz SCS.

- For LP-SS.

- Single overlaid sequence is on each OOK ‘ON’ symbol.

- Prefer to use different roots, i.e. 4 roots to generate the overlaid OFDM sequence for different cells to minimize the interference.

- In case of LP-SS reuse a specified overlaid OFDM sequence for LP-WUS, if same M value is configured for LP-SS and LP-WUS. It needs to determine which overlaid OFDM sequence is reused to generate the waveform of LP-SS.

  - At least for case2, single overlaid OFDM sequence on each OOK ‘ON’ symbol not carrying information for LP-WUS and LP-SS, support to use the same overlaid OFDM sequence.

- It needs to clarify which number of candidates overlaid sequences is supported except for the maximum value.

- Support same number of roots to generate the overlaid OFDM sequence for different M values. The number of roots is 4.

  - M=1,2: different roots with cyclic shift to generate the overlaid sequences

  - M=4: different roots to generate the overlaid sequences.

- In case of one LO is associated with multiple POs, it needs to determine which PO’s subgroup is woke-up by the codepoint in LP-WUS.

- In case of one LO is associated with multiple POs, multiple fields similar as PEI could not work directly.

- In case of one LO is associated with multiple POs, support additional field to indicate the associated PO. The codepoint corresponding to the indicated PO is transmitted in another field.

  - Codepoint with 5bits payload is enough.

- For connected mode, support multiple fields to indicate different codepoints in one LP-WUS.

- Support alt2: Raw information bits are mapped to sequence(s) after channel coding.

- No need to consider repetition before or after Manchester coding. Rate matching could have the same effect if gNB configure the enough resources.

- Support rate matching after Manchester coding, regardless of whether channel coding or not before Manchester coding.

- LP-SS signal

- LP-SS uses a binary sequence associated to the cell ID by configuration. FFS: mapping schemes between cell ID and LP-SS sequences.

- If support one LP-SS set for each M value. Considering same time duration of LP-SS with different M values. We suggest alt 1 or alt3.

  - Alt1: M=1, L=4; M=2, L=8, M=4, L=16;

  - Alt3: M=1, L=8; M=2, L=16, M=4, L=32;

- If support only one LP-SS set. Considering the best sync performance. We suggest M=4, L=32.

- Support sequence set2 for M=4.

- Support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Considering timing error requirement for M=4, support option 1A: No additional sync signal, LP-SS periodicity =320ms, and additional 80ms periodicity.

- LP-WUS and LP-SS share the same frequency location, SSB location should be associated with LP-WUS/LP-SS.

- Consider shorter periodicity like 80ms or 160ms for LP-SS.

- Multiple LP-SSs can be transmitted in a period. Each LP-SS can be associated with a beam/SSB.

- Support LP-WUS/LP-SS frequency resource can be out of initial DL BWP.

## R1-2502580 Panasonic

Proposal 1: If OOK-1 is supported and specified, no need to support M=1 for OOK-4, which duplicates the feature design.

Proposal 2: For FR1, OOK-4 M=4 for 15kHz and M=2 for 30kHz are supported. Either 15 or 30kHz is specified for OOK-1.

Proposal 3: For FR2, OOK-4 with M=2 for 60 kHz SCS and OOK-1 for 120kHz SCS are supported.

Proposal 4: For Option 2-2, at least detailed design of Option 2-2-1 should be supported. FFS Option 2-2-2.

  - Option 2-2-1: Overlaid sequence per OOK ON chip is same and associated with all the OOK chips of LP-WUS. Overlaid sequence in each individual OOK ON chip conveys all the information bits carried by OOK binary sequence.

  - Option 2-2-2: Overlaid sequence per OOK ON chip is associated with an OOK chip group. An overlaid sequence per OOK ON chip conveys the information bits carried by the OOK chip group associated with the OOK chip where overlaid sequence is transmitted. One LP-WUS consists of multiple concatenating OOK chip groups.

Proposal 5: For WUS information carried by the overlaid OFDM sequence(s), raw information bits are mapped to sequence(s), i.e., Alt 1 is supported.

Observation 1: For OFDM-based LP-WUR, it is beneficial to reuse MR RF by limiting frequency location of LP-WUS/LP-SS within a maximum gap from CD-SSB.

Proposal 6: To facilitate the implementation and deployment of the OFDM-based LP-WUR, it is proposed to define frequency location of LP-WUS/LP-SS with below restriction,

- LP-WUS/LP-SS frequency location has a maximum gap of X MHz from the CD-SSB. The candidate value of X should consider and balance the need from UE side to reuse MR RF and network side LP-WUS/LP-SS resource allocation flexibility.

- It can be up to UE capability whether confined by such limitation.

Proposal 7: For LP-SS, sequence hopping should be studied to mitigate the inter-cell interference, while the complexity of the design is managed with sufficient simplicity.

Proposal 8: The M values for LP-WUS and LP-SS are always same. Our second preference is Alt 3: The M values for LP-WUS and LP-SS can be configured to be same or different by configuration, if more flexibility is desired.

## R1-2501710 FUTUREWEI

LP-WUS Structure

Proposal 1: Support Option 1A (No additional sync signal with LP-SS periodicity ∈{80,160,320}ms) if the 90% target residual time error of T=0.5 μs is considered for OOK-4/M=2 and OOK-4/M=4 waveforms and the residual frequency error is restricted to ≤ 5 ppm for OOK-4/M=4 waveform. Otherwise, support Option 2.

Proposal 2: Support LP-WUS to carry a code block length of at least 12 bits and up to 6 bits of message length with repetition after Manchester encoding for LP-WUS information carried by OOK.

Proposal 3: Support Alt 2, i.e., “Raw information bits are mapped to sequence(s) after channel coding”, with different rate matching and/or repetition factor as OOK for LP-WUS information carried by the overlaid OFDM sequence(s).

LP-WUS Waveform

Proposal 4: Confirm the working assumption that a LP-WUR-enabled UE supports OOK-4 based LP-WUS design with M=4 regardless of SCS at least in RRC Connected state.

Proposal 5: Confirm the RAN1#118b working assumption that for OOK-1 and OOK-4 M=1, the overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing.

Proposal 6: For LP-WUS information carried by overlaid OFDM sequences, support the use of combination of ZC sequence roots and cyclic shifts at least for Nc=8 candidate overlaid OFDM sequences with at least two repetitions.

Proposal 7: Support the use of OOK symbols and overlaid sequences to detect the LP-WUS source bits for shorter OFDM based LP-WUR’s detection time and improved power saving.

Proposal 8: Support UE’s determination of an overlaid OFDM sequence root/root pair based on cell identifier and cyclic shifts C_v according to C_v∈{l×⌊(M_q×B_ZC)/N_c⌋, l=0,1,…,N_c/M_q}, where M_q=1 for single roots and M_q=2 for root pairs.

Proposal 9: Support OOK-1 and OOK-4 based LP-WUS design with low frequency envelope channels to enable ED-based LP-WURs robustness against narrowband and inter-cell interference.

LP-SS Design

Proposal 10: Do not support the LP-SS sequence sets for M=1 and L∈{4,6,8}.

Proposal 11: Do not support the LP-SS sequence sets for M=2 and L∈{8,12}.

Proposal 12: Do not support LP-SS sequence sets for M=2 & L=16, or LP-SS sequence Set 1 for M=4 & L∈{16, 32}. Only support LP-SS sequence Set 2 for M=4 and L∈{16, 32} to benefit from:

- lower resource overhead,

- lower number of required measurement samples, and

- improved timing synchronization performance.

Proposal 13: Support Alt2 for the M value, i.e., “the M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS”.

RAN4#114 LS

Proposal 14: For RAN4 LS, the chip rate of LP-WUS or LP-SS OOK sequence can range from 14 kbps (OOK-1/M=1 at 15kHz SCS) to 112 kbps (OOK-4/M=4 at 30kHz) if the working assumption “M=4 for 30kHz SCS” is confirmed.

Proposal 15: For RAN4 LS, the length of information bits (the number of ON/OFF symbols for the single LP-WUS transmission) of OOK sequence can be ≥ 24 based on a 12 bit code block length and Manchester encoding.

Proposal 16: For RAN4 LS, the LP-WUS coding of information bits carried by OOK is based on Section 5.3.3.1, Section 5.3.3.2, and Section 5.3.3.3 for 1, 2, and >2 information bits, respectively, without CRC and with potential repetition before or after Manchester coding.

## R1-2502694 Honor

Proposal 1: Support alt3 for the M value for LP-WUS and LP-SS.

Proposal 2：Support M=4 for 30kHz SCS.

Proposal 3: The value of M is independent of SCS.

Proposal 4：Specify only the necessary steps for the design of OOK-1 and OOK-4.

Proposal 5: Specifies only the overlaid sequence for OOK-1.

Proposal 6: Specifies the two steps of sequence mapping and DFT for OOK-4.

Proposal 7: Further discuss how the UE obtains the OOK waveform generation scheme.

Proposal 8: The candidate overlaid sequence set of the cell is configurable.

Proposal 9: The overlaid sequence configuration reuses the configuration method of PRACH preamble sequences.

Proposal 10: Support alt3 in carrying information for overlaid sequences.

Proposal 11: Confirm the following working assumption:

Support the following options for LP-SS

- Option 1: OOK-1

- Option 2: OOK-4 with M=2,4

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

Proposal 12: The following options are supported for the length L of LP-SS binary sequence:

- M=1, L= {8}

- M=2, L= {16}

- M=4, L= {32}

## R1-2502302 InterDigital, Inc

Proposal 1. Support Option 1B with no additional sync signal, LP-SS periodicity = 320ms, and no additional support of other periodicities.

Proposal 2.  Confirm the working assumption on modulation orders for LP-SS to include both Option 1 with OOK-1 and Option 2 with OOK-4 and M = 2 or 4.

- Do not support additional M values.

Proposal 3.  Support Alt1 for M values selection, that is, the M values for LP-WUS and LP-SS are always same.

Proposal 4. In determining the binary sequences for LP-SS, support selecting one L-value applied to each M-value.

Proposal 5. In determining the binary sequences for LP-SS, support selecting the L-value applied to different M-values, such that the number of OFDM symbols is the same for different M-values.

Proposal 6. In determining the binary sequences for LP-SS, support following L-values:

- For M = 1, support L = 8,

- For M = 2, support L = 16,

- For M = 4, support L = 32.

Proposal 7. Procedures for handling inconsistencies in RRM measurements based on LP-SS and RRM measurements based on NR-SS, e.g., due to RRM measurements inaccuracies based on LP-SS, should be supported.

Proposal 8. LP-WUS/LP-SS frequency resource can be configured out of initial DL BWP’.

Proposal 9: Support 5 bits as the maximum number of information bits.

Proposal 10: Down select raw information bits after channel coding (Alt 2) and codepoint/subgroup mapped to sequence(s) (Alt 3) are down selected for further down selection.

Proposal 11: The OOK waveform without overlaid OFDM sequence is not specified in RAN1 and define RAN4 requirements to guarantee the OOK detection performance.

## R1-2502322 Sony

Proposal 1 – RAN1 to support setting M value to its maximum, i.e., M=4 for 30 kHz SCS and M=8 for 15 kHz to allow for low system overhead, lower latency and less power consumption at the LP-WUR.

Proposal 2 – RAN1 to support the same SCS configuration as other NR as it has less complexity for gNB.

Proposal 3 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of subsequent actions and/or wake-up group identity, depending on state of the operation.

Proposal 4 – RAN1 to support the use of one configured OFDM overlaid ZC sequence per UE and carry the LP-WUS data in a cyclic shift of the overlaid sequence.

Proposal 5 – RAN1 to support carrying information bits in cyclic shifts of the ZC sequence configured as  the OFDM LP-WUR synchronization sequence for the overlaid OFDM LP-WUS.

Proposal 6 – RAN1 to support that the OFDM sequence overlaid on the LP-SS is the same sequence used to carry data in its cyclic shifts on LP-WUS data symbols.

Proposal 7 – RAN1 to consider cell-ID, UE-ID and/or paging group-ID in the choice of the ZC sequence configured as for the overlaid OFDM LP-WUS.

Proposal 8 – When carrying information, the information carried in the overlaid sequence should be only for the purpose of reducing the total length of the LP-WUS received by OFDM based LP-WUR without degrading performance of the OOK-based LP-WUR.

Proposal 9 – The OFDM overlaid sequence carrying cell information, should be only used for the purpose of assisting OFDM-based LP-WUR detection and not carrying any additional information that cannot be detected by the OOK-based LP-WUR.

Proposal 10 – RAN1 to consider having the same value of M for both LP-SS and LP-WUS.

Proposal 11 – Support inclusion of association of cell ID to the LP-SS sequences, e.g., by choosing LP-SS with max. value of M and L.

Proposal 12 – RAN1 to revise the agreed number of sequences and allow for larger number of sequences.

Proposal 13 – Consider additional aperiodic synchronization signal where the signal/sequence is transmitted as part of LP-WUS.

Proposal 14 - Support longer periodicity of LP-SS to allow reduced resource, power consumption and spectrum usage.

## R1-2501845 TCL

Proposal 1: RAN1 to consider including the physical cell ID (PCI) into the LP-SS signal

Proposal 2: For the candidate values of LP-SS periodicities, support 80ms, 160ms, 320ms, 640ms, 1280ms, and 2560ms for UEs in both RRC idle/inactive and connected states.

Proposal 3: Consider multiple candidate payload sizes for the LP-WUS, which can be configured to the UEs in both idle/inactive and connected states.

Proposal 4: For LP-WUS information carried by the overlaid OFDM sequence(s), support Alt 3: Codepoint/Subgroup is directly mapped to sequence(s).

Proposal 5: Support the following working assumption:

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8

Proposal 6: For the LP-WUS information used to trigger PDCCH monitoring for RRC connected UEs, to support the checking of up to 8 codepoints in a MO, the association/mapping between the codepoints in an LP-WUS and the MOs is configured to the UEs.

## R1-2502481 LG Electronics

Proposal #1: Confirm to support OOK-4 with M=4 for 30 kHz SCS

Proposal #2: Not support M=4 for LP-WUS/LP-SS in FR2

Proposal #3: The maximum information bit size for both Idle/Inactive mode and Connected mode should be 5

Proposal #4: Support the following code block length for LP-WUS

- For more than 2 information bits: gNB configures the code block length with maximum length of 32

- For 2 information bits: code block length is 3 bits

- For 1 information bit: code block length is 1 bit

Proposal #5: Not support adding scrambling bits for LP-WUS channel coding

Proposal #6: Additional repetition can be considered at least when the information bits carried by LP-WUS is 1 bit (FFS: for 2-bit information)

- For more than 2 bits, no need to apply the additional repetition on top of RM coding

Proposal #7: For the overlaid OFDM sequence of OOK-4 with M=1, the sequence length LZC can be determined as LZC=2k, where 2k≤12∗X and X is the number of RBs of LP-WUS bandwidth (blanked guard RBs are not included)

Proposal #8: For the maximum number of candidates for the overlaid OFDM sequence per OOK ON symbol for a serving cell, consider the followings as a baseline capability for Connected mode UE

- Maximum 16 candidates overlaid sequences for M=1

- Maximum 8 candidates overlaid sequences for M=2

- Maximum 4 candidates overlaid sequences for M=4

Proposal #9: For a given cell, different CS can be used to differentiate sequences

- Different root for different cells to mitigate inter-cell interferences, if necessary

Proposal #10: Support the overlaid OFDM sequence carries the information bits without channel coding

- FFS: Simple repetition can be further discussed

Proposal #11: Discuss the applicable M value for the small LP-WUS bandwidth

Proposal #12: For the overlaid OFDM sequence on each OOK ON symbol within an OFDM symbol, two sequences with different phases are alternatively used on each OOK ON symbol

Proposal #13: The bit ordering of information carried by overlaid OFDM sequence can be a shifted version of bit ordering for OOK symbols

Proposal #14: Discuss further the bit ordering of information carried by overlaid OFDM sequence when LP-WUS repetition is configured

Proposal #15: In case of overlaid OFDM sequence not carrying information, one sequence is selected from multiple candidates overlaid OFDM sequences which can be configured for the overlaid sequences carrying information bits

Proposal #16: Support the frequency resource for LP-WUS/LP-SS is confined within initial DL BWP

Proposal #17: Support the same frequency resource for LP-WUS and LP-SS

Proposal #18: Confirm the working assumption for LP-SS waveform generation

Proposal #19: At least for FR2, the value of M could be determined based on the SCS of LP-WUS/LP-SS

Proposal #20: The M value for LP-SS can be configured to be same or greater than that for LP-WUS

Observation #1: Applicable L for LP-SS can be configured depending on SCS for LP-WUS and SCS for LP-SS when multiple L values are configured to each M

Proposal #21: Discuss whether and how to configure the value of M for LP-SS with consideration of the followings:

- Value of M for LP-WUS waveform

- SCS of LP-WUS/LP-SS

- LP-SS periodicity

- Sequence length of LP-SS (i.e., LP-SS duration)

Proposal #22: Support multiple L values for each M considering flexibility for gNB perspective

- L=4 and 8 for M=1

- L=8 and 16 for M=2

- L=16 and 32 for M=4

Proposal #23: Specify the overlaid OFDM sequence in frequency domain for OOK-1

Proposal #24: Overlaid OFDM sequence for LP-SS can be determined as one of the overlaid sequence candidates for LP-WUS

Proposal #25: Multiple LP-SS periodicities need to be supported for various scenarios

Proposal #26: Additional sync signal needs to be supported to accommodate M=4 for LP-WUS

Proposal #27: On the additional sync signal, Option 2 should be supported to receive LP-WUS which accommodates various M values

- Option 2: With additional sync signal, LP-SS periodicity =320ms

- Reuse the agreed binary sequence for additional synchronization signal

## R1-2502075 NEC

Proposal 1: support flexibly configuring frequency locations of one or more LP-WUS bands within a carrier, UE can select an LP-WUS band based on its UE ID or a PF/PO it is intended to monitor.

Proposal 2: support message based LP-WUS structure with a preamble and a CRC.

Proposal 3: support repetition of LP-WUS to improve the coverage.

Observation 1: for UE with OFDM LR, it is beneficial if UE is allowed to receive only partial OOK symbols of LP-WUS and acquire the information bits based on the OOK on-off pattern and overlaid sequences of the partial OOK symbols.

Proposal 4: the order/pattern of information bits mapped to overlaid sequences should facilitate UE to acquire information bits based on partial OOK symbols, e.g., first half of the OOK symbols, of the LP-WUS.

Proposal 5: support FDM multiplexing of an LP-SS and its QCLed SSB.

Proposal 6: for each M values for LP-SS, support at least the following sequence length L:

- M=1, L= 4

- M=2, L= 8

## R1-2502773 NTT DOCOMO, INC

Proposal 1

- LP-WUS/LP-SS frequency resource is confined within legacy initial DL BWP

Proposal 2

- The way of carrying the overlaid OFDM sequences is selected from the following options

  - Option 1: The information bits are arranged in order from front to back

  - Option 2: The information bits are set in order from the back

Proposal 3

- The way of the overlaid OFDM sequences repetition is selected from the following options

  - Option 1: All information bits level repetition

  - Option 2: On chip bit group level repetition

Proposal 4

- Overlaid OFDM sequence is used for both repetition and early termination

Proposal 5

- For OOK-4 M=1, L=8 is supported

- For OOK-4 M=2, L=16 is supported

- For OOK-4 M=4, L=32 is supported

Proposal 6

- Preamble is not supported

- For OOK-4 M=4, Shorter LP-SS periodicity is supported e.g., 80ms, 160ms

Proposal 7

- Remove the higher layer parameter [LP-SS_Mvalue] in higher layer parameters list

- LP-WUS_Mvalue_IDLE/INACTIVE is used to configure the M values for both LP-WUS and LP-SS

Proposal 8

- Regarding [LP-WUS/LP-SS_startRB_IDLE/INACTIVE] in higher layer parameters list,

  - Relative RB position from the edge or center of the initial DL BWP is configured

Proposal 9

- Remove the bracket of [1…[4]]

Proposal 10

- Regarding LP-SS_periodicityoffset in higher layer parameters list,

  - For M=1,2, UE assumes 320ms periodicity without higher layer configuration

  - For M=4, UE assumes [80 or 160]ms (to be down-selected) periodicity without higher layer configuration

Proposal 11

- Regarding [additionalsync] in higher layer parameters list,

  - Remove [additionalsync]

Proposal 12

- Before discussing the details on LP-WUS_overlaidSeq_IDLE/INACTIVE and LP-WUS_overlaidSeq_CONNECTED  in higher layer parameters list, followings should be firstly discussed

  - How to send information via overlaid OFDM for LP-WUS in IDLE/INACTIVE mode and CONNECTED mode

Proposal 13

- Regarding [Indicates the starting RB of LP-WUS in RRC CONNECTED] in higher layer parameters list,

  - Remove the bracket of [Indicates the starting RB of LP-WUS in RRC CONNECTED]

Proposal 14

- Before discussing the details on WUS_codepoint_CONNECTED in higher layer parameters list, following should be firstly discussed

  - Whether a codepoint corresponds to [one or more] UEs

## R1-2502675 Sharp

Proposal 1: No need to specify repetition after channel coding with rate matching for LP-WUS information.

Proposal 2: Support CRC attachment after RM encoding.

Proposal 3: Consider the group size of different LP-WUS information within one LO in the LP-WUS design and apply different code rates accordingly.

Proposal 4：Support Lzc = 132 as the overlaid sequence length for M = 1.

Proposal 5: Support Option 1A, where no additional synchronization signal is required, and allow a smaller configurable periodicity for LP-SS.

Proposal 6: Consider two methods for configuring LP-WUS resources:

- BWP-specific mode, where LP-WUS resources are configured per BWP.

- Cell-specific mode, where LP-WUS resources are configured at the cell level.

## R1-2502880 Nordic

Proposal-1: Set of root sequence indices and corresponding cyclic-shifts are configured to a UE according to table below. NCS=16.

|  | Max # of roots defined in 38.211 | Max # CS per root defined in 38.211 | Max# of root configured to a UE | Max# of combinations configured to UE for monitoring |
| --- | --- | --- | --- | --- |
| M=1 | 127 | 8 | 2 | 16 |
| M=2 | 61 | 4 | 2 | 8 |
| M=4 | 31 | 2 | 2 | 4 |

Proposal-2: Manchester coding is applied to every LP-WUS by default, i.e. irrespective of M and SCS.

Proposal-3: To simplify LP-WUS design, support only payloads of 3 and 5bits for LP-WUS

- take 32bit codeblock length as baseline.

- consider the same LP-WUS length for 3 and 5 bit payloads.

- for repetitions consider sequential reading from circular buffer.

Proposal-4: For information mapping to overlaid sequences, in RAN1#120b collect proposed mapping schemes from contributions and discuss KPIs.

Proposal-5: Length of binary sequence is L=M∗[4 or 8], i.e. there are three different sequence length, one for each M.

Proposal-6: Our preference is Option 1A

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms] for M=4

## R1-2502908 Lenovo

Proposal 1: Consider preamble transmission before LO when the offset between LP-SS and the starting of LO can be more than configured value.

Proposal 2: UE can benefit from power consumption, when the preamble can be configured closer to the monitoring occasion containing codepoint corresponding to all subgroups in a LO