3GPP TSG RAN WG1 #120bis	                                   R1-2502959

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.1

Agenda Item:		9.15.1

Document for: 	Endorsement

- 9.15.1	UE features for AI/ML for NR Air Interface

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

R1-2502738	Summary of UE features for AI/ML for NR Air Interface	Moderator (AT&T)

#### Beam management

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agree

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {FFS} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for component 2 | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] [11. Supported maximum number of predicted beams in each predicted time instance] [12. Supported maximum number of predicted time instances] [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Positioning

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | [1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | [1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. [the reported value is the total number across all bands in the corresponding BC] Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

#### CSI prediction

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 [8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) ] [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] [11. Scaling factor for active resource counting Kp] | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR [5. Support for the size of DD-basis, N4>1] [6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] [8. Supported values of the maximum number of observation number Candidate values: {FFS}] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |

R1-2501829	Discussion on UE features for AI/ML for NR Air Interface	vivo

R1-2501920	Discussion on UE features for AI/ML for NR Air Interface	ZTE Corporation, Sanechips

R1-2501979	Discussion on UE features for AI/ML for NR Air Interface	CATT, CICTCI

R1-2502047	Views on UE features for AI/ML for NR Air Interface	Ofinno

R1-2502102	Discussions on UE features for AI/ML for NR Air Interface	LG Electronics

R1-2502133	Views on UE features for AI/ML for NR Air Interface	Nokia

R1-2502178	Discussion on UE features for AI/ML for NR air Interface	CMCC

R1-2502246	UE features for AI/ML for NR air interface	Huawei, HiSilicon

R1-2502292	UE features for AIML for NR air interface	OPPO

R1-2502391	UE features for AI/ML for NR Air Interface	Samsung

R1-2502459	Discussion on UE features for AI/ML for NR Air Interface	Xiaomi

R1-2502639	View on UE features for R19 AI/ML for NR Air Interface	Apple

R1-2502661	UE Features for Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2502785	Discussion on UE features for AI/ML for NR Air Interface	NTT DOCOMO, INC.

R1-2502863	Rapporteur's view on Rel-19 UE features for AI/ML air interface	Qualcomm Incorporated

R1-2502864	UE features for AI/ML for NR air interface	Qualcomm Incorporated