3GPP TSG RAN WG1 #121	 R1-2503547

# Malta, MT, May 19th – 23rd, 2025

Agenda item:	7

Source:	Samsung

Title:	Discussion for HARQ-ACK aspects of Rel-17 multi-PDSCH scheduling

Document for:	Discussion and decision

# Introduction

The Rel-17 WI on “Extending current NR operation to 71 GHz” specified support for multi-PDSCH scheduling, where a single DCI format can schedule multiple PDSCHs on a same serving cell, and corresponding HARQ-ACK information can be provided per transport blocks (TBs) or per transport block groups (TBGs) / PDSCH reception groups.

This contribution discusses an issue related to HARQ-ACK information for Rel-17 multi-PDSCH scheduling in case of multiple overlapping DCIs/PDCCHs that at least some of them schedule multiple PDSCHs. A text proposal (TP) is discussed here and formally presented in the draft CR in [1].

# Counting of overlapping PDCCHs that schedule multiple PDSCHs

The Rel-16 WI on MR-DC/CA enhancements introduced a UE capability “type2-HARQ-ACK-Codebook” (FG 18-9) to support Type-2 HARQ-ACK codebook for more than one PDCCHs/DCIs that are received in the same PDCCH monitoring occasion (MO) and schedule PDSCHs on the same serving cell, where the corresponding DAI / HARQ-ACK information is in ascending order of PDSCH reception starting time.

In particular, the UE behavior for FG 18-9 was clarified in CRs [2] and [3] for Rel-17 and Rel-18, respectively, as follows:

[Excerpt #1 from TS 38.213 v17.12.0, Clause 9.1.3.1]

However, as can be inferred from the CRs and the discussion summary in [4], the clarification only considered the case of single-PDSCH scheduling, and the case of a DCI format 1_1 with multi-PDSCH scheduling as per Rel-17 was not considered.

In fact, the support for receiving more than one multi-PDSCH scheduling DCI formats in a same PDCCH MO (or at least one multi-PDSCH scheduling DCI format together with at least one single-PDSCH scheduling DCI format) is already supported in the RAN1 specifications, such as the following Excerpt #2:

[Excerpt #2 from TS 38.213 v17.12.0, Clause 9.1.3.1]

If the current Rel-17 specifications, as per Excerpt #1 / the CR in [2], is directly applied to Rel-17 multi-PDSCH scheduling, the UE needs to count (and run the pseudo-code for) a corresponding serving cell based on the number of scheduled PDSCHs. For example, if a UE receives 2 DCI formats, in the same PDCCH MO, each scheduling 4 PDSCHs on a same serving cell, the current specifications would count the serving cell a total of 8 times. Such counting would be against the DAI principle agreed in Rel-17 multi-PDSCH scheduling, that DAI counting is per DCI format, and not per scheduled PDSCH. In the previous example, the UE is expected to count the serving cell only 2 times in that MO.

Therefore, clarification is necessary that the agreed CRs in [2] and [3] were for the case of single-PDSCH scheduling, and that counting for the case of multi-PDSCH scheduling is based on the number of DCI formats received for a serving cell in a same PDCCH MO. Such clarification is captured in the following TP, which is formally presented in the draft CR in [1].

[Suggested TP for TS 38.213 v17.12.0, Clause 9.1.3.1] - if the UE indicates type2-HARQ-ACK-Codebook and receives a number NPDSCH, cm>1 of PDSCHs on a serving cell c that are scheduled by a number NPDCCH, cm>1 of DCI formats, where NPDCCH, cm≤NPDSCH, cm, in PDCCH receptions at a same PDCCH monitoring occasion m, the serving cell c is counted NPDSCH, cmNPDCCH, cm times for PDCCH monitoring occasion m in increasing order of the PDSCH reception starting time associated with the respective NPDCCH, cm DCI formats

Proposal: Clarify that, when a UE receives multiple DCI formats in a same PDCCH MO, and at least one of the DCI formats schedules multiple PDSCHs on a same serving cell, the UE counts (and runs the Type-2 HARQ-ACK pseudo-code for) the serving cell a same number of times as the number of DCI formats – and not the number of scheduled PDSCH receptions.

- Adopt the TP proposed in draft CR [1].

# Conclusions

This contribution considered the issue of counting overlapping DCIs / PDCCHs as related to HARQ-ACK information for multi-PDSCH scheduling, and made the following proposal.

Proposal: Clarify that, when a UE receives multiple DCI formats in a same PDCCH MO, and at least one of the DCI formats schedules multiple PDSCHs on a same serving cell, the UE counts (and runs the Type-2 HARQ-ACK pseudo-code for) the serving cell a same number of times as the number of DCI formats – and not the number of scheduled PDSCH receptions.

- Adopt the TP proposed in draft CR [1].

# References:

- R1-2503980, Draft CR for HARQ-ACK aspects of Rel-17 multi-PDSCH scheduling, RAN1#121, Samsung.

- R1-2310666, Correction on multiple DCIs per MO per CC for the Type-2 HARQ-ACK codebook (Rel-17), RAN1#114bis, Moderator (Samsung) and Ericsson.

- R1-2310667, Correction on multiple DCIs per MO per CC for the Type-2 HARQ-ACK codebook (Rel-18), RAN1#114bis, Moderator (Samsung) and Ericsson.

- R1-2310512, Summary of discussion on the correction on Type-2 HARQ-ACK codebook for receiving multiple DCIs per MO per CC, RAN1#114bis, Moderator (Samsung).