3GPP TSG RAN WG1 #121		                                  R1-2503598

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.15.8

Source:			Samsung

Title:			UE features for NTN for NR Phase 3

Document for:		Discussion and decision

- Introduction

This contribution considers UE features for NTN for NR Phase 3.

Agreement: Introduce the following FG for the support of PDCCH repetition with CSS for Rel-19 NR-NTN: 65-1-2; PDCCH repetition for Type0 PDCCH CSS; 1. Support reception of PDCCH repetition for Type0 PDCCH CSS [of searchSpaceZero configured within MIB pdcch-ConfigSIB1] FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition; FFS:; FFS; N/A; PDCCH repetition for Type0 PDCCH CSS is not supported; Per band; N/A; N/A; N/A; Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN]; Optional [with/without] capability signaling. 65-1-3; PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS; 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS; FFS:; FFS; N/A; FFS; Per band; N/A; N/A; N/A; Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN]; Optional [without] capability signaling. Agreement: Introduce the following FG for the support of handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE: 65-2-1; Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE; [1. Support handling collision Case 3] [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state; FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2; YES; N/A; Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported; Per band; N/A; FFS; N/A; Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only; Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG.

- Discussion

In [1], following was agreed as baseline for the support of 160ms SSB periodicity during initial access for Rel-19 NR-NTN.

| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access] [FFS Other components] | FFS | FFS | N/A | FFS: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 | Optional [with/without] capability signaling FFS: A UE that supports Rel-19 NR-NTN must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Components: It is preferable to reuse the WID description, i.e., “initial access.”

- Prerequisite: It could consider the basic NTN feature component “26-1 (Uplink Time and Frequency Pre-Compensation and Timing Relationship Enhancements)” as this is a mandatory feature in NTN UE. That is, the UE must indicate that this FG is supported for the UE to support NR communication via satellite. However, if this feature group is confined within NTN-specific bands, then it is not necessary to consider this FG as a prerequisite.

- Need for the gNB to know if the feature is supported: It is unclear how the gNB could know this capability previously, as this feature is related to SSB reception. In that sense, it should be “No.”

- Consequence if the feature is not supported by the UE: It is sufficient not to support additional default values rather than explaining legacy SSB periodicity. For example, the UE does not support the default value (apart from the 20 ms value) of 160 ms periodicity of the half frames with SS/PBCH blocks during initial access.

- Note: Though this feature is considered based on FR1 NTN evaluation, it is open to expanding to FR2-NTN if there is strong motivation. Otherwise, it is preferable not to add [5.2.3-1].

- Mandatory/Optional: It should be optional without capability signaling, as there is no mechanism to report this capability before initial access. Regarding FFS, it is open to making all Rel-19 NR-NTN UEs support this feature.

Proposal 1: Consider the above updates for the support of 160ms SSB periodicity during initial access for Rel-19 NR-NTN.

For uplink capacity/throughput enhancement, it was agreed to support separate UE capabilities for OCC lengths of 2 and 4. Therefore, at least two UE features are required.

Proposal 2: Introduce the following two UE capabilities to support OCC for NPUSCH format 1 as a baseline.

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z-1 | Inter-slot OCC based PUSCH transmission with OCC length 2 | 1. Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 2. Orthogonal sequences [1 1; 1 -1] are used 3. RV cycling is used across OCC groups, and same RV value is used in one OCC group |  |  |  |  |  | Optional with capability signaling |
| Z-2 | Inter-slot OCC based PUSCH transmission with OCC length 4 | 1. Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 4 2. Hadamard sequences are used 3. RV cycling is used across OCC groups, and same RV value is used in one OCC group | Z-1 |  |  |  |  | Optional with capability signaling |

- Reference

[1] R1-2502967, Session Notes of AI 9.15.8