3GPP TSG RAN WG1 #121		                                    R1-2503604

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.15.14

Source:			Samsung

Title:			UE features for Rel-19 TEI

Document for:		Discussion and decision

- Introduction

This contribution provides Samsung’s view regarding UE features for supporting Rel-19 TEI.

- Discussion

In RAN1#120bis, the following agreement was made in Rel-19 TEI session [1].

Agreement in RAN1#120bis For simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41, 2-43, 16-3a and 16-3b, define 1 new UE capability: - UE capability 1: To allow the UE to indicate that CSI-RS ports within one periodic/semi-persistent CSI-RS resource, as well as the periodic/semi-persistent CSI-RS resource, are counted as one resource, even if the periodic/semi-persistent CSI-RS resource is referred by N Report Settings.

Hence, we would like to suggest the following FG supporting the relevant functionality.

- Component: component 1 is basic description on the feature. Considering such new UE behavior, it is good for UE to report new values for maximum resource/port counting per CC and across CC via component 2, 3, 4, and 5.

- Pre-requisite: as agreed, it could be at least one of {2-33, 2-36, 2-40, 2-41, 2-43, 16-3a, and 16-3b}.

- Type: It could be per Band considering FG 2-33 and component 4 and 5. Component 6 and 7 can be reported per band combination.

Proposal 1. Support the following FG supporting relevant functionality of the above agreement.

| X-1 | Enhanced active resource counting for periodic/semi-persistent CSI-RS resource | 1. Support to count as one CSI-RS resource for periodic/semi-persistent CSI-RS resource/port, even if the periodic/semi-persistent CSI-RS resource is referred by N CSI reporting settings. 2. Supported max # simultaneous NZP-CSI-RS resources per CC 3. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 4. Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs | At least one of {2-33, 2-36, 2-40, 2-41, 2-43, 16-3a, and 16-3b} | FFS | FFS | Active resource/port counting for periodic/semi-persistent CSI-RS is done by N times when the periodic/semi-persistent CSI-RS resource is referred by N CSI report Settings | Per band | FFS | FFS | FFS | Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } Note: Component 6 and 7 are reported per band combination. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. Support the following FG supporting relevant functionality of the above agreement.

| X-1 | Enhanced active resource counting for periodic/semi-persistent CSI-RS resource | 1. Support to count as one CSI-RS resource for periodic/semi-persistent CSI-RS resource/port, even if the periodic/semi-persistent CSI-RS resource is referred by N CSI reporting settings. 2. Supported max # simultaneous NZP-CSI-RS resources per CC 3. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 4. Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs | At least one of {2-33, 2-36, 2-40, 2-41, 2-43, 16-3a, and 16-3b} | FFS | FFS | Active resource/port counting for periodic/semi-persistent CSI-RS is done by N times when the periodic/semi-persistent CSI-RS resource is referred by N CSI report Settings | Per band | FFS | FFS | FFS | Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } Note: Component 6 and 7 are reported per band combination. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- Chair’s note, RAN1#120bis.