3GPP TSG RAN WG1 #121		                        R1-2504178

St Julian’s, Malta, May 19th-23th, 2025

Source: 	Transsion Holdings

Title:	Discussion on adaptive transmission of common signal or common channel

Agenda item:	9.5.3

Document for:	Discussion & Decision

Introduction

In RAN1#120bis meeting, the following agreements[1] on adaptive transmission of common signal/channel were achieved：

Agreement (Legacy RO) For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon. Agreement (Bit in DCI P-RNTI) For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers). - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs. Agreement (Validity duration) For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI: - the validity duration is configured via higher layer signalling. Agreement (Unit of PRACH subset mask) For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of - PRACH association period. - This PRACH mask applies to every [configured] K SSB RO association pattern period(s). Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell, - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter, - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell. Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB Agreement For adaptation of PRACH in time-domain, for a connected mode UE, - One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources. Agreement For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by: - Option 1-2: Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods). - For K: one from up to four candidate values {2,4,8, [1 or 16]}. Mask index; Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP). 0; 1st half of the APs in K APPs. 1; 1st quarter of the APs in K APPs. 2; 1st eighth of the APs in K APPs. 3; 1st sixteenth of the APs in K APPs. Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI. - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..}).

In this contribution, some considerations on adaptation of common signal/channel will be discussed.

Adaptation of common signal/channel transmission

Adaptation of SSB in time domain

In the RAN1#120bis meeting, we discussed the application time of DCI format 2_9 and came up with the following working assumption.

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst.

Based on the agreement in AI 9.5.1, the UE can assume that the SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first actually transmitted SSB within the first possible SSB burst according to the indicated SSB burst periodicity.

For the first possible SSB burst, it may have the same half-frame index and SFN index as the SSB prior to the period change, and it differs from the SSB prior to the period change only in terms of the periodicity.

Proposal 1  For the first possible SSB burst, it may have the same half-frame index and SFN index as the SSB prior to the period change.

Since SSB is mainly used for AGC or cell synchronization, when the channel environment or service requirements change, it may be necessary to switch the SSB burst period in various scenarios. In summary, switching the SSB burst periodicity using DCI format 2_9 can be applied to all scenarios of AI 9.5.1.

Proposal 2  Switching the SSB burst periodicity using DCI format 2_9 can be applied to all scenarios of AI 9.5.1.

Adaptation of PRACH

In RAN1#120bis meeting, the following agreement was achieved.

Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB

First, according to TS 38.331, totalNumberOfRA-Preambles is mainly used to indicate the total number of preambles used for contention based and contention free 4-step random access in the RACH resources defined in RACH-ConfigCommon. Therefore, totalNumberOfRA-Preambles can be configured separately for the additional PRACH resources at least for 4-step RACH.

Second, according to TS 38.331, numberOfRA-PreamblesGroupA is mainly used to explicitly indicate the number of contention-based preambles per SSB in preamble group A and implicitly indicate the number of contention-based preambles per SSB in preamble group B. In addition, grouping preambles is beneficial for the gNB to allocate appropriate Msg3 uplink resources in RAR. Therefore, numberOfRA-PreamblesGroupA can be configured separately for the additional PRACH resources at least for 4-step RACH.

In summary, totalNumberOfRA-Preambles and numberOfRA-PreamblesGroupA can be configured separately for the additional PRACH resources at least for 4-step RACH.

Proposal 3  TotalNumberOfRA-Preambles and NumberOfRA-PreamblesGroupA can be configured separately for the additional PRACH resources at least for 4-step RACH.

In previous meeting, the following agreements were reached.

Agreement(RAN1#120 meeting) For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement (RAN1#120bis meeting) For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling.

since DCI 1_0 with P-RNTI can indicate the activation of additional PRACH adaptation, DCI 1_0 with P-RNTI can also be used to indicate the deactivation of additional PRACH resources. In addition, deactivation of additional PRACH resources by DCI 1_0 with P-RNTI is more flexible than deactivation of additional PRACH resources by validity time duration. And deactivation of additional PRACH resources by DCI 1_0 with P-RNTI does not increase the payload size of DCI compared to activation of additional PRACH resources by DCI 1_0 with P-RNTI. In summary, DCI 1_0 with P-RNTI can be used to explicitly deactivate the additional PRACH resources.

Proposal 4  DCI 1_0 with P-RNTI can be used to explicitly deactivate the additional PRACH resources.

Conclusions

In this contribution, some considerations on adaptation of common signal/channel are discussed, and the following proposals are made.

Proposal 1  For the first possible SSB burst, it may have the same half-frame index and SFN index as the SSB prior to the period change.

Proposal 2  Switching the SSB burst periodicity using DCI format 2_9 can be applied to all scenarios of AI 9.5.1.

Proposal 3  TotalNumberOfRA-Preambles and NumberOfRA-PreamblesGroupA can be configured separately for the additional PRACH resources at least for 4-step RACH.

Proposal 4  DCI 1_0 with P-RNTI can be used to explicitly deactivate the additional PRACH resources.

References

- “Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0”, RAN1#120bis