3GPP TSG RAN WG1 #121		                                           R1-2504187

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.6.1

Source: 	OPPO

Title:	Signal design for LP-WUS and LP-SS

Document for:	Discussion & Decision

Introduction

In RAN#102 meeting, the WI [1] of LP-WUS was agreed, and it includes the following aspects that related to signal design for LP-WUS and LP-SS.

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In this contribution, we further discuss the design details for LP-WUS and LP-SS.

LP-WUS signal

Waveform design of OOK-1 and OOK-4

M value for LP-WUS

Regarding the value of M for OOK-4, following agreement were reached in RAN1#118.

Agreement Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

In our views, the time domain resources required for M=4 are half of those for M=2. Thus, if the channel quality is good and the LP-WUS transmission performance is met, gNB could configure M=4 to reduce system resource overhead. On the other hands, M=4 for 15kHz SCS has been supported, it is not necessary to limit the value of M for different SCS. gNB could configure all supported value of M independently regardless of the SCS.

- Confirm the working assumption. Support M=4 for 30KHz SCS.

Root and Cyclic shift for overlaid OFDM sequence

Regarding the overlaid OFDM sequence, following agreements had been made in previous meeting.

Agreement [RAN1#118] The number of binary LP-SS sequences is 4. Agreement [RAN1#118bis] In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. Agreement [RAN1#119] Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement [RAN1#119] CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement [RAN1#119] Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4 Agreement [RAN1#120] For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement [RAN1#120] Proposal 4.3-1: Update the agreements in RAN1 #118bis as below Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Applicable to both OOK-1 and OOK-4 - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at leastfor OOK-1 only and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Working Assumption [RAN1#120b] At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). - Different roots can be picked for different M values. Agreement [RAN1#120b] For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where LZC is the overlaid OFDM sequence length.

Increasing the number of roots will impact the complexity of UE implementation, maximum 2 roots is acceptable to UE implementation. As for the roots picked for different M values, it was agreed gNB can configure any value from 1 to Bzc-1, where Bzc is given by the largest prime number such that BZC<LZC. Different M values have different Bzc. Thus, gNB could configure different range for the roots corresponding to the different M value.

Following agreement about the configuration of M values has been reached.

Agreement For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS) - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both

When overlaid OFDM sequences is used for LP-WUS and LP-SS.

  - In case of same M value is configured for LP-WUS and LP-SS, no need to configure additional root for LP-SS other than the root(s) configured for LP-WUS.

  - In case of different M value is configured for LP-WUS and LP-SS, maximum 2 roots is enough, e.g., root A for LP-SS, one or two roots of {rootA, rootB} for LP-WUS.

- Confirm the Working Assumption with following update.

Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for the transmission of LP-WUS and LP-SS each M value). - Different roots can be picked for different M values.

- LP-WUS

Regarding the overlaid OFDM sequence for LP-WUS, the number of candidates overlaid sequences for different M values is different. Per agreements, the maximum number of candidates overlaid sequences has been agreed. It is not clear which exactly number of candidates overlaid sequences is supported. For example, it supports maximum 16 candidates overlaid sequences for M=1, whether it also supports 8 or 4 candidates overlaid sequences for M=1.

- RAN1 to clarify the numbers of candidates overlaid sequences supported other than the maximum value.

Information bits

Regarding the information bits, following agreements were reached in last meeting.

Agreement [RAN1#120b] From specification design perspective, for both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints for a LP-WUS. Working Assumption Support 1 & 2 information bits for LP-WUS. No additional restriction is introduced on the supported number of subgroups/codepoints by LP-WUS. - 2 information bits: coding is done as described in the first row in the table in section 5.3.3.2 of 38.212 (scrambling is not applied) - 1 information bit: repetition is done as described in the first row in the table in section 5.3.3.1 of 38.212 (scrambling is not applied)

In CONNECTED mode, it makes sense to support 1&2 information bits. However, in IDLE/INACTIVE mode, we don’t see the necessary to support 1&2 information bits. In the specification, it seems we may not actually support that in the connection mode. Thus, we can confirm the working assumption. In the specification, we expect only CONNECTED mode may use the coding in 5.3.3.2/5.3.3.1.

- RAN1 can confirm the working assumption on the supported information bits.

Coded bits length after rate matching

Following agreement was reached in last meeting.

Agreement For > 2 information bits with RM coding, support candidate code block (coded bits) length after rate matching within the below range: - Alt 1a: code block (coded bits) length: [3,4]~[32,64] - FFS whether all or a subset of lengths is selected. - FFS whether same range applies to all M values - FFS: above lengths do not consider repetition (if supported) after rate matching

Coded bits length after rate matching depends on the resource allocation. For example, gNB configure the M value and time domain resource, e.g. L OFDM symbols. UE can determine the number of OOK chips, i.e., M*L. The coded bits length after rate matching is M*L/2. If we limit the subset of lengths, it means limit the time domain resource allocation for LP-WUS.

- Coded bits length after rate matching depends on the resource allocation, if we limit the coded bits length, it will limit the time domain resource allocation for LP-WUS.

- No restriction on the coded bits length. Exactly code block length depends on the resource allocation

Wake-up indication in IDLE/INACTIVE mode

In IDLE/INACTIVE mode, codepoint based indication has been supported. Following agreements were reached in previous meetings.

9.6.1 Agreement [RAN1#118] For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X. Agreement [RAN1#118bis] At least the following codepoints are supported for LP-WUS: - One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS - One codepoint corresponding to all the subgroups that can be indicated by LP-WUS - FFS: Additional codepoints Agreement [RAN1#120] For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - Down-select between 2 and 4 for max number of POs per LO. Agreement [RAN1#120] Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32. Agreement [RAN1#120b] From specification design perspective, for both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints for a LP-WUS.

9.6.2 Agreement [RAN1#120b] The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. Agreement [RAN1#120b] For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported. - For codepoint corresponding to more than one subgroups: - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO Agreement [RAN1#120b] For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO Agreement Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO

As per the agreement, the number of POs per LO can be 1,2 or 4. If multiple POs are associated with the same LO, it means different subgroups in different POs need to monitor the same LO. And we have the agreement that “no additional constraint for mapping multiple POs to one LO”. In our understanding, even though one LO has multiple MOs, it won’t consider the mapping rule between POs and MOs in case of multiple POs associated with one LO. In this case, it needs to determine how to wake-up the subgroup within different POs.

- Based on the agreement, no further mapping rule between POs and MOs in case of multiple POs associated with one LO.

- In case of multiple POs associated with one LO, it needs to determine how to wake up subgroups in different POs by LP-WUS.

In case of multiple POs associated with one LO, e.g. the number of POs per LO is 4, and the number of subgroups per PO is 7. For the wake-up indication, 3 bits codepoint is enough for one PO. If codepoint values 000~111 are configured for the subgroups within one PO, e.g. 000~110 corresponding to each of the subgroups in each PO, and 111 corresponding to all the subgroups within one PO. When gNB needs to wake up the subgroup configured with codepoint ‘010’ in PO1, it would transmit LP-WUS signal carrying the codepoint ‘010’. However, there is one subgroup configured with codepoint ‘010’ in each PO. Thus, 3 subgroups in other POs would be woke-up unnecessarily, it will decrease the power saving gain in IDLE/INACTIVE mode.

Figure 1: LP-WUS indication in case of multiple POs associated with one LO

- In case of multiple POs associated with one LO, if one codepoint value is configured for the subgroups in different POs, it will decrease the power saving gain.

- For the power saving gain, avoid the case that non-target subgroup(s) are woke-up by LP-WUS in case of multiple POs associated with one LO.

In the design of PEI, one PEI-O could also be associated with multiple POs, for the indication of multiple POs, it introduced multiple fields for PEI. Each field is associated with one PO. But for LP-WUS, based on the agreement that “support up to 5 information bits for LP-WUS”, it is impossible to support the similar design as PEI.

In order to avoid the case that non-target subgroup(s) are woke-up by LP-WUS, UEs in different POs should be aware of which PO is the target PO for the LP-WUS, i.e., LP-WUS should carry the indication of POs.

- Support the PO indication in LP-WUS.

One simple way is to introduce one PO indication field in LP-WUS as following.

If the number of POs per LO is larger than 1, 1 or 2 bits of PO indication field can be used to indicate the target PO.

- The number of POs per LO is 1

  - The maximum number of subgroups per PO is 31.

  - 0 bit for PO indication field, up to 5 bits for the codepoint indication.

- The number of POs per LO is 2

  - The maximum number of subgroups per PO is 15.

  - 1 bit for PO indication field, up to 4 bits for the codepoint indication.

- The number of POs per LO is 4

  - The maximum number of subgroups per PO is 7.

  - 2 bits for PO indication field, up to 3 bits for the codepoint indication.

From UE perspective, when UE monitors the LP-WUS, it could determine the target PO based on the PO indication field. If the target PO is not the PO which UE belongs to, UE can stop the LP-WUS monitoring for the early termination.

- Introducing PO indication field won’t change the maximum number of LP-WUS information bits.

- Introducing PO indication field could achieve the early termination for non-target PO’s UE.

If no PO indication field, it needs to increase the number of bits for codepoint to avoid the case that non-target subgroup(s) are woke-up by LP-WUS as following.

- The number of POs per LO is 1

  - The maximum number of subgroups per PO is 31.

  - up to 5 bits for the codepoint indication.

- The number of POs per LO is 2, if the number of subgroups per PO is 15.

  - The number of bits for codepoint associated with one subgroup needs to be 5.

- The number of POs per LO is 4, if the maximum number of subgroups per PO is 7.

  - The number of bits for codepoint associated with one subgroup needs to be 5.

- If no PO indication field, it needs to increase the bit number for codepoint. The total bits for LP-WUS is same.

- Support PO indication field in LP-WUS, i.e. LP-WUS consist of 2 fields, one is used to indicate the associated PO, the other one is used to indicate the codepoint.

Wake-up indication in CONNECTED mode

During the RAN1#120 meeting, working assumption finally made for the codepoint scheme with some conditions.

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8

It is essentially similar mechanism as that for idle mode. The mechanism for CONNECTED mode may have higher number of detecting sequences/codepoints. This can allow flexible usage of codepoints. UE can be configured with up to 8 codepoints.

Further on the number of codepoint capability, there should be some lower bound of codepoint. To allow better scheduling coordination, we suggest to allow {4, 8} to be reported by UE capability. Comparing with the idle mode, 4 as minimum is acceptable, as the CONNECTED mode should be in more active state.

gNB can configure UE with codepoints, which do not exceed the capability. That can be supported by RRC configured codepoints mapping specific to each UE. A UE in CONNECTED mode should be UE-specifically configured with the LP-WUS resources and codepoints. And, one codepoint can be mapped to multiple UEs.

We suggest to confirm the working assumption and further agree the reporting of number of codepoints.

- A UE can report supported number of detecting codepoints for monitoring wake-up indication in CONNECTED mode, from 2 candidates {4, 8}.

LP-SS signal

Information for determining the LP-SS sequence

We had agreements in RAN1 #117 for the determination of sequences on LP-SS.

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

Regarding the sequence selection, it was agreed to support Option 1 for binary LP-SS sequences for the ‘ON-OFF’ pattern.

Further, we can consider simple way to configure the sequence. The binary sequence can be associated with the cell ID for reducing neighboring cell interference and identification. The PCI have larger number of IDs than the current agreed number (up to 16) of LP-SS sequences. A mapping schemes can be introduced between cell ID and LP-SS sequences.

- LP-SS sequence is associated to the cell ID by pre-determined rules.

LP-SS overlaid sequences

For LP-SS, the overlaid OFDM sequence does not need to carry additional information, it is used to flatten the spectrum and help OFDM based receiver to do the sync and measurement. Thus, single overlaid sequence is enough for the LP-SS transmission.

- For LP-SS.

- Single overlaid sequence is on each OOK ‘ON’ symbol.

Regarding the overlaid OFDM sequence(s) for LP-SS. When M=2 or 4, LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. When M=1, whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

In case of M=2/4 or M=1 with using a specified overlaid OFDM sequence, it needs to determine how to reuse the overlaid OFDM sequence(s) specified for LP-WUS.

- When LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS.

- It needs to determine how to reuse the overlaid OFDM sequence(s) specified for LP-WUS, e.g. the overlaid OFDM sequence for LP-SS is associated with the candidate overlaid OFDM sequence.

LP-SS in time and frequency

As the LP-SS is for assisting LP-WUS, the closest or same frequency location should be assumed. And the same occupation of bandwidth should be used. For LP-WUS signal bandwidth 11RBs in 30KHz SCS as example, ideally the LP-SS should be with same SCS and number of RBs.  Further as also been provided by LP-WUS, same occupied bandwidth would help for reducing LR receiver complexity.

Figure 5. TDM based multiplexing for LP-WUS and LP-SS.

The wake-up schemes support different receivers. The overlaid sequences of WUS would depend on the measurement of PSS/SSS. Thus, the LP-WUS may be configured with relative location in frequency with SSB. If the LP-WUS and LP-SS share the same frequency location, SSB location should be associated with LP-SS.

RAN1 is to further discuss the frequency gap between LP-WUS/LP-SS and SSB. It can be set by frequency starting points. Considering the maximum single carrier, it can be set to up to 100MHz for FR1.

Figure 6. Time and frequency location between SSB and LP-WUS/SS

In time domain, the LP-SS should be periodically transmitted. The LP-SS transmission should consider the multi-beam operations. A set of LP-SSs can be transmitted in a period. Each LP-SS may be associated with a beam.

- LP-WUS and LP-SS share the same frequency location, SSB location should determine LP-WUS/LP-SS location with an offset.

  - Frequency gap between starting of LP-WUS/LP-SS and starting of SSB can be set to up to 100MHz for FR1.

M values configuration for LP-WUS and LP-SS

Following agreement was reached in last meeting.

Agreement For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS) - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both

If M>1 is configured for LP-WUS, LP-SS could be configured with the same or larger M value. In case of M=1 for LP-WUS, the power of OOK-ON symbol depends on the EPRE configuration. Then it is not necessary to limit the M value configuration for LP-SS.

- No to consider the restriction on the M value configuration in case of M=1.

Conclusion

In this contribution, we discussed the signal design for LP-WUS and LP-SS.  Observations and proposals are summarized as following.

- Confirm the working assumption. Support M=4 for 30KHz SCS.

- Confirm the Working Assumption with following update.

Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for the transmission of LP-WUS and LP-SS each M value). - Different roots can be picked for different M values.

- Proposal 3: RAN1 to clarify the numbers of candidates overlaid sequences supported other than the maximum value.

- RAN1 can confirm the working assumption on the supported information bits.

- Coded bits length after rate matching depends on the resource allocation, if we limit the coded bits length, it will limit the time domain resource allocation for LP-WUS.

- Based on the agreement, no further mapping rule between POs and MOs in case of multiple POs associated with one LO.

- No restriction on the coded bits length. Exactly code block length depends on the resource allocation

- In case of multiple POs associated with one LO, it needs to determine how to wake up subgroups in different POs by LP-WUS.

- In case of multiple POs associated with one LO, if one codepoint value is configured for the subgroups in different POs, it will decrease the power saving gain.

- For the power saving gain, avoid the case that non-target subgroup(s) are woke-up by LP-WUS in case of multiple POs associated with one LO.

- Support the PO indication in LP-WUS.

- Introducing PO indication field won’t change the maximum number of LP-WUS information bits.

- Introducing PO indication field could achieve the early termination for non-target PO’s UE.

- If no PO indication field, it needs to increase the bit number for codepoint. The total bits for LP-WUS is same.

- Support PO indication field in LP-WUS, i.e. LP-WUS consist of 2 fields, one is used to indicate the associated PO, the other one is used to indicate the codepoint.

- A UE can report supported number of detecting codepoints for monitoring wake-up indication in CONNECTED mode, from 2 candidates {4, 8}.

- LP-SS sequence is associated to the cell ID by pre-determined rules.

- For LP-SS.

  - Single overlaid sequence is on each OOK ‘ON’ symbol.

- When LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS.

- It needs to determine how to reuse the overlaid OFDM sequence(s) specified for LP-WUS, e.g. the overlaid OFDM sequence for LP-SS is associated with the candidate overlaid OFDM sequence.

- LP-WUS and LP-SS share the same frequency location, SSB location should determine LP-WUS/LP-SS location with an offset.

  - Frequency gap between starting of LP-WUS/LP-SS and starting of SSB can be set to up to 100MHz for FR1.

- No to consider the restriction on the M value configuration in case of M=1.

Reference

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR.