3GPP TSG RAN WG1 #121		                                      R1-2504231

St Julian’s, Malta, May 19th–23rd, 2025

Source:	OPPO

Title:	Enhancements on asymmetric DL STRP/UL MTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item are listed below

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Scenarios and assumptions

The deployment of heterogeneous networks (HetNets) is a strategy to improve user experience by combining macro cells with smaller cells (e.g., micro, pico, and femtocells). This setup caters to the differing needs of users and helps improve overall network coverage and capacity, particularly for uplink (UL) throughput.

This scenario provides a strategic advantage in network topology, catering to dense urban environments where uplink data traffic is significantly high. By offloading the uplink communication to dedicated UL TRPs, NW can efficiently manage the increased uplink traffic without overburdening the regular TRP, which manages both uplink and downlink. Such a deployment scenario is instrumental in improving uplink throughput and enhances network capacity and coverage.

The basic scenario of asymmetric deployment is illustrated in Figure 1. UE is engaged in downlink communication with a regular TRP capable of both downlink and uplink transmission. However, for UL communication, the UE targets a separate TRP, designated as an UL TRP.

- : the asymmetric deployment scenario in which a UE communicates with one single TRP for DL and with one or more TRPs for UL

In the WID [1], the assumptions for the enhancement in this agenda item were explicitly listed in what follows. From our point of view, those assumptions underlying this deployment are constrained to ensure focus and manageability in standardization efforts.

  - DL sTRP/UL mTRP deployment

  - Intra-band & intra DU & non-co-located mTRP

  - No definition change on cell

  - Reuse Rel-17/18 unified TCI framework

  - Applicable to both FR1 and FR2

# Remaining issues

For the asymmetric deployment scenario, we believe most of essential issues have been addressed until RAN1#120bis. In this section, we present remaining issue(s) for certain scenario.

For UE configured with Rel-19 2 TAGs, UE needs to send PDCCH-order PRACH to one particular TRP to obtain its corresponding TA value. But if the UE is not configured with PL offset in joint/UL TCI state(s), it is not clear to UE which PL RS to be applied for PDCCH-order PRACH. A few of options are still available, e.g. obtaining PL RS from the DMRS of PDCCH-order or from the SSB indicated in the PDCCH order.

To address this issue, we may borrow the solution (by reusing the field of PRACH association indicator in PDCCH-order) from Rel-18 2 TA enhancement for M-DCI MTRP. The only difference lies in the fact that in Rel-19 2 TA, the restriction of M-DCI MTRP, i.e. the RRC parameter CORESETPoolIndex can be removed.

- For UE configured with Rel-19 2 TAGs, when the UE is not configured with PL offset in joint/UL TCI state(s), support to reuse the DCI field ‘PRACH association indicator’ to indicate PL RS for PDCCH-order PRACH.

# Conclusion

In this section, allow us to repeat our proposals and observations.

- For UE configured with Rel-19 2 TAGs, when the UE is not configured with PL offset in joint/UL TCI state(s), support to reuse the DCI field ‘PRACH association indicator’ to indicate PL RS for PDCCH-order PRACH.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024