3GPP TSG RAN WG1 #121	R1-2504445

St Julian’s, Malta, May 19th – 23th, 2025

Source:	Sharp

Title:	CSI enhancements

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Background

In RAN#102 and #105, the working item to enhance NR MIMO operations in Rel-19 was agreed and revised, respectively [1,2].

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

Left-over issues

Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for a given P∈{48, 64, 128} CSI-RS ports, the mapping of i1,3 to (k1,k2) for RI=2-4 is the same as that used in the Rel-19 Type-I SP Scheme-A for RI=2-4 and N1>N2>1.. - Note: How this is captured is up to the TS38.214 editor. Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, support only the following (Ng, N1, N2) values (revision from the current version of editor draft CR on TS 38.214): Number of CSI-RS antenna ports, PCSI-RS; (Ng,N1,N2). 48; (2,4,3). (2,6,2). (2,12,1). 64; (2,4,4). (2,8,2). (2,16,1). (4,4,2). (4,8,1). 128; (4,4,4). (4,8,2). (4,16,1). Conclusion For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when the Rel-18 SD NES Type-I is configured for the Rel-19 Type-I SP codebook, the powerOffset parameter can be configured in all the respective subConfiguration IEs - The supported values for powerOffset follow the legacy specification Conclusion There is no RAN1 consensus on the following proposal: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, also support AD = {0.1CP, 0.2CP}. - Each of the above values of the dynamic ranges is subject to a separate UE capability. - Each of the above values of the dynamic ranges is applicable when the bandwidth of configured TRS is no smaller than 52 PRBs Conclusion For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, there is no need to support any mapping from CSI-RS resource index and CSI-RS resource port index per resource to port index for CSI/PMI calculation Conclusion There is no consensus to support the following proposal: - For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, support configuring, via higher-layer (RRC) signalling, an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler

  - Joint/separate report of Rel-18 eType-II CJT CSI and CJTC

In the previous meeting, the following proposal was raised:

Proposal 3.E.2: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports, and if the UE performs delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, the UE can assume that δi of the reported DO interval δi, δi+1 is used for the DO compensation.

The definition of the compensated DO value is simple and helpful to avoid the misalignment between NW and UE, even though this impact on spec is very small. For 30kHz SCS, the interval of the reported DO is AD/(MD-1) = 75.5 ns, when AD = 2.34 μs and MD = 32. As a reference, this is larger than 65 ns that is the maximum BS MIMO TAE. We think the misalignment in the exact DO value between NW and UE is not negligible.

If the 1-bit indicator indicates to perform the DO compensation, the delay offset value to be compensated is the reported DO. However, since the reported DO is expressed as the interval of δi, δi+1, the DO to be compensated is unclear.

We think there are two understanding about the detail on the DO compensation.

- In the first understanding, the UE reports Rel-18 Type-II CJT CSI including the phase rotation of the reported DO. For the CJT transmission, the NW can use only one precoder matrix corresponding to the reported Rel-18 Type-II CJT CSI that includes the phase rotation of the reported DO. However, in this case, the phase rotation of the reported DO is further quantized due to CJT CSI reporting in addition to the quantization of the DO reporting. That causes performance loss.

- In the second understanding, the UE reports Rel-18 Type-II CJT CSI separately from the phase rotation of the reported DO. For the CJT transmission, the NW can use two precoder matrices corresponding to the reported Rel-18 Type-II CJT CSI and the phase rotation of the reported DO, respectively. In this case, the phase rotation of the reported DO is not quantized by the reported Rel-18 Type-II CJT CSI, and is calculated at both UE side and NW side.

If the DO compensation is up to UE implementation, these two understandings remain. In this case, the NW cannot identify whether to use only the reported Rel-18 Type-II CJT CSI or both reported Rel-18 Type-II CJT CSI and phase rotation of the reported DO.

In our view, the second understanding is technically reasonable. Furthermore, this may be an issue for smaller MD value, i.e., misalignment in the exact DO value between NW and UE can cause significant performance loss. Therefore, we support the proposal to define the DO compensation value.

Proposal 1: When linking CJTC and CJT PMI reports is configured, the lower limit value δi of the reported DO δi, δi+1 should be used for the DO compensation.

Additionally, in the current spec, for CQI calculation, the UE assumes PDSCH signals as the following equation:

##### 5.2.2.5.1b UE assumptions for CQI/PMI/RI calculation for CJT If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', the higher layer parameter codebookType is set to 'typeII-CJT-r18' or ' typeII-CJT-PortSelection-r18', and the corresponding CSI-RS Resource Set for channel measurement is configured with 1≤NTRP≤4 CSI-RS resources, for CQI calculation - a UE should assume PDSCH signals on antenna ports in the set [1000,…,1000+υ-1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N0 selected CSI-RS resources, as given by yσ13000(i)⋮yσ13000+P-1(i)yσ23000(i)⋮yσ23000+P-1(i)⋮yσN03000(i)⋮yσN03000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.8 and 5.2.2.2.9 for codebookType set to 'typeII-CJT-r18' and ' typeII-CJT-PortSelection-r18', respectively, and {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP. A UE should assume that the signals yσj, j=1,…,N0, fully overlap in time and frequency. - a UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources σj, with j=1,…,N0, equal to N0NTRP times the powerControlOffset (in linear scale) of the respective CSI-RS resource.

However, if the DO compensation is up to UE implementation, the above two understandings remain, and the NW cannot identify whether to use only the reported Rel-18 Type-II CJT CSI or both reported Rel-18 Type-II CJT CSI and phase rotation of the reported DO.

The following equation is referred from ZTE’s contribution [3]:

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

We support the above equation to clarify that the reported Rel-18 Type-II CJT CSI is independent from phase rotation of the reported/compensated DO. Additionally, the compensated DO value should be the lower limit of the reported DO interval as τσn, where n=1,2,…,NTRP.

Proposal 2: When linking CJTC and CJT PMI reports is configured, and the reported DO value is compensated, the UE should assume the following PDSCH signal to calculate CQI [3]:

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

We summarize our view of this section as the following:

- UE needs to calculate a CQI by assuming PDSCH signal for CJT.

- In the current spec, the assumed PDSCH signal for CJT is composed of a precoding matrix and signals.

- However, if UE performs DO compensation, the assumed PDSCH signal for CJT is unclear.

- We support that the assumed PDSCH signal for CJT is composed of a precoding matrix, signals, and the phase rotation of the compensated DO.

- However, the compensated DO is unclear.

- We support that the compensated DO is the lower limit value of the reported DO [δi, δi+1).

Conclusion

In this contribution, we have the following proposal:

Proposal 1: When linking CJTC and CJT PMI reports is configured, the lower limit value δi of the reported DO δi, δi+1 should be used for the DO compensation.

Proposal 2: When linking CJTC and CJT PMI reports is configured, and the reported DO value is compensated, the UE should assume the following PDSCH signal to calculate CQI [3]:

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

References

- [1]	RP-234007, New WID: NR MIMO Phase 5, Dec., 2023.

- [2]	RP-242394, WID revision: NR MIMO Phase 5, Sep., 2024.

- [3]	R1-2500242, ZTE, Discussion on CSI enhancements, Feb., 2025.