3GPP TSG-RAN WG1 Meeting #121	R1-2504451

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Remaining issues on CSI enhancements for large antenna arrays and CJT

Document for:	Discussion, Decision

# 1	Introduction

In the work item description (WID) for NR MIMO Phase 5 we have the following objectives related to CSI enhancement and CJT.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this paper, we provide Ericsson’s views on the maintenance issues related to the above objectives.

# 2	Discussion

## 2.1 Extension of Rel-19 Type I codebooks to number of ports ≤32

During pre-RAN1#119 offline email discussion [3], it was discussed whether to extend Rel-19 Type I codebooks introduced for 48, 64, and 128 ports to smaller of ports as well.  We provide some simulation results to compare the performances between Rel-15 and Rel-19 Type I codebooks for 32 ports.  The results are provided in Table 1 for ranks 1-4.

As can be seen in the results, Rel-19 Type I Scheme A (no array splitting) shows around 3% (4%) gain at 50% RU and around 7% (8%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput).  Rel-19 Type I Scheme B shows around 7% (12%) gain at 50% RU and around 16% (24%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput). ​

Our results suggest that Rel-19 Type I Scheme B provides larger gains than Rel-19 Type I Scheme A.  Based on the results, we support extension of Rel-19 Type I codebooks to number of ports smaller or equal to 32.  To avoid introduction of too many UE capabilities, our preference is to introduce this extension to only 16, 24, and 32 ports.

- Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

Table 1: Comparison between Rel-15 and Rel-19 Type I codebooks for 32 ports for ranks 1-4

## 2.2 Need tor new RRC parameter for Rank 2 soft scaling

In [1], the following proposal was discussed regarding the need for introducing a new RRC parameter for enabling the per-layer scaling factor applied to SD-basis vectors in Rel-19 Type-I SP codebook:

“For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), introduce a new RRC parameter (e.g. enable-typeI-softScalingRank2-r19 with value {enabled}) to enable the per-layer scaling factor for RI=v=2.”

As discussed in [1], there was an earlier agreement in RAN1#118bis to introduce such RRC parameter.  However, in RAN1#119, it was agreed that the 3-bit scaling factor per SD-basis vector group is RI-common which means that the same 3-bit scaling factor configuration is applied to both ranks 1 and 2.  In Table 2, we show evaluation results that indicate that the UE only applying the scaling factor to rank 1 does not yield any performance improvement over a network implementation based baseline.  Hence, if the UE supports the capability for applying SD-basis vector group specific scaling factor for ranks 1-2, there is no motivation for the gNB to disable the application SD-basis vector group specific scaling factor to rank 2.  Hence, the introduction of the new RRC parameter for enabling the application of the SD-basis vector group specific scaling factor is not needed for rank 2.  We propose the following:

- Proposal 2:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), a new RRC parameter to enable the application of the scaling factor for RI=v=2 is not introduced.

Table 2: Throughput comparison

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | Mean user TP gain (%) | 5th percentile TP gain (%) | Mean user TP gain (%) | 5th percentile TP gain (%) |
| Beam specific backoff at the network (via network implementation) – ranks 1-4 | Baseline | Baseline | Baseline | Baseline |
| Beam-group specific scaling factor aware PMI/CQI selection at the UE for rank 1 only, no power scaling for ranks 2-4 | -3 | -6 | -10 | -10 |
| Beam-group specific scaling factor aware PMI/CQI selection at the UE, rank restricted to ranks 1-2 only | 13 | 61 | 14 | 60 |

# 3. Conclusion

- Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

- Proposal 2:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), a new RRC parameter to enable the application of the scaling factor for RI=v=2 is not introduced.

# 4. References

R1-2502357, Moderator Summary#1 on Rel-19 CSI enhancements: Round 1, RAN1#120bis, Wuhan, China, April 7-11, 2025.

# 5. Appendix

Table 3 – Evaluation assumptions for results presented in Table 2

| Parameter | Value |
| --- | --- |
| Scenarios | Uma with 500m ISD Mobility model: Random UE direction. UE speed: 3 km/h |
| Frequency Range | 6.5 GHz |
| Channel generation model | TR 38.901. No spatial consistency. No vehicles penetration loss modeled. |
| Antenna setup and port layouts at gNB | 128 ports: (16,16,2,1,1,4,16) (dV,dH) = (0.8,0.5)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,2,2), (dV, dH) = (0.5, 0.5)λ |
| CSI-RS configuration | 128 ports: 4 resources x 32 port CSI-RS |
| CSI feedback | Periodic CSI feedback: 10 slot, 4 slot delay |
| BS Tx power | Uma: 41 dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 7 dB |
| Modulation | Up to 256QAM |
| SCS | 30 kHz |
| Number of RBs | 52 for 30 kHz SCS |
| Simulation bandwidth | 20 MHz DL |
| Sub-band size | 4 PRBs |
| Overhead | CSI-RS, DMRS overhead included in all results |
| Traffic model | FTP model 1, 5 drops, with packet size 0.5 Mbytes |
| UE distribution | 20% outdoor, 80% indoor |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |