3GPP TSG RAN WG1 #121	R1-2504546

St Julian’s, Malta, May 19th – 23th, 2025

Source:	Sharp

Title:	Discussion on measurements related enhancements for LTM

Agenda item:	9.9.1

Document for: 	Discussion

Introduction

In RAN1#120bis meeting [1], the following agreements and conclusions were made as follows:

Agreement - Repetition=off is supported for candidate cell CSI-RS in Rel-19. - Repetition=on is not supported for candidate cell CSI-RS in Rel-19. Conclusion RAN1 assumes the SSB QCLed with CSI-RS is used for timing reference of candidate cell CSI-RS measurement in LTM. Agreement In the resource set configured for candidate cell L1-RSRP measurement, a sequence of candidate cell indices is included together with a sequence of CSI-RS resource indices, where each CSI-RS resource index is associated with one candidate cell index. Agreement Regarding the LS from RAN2 (R1-2501686) the following contents are included in the reply LS. - RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting - Regardless whether or not the report includes the current beam - Note: The maximum number of beams that can be reported by a UE subject to UE capability is under discussion in RAN1 Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Agreement Reply LS on number of beam measurements in the measurement report MAC CE is agreed. RAN1 would like to thank RAN2 for their LS on number of beam measurements in the measurement report MAC CE. RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting regardless whether or not the report includes the current beam. It is noted that the maximum number of beams that can be reported by a UE is subject to UE capability which is under discussion in RAN1. Agreement LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells is agreed. Final LS in R1-2503079. Agreement For the RS type determination for event triggered reporting with event LTM2, - Alt 1-2: At least one candidate RS shall be configured Agreement For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available - FFS: the definition of valid CSI Agreement For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available - Option 1-1-1: Codepoint with the lowest CQI index is reported Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability

In this contribution, we provide our views on L1 measurement enhancements for the purpose of supporting LTM.

Discussion

Support for CSI acquisition for candidate cell

Subset of CSI-RS resources for CSI measurement before CSC

In last FL summary, a proposal related to the subset selection of the measurement CSI-RS resource(s) from the configured CSI-RS resources before CSC MAC CE was made, and whether and how to introduce a mechanism to indicate subset of CSI-RS resources for CSI measurement is to be discussed. For whether the subset selection is needed, implementing a strategy that allows the UE to measure only a subset of the CSI-RS can help ensure efficient use of resources due to the potential for excessive overhead on the UE caused by an abundance of CSI-RS. Thus, to effectively manage the overhead caused by CSI measurement before the reception of the CSC MAC CE, we propose the following methods. First, employing MAC CE allows for the dynamic activation and deactivation of specific subsets of CSI-RS resources, meaning the UE will only measure those that are currently activated, thus enhancing efficiency and reducing overhead. Furthermore, the subset indicated in a new MAC CE is preferred since the timing of performing subset selection and the timing of activating/deactivating TCI states may be different. Second, incorporating a field within the DCI can explicitly indicate which CSI-RS resources need to be measured, providing the UE with precise guidance. Thus, using a DCI field to indicate a subset of CSI-RS resources can also be considered.

Support implementing a strategy that allows the UE to measure only a subset of CSI-RS resources.

Consider one of the following methods to select a subset of CSI-RS resources

- A new MAC CE to activate/deactivate a subset of CSI-RS resources

- A DCI field to indicate a subset of CSI-RS resources

For the CSI report sent to the target cell, the use of UCI for CSI reporting was agreed. However, the timing of the CSI report has not been determined yet. To acquire appropriate coding scheme scheduling for the target cell immediately, carrying the CSI report on the first UL transmission for RACH-less procedure and PUSCH scheduled by RAR UL grant for RACH-based procedure should be supported.

CSI report for CSI acquisition is carried on the first UL transmission for RACH-less procedure.

CSI report for CSI acquisition is carried on the PUSCH scheduled by RAR UL grant for RACH-based procedure.

Conditional intra-CU LTM

How to determine subcarrier spacing of timing advance for CLTM

With the introduction of the LTM Candidate Timing Advance Command MAC CE for CLTM, its timing advance command field is used to indicate an index value TA. This TA is associated with the time alignment amount NTA, where NTA=TA∙16∙642μ.

To determine NTA for conditional LTM, the value TA will be indicated by the timing advance command field in the LTM Candidate Timing Advance Command MAC CE. However, how to determine the subcarrier spacing (i.e., μ) for NTA is not determined yet.

According to current TS38.213 [2] specification, NTA is defined as follows: “A timing advance command  in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTAis relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command.”

From our perspective, this method － where NTA is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command － cannot be directly applied to the case of the LTM Candidate Timing Advance Command MAC CE. In that case, it would imply: “NTA is relative to the SCS of the first uplink transmission from the UE after the reception of the LTM Candidate Timing Advance Command MAC CE”. However, under CLTM, the UE may not apply the TA in the LTM Candidate Timing Advance Command MAC CE immediately after receiving it. Instead, the UE may store the TAvalue and only determine NTA after it decides which candidate cell to switch to. Therefore, the existing method for determining the SCS for NTA－based on the first uplink transmission following the LTM Candidate Timing Advance Command MAC CE － cannot be reused.

As a result, a new method for determining the subcarrier spacing for NTA may require further discussion. For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

The legacy method － where NTA is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command － cannot be directly applied to the case that the timing advance command is received in the LTM Candidate Timing Advance Command MAC CE.

How to determine the subcarrier spacing for an amount of time alignment, NTA=TA∙16∙642μ, may require further discussion in the case that the timing advance command is received in  the LTM Candidate Timing Advance Command MAC CE for conditional LTM.

  - For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

Conclusions

In this contribution, we discussed the issues regarding enhancement of measurements to support LTM which includes CSI acquisition for candidate cell and Conditional intra-CU LTM. And we have proposals as follows.

- Support implementing a strategy that allows the UE to measure only a subset of CSI-RS resources.

Consider one of the following methods to select a subset of CSI-RS resources

- A new MAC CE to activate/deactivate a subset of CSI-RS resources

- A DCI field to indicate a subset of CSI-RS resources

CSI report for CSI acquisition is carried on the first UL transmission for RACH-less procedure.

CSI report for CSI acquisition is carried on the PUSCH scheduled by RAR UL grant for RACH-based procedure.

The legacy method － where NTA is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command － cannot be directly applied to the case that the timing advance command is received in the LTM Candidate Timing Advance Command MAC CE.

How to determine the subcarrier spacing for an amount of time alignment, NTA=TA∙16∙642μ, may require further discussion in the case that the timing advance command is received in  the LTM Candidate Timing Advance Command MAC CE for conditional LTM .

  - For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

References

Chair notes RAN1#120bis

5G; NR; Physical layer procedures for control (3GPP TS 38.213 version 18.6.0 Release 18).