3GPP TSG RAN WG1 #121												  R1-2504792

St Julian's, Malta, May 19th – 23rd, 2025

Agenda item:	7

Source:	Moderator (NEC)

Title:              	Summary on starting slot for typeII Doppler CSI

Document for:	Discussion and Decision

- Introduction

In RAN1#121, the following discussion paper and draft CR were submitted for starting slot for typeII Doppler CSI.

R1-2503919	Discussion on starting slot for typeII Doppler CSI	NEC

R1-2503920	Draft CR on starting slot for typeII Doppler CSI in TS38.214	NEC

This contribution collects companies’ views on the issues and proposals discussed in the above Tdocs.

Section 2 is for detail of the issue. Section 3 is for collection companies’ views.

- Discussion

For Rel-18 Type-II codebook refinement for high/medium velocities, the prediction delay δ was agreed to determine the earliest slot l for the predicted CSI.

Agreement@RAN1#110bis On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after slot l where the location of slot l is configured (from multiple candidate values) by gNB via higher-layer signalling - Candidates of slot l location include the legacy CSI reference resource location (n – nCSI,ref ) and slot (n+δ) where δ ≥ 0 - FFS: Possible value(s) of δ and possible value(s) of WCSI Note: Per legacy behavior, the legacy CSI reference resource, i.e., (n – nCSI,ref ), is reused for locating the last CSI-RS occasion used for a CSI report For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional.

Agreement@RAN1#110bis On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, study the supported value(s) for δ and WCSI from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units: - δ (slots): {0, 1, 2, 3, 4, 6, 8}, or a subset thereof with at least two values including 0, or a single fixed value (e.g. 0 or 1) - WCSI (slots): 1, N4, following periodicity of P/SP-CSI-RS or SP-CSI (e.g., 4, 5, 8, 10, 16, 20, 40), dN4 (d=DD unit size in slots, N4 is unit-less) FFS: Dependence on sub-carrier spacing should also be studied

Agreement@RAN1#111 For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values: - First candidate value: δ=0, - 2 additional non-zero values of parameter δ are supported - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8

In the current TS 38.214, the earliest of the N4 slot intervals for typeII doppler CSI is defined as slot l=n+δ, and n is the uplink slot in which the CSI is reported. But there are several issues unclear:

- In case of δ=-nCSI_ref, it’s not suitable to directly add slot n (defined as uplink slot) with δ=-nCSI_ref, as the subcarrier spacing can be different for uplink and downlink

- For the predictionDelay δ, there is no definition of the corresponding subcarrier spacing.

- For slot l=n+δ, it’s unclear to be downlink slot or uplink slot.

The issue also exists for CSI reference slot, at that time, we defined the slot considering the uplink slot n', the downlink subcarrier spcaing, and the uplink subcarrier spacing. The reference TS part as follows:

5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. <Irrelavant part omitted>

In the draft CR [2], NEC proposed to reuse the formula for CSI reference slot, and capture the following.

Reason for change:; In current TS, the earliest of the N4 slot intervals for typeII doppler CSI is defined as slot l=n+δ, and n is the uplink slot in which the CSI is reported. There are several issues unclear, also discussed in our companion contribution [1]: - No clear definition of the starting slot l as uplink slot or downlink slot - No clear definition of subcarrier spacing for the predictiondelay δ - At lesat when δ=-nCSI_ref, it’s clearly defined nCSI_ref corresponding to downlink subcarrier spacing in section of CSI reference resource, then it’s not suitable to directly add slot n (defined as uplink slot) with δ=-nCSI_ref (which corresponds to downlink). Summary of change:; - Clarifiying the definition of starting slot l corresponding to downlink slot - Reusing the definition of the slot n for CSI reference resource, as +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL - Clarifying the predictiondelay δ corresponding to subcarrier spacing configuration for downlink. Consequences if not approved:; - Ambiguous on UE to determine the starting slot l for typeII doppler CSI.

##### 5.2.1.4.2 Report quantity configurations <Unchanged part omitted> Subject to UE capability, a UE configured with a CodebookConfig with the higher layer parameter vectorLengthDD and a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter vectorLengthDD. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter unitDurationDD, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and μDL and μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and μoffset are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211], and where n' n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay and corresponds to subcarrier spacing configuration for DL, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability. <Unchanged part omitted>

- Collection of companies’ view

Q1: Do you agree on the issue and changes with reusing formula for CSI reference slot definition to doppler based CSI in the draft CR [2], if no, please provide your views or suggestions on the following issues, e.g.

- whether we need to clarify the starting slot l is downlink slot or uplink slot,

- if no change on the formula, do we need to clarify the subcarrier spacing for downlink and uplink should be same for doppler CSI

| Company | Y or N | Comments |
| --- | --- | --- |
| Moderator (NEC) | Y | We support to update the formula for doppler CSI, which can reuse similar formula from CSI reference slot, as subcarrier spacing for downlink and uplink can be possibly different. Otherwise, it’s not correct to add a slot index (n) for uplink and an offset (δ) for downlink. |
| OPPO | Y | We understand the issue and support to update the formula for Doppler CSI. Whether downlink SCS or uplink SCS is applied can be further studied. Restriction like the SCS for downlink and uplink should be same for doppler CSI is not a good way to go. [Mod]: Thank you. |
| ZTE | Y | We think this issue is valid. The suggested formula is fine to us. [Mod]: Thank you. |
| Ericsson | Y | Ok. But the current changes make the sentence very long and quite difficult to parse and read. May be better to break the introduced changes in to a separate sentence. A suggested change is given below: earliest of the N4 slot intervals starts at slot l=n+δ, where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL, where n' n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay and corresponds to subcarrier spacing configuration for DL, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability. In the above, μDL and μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and μoffset are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]. [Mod]: Thank you. We think this version is OK. Let’s check whether other companies are also OK. |
| Apple | N | We fail to see the issue considering that deployment is the same SCS between DL and UL. But we are okay with the CR if we are minority. The new spec is just more complicated to read. [Mod]: Thanks for being flexible. As spec doesn’t exclude different SCS case, update by reusing the formula for CSI reference slot is a better way, which can make the spec complete and correct. |
| Mod |  | @All. Thanks for the input. Let’s further check whether Ericsson’s update can be accepted. 5.2.1.4.2 Report quantity configurations <Unchanged part omitted> Subject to UE capability, a UE configured with a CodebookConfig with the higher layer parameter vectorLengthDD and a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter vectorLengthDD. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter unitDurationDD, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL , where n' n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay and corresponds to subcarrier spacing configuration for DL, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability. In the above, μDL and μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and μoffset are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]. <Unchanged part omitted>. |
| ETRI | Y | We are okay with the updated version. |

- Conclusion

Thank you very much for sharing the views and being flexible. According to companies’ input, it is proposed to adopt the TP with update from Ericsson.

Reason for change:; In current TS, the earliest of the N4 slot intervals for typeII doppler CSI is defined as slot l=n+δ, and n is the uplink slot in which the CSI is reported. There are several issues unclear, also discussed in our companion contribution [1]: - No clear definition of the starting slot l as uplink slot or downlink slot - No clear definition of subcarrier spacing for the predictiondelay δ - At lesat when δ=-nCSI_ref, it’s clearly defined nCSI_ref corresponding to downlink subcarrier spacing in section of CSI reference resource, then it’s not suitable to directly add slot n (defined as uplink slot) with δ=-nCSI_ref (which corresponds to downlink). Summary of change:; - Clarifiying the definition of starting slot l corresponding to downlink slot - Reusing the definition of the slot n for CSI reference resource, as +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL - Clarifying the predictiondelay δ corresponding to subcarrier spacing configuration for downlink. Consequences if not approved:; - Ambiguous on UE to determine the starting slot l for typeII doppler CSI.

5.2.1.4.2 Report quantity configurations <Unchanged part omitted> Subject to UE capability, a UE configured with a CodebookConfig with the higher layer parameter vectorLengthDD and a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter vectorLengthDD. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter unitDurationDD, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL , where n' n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay and corresponds to subcarrier spacing configuration for DL, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability. In the above, μDL and μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and μoffset are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]. <Unchanged part omitted>

- References

- R1-2503919	Discussion on starting slot for typeII Doppler CSI	NEC

- R1-2503920	Draft CR on starting slot for typeII Doppler CSI in TS38.214	NEC