3GPP TSG RAN WG1 #121			R1-2504802

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.7

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary#2 of discussion on Rel-19 NR UE features for XR Phase 3

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.15.7. regarding Rel-19 NR UE features for XR Phase 3.

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| Index | Companies | Proposals |
| --- | --- | --- |
|  | Huawei, HiSilicon | In RAN1#120, some companies proposed the UE FGs should be separated corresponding to different DCI formats. So far, we do not see strong motivation to have separate FGs. Meanwhile, a single FG seems sufficient to support of R19 XR for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3). The support of R19 XR feature for DCI format 0_2/1_2 and 0_3/1_3 are subject to the support of DCI format 0_2/1_2 and DCI format 0_3/1_3 themselves, respectively. Thus, we suggest to have a single FG for all the DCI formats. Proposal 1: A single FG is preferred for the support of R19 XR for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3). For the granularity of this FG, a UE may measure multiple neighbour cells and these cells may locate in different bands. Hence, the granularity of this FG should be per UE for simplicity. Proposal 2: Endorse updated UE feature list for R19 XR in Appendix. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. NR_XR_Ph3-Core; X-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 2. 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 3. 4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction FFS: other component (which may be separate FG); FFS N/A; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; Per UE; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. |
|  | ZTE Corporation, Sanechips | # Remaining issues on UE features on Rel-19 XR enhancements ## 2.1 Separation of X-1 per DCI format According to previous discussion on UE features for Rel-19 XR enhancement, we discussed three alternatives for separation of X-1 per DCI format(s) as follows [2]. For separation of X-1 per DCI format(s), companies are encouraged to provide their own preference/concern for the following alternative: - Alt-1: A FG (i.e., X-1) is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3) - Note: The support of XR feature for DCI format 0_2/1_2 and for DCI format 0_3/1_3 are subject to the support of DCI format 0_2/1_2 itself and DCI format 0_3/1_3 themselves, respectively - Alt-2: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1/0_2/1_2. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1 and X-2 - Alt-3: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_2/1_2. Another FG (e.g., X-3) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1, X-2 and X-3. In our opinion, we think Alt 1 is more reasonable. Considering one FG for all DCI signaling indication, it may not cause extra complexity for DCI format x_1, x_2, and x_3, respectively. The gNB knows the scheduling DCI UE supports, since UE can report UE capability of scheduling (e.g., cross carrier scheduling, multi-carrier scheduling, etc.). Based on that, gNB is capable of configuring different RRC signalings for different UEs. Proposal 1: Support Alt 1, i.e., a FG is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3). ## 2.2 Granularity of Rel-19 XR UE feature In our opinion, the granularity of Rel-19 XR UE feature should be ‘per UE’. If smaller granularity (e.g., per band) is considered, it may cause complexity on gNB indication to different UEs in different bands. To this end, we think granularity of ‘per UE’ is a suitable granularity for Rel-19 XR enhancement. Proposal 2: The granularity of Rel-19 XR UE feature is ‘per UE’. ## 2.2 Necessity of FR1/FR2 differentiation Since there exists per-FR type measurement gap in current specification, we think the column labeled as ‘Need of FR1/FR2 differentiation’ should be set to ‘YES’. Proposal 3: ‘Need of FR1/FR2 differentiation’ should be set to ‘YES’ We provide the modification for UE features on Rel-19 XR enhancements in summary in Appendix. |
|  | vivo | - Discussion To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements support, the following working assumption and agreements related with UE capability were made during RAN1 normative phase. Working Assumption RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. Note: UE features related to the developed solution(s) is a separate discussion. Agreement Confirm the working assumption from RAN1 #116 with updates: - RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. - It is up to RAN4 to discuss which type of gaps/restrictions caused by RRM measurements can be cancelled/skipped - Note: UE features related to the developed solution(s) is a separate discussion Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561. Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell. Agreement Clarify the note from previous agreement as follows: - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. Agreement A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells). Agreement Agreement from RAN1#119 is updated (red part) For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells). - Alt1: If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band (and have same SCS) can be co-scheduled. In other words, if co-scheduled cells could be inter-band, skipping indication field cannot be configured.. In RAN1#120 meeting, the following agreements were made for UE features for XR for NR Phase 3 [1]. Agreement: Introduce following basic FG for XR Phase 3, and FFS on some potential components (that may be separate FGs). X-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. [4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction] FFS: other component (which may be separate FG); FFS; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; [Per UE or Per band]; Candidate values of component 3: {Option 1, Option 2} [Candidate values of component 4: {5ms, 3ms}]; Optional with capability signalling. Agreement: Confirm the following component 4 in X-1 for XR - [4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction] Agreement: Confirm the following candidate values for component 4 in X-1 for XR - [Candidate values of component 4: {5ms, 3ms}]. Per the above agreeements, DCI format x_1, x_2 and x_3 are supported to have 1-bit indication field to indicate UE whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Considering that the support of DCI format x_1, x_2 and x_3 are based on different UE capabilities, one way is to separately define UE feature groups for XR for NR phase 3 based on DCI format x_1, DCI format x_2 and DCI format x_3. For this way, when a UE supports of DCI format x_2, x_3, UE has the freedom to report whether gap skiiping with DCI format x_2, or x_3 is supported. Another way is to define one UE feature group for all DCI formats, including all DCI format of x_1, x_2 and x_3. Some companies think there is no difference for UE to support gap skipping once it supports the corresponding DCI format. But if so, that means, if the UE supports DCI format x_2, or x_3 and gap skipping, the UE has to support gap skipping with all DCI formats of DCI format x_1, x_2 and x_3. From this perspective, the former is perferred Proposal 1: For UE features for NR_XR_ph3, separate UE feature groups are defined for DCI format x_1, x_2, 0_3 and 1_3. Proposal 2: UE features for NR_XR_ph3 are suggested as following. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 64. NR_XR_ph3; 64-1; Enable Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_1/1_1; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. DCI format 0_1/1_1 indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the PDCCH carrying the gap/restriction cancellation to the cancelled gap/restriction; 3-1; YES; Not support Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_1 or 1_1; Per band; N/A; YES; If UE only supports this feature for FR1 or FR2, gNB can only indicate the UE to skip a per FR gap for FR1 or FR2, respectively.; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. 64. NR_XR_ph3; 64-2; Enable Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_2/1_2; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. DCI format 0_2/1_2 indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the PDCCH carrying the gap/restriction cancellation to the cancelled gap/restriction; at least one of {11-1, 11-1a}; YES; Not support being indicated to perform Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_2 or 1_2; Per band; N/A; YES; If UE only supports this feature for FR1 or FR2, gNB can only use DCI format 0_2 or 1_2 to indicate the UE to skip a per FR gap for FR1 or FR2, respectively.; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. 64. NR_XR_ph3; 64-3; Enable Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 1_3; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. DCI format 1_3 indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the PDCCH carrying the gap/restriction cancellation to the cancelled gap/restriction; at least one of {49-1, 49-1b}; YES; Not support being indicated to perform Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 1_3; Per band; N/A; YES; If UE only supports this feature for FR1 or FR2, gNB can only use DCI format 1_3 to indicate the UE to skip a per FR gap for FR1 or FR2, respectively.; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. 64. NR_XR_ph3; 64-4; Enable Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_3; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. DCI format 0_3 indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the PDCCH carrying the gap/restriction cancellation to the cancelled gap/restriction; at least one of {49-2, 49-2b}; YES; Not support being indicated to perform Tx/Rx in gaps/restrictions that are caused by RRM measurements by DCI format 0_3; Per band; N/A; YES; If UE only supports this feature for FR1 or FR2, gNB can only use DCI format 0_3 to indicate the UE to skip a per FR gap for FR1 or FR2, respectively.; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. |
|  | Nokia | Discussion and proposals The main open issue is related on the support for different DCI formats. While it would be attractive to require that if the functionality is supported, then it is supported with all the DCI formats that the UE otherwise supports, this maybe too much to require. Proposal 1: Allow UE to separately indicate the support for DCI formats x_1, x_2 and x_3. Proposal 2: Make the following updates to the FG64-1 Feature group; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]. Component 1; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI  1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI. Component 2 and related candidate values; 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction Candidate values of component 2: {Format 0_1/1_1 only, format 0_1/1_1 and 0_2/1_2, format 0_1/1_1 and 0_3/1_3, format 0_1/1_1 and 0_2/1_2 and 0_3/1_3}. Component 3; 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour.. Component FFS; FFS: other component (which may be separate FG). Pre-requisite Feature groups; N/A FFS. Consequence if the feature is not supported by the UE; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]. Type; [Per UE or Per band]. Need of FDD/TDD differentiation; No. Need of FR1/FR2 differentiation; No. Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; No. Note; The supported band indicates the band where the DCI format is supported. |
|  | Samsung | # Discussion In [1], following agreements were made. Agreement: Introduce following basic FG for XR Phase 3, and FFS on some potential components (that may be separate FGs). X-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. [4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction] FFS: other component (which may be separate FG); FFS; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; [Per UE or Per band]; Candidate values of component 3: {Option 1, Option 2} [Candidate values of component 4: {5ms, 3ms}]; Optional with capability signalling. Agreement: Confirm the following component 4 in X-1 for XR - [4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction] Agreement: Confirm the following candidate values for component 4 in X-1 for XR - [Candidate values of component 4: {5ms, 3ms}]. Regarding whether to introduce a separate UE feature per DCI format or not, there is no apparent reason why a UE can support MG skipping indication when it is provided by, e.g., DCI format 0_1/1_1, but cannot support it when it is provided by e.g., DCI format 0_2/1_2. In such a case, only a single FG would be needed—the one for the offset value. Thus, [by DCI format 0_1/1_1] can be deleted in the column of ‘feature group’ and the column of ‘consequence if the feature is not supported by the UE.’ As for the prerequisite FG, it can consider: - 3-1 (Monitoring DCI formats 0_0, 1_0, 0_1, 1_1) - 11-1 (Monitoring DCI format 1_2 and DCI format 0_2) - 49-1 (Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with the same SCS between the scheduling cell and cells in the set) - 49-2 (Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with the same SCS between the scheduling cell and cells in the set) Regarding the first component, “is enabled” can be confirmed. Or, if there is no consensus for this part, considering endorsed spec text in TS38.213 [2] can be considered. For example: “Explicit indication by a DCI format in PDCCH reception is used to cancel an earliest RM measurement gap occasion where the UE would perform RRM measurements.” Regarding the third component, it can be updated to have similar wording to the spec text in TS38.213 [2]. For example: “For explicit indication by DCI, a bit value equal to ‘0’ of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception, where the UE would perform RRM measurements on cells in the same frequency band as the one or more scheduled cells associated with the DCI format, is as described in [10, TS 38.133].” With these understandings, the following proposal is suggested. Proposal 1: Consider the following UE capabilities for enable TX/RS for XR during RRM measurement X-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. - Option 2: For explicit indication by DCI, bit value equal to “0” of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [TS 38.133]. 4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG); FFS 3-1, 11-1, 49-1, 49-2; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; [Per UE or Per band]; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. |
|  | OPPO | Discussion The main remaining issue of UE feature for R19 XR is whether separate FGs should be defined for different DCI formats. DCI format 0_1/1_1 is mandatory UE feature, but DCI format 0_2/1_2 and 0_3/1_3 are optional UE features defined for R17 URLLC and R18 MC, respectively. To avoid additional implementation and testing of R17 URLLC and R18 MC for UEs supporting R19 XR, we propose that three UE FGs corresponding to different DCI formats should be defined for 19 XR. Proposal 1: Three UE FGs corresponding to different DCI formats should be defined for 19 XR: - FG X-1: Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_1/1_1 without prerequisite; - FG X-2: Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2 with prerequisite dependency on FG11-1; - FG X-3: Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_3/1_3 with prerequisite dependency on at least one of {FG49-1,FG 49-2}. |
|  | Qualcomm Incorporated | Minimum time offset In RAN1 #120, the Rel-19 UE capabilities for XR were discussed. Status and agreements were captured in [1] and [2]. One debate in RAN1#120 was whether the same FG or separate FGs should be defined for all the DCI formats 0_1/1_1, 0_2/1_2 and 0_3/1_3. Our understanding is there should be separate FGs for DCI format 0_3/1_3. The reason is DCI 0_3/1_3 were defined for one cell scheduling multiple cells. In this CA scenario, we do not think the UL/DL data scheduling latency issue exists. For DCI formats 0_2/1_2, it is also preferrable to define separate FGs as they were originally for a different use case, i.e., URLLC. For all FGs, the FG type should be per band for the benefits of flexible implementation and testing for the optional feature. This allows both network and UE to support a subset of frequency bands for XR traffic communication from all frequency bands the corresponding DCI formats are supported. Proposal 1: Support to define separate UE capabilities for DCI formats 0_1/1_1, 0_2/1_2, 0_3, and 1_3 - Type: Per band, for the band where UE Tx/Rx is enabled in skipped measurement gap - For DCI format 0_2/1_2, the pre-requisite includes FG 11-1 - For DCI format 1_3, the pre-requireiste includes one of {FG 49-1, FG 49-1b} - For DCI format 0_3, the pre-requireiste includes one of {FG 49-2, FG 49-2b} Details of the proposal are captured in the table below in the red text. 64-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG); FFS; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; [Per UE or Per band]; Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms}; Optional with capability signalling. 64-2; Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2; 1. DCI format 0_2/1_2 indication for enabling reception/transmission in gap/restriction; 64-1 and 11-1; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication by DCI format 0_2/1_2; Per band; Optional with capability signalling. 64-3a; Enabling TX/RX during measurement gap scheduling restriction by DCI format 1_3; 1. DCI format 1_3 indication for enabling reception/transmission in gap/restriction; 64-1 and one of {FG 49-1, FG 49-1b}; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication by DCI format 1_3; Per band; Optional with capability signalling. 64-3b; Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_3; 1. DCI format 0_3 indication for enabling reception/transmission in gap/restriction; 64-1 and one of {FG 49-2, FG 49-2b}; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication by DCI format 0_3; Per band; Optional with capability signalling. |
|  | NTT DOCOMO, INC. | - Discussions At the RAN1#120 meeting, it was agreed to introduce FG 64-1 for enabling TX/RX during measurement gap scheduling restriction. Following proposal regarding separation of FG 64-1 for different DCI formats was discussed, while no consensus was reached. Proposal 2: For separation of X-1 per DCI format(s), support Alt-1 - Alt-1: A FG (i.e., X-1) is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3) - Alt-2: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1/0_2/1_2. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1 and X-2 - Alt-3: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_2/1_2. Another FG (e.g., X-3) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1, X-2 and X-3. We prefer to have a single FG for DCI format x_1/x_2/x_3, since we think there is not much complexity difference for UE to support DCI format x_1/x_2/x_3 for gap/restriction occasion skipping indication. If UE supports FG 64-1, and if UE supports DCI format x_2, then UE supports DCI format x_2 for the skipping indication. If UE supports FG 64-1 and UE supports DCI format x_3, UE supports DCI format x_3 for the skipping indication. Proposal 1: FG 64-1 is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3). - If UE supports FG 64-1, and if UE supports DCI format x_2, then UE supports DCI format x_2 for the skipping indication. - If UE supports FG 64-1 and UE supports DCI format x_3, UE supports DCI format x_3 for the skip-ping indication. For type of this FG, we do not identify a strong need to define this with finer granularity, such as per BC/FC/FCPC. From technical point of view, we believe per UE or per band should be sufficient, and our slight preference is put on per UE given its smaller reporting overhead. Note that, even if it is defined per UE, we do not see a strong need of either FDD/TDD differentiation or FR1/FR2 differentiation, although we do not have strong opinion on this. If it is defined for per band, whether the capability is applied to the scheduling cell and/or scheduled cell needs to be clarified, considering cross-scheduling case. Since it was agreed that the 1 bit indication is applied to scheduled cell for cross-carrier scheduling case, it is straightforward that the capability is applied to the band of the scheduled cell. Proposal 2: Type of FG 64-1 is per UE or per band. |
|  | Ericsson | # 2 Discussions The summary of the discussion for the UE feature for Rel-9 XR, is available in [1]. The main open issue is whether to have the FG per DCI format X_1/X_2/X_3, or jointly. This issue is formulated by the following question in [1]. Question: For separation of X-1 per DCI format(s), companies are encouraged to provide their own preference/concern for the following alternative: - Alt-1: A FG (i.e., X-1) is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3) - Note: The support of XR feature for DCI format 0_2/1_2 and for DCI format 0_3/1_3 are subject to the support of DCI format 0_2/1_2 itself and DCI format 0_3/1_3 themselves, respectively - Alt-2: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1/0_2/1_2. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1 and X-2 - Alt-3: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_2/1_2. Another FG (e.g., X-3) is defined for the support of XR basic feature for DCI format 0_3/1_3. - FFS: Prerequisite relationship between FG X-1, X-2 and X-3 If the UE supports any of the DCI formats x_1/x_2, the feature functionality is no difference, and there is no motivation to have separate feature. Furthermore, considering the conditions imposed for the support of this feature for DCI format x_3, the operation would be effectively similar to single cell scheduling. It has been argued that DCI format x_2 is originally specified for URLLC. For URLLC traffic, the measurement gaps should be already conservatively configured to avoid the latency impact to scheduling. In our view, this argument is not well motivated. We disagree with this argument. Measurement gaps are provided such that the UE can be connected to be best possible serving cell which is crucial for proper operation of URLLC traffic. Moreover, upon arrival of URLLC traffic, the need for cancellation of the gap is higher to meet the delay requirements when the conditions for the serving cell is satisfactory. It has been argued that for DCI format x_3 when CA is configured, the motivation for cancelling measurement gaps for Tx/Rx is not critical. In our view, the opponents question the support of feature for CA operation rather than DCI format x_3 which a strong limitation that is not supported by any agreements. Therefore, we make the following observations and propose: Considering the conditions imposed for the support of this feature for DCI format x_3, The functionality of this feature is no difference for DCI format x_1/x_2/x_3. Upon arrival of URLLC traffic, the need for cancellation of a gap is higher to meet the delay requirements when the conditions for the serving cell is satisfactory. The argument of coupling DCI format x_3 for CA and discouraging cancelling measurement gaps, in fact questions the support of feature for CA operation rather than DCI format x_3 which a strong limitation that is not supported by any agreements. For separation of X-1 per DCI format(s), support Alt. 1. Regarding the feature type, our preference is “Per UE” as it simplified the NW operation. However, it case of strong interest from majority of companies, we can be supportive of “Per Band”. Support “Per UE” for the feature Type. - Note: In case, there is a strong interest, we can be supportive of “Per Band”. |

- Discussion

Summary of companies’ views and discussion

Per DCI format separation: - One FG for all DCI x_1/x_2/x_3: HW/Hisi, ZTE, Nokia, Samsung, DCM, E/// - Still DCI x_2/x_3 is optional: Nokia, - Different FG for different DCI x_1/x_2/x_3: vivo (DL/UL separation for x_3) - Vivo, OPPO, QC, Component 1: - Confirm “is enabled”: Nokia Component 3: - Confirm “as of Rel-18”: Nokia, - Wording change: Samsung - Samsung: “Option 2: For explicit indication by DCI, bit value equal to “0” of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [TS 38.133].” Others for components - Remove “other components”: HW/Hisi, vivo, Nokia, Samsung, Type - Per UE: HW/Hisi, ZTE, vivo, E///, - Per band: Nokia, QC, DCM, Prerequisite FG for FG 64-1 - n/a: HW/Hisi, Nokia - 3-1: vivo, Samsung, - 11-1 (DCI 2_x): Samsung, - 49-1/2: Samsung FR1/FR2 differentiation - Yes: ZTE - n/a: vivo, Note: - “The supported band indicates the band where the DCI format is supported”: Nokia

Proposal 1:

For Component 2 in FG 64-1, support Alt-1 below:

- Alt-1: Confirm all DCI formats (i.e., DCI format 0_1/1_1/0_2/1_2/0_3/1_3)

  - Prerequisite FG: No

  - Add the following three notes in Note column:

    - “If UE supports FG 64-1, and if UE supports FG 11-1, then the UE supports DCI format 0_2/1_2 indication for enabling reception/transmission in gap/restriction”

    - “If UE supports FG 64-1, and if UE supports FG 49-1 or 49-1b, then the UE supports DCI format 1_3 indication for enabling transmission in gap/restriction”

    - “If UE supports FG 64-1, and if UE supports FG 49-2 or 49-2b, then the UE supports DCI format 0_3 indication for enabling reception in gap/restriction”

- Alt-2: Confirm only DCI format 0_1 and 1_1. Introduce separate FG(s) for enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2/0_3/1_3, i.e.:

  - Revise FG 62-1 as follows:

    - FG name: confirm “by DCI format 0_1/1_1”

    - Component 2: Confirm by removing [/0_2/1_2/0_3/3_3]

    - Consequence if not supported: confirm “by DCI format 0_1/1_1”

    - Prerequisite FG: No

  - Define FG 64-2 as follows:

    - FG name: “Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2”

    - Component:

      - 1. DCI format 0_2/1_2 indication for enabling reception/transmission in gap/restriction

    - Prerequisite FG: 64-1 and 11-1

  - Define FG 64-3 as follows:

    - FG name: “Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_3[/1_3]”

    - Component:

      - 1. DCI format 0_3[/1_3] indication for enabling [reception/]transmission in gap/restriction

    - Prerequisite FG: 64-1 and at least one of {[49-1, 49-1b,] 49-2, 49-2b}

    - FFS: Whether to separate this FG into two; one for uplink and the other for downlink

| Companies | Proposals |
| --- | --- |
| Qualcomm | We support Alt-2. Through the design of the feature, we do not think DCI x_2 and x_3 are useful for indicating MG skipping. Hence, we do not think UE has to assume the presence of the additional bit in its UE specific signaling. |
| ZTE, Sanechips | Prefer Alt 1, and fine with the additions in the note. |
| Apple | We support Alt. 2 |
| DOCOMO | We prefer Alt-1. there is not much complexity difference for UE to support DCI format x_1/x_2/x_3 for gap/restriction occasion skipping indication. |
| vivo | We support Alt-2. |

Proposal 2 (closed):

Update Component 1 of FG 64-1 as follows (i.e., confirm “is enabled”):

- “1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI”

| Companies | Proposals |
| --- | --- |
| Qualcomm | Support Proposal 2. |
| ZTE, Sanechips | We are fine with the proposal. |
| DOCOMO | Support the proposal. |
| vivo | Support the proposal. |
| Moderator | With the following agreement, this discussion has been closed. Agreement: Update Component 1 of FG 64-1 as follows (i.e., confirm “is enabled”): - “1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI” |

Question 3 (closed):

Companies are encouraged to share views on whether to update Option 2 text in Component 3 of FG 64-1 as follows:

- Alt-1: Confirm “of Rel-18”, i.e., the following:

3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour.

- Alt-2: Revise such that “Option 2: For explicit indication by DCI, bit value equal to “0” of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [TS 38.133].”

| Companies | Proposals |
| --- | --- |
| Qualcomm | We prefer Alt-1. The Alt-2 wording “UE would perform RRM measurements” seems to imply that the UE is required to perform RRM measurement in the MG if it is not skipped. |
| ZTE, Sanechips | We are open to discuss. For Alt-1, if the ‘as Rel-18 behavior’ is used, it should be clarified that whether Rel-15 to Rel-17 UE behaviors are included. |
| DOCOMO | We are fine with either Alt-1 or Alt-2. Alt-1 is simpler for reading. |
| vivo | We have no strong preference, alt 1 seems better considering the concern from Qualcomm. |
| Moderator | With the following agreement, this discussion has been closed. Agreement: On whether to update Option 2 text in Component 3 of FG 64-1, support Alt-1 as follows: - Alt-1: Confirm “of Rel-18”, i.e., the following: 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour.. - Alt-2: Revise such that “Option 2: For explicit indication by DCI, bit value equal to “0” of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [TS 38.133].” |

Proposal 4:

Companies are encouraged to share views on Type for FG 64-1:

- Alt-1: Per band

- Alt-2: Per UE

| Companies | Proposals |
| --- | --- |
| Qualcomm | We support Alt-1 per band for the band where the MG is skipped. Similar to other per band UE features, this allows for network and UE implementation and test of the feature for each band separately which encourages network and UE to support the feature. Besides, the MG itself can be per band. Supporting per band capability for the MG skipping feature is more aligned with per band MG. |
| ZTE, Sanechips | Support Alt-2 Per UE. If granularity of per band is considered, it may cause complexity on gNB indication to different UEs in different bands. |
| Apple | Per Band or Per band per band combination |
| DOCOMO | Prefer Alt 2. |
| vivo | We support Alt-1. |

Question 5 (new):

Companies are encouraged to share views on the following questions on FG 64-1:

- Q5-1: If per UE is adopted for FG 64-1, if a UE supports FG 64-1, does it imply the UE supports the behavior of “Enabling TX/RX during measurement gap scheduling restriction” and monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction as follows?

  - For the behavior of “Enabling TX/RX during measurement gap scheduling restriction”, it is supported in any band(s) that the UE supports

  - For monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction, it is supported in any band(s)

- Q5-2: If per band is adopted for FG 64-1, if a UE supports FG 64-1, does it imply the UE supports the behavior of “Enabling TX/RX during measurement gap scheduling restriction” and monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] as follows?

  - Alt-1: The behavior of “Enabling TX/RX during measurement gap scheduling restriction” is supported in the band(s) that the UE reports FG 64-1. Monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction is supported in any band(s) that the UE supports as scheduling band.

    - E.g., If UE reports FG 64-1 for band A, the behavior of “Enabling TX/RX during measurement gap scheduling restriction” is supported in band A. Monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction is supported in any scheduling band(s).

  - Alt-2: The behavior of “Enabling TX/RX during measurement gap scheduling restriction” is supported in any band(s) that the UE supports as scheduled band(s). Monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction is supported in the band(s) that the UE reports FG 64-1.

    - E.g., E.g., If UE reports FG 64-1 for band A, the behavior of “Enabling TX/RX during measurement gap scheduling restriction” is supported in any scheduled band(s). Monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction is supported in band A

  - Alt-2: Both the behavior of “Enabling TX/RX during measurement gap scheduling restriction” and monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction are supported in the band(s) that the UE reports FG 64-1.

  - Any other alternative?

| Companies | Proposals |
| --- | --- |
| Moderator | I am trying to capture a discussion point that is raised by OPPO during online. After some thinking, if FG 64-1 is per UE, both skipping behavior and monitoring of corresponding DCI formats are supported in any band(s) that the UE supports. While, if per band is applied, there may not be common understanding on what is exactly supported in the band (i.e., skipping behavior and/or monitoring of corresponding DCI formats). For alt-1, cross carrier indication is possible even when a UE reports FG 64-1 in a single band only, in which scheduling cell could be anywhere that the UE supports as scheduling cell(s). For alt-2, cross carrier indication is possible even when a UE reports FG 64-1 in a single band only, in which scheduled cell could be anywhere that the UE supports as scheduled cell(s). For alt-3: cross carrier indication is possible only when a UE supports FG 64-1 in two or more bands. In the band(s) that the UE reports FG 64-1, both skipping behavior and monitoring of corresponding DCI format(s) are supported. |
| OPPO | In our thinking, there can be three options to spell out the FG64-1. Opt-1: FG 64-1 is about “capability of enabling Tx/Rx” but not about “capability of retrieving the skipping indication from a DCI format”. Then because the band where Tx/Rx can be enabled is not necessarily the band where the corresponding DCI is/can be received, the legacy FGs for specific DCI format reception cannot be the prerequisite of FG64-1. Instead, to maintain the capability logic that the UE should report to gNB what DCI format it is capable to read the skipping indication, the supported DCI formats should be in a selective list in feature component for UE to report, something like: Component #x: UE reports the supported DCI formats from set of {0_0, 0_1, 1_0, 1_1, 2_0, 2_1, 3_0, 3_1}, in which it can read the skipping indication. Note: this does not impact the interpretation of legacy FGs on receptions of different DCI formats In Opt-1, FG64-1 can be either per band or per UE. Opt-2: FG 64-1 is about “capability of retrieving the skipping indication from a DCI format”, but not about “capability of enabling Tx/Rx”. In this Opt-2, the FG can base on legacy capabilities on receptions of different DCI formats as individual prerequisite, and use a FG granularity that is compatible to that of prerequisite FG (e.g., if the prerequisite FG is per BC, the new FG is better not to be per UE). For the granularity, FG64-1 can be per band or per BC in order to match that of prerequisite FG. However, the unfortunate thing for Opt-2 is that it does not seem to fit well with the following agreed Component #1: Agreement: Update Component 1 of FG 64-1 as follows (i.e., confirm “is enabled”): - “1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI” Opt-3: FG64-1 is about both “capability of retrieving the skipping indication from a DCI format” and “capability of enabling Tx/Rx”. As we commented online, this Opt-3 does not seem quite compatible with per-Band or per-BC granularity. Per-UE seems to be a better fit. Meanwhile, we think it is an overkill to mandate UE to support retrieving indication bit from all supported DCI formats (e.g., supporting DCI format 3_1 does not necessarily mean the UE should support having skipping indication bit in DCI format 3_1). So maybe it is better to have the same thing above in Opt-1. With above in mind, back to moderator’s questions, below are our considerations. Q5-1: If per UE is adopted for FG 64-1, a support of FG 64-1 can mean that - For the behavior of “Enabling TX/RX during measurement gap scheduling restriction”, it is supported in any band(s) that the UE supports. - For monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction, it is supported in any band(s) according to a reported FG component describing the list of DCI formats in which the UE is capable to obtain the indication bit. The capability of receiving the corresponding DCI format is still managed by the legacy FGs. - The legacy FGs for reception of different DCI formats are NOT prerequisites of FG64-1. Q5-2: If per band is adopted for FG 64-1, a support of FG 64-1 can mean that - Alt-1: The behavior of “Enabling TX/RX during measurement gap scheduling restriction” is supported in the band(s) that the UE reports FG 64-1. Monitoring of DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction is supported in any band(s) that the UE supports as scheduling band, subject to a reported FG component describing the list of DCI formats in which the UE is capable to obtain the indication bit. The capability of receiving the corresponding DCI format is still managed by the legacy FGs. - Alt-2: Same understanding as moderator. But for Alt-2, the agreed (updated) component #1 may need a bit re-wording, since it does not seem to say the support of “enabling of Tx/Rx” is on any band. - Alt-3: The Alt-3 from moderator is certainly one alternative, but in our view it is restrictive. |

- Conclusions

TBD

References

- R1-2503289	UE feature for R19 XR	Huawei, HiSilicon

- R1-2503324	Discussion on XR features	ZTE Corporation, Sanechips

- R1-2503391	Discussion on Rel-19 UE features for XR for NR Phase 3	vivo

- R1-2503411	UE features for XR for NR Phase 3	Nokia

- R1-2503597	UE features for XR for NR Phase 3	Samsung

- R1-2504215	Discussion on UE features for Rel-19 XR	OPPO

- R1-2504425	UE features for Rel-19 XR for NR Phase3	Qualcomm Incorporated

- R1-2504529	Discussion on UE features for Rel-19 XR enhancements	NTT DOCOMO, INC.

- R1-2504621	UE features for XR enhancements Phase 3	Ericsson

Appendix: XR UE features after RAN1#120

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 64. NR_XR_Ph3 | 64-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) | FFS | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | [Per UE or Per band] |  |  |  | Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms} | Optional with capability signalling |