3GPP TSG RAN WG1 #122                                                            R1- 2505367

Bengaluru, India, Aug 25th – 29th, 2025

Source:	vivo

Title:	Maintenance on specification support for beam management

Agenda Item:	8.1.1

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, WID [1] for NR AI/ML air interface was approved which includes enhancement on beam management, which is copied below for reference.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we focus on the remaining issues for the use case of beam management.

- Remaining issue on associated ID

It is necessary to discuss how associated IDs are determined or allocated, e.g., global ID or local ID. The details of global associated ID and local associated ID are provided in the following,

- Global associated ID would refer to NW side additional conditions of different NW-side implementation/configurations. Different global associated ID would refer to different Tx beam boresight direction (azimuth and elevation), 3dB beam-width and mapping of actual beams to beam indices, etc.

- Local associated ID is cell/site/region specific. NW may use different beam codebook in the daytime and night, since the UE distribution and UE requirement may be different in the daytime and night. Then NW could use different local associated ID to indicate different beam codebook.

The local associated ID requires large efforts at UE side to categorize the collected data to maintain consistency between training and inference. Depending on how local the associated ID is applied, the trained model may only be applicable to a relatively small region thus may potentially require cell/site/region specific model development. Conversely, global associated ID is useful for maintaining consistency between training and inference and help UE to train a model with good generalization performance globally and thus would be friendlier to UE side management.

However, global associated ID may potentially expose deployment choices of NW side, which results in the working assumption agreed in AI 9.1.3.3 for local associated ID to address consistency issues. Along the line of the spirit of the working assumption in AI 9.1.3.3, local associated ID can be prioritized. On the other hand, ways to resolve UE side complexity on managing local associated IDs should be further considered.

For the case when UE moves to a cell that UE has not collected data before, we can consider the following procedures to make UE side mode more efficient.

Figure 2-2: associated ID confirmation for UE-side AI model

For data collection and model training:

- Step 1: Beam related data is collected with associated ID and corresponding cell ID(s), e.g. associated ID + {cell 1, cell 2, cell 3, …, cell N}, or associated ID + area ID

- Step 2: AI model is trained by collected data, and mapping to the associated ID + cell ID(s) or area ID

For inference phase:

- Step 1: gNB configures associated IDs, corresponding cell IDs or area ID with RS/Report configuration to the UE, e.g. associated ID + cell 1, another associated ID + another cell, or associated ID + area ID

- Step 2: The supported associated IDs can be report by UE with applicable report.

The aforementioned process, i.e. introduce an indicator to indicate whether to interpret the associated ID as for a target cell/area in data collection procedure, ensures the UE-side AI model to be working with multi-cell associated ID.

Based on the above, there are use cases for interpreting the associated ID either as for target cell or source cell. Hence, we suggest to include a new indicator to indicate whether to interpret the associated ID as for target cell or source cell, e.g., such indicator can be a cell ID to interpret the associated ID.

For inference, for UE-side model, introduce an area ID along with the configuration of associated ID to indicate how to interpret the associated ID for further address consistency issue.

- Remaining issue on performance monitoring

How to report BAI for initial monitoring report

For performance monitoring report, upon network-side configuration of such reports, they are designed to deliver performance metrics strictly adhering to predefined parameters. A procedural ambiguity emerges when the report occurs prior to the UE accumulating sufficient monitoring instances, in the conditions when Scell activation, BWP switch, CSI reconfiguration, etc., happens. It shall be clarified how the UE should handle and format incomplete monitoring results when the reporting interval precedes completion of the configured monitoring instances.

Figure 3-9: Report BAI for initial monitoring report

For example, in above figure, if the report configuration mandates M+1 monitoring instance but the UE has only executed M observations at the first reporting occasion, it takes two risks:

- Unqualified Reporting: Reporting M1 successful predictions without explicitly indicating the denominator (M vs. M+1) would lead the network to erroneously calculate accuracy as M1/(M+1) rather than M1/M.

- Consequential Network Misinterpretation: This accuracy could falsely suggest degraded AI inference performance, potentially triggering unnecessary network reconfiguration actions.

To address the aforementioned issue, the following potential solutions are proposed:

- Option 1: Restrict UE from submitting reports until the configured number of monitoring instances is achieved.

- Option 2: Prohibit the network from deactivating inference configurations prior to a reference point, and the reference point can be FFS.

- Option 3: Reporting the number of accurate predictions and actual monitoring instances in the monitoring report.

- Option 4: Reporting the number of accurate predictions and accurate ratio in the monitoring report, which accurate ratio can be defined as the number of accurate predictions divided by actual monitoring instances

For performance monitoring, for UE-side model, support one of the following options for BAI reporting at initial monitoring procedure.

- Option 1: Restrict UE from submitting reports until the configured number of monitoring instances is achieved

- Option 2: Prohibit the network from deactivating inference configurations prior to a reference point, and the reference point can be FFS.

- Option 3: Reporting the number of accurate predictions and actual monitoring instances in the monitoring report.

- Option 4: Reporting the number of accurate predictions and accurate ratio in the monitoring report, which accurate ratio can be defined as the number of accurate predictions divided by actual monitoring instances

- Remaining issue on AI processing capability

- In previous RAN meeting, the following agreement was endorsed for AI capability,

Agreement

A UE can report support for N (up to 2) AI/ML PU pools for AI/ML features

- For each of the N AI/ML PU pools, UE reports the maximum number of simultaneous AI/ML PUs, respectively.

- If N = 2, for each AI/ML feature, UE reports which AI/ML PU pool it belongs to.

There can be ambiguity regarding the interpretation of “each AI/ML feature”, particularly in beam management use case, which includes two sub use cases: BM case 1 for spatial domain prediction and BM case 2 for temporal domain prediction. It's crucial to clarify that what “each AI/ML feature” in the above agreement refers to. We think a natural way is to report such PU information in relevant AI/ML FGs. Then RAN1 should decide which FGs should contain such PU information or two FGs can correspond to different TU pools.

One of the simplest ways is to allow each FGs can indicate which TU pool it belongs to. Based on the current UE feature list, at least FG, UE-side beam prediction for BM Case1 [for inference] (FG 58-1-2), UE-side beam prediction for BM Case1 with predicted RSRP [for inference] (FG 58-1-3), UE-side beam prediction for BM Case2 [for inference] (FG 58-1-4) and UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] (FG 58-1-5), should contain such information.

It's necessary to clarify that “each AI/ML feature” refers to relevant AI/ML FGs. Based on the current AI/ML UE feature list, at least FG, UE-side beam prediction for BM Case1 [for inference] (FG 58-1-2), UE-side beam prediction for BM Case1 with predicted RSRP [for inference] (FG 58-1-3), UE-side beam prediction for BM Case2 [for inference] (FG 58-1-4) and UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] (FG 58-1-5), shall contain which AI/ML PU pool it belongs to when N=2.

- Conclusions

In this contribution, we discuss some issues on AI/ML for beam management and have the following proposals:

- For inference, for UE-side model, introduce an area ID along with the configuration of associated ID to indicate how to interpret the associated ID for further address consistency issue.

For performance monitoring, for UE-side model, support one of the following options for BAI reporting at initial monitoring procedure.

- Option 1: Restrict UE from submitting reports until the configured number of monitoring instances is achieved

- Option 2: Prohibit the network from deactivating inference configurations prior to a reference point, and the reference point can be FFS.

- Option 3: Reporting the number of accurate predictions and actual monitoring instances in the monitoring report.

- Option 4: Reporting the number of accurate predictions and accurate ratio in the monitoring report, which accurate ratio can be defined as the number of accurate predictions divided by actual monitoring instances

It's necessary to clarify that “each AI/ML feature” refers to relevant AI/ML FGs. Based on the current AI/ML UE feature list, at least FG, UE-side beam prediction for BM Case1 [for inference] (FG 58-1-2), UE-side beam prediction for BM Case1 with predicted RSRP [for inference] (FG 58-1-3), UE-side beam prediction for BM Case2 [for inference] (FG 58-1-4) and UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] (FG 58-1-5), shall contain which AI/ML PU pool it belongs to when N=2.

References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface