3GPP TSG RAN WG1 #122bis			R1-2507980

Prague, Czech Republic, October 13th –17th, 2025

Title:	DRAFT LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122bis

Response to:	-

Release:	Rel-19

Work Items:	NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_NTN_Ph3, NR_MC_enh2, TEI19, NR_LBCA_Sw

Source:	Moderators (AT&T, NTT DOCOMO, INC.) [RAN WG1]

To:	RAN WG2

CC:	RAN WG4

Contact Person:

Name:	Naoya Shibaike, Ralf Bendlin

E-mail Address:	naoya.shibaike.eg@nttdocomo.com, ralf_bendlin@labs.att.com

Attachment:	               R1-2507979.zip (Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis)

1. Overall Description:

RAN1 has discussed the Rel-19 RAN1 UE features list for NR and would like to share the latest version with RAN2 and RAN4 in the attachment R1-2507979.

RAN1 would like to thank RAN2 for the LS on per band and per BC capability in R1-2506724 (R2-2506565). The following is RAN1’s understanding.

Question 1: what is the relationship between per band and per BC capabilities i.e. what is the final capability in the following cases? - Case 1: when UE indicates both per band and per BC capability - Case 2: when per BC capability is indicated but one of the bands in the BC doesn’t indicate per band capability - Case 3: when the UE reports the per band capability but does not include the per BC capability for a certain BC

Case 1:

- When UE indicates both per band and per BC capability, if the capability/component is not counted across CCs, then the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA)

  - RAN1 will continue to discuss the cases

    - When the capability/component is counted across CCs regardless of whether CA is configured or not

    - When the capability/component is not counted across CCs and CA is not configured

  - RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19, e.g., in some cases a clear “ranking” between capabilities may need to be clarified in the Notes

Case 2:

- RAN2’s understanding is generally correct

- RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19

Case 3:

- RAN2’s understanding is generally correct

- RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19

Question 2: how do we define pre-requisite if pre-requisite is also per band and per BC capabilities?

For per band and per BC capabilities, it is expected that UE indicates the same granularity as the pre-requisite

For NR_MC_enh2,

RAN1 would like to inform RAN2 of the following agreement:

Agreement:

Ask RAN2 to feedback if it is possible to update prerequisite FG(s) for Rel-18 RAN1 FG 49-4a/4b/4c/4d/5a/5b/6/7/8/9/10/12/12a/13/14 in Rel-19 as follows (red fonts):

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b, 66-2} |
| 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value | At least one of {49-1, 49-1b, 66-1} |
| 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | At least one of {49-1, 49-1b, 66-1} |
| 49-6 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and 'codeBlockGroupTransmission" for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot - Candidate values for the component 6 of this FG is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration 7. Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK | At least one of {49-1, 49-1b, 66-1} |
| 49-7 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 1_3/0_3 | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) for DCI format 1_3/0_3 1) Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 0_3 2) Multiplexing/prioritization between UL channels/signals with the same PHY priority level 3) Prioritization between UL channels/signals with different PHY priority levels 4) Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. 5) Additional number of symbols (d2) of the preparation time needed for the high priority UL transmission that cancels a low priority UL transmission | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-8 | Triggered HARQ-ACK codebook re-transmission for DCI format 1_3 | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset | at least one of {49-1, 49-1b, 66-1} |
| 49-9 | SCell dormancy indication within active time in DCI format 0_3/1_3 | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_3/1_3 | 6-5, at least one of {49-1, 49-1b, 49-2,49-2b, 66-1, 66-2} |
| 49-10 | Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3/1_3 | 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 and 1_3 2) minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset 3) minimumSchedulingOffset K2 configuration for PUSCH 4) Support of extended value range for aperiodic CSI-RS triggering offset | At least one of {49-1, 49-1b, 49-2,49-2b, 66-1, 66-2} |
| 49-12 | Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra-cell and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | 1: TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2: The minimum beam application time in Y symbols per SCS 3: The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1, At least one of {49-1, 49-1b, 66-1} |
| 49-12a | Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC | - TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 23-10-1, At least one of {49-1, 49-1b, 66-1} |
| 49-13 | Default QCL assumption for multi-cell scheduling by DCI format 1_3 | Indicates whether the UE can be configured with enabledDefaultBeamFormultiCellScheduling for default QCL assumption for multi-cell scheduling by DCI format 1_3 for same/different numerologies - Candidate values are {different only, both} - When "both" is reported, the UE supports this feature for same SCS and for different SCS combination(s) (low-to-high, high-to-low or both) reported for 49-1b | At least one of {49-1, 49-1b, 66-1} |
| 49-14 | Support of BWP switch indication by DCI format 0_3/1_3 | Support of BWP switch indication by DCI format 0_3/1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} for the BC At least one of {6-2, 6-3, 6-4} for at least one band of the BC |

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully asks RAN2 to take into account above information and the RAN1 NR UE features in the attachment for designing corresponding capability signalling in Rel-19.

3. Dates of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #123				November 17 to 21	Dallas, US

TSG-RAN WG1 Meeting #124				February 9 to 13	Gothenburg, SE