3GPP TSG RAN WG1 #122bis		R1-2508086

Prague, Czech, October 13th – 17th, 2025

Source: 	Moderator (CMCC)

Title:	FL summary for Frame Structure (3rd round)

Agenda:	11.3.2

Document for:	Discussion & Decision

# Background

The issues/proposals related to agenda 11.3.2 in this document are marked with [H]/[M]/[L] priority (for RAN1#122bis).

## Workplans

## Contacts

Please see contacts list in Annex 1.

# Online/offline proposals

## Monday Online

###### [004r1] FL Proposal

As a start point, 6GR study assumes SSB and all channels/signals (except PRACH) use the same SCS for a given band.

- Other options are not precluded to be discussed in the future in initial access discussion.

- Note: ISAC is separate discussed in ISAC session.

###### [101r1] FL proposal

- 6GR considers NR frame structure is used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12.

  - Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier.

  - Symbol length follows NR design as a start point, details FFS.

  - FFS: Maximum number of subcarriers/RBs per NR carrier from RAN1 specification perspective.

###### [102r1] FL proposal

- A slot is defined as supporting 14 symbols for normal CP case and all subcarrier spacings all numerologies.

  - FFS: Number of OFDM symbols within a slot for longer CP case if supported

###### [005] FL proposal

- Extended CP is not supported for general communication.

- If RAN1 agree to study in the study item, for extend CP, focused on NTN or sensing cases.

###### [006r1] FL proposal

- From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is

- [800MHz or 400MHz or 200MHz] for FR2

- 200MHz for Sub-6GHz

  - FFS: limit to 100MHz for FDD

- [400MHz or 200MHz] for around 7GHz

- [400MHz or 200MHz] for around 15GHz

###### [007r1] FL proposal

- The maximum supported FFT size from RAN1 perspective for study is as follows,

  - 8192, 16384

  - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier.

## Tuesday Online

###### [004r3] FL Proposal

As a start point, 6GR study assumes all channels/signals (except PRACH) use the same SCS for a given band.

- Other options assume for different SCS for 6GR sync signal and other channels/signals (except PRACH) are not precluded to be discussed in initial access discussion.

- Note: ISAC is separately discussed in ISAC session.

###### [101r3] FL proposal

- 6GR considers NR frame structure is used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12.

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - Each symbol within 1ms is aligned with NR design.

  - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms

  - Symbol length follows NR design as a start point, details FFS.

  - FFS: Maximum number of subcarriers/RBs per NR carrier from RAN1 specification perspective.

###### [102r1] FL proposal

- A slot is defined as supporting 14 symbols for normal CP case and all subcarrier spacings all numerologies.

  - FFS: Number of OFDM symbols within a slot for longer CP case if supported

###### [005] FL proposal

- Extended CP is not supported for communication.

- If RAN1 agree to study in the study item, for extend CP, focused on NTN or sensing cases.

###### [006r1] FL proposal

- From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is

- 800MHz or 400MHz or 200MHz for FR2

- 200MHz for Sub-6GHz

  - FFS: limit to 100MHz for FDD

- 400MHz or 200MHz for around 7GHz

- 400MHz or 200MHz for around 15GHz

###### [007r1] FL proposal

- The maximum supported FFT size from RAN1 perspective for study is as follows,

  - 8192, 16384

  - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier.

## After Tuesday Online

###### [004r3] FL Proposal

As a start point, 6GR study assumes all channels/signals (except PRACH) use the same SCS for a given band.

- Other options assume for different SCS between 6GR sync signal and other channels/signals (except PRACH) are not precluded to be discussed in initial access discussion.

- Note: ISAC is separately discussed in ISAC session.

###### [006r2] FL proposal

- From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier for study is down-selected from the followings,

- 800MHz, 400MHz or 200MHz for FR2

- 200MHz for Sub-6GHz

  - FFS: limit to 100MHz for FDD

- 400MHz or 200MHz for around 7GHz

- 400MHz or 200MHz for around 15GHz

###### [006r3] FL proposal

- From RAN1 perspective, maximum channel bandwidth and SCS per 6GR carrier around 7GHz is

- 400MHz

  - Note: a smaller maximum channel bandwidth supported by UE is allowed.

  - Note: the above doesn’t preclude to have multiple FFT operations within a carrier

- 30kHz SCS

###### [001r2] FL Proposal

Observation for around 7GHz:

30kHz and 60kHz SCS have been studied. Based on the preliminary evaluations from companies’ contributions (Apple [1], DOCOMO [2], Ericsson [3], Huawei [4], Lenovo [5], MediaTek [6], Nokia [7], OPPO [8], Spreadtrum [9], ZTE [10], Kyocera[11], Interdigital[12], Qualcomm[13]), the followings are observed:

- With large delay spread (e.g., 300ns),

  - 30 kHz performs better with marginal or significant (e.g., up to ~4dB [9]) SNR gain for all MCS.

  - [2, 3, 6] reported for high-order modulation (e.g., for 256QAM/1024QAM), 60kHz SCS cannot achieve 10% BLER target.

  - [13] observed that using legacy comb-2 DMRS with FD-OCC2, there is a performance degradation when using 60KHz due to lower frequency domain sampling density. However, this performance loss can be reduced by removing FD-OCC (using TD-OCC only instead). Hence 60kHz can achieve better throughput.

- With small delay spread (e.g., 30ns),

  - 60 kHz performs better with marginal or small (<1dB) SNR gain for all MCS.

  - [check on delay and doppler spread]

- With medium delay spread (e.g., around 100ns),

  - 2 sources ([8, 10]) observers 30 kHz performs better with marginal or small (up to 1.2 dB) SNR gain for QPSK and 3km/h UE speed,

  - 2 sources ([2, 6]) observers 60 kHz performs better with marginal or significant (up to 2 dB) SNR gain for higher modulation layer (e.g., 64QAM/256QAM/1024QAM),

  - 1 source ([3]) observers either 30 kHz or 60kHz SCS performs better under different modulation layer and UE speed.

Proposals for simulation (will pick up a few cases if agreed)

For the study of 6GR numerologies (including SCS, CP), the following simulation assumptions for studying of the 6GR numerologies are considered,

- Companies are encouraged to provide simulation results in RAN1#[123?/124?]

- System level simulation results can be optionally provided by company.

|  | Around 7GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Channel Model | FFS: TDL-x, CDL-x TDL-C TDL-D CDL-A TDL-A with medium correlation Delay Spread: {30 ns, 90/100 ns, 300 ns}? 30ns, 100ns, 300ns |  |  |  |
| DL/UL | Company report |  |  |  |
| Carrier frequency | 7GHz |  |  |  |
| Antenna configuration | 4Tx4Rx, 2Tx2Rx? Follow 11.2 outcome when available ? |  |  |  |
| Simulation Bandwidth | 100MHz, 200MHz, 400MHz Can we agree on 50MHz instead? |  |  |  |
| SCS | 30kHz, 60kHz |  |  |  |
| FFT size | Reported by companies |  |  |  |
| Cyclic Prefix | Normal |  |  |  |
| UE speed | 3/30/60/120[/500][/300][/100][/1000] km/h |  |  |  |
| Waveform | 5G NR CP-OFDM, other waveforms can be reported by companies. |  |  |  |
| MCS | [QPSK], [16QAM, ]64QAM, 256QAM, 1024QAM FFS: code rate |  |  |  |
| FEC | 5G NR LDPC, Other coding scheme is not precluded and reported by companies. |  |  |  |
| Other assumptions | The followings are reported by companies, - DMRS or CSI-RS configuration, - Phase noise model, - Carrier frequency offset, - … |  |  |  |
| Performance Metrics | BLER, system overhead, throughput, other qualitative / quantitative results can be reported by companies |  |  |  |

###### [106] FL proposal

- For the study of 6GR frame structure configuration / indication

  - Support of cell and UE specific RRC configuration

    - FFS necessity to remove UE specific RRC configuration

  - Support of multiple TDD-pattern combinations, i.e., similar to NR

###### [109r1] FL proposals:

Version 1

6GR considers the following symbols/slots types,

- Support at least DL, UL and additional symbol type(s) for 6GR

- Study of the new additional symbol type(s) in addition to DL and UL to support

  - BS-side semi-static SBFD

  - Dynamic TDD

  - Reservation of resources

  - Energy saving, e.g., Sleeping

Version 2

6GR considers NR symbol/slot types as a starting point for the study item,

- study further additional symbol type(s) to support

  - BS-side semi-static SBFD

  - Dynamic TDD

  - Reservation of resources

  - Energy saving , e.g., Sleeping

###### [107r1] FL observation and suggestion:

Study 6GR frame structure configuration/indication to allow dynamic TDD, e.g.,

- Restriction of one DL-UL switching point within a D to ‘X’ to U TDD pattern, ‘X’ is flexible symbols/slots

- Avoid slot-level SFI while a few long term TDD configurations can be supported to match long-term statistical characteristics

- Frame Pattern Indicator (FPI) to switch to a particular frame pattern from the pre-configured option

- For flexible symbol configured by RRC, UE will follow dynamic grants to either transmit or receive

Dynamic Indication (i.e., dynamic SFI indication by DCI) is deprioritized for study due to complexity of UE implementation

###### [108r1] FL observation and suggestion:

Study 6GR frame structure configuration/indication to allow BS-side semi-static SBFD, e.g.,

- RRC-configured link direction in SBFD symbols

- Subband-level TDD UL/DL configuration

###### [201r2] FL proposals:

- 6GR considers the following frame structure design,

  - Harmonized Unified frame structure to support TN and NTN.

- Study nessassity and feasibility of frame structure to support TDD NTN in NTN session

#### Company’s comment here

| Company | Comments |
| --- | --- |
| MTK | - [004r3]: Support. - [006r2]: Support. - [001r2]: We are fine with the observation and thanks for FL’s great efforts. For the channel model alignment, if companies have consensus to have this table, our suggestion is using some offline/online time to discuss each parameter one-by-one and check whether any simulation assumption is redundant/missing. At the same time, with the observation available, if further simulation is aligned, we can focus on scenarios where companies are having conflict observations, for example, we only need simulation alignment for medium/large delay spread. - [109r1]: We prefer version 2 while keeping D/U symbol only from NR, as the flexible symbol defined in NR may be simplified/optimized in 6GR. - [107r1]: For the name of “dynamic TDD”, it is a little bit confusing to us as 5G uses the same name with DCI indication. We suggest to change it into something like “TDD pattern adaptation faster than RRC reconfiguration”. - [108r1]: Fine. - [201r2]: Support. |
| Google | - [004r3]: Support. - [006r2]: OK but can we make it clear it is for network side, as we want to decouple the discussion for network and UE. It is a good starting point as it correctly captures the candidate bandwidths for further study and down-selection. We support keeping these options on the table. - [001r2]: OK - [109r1]: Support Version 2. We support defining new symbol/slot types for SBFD (and ISAC). - [107r1]: Support - [108r1]: Support. - [201r2]: Support |
| Tejas | - [006r2]: Support. - [006r3]: Support. (30 KHz SCS should also analyse other CP options) - [001r2]: We support simulation results to be provided by #124. For Simulation conditions we recommend following 11.2 till then companies can show benefits with minimal agreed simulation conditions like Channel Model : TDL-A with 30 , 100 & 300ns , UE Speeds up to 120 , waveform is CP-OFDM(till convergence on any new Waveform) , MCS (QPSK,16-QAM,256,QAM) , FEC should stay with LDPC, Support the other options & performance metrics recommended by Moderator. Proposed simplified simulation SLS setup Around 7GHz. Channel Model; FFS: TDL-x, CDL-x TDL-D TDL-A with medium correlation Delay Spread: 30ns, 100ns, 300ns. DL/UL; Company report. Carrier frequency; 7GHz. Antenna configuration; 4Tx4Rx,. Simulation Bandwidth; 50 MHz, 100MHz,. SCS; 30kHz, 60kHz. FFT size; Reported by companies. Cyclic Prefix; Normal. UE speed; 3/30/60/120km/h. Waveform; 5G NR CP-OFDM, other waveforms can be reported by companies.. MCS; 64QAM, 256QAM, 1024QAM FFS: code rate. FEC; 5G NR LDPC, Other coding scheme is not precluded and reported by companies.. Other assumptions; The followings are reported by companies, - DMRS or CSI-RS configuration, - Phase noise model, - Carrier frequency offset, - …. Performance Metrics; BLER, system overhead, throughput, other qualitative / quantitative results can be reported by companies. - [109r1]: We prefer version 2 - [107r1]: Support - [108r1]: Support. |
| ZTE | [004r3] FL Proposal OK [006r2] FL proposal For the first bullet, 200MHz should be removed. We think 3GPP should focus on the frequency ranges that have been confirmed for mobile communication. Thus, we think around 15GHz should be postponed until the usage is clear. However, if the other companies have strong view to discuss this right now, we would suggest making this frequency range more general to include both around 10GHz and around15GHz because 10-10.5GHz has been identified for IMT usage for some countries in region 2 in WRC-23. In addition, we suggest adding a note for this frequency range, which has been agreed in RAN plenary. The proposal is updated as below. - From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier for study is down-selected from the followings, - 800MHz, 400MHz or 200MHz for FR2 - 200MHz for Sub-6GHz - FFS: limit to 100MHz for FDD - 400MHz or 200MHz for around 7GHz - 400MHz or 200MHz for around 15GHz frequency range between upper bound of around 7GHz and FR2-1 (e.g. around 10GHz and around 15GHz) - Note: Carrier frequency of 15GHz has not been confirmed by ITU-R/WRC [006r3] FL proposal OK [001r2] FL Proposal For the simulation assumption, how do we understand the system overhead and/or evaluate it? [109r1] FL proposals: We don’t think the additional symbol type for energy saving is needed. The power saving can be achieved by many other solutions. The motivation of introducing such symbol type is not clear. Whether it is needed should be discussed in the energy efficiency session. [07r1] FL observation and suggestion We understand is the proposal is to just list some high-level principle for dynamic TDD. The third bullet is the specific solution, which is too early to discuss. We think it should be removed. The second bullet can be split into two bullets. And the last one should be the sub-bullet of this proposal. [108r1] FL observation and suggestion: The first bullet is not needed. This is not even supported in NR SBFD. We don’t see the need to consider this in 6GR. [201r2] FL proposals: For the last bullet, it seems we make a decision for NTN guys. We are not sure whether this is suitable. In addition, if the necessity of the TDD NTN is studied in NTN session, it is a bit early to have the first bullet. If we really need this, we suggest the following updates. - 6GR considers the following frame structure design, - Harmonized Unified frame structure design should also consider to support TN and NTN if TDD NTN is supported. - Note: The support of TDD NTN is discussed in NTN session. |
| FL4 | Added proposal[106] about frame structure configuration/indication which is missed by FL unintentionally. [006r2]->[006r3] to see whether we can have some level of consensus in SCS and max channel BW. |

## Wednesday Online

###### [004r3] FL Proposal

As a start point, 6GR study assumes all channels/signals (except PRACH) use the same SCS for a given band.

- Other options assume for different SCS between 6GR sync signal and other channels/signals (except PRACH) are not precluded to be discussed in initial access discussion.

- Note: ISAC is separately discussed in ISAC session.

###### [006r3] FL proposal

- From RAN1 perspective, maximum channel bandwidth and SCS per 6GR carrier around 7GHz is

- 400MHz

  - Note: a smaller maximum channel bandwidth supported by UE is allowed.

  - Note: the above doesn’t preclude to have multiple FFT operations within a carrier

- 30kHz SCS

###### [106] FL proposal

- For the study of 6GR frame structure configuration / indication

  - Support of cell and UE specific RRC configuration

    - FFS necessity to remove UE specific RRC configuration

  - Support of multiple TDD-pattern combinations, i.e., similar to NR

###### [201r2] FL proposals:

- 6GR considers the following frame structure design,

  - Harmonized Unified frame structure to support TN and NTN.

- Study necessity and feasibility of frame structure to support TDD NTN in NTN session

###### [109r2] FL proposals:

Version 1

6GR considers the following symbols/slots types,

- Support at least DL, UL and additional symbol type(s) for 6GR

- Study of the new additional symbol type(s) in addition to DL and UL to at least support

  - BS-side semi-static SBFD

  - Dynamic TDD

  - Reservation of resources

  - Energy saving, e.g., Sleeping

  - Other duplex types are not precluded if supported later

Version 2

6GR considers NR symbol/slot types as a starting point for the study item,

- Study further additional symbol type(s) to at least support

  - BS-side semi-static SBFD

  - Dynamic TDD

  - Reservation of resources

  - Energy saving, e.g., Sleeping

  - Other duplex types are not precluded if supported later

# Discussions

## Terminologies

Note: the following is used in this document,

MRSS		Multi-RAT Spectrum Sharing

SBFD		Sub-band Full Duplex

## SCS, Bandwidth and FFT size

### Discussion (before meeting)

Agreement from RAN#109

Agreement Regarding the RAN1 agreement below, the following is agreed: Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Agreement from RAN1#122

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

#### SCS/CP Around 7GHz

- Around 7GHz

During RAN1#122, 30kHz and 60kHz are agreed as options for candidate subcarrier spacing for further study. Among 30kHz and 60kHz,

- 30 kHz is the most widely supported option (Apple, AT&T, CMCC, CATT, China Telecom, Ericsson, Nokia, Huawei, MediaTek, Samsung, Xiaomi, OPPO, ZTE, FUTUREWEI, etc.).

- 60 kHz is FFS and supported by some (CMCC, KT, Qualcomm, OPPO), but others are cautious due to delay spread and ISI concerns (InterDigital, MediaTek).

Summary of evaluation results for SCS/CP

| Company | Results |
| --- | --- |
| Nokia | Observation 4: For most of the simulation cases 30 kHz and 60 kHz SCSs perform almost equally . Observation 5: 30 kHz SCS provides better performance for a combination of high delay spread and high MCS For 60 kHz SCS, performance improvements may be difficult to achieve, as the limiting factors are the CP length and the DMRS frequency-domain density. |
| ZTE | Observation 2: Based on numerical analysis, compared with 30kHz SCS, transmission with 60kHz SCS causes more ISI due to transmission power leakage. Proposal 5: Regarding SCS, for frequency bands around 7GHz, 30kHz SCS is adopted. Figure 3 PDSCH performance in DS=100ns Figure 4 PDSCH performance in DS=300ns |
| Spreadtrum, UNISOC | Proposal 1: Don’t support 16k FFT in 6GR day1. Proposal 2: Around 7GHz, support 200MHz as the max.CBW from UE side and 30kHz as the only SCS value. Proposal 3: Around 15GHz, the used numerology can be inherited directly from around 7GHz. Proposal 4: If 400MHz is supported around 7GHz from network perspective, only consider CA-implementation at UE side, e.g., 2*200MHz via 2 CCs, and this feature should be optional. Figure-1. Comparisons between 30kHz SCS and 60kHz SCS for BLER performance |
| Huawei, HiSilicon | Observation 1: According to link-level simulation results and the discussion above, we can observe that 30 kHz performs better than 60 kHz around 7 GHz. Figure 2. Initial simulation results of numerologies in 7GHz |
| OPPO | 30kHz is supported as the single SCS for 6GR baseline numerology in around 7GHz (i.e., 6.425GHz 8.4GHz) spectrum. Table 1: Simulation assumptions for 30kHz vs 60kHz SCS in 7GHz band Assumptions; Value. Carrier frequency; 7GHz. System Bandwidth; 20 MHz. Subcarrier spacing; 30kHz, 60kHz. Guard time interval; Normal CP. Data transmission bandwidth; 40PRBs for 30kHz; 20PRBs for 60kHz. Antenna configuration; 2T2R. MIMO mode; PMI based with 2Tx codebook. Rank per UE; Fixed rank=1. MCS; Fixed. MCS index = 7(QPSK) in Table 5.1.3.1-1 of 38.214. Channel estimation; Real. Channel Model; TDL-A with low correction with DS=30 ns. Speed; 3km/h. Figure 3: Performance comparison between 30kHz and 60kHz SCSs for 7GHz carrier frequency |
| Ericsson | For around 7GHz, support (only) 30kHz SCS. Many figures in the contributions R1-2507946 including - 30 km/h and 64QAM - 30 km/h and 64QAM - 3 km/h and 256QAM - 30 km/h and 256QAM - 3 km/h and 1024QAM - 30 km/h and 1024QAM |
| Interdigital | Observation 1: For UMi, UMa, and SMa scenarios at 7 GHz carrier frequency, 17% - 19% of the links showed lower than 30 dB SIR measuring inter-OFDM symbol interference. Observation 2: For UMi, UMa, and SMa scenarios at 7 GHz carrier frequency, preliminary analysis using system level evaluations shows the use of 60 kHz SCS has significant effect on ISI. Proposal 1: Further study impact of inter-symbol interference for 30 kHz and 60 kHz SCS usage at around 7 GHz frequency range with system level evaluations. Proposal 2: Use system level evaluations for investigations of appropriate SCS for frequency bands of interest. Figure 1. CDF of RMS delay spread and SIR of links for UMi, UMa, and SMa scenarios Table 1. Percentile of links with (Intersymbol) SIR <= 30 dB (for serving cell) SCS; Scenario; Percentile of links with SIR < 30dB. 30 kHz; UMi; 4.63. UMa; 7.52. SMa; 8.23. 60 kHz; UMi; 18.92. UMa; 19.04. SMa; 17.0. |
| Kyocera | Figure 1 BER performance for 200 MHz BW, 15 GHz, 150 km/h, QPSK |
| MediaTek | Observation 3: For 7GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread. Observation 4: For 15GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread. Multiple figures are provided in Tdoc R1-2507609 |
| Qualcomm | Observation 6. For 30KHz and 60KHz SCS comparisons, we have the following observation: - For small delay spread channel, there is no performance different between 30KHz and 60KHz SCS - For large delay spread channel (300ns rms delay), using legacy comb-2 DMRS with FD-OCC2, there is a performance degradation when using 60KHz due to lower frequency domain sampling density. However, this performance loss can be reduced by removing FD-OCC (using TD-OCC only instead) - If we use TD-OCC only, the DMRS overhead increases with the same number of symbols for PxSCH in time domain. However, if we use the same time duration (2x symbols in time domain for 60KHz), we will have the same DMRS overhead and there is no performance loss \ Figure 5. Performance of PDSCH with 10 symbol duration for 30KHz and 60KHz SCS with different DMRS patterns (CDL-C 300ns, rank 4, 10Hz Doppler) Figure 6. Performance of PDSCH with 10 symbol duration for 30KHz and 20 symbol duration for 60KHz with different DMRS patterns (CDL-C 300ns, rank 4, 10Hz Doppler) |
| DOCOMO | Observation 2: - For around 7 GHz, there seems to be no impact on phase noise. Observation 3: - For around 7 GHz, for a comparison between 30 kHz SCS and 60 kHz SCS, - In case of 64 QAM with R = 666/1024, - When low UE speed and DS = 100 ns / 300 ns, similar performance is observed. - When high UE speed and DS = 100 ns / 300 ns, 60 kHz SCS shows better performance in most SNR region, probably due to larger impact of inter-subcarrier inference. - For wider BW, the performance gap becomes larger. - In case of 256 QAM with 885/1024, - When DS = 100 ns, the same tendency as in case of 64 QAM is observed. - When DS = 300 ns, 60 kHz SCS shows significantly degraded performance, probably due to delay paths beyond CP length and low tolerance of 256 QAM. Proposal 2: - For around 7 GHz, RAN1 to discuss which scenario (which DS value) should more prioritized when down-selecting either 30 kHz SCS or 60 kHz SCS. |

###### [001] FL proposal

For the study of 6GR numerologies (including SCS, CP), the following simulation assumptions for studying of the 6GR numerologies are considered,

- Companies are encouraged to provide simulation results in RAN1#[123?/124?]

|  | Around 7GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Channel Model | FFS: TDL-x, CDL-x Delay Spread: {30 ns, 90/100 ns, 300 ns}? |  |  |  |
| Carrier frequency | 7GHz |  |  |  |
| Antenna configuration | FFS |  |  |  |
| Simulation Bandwidth | 100MHz, 200MHz, 400MHz Can we agree on 50MHz instead? |  |  |  |
| SCS | 30kHz, 60kHz |  |  |  |
| FFT size | Reported by companies |  |  |  |
| Cyclic Prefix | Normal |  |  |  |
| UE speed | 3/30/60/120[/500][/300][/100][/1000] km/h |  |  |  |
| Waveform | 5G NR CP-OFDM, other waveforms can be reported by companies. |  |  |  |
| MCS | [QPSK], [16QAM, ]64QAM, 256QAM, 1024QAM FFS: code rate |  |  |  |
| FEC | 5G NR LDPC, Other coding scheme is not precluded and reported by companies. |  |  |  |
| Other assumptions | The followings are reported by companies, - DMRS or CSI-RS configuration, - Phase noise model, - Carrier frequency offset, - … |  |  |  |
| Performance Metrics | BLER, system overhead, throughput, other qualitative / quantitative results can be reported by companies |  |  |  |

#### SCS/CP Around 15GHz

Around 15GHz

- 60 kHz is the most commonly proposed baseline (FUTUREWEI, KT, Qualcomm, Samsung, OPPO, ZTE, etc.).

- 30 kHz and 120 kHz are also FFS (AT&T, CMCC, ETRI, Nokia, OPPO).

Some companies noticed that the study for 15GHz can be postponed.

- Postpone discussion on SCS for around 15GHz until spectrum availability is clear, further analysis is made on delay spread in that range and 7 GHz should be concluded first prior (Apple P2, Nokia P6, CMCC P2)

###### [002] FL suggestion

Postpone discussion on SCS for around 15GHz.

###### [002r1] FL suggestion

Focus on 7GHz first before discuss SCS for around 15GHz.

#### SCS/CP sub-6GHz

The status quo is as follows:

- FDD Mode:

  - 15 kHz is widely supported as the baseline

  - 30 kHz for FDD is FFS in some bands like 1–2.5 GHz

  - 7.5 kHz is proposed for sub-1 GHz and NTN/IoT scenarios (Rakuten, CEWiT, Lenovo, TCL, Sharp).

- TDD Mode:

  - 30 kHz is the dominant proposal

RAN#109 discussed on whether 30kHz for FDD should be introduced and no conclusion is achieved.

###### [003] FL suggestion

Additional sub-6GHz SCS is not discussed in this RAN WG meeting. Continue discussion in RAN#110.

#### additional subcarrier spacing for SSB

- Most companies propose same SCS as data channels (Apple, CATT, Ericsson, Fujitsu, Huawei, Nokia, OPPO, Samsung, etc.). Reason includes system simplification in 6GR.

- 240 kHz SSB is FFS for FR2-1 (MediaTek). FFS whether to allow using additional subcarrier spacing for SSB (ATT)

###### [004] FL Proposal

As a start point, 6GR study assumes SSB and data use the same SCS.

- Other options are not precluded to be discussed in the future.

#### Additional CP type

Extended CP is not supported for general communication but may be considered for NTN or sensing (CATT, CEWiT, Lenovo, OPPO, LGE.)

Shorter CP: field and simulation observations (UMa/UMi) show that >80% of UEs experience RMS delay spread below 1 µs at 7–8 GHz, while 30 kHz CP = 2.33 µs, leaving a large unused CP margin (R1-2507054, section 2.6). This motivates a study on CP optimization for efficiency, without compromising the robustness of baseline 6GR numerologies. By keeping SCS as 30 kHz, we are ensuring that the density of DMRS is not affected and increases throughput. Propose to study the feasibility of flexible CP-based symbol extension for 30 kHz SCS around 7 GHz and other 6GR bands, without modifying the baseline of subframe duration of 1ms and frame duration of 10ms. (Tejas)

###### [005] FL suggestion

- Extended CP is not supported for general communication

- If RAN1 agree to study in the study item, for extend CP, focused on NTN or sensing cases.

###### [005A] FL suggestion

Study the feasibility of flexible CP-based symbol extension for 30 kHz SCS around 7 GHz and other 6GR bands.

#### Maximum BW and FFT size

According to the contributions,

The most controversial is around 7GHz, it is summarized as follows,

- 400MHz “single-wide”: (Qualcomm, Huawei, CMCC, KT,  CATT, Nokia, ZTE, Ericsson)

  - Arguments: One 400 MHz PHY carrier gives higher trunking gain, smaller guard-band, no inter-carrier MPR edges, lower PAPR, IMD Avoidance

  - Weakness: needs 16k FFT in UE BB; PA non-linearities for large BW; EVM budget for wideband operation ; ADC/DAC power increase; UE RF chain must cover 400 MHz instantaneous BW; ACP/SEM may need to be relaxed

  - If 30 kHz SCS → 16k FFT

  - If 60kHz SCS→ 8k FFT

- “Dual-200MHz”: (MediaTek, Spreadtrum/UNISOC, Xiaomi, Samsung (UE view), Apple, vivo, HONOR)

  - Arguments: Two 200 MHz CCs aggregated is enough; 400 MHz UE BB is over-kill

  - Weakness: 2 PLLs / 2 RF front-ends → cost & isolation; Scheduler must align PRB grids of both CCs for MRSS; spectrum lost in guard-band between CCs;

  - If 30 kHz SCS → 2×8k FFT (easy reuse)

  - each CC can be switched on/off independently;

For FR2,

- 800 MHz is the de-facto target for FR2-1 in all submitted proposals except KT (400 MHz), Ericsson(400Mhz), Xiaomi (400 MHz).

###### [006] FL proposal

- From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is

- [800MHz or 400MHz] for FR2

- 200MHz for Sub-6GHz

  - FFS: limit to 100MHz for FDD

- [400MHz or 200MHz] for around 7GHz

- FFS: around 15GHz

###### [007] FL proposal

- The maximum supported FFT size from RAN1 perspective for study is as follows,

  - 8192, 16384

  - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier.

###### [008] FL suggestion

FL propose companies to further clarification on

- BS and UE bandwidth,

- What’s the meaning of ‘FFT size’ and how to capture in the study.

#### Company’s comment here

| Company | Comments |
| --- | --- |
| DCM | [001] OK, but with 15 GHz as mentioned below. [002] Not accept. We are confused the FL suggestion as much more companies propose to discuss 30/60/120 kHz and only 3 companies propose to postpone discussion, but FL suggestion is postponing. This is really FL suggestion? Or CMCC’s preference? In our understanding, why “FFS” was added before “around 15 GHz” at the last RAN1 meeting is to follow the RAN plenary discussion status. Then the last RAN plenary agreed 15 GHz for 6G scenario. Why still is discussion blocked? BTW, our simulation results are not captured in the above list. [003] OK [004] OK [005] Is different CP length between TN and NTN really desirable? This may not be aligned with TN/NTN harmonization.. [006] 15 GHz should also be discussed as abovementioned. [400MHz or 200MHz] can be applied. [007] OK [008] Intention is unclear, especially for the first bullet. |
| TCL | [001] Support the proposal. [002] Not sure why postpone the discussion around 15KHz. [003] [004] Support the proposal. [005] Support the proposal. [006] Support the proposal. [007] Support the proposal. [008] Support the proposal. |
| Spreadtrum | [001]: Larger DS value > 300ns should be considered for evaluation, e.g., 500ns and/or 1000ns, which represents larger cell coverage. [002]: Support [003]: Support [004]: Support [005]: Support [006] [007] [008]: not OK In NR FR2, the max.CBW is 200MHz which can be viewed as a starting point for 6GR FR2 as FR2 is unchanged comparing with NR. Thus, we recommend adding 200MHz back for FR2, i.e., [800MHz or 400MHz or 200MHz] for FR2. From our perspective, the maximum channel bandwidth of BS and UE bandwidth should be decoupled and should be discussed separately. Current wording in [006] seems to mix the discussion, which is not the right suitable way to make progress. Besides, the parameters of CBW and FFT are highly correlated. It is better to combine [006] [007] [008] together for discussion. For around 7GHz, we think the most potential popular 6GR eMBB device with supporting 200MHz CBW should be studied with high priority firstly. After we have a clear picture on the capabilities (CBW, SCS, FFT size, etc.) of this most important 6GR eMBB device, then, we can discuss on how to support 400MHz bandwidth, e.g., by advanced implementation or other methods. |
| MTK | FL proposal 001 - For simulation bandwidth, we suggest to keep only 200MHz and 400MHz to maintain reasonable simulation efforts as 100MHz maximum bandwidth seems ruled out for around 7GHz. - For UE speed, we suggest to keep only 3/30/60/120 km/h to maintain reasonable simulation efforts. Higher UE speed can be optionally reported by companies. - For MCS, we suggest to keep only 64QAM, 256QAM, 1024QAM to maintain reasonable simulation efforts. - For performance metric, we suggest to keep only BLER. For throughput, it is calculated through BLER curve for LLS simulator and seems a little bit redundant to us. - As we are testing the feasibility for various settings, and 6G UE may target 6 or 8 layers, which should be ensured workable under the selected SCS. Hence, we suggest to add 6 or 8 MIMO layers (one of them should be enough) to the evaluation. FL suggestion 002/003 - Support FL proposal 004 - Basically fine. FL suggestion 005 - Support FL proposal 006 - Support, while we are not sure about the necessity of 800MHz for FR2. FL proposal 007 - Fine. FL suggestion 008 - For FFT size, if its accurate meaning is hard to be captured in the study, maybe we can just focus on maximum channel bandwidth and SCS, as these two parameters can already determine the FFT size. |
| LG Electronics | We appreciate FL for providing the summary. General Comment First of all, regarding SCS and CP length, LGE has submitted a proposal, but it was omitted in FL’s summary. We hope this can be remembered and included in future summaries. LGE prefers using 30kHz SCS around the 7GHz band, assuming normal CP is applied. This reflects the proposal we submitted. [001] FL Proposal Regarding the FL proposal: We are generally fine with the [001] FL proposal for simulation assumptions. (1) UE speed: In low-speed and wide coverage scenarios, 30kHz SCS around the 7GHz band is sufficient. However, in high-speed and wide coverage scenarios, 30kHz SCS around 7GHz may suffer from performance degradation due to ICI. If this configuration is to be selected despite the degradation, it is important to verify the extent of the performance loss through simulations. Therefore, including tests at 500, 300, and 1000 km/h is crucial. Additionally, it is necessary to assess how frequently high-speed and wide coverage scenarios actually occur. (2) Regarding Coverage, SCS and CP length: Initial simulation results clearly show that 60kHz SCS performs better at higher speeds, but suffers from ISI under longer delay spreads. As UE speed increases (e.g., beyond 120 km/h), it is evident that 30kHz SCS will experience further performance degradation due to ICI. In high-speed and long delay spread environments, both 30kHz and 60kHz SCS with normal CP are likely to underperform. This is precisely why extended CP was considered in NR and advocated in 6GR discussions. It was premature to conclude based solely on normal CP without any experimental or observational data. Hence, we proposed exploring other CP lengths. To provide wide coverage around 7GHz—comparable to what is achievable around 3GHz—it is essential to consider longer delay spreads and multi-TRP operation. If wide coverage is to be supported at 7GHz, 30kHz SCS with relatively long CP is indispensable. Alternatively, applying a CP length equivalent to that of 30kHz SCS to 60kHz SCS could be considered. Although this may reduce throughput due to increased overhead, it could offer better performance in high-speed scenarios. (3) Regarding maximum bandwidth: To support 400MHz around the 7GHz band, 30kHz SCS requires a 16k FFT, while 60kHz SCS can operate with an 8k FFT. Some companies are advocating for extending up to 8k FFT. Using 8k FFT with 30kHz SCS only supports up to 200MHz, and operating at 400MHz would introduce overhead, resulting in reduced throughput. [002] FL Proposal As the frequency range around 15GHz is already included in several deployment scenarios agreed in RANP, RAN1 can discuss which SCS will be used in that frequency range. [003] FL Proposal Fine with the [003] FL proposal. [004] FL Proposal Fine with the [004] FL proposal. [005] FL Proposal Generally fine with the [005] FL proposal. However, we suggest replacing the term "extend CP" with "longer CP" for clarity. [006] FL Proposal Generally fine with the [006] FL proposal. Question: Is the intention of the FL proposal to select one value among the two candidates as the maximum channel bandwidth per 6GR carrier? [007] FL Proposal Regarding the FFT size of the UE: In NR, the assumed FFT size was 4096, which is twice that of LTE. We believe that doubling the FFT size again for 6GR is a reasonable direction. However, increasing it by a factor of four may impose implementation burdens on the UE. An FFT size of 8192 is acceptable. As for 16384, we suggest discussing it more carefully. |
| HONOR | [001] OK. [002] OK. [003] Not sure whether RAN meeting will discuss it. Suggest to make it clear that 15kHz for FDD is supported and 30kHz and 7.5kHz for FDD is up to the RAN decision if any. [004] OK. [005] OK. [006] OK. [007] OK. [008] Not very clear about the proposal. Dose that mean the BS and UE can have different maximum CBW and FFT size? |
| InterDigital | [001] Suggest adding assumptions relevant for system-level evaluations as well. Link-level evaluations can show performance for specific values of DS (e.g. 300 ns) but do not provide any insight about the probability of the value being observed in practical deployments. In addition, results are sensitive on the assumed RS configuration especially for UE speed. The missing assumptions for system-level are: “deployment” channel model (e.g. UMi, UMa, etc.), Layout, Inter-site distance, BS assumptions (antenna height, antenna configuration, antenna pattern, port mapping, Tx power) and UE assumptions (antenna configuration, distribution, height). Performance metrics can include at least SIR from inter-symbol interference. [002] we are open to study 15 GHz. [003][004][005][006][007] ok |
| CATT | [001]: OK [002, 003, 004] Support [005] current wording is a ambiguous ( what is ‘if ran1 agrees to’ referring to ), it is suggested to change to : For extend CP, focused on NTN or sensing cases in the future respective study item. [006] [008] ： OK [007]: support |
| QC | [001] Ok. We think throughput is more appropriate KPI for the evaluation, since link-adaptation is applied in reality. In that case, instead of fixed MCS evaluation, we also need evaluation with link adaptation. [002] Ok [003] We think we do not need to discuss this proposal, as there is already a chairman’s guidance. [004] Ok [005] We suggest to reformulate the proposal as follows “For communication over TN, only normal CP is studied.” [006] For around 7GHz, we believe support of maximum channel bandwidth of 400MHz per 6GR carrier is important. Given there would be wider (> 200MHz) contiguous spectrum availability for some operators around 7GHz, there is no point to limit the carrier bandwidth to be 200MHz. As we pointed out in our contribution, single wideband carrier offers various advantages (including efficiency, performance, complexity) over multiple carriers for the given contiguous wider spectrum. Therefore, we propose “[400MHz or 200MHz] for around 7GHz”. For sub-6GHz, we would like to hear operators’ views. [007] [008] If the FFT size in this proposal indicates “how many subcarriers can be accommodated within the carrier” as in the note of the proposal, it is not so sure whether this proposal is really necessary. The number can be simply derived from the maximum carrier bandwidth per proposal 006. |
| ZTE | [001] FL proposal We think the 300ns delay spread should also be included. For the bandwidth, we don’t think 50MHz is needed since 6GR should support a large bandwidth. For the UE speed, we don’t think 100km/h is needed since we already have 120km/h and there may be no difference between of them. The maximum UE speed can be 300km/h. For MCS, QPSK can be supported since it may be used by the cell-edge UE. For the performance, how to evaluate the system overhead? [002] FL proposal Support [003] FL proposal Support [005] FL proposal We can make the proposal clearer. For example, add a new bullet: Extended CP is not supported for general communication. [006] FL proposal For FR2, we think 800MHz is better considering the 120kHz SCS and 8k FFT size in the next proposal. For around 7GHz, we can keep the two candidates right now. [008] FL suggestion It seems that the meaning of FFT size has been clarified in [007] FL proposal. |
| Xiaomi | FL suggestion 002 - Support FL suggestion 003 - Support FL suggestion 004 - Support FL suggestion 005 - Support FL suggestion 006 - (1) We think the maximum channel BW for UE and BS should be decoupled and separately discussed. - (2) We are not sure 800MHz channel bandwidth for FR2 is necessary. - (3) For around 7GHz, we think 200MHz for 30kHz SCS and 8k FFT size should be prioritized. FL suggestion 007 - (1) We think 8k FFT size should be prioritized than 16k FFT size. FL suggestion 008 - UE maximum channel BW can be discussed firstly then BS side. - For FFT size, agree with MTK to just focus on maximum channel bandwidth and SCS. |
| Panasonic | [005] Support. [008] Instead of FFT size, the discussion should focus on the number of SCS. |
| OPPO | [001] In general OK. But the simulation assumption should be simplified as possible. We suggest to take one BW, e.g., 100MHz or 50MHz, for simulation. And we do not support to take throughput for SCS simulation. It is not necessary. [002] Support. [003] Support. [004] Support. [005] Support the first bullet. But for the second bullet, “longer CP” should be used instead of “extended CP” - Extended CP is not supported for general communication - If RAN1 agree to study in the study item, for extend longer CP, focused on NTN or sensing cases. [006] Generally fine. But we prefer to postpone the discussion after SCS is decided. [007] We do not support the FFT size of 16K. [008] Support. |
| vivo | [001] FL Proposal – these assumptions listed in the proposal could be starting point. However, some parameters still have multiple options, which may lead to increased simulation work. So, further discussion or downselection on the parameters may be needed, e.g., UE speed, MCS... For antenna configuration, there is discussion about antenna model in AI 11.2. It is better to wait for the progress of antenna model discussion in the AI 11.2. [004] FL Proposal – we are fine. [006] FL Proposal -- We suggest to make it clear in the proposal that the maximum channel bandwidth is from BS or from UE perspective, or both. [007] FL Proposal – Ok (if this also should be discussed separately for BS and UE?) [008] FL Proposal -- Ok |
| Futurewei | - OK - OK - OK - OK - We prefer to retain only the normal CP in this AI, the NTN CP can be left to the AI NTN - OK in principle - OK - OK |
| Lenovo | [01]: Large DS value like 1000ns should be evaluated for IoT coverage extension. [02]: OK [03]: OK [04]: OK [005]: OK [006]: OK [007]: OK [008]: OK |
| Nokia | [001] Evaluation assumptions: - Antenna configuration: we think that 4RX to be appropriate The number of required TX antennas may be depending on potential higher-rank MIMO evaluations. - Bandwidth: we think that a single BW, such as 100MHz could be sufficient. There is no added benefit for the SCS investigations to run performance evaluations for different CBWs. Moreover, we think that 100MHz is a good trade-off between simulation load and still having sufficiently wide carrier evaluated. - Delay spread: we think that up to 300ns should be evaluated. - Waveform: we are fine to focus on CP-OFDM and are open to investigate DFT-s-OFDM for UL evaluations. NR waveforms are baseline for 6GR, as described in the SID. Hence, for SCS/numerology evaluations we should use only those waveforms until any alternative waveform is agreed on in the dedicated waveform AI. - MCS: we think that only NR supported combinations of modulation and code rates should be evaluated (to allow for alignment, NR MCS table and index to be reported). We further think that a reasonable subset of MCS could be selected by the group. - FEC: 5G NR LDPC is the baseline scheme for data FEC in 6GR, as described in the SID, hence we should only use 5G NR LDPC for SCS/numerology related investigations until any other coding scheme is agreed on in the dedicated coding AI. - Channel estimation assumption: should be reported, realistic ChEst should be mandatory - Link direction and TDRA: we think that PDSCH for CP-OFDM (.. and PUSCH with DFT-s-OFDM) could be evaluated. But the focus could be on e.g. 12 symbols PDSCH allocation (allowing for 2 symbol PDCCH/CORESET) with CP-OFDM. - DM-RS assumptions: it would be good to align (or at least report) also the number of DM-RS symbols that may affect the performance for different speeds. [002] OK in principle. But it would be good to clarify when (or if) the around 15GHz is to be investigated. [003] OK [004] OK [005] ECP: We think the NTN and ISAC specific discussion should be handled in the NTN AND ISAC specific AIs. At least for NTN we don’t see a need for the support of ECP. ECP for ISAC is for further discussion. [006] max. BW - For FR2-1, we think that 800MHz could be supported. - OK with sub-6GHz - for around 7GHz we suggest supporting 400MHz irrespective of the SCS to be chosen [007] FFT size we think that support of 8k FFT should be the baseline. For 30kHz SCS for around 7GHz, we think that 16k FFT could be supported to enable up to 400MHz channel bandwidths. [008] FFT vs. UE and BS BW On the BW: we think there is a need to distinguish between channel bandwidth (CBW) and ‘UE channel bandwidth’ (UE CBW). When e.g. discussing the minimum ‘channel bandwidth’, the operator (and therefore the gNB) may need to support e.g. 3MHz channel bandwidth for specific sync raster points of specific bands. But from UE perspective, the UE should support for other bands (and sync raster points) a larger channel bandwidth (SCS dependent - e.g. at least 5MHz for 15kHz FDD). Bandwidth versus FFT: we think that we could agree on the supported max. FFT size, which would give some information about the max. number of subcarriers or RBs that would be supported. How the UE would then implement the required operation to support such number of subcarriers or RBs is still an implementation specific decision (using a single FFT or multiple smaller FFTs). Although RAN1 agrees on the intended FFT size, this would not need to show up in the specifications – but just show up in the channel bandwidth options with the associated number of RBs. Of course, RAN4 would need to confirm the feasibility of the considered FFT/SCS/CBW combinations. |
| Ofinno | [001] Support. [002] Similar view with Docomo. We should keep RAN plenary decision. [003] Support. [004] Support. [005] Based on the agreement in the previous meeting, extended CP could be one of the other CPs. Is the intention of the proposal to remove whole FFS bullet point by clarifying that other CP in the agreement is extended CP in NR? If so, it would be helpful to clarify that. [006] Clarification on what the maximum channel bandwidth per 6GR is needed. Considering supports of the Dual-200 Mhz, maximum channel bandwidth in that case may refer non-contiguous frequency bandwidth and the aggregated bandwidth is 400 Mhz. [007] Support. [008] Support. |
| Samsung | Pls find our views for following proposals/suggestions: ###### [003] FL suggestion Additional sub-6GHz SCS is not discussed in this RAN WG meeting. Continue discussion in RAN#110. [SS]: remove “Continue discussion in RAN#110.” And since RAN P did not conclude to continue FFS, we think it’s not to pursue this additional sub-6GHz SCS. Further revise to “Additional sub-6GHz SCS is not discussed in RAN1 in Q4, 2025” This is based on RAN1 chair’s guideline in R1-2506701 as following: - RAN1 will suspend the discussion on second numerology for FDD frequency, i.e., RAN1 will not discuss the two “FFS” of “for sub 6GHz” in Q4, 2025. ###### [005] FL suggestion For extend CP, focused on NTN or sensing cases if RAN1 agree to study in the study item. [SS]: for different/extended CP, it’s no need to have another proposal for NTN, and sensing, it will be up to further discussion in NTN and sensing session. ###### [006] FL proposal - From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is - [800MHz or 400MHz] for FR2 - 200MHz for Sub-6GHz - FFS: limit to 100MHz for FDD - [400MHz or 200MHz] for around 7GHz - FFS: around 15GHz [SS]: we are general OK the proposal for further down selection later. ###### [007] FL proposal - The maximum supported FFT size from RAN1 perspective for study is as follows, - 8192 as starting point, study necessity of 16384 - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier. [SS]: we think it’s better to take 8192 as a starting point. |
| FL | ###### [001r1] FL proposal WF1: Draft text based on the submitted results/analysis. RAN1 will not discuss a common simulation assumption. Companies reported by own. WF2: For the study of 6GR numerologies (including SCS, CP), the following simulation assumptions for studying of the 6GR numerologies are considered, - Companies are encouraged to provide simulation results in RAN1#[123?/124?] - [Add system level simulation] - Rx / MIMO layer Around 7GHz. Channel Model; FFS: TDL-x, CDL-x Delay Spread: {30 ns, 90/100 ns, 300 ns}?. Carrier frequency; 7GHz. Antenna configuration; FFS. Simulation Bandwidth; 100MHz, 200MHz, 400MHz Can we agree on 50MHz instead?. SCS; 30kHz, 60kHz. FFT size; Reported by companies. Cyclic Prefix; Normal. UE speed; 3/30/60/120[/500][/300][/100][/1000] km/h. Waveform; 5G NR CP-OFDM, other waveforms can be reported by companies.. MCS; [QPSK], [16QAM, ]64QAM, 256QAM, 1024QAM FFS: code rate. FEC; 5G NR LDPC, Other coding scheme is not precluded and reported by companies.. Other assumptions; The followings are reported by companies, - DMRS or CSI-RS configuration, - Phase noise model, - Carrier frequency offset, - …. Performance Metrics; BLER, system overhead, throughput, other qualitative / quantitative results can be reported by companies. [002] From FL perspective, although there were some companies proposed SCS for 15GHz, but some are based on the results based on 7GHz. The discussion 15GHz can still be continued, but we make clear proposal when 7GHz SCS is more clear. [005] ###### [005r1] FL suggestion - Extended CP is not supported for general communication - If RAN1 agree to study in the study item, for extend CP, focused on NTN or sensing cases. ###### [006r1] FL proposal - From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is - [800MHz or 400MHz or 200MHz] for FR2 - 200MHz for Sub-6GHz - FFS: limit to 100MHz for FDD - [400MHz or 200MHz] for around 7GHz - [400MHz or 200MHz] for around 15GHz ###### [007r1] FL proposal - The maximum supported FFT size from RAN1 perspective for study is as follows, - 8192, 16384 - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier. |

### Discussion (after Monday online)

###### [001r2] FL Proposal

Observation for around 7GHz:

30kHz and 60kHz SCS have been studied. Based on the preliminary evaluations from companies’ contributions (Apple [1], DOCOMO [2], Ericsson [3], Huawei [4], Lenovo [5], MediaTek [6], Nokia [7], OPPO [8], Spreadtrum [9], ZTE [10], Kyocera[11], Interdigital[12], Qualcomm[13]), the followings are observed:

- With large delay spread (e.g., 300ns),

  - 30 kHz performs better with marginal or significant (e.g., up to ~4dB [9]) SNR gain for all MCS.

  - [2, 3, 6] reported for high-order modulation (e.g., for 256QAM/1024QAM), 60kHz SCS cannot achieve 10% BLER target.

  - [13] observed that using legacy comb-2 DMRS with FD-OCC2, there is a performance degradation when using 60KHz due to lower frequency domain sampling density. However, this performance loss can be reduced by removing FD-OCC (using TD-OCC only instead). Hence 60kHz can achieve better throughput.

- With small delay spread (e.g., 30ns),

  - 60 kHz performs better with marginal or small (<1dB) SNR gain for all MCS.

  - [? Any impact from speed and high order modulation?]

- With medium delay spread (e.g., around 100ns),

  - 2 sources ([8, 10]) observers 30 kHz performs better with marginal or small (up to 1.2 dB) SNR gain for QPSK and 3km/h UE speed,

  - 2 sources ([2, 6]) observers 60 kHz performs better with marginal or significant (up to 2 dB) SNR gain for higher modulation layer (e.g., 64QAM/256QAM/1024QAM),

  - 1 source ([3]) observers either 30 kHz or 60kHz SCS performs better under different modulation layer and UE speed.

Proposals for simulation (will pick up a few cases if agreed)

For the study of 6GR numerologies (including SCS, CP), the following simulation assumptions for studying of the 6GR numerologies are considered,

- Companies are encouraged to provide simulation results in RAN1#[123?/124?]

- [System level simulation can be conducted if system performance is evaluated. ]

|  | Around 7GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Channel Model | FFS: TDL-x, CDL-x TDL-C TDL-D CDL-A TDL-A with medium correlation Delay Spread: {30 ns, 90/100 ns, 300 ns}? 30ns, 100ns, 300ns |  |  |  |
| DL/UL | Company report |  |  |  |
| Carrier frequency | 7GHz |  |  |  |
| Antenna configuration | 4Tx4Rx, 2Tx2Rx? Follow 11.2 outcome when available ? |  |  |  |
| Simulation Bandwidth | 100MHz, 200MHz, 400MHz Can we agree on 50MHz instead? |  |  |  |
| SCS | 30kHz, 60kHz |  |  |  |
| FFT size | Reported by companies |  |  |  |
| Cyclic Prefix | Normal |  |  |  |
| UE speed | 3/30/60/120[/500][/300][/100][/1000] km/h |  |  |  |
| Waveform | 5G NR CP-OFDM, other waveforms can be reported by companies. |  |  |  |
| MCS | [QPSK], [16QAM, ]64QAM, 256QAM, 1024QAM FFS: code rate |  |  |  |
| FEC | 5G NR LDPC, Other coding scheme is not precluded and reported by companies. |  |  |  |
| Other assumptions | The followings are reported by companies, - DMRS or CSI-RS configuration, - Phase noise model, - Carrier frequency offset, - … |  |  |  |
| Performance Metrics | BLER, system overhead, throughput, other qualitative / quantitative results can be reported by companies |  |  |  |

###### [004r2] FL Proposal

As a start point, 6GR study assumes all channels/signals (except PRACH) use the same SCS for a given band.

- Other options assume are not precluded to be discussed in initial access discussion.

- Note: ISAC is separate discussed in ISAC session.

###### [005] FL Proposal

- Extended CP is not supported for general communication

- If RAN1 agree to study in the study item, for extend CP, focused on NTN or sensing cases.

###### [006r1] FL proposal

- From RAN1 specification perspective, maximum channel bandwidth per 6GR carrier is

- [800MHz or 400MHz or 200MHz] for FR2

- 200MHz for Sub-6GHz

  - FFS: limit to 100MHz for FDD

- [400MHz or 200MHz] for around 7GHz

- [400MHz or 200MHz] for around 15GHz

###### [007r1] FL proposal

- The maximum supported FFT size from RAN1 perspective for study is as follows,

  - 8192, 16384

  - Note: The “FFT size” is related to how many subcarriers can be accommodated within the carrier.

The following is some suggestions that only for clarification and is not intended to be proposed in the online session

###### [002r1] FL suggestion

Focus on 7GHz first before discuss SCS for around 15GHz.

###### [003r1] FL suggestion

Additional sub-6GHz SCS is not discussed in this RAN WG meeting.

## Frame structure

### Discussion (before meeting)

#### Align with 5G NR Frame Structure

- Most companies (e.g., Nokia, Huawei, Samsung, Apple, Qualcomm, OPPO, Vivo, ZTE, MediaTek, Xiaomi) agree that:

  - 6G should reuse the 5G NR frame structure:

    - 10 ms frame

    - 1 ms subframe

    - 14 symbols per slot

    - PRB = 12 subcarriers

  - This simplifies MRSS and ensures time-frequency compatibility between 5G and 6G.

- Most companies think alignment in symbol boundaries, subcarrier spacing, and resource block definitions is essential for symbol-level scheduling, RE-level rate matching, efficient spectrum sharing

- Radio Frame / subframe

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms (Huawei, Ericsson, CMCC, Samsung, Nokia, vivo, DoCoMo, KT, FUTUREWEI, Spreadtrum, TCL, Transsion Holdings, Ofinno, InterDigital, Google, ITL, CEWiT)

    - Negative: OPPO (study the necessity of subframe concept)

    - Views: Panasonic (To clarify whether the alignment of symbol boundary among different numerologies are required or not related to 1ms boundary definition)

- Symbol length

  - Follows NR design as a start point (CMCC, InterDigital)

###### [101] FL proposal

- 6GR considers NR frame structure is used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12.

  - Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier.

  - Symbol length follows NR design as a start point, details FFS.

  - FFS: Maximum number of subcarriers per NR carrier from RAN1 specification perspective.

#### Slot concept

- Majority support retaining slot

Majority support retain the slot concept as a basic time-domain resource unit (i.e., A slot is defined as supporting 14 symbol for all numerologies), (e.g.,  Huawei, CATT, Qualcomm, Samsung, CMCC, Nokia, OPPO, DoCoMo, China Telecom, KT, FUTUREWEI, Spreadtrum, TCL, LG, Fujitsu, InterDigital, Panasonic, ETRI, Google, Sharp, CSCN, ITL, CEWiT) considering Slot is a well-established time unit in 5G for Resource allocation, HARQ timing, scheduling, hence aligns with current specifications and avoids unnecessary complexity.

- Some suggest flexibility

FFS the association between slot and timing unit for scheduling:

- FFS time unit for scheduling: Nokia, DoCoMo

- PDSCH and PUSCH transmissions are not constrained to a slot: InterDigital

- Support NR based solution:

  - mini-slot scheduling: CATT, OPPO (symbol-level resource allocation and configuration), China Telecom (symbol-level resource allocation and configuration), InterDigital (symbol level scheduling), Sony

  - multi-slot scheduling: CATT, ITL

- Possible enhancements:

  - Consider allowing PxSCH to cross slot boundaries: CMCC (further studied under the scheduling agenda. i.e., AI 11.9), Qualcomm (Both mini-slot PxSCH and long PxSCH are supported), vivo (retain slot definition), China Telecom, Tejas, MediaTek, Google

  - Time-domain scheduling with flexible location and duration: Huawei, vivo (no slot definition), China Telecom (allocation can start at any symbol within a slot)

  - A single PDSCH or PUSCH spans more than 14 OFDM symbols: Samsung (e.g., multiples of 14 symbols), vivo

- Negative: Apple (Do not support a floating frame structure with crossing the slot boundary for Day-1 6GR air interface)

- Other views:

  - Panasonic: Study whether/how to support longer slot lengths for 6GR

    - Option 2: Slot length is defined depending on frequency range, sub-range, or SCS.

    - Option 3: Slot length is cell-specifically configured.

    - Option 4: Slot length is UE-specifically configured.

  - Panasonic: Study whether/how to support mini-slot based scheduling.

###### [102] FL proposal

- A slot is defined as supporting 14 symbols for all numerologies.

  - FFS: longer CP case if supported

###### [103] FL proposal

- 6GR supports slot-based scheduling/transmission.

###### [104] FL proposal

- Study further for scheduling/transmission not across slot-boundary, e.g.,

  - Replace slot-based scheduling with symbol-based allocation for finer granularity to improve low-latency services

  - Time-domain scheduling with flexible location and duration within a slot

###### [105] FL proposal

- Study further for PDSCH/PUSCH scheduling/transmission across slot boundary, e.g.,

  - support PDSCH/PUSCH transmissions across slot boundaries / multiple of slots to improve resource efficiency.

#### Frame structure configuration/indication

- Cell-specific semi-static RRC configuration is baseline

  - Most companies (e.g., Nokia, Huawei, Samsung, Qualcomm, OPPO, Vivo, ZTE, MediaTek, Xiaomi, Apple, LG, CMCC, etc.) agree that:

  - Frame structure should be configured semi-statically via RRC.

  - Configuration should be cell-specific, applying to all UEs in a cell.

- Support for UE-specific/Group-specific RRC configuration

  - Some companies (e.g., Huawei, CATT, CEWiT, Ericsson, CMCC) suggest:

  - UE-specific RRC configuration can be supported in addition to cell-specific config.

  - Time-frequency resource reservation to user groups with specific requirements and prevent access of those resources by other user groups (Ericsson)

  - Negative: Qualcomm, Samsung (6G day-1), Nokia, DoCoMo (assess the need of UE-dedicated semi-static configuration)

- 6GR frame pattern time domain periodicity

  - Support multiple TDD-pattern combinations, i.e., similar to NR: CATT, CMCC, Indian Institute of Tech (M)

  - Down-select to a limited number of DL-UL configurations from those supported in 5G NR: vivo (FFS)

Meanwhile, there are some other general viewpoints,

- Nokia: The support of advanced duplexing schemes in 6G should not come at the cost of increased complexity for basic TDD operation.

- LG: RAN1 to support placing DL resources at the front part and UL resources at the rear part of the period of UL/DL resource configuration

###### [106] FL proposal

- For the study of 6GR frame structure configuration / indication

  - Support of cell and UE specific RRC configuration

    - FFS necessity to remove UE specific RRC configuration

  - Support of multiple TDD-pattern combinations, i.e., similar to NR

#### Dynamic TDD

Support of dynamic TDD

- Support: Huawei (support a few long term TDD configurations), CATT, CMCC, Qualcomm (A single DL/UL switching point within the TDD pattern, and only one of dynamic TDD and network side semi-static SBFD are supported at the same time in a cell), Nokia, vivo, Tejas, [Lenovo], Fujitsu, Ofinno, InterDigital, Panasonic

  - [Qualcomm] A maximum of one DL-UL switching point within a D to X to U TDD pattern

  - [Qualcomm] Dynamic TDD is useful for small cell hot spot use case and semi-static SBFD for macro cell UL extension use case.

  - [CMCC] dynamic TDD for fast traffic adaptation for various service requirements in 6G

- Negative: [Samsung (not prioritized)], Apple, [DoCoMo]

- Open: AT&T, [China Telecom (FFS)]

Dynamic Indication (DCI-based)

- Positive: Spreadtrum (simplified SFI table)

- Negative: CATT, CMCC (re-evaluates the necessity), Qualcomm (6G day-1), Huawei, Samsung (6G day-1), vivo, DoCoMo, LG (study on necessity of dynamic SFI), Fujitsu, CEWiT

- Open: HONOR (FFS)

How to do if dynamic SFI is not supported?

- [Huawei] One possibility is to avoid slot-level SFI while a few long term TDD configurations can be supported to match long-term statistical characteristics.

- Nokia: Frame Pattern Indicator (FPI), via L1/L2 signaling (e.g., group common DCI), to switch to a particular frame pattern from the pre-configured options

Other views:

- Simplify 6G slot format design and avoid over-optimization: Xiaomi

- Study how to support dynamic TDD through SBFD as opposed to separate features: NEC

###### [107] FL observation and suggestion:

- Considering many supports of dynamic TDD, FL propose to consider to study 6GR frame structure configuration/indication to allow dynamic TDD

- Dynamic Indication (DCI-based) is deprioritized for study due to complexity of UE implementation

- Companies proposed some restrictions/simplifications/alternatives to allow dynamic TDD for 6GR, e.g.,

  - Restriction of one DL-UL switching point within a D to ‘X’ to U TDD pattern, ‘X’ is flexible symbols/slots

  - Avoid slot-level SFI while a few long term TDD configurations can be supported to match long-term statistical characteristics

  - Frame Pattern Indicator (FPI) to switch to a particular frame pattern from the pre-configured options

#### BS-side semi-static SBFD

Support of semi-static SBFD@BS

- Support: Huawei, ZTE, CATT, CMCC (for all RRC states), Qualcomm (only one of dynamic TDD and network side semi-static SBFD are supported at the same time in a cell), Samsung (priority), Nokia, vivo, Xiaomi, China Telecom, KT, Tejas, FUTUREWEI, Spreadtrum, Lenovo, LG, MediaTek, Fujitsu, Ofinno, InterDigital, Panasonic, Google, Sharp, ITL

- Open: AT&T

Potential enhancement in 6G:

- Natively support link direction configuration/indication in time-frequency domain: CMCC, Ericsson (natively support flexible time-frequency configurations), OPPO (semi-static assignment for transmission direction for a 2-D time-frequency resource block), [LG]

- Subband-level TDD UL/DL configuration: Huawei, ZTE

  - [Huawei] Time-frequency domain configuration, e.g., subband-level TDD UL/DL configuration, shall be further considered.

  - [ZTE] Subband based frame structure configuration

- RRC-configured link direction in SBFD symbols: Apple, MediaTek (link direction should be provided to the UE, e.g., via RRC or MAC-CE (half-duplex UEs)), CEWiT

- CORESET overlapped with resources outside DL subband: CATT

  - [CATT] CORESET can be configured to overlap with resources outside DL subband, but PDCCH candidates outside the DL subband will not be detected.

- Dedicated slot formats for SBFD: TCL (Option 2)

- Support SBFD operation, independent of the TDD, leveraging UL sub-band resources for UL transmissions without requiring dedicated UL slots or symbols, e.g., SBFD only operation: Google

- Study how to support the changing of BW during semi-static SBFD configuration: NEC

- Other views:

  - RAN1 studies whether all UEs in 6G day 1 is SBFD-aware UEs: OPPO

  - The intermediate approach between the transparent and non-transparent approach of gNB semi-static SBFD for 6GR should be considered: Panasonic (For instance, it would be considered that SBFD subband locations are explicitly informed similar to the non-transparent approach, but the resource allocation handling is the same regardless of whether the resource is on SBFD subband, similar to the transparent approach)

###### [108] FL observation and suggestion:

- Considering many supports of BS-side semi-static SBFD, FL propose to consider to study 6GR frame structure configuration/indication to allow BS-side semi-static SBFD.

- Companies proposed detailed configuration/indications for BS-side semi-static SBFD, e.g.,

  - RRC-configured link direction in SBFD symbols

  - Subband-level TDD UL/DL configuration

- FFS symbol/slot type to support BS-side semi-static SBFD

#### Symbols/slots type

Besides DL and UL, the following symbol types are considered,

- A new symbol type for SBFD:

  - ZTE (DL & UL), CATT (duplex symbol), CMCC (time-frequency), Qualcomm, [China Telecom], KT, Spreadtrum, TCL (Option 1), Lenovo, Transsion Holdings, Fujitsu, Google, Indian Institute of Tech (M), CEWiT

- Flexible symbol:

  - Huawei, [ZTE], CATT, CMCC, Nokia (a key enabler for different duplexing schemes), China Telecom, Spreadtrum, Lenovo, LG, Transsion, Fujitsu, Indian Institute of Tech (M), CEWiT, FUTUREWEI

  - Negative: Samsung (reconsiders the necessity)

- New ‘guard’ or ‘reserved’ resource type:

  - Nokia (for the purposes of at least UE UL-DL transition periods, SBFD UL-DL subband separation, and gNB mono-static sensing)

  - Reserved symbols: Fujitsu

- ‘X’ symbol type configuration in addition to DL and UL symbols: Qualcomm

  - [Qualcomm]

    - X symbol represents the resources where the link direction is not determined semi-statically as DL or UL.

    - Only one of dynamic TDD and network side semi-static SBFD are supported at the same time in a cell

    - The X symbols are interpreted as SBFD symbols in the presence of the UL/DL SBFD configurations.

###### [109] FL proposals:

6GR considers the following symbols/slots types,

- Support at least DL, UL for 6GR

- Study of new type(s) in addition to DL and UL to support

  - BS-side semi-static SBFD

  - Dynamic TDD

  - Reservation of resources

#### Other duplex

Dynamic SBFD

- Positive: Huawei, CATT, vivo, KT (FFS), [Lenovo], LG, Ofinno, InterDigital, ETRI, Google

  - Dynamically switching SBFD symbols to full downlink symbols: CATT, KT, Ofinno, ETRI, Google

  - Dynamic adjustments to SBFD sub-band sizes or positions:

    - Support: Huawei, Ofinno, ETRI, Google

      - [Huawei] The uplink subband width can be adjusted. This adjustment can be made through DCI or MAC CE mechanisms, allowing the network to configure the uplink subband width according to the specific uplink requirements and minimize downlink loss.

    - Negative: CATT

      - [CATT] For dynamic SBFD, focus on simpler implementation schemes, such as dynamically switching SBFD symbols to full downlink symbols, while avoiding complex dynamic adjustments to SBFD sub-band sizes or positions.

  - Dynamic indication of transmission direction of UL/DL SBFD sub-bands: Google

- Negative: Nokia (Deprioritize dynamic SBFD from the current Rel-20. If dynamic SBFD is to be supported, the feature should be limited to allowing the network to turn on and off the SBFD symbols (e.g. fallback to legacy DL symbols) without need for RRC reconfiguration), Xiaomi, DoCoMo

- Open: [OPPO (If dynamic SBFD is to be studied, unified design (if can be achieved) for dynamic TDD and dynamic SBFD is preferred)]

UE side SBFD

- Positive: MediaTek, NEC, Ofinno, ETRI

- Open: [OPPO], [Lenovo (Day-1 6GR duplexing should be designed with forward compatibility to accommodate future advanced duplexing in the future)]

- Negative: Nokia, vivo (via SLS: UE-side SBFD shows marginal improvement for eMBB traffic on top of BS-side SBFD), Xiaomi, DoCoMo

- Other views:

  - FUTUREWEI: Further investigate the impact of duplex solutions such as SBFD and SSFD at UE on the frame structure design.

gNB Full Duplex

- Positive: NEC, [Indian Institute of Tech (M)]

- Open: [Lenovo (Day-1 6GR duplexing should be designed with forward compatibility to accommodate future advanced duplexing in the future)]

- Negative: Nokia, vivo, Xiaomi, DoCoMo

###### [110] FL suggestion:

Considering less interests and issues brought up by some other companies, FL suggests no conclusion on UE side SBFD and gNB Full Duplex this meeting. Companies can continue discuss offline.

#### MRSS

In 6G SID, migration from 5G NR to 6GR is considered, especially 5G-6G Multi-RAT Spectrum Sharing for migration is considered as a task for study.

- Several companies mention that in order to achieve MRSS, certain restrictions might be needed, such as, same SCS between 5G-6G, frame structure boundary, switching point, etc.  (xiaomi, InterDigital, Spreadtrum, CMCC, ZTE, CATT, Fujitsu)

- 6GR study of solutions for 5G-6G Multi-RAT Spectrum Sharing (MRSS) can be postponed after numerology and frame design are stable (Futurewei)

###### [111] FL proposal:

- 6GR studies for 5G-6G Multi-RAT Spectrum Sharing (MRSS) assumes

  - 5G-6G aligned numerology, e.g., common/compatible SCS, aligned symbols,

  - 5G-6G aligned TDD grid, e.g., aligned TDD switch points,

#### Collision handling

[Nokia] Consider the following enhancements for collision handling in 6GR:

- Unified design for different half-duplex scenarios: HD-FDD, HD-SBFD and HD-CA

- Simplifying/reducing the number of rules for link direction determination in flexible symbols

- For advanced duplexing (e.g. SBFD at gNB), improve the handling (or remove) some of the error cases present in NR, e.g. overlapping of (dynamic or semi-static) UL and DL channels/signals and insufficient time for UE UL-DL switching.

#### Company’s comment here

| Company | Comments |
| --- | --- |
| DCM | [101] OK [102] OK [103] OK [104] the main bullet is OK, but the first sub-bullet is unclear. Although we have not discussed anything on scheduling for 6GR, why is “replace” used? [105] OK [106] UE specific RRC configuration should be FFS, i.e., “UE specific RRC configuration” should be removed from the first bullet. [107] We do not prefer to continue discussion on dynamic TDD. At least DCI-based mechanism we suggest deprioritizing. [108] OK, and for “detailed configuration/indications for BS-side semi-static SBFD”, e.g., part should be removed; any option can be discussed for the next meeting. [109] “reservation of resources” is unclear for us. Whether to include ‘Dynamic TDD’ is up to [107] [110] OK [111] OK |
| TCL | [101] Support the proposal. [102] Support the proposal. [103] Support the proposal. [104] We do not see a clear justification for restricting cross-slot scheduling. In our perspective, such a restriction would limit flexibility and the application of nominal and actual durations, particularly in scenarios involving multiple PDSCH/PUSCH transmissions. Therefore, we do not support this proposal. [105] Support the proposal. [106] Support the proposal. [107] Support the proposal. [108] We generally support the proposal. However, the sub-bullet regarding “RRC-configured link direction in SBFD symbols” is unclear to us. If this sub-bullet intends to specify the link direction in addition to the DL/UL subband indication, we believe it is unnecessary, as the link direction can be determined based on gNB scheduling. [109] Support the proposal. [110] Support the proposal. UE side SBFD has no clear motivation. [111] Support the proposal. |
| Spreadtrum | [101]:2nd bullet, PRB can change into RB to aligned with NR, and further it can include PRB and CRB if agreed. Last FFS can removed, due to it is covered by other proposals. [103]:in NR we support slot based and mini-slot based, so mapping type A and mapping type B were defined, with two set of DMRS locations. To be honest, do we really need those distinguishes? Slot based, mini-slot based or multi-slot based? What will impact those terminology have, always within a slot with fixed loaded DMRS, indicated in TDRA or other aspects. In short, we suggest to say scheduling/transmission within a slot. ###### [104]: We do not understand, scheduling in two sub-bullets are all supported by NR, they are legacy. What is the intention of the proposal? [105]: These two mechanisms can be further studied; we suggest to list them separately. - One occasion across slot boundary. - Multiple occasions in multiple slots, but each occasion in one slot. [106]: We suggest to agree cell specific RRC configuration. And left UE specific RRC for further study. Since the main function of UE specific RRC is for a UE can have its own configuration according to its traffic, and TA differences. According to TA differences, it may be configured as symbol type such as reserved symbol. For traffic specific configuration, it related with dynamic TDD, which can provide fast adjustment. So anyway, cell common RRC configuration should support. For multiple TDD-pattern combinations, it also can be further studied. It is too early for it, before enough discussion on period and form of cell specific RRC configuration. [109]: We support the first bullet. And we are also open for new type of symbol. However, why dynamic TDD is a new type of symbol? |
| MTK | FL proposal 101 - Support FL proposal 102 - Fine FL proposal 103 - Although we prefer to discuss the support of PXSCH across slot boundary first, but can be fine with it if majority companies want to have it. Suggest to add “at least” in the proposal. FL proposal 104 - Fine FL proposal 105 - Support FL proposal 106 - Support the first sub-bullet. For the second sub-bullet, we do not support it and we think RAN1 should try to reduce the number of different TDD configurations compared to NR (more similar to LTE) as the flexibility is over-optimized in NR and only a few TDD configurations are really used in field. FL proposal 107 - We do not think dynamic TDD is really showing its benefits in 5G NR. If a clear majority of companies still want to study it, we can be fine while also adding a sub-bullet saying: “study extended application time for dynamic TDD adaptation”. FL proposal 108 - Generally fine. We also support the study of UE-side SBFD. FL proposal 109 - Generally fine, while for dynamic TDD we should keep the restrictions mentioned in FL proposal 107 in mind. FL proposal 110 - We support the study of UE-side SBFD. FL proposal 111 - Support. |
| LG Electronics | We appreciate FL for providing the summary. [101] FL Proposal General Comment Similar to previous cases, LGE has submitted a proposal regarding the frame structure, but it was not reflected in the FL summary. We hope such omissions do not happen again in future summaries. FL Proposal – Specific Comment We are generally fine with the FL proposal. We believe the NR frame structure is a good starting point for the 6GR study. Further enhancements and modifications can be considered. Please add the following sentence: - Other enhancements are not precluded for further studies. One clarification: The alignment should be on symbol boundary every 0.5ms in the carrier, not 1ms. [102] FL proposal We are generally fine with having a slot definition with 14 symbols for 6GR. We would like to clarify our understanding that study of slot definition with less than 14 symbols is allowed by the FFS bullet. ‘Numerologies’ seems ambiguous. To be clear the intention of the main bullet of [102] FL proposal and point out the normal CP case, we propose to modify as follow: ###### [102] FL proposal - A slot is defined as supporting 14 symbols for normal CP case and all subcarrier spacings all numerologies. - FFS: Number of OFDM symbols within a slot for longer CP case if supported [103] FL Proposal & [104] FL Proposal & [105] FL Proposal We think that it is too early to make decision on this issue and it is ambiguous how we define slot-based scheduling/transmission.Also we have several issues that should be handled with higher priority in frame structure perspective. So it would be better to discuss scheduling aspects in the agenda item which will be started next year. [107] FL Proposal We support that dynamic Indication (DCI-based) is deprioritized for study due to complexity of UE implementation. However, the second main bullet (Dynamic Indication (DCI-based) …) and the last sub-bullets (Frame Pattern Indicator (FPI) to…) seems to be complicit. Frame Pattern Indicator (FPI) may work based on simple DCI-based indication to switch to a particular frame pattern from the pre-configured options. Furthermore, for the third main bullet “companies proposed…” it seems not be necessary to contain the companies’ suggestion in proposal. It can be used only for discussion. [108] FL Proposal For “Subband-level TDD UL/DL configuration”, it could be generalized “time-frequency resource configuration” as companies mentioned. In NR, resource configuration is mainly designed to assign the time resource since the basic assumption is to use entire frequency band for TDD operation. For “FFS symbol/slot type to support BS-side semi-static SBFD”, symbol/slot type could be generalized such as “FFS resource type to support BS-side semi-static SBFD” aligned with time-frequency resource configuration to study 6GR frame structure. [109] FL Proposal Similar comment in [108]. Instead of symbol/slot type, resource type is the general description. Suggest as below: 6GR considers the following symbols/slots resource types, As for reservation of resources, we may need reserved resource for different reasons such as guard time resource for switching between DL and UL, switching between SBFD and non-SBFD, guard time resource between UL subband and DL subband, and/or resources exclusively reserved for a specific services. However, it is not clear whether it could be configured by cell-specific resource configuration like DL, UL and/or (flexible) resource. Suggest updating as “FFS Reservation of resources” [110] FL Proposal Dynamic SBFD is missing for further discussion in 3.3.1.7 (Other duplex). Considering more companies support to study dynamic SBFD than other advanced duplex schemes (UE side SBFD and gNB Full Duplex) for 6G duplex study, dynamic SBFD should be on the table for further discussion whether to include for 6G duplex study. [111] FL Proposal We are generally fine with the first bullet. For the second bullet, it should be discussed carefully including consideration of useful scenario of MRSS. Given that 5G cells utilize a DL-heavy TDD pattern, imposing TDD grid restrictions on 6G cells operating on the MRSS carrier may hinder the allocation of additional UL resources needed for coverage extension. |
| HONOR | [101] OK [102] OK [103] OK [104] OK to study enhanced scheduling but the sub-bullet should be removed. A question about whether scheduling should be discussed in the sub agenda? [105] OK to study and same question as above. [106] OK [107] Support dynamic TDD not using DCI/SFI based mechanism. [108] OK [109] Not clear of “reservation of resources”. Suggest to remove. [110] OK [111] OK |
| InterDigital | [101] ok but curious about symbol length, what are the “details” to be discussed? [102] ok [103][104] a bit unclear on what “slot-based scheduling” means? Should we say “6GR supports at least scheduling every 1 slot”? [105] ok [106] ok [107] ok to allow dynamic TDD [108] ok to allow BS-side semi-static SBFD, with details FFS [109] ok [110] agree there is no need to conclude in this meeting. Open to study UE-side SBFD for feasibility and benefits. |
| CATT | [101] OK [102] Not support. The proposal seems to suggest that some type of CP is supported. But what exactly is supported here? We suggest to change the proposal to : A slot is defined as supporting 14 symbols for normal CP. [103] [104]: change: slot-based scheduling and symbol-based allocation for finer granularity to improve low-latency services [105][106]:ok [107] [108] [109] [110] support. [111] ok |
| QC | [103] Actually not clear what exactly is slot-based scheduling/transmission. Is this about PxSCH exactly 14-symbols long? Or the FL is talking about “6GR supports PxSCH not crossing slot boundary”. If it is the later case, what is different from [104]? [104] & [105] proposals should be merged ? [106] If cell-specific configuration is the common ground and there are different opinions on UE-specific configuration, we believe we should agree on cell-specific configuration first and FFS UE-specific configuration. [107] It is good to clarify in the second subbullet that “Dynamic Indication (DCI-based)” refers to GC-DCI format 2_0 (SFI) [108] We don’t see the need to spell out the examples of detailed configuration/indications for BS-side semi-static SBFD. This can be further discussed in [109]. - RRC-configured link direction in SBFD symbols - Subband-level TDD UL/DL configuration [109] Generally okay. In addition to DL and UL, we believe only one additional symbol type X is needed for SBFD and FL where symbol direction is determined by dynamic grant. No need to have specific reserved symbols. |
| ZTE | [101] FL proposal We understand the PRACH is not included for the fifth bullet. For the last bullet, we suggest changing ‘subcarriers’ to ‘RBs’. [105] FL proposal For transmission across slot boundary, maybe we can also consider control channel. [107] FL proposal First, the dynamic TDD has been agreed to consider in 6GR. For the second bullet, if DCI-based indication is deprioritized, how to achieve dynamic TDD. In addition, it seems that this bullet is the same as the last bullet. However, they have contradictory treatments, i.e., one is deprioritized and the other one is considered. [109] FL proposals We think the flexible symbol should be considered. |
| Xiaomi | FL suggestion 101 - Support FL suggestion 102 - Generally support. Suggest removing the FFS since it is not relevant to the main bullet. FL suggestion 103 - Generally support. Suggest add “at least” in the bullet to cover the possibility of mini-slot based scheduling FL suggestion 104 - Based on proposal 103, proposal 104 is not quite clear. In Proposal 103, slot-based scheduling should be not across slot boundary. Why is it mentioned for further study? FL suggestion 105 - Generally OK except removing “scheduling” in the main bullet because cross-slot scheduling has been supported in spec since 4G. - The intention of this proposal is further study cross-slot transmission not scheduling. FL suggestion 106 - We have concern on “UE-specific RRC configuration” and more justification is needed. - For the 2nd bullet, still concern on the necessity of symbol-level slot format. We need simplify NR frame structure mechanism. FL suggestion 107 - We are open to dynamic TDD and think how to simplify it should be discussed firstly. FL suggestion 108 - OK. FL suggestion 109 - OK. FL suggestion 110 - OK. FL suggestion 111 - OK. |
| Panasonic | [101] We are generally fine with the proposal. However, regarding the 5th sub-bullet (i.e.., "All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier"), as one SCS per carrier principle is under the discussion, the relation among different numerologies may not be required to be specified. Thus, we would like to clarify whether the alignment of symbol boundaries among different numerologies in the carrier is required or not. [102] Although we are OK with the fixed slot length with 14 symbols, if scheduling/transmission across slot boundary is needed, as a slot is used as a scheduling unit, it would be straightforward to support a longer slot rather than supporting cross-slot transmission. [103] Support. [106] We are fine to support both cell and UE specific RRC configuration. For PDSCH/PUSCH scheduling, UE specific RRC configuration can be used as the tool to allow "cell level dynamic adaptation, but UE is semi-static configuration" when SFI is not used. By scheduling different UEs, DL or UL are adjusted. [107] We support dynamic TDD. Regarding the dynamic indication, in NR, SFI is used to indicate enabling/disabling of periodically configured transmissions/receptions, such as PDCCH, CSI-RS, SPS, PRACH, PUCCH, and SRS. Before SFI is excluded, it should be studied how to indicate the direction indication for enabling/disabling of periodically configured transmissions/receptions. [109] Support. |
| OPPO | [101] We suggest to study the necessity of subframe in 6GR frame structure. Since a single SCS per FR/sub-FR is supported, the subframe is redundant since the fixed number of slots is in a subframe. [102] Support. [103] Support. [104] Support in general. [105] Support. [106] Support. [107] We think the semi-static TDD should be prioritized. We are open to study dynamic TDD. But no need to mention so many details in the proposal. [108] Support. [109] Support. [110] Support. [111] Support in general. |
| vivo | [101] FL Proposal – ok [102] FL Proposal – before agreeing the slot definition, we should discuss the necessity of slot definition and clarity what specific usage for slot. For example, clarify the slot concept here is used for resource allocation, timeline and/or it is kind of time scale？ - If it is just used for defining a time duration for resource allocation or timeline counting, then the starting and ending point for a slot is not necessary to be fixed. - If it is kind of time scale, then the starting and ending point for a slot needs to be fixed, as in NR. For resource allocation, cross-slot boundary should be allowed. It seems we define slot for 6GR at the beginning which may not be necessary since we later we may agree on the resource allocation to cross slot boundary. Therefore, before discussing the slot concept, its purpose should be clarified with the necessity in 6GR, whether symbol, symbol group can be used as resource allocation, timeline unit and subframe can be used as time scale (or half-subframe can be used as time scale for higher SCS if we consider subframe as time scale is too coarse for higher SCS) Regarding [103] FL Proposal, [104] FL Proposal, [105] FL Proposal, These proposals need to study together with [102] FL Proposal. Only supporting slot based scheduling/transmission will put unnecessary restriction on scheduling as NR, which is not preferable. We think scheduling granularities should be discussed first for different purposes (symbol level, symbol group level, multiple slots, etc.), then we can determine how to accommodate all the granularities in 6G frame structure. [106] FL Proposal – we are open to discuss UE-specific RRC configuration. However, we suggest to discuss what frame structure can be configured first, before agreeing how to indicate them. For the maximum number of TDD patterns, we think the maximum number can be limited for simplicity. [107] FL Proposal -- We are open to study potential enhancement for dynamic TDD, which is dynamic DL/UL resource allocation. In NR, if SFI or scheduling DCI indicates a transmission direction for a flexible symbol, the whole frequency bandwidth of the symbol will be treated as the indicated transmission direction. However, for UE, UE only needs to know the transmission direction for the allocated resources in the frequency bandwidth of the symbol. That is, outside the allocated resources, UE does not need to know how NW is allocating those resources. In this case, for UE, the behaviour is the same as dynamic TDD. For BS, it is up to BS to allocate different transmission direction on different frequency resources in the same symbol, which is SBFD, but transparent to UEs. UE only needs to know what frequency resources are to be used, and whether transmission or reception will be performed in the symbol, according to NW’s indication, such as scheduling DCI. We suggest to add a sub-bullet that “study dynamic determination of DL/UL resource by scheduling DCI” For Dynamic Indication (DCI-based) dynamic TDD, it is not clear. We think Dynamic Indication (DCI-based) means SFI based indication, not scheduling DCI based. In fact, dynamic TDD can be achieved by scheduling DCI, which is also DCI based. So, we suggest to clarify in the proposal that “non-scheduling DCI indication is deprioritized”. For “Frame Pattern Indicator (FPI) to switch to a particular frame pattern from the pre-configured options”, not clear to us how is it different from SFI and given there are significant concerns on SFI based approach why do we study such similar approach instead? [109] FL Proposal -- As comment of [107] FL Proposal, we suggest to change dynamic TDD to dynamic determination of DL/UL resource. Prefer to remove “new” and clarify it is symbol/slot type, for dynamic TDD or dynamic determination of DL/UL resource, traditional flexible symbol may be reused. 6GR considers the following symbols/slots types, - Support at least DL, UL for 6GR - Study of new symbol/slot type(s) in addition to DL and UL to support - BS-side semi-static SBFD - Dynamic determination of DL/UL resource - Reservation of resources [111] FL Proposal – there are a few things that need to be clarified. How to understand the “aligned numerology”? it is common or it is compatible SCS? How to understand the aligned TDD grid? It is early to make such assumptions. |
| Futurewei | 101 – OK 102 – OK 103 - OK 104 – OK 105 – OK 106 -OK 107 – OK for study 108 – OK, agree with QC that the sub-bullets not necessary 109 – OK 110 -OK to postpone |
| Lenovo | [101]: OK [102]: OK [103]: We think the details of scheduling should be left for other AI (scheduling/HARQ). As a starting point, slot-based scheduling/transmission can be supported. [104]: Details of scheduling not across slot-boundary should be left for other AI (scheduling/HARQ). [105]: Details of scheduling across slot-boundary should be left for other AI (scheduling/HARQ). [106]: UE specific RRC configuration should be FFS. It can be supported only if significant benefit is shown. “UE specific RRC configuration” should be removed from the first bullet at this time. [107]: OK [108]: OK [109]: OK [110]: OK [111]: OK |
| Huawei & HiSilicon 1 | Generally, we are fine with the proposals. However, we have the following proposed updates #### In 3.3.1.2 Slot concept section, we support mini-slot scheduling and multi-slot scheduling . We suggest the following changes in red ###### [103] FL proposal - 6GR supports NR slot-based scheduling/transmission as a baseline including - mini-slot scheduling - multi-slot scheduling - FFS: Considering possible enhancements e.g., - flexible location and duration, i.e., PxSCH can start at any symbol within a slot and finish in any symbol of another slot #### For 3.3.1.5 BS-side semi-static SBFD section, we propose small modification in red on companies observed details ###### [108] FL observation and suggestion: - Considering many supports of BS-side semi-static SBFD, FL propose to consider to study 6GR frame structure configuration/indication to allow BS-side semi-static SBFD. - Companies proposed detailed configuration/indications for BS-side semi-static SBFD, e.g., - RRC-configured /dynamic indication link direction in SBFD symbols - Subband-level TDD UL/DL configuration - FFS symbol/slot type to support BS-side semi-static SBFD #### For 3.3.1.6 Symbols/slots type section, new types could not be a new type and could be realized by NR like DL, UL, or flexible symbols/slots. ###### [109] FL proposals: 6GR considers the following symbols/slots types, - Support at least DL, UL and flexible for 6GR - Study of new type(s) in addition to DL and UL to weather/how to support the following type(s) - BS-side semi-static SBFD - Dynamic TDD - Reservation of resources - Energy saving types, e.g., Sleeping #### For 3.3.1.3 Frame structure configuration/indication section, we propose to add - For the study of 6GR frame structure configuration / indication - Support of cell and UE specific RRC configuration - FFS necessity to remove UE specific RRC configuration - Support of multiple TDD-pattern combinations, i.e., similar to NR - Study time-frequency domain configuration/indication, including single carrier (SBFD) and multi-carrier cases (flexible pairing of DL/UL) |
| Nokia | Proposal [101]: OK in principle On the PRB definition  RB definition: we think it would be more correct to talk about Resource Block (RB) and not about a physical resource block. Subframe alignment: just editorial - we should be talking about CP length and not CP overhead. Moreover, as we try to go for a single SCS per carrier, maybe the formulation there should not talk about carrier but just alignment of the symbol boundaries with the subframe boundaries. Moreover, we think that if we support 7.5kHz SCS we should still align with the subframe boundaries. i.e. o All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP length overhead, align on symbol boundaries every 1ms in the carrier. Proposal [102]: OK Proposal [103]: OK Proposal [104]: The proposal is not fully clear to us. NR supports shorter PDSCH/PUSCH resource allocations than the slot duration already. Moreover, we think that TDRA studies could be still done a later state and different AI (such as “scheduling”). Proposal [105]: We are fine to study PDSCH/PUSCH allocations to cross the slot boundary. But it is not clear to us what it means to “improve resource efficiency”. We think some more discussion is needed here, to set the focus of the related studies and agree on studying incl. the related target use case (incl. what to focus – coverage- and what not to focus – e.g. latency). Moreover as for [105], we think that TDRA studies could be still done a later state and different AI (such as “scheduling”). Proposal [106]: We do not consider a UE-specific RRC configuration necessary. We would appreciate clarification on the potential advantages of supporting such a configuration. Besides in the multiple TDD-pattern combination, we prefer to make it clearer by adding text stating that the patterns are concatenated one after another. Accordingly, we propose the following modifications to the FL proposal: [106] FL proposal - For the study of 6GR frame structure configuration / indication - Support of cell and UE specific RRC configuration - FFS necessity of to remove UE specific RRC configuration - Support of multiple concatenated TDD-pattern combinations, i.e., similar to NR Proposal [107]: We acknowledge the complexity of dynamic SFI indication and agree that the SFI mechanism should be replaced with a simpler solution. However, we do not see the need to deprioritize the dynamic indication itself. The main complexity lies in the SFI design rather than in the DCI-based indication. Therefore, we believe it is better to remain open to the dynamic indication and suggest the following: [107] FL observation and suggestion: - Considering many supports of dynamic TDD, FL propose to consider to study 6GR frame structure configuration/indication to allow dynamic TDD - Dynamic Indication (DCI-based) is deprioritized for study due to complexity of UE implementation - Companies proposed some restrictions/simplifications/alternatives to allow dynamic TDD for 6GR, e.g., - Restriction of one DL-UL switching point within a D to ‘X’ to U TDD pattern, ‘X’ is flexible symbols/slots - Avoid slot-level SFI while a few long term TDD configurations can be supported to match long-term statistical characteristics - Frame Pattern Indicator (FPI) to switch to a particular frame pattern from the pre-configured options Proposal [108]: Ok. Proposal [109]: The expression “new type(s)” is confusing. Does this imply that FL types are not included, since they were already supported in 5G NR and thus cannot be considered as new? We suggest replacing “new” with “additional” types. [109] FL proposals: 6GR considers the following symbols/slots types, - Support at least DL, UL for 6GR - Study of additional type(s) in addition to besides DL and UL to support - BS-side semi-static SBFD - Dynamic TDD - Reservation of resources Suggestion [110]: Support Proposal [111]: Support |
| Ofinno | [101] Support. [102] If the slot is defined as 14 symbols, and if the longer/extended CP is supported on top of that, we think the slot length of the longer/extended CP should be longer than that of normal CP. [103] Support the proposal and we think it is transmission and reception, right? [104] Considering the stage of the discussion, we prefer to start with main bullet without second bullets since it seems not clear enough. For example, what is the “replace slot-based scheduling” and “flexible location”? [105] Similar view with [104]. We can take first bullet and further discuss the details. [106] It seems too early stage to discuss the signalling details. Support to have multiple TDD-pattern combination. [107] similar view with [106]. It seems too early to discuss signalling details. Support dynamic TDD. [108] Support. [109] We think support of the flexible symbol should be considered too. Is the reservation of resources are resource those are not allowed to be used? [110] Support. [111] Support the first bullet, not for the second. If the TDD switch point is from UE’s point of view, it could be left for UE implementation and if it is gNB’s point of view, it seems too restrictive and may not be known. |
| Samsung | ###### [101] FL proposal - 6GR considers NR frame structure is used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12. - Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier. - Symbol length follows NR design as a start point, details FFS. - FFS: Maximum number of subcarriers per NR carrier from RAN1 specification perspective. [SS]: for last FFS, it’s may also involve RAN4 for determination. ###### [102] FL proposal - A slot is defined as supporting 14 symbols for all numerologies. - FFS: longer CP case if supported [SS]: the FFS is not needed, we did not see strong promotion to propose that. ###### [103] FL proposal - 6GR supports slot-based scheduling/transmission. ###### [104] FL proposal - Study further for scheduling/transmission not across slot-boundary, e.g., - Replace slot-based scheduling with symbol-based allocation for finer granularity to improve low-latency services - Time-domain scheduling with flexible location and duration within a slot ###### [105] FL proposal - Study further for scheduling/transmission across slot boundary, e.g., - support PDSCH/PUSCH transmissions across slot boundaries / with multiple of slots to improve resource efficiency. [SS]: first of all, this is scheduling design which is better to discuss in the future scheduling/harq session. Secondly, it seems the intention of the 104 is not so clear given 103 which says support slot based scheduling and 104 saying to replay slot based scheduling. Could FL clarify the intention? In addition, ###### [106] FL proposal - For the study of 6GR frame structure configuration / indication - Support of cell and UE specific RRC configuration - FFS necessity to remove include UE specific RRC configuration - FFS: Support of necessity to keep multiple TDD-pattern combinations, i.e., similar to NR [SS]: first of all, this is scheduling design which is better to discuss in the future scheduling/harq session. Secondly, clarify the cell and UE specific RRC configuration, if the RRC configuration is meant the TDD-UL/DL configuration, we don’t think the UE specific RRC configuration is needed. Based on the experience in 5G NR, such “flexibility” did not provide much actual gain in the deployment, most of the time the TDD-UL/DL configuration is based on the cell specific one. For multiple TDD pattern combination, further to clarify if the “multiple TDD-pattern combinations” means multiple TDD-patterns cascaded? If so, we think that’s another flexlibity in NR but not widely used, we would like to hear more justification. ###### [107] FL observation and suggestion: - Considering many supports of dynamic TDD, FL propose to consider to study 6GR frame structure configuration/indication to allow dynamic TDD - Dynamic Indication (DCI-based) is deprioritized for study due to complexity of UE implementation - Companies proposed some restrictions/simplifications/alternatives to allow dynamic TDD for 6GR, e.g., - Restriction of one DL-UL switching point within a D to ‘X’ to U TDD pattern, ‘X’ is flexible symbols/slots - Avoid slot-level SFI while a few long term TDD configurations can be supported to match long-term statistical characteristics - Frame Pattern Indicator (FPI) to switch to a particular frame pattern from the pre-configured options [SS]: We have different views, first, although dynamic TDD is mentioned by a few companies but the real reasons and evaluations are still not clear. Second, Dynamic indication should be replaced by “dynamic SFI”. Third, our concern is the applicable scenarios and its feasibility. As our tdoc discussed, Rel-16 CLI study clearly indicates it is infeasible for a certain scenario (e.g. FR1 Macro). Forth, in every releases, RAN1 faced long discussions about UE’s behaviors on Flexible symbols, even though the flexible symbols are only used for guard period. For 6G, we need to avoid such an unnecessary discussion and UE behaviors. ###### [108] FL observation and suggestion: - Considering many supports of BS-side semi-static SBFD, FL propose to consider to study 6GR frame structure configuration/indication to allow BS-side semi-static SBFD. - Companies proposed detailed configuration/indications for BS-side semi-static SBFD, e.g., - RRC-configured link direction in SBFD symbols - Subband-level TDD UL/DL configuration - FFS symbol/slot type to support BS-side semi-static SBFD [SS]: We think Rel-19 SBFD is a starting point. And, as we mention in [107], RAN1 needs to first figure out what is limitation on Rel-19 SBFD. Then, we can further consider what is updated from Rel-19 SBFD. To us, the Rel-19 SBFD configuration is not a big issue. ###### [111] FL proposal: - 6GR studies for 5G-6G Multi-RAT Spectrum Sharing (MRSS) assumes - 5G-6G aligned numerology, e.g., same common/compatible SCS, aligned symbols, - 5G-6G aligned TDD grid, e.g., aligned TDD switch points, [SS]: Suggest to replace “common/compatible” to “same”. |
| FL | [101] To LGE, I have checked the NR TR38.802, it says, ‘All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in NR carrier’. Actually here 1ms refers to subframe and should be OK. To Panasonic, NR has the 5th sub-bullet (i.e.., "All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier"). So that’s the reason why I think have this sentence should be here. We can discuss that during offline. ###### [101r1] FL proposal - 6GR considers NR frame structure is used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12. - Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms in the carrier. - Symbol length follows NR design as a start point, details FFS. - FFS: Maximum number of subcarriers/RBs per NR carrier from RAN1 specification perspective. [002] To Panasonic: to support longer scheduling unit, there could be two way: - Scheduling can across multiple slots - Scheduling unit is slot, but slot can be more than 14 symbols. So far, the proposal is based on understanding (1), but let’s discuss that in the offline discussion. ###### [102r1] FL proposal - A slot is defined as supporting 14 symbols for normal CP case and all subcarrier spacings all numerologies. - FFS: Number of OFDM symbols within a slot for longer CP case if supported [003] ###### [103r1] FL proposal - 6GR supports at least slot-based scheduling/transmission. [009] ###### [109] FL proposals: 6GR considers the following symbols/slots types, - Support at least DL, UL and at least one new symbol type for 6GR - Study of the new type(s) in addition to DL and UL to support - BS-side semi-static SBFD - Dynamic TDD - Reservation of resources - Energy saving |

### Discussion (after Monday online)

###### [101r2] FL proposal

- 6GR considers NR frame structure is used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Physical resource block (PRB) is defined where the number of subcarriers per PRB is the same for all numerologies and the number of subcarriers per PRB is 12.

  - Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - Each symbol within 1ms is aligned with NR design.

  - All numerologies with 15 kHz and larger subcarrier spacing, regardless of CP overhead, align on symbol boundaries every 1ms

  - Symbol length follows NR design as a start point, details FFS.

  - FFS: Maximum number of subcarriers/RBs per NR carrier from RAN1 specification perspective.

###### [102r1] FL proposal

- A slot is defined as supporting 14 symbols for normal CP case and all subcarrier spacings all numerologies.

  - FFS: Number of OFDM symbols within a slot for longer CP case if supported

## NTN related

### Discussion (before meeting)

NTN related:

- Consider TDD NTN (5): Lenovo, CATT, CMCC, vivo, CSCN

- TDD pattern period longer than 10ms (4): CATT (for both NTN and TN), CMCC (for TDD NTN), vivo (FFS periodicity larger than 20ms for NTN), CSCN

- Unified frame structure to support TN and NTN: Tejas, TCL

- Design NTN-aware duplex structures for long delays and half-duplex UEs: Ofinno

- UE specific TA adjustment: CEWiT

###### [201] FL proposals:

- 6GR considers the following frame structure design,

  - Unified frame structure to support TN and NTN.

  - TDD pattern period longer than 10ms in order to support NTN scenario.

#### Company’s comment here

| Company | Comments |
| --- | --- |
| DCM | We do not support TDD for NTN directly. Whether NTN scenario can/should support TDD should be discussed first. Especially, interference from NTN to TN could be a large problem; at least companies should be on the same page for this problem. |
| TCL | We support the proposal and advocate for a unified frame structure for TN and NTN to enable seamless connectivity. In addition, we recommend the use of TDD for NTN as mentioned in the proposal. However, considering the longer RTT associated with NTN, the pattern period for NTN can be extended compared to TN. To ensure alignment between the TN and NTN pattern period, the NTN pattern period could be set as an integer multiple of the TN pattern period. For instance, a baseline of 20 msec could be explored further in discussions. |
| Spreadtrum | This proposal can be postponed until there is an outcome whether 6GR support TDD NTN. |
| MTK | Generally support FL Proposal 201. Or we can discuss it later under NTN agenda. |
| LG Electronics | [201] FL proposals Even in NTN scenario, we can further study the dynamic TDD. To be specific, in case of LEO/MEO, due the satellite moving, the distance between UE and the satellite could be varying in time. In this case, the suitable gap between DL and UL would be changing as well. So, when we discuss the dynamic TDD for 6GR or discuss the TDD frame structure for NTN separately, we also need to care the NTN scenario. At this moment, we can add “dynamic TDD to support NTN scenario” in the [201] FL proposal. |
| HONOR | We support to design Unified frame structure to support TN and NTN and extend TDD pattern period longer than 10ms |
| CATT | We support the FL’s proposal. Especially TDD for NTN. This is very likely next step and there is already many interests for this. We should consider all operator’s requirement. |
| CSCN | Support. Due to the limited FDD spectrum, we cannot rule out that some TDD bands may be allocated to 6GR NTN. It's not necessary to exclude the posibility of extending TN TDD to NTN TDD in the 6GR study phase. Extending the TDD pattern period to longer than 10ms can better enable the support of NTN TDD in LEO system. Besides, NTN charactersitics should be incorperated in the TDD frame structure discussions, rather than seperately determined in NTN agenda item, to align with the unified design principle. |
| ZTE | For the second bullet, we think it should be discussed in the NTN session. We think it is difficult for us to determine the value without the basic information of NTN requirements. |
| Xiaomi | We agree with first bullet and suggest discussing the 2nd bullet in NTN agenda |
| OPPO | OK in general. |
| vivo | For “TDD pattern period longer than 10ms in order to support NTN scenario.” Our comments/questions are: - Is it the intention of the proposal that the consideration for TDD pattern period of >10ms only applicable to NTN, but not for TN? Would be good to give a upper limit (for the TDD pattern period) for consideration. |
| Futurewei | We are OK using a unified frame structure. |
| Lenovo | [201]: Whether to use TDD for NTN is subject to spectrum allocation. Given the limited FDD band available for NTN, it is important to support TDD. Considering the long RTT for NTN, it is necessary to support pattern period longer than 10ms. While it will be great to have a unified frame structure for TN and NTN, the feasibility is subject for further study. Suggest to change first sub-bullet to “ study unified frame structure to support TN and NTN”. |
| Huawei & HiSilicon 1 | ###### We think the second bullet is not needed at this stage and can wait more progress in NTN SI. ###### [201] FL proposals: - 6GR considers the following frame structure design, - Unified frame structure to support TN and NTN. - TDD pattern period longer than 10ms in order to support NTN scenario. |
| Nokia | Do not support Support only the first bullet – but not the second bullet. - we think that the first and second bullet are contradicting. There is no need to support longer TDD patterns than 10ms for TN operation – so then it would not be unified anymore. - we don’t see a need to support TDD NTN operation – and this discussion is happening in AI 11.1. So we think the related discussions & decision on the TDD support for NTN to happen there. In case only NTN FDD would be supported, again this be contradicting the first bullet as then TDD would only be applicable to TN. |
| Ofinno | Support the proposal but we are also not sure whether this is the right place to discuss. More importantly, whether to keep BWP in 6G or not should be discussed first from our understanding. |

### Discussion (after Monday online)

It seems most companies can agree on the first sub-bullet.

###### [201r1] FL proposals:

- 6GR considers the following frame structure design,

  - Harmonized Unified frame structure to support TN and NTN.

  - TDD pattern period longer than 10ms in order to support NTN scenario.

###### [201r2] FL proposals:

- 6GR considers the following frame structure design,

  - Harmonized Unified frame structure to support TN and NTN.

- Study nessassity and feasibility of frame structure to support TDD NTN in NTN session

## ISAC related

### Discussion (before meeting)

ISAC

- Strive to reuse the frame structure designed for communication for sensing (3): China Telecom, Tejas (unified frame structure to support ISAC), Lenovo (the unified frame structure must support coexistence of sensing and communication if they use different numerologies)

- Sensing resources reservation (3): Ericsson (User group based time-frequency resource reservation), CATT (transparent sensing resources reservation for non-sensing UEs), LG (configuration of a silent time gap before and/or after the sensing signal in a frame structure)

- Dedicated ISAC symbol or symbol structure (4): China Telecom (FFS), OPPO (symbol structure can be studied for sensing without changing symbol boundary), TCL (Option 1), Google

- Dedicated slot formats for ISAC: TCL (Option 2)

- Time domain resources for sensing RS: ZTE

  - TDM: ZTE

  - FDM: ZTE (subband based time domain resources for sensing RS)

###### [301] FL proposals:

- 6GR considers the following frame structure design,

#### Company’s comment here

| Company | Comments |
| --- | --- |
| DCM | Probably sub-bullets are missed. Before the discussion for each bullet, whether this proposal should be discussed in this agenda or in sensing agenda is unclear. |
| TCL | ISAC is expected to be a key service supported from day 1 in 6G deployment, and it may necessary to allocate the time and frequency resources for sensing in Frame structure. Therefore, the frame structure design should include the following: - Dedicated symbols/slots for sensing. |
| MTK | Probably sub-bullets are missing. We think it can be discussed under sensing agenda later. |
| LG Electronics | [301] FL proposals The coexistence between communication and sensing signal needs to be considered in 6GR frame structure design. The details of the sensing signal structure were not decided yet, but 6GR frame structure should be flexible to support the inclusion of sensing signal. It should be flexible to support e.g. SCS, guard interval and multiplexing necessary for sensing signal transmission. |
| InterDigital | Prefer to defer this discussion to sensing agenda. |
| CATT | Sensing is supported via resource reservation. |
| Xiaomi | We think it can be discussed under ISAC agenda later. |
| OPPO | We suggest to have unified baseline frame structure for 6G Communication and 6G sensing. Sensing-specific FS design can be studied based on the symbol/slot/frame boundary in baseline frame structure. |
| Futurewei | We prefer to defer this discussion, or to be addressed in the sensing AI |
| Lenovo | Frame structure must support coexistence of sensing and communication. |

## Frequency domain related

### Discussion (before meeting)

BWP enhancement: [Huawei], CATT, LG, ETRI, Google, Hanbat National University, ITL

  - [Huawei] optimized BWP configuration, low switching delay, and enhanced robustness, etc.

  - [LG] e.g., in unpaired spectrum scenarios, the center frequencies of DL and UL BWPs may be either aligned or offset. Non-contiguous resource allocation may also be supported to support SBFD scenario or Multi-carrier Single Cell scenario.

[CATT] The following lessons are learned from 5G NR BWP framework:

- Too many physical layer parameters are per-BWP configured, which are redundant;

- Per-BWP configuration of physical layer parameters increases the overhead, complexity of UE implementation, and probably the latency of BWP switching;

- Multiple BWP adaptation options have been introduced but not all are useful in practical.

[CATT] Minimize unnecessary per-BWP configuration parameters and simplify BWP switching options.

- Reducing signaling overhead by avoiding redundant parameter configurations across BWPs.

- Simplifying UE complexity during BWP switching, so that latency and processing burden are minimized.

- Preserving the benefits of BWP (support for low-capability devices and energy savings) while ensuring efficient support of control and data channels.

[Hanbat National University]

- Simple operation, e.g., one initial BWP + optional dedicated BWP per UE/type

- Group/cell-level BWP switching

- Non-continuous and misaligned DL/UL BWPs

###### [401] FL proposals:

6GR studies Bandwidth Part (BWP) with further considerations focusing on following potential enhancements,

- Essential/relevant configurations under BWP configurations to enable

  - Faster BWP switching delay

  - Frequency region adaptation while minimizing UE complexity and signalling overhead.

- non-continuous frequency resource configuration

- Center frequencies of DL and UL BWPs should be aligned or allowed to differ to support more flexible duplexing schemes

- Other enhancements are not precluded for further studies.

#### Company’s comment here

| Company | Comments |
| --- | --- |
| DCM | Not support to discuss this in this AI. 11.1 is the right place based on discussion at the last meeting. |
| TCL | Support the proposal. |
| Spreadtrum | For BWP, it can start from 11.1 to have a clear picture of lessons in NR. And then decide whether a totally new BWP design or only some enhancement. Especially some contents overlaps with proposal in 11.1. |
| MTK | For FL Proposal 401, the directions seem good to us, but the same topic is under discussion in 6G energy efficiency agenda and overviews agenda, and we think it is more proper to discuss BWP there. |
| LG Electronics | [401] FL proposal We are generally fine with the proposal; however, we prefer to discuss the details of BWP operation under a dedicated agenda item. It would also be helpful to clarify which agenda item will address these issues to avoid duplicated discussions, as similar topics are already suggested under the overview and energy efficiency agenda. |
| HONOR | Support study BWP enhancement but not sure this AI is the right place to discuss it. |
| InterDigital | As per chair guidance, this is discussed under 11.1. |
| CATT | Support. |
| Xiaomi | Suggest deferring this proposal or discuss it in overview agenda |
| OPPO | Fine in general. But prefer to discuss BWP issue in Agenda 11.1. |
| vivo | Can be discussion in AI 11.1. There are similar proposals already in the FL summary of AI 11.1. |
| Futurewei | We should discuss the BWP in 11.5 as the BWP is related to EE. We do not think that the details should be discussed in 11.1 , which the overall discussion. In 11.1 should be discussed the basic principles and the details left to other Ais. |
| Lenovo | This should be discussed under 11.1. |
| Huawei&HiSilicon 1 | We agree that several usage scenarios and technique directions of 6GR BWP should be further studied, including optimized BWP configuration, low switching delay, and enhanced robustness, etc. However, maybe this is not the right AI to discuss 6GR BWP and its lesson learned from NR. |
| Nokia | Thanks for the moderator for the summary here. One thing that we are a bit missing from the list of items to be studied is the BWP switching robustness and the number of ‘BWP’s (i.e. if there is a need for all of the current BWP types).Moreover, there seems to be some overlap with discussions happening in AI 11.1. |

## Others

### Discussion (before meeting)

DC handling (DOCOMO)

DC subcarrier handling in NR can be start point for further study in 6GR

- For DL: TX DC subcarrier is indicated to UE; no RX-side behavior is defined

- For UL: TX DC subcarrier (w/ or w/o 7.5 kHz offset) is reported to NW in RRC reconfiguration complete message if reporting is requested, and no TX-side behavior is defined

Other views:

- For FDD, consider whether dynamic adaptation of DL and UL carrier bandwidth and/or locations and/or pairing are supported: Ofinno

- Flexible slot duration or finite set of slot duration per numerology for 6GR: Tejas

- AI/ML enabled frame structure: Tejas

- Fractional symbol: Tejas

- A flexible frame structure configuration on a virtual continuous carrier: TCL

- Retransmission mechanisms for different channels: Transsion Holdings

- Multi-subframe continuous transmissions: Transsion Holdings

#### Company’s comment here

| Company | Comments |
| --- | --- |
| Tejas | We would like to discuss the following topics, - Flexible slot duration or finite set of slot duration per numerology for 6GR - Fractional symbol support |

# Agreements (post meeting)

RAN1#122 Bengaluru

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

Numerologies for sensing is up to sensing agenda discussion.

Agreement

6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

- FFS potential need for other CP.

# Proposals from contributions

# Reference

Section 11.3.2

- R1-2505128	Frame Structure and Numerology in 6G Radio Air Interface	Nokia

- R1-2505144	Discussion on 6G numerology and frame structure	FUTUREWEI

- R1-2505173	Discussion on frame structure for 6GR	Spreadtrum, UNISOC

- R1-2505184	Numerology and frame structure for 6GR air interface	Huawei, HiSilicon

- R1-2505265	Frame Structure for 6GR Air Interface	Google

- R1-2505309	Frame structure for 6GR	CATT

- R1-2505417	Discussion on 6GR frame structure	vivo

- R1-2505464	Discussion on 6G frame structure	Xiaomi

- R1-2505494	Discussion on 6G frame structure	ZTE Corporation, Sanechips

- R1-2505517	Discussion on frame structure	China Telecom

- R1-2505526	Discussion on numerology and frame structure for 6GR air interface	Lenovo

- R1-2505585	Discussion on frame structure design for 6GR	Samsung

- R1-2505611	On 6G frame structure	Ericsson

- R1-2505618	Discussion on 6GR Frame structure 	TCL

- R1-2505628	Zak-OTFS Supporting MRSS	Cohere Technologies

- R1-2505634	Frame Structure design	Tejas Network Limited

- R1-2505676	Initial views on 6G frame structure	Ofinno

- R1-2505681	6G and 5G Coexistence Considerations	IITH and WiSig

- R1-2505682	Dual Waveform for 6G DL	IITH and WiSig

- R1-2505694	Discussion on 6G frame structure	Transsion Holdings

- R1-2505758	Numerology and frame/slot structure for 6G Radio	OPPO

- R1-2505772	6GR frame structure	InterDigital, Inc.

- R1-2505780	Discussion on frame structure for 6GR air interface	Panasonic

- R1-2505788	Discussion on frame structure for 6GR	LG Electronics

- R1-2505797	Frame Structure for 6GR	Lekha Wireless Solutions

- R1-2505914	Numerology and frame structure for 6GR air interface	Apple

- R1-2505919	On Efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS)	Boost Mobile Network

- R1-2505968	Discussion on frame structure for 6GR	Fujitsu

- R1-2506000	Views on numerology and frame structure design for 6GR	HONOR

- R1-2506021	6G frame structure and numerology	MediaTek Inc.

- R1-2506066	Discussion on 6GR frame structure	ETRI

- R1-2506098	Discussion on frame structure for 6GR interface	CMCC

- R1-2506118	Considerations on 6GR frame structure	Sony

- R1-2506141	Discussion on Frame Structure for 6GR Air Interface	Rakuten Mobile, Inc

- R1-2506163	Frame Structure for 6GR	Sharp

- R1-2506219	Frame structure for 6GR	Qualcomm Incorporated

- R1-2506240	Requirements for 6GR Frame Structure Design	AT&T

- R1-2506307	Discussion on Frme structure for 6GR	NTT DOCOMO, INC.

- R1-2506360	Views on 6GR Frame structure and related aspects	CEWiT, IITM, Tejas, IITK

- R1-2506379	Considerations on frame structure for 6GR air interface 	ITL

Others

# History

RAN1#122

R1-2506603	FL summary for Frame Structure (round 2)	Moderator (CMCC)

R1-2506542	FL summary for Frame Structure (round 1)	Moderator (CMCC)

# Annex: Contact list

| Company | First/Last name | Email address |
| --- | --- | --- |
| ZTE | Shuaihua KOU Xingguang WEI | kou.shuaihua@zte.com.cn wei.xingguang@zte.com.cn |
| Sharp | Majid Ghanbarinejad | ghanbarinejadm@sharplabs.com |
| Ofinno | Jae-Nam Shim | jshim@ofinno.com |
| Xiaomi | Haipeng Lei Yanping Xing | leihaipeng@xiaomi.com xingyanping@xiaomi.com |
| NTT DOCOMO | Shohei Yoshioka Takanobu Onoda | syouhei.yoshioka.py@nttdocomo.com takanobu.onoda.un@nttdocomo.com |
| OPPO | Jia SHEN Kevin LIN | sj@oppo.com kevin.lin@oppo.com |
| CEWiT | Dhivagar B Abhijeet Masal Deepak P M | dhivagar.b@cewit.org.in abhijeetmasal@cewit.org.in deepakpm@cewit.org.in |
| Tejas Networks | Shrinivas Bhat Virendra | Shrinivasb@tejasnetworks.com virendrar@tejasnetworks.com |
| ASUSTeK | Eddie Lin | eddie1_lin@asus.com |
| TCL | Yiwei Deng Xingya Shen Jan Shahid Pu Yuan | yiwei1.deng@tcl.com xingya.shen@tcl.com shahid.jan@tcl.com pu.yuan@tcl.com |
| vivo | Xiaohang Chen Lihui Wang | chenxiaohang@vivo.com wanglihui@vivo.com |
| Fujitsu | Qinyan JIANG Taewoo LEE Lei ZHANG | jiangqinyan@fujitsu.com lee.taewoo@fujitsu.com zhanglei@fujitsu.com |
| Panasonic | Hidetoshi Suzuki Tomoya Nunome | suzuki.hidetoshi@jp.panasonic.com nunome.tomoya@jp.panasonic.com |
| Cohere Technologies | Yoav Hebron | Yoav.hebron@cohere-tech.com |
| LG Electronics | Hyunsoo Ko Seungmin Lee Daesung Hwang Hanjun Park Woo Suk Ko Seunggye Hwang Yujin Noh Hyangsun You | hyunsoo.ko@lge.com edison.lee@lge.com daesung.hwang@lge.com hanjun0128.park@lge.com woosuk.ko@lge.com seunggye.hwang@lge.com yujin.noh@lge.com sssun.you@lge.com |
| Lenovo | Chenxi Zhu Karthik Ganesan | Zhucx1@lenovo.com kganesan@lenovo.com |
| Fraunhofer | Olivier Renaudin | olivier.renaudin@iis.fraunhofer.de |
| Fraunhofer | Geordie George | geordie.george@iis.fraunhofer.de |
| Fraunhofer | Martin Leyh | martin.leyh@iis.fraunhofer.de |
| Futurewei | Hussain Elkotby George Calcev | helkotby@futurewei.com gcalcev@futurewei.com |
| Ericsson | Claes Tidestav Narendar Madhavan | claes.tidestav@ericsson.com narendar.madhavan@ericsson.com |
| Fraunhofer | Rashid Khouli | rashid.khouli@iis.fraunhofer.de |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| MediaTek | James (CH) Hsieh Mohammed AI-Imari | ch.hsieh@mediatek.com mohammed.al-imari@mediatek.com |
| China Telecom | Nanxi Li | Linanxi@chinatelecom.cn |
| NICT | Michiharu Nakamura | michi.nakamura@nict.go.jp |
| ETRI | Cheulsoon Kim Hoondong Noh | cs.kim@etri.re.kr hoondong.noh@etri.re.kr |
| Spreadtrum | Huan Zhou Yu Ding | huan.zhou@unisoc.com Yu.Ding@unisoc.com |
| Kyocera | Keita Tominaga | keita.tominaga.hs@kyocera.jp |
| CMCC | Xiaodong Shen Xiaoran Zhang Ting Ke | shenxiaodong@chinamobile.com zhangxiaoran@chinamobile.com keting@chinamobile.com |
| Orange | Raphael Visoz | raphael.visoz@orange.com |
| AT&T |  | ahmed.hindy@att.com rb691m@att.com |
| HONOR | JIAN ZHANG | zhangjian23@honor.com |