3GPP TSG RAN WG1 #122bis	        		              R1-250xxxx

Prague, Czech Republic, October 13th – 17th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.213 (mirrored to Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core, NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2025-10-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Missing part of Type-2 HARQ-ACK codebook for Uu link in the corresponding description for sidelink in Clause 16.5.2.2. - Incorrect reference to “DL PRS” as “PL RS” in Clause 17.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing part of Type-2 HARQ-ACK codebook for Uu link in the corresponding description for sidelink in Clause 16.5.2.2. - Change “PL RS” to “DL PRS” in Clause 17.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous/incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.2.2, 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

#### 16.5.2.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format without an SAI field, then

-	if the UE

-	has not received any PDCCH within the monitoring occasions for DCI format 3_0 for scheduling PSSCH with corresponding PSFCH reception occasions on any serving cell, and

-	does not have HARQ-ACK information in response to a PSSCH transmission with corresponding PSFCH reception occasions associated with a SL configured grant to multiplex in the PUSCH, as described in clause 16.5.2.1,

the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates and multiplexes in the PUSCH transmission the HARQ-ACK codebook as described in clause 16.5.2.1.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a SAI field, the UE generates the HARQ-ACK codebook as described in clause 16.5.2.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in clause 16.5.2.1, after the completion of the m loop, the UE sets j=j+1 if Vtemp>VT-SAIUL, and sets Vtemp=VT-SAIUL where VT-SAIUL is the value of the SAI field in the DCI format according to Table 16.5.2.2-1.

*** unchanged text omitted ***

## 17.2	Half-Duplex UE in paired spectrum

*** unchanged text omitted ***

If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols.

If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks.

*** unchanged text omitted ***