3GPP TSG- RAN WG1 #122bis	R1-250xxxx

Prague, Czech Republic, Oct 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 editorial corrections for TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current priority formula for CSI based on Doppler codebook, τ is used instead of j, but j is still used in function Pril,i,f, j, which is obviously a typo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | τ is used to replace j in Pril,i,f, j, to be consistent with the formula. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The priority formula for CSI based on Doppler codebook is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, Further Enhanced Type II Port Selection for predicted PMI and TDCP reporting.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4. For a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s). For a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for Type I CSI feedback for one or more of the sub-configurations, Part 1 for a sub-configuration contains corresponding RI (if reported), CRI (if reported) and CQI for the first codeword (if reported). Part 2 for a sub-configuration contains the corresponding CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI (if reported).

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,1 are reported in the increasing order of their indices, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded.

-	For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10) and Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1 or Further Enhanced Type II Port Selection for predicted PMI. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for CJT (see Clause 5.2.2.2.8) and Further Enhanced Type II Port Selection for CJT (see Clause 5.2.2.2.9), Part 1 contains RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} or {α1,…,αNTRP} (if reported). The fields of Part 1 – RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} or {α1,…,αNTRP} (if reported) – are separately encoded. Part 2 contains the PMI of the Enhanced Type II for CJT or Further Enhanced Type II Port Selection for CJT. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for predicted PMI with N4>1 (see Clause 5.2.2.2.10), Part 1 contains RI (if reported), the CQI (if the higher layer parameter tdCQI is set to '1-1' or '1-2') or the first CQI (if the higher layer parameter tdCQI is set to '2') and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the second CQI (if the higher layer parameter tdCQI is set to '2') and the PMI of the Enhanced Type II for predicted PMI with N4>1. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see Clause 5.2.4 and 5.2.2).

When the higher layer parameter reportQuantity is configured with one of the values 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', or 'cri-RSRP-Index', 'ssb-Index-RSRP-Index', 'cri-SINR-Index', 'ssb-Index-SINR-Index', 'tdcp', the CSI feedback consists of a single part.

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Clause 5.2.4.

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand with value '1' are numbered continuously in increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level, except for Part 2 subband CSI when the corresponding CSI report contains one or more CSI sub-reports with Part 2 each corresponding to a sub-configuration from a list of sub-configurations provided by csi-ReportSubConfigToAddModList contained in the CSI-ReportConfig as described in Clause 5.2.1.1.

-	For Enhanced Type II reports and Enhanced Type II for predicted PMI configured with higher layer parameter N4=1, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection reports and Further Enhanced Type II Port Selection for predicted PMI, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, i and f, is associated with a priority value Pril,i,f=K1⋅υ⋅f+υ⋅i+l, with l=1,2,…,υ, i=0,1,…,K1-1 and f=0,…,M-1. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υK1M-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,ij, f and j, is associated with a priority value Pril,ij,f,j=2⋅k=1N0Lσk⋅υ⋅πf+υ⋅2k=1j-1Lσk+ij+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1), for l=1,…,υ, ij=0,1,…,2Lσj-1, f=0,1,…,Mυ-1 and j=1,…,N0, and where n3,l(f) and Lσj are defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2Mυk=1N0Lσk-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, ij, f and j, is associated with a priority value Pril,ij,f,j=k=1N0K1,σk⋅υ⋅f+υ⋅k=1j-1K1,σk+ij+l, for l=1,…,υ, ij=0,1,…,K1,σj-1, f=0,…,M-1 and j=1,…,N0,  and where K1,σj is defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υMk=1N0K1,σk-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-ν,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for predicted PMI configured with N4>1, for a given CSI report n, each reported element of  i2,4,l i2,5,l and i1,7,l, indexed by l,i, f and τj, is associated with a priority value Pril,i,f,τj=2L⋅υ⋅Mν⋅τ+2L⋅υ⋅f+υ⋅i+l, for l=1,…,υ, i=0,1,…,2L-1, f=0,1,…,Mυ-1 and τ=0,1. The element with the highest priority has the lowest associated value Pril,i,f,τj. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported), i1,8,l (l=1,…,υ) and the second wideband CQI (if reported).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυQ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ), i1,10,l (if reported) and the second subband CQI of even subbands (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ) and the second subband CQI of odd subbands (if reported).

<omitted text>