3GPP TSG RAN WG1#123                                              R1-2508595

Dallas, USA, Nov.17th – 21st, 2025

Source:         CATT

Title:            Discussions on waveform for 6GR

Agenda Item:    11.3.1

Document for:   Discussion and Decision

Introduction

In RAN1#122bis meeting, following agreements were achieved [1].

Agreement Draft LS R1-2508068 is endorsed with following revision: - removing “Additionally, if time permits, any feedback for CP-OFDM PAPR reduction/MPR values achievable by implementation is also appreciated.” Agreement Final LS R1-2508069 is endorsed. Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

Waveform and modulations are the core techniques in communication and sensing systems. As shown in SID, 5G NR waveforms and modulation should be considered for 6GR with potential enhancement. In this contribution, the requirement on waveform, candidate lower PAPR schemes on communication and sensing, the feasibility of DFT-s-OFDM for downlink and the DFT-s-OFDM with RANK>1 for UL are discussed.

Disucssion

Waveform for communication

    - Waveform requirements

In 6GR design, to meet higher peak rate requirement, the maximum channel bandwidth will be increased, e.g. increasing from 100 MHz to 200 MHz or 400 MHz (SCS = 30 kHz). Following issues arise:

- With the increasing of FFT size (e.g. from 4096 to 8192 or 16384), the PAPR will increase about 0.3-0.5dB due to the doubled FFT size

- To maintain the same coverage (i.e. same power spectral density), the transmission power must also be increased, which may lead to PA (power amplifier) OBO (output power back-off). Thus, reducing PAPR is needed to maintain the coverage of 6GR

- The propagation distance in NTN is large. Taking LEO600 as an example, the downlink free space path loss (FSPL) can reach up to 159.1 dB [2]. Consequently, higher transmission power at BS is required, and a new requirement for PAPR performance is raised. Meanwhile, given the limited energy resources on satellites, efficiency improvement can reduce energy consumption and extend their operational lifespan. Additionally, high-efficiency PA can reduce the size of satellite solar panels and batteries, thereby lowering launch costs.

Observation 1: The PAPR is increasing with the increasing of DFT points e.g. 4096 to 8192 or 16384.

Observation 2: To maintain the same coverage (i.e. same power spectral density), transmission power will be doubled with the doubled channel bandwidth.

Observation 3: Since downlink free space path loss in NTN is large, higher transmission power at BS is required.

Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR:

- Larger FFT size (e.g. from 4096 to 8192 or 16384)

- Larger transmission channel bandwidth

- Increased downlink free space path loss in NTN due to large propagation distance.

    - PAPR reduction for CP-OFDM

In this section, candidate PAPR reduction schemes for CP-OFDM waveform such as Tone Reservation (TR), Selective Mapping (SLM) are discussed.

SLM for CP-OFDM waveform

SLM is a PAPR reduction technique that generates multiple candidate OFDM symbols by multiplying the input data with different phase sequences. The transmitter computes M different representations of the same data and selects the one with the lowest PAPR for transmission. SLM can achieve significant PAPR reduction without signal distortion. However, it not only requires M IFFT operations and side information transmission of ⌈log₂M⌉ bits per scheduled symbol, but also has high computational complexity proportional to M and channel rank. This prohibitive computational complexity scales poorly with increasing subcarrier numbers. Also, the side information overhead will reduce spectral efficiency. The impact of side information overhead is more significant in the uplink due to the limitations of the UL capability. The large side information overhead must be included in the UCI, which not only increases the header overhead but also complicates the reusing of UCI in the PUSCH. The prohibitive computational complexity and the side information transmission will pose challenges for practical deployment in 6GR, especially for massive bandwidth and massive MIMO configurations. Therefore, optimizations such as reducing overhead and indicating complexity of side information shall be studied if SLM is adopted in 6GR.

Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, scheme on reducing the overhead and indicating complexity of side information shall be studied.

TR for CP-OFDM waveform

TR is a PAPR reduction technique thorough exclusively allocating a subset of OFDM subcarriers for peak reduction purposes, without carrying data information. These reserved tones are optimized to generate a peak-reducing signal in the time domain, which is added to the original OFDM signal to suppress peaks. As the reserved subcarriers are predefined and do not interfere with data-carrying subcarriers, so no side information needs to be transmitted. TR preserves the BER performance of data subcarriers, remains compatible with existing receiver structures, and provides flexibility in trading spectral efficiency for reduced PAPR.

Proposal 3: It is proposed to study Tone Reservation (TR) for CP-OFDM waveform in 6GR.

In RAN1#122, CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink and uplink. In NR, the CP-OFDM waveform can be applied in all channel/signals e.g. PSS/SSS /PBCH/PDSCH/PDCCH/CSI-RS/PUSCH/PRACH/PUCCH etc. Similarly, the CP-OFDM waveform in 6GR can also be applied for those channel/signals. In 6GR, the schemes e.g. SLM/TR can be applied for CP-OFDM to reduce the PAPR as discussed above. The target channels/signals for SLM/TR with CP-OFDM should be discussed. Considering the synchronization/reference signals is not power limited because of PAPR issue, it is not necessary that the TR/SLM for CP-OFDM waveform applies to synchronization/reference signals. In other words, when design the TR/SLM schemes, the target channels/signals e.g. Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH should be considered.

Proposal 4: If SLM/TR are applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH.

    - PAPR reduction for DFT-s-OFDM

In this section, candidate PAPR reduction schemes for DFT-s-OFDM waveform such as Frequency Domain Spectrum Shaping (FDSS)/ FDSS-spectrum expansion (FDSS-SE), 8QAM/32QAM modulation are discussed.

FDSS/FDSS-SE for DFT-S-OFDM waveform

Frequency Domain Spectrum Shaping (FDSS) is a PAPR reduction technique by applying phase rotations or amplitude adjustments to OFDM subcarriers in the frequency domain without requiring reserved tones or side information. FDSS-SE (FDSS-spectrum expansion) extends the spectrum beyond the original bandwidth for better PAPR reduction. However, the bandwidth expansion in FDSS-SE leads to lower spectrum efficiency.  There is a tradeoff between PAPR reduction and spectrum expansion. Thus, if FDSS-SE is adopted in 6GR, the scheme on improving spectrum efficiency shall be studied.

Furthermore, FDSS-SE can be categorized into ‌symmetric FDSS-SE‌, which performs spectrum expansion on both sides of the original bandwidth, and ‌asymmetric FDSS-SE‌, which performs spectrum expansion on only one side of the original bandwidth. The PAPR of the transmitted signal depends on not only the amount of spectrum expansion but also the signal's inherent characteristics e.g., low PAPR characteristic. When the signal's inherent PAPR is not very high, asymmetric FDSS-SE can effectively reduce PAPR while minimizing the amount of spectrum expansion. This approach thereby reduces the frequency overhead of FDSS-SE. When the signal's inherent PAPR is high, symmetric FDSS-SE can be applied for PAPR reduction while increasing the amount of spectrum expansion. Thus, we propose that in 6GR, ‌both symmetric and asymmetric FDSS-SE should be supported‌ for low PAPR and spectrum efficiency.

Proposal 5: Both symmetric and asymmetric FDSS-SE can be supported in 6GR‌.

Odd-order modulation 32-QAM

Based on [3], even-order modulation schemes i.e., BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM, and 1024-QAM are supported in 5G NR, while odd-order modulation schemes (e.g., 32-QAM) are not supported. The 16-QAM with four high-power corner points brings higher PAPR than 32-QAM. Without four points in the four corners, the 32-QAM does not have large instantaneous signal amplitudes, which leads to lower PRPR, as shown in Figure 1. Figure 2 shows that the 32-QAM can bring 0.23dB lower PAPR than 16-QAM at CCDF (Complementary Cumulative Distribution Function) of 1e-3. Since the PAPR of 32-QAM is lower than that of 16-QAM, the MPR of 32-QAM is expected to be lower than that of 16-QAM. The lower MPR of 32-QAM can enable higher transmission power, which can bring performance gains such as in terms of BER. Moreover, 32-QAM offers a higher modulation rate (5 bits per symbol) compared to 16-QAM (4 bits per symbol), resulting in greater throughput. Therefore, 32-QAM outperforms 16-QAM in in terms of PAPR, BER and throughput.

Figure 1: The constellation of 32-QAM

Figure 2: The PAPR performance of 16-QAM, 32-QAM

Observation 4: The corner constellation points in odd-order quadrature amplitude modulation (e.g., 16-QAM) have higher power, resulting in a higher peak-to-average power ratio (PAPR). This increases power amplifier linearity requirements and reduces power efficiency.

Observation 5: 32-QAM outperforms 16-QAM in in terms of PAPR, BER and throughput.

Proposal 6: It is proposed to study 32-QAM schemes for PAPR reduction for DFT-S-OFDM in 6GR.

    - The feasibility of DFT-s-OFDM for NTN downlink

As previously mentioned in section 2.1.2, SLM technology can be used to reduce the PAPR of CP-OFDM waveform, but simultaneously would introduce extreme high complexity and reduce spectral efficiency. DFT-s-OFDM waveform had already been widespread application in the uplink. In this section, we will discuss the feasibility of DFT-s-OFDM for NTN downlink, addressing both main motivation and potential impacts.

Motivation

As describe in TR38.811 [5], the free space path loss (FSPL) in dB for a separation distance d in meter and frequency fc in GHz is given by

Based on different satellite orbit type, central beam elevation, and operating frequency, the FSPL in NTN ranges from 159.1 dB to 214.1 dB, which is higher than that in TN. On the other hand, BS in TN can obtain abundant energy supply by connected to the mains power supply, making efficiency losses from appropriate power backoff acceptable. However, the energy supply for BS is limited in satellite communications, which necessitates that the power amplifier achieves high-power output while operating at low power consumption.

Observation 6: The FSPL in NTN is higher than that in TN. Due to limited energy supply, higher efficient waveform is necessary for satellite communication.

To ensure adequate terminal reception power, it is necessary to increase the output power of the PA. In other words, OBO must be minimized as much as possible to enhance PA efficiency. In conjunction with PA employed in actual satellite communications, a set of tests and measurements on DFT-s-OFDM and CP-OFDM waveforms are conducted. Typically, a kind of traveling-wave tube amplifier (TWTA) used in satellite is selected for real test.. It was observed that the PAPR of the CP-OFDM waveform was larger than that of the DFT-s-OFDM waveform without passing through PA, as shown in Table 1.

Table 1: PAPR Test Results for Different Waveforms

| Parameter | CP-OFDM waveform | DFT-s-OFDM waveform |
| --- | --- | --- |
| PAPR | 9.64 | 7.42 |

The high PAPR of the CP-OFDM waveform leads a greater OBO after passing through the PA, as shown in Table 2 and Table 3.

Table 2: The EVM (Error Vector Magnitude) test results for QPSK modulation scheme

| Parameter | CP-OFDM waveform | DFT-s-OFDM waveform | The EVM gain |
| --- | --- | --- | --- |
| OBO=1dB | 23.9% | 8.79% | 15.11% |
| OBO=2dB | 16.1% | 6% | 10.1% |
| OBO=3dB | 11.1% | 4.22% | 6.88% |
| OBO=4dB | 7.8% | 3.2% | 4.6% |
| OBO=5dB | 5.5% | 2.87% | 2.63% |

Table 3: The EVM (Error Vector Magnitude) test results for 16QAM modulation scheme

| Parameter | CP-OFDM waveform | DFT-s-OFDM waveform | The EVM gain |
| --- | --- | --- | --- |
| OBO=1 dB | 23.2% | 14.4% | 8.8% |
| OBO=2 dB | 16.9% | 8.7% | 8.2% |
| OBO=3 dB | 11.2% | 5.7% | 5.5% |
| OBO=4 dB | 7.6% | 4% | 3.6% |
| OBO=5 dB | 5.4% | 3.3% | 2.1% |

For the PDSCH, the EVM of each NR carrier shall be less than 17.5% for QPSK modulation and less than 12.5% for 16QAM modulation [4]. To satisfy the EVM requirements,

- For QPSK, no OBO is required when using DFT-s-OFDM waveform, whereas a 2dB OBO is required for CP-OFDM waveform.

- For 16QAM, a 1dB OBO is required when using DFT-s-OFDM waveform, whereas a 3dB OBO is required for CP-OFDM waveform.

It can be seen that, an additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme after PA.

Observation 7: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet EVM requirements.

A higher EVM leads to poorer demodulation performance. As shown in Figure 3, to achieve equivalent demodulation performance (approximately 0.2 dB worse than that without passing through PA), an OBO of 3 dB is required when using CP-OFDM to align the performance with the no PA case, whereas only a OBO of 1 dB is needed when using the DFT-S-OFDM waveform.

Observation 8: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet equivalent demodulation performance.

Figure 3: The BLER performance of CP-OFDM and DFT-s-OFDM waveform

Proposal 7: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA.

Based on last meeting discussion, for low PAPR benefit, similar performance evaluation method in UL waveform evaluation can be used in the DL. The net gain can be considered to evaluate the new waveform by taking into account TX power gain and link loss.

Proposal 8: For downlink low-PAPR proposals the primary evaluation criterion may use the following criterion:

- Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal.

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - FFS: Other evaluation metrics

    - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Potential impacts

Target channels/signals

The network can select different waveforms for downlink transmission based on terminal types. For instance, the DFT-s-OFDM waveform is employed for handheld terminals, while for Very Small Aperture Terminal (VSAT), CP-OFDM waveform can be used. The transform precoding is applied across all channels/signals when using DFT-s-OFDM waveform. The network can do pre-configuration of the mapping between cell ID and waveform, so that the UE can also obtain the waveform to be used, such as the cell ID, via SSB detection. Subsequently, a waveform type is uniformly utilized for all channels within a single cell although two waveforms can be supported in NTN. As only one type of waveform is chosen in a single cell, there is no issue of compatibility between different waveforms in different channels/signals.

Observation 9: DFT-s-OFDM waveform can be used across all channels/signals.

Observation 10: The issue of multiplexing/coexistence with baseline waveforms does not exist as only one type of waveform is chosen in a single cell in one certain scenario.

MIMO (SU and MU-MIMO) compatibility

For satellite communication applications, line-of-sight (LOS) paths are predominantly employed, with MIMO transmission being relatively uncommon particularly for handheld terminals in Direct-to-Satellite Mobile Communication. For VSAT UE, in some cases it is possible to extend Rank 2 transmission via antenna polarization. In general, DFT-s-OFDM waveform is optimal for single layer transmission. Whether to use DFT-s-OFDM for DL rank 2 transmissions, it can be jointly discussed with UL waveform.

Observation 11: For NTN scenario, rank 1 is the baseline assumption.

Multi-user multiplexing/scheduling flexibility

When multiple users multiplex into a single symbol, the network must cascade multiple users' data according to certain rules in the time domain to perform a DFT transformation into the frequency domain as shown in Figure 4. Consequently, at the receiver, the UE performs full-bandwidth reception, i.e., extracts its own data after IDFT processing. Although the UE must buffer the entire BWP data, its frequency domain position can be determined via the FDRA field in the DCI after decoding the PDCCH when CP-OFDM waveform is used. In contrast, when using the DFT-s-OFDM waveform, the UE needs an IDFT transform before getting its own data. This may need a higher FFT-points compared to the CP-OFDM waveform used in uplink. However, considering the limited bandwidth in NTN, particularly in Direct-to-Satellite Mobile Communication, the FFT-points remain manageable.

Figure 4: Transmitter Block Diagram at BS

Observation 12: When DFT-s-OFDM waveform is used in DL, multi-user resource mapping before DFT precoding can be implemented.

Proposal 9: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement.

    - The enhancement on DFT-S-OFDM waveform

In this section, the enhancements e.g. two segments DFT-S-OFDM scheme and multiplexing between data and DMRS are discussed.

Two segments DFT-S-OFDM scheme

In LTE, the DFT-S-OFDM is used on physical uplink channel due to its lower PAPR and higher energy efficiency. In 5G NR, the DFT-S-OFDM is also used on physical uplink channel to guarantee the coverage for scenarios where the coverage performance is limited. In RAN1#122, the DFT-S-OFDM is agreed to use on physical uplink channel in 6GR. Further, the bandwidth of the PUSCH i.e. MRBPUSCH in terms of resource blocks in 5G NR for applying the DFT [3], fulfils

Where is a set of non-negative integers. With above restriction, the REs of the PUSCH can be enabled easily for the DFT implementation. Similarly, we assume that same limitation of RBs should be fulfilled for the uplink channel in 6GR if DFT-S-OFDM is applied. However, to support 200 MHz (SCS = 30 kHz) bandwidth on PUSCH, the 8192 points DFT size may be used, that is about 550 RBs when considering guard bandwidth. From 1-550RB, only 12.5% RB number fulfills the above restriction, it means more than 87% of RB number values cannot be indicated, which will reduce the scheduling flexibility. More details can be found in Appendix.

To increase the scheduling flexibility, one possible scheme is to divide MRBpusch RB into two segments i.e. M1RBpusch and M2RBpusch, where MRBpusch =M1RBpusch+M2RBpusch, M1RBpusch=2α2*3α3*5α5, M2RBpusch=2α2*3α3*5α5. The principle of scheme with performing two DFT operations is shown as Figure 5.

Figure 5: Two segments DFT-s-OFDM scheme

For example, MRBpuach=205, cannot satisfy the restriction condition. Then, 205 can be divided into M1RBpusch=200 and M1RBpusch= 5 where 200 is equal to 22*52, and 5 is equal to 51,which satisfy the restriction condition. In this way, one DFT operation with 200 RB and another DFT operation with 5 RB can be indicated to increase the scheduling flexibility.

Proposal 10: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied.

Multiplexing between data and DMRS

In 5G-NR, when DFT-S-OFDM waveform is used for PUSCH transmission, the REs that no using for DMRS signal transmission is blank and cannot be used for PUSCH transmission. It is understood that only DMRS transmitted in DMRS OFDM symbol can keep lower PAPR than both PUSCH and DMRS transmitted in an OFDM symbol. Our simulation show that the PAPR of DMRS multiplexing with PUSCH on a OFDM symbol is increasing, but is not much to block the multiplexing between PUSCH data and DMRS in a symbol.

Figure 6: CCDF of PARP for frequency multiplexing

Assuming waveform for PUSCH transmission is DFT-S-OFDM, 16-QAM and low PAPR ZC sequence. Comparing to the only PUSCH data transmission, the DMRS multiplexing with PUSCH on an OFDM symbol has about 0.6dB PAPR increasing, as shown in Figure 6.

In the 1% CCDF point, the PAPR of only PUSCH data transmitted in an OFDM symbol is about 8.3dB, and it means that PUSCH can be transmitted efficiently when the PAPR is not larger than 8.3dB. We can see that it is about 91% chance to allow for frequency multiplexing between PUSCH data and DMRS since the CCDF point is 9% that PAPR is larger than 8,3dB. More specifically, how to multiplex can be implemented as shown in Figure 7.

Figure 7: Multiplexing between PUSCH and DMRS

Proposal 11: For improving spectrum efficiency, multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied.

    - DFT-s-OFDM with RANK>1 for UL

In RAN1#122bis meeting, it was agreed to study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1 as follows:

Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

In NR, DFT-s-OFDM only with layer = 1 and CP-OFDM with number of layers ≥ 1 were supported. The DFT-s-OFDM and CP-OFDM can be switched by a dynamic indication for coverage enhancement. The scheduling for layers > 1 is implemented when the switching to CP-OFDM is indicated. Thus, to study whether to support DFT-s-OFDM for UL with number of layers > 1, the potential benefits of DFT-s-OFDM with number of layers > 1 should be compared with CP-OFDM with number of layers > 1 based on the link level simulation. In this case, the link level performance evaluation criterion, e.g. Net Gain (Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER), can be considered. The reference is the link level performance of CP-OFDM with corresponding number of layers. Further, a realistic PA model should be used. When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration and equalization should be considered. Moreover, the evaluation on the new frequency range, e.g., around 7GHz should be considered, as well as the potential higher bandwidth for 6GR, e.g., 100MHz transmission bandwidth.

Proposal 12: For evaluating DFT-s-OFDM for UL with number of layers > 1, link level simulation is considered:

- Net gain = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - The reference is the link level performance of CP-OFDM with corresponding number of layers.

    - A realistic PA model should be used.

    - Both low (e.g., 20MHz) and high (e.g., 100MHz) bandwidth.

Apart from the simulation, other issues should also be considered for evaluation on DFT-s-OFDM with higher layers. One critical issue is the real implementation complexity in receiver on gNB side, e.g., complex joint FD equalization and MIMO decoder. When DFT-s-OFDM with number of layers > 1 is implemented, the complexity of the receiver will increase dramatically. Such complexity should be avoided. Another issue is the spec impact, e.g., reference signals design and codebook design. These designs need further modifications if DFT-s-OFDM with number of layers > 1 is implemented, which may lead to huge spec impact. Overwhelming complicated designs considering such issues should also be avoided.

Proposal 13: Besides the link level simulation, following aspects should be analyzed:

- Implementation complexity in receiver.

- Spec impact, e.g., reference signals design, codebook design.

Waveform for sensing

To enable sensing functionality, both mono-static mode and bi-static mode configurations can be employed. In mono-static mode, the transmitter and receiver are co-located, offering clear advantages. For example, there is no inter-node synchronization issue, and communication signals can be reused for sensing. The following two types sensing waveforms are needed for mono-static mode:

- The Pulse wave (PW) with short duration transmission and power boosting provides better coverage. Also, PW is self-interference free because the TX and RX are time-division-multiplexed (TDM).

- Continuous wave (CW) with long duration transmission can provide a full coverage. In mono-static mode, TX and RX are operating simultaneously. The CW with self-interference limits the transmission power and sensing coverage (e.g., 0-200 meters)

For bi-static mode, TX and RX are separated in spatial domain, and the CW can be used without considering self-interference.

In general, there are two kinds of waveforms for PW. One is Linear Frequency Modulation (LFM) chirps-based PW, which is widely proven in radar system. Another is OFDM-based PW with larger SCS (e.g. SCS=960 kHz or 1920 kHz), which has similar waveform with communication and can reuse the existing communication transmitter chain.

For CW, the OFDM-based can be used, which has same waveform and same SCS with communication. Moreover, some enhancement schemes can be studied to improve sensing performance, such as AFDM (Affine Frequency Division Multiplexing) or OCDM (Orthogonal Chirp Division Multiplexing) which are based on CP-OFDM waveform. Compared to OFDM-based waveform, the AFDM/OCDM features a sharper main lobe in the range ambiguity function.

Proposal 14: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW):

- OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication

- OFDM-based CW with same SCS as communication

Proposal 15: For enhance the sensing, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing).

Conclusion

In this contribution, the waveform requirement and candidate schemes on sensing and communication are discussed. The proposals and observations are summarized as follows:

Observation 1: The PAPR is increasing with the increasing of DFT points e.g. 4096 to 8192 or 16384.

Observation 2: To maintain the same coverage (i.e. same power spectral density), transmission power will be doubled with the doubled channel bandwidth.

Observation 3: Since downlink free space path loss in NTN is large, higher transmission power at BS is required.

Observation 4: The corner constellation points in odd-order quadrature amplitude modulation (e.g., 16-QAM) have higher power, resulting in a higher peak-to-average power ratio (PAPR). This increases power amplifier linearity requirements and reduces power efficiency.

Observation 5: 32-QAM outperforms 16-QAM in in terms of PAPR, BER and throughput.

Observation 6: The FSPL in NTN is higher than that in TN. Due to limited energy supply, higher efficient waveform is necessary for satellite communication.

Observation 7: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet EVM requirements.

Observation 8: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet equivalent demodulation performance.

Observation 9: DFT-s-OFDM waveform can be used across all channels/signals.

Observation 10: The issue of multiplexing/coexistence with baseline waveforms does not exist as only one type of waveform is chosen in a single cell in one certain scenario.

Observation 11: For NTN scenario, rank 1 is the baseline assumption.

Observation 12: When DFT-s-OFDM waveform is used in DL, multi-user resource mapping before DFT precoding can be implemented.

Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR:

- Larger FFT size (e.g. from 4096 to 8192 or 16384)

- Larger transmission channel bandwidth

- Increased downlink free space path loss in NTN due to large propagation distance.

Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, scheme on reducing the overhead and indicating complexity of side information shall be studied.

Proposal 3: It is proposed to study Tone Reservation (TR) for CP-OFDM waveform in 6GR.

Proposal 4: If SLM/TR are applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH.

Proposal 5: Both symmetric and asymmetric FDSS-SE can be supported in 6GR‌.

Proposal 6: It is proposed to study 32-QAM schemes for PAPR reduction for DFT-S-OFDM in 6GR.

Proposal 7: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA.

Proposal 8: For downlink low-PAPR proposals the primary evaluation criterion may use the following criterion:

- Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal.

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - FFS: Other evaluation metrics

    - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Proposal 9: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement.

Proposal 10: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied.

Proposal 11: For improving spectrum efficiency, multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied.

Proposal 12: For evaluating DFT-s-OFDM for UL with number of layers > 1, link level simulation is considered:

- Net gain = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - The reference is the link level performance of CP-OFDM with corresponding number of layers.

    - A realistic PA model should be used.

    - Both low (e.g., 20MHz) and high (e.g., 100MHz) bandwidth.

Proposal 13: Besides the link level simulation, following aspects should be analyzed:

- Implementation complexity in receiver.

- Spec impact, e.g., reference signals design, codebook design.

Proposal 14: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW):

- OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication

- OFDM-based CW with same SCS as communication

Proposal 15: For enhance the sensing, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing).

References

- Chairman notes RAN1#122bis, Prague, Czech, Oct 13th – 17th, 2025.

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), v16.1.0.

- TR 38.211, NR Physical channels and modulation, v19.0.0.

- TS 38.104, Base Station (BS) radio transmission and reception, v15.1.0.

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks, v15.4.0.

Appendix

With limitation, there are 69 candidate frequency resource from 1~550PRB {1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 15, 16, 18, 20, 24, 25, 27, 30, 32, 36, 40, 45, 48, 50, 54, 60, 64, 72, 75, 80, 81, 90, 96, 100, 108, 120, 125, 128, 135, 144, 150, 160, 162, 180, 192, 200, 216, 225, 240, 243, 250, 256, 270, 288, 300, 320, 324, 360, 375, 384, 400, 405, 432, 450, 480, 486, 500, 512, 540} PRBs. In other words, around 12.5% (69/551) resource allocation cannot be scheduled.