3GPP TSG RAN WG1 #123	R1-2508970

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	10.6.1

Source: 	ETRI

Title:	Discussion on NR-NTN GNSS resilient operations

Document for:	Discussion/Decision

Introduction

In [1], the following study item objectives on NR-NTN GNSS resilient operations are available:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

This contribution presents ETRI’s views on NR-NTN GNSS resilient operations, focusing on the following aspects:

- GNSS performance degradation in real world

- Preliminary evaluation results on NR-NTN GNSS resilience

- Potential solutions

GNSS performance degradation in real world

In environments where satellite signals are physically blocked—such as tunnels, underground spaces, and building interiors—GNSS operation is fundamentally infeasible. When a vehicle enters a tunnel, all GNSS satellite signals are typically lost within just a few meters due to obstruction by concrete structures. The receiver loses signal tracking within seconds, and position updates are halted. Depending on tunnel length, GNSS may be completely unavailable for several minutes, leading to frequent navigation malfunctions or interruptions in road and railway tunnels. Urban canyons, surrounded by tall buildings, can be considered quasi-indoor environments. In such areas, direct satellite signals are often blocked, and only multipath signals reflected off building surfaces are received. In these cases, the receiver may continuously output significantly inaccurate positions.

In mountainous terrain, surrounding ridgelines and cliffs can obstruct the line-of-sight (LOS) between GNSS satellites and the receiver, resulting in a reduced number of visible satellites. When located in deep valleys or gorges, only a limited portion of the sky is visible, causing satellites to cluster within a narrow section of the sky. This leads to poor satellite geometry and significantly increases the Dilution of Precision (DOP), thereby amplifying positioning errors. For instance, when satellites are confined to a narrow angular sector, a position fix may still be computed in theory, but the resulting error margin can be substantially larger—ranging from several tens to hundreds of meters. In practice, there have been numerous reports of GNSS positions deviating by hundreds of meters in canyons or along steep slopes. Furthermore, mountainous regions often experience signal shadowing (zones of shadow) caused by terrain obstructions, resulting in periods during which satellite signals cannot be received at all.

Jamming is a relatively simple method for disrupting GNSS operations. It involves emitting strong radio signals in the same frequency bands used by GNSS systems, such as GPS, via a signal generator (jammer), thereby preventing nearby GNSS receivers from acquiring satellite signals. Under jamming attacks, the signal-to-noise ratio (SNR) at the receiver rapidly deteriorates, eventually resulting in loss of satellite tracking. This leads to degraded GNSS availability or complete service interruption, rendering navigation systems unable to compute positions or significantly increasing their errors.

Modern jammers have evolved into multi-band devices, making it increasingly common for civilian receivers to experience complete GNSS signal loss under jamming conditions. For example, since 2017, recurring GPS jamming—suspected to originate from across the nearby border—has been reported in northern Norway, affecting systems such as ambulance tracking and personal emergency devices. In another incident in 2019, intermittent GPS signal loss occurred for aircraft approaching Harbin Airport in China due to an illegal jammer operated at a nearby pig farm. The jammer had been installed to repel drones, but unintentionally interfered with aircraft navigation.

Spoofing is a more sophisticated form of interference than jamming, aiming not to disable the receiver entirely but to deceive it into accepting false position or time information. A spoofer transmits counterfeit GNSS signals that mimic authentic satellite signals but with slightly higher power, thereby hijacking the receiver’s tracking loop and forcing it to output a fabricated position. During a spoofing attack, the GNSS receiver appears to function normally from the user's perspective, but the displayed position or time is falsified. As a result, the integrity of the GNSS is severely compromised, potentially leading to secondary damages caused by incorrect information.

In 2013, a research team from the University of Texas Austin used GPS spoofing to manipulate the navigation of a vessel sailing in the Mediterranean Sea. They successfully induced the vessel to deviate from its original route by hundreds of meters away, and the vessel’s navigation system failed to recognize the GPS spoofing attack. In 2020, a U.S. light aircraft pilot encountered a military drone during flight, after which the aircraft’s navigation system suddenly reported a location several hundred km away in a different state. While flying near Los Angeles, the display abruptly showed place names in Utah and Montana, triggering off-route warnings and forcing the pilot to abandon the instruments and land visually. Recently, abnormal vessel tracks have also been reported in the Red Sea, where ships appeared on land or were shown traveling at impossible speeds exceeding 100 knots.

Proposal 1: RAN1 to consider intentional threats, such as jamming and spoofing, as well as environmental impairments for practical GNSS performance degradation modelling.

- Intentional threats on GNSS may last several hours.

Detailed modeling on intentional threats on GNSS

As aforementioned in previous sections, it is critical to reflect GNSS performance degradation from either of(or from both of) environmental impairments or(and) intentional threats for fair comparison between different GNSS resilient operations. Although the GNSS performance degradation modelling does not affect the NTN channel model itself, several key assumptions in NTN link-level and system-level evaluations, including pre-/post-time and frequency compensations, should be revisited. In this section, we provide 2 examples on GNSS performance degradations, which would be converted into lower T/F compensation accuracies for the NTN link-level and system-level evaluations.

Figure 1 shows an example of GNSS performance degradation modelling due to jamming. In this scenario, the CNR for GNSS reception (identically, inverse of UE location error) starts to decrease after the jamming signal is generated, until the transmission power of jammer is reached to its maximum power (deconditioning duration). The maximum CNR degradation level would be kept during the jamming duration. After the jamming duration, the CNR degradation would be recovered to 0 dB within a certain recovery duration. So, the GNSS-unavailable time duration is equal to or less than the sum of deconditioning duration, jamming duration, and recovery duration.

Figure 1. An example of GNSS performance degradation modelling due to jamming

Figure 2 depicts an example of GNSS performance degradation modelling due to spoofing. In this scenario, the estimated UE position can be intentionally deceived by, e.g., position jump, position drift, etc. For instance, after the GNSS spoofing is started, there could be one or more position jump, even if the UE actually has stayed in the same location. Furthermore, the distorted UE location may keep varying by a certain level of position drift (or by drift variation, as well) during the spoofing duration. After the spoofing duration, the (additional) UE position error would be recovered to 0 m within a certain recovery duration. So, the GNSS-unavailable time duration is equal to or less than the sum of spoofing duration, and recovery duration.

Figure 2. An example of GNSS performance degradation modelling due to spoofing

According to the above discussions on GNSS performance degradation modelling, we think the following aspects can be studied from UE perspectives (i.e., can be different per UE type/condition):

- Detection mechanism to clarify GNSS status (and corresponding detection probability/latency on GNSS status)

- Time-To-Recover

- Max position drift / drift variation

- Etc.

On top of the UE-specific parameters above, the following aspects can be further studied from system perspectives:

- (Timing) pattern of GNSS-unavailability occurrence

- Maximum duration of GNSS-unavailable state (i.e., max outage duration)

- Etc.

Proposal 2: RAN1 to consider the following aspects from UE perspectives for GNSS performance degradation modelling:

- Detection mechanism to clarify GNSS status (and corresponding detection probability/latency on GNSS status)

- Time-To-Recover

- Max position drift / drift variation

- FFS, other aspects

Proposal 3: RAN1 to consider the following aspects from system perspectives for GNSS performance degradation modelling:

- (Timing) pattern of GNSS-unavailability occurrence

- Maximum duration of GNSS-unavailable state (i.e., max outage duration)

- FFS, other aspects

Preliminary evaluation results on NR-NTN GNSS resilience

Assumptions and parameters

For the evaluation of GNSS resilient NR-NTN operation, the following agreements were made in RAN1#122 [2] and RAN1#122bis [3].

Agreement For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes: Satellite orbit; GSO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Satellite scenario parameters; At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered.. Beam size (note 3); At least values captured in Table 6.1.1.1-1/2 are considered,. Minimum elevation angle; 30° (LEO), 12.5° (GSO). Frequency ranges/bands; FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz). UE type; Handheld (L/S band), VSAT (Ku/Ka). UE speed (note 1); L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2). Beam/cell type (note 4); (Quasi)-Earth-fixed beams/cells and Earth-moving. Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies.. Note: The cyan highlighted parameters are prioritized for evaluation purposes. Agreement: For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area - Case a: the location uncertainty area is the area served by the cell or beam. - Case b: the location uncertainty area is a circle of radius X km - X to be reported UE altitude error should be taken into account. - UE altitude error to be reported. For both Case a and Case b above, RAN1 to evaluate at least: - Differential delay values - Differential frequency offset/Doppler values Agreement: For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions: - For scenario 1 and 2 - The following time/frequency differences should be evaluated: - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area. - For UL performance evaluation 2 times differential one-way delay should be considered. - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area. - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported. Agreement: In RAN1#123, based on the following alternatives representing two calculation methodologies, select a calculation methodology with potential adjustments, including correction of equations: Alt 1: For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered: Geometry model and equations could be considered. For the variables in the equations, refer to Figure below. For non-nadir beam For nadir beam dnear=hs-hue. Note 1: UE is at same height hue as the reference point. Note 2: For the purpose of evaluation, fixed size of uncertainty area is used within the whole satellite coverage footprint is a baseline. Other size of uncertainty area is not precluded. FFS: Whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area or at the center of the uncertainty area FFS: How to take into account the UE altitude uncertainty. Alt 2: For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X. Considering satellite location at s=0,0,RE+h One-way delay between a point u and satellite: done-wayu=¦u-s¦2c One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along with speed vue: dopplerone-wayu=vcfc<u-s,x>done-wayu+vuecfc<s-u,k>done-wayu Let R be the location uncertainty area. Differential one-way delay = maxu∈Rdone-wayu-minu∈Rdone-wayu Differential one-way doppler = maxu∈R dopplerone-wayu-minu∈R dopplerone-way(u) For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane). Note: Assume satellite and UE moves parallel to the x-y plane. <∙,∙> is the vector inner product, ∙2 is the vector magnitude.

For Alt1 in the Agreement for the two calculation methodologies, the equations for ∆t and ∆fd contain typos, and should be corrected as follows:

∆t=dfar-dnearc and ∆fd=fdfar-fdnear.

Proposal 4: RAN1 to select Alt 1 for future evaluations:

- The typos in Alt1 from the Agreement need to be corrected (the version from final FLS is correct) as follows:

  - ∆t=dfar-dnearc and ∆fd=fdfar-fdnear

  - fdnear=vssinβ1-vuecosγcfc

  - φ1=φ-2RcRE+hue (for non-nadir beam) and φ=RcRE+hue (for nadir beam)

- It is preferred to capture Alt 1 for the NTN channel model TR

The shape and size of the satellite beams illustrated onto the Earth’s surface differ from the beam pattern in Table 6.1.1.1-1 [4]. When considering the LEO-600 S-band scenario in Table 6.1.1.1-1 [4] with a 3dB beamwidth of 4.4127° and a bore-sight angle of 45° Figure 3 illustrates the beam mapping from the UV plane onto the Earth’s surface.

Figure 3. Beam mapping from the UV plane (eft) onto Earth’s surface (right)

The projected beam pattern varies with the satellite altitude and elevation angle. For LEO-600 S-band with the 3dB beamwidth of 4.4127°, the beam footprints projected onto the Earth’s surface in the x-y-z coordinate system are illustrated in Figure 4 according for different elevation angles, and the corresponding differential beam radii are provided in Table 1. It is observed that the differential beam radii are non-trivial particularly when the elevation angle is small.

Figure 4. Illuminated beam footprint: 600km circular orbit with 3dB beamwidth of 4.4127°

Table 1. Differential beam radius for LEO-600 S-band

| Elevation angle [deg] | 90 | 80 | 70 | 60 | 50 | 40 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Differential beam radius [km] | -1.882 | -0.636 | 3.034 | 10.524 | 25.378 | 57.306 | 139.378 |

Observation 1: The differential beam radius between the beam footprints projected onto the Earth’s surface and the circular beam in Table 6.1.1.1-1 TR 38.821 are non-trivial.

- The radius of edge beam can be multiple times of that of Nadir beam

Proposal 5: RAN1 to consider the beam footprints projected onto the Earth’s surface for evaluation of NR-NTN GNSS resilience.

- The values of X and T should be determined by considering the radius of edge beam, rather than by the radius of Nadir beam

When considering the projected beam pattern, the following geometry model and the equations are considered for the calculation of differential one-way delay and differential Doppler.

Figure 5. Geometry model for the evaluations with projected beam pattern

αnear=acosRE+hssinθ-θ3dB2RE+hue αfar=acosRE+hssinθ+θ3dB2RE+hue αRL=acosRE+hssinθRE+hue dnear=-RE+huesinαnear+RE+hue2sin2αnear+hs-hue2+2RE+huehs-hue dfar=-RE+huesinαfar+RE+hue2sin2αfar+hs-hue2+2RE+huehs-hue dRL=-RE+huesinαRL+RE+hue2sin2αRL+hs-hue2+2RE+huehs-hue fnear=fccVs-Vuecosπ2-θ-θ3dB2 ffar=fccVs+Vuecosπ2-θ+θ3dB2 fRL=fccVscosπ2-θ ΔtRL-far=dfar-dRLc Δtnear-far=dfar-dnearc ΔfRL-far=fRL-ffar Δfnear-far=fnear-ffar

Evaluation results

In this subsection presents the evaluation results for NR-NTN GNSS resilience. The following assumptions are considered:

- For the beam pattern, two cases are considered as

- BP 1: Ideal circular beam pattern in Table 6.1.1.1-1 [4], and the uncertainty area has a fixed size

- BP 2: Practically illuminated beam pattern projected onto the Earth’s surface in the x–y–z coordinate system, and the size of the uncertainty area varies in accordance with the elevation angle

- (Quasi)-Earth fixed beams are considered

- The UEs are located on the orbital plane, positioned at the farthest and closest points from the satellite, respectively

- The farthest and the closest UEs move in the same and opposite direction of the satellite, respectively

- The elevation angle is measured at the reference location

The following tables shows the evaluation results for differential position (Δp), differential delay (Δt), and differential Doppler (Δfd). The Δt and Δfd are evaluated between the UEs at the farthest and closest points from the satellite (Table 2 - Table 9), and between the farthest UE and the reference location (Table 10 - Table 17).

Table 2. Δp, Δt, and Δfd between the farthest and closest UEs: 2 GHz, 0 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 50 | 281 | 144.43 | 816.01 | 0.36 | 1.94 | 90 | 465 | 259.98 | 1350.95 | 0.37 | 1.86 |
| 40° | 50 | 150 | 127.74 | 384.43 | 0.66 | 1.94 | 90 | 268 | 229.94 | 688.62 | 0.63 | 1.86 |
| 50° | 50 | 95 | 107.17 | 204.25 | 1.03 | 1.94 | 90 | 179 | 192.92 | 384.24 | 0.94 | 1.86 |
| 60° | 50 | 68 | 83.35 | 114.43 | 1.42 | 1.94 | 90 | 133 | 150.05 | 221.90 | 1.27 | 1.86 |
| 70° | 50 | 55 | 57.00 | 62.69 | 1.77 | 1.94 | 90 | 108 | 102.62 | 123.80 | 1.55 | 1.86 |
| 80° | 50 | 48 | 28.94 | 27.98 | 2.01 | 1.94 | 90 | 96 | 52.10 | 55.80 | 1.74 | 1.86 |
| 90° | 50 | 46 | 0.00 | 0.00 | 2.10 | 1.94 | 90 | 92 | 0.00 | 0.00 | 1.81 | 1.86 |

Table 3. Δp, Δt, and Δfd between the farthest and closest UEs: 2 GHz, 3 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 50 | 281 | 144.43 | 816.01 | 0.36 | 1.95 | 90 | 465 | 259.98 | 1350.95 | 0.38 | 1.87 |
| 40° | 50 | 150 | 127.74 | 384.43 | 0.66 | 1.95 | 90 | 268 | 229.94 | 688.62 | 0.64 | 1.87 |
| 50° | 50 | 95 | 107.17 | 204.25 | 1.03 | 1.95 | 90 | 179 | 192.92 | 384.24 | 0.95 | 1.87 |
| 60° | 50 | 68 | 83.35 | 114.43 | 1.42 | 1.94 | 90 | 133 | 150.05 | 221.90 | 1.27 | 1.87 |
| 70° | 50 | 55 | 57.00 | 62.69 | 1.77 | 1.94 | 90 | 108 | 102.62 | 123.80 | 1.55 | 1.87 |
| 80° | 50 | 48 | 28.94 | 27.98 | 2.01 | 1.94 | 90 | 96 | 52.10 | 55.80 | 1.75 | 1.86 |
| 90° | 50 | 46 | 0.00 | 0.00 | 2.10 | 1.94 | 90 | 92 | 0.00 | 0.00 | 1.81 | 1.86 |

Table 4. Δp, Δt, and Δfd between the farthest and closest UEs: 2 GHz, 120 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 50 | 281 | 144.43 | 816.01 | 0.55 | 2.12 | 90 | 465 | 259.98 | 1350.95 | 0.57 | 2.03 |
| 40° | 50 | 150 | 127.74 | 384.43 | 0.83 | 2.10 | 90 | 268 | 229.94 | 688.62 | 0.80 | 2.01 |
| 50° | 50 | 95 | 107.17 | 204.25 | 1.17 | 2.07 | 90 | 179 | 192.92 | 384.24 | 1.09 | 1.98 |
| 60° | 50 | 68 | 83.35 | 114.43 | 1.53 | 2.04 | 90 | 133 | 150.05 | 221.90 | 1.38 | 1.96 |
| 70° | 50 | 55 | 57.00 | 62.69 | 1.85 | 2.01 | 90 | 108 | 102.62 | 123.80 | 1.63 | 1.93 |
| 80° | 50 | 48 | 28.94 | 27.98 | 2.05 | 1.98 | 90 | 96 | 52.10 | 55.80 | 1.78 | 1.90 |
| 90° | 50 | 46 | 0.00 | 0.00 | 2.11 | 1.94 | 90 | 92 | 0.00 | 0.00 | 1.82 | 1.86 |

Table 5. Δp, Δt, and Δfd between the farthest and closest UEs: 2 GHz, 1500 km/h, hue=10 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 50 | 245 | 144.65 | 805.89 | 2.77 | 4.15 | 90 | 440 | 260.38 | 1343.73 | 2.78 | 3.89 |
| 40° | 50 | 128 | 127.94 | 378.94 | 2.80 | 3.89 | 90 | 251 | 230.30 | 684.14 | 2.77 | 3.66 |
| 50° | 50 | 80 | 107.33 | 201.11 | 2.83 | 3.58 | 90 | 166 | 193.21 | 381.44 | 2.74 | 3.37 |
| 60° | 50 | 57 | 83.47 | 112.60 | 2.84 | 3.21 | 90 | 122 | 150.26 | 220.17 | 2.67 | 3.03 |
| 70° | 50 | 46 | 57.08 | 61.66 | 2.75 | 2.81 | 90 | 100 | 102.76 | 122.80 | 2.51 | 2.66 |
| 80° | 50 | 40 | 28.97 | 27.52 | 2.53 | 2.38 | 90 | 88 | 52.16 | 55.34 | 2.24 | 2.27 |
| 90° | 50 | 39 | 0.01 | 0.00 | 2.26 | 1.94 | 90 | 85 | 0.02 | 0.00 | 1.95 | 1.86 |

Table 6. Δp, Δt, and Δfd between the farthest and closest UEs: 30 GHz, 0 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 20 | 109 | 57.77 | 315.04 | 0.14 | 0.78 | 40 | 181 | 115.55 | 522.61 | 0.17 | 0.75 |
| 40° | 20 | 59 | 51.10 | 151.68 | 0.26 | 0.78 | 40 | 106 | 102.21 | 271.78 | 0.28 | 0.75 |
| 50° | 20 | 38 | 42.88 | 81.15 | 0.41 | 0.78 | 40 | 71 | 85.76 | 152.64 | 0.42 | 0.75 |
| 60° | 20 | 27 | 33.35 | 45.59 | 0.57 | 0.78 | 40 | 53 | 66.71 | 88.39 | 0.56 | 0.75 |
| 70° | 20 | 22 | 22.81 | 25.01 | 0.71 | 0.78 | 40 | 43 | 45.63 | 49.37 | 0.69 | 0.75 |
| 80° | 20 | 19 | 11.58 | 11.17 | 0.81 | 0.78 | 40 | 38 | 23.17 | 22.27 | 0.78 | 0.75 |
| 90° | 20 | 18 | 0.00 | 0.00 | 0.84 | 0.78 | 40 | 37 | 0.00 | 0.00 | 0.81 | 0.75 |

Table 7. Δp, Δt, and Δfd between the farthest and closest UEs: 30 GHz, 3 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 20 | 109 | 57.77 | 315.04 | 0.15 | 0.78 | 40 | 181 | 115.55 | 522.61 | 0.17 | 0.75 |
| 40° | 20 | 59 | 51.10 | 151.68 | 0.27 | 0.78 | 40 | 106 | 102.21 | 271.78 | 0.29 | 0.75 |
| 50° | 20 | 38 | 42.88 | 81.15 | 0.41 | 0.78 | 40 | 71 | 85.76 | 152.64 | 0.42 | 0.75 |
| 60° | 20 | 27 | 33.35 | 45.59 | 0.57 | 0.78 | 40 | 53 | 66.71 | 88.39 | 0.57 | 0.75 |
| 70° | 20 | 22 | 22.81 | 25.01 | 0.71 | 0.78 | 40 | 43 | 45.63 | 49.37 | 0.69 | 0.75 |
| 80° | 20 | 19 | 11.58 | 11.17 | 0.81 | 0.78 | 40 | 38 | 23.17 | 22.27 | 0.78 | 0.75 |
| 90° | 20 | 18 | 0.00 | 0.00 | 0.84 | 0.78 | 40 | 37 | 0.00 | 0.00 | 0.81 | 0.75 |

Table 8. Δp, Δt, and Δfd between the farthest and closest UEs: 30 GHz, 120 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 20 | 109 | 57.77 | 315.04 | 0.34 | 0.95 | 40 | 181 | 115.55 | 522.61 | 0.36 | 0.91 |
| 40° | 20 | 59 | 51.10 | 151.68 | 0.43 | 0.93 | 40 | 106 | 102.21 | 271.78 | 0.45 | 0.89 |
| 50° | 20 | 38 | 42.88 | 81.15 | 0.55 | 0.91 | 40 | 71 | 85.76 | 152.64 | 0.56 | 0.87 |
| 60° | 20 | 27 | 33.35 | 45.59 | 0.68 | 0.88 | 40 | 53 | 66.71 | 88.39 | 0.67 | 0.84 |
| 70° | 20 | 22 | 22.81 | 25.01 | 0.79 | 0.85 | 40 | 43 | 45.63 | 49.37 | 0.77 | 0.81 |
| 80° | 20 | 19 | 11.58 | 11.17 | 0.84 | 0.81 | 40 | 38 | 23.17 | 22.27 | 0.81 | 0.78 |
| 90° | 20 | 18 | 0.00 | 0.00 | 0.84 | 0.78 | 40 | 37 | 0.00 | 0.00 | 0.81 | 0.75 |

Table 9. Δp, Δt, and Δfd between the farthest and closest UEs: 30 GHz, 1500 km/h, hue=10 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 20 | 108 | 57.86 | 311.11 | 2.55 | 2.98 | 40 | 180 | 115.73 | 519.81 | 2.57 | 2.77 |
| 40° | 20 | 58 | 51.18 | 149.51 | 2.40 | 2.73 | 40 | 106 | 102.37 | 270.01 | 2.41 | 2.54 |
| 50° | 20 | 37 | 42.95 | 79.90 | 2.20 | 2.41 | 40 | 71 | 85.89 | 151.53 | 2.21 | 2.25 |
| 60° | 20 | 27 | 33.40 | 44.86 | 1.97 | 2.05 | 40 | 53 | 66.81 | 87.70 | 1.96 | 1.92 |
| 70° | 20 | 22 | 22.85 | 24.60 | 1.67 | 1.65 | 40 | 43 | 45.70 | 48.97 | 1.65 | 1.55 |
| 80° | 20 | 19 | 11.60 | 10.98 | 1.30 | 1.22 | 40 | 38 | 23.20 | 22.08 | 1.27 | 1.15 |
| 90° | 20 | 18 | 0.00 | 0.00 | 0.91 | 0.78 | 40 | 37 | 0.00 | 0.00 | 0.87 | 0.75 |

Table 10. Δp, Δt, and Δfd between the farthest and reference location: 2 GHz, 0 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 25 | 164 | 72.79 | 487.17 | 0.17 | 0.97 | 45 | 270 | 131.50 | 801.15 | 0.18 | 0.93 |
| 40° | 25 | 82 | 64.62 | 216.30 | 0.32 | 0.97 | 45 | 147 | 116.64 | 386.92 | 0.31 | 0.93 |
| 50° | 25 | 50 | 54.61 | 111.63 | 0.50 | 0.97 | 45 | 95 | 98.50 | 210.29 | 0.46 | 0.93 |
| 60° | 25 | 36 | 43.01 | 61.78 | 0.69 | 0.97 | 45 | 69 | 77.52 | 120.16 | 0.62 | 0.93 |
| 70° | 25 | 28 | 30.13 | 33.97 | 0.87 | 0.97 | 45 | 55 | 54.24 | 67.41 | 0.76 | 0.93 |
| 80° | 25 | 24 | 16.30 | 15.83 | 1.00 | 0.97 | 45 | 49 | 29.28 | 31.86 | 0.86 | 0.93 |
| 90° | 25 | 23 | 1.90 | 1.62 | 1.05 | 0.97 | 45 | 46 | 3.34 | 3.53 | 0.91 | 0.93 |

Table 11. Δp, Δt, and Δfd between the farthest and the reference location: 2 GHz, 3 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 25 | 164 | 72.79 | 487.17 | 0.18 | 0.97 | 45 | 270 | 131.50 | 801.15 | 0.18 | 0.93 |
| 40° | 25 | 82 | 64.62 | 216.30 | 0.32 | 0.97 | 45 | 147 | 116.64 | 386.92 | 0.31 | 0.93 |
| 50° | 25 | 50 | 54.61 | 111.63 | 0.50 | 0.97 | 45 | 95 | 98.50 | 210.29 | 0.46 | 0.93 |
| 60° | 25 | 36 | 43.01 | 61.78 | 0.69 | 0.97 | 45 | 69 | 77.52 | 120.16 | 0.62 | 0.93 |
| 70° | 25 | 28 | 30.13 | 33.97 | 0.87 | 0.97 | 45 | 55 | 54.24 | 67.41 | 0.76 | 0.93 |
| 80° | 25 | 24 | 16.30 | 15.83 | 1.00 | 0.97 | 45 | 49 | 29.28 | 31.86 | 0.86 | 0.93 |
| 90° | 25 | 23 | 1.90 | 1.62 | 1.05 | 0.97 | 45 | 46 | 3.34 | 3.53 | 0.91 | 0.93 |

Table 12. Δp, Δt, and Δfd between the farthest and the reference location: 2 GHz, 120 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 25 | 164 | 72.79 | 487.17 | 0.27 | 1.06 | 45 | 270 | 131.50 | 801.15 | 0.28 | 1.02 |
| 40° | 25 | 82 | 64.62 | 216.30 | 0.40 | 1.05 | 45 | 147 | 116.64 | 386.92 | 0.39 | 1.01 |
| 50° | 25 | 50 | 54.61 | 111.63 | 0.57 | 1.04 | 45 | 95 | 98.50 | 210.29 | 0.53 | 1.00 |
| 60° | 25 | 36 | 43.01 | 61.78 | 0.75 | 1.03 | 45 | 69 | 77.52 | 120.16 | 0.68 | 0.98 |
| 70° | 25 | 28 | 30.13 | 33.97 | 0.91 | 1.01 | 45 | 55 | 54.24 | 67.41 | 0.80 | 0.97 |
| 80° | 25 | 24 | 16.30 | 15.83 | 1.02 | 0.99 | 45 | 49 | 29.28 | 31.86 | 0.89 | 0.95 |
| 90° | 25 | 23 | 1.90 | 1.62 | 1.06 | 0.98 | 45 | 46 | 3.34 | 3.53 | 0.91 | 0.94 |

Table 13. Δp, Δt, and Δfd between the farthest and the reference location: 2 GHz, 1500 km/h, hue =10km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 25 | 162 | 72.80 | 481.24 | 1.39 | 2.13 | 45 | 268 | 131.50 | 796.91 | 1.40 | 2.00 |
| 40° | 25 | 81 | 64.63 | 213.22 | 1.41 | 2.00 | 45 | 146 | 116.65 | 384.40 | 1.39 | 1.88 |
| 50° | 25 | 50 | 54.62 | 109.91 | 1.43 | 1.84 | 45 | 94 | 98.51 | 208.75 | 1.39 | 1.74 |
| 60° | 25 | 35 | 43.03 | 60.78 | 1.44 | 1.66 | 45 | 68 | 77.54 | 119.22 | 1.36 | 1.57 |
| 70° | 25 | 28 | 30.15 | 33.41 | 1.41 | 1.46 | 45 | 55 | 54.26 | 66.86 | 1.30 | 1.39 |
| 80° | 25 | 24 | 16.32 | 15.56 | 1.32 | 1.25 | 45 | 48 | 29.31 | 31.59 | 1.17 | 1.19 |
| 90° | 25 | 23 | 1.93 | 1.60 | 1.13 | 1.02 | 45 | 46 | 3.37 | 3.49 | 0.98 | 0.99 |

Table 14. Δp, Δt, and Δfd between the farthest and the reference location: 30 GHz, 0 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 10 | 58 | 28.98 | 169.67 | 0.07 | 0.39 | 20 | 96 | 58.07 | 280.59 | 0.08 | 0.37 |
| 40° | 10 | 31 | 25.67 | 79.64 | 0.13 | 0.39 | 20 | 55 | 51.43 | 142.61 | 0.14 | 0.37 |
| 50° | 10 | 19 | 21.60 | 42.08 | 0.20 | 0.39 | 20 | 36 | 43.28 | 79.21 | 0.21 | 0.37 |
| 60° | 10 | 14 | 16.89 | 23.52 | 0.28 | 0.39 | 20 | 27 | 33.85 | 45.66 | 0.28 | 0.37 |
| 70° | 10 | 11 | 11.67 | 12.92 | 0.35 | 0.39 | 20 | 22 | 23.39 | 25.57 | 0.34 | 0.37 |
| 80° | 10 | 10 | 6.08 | 5.88 | 0.40 | 0.39 | 20 | 19 | 12.22 | 11.77 | 0.39 | 0.37 |
| 90° | 10 | 9 | 0.30 | 0.26 | 0.42 | 0.39 | 20 | 18 | 0.66 | 0.56 | 0.40 | 0.37 |

Table 15. Δp, Δt, and Δfd between the farthest and the reference location: 30 GHz, 3 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 10 | 58 | 28.98 | 169.67 | 0.07 | 0.39 | 20 | 96 | 58.07 | 280.59 | 0.08 | 0.37 |
| 40° | 10 | 31 | 25.67 | 79.64 | 0.13 | 0.39 | 20 | 55 | 51.43 | 142.61 | 0.14 | 0.37 |
| 50° | 10 | 19 | 21.60 | 42.08 | 0.20 | 0.39 | 20 | 36 | 43.28 | 79.21 | 0.21 | 0.37 |
| 60° | 10 | 14 | 16.89 | 23.52 | 0.28 | 0.39 | 20 | 27 | 33.85 | 45.66 | 0.28 | 0.37 |
| 70° | 10 | 11 | 11.67 | 12.92 | 0.35 | 0.39 | 20 | 22 | 23.39 | 25.57 | 0.34 | 0.37 |
| 80° | 10 | 10 | 6.08 | 5.88 | 0.40 | 0.39 | 20 | 19 | 12.22 | 11.77 | 0.39 | 0.37 |
| 90° | 10 | 9 | 0.30 | 0.26 | 0.42 | 0.39 | 20 | 18 | 0.66 | 0.56 | 0.40 | 0.37 |

Table 16. Δp, Δt, and Δfd between the farthest and the reference location: 30GHz, 120 km/h, hue=0 km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 10 | 58 | 28.98 | 169.67 | 0.17 | 0.48 | 20 | 96 | 58.07 | 280.59 | 0.18 | 0.46 |
| 40° | 10 | 31 | 25.67 | 79.64 | 0.22 | 0.47 | 20 | 55 | 51.43 | 142.61 | 0.22 | 0.45 |
| 50° | 10 | 19 | 21.60 | 42.08 | 0.28 | 0.46 | 20 | 36 | 43.28 | 79.21 | 0.28 | 0.43 |
| 60° | 10 | 14 | 16.89 | 23.52 | 0.34 | 0.44 | 20 | 27 | 33.85 | 45.66 | 0.34 | 0.42 |
| 70° | 10 | 11 | 11.67 | 12.92 | 0.39 | 0.42 | 20 | 22 | 23.39 | 25.57 | 0.38 | 0.41 |
| 80° | 10 | 10 | 6.08 | 5.88 | 0.42 | 0.41 | 20 | 19 | 12.22 | 11.77 | 0.41 | 0.39 |
| 90° | 10 | 9 | 0.30 | 0.26 | 0.42 | 0.39 | 20 | 18 | 0.66 | 0.56 | 0.41 | 0.37 |

Table 17. Δp, Δt, and Δfd between the farthest and the reference location: 30 GHz, 1500 km/h, hue=10km

|  | Set-1 LEO 600km |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  | Δp (km) |  | Δt (μs) |  | Δfd (ppm) |  |
| Elevation | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 30° | 10 | 57 | 28.98 | 167.57 | 1.28 | 1.51 | 20 | 96 | 58.07 | 279.10 | 1.29 | 1.41 |
| 40° | 10 | 30 | 25.67 | 78.50 | 1.20 | 1.38 | 20 | 55 | 51.43 | 141.68 | 1.21 | 1.29 |
| 50° | 10 | 19 | 21.61 | 41.44 | 1.11 | 1.23 | 20 | 36 | 43.28 | 78.63 | 1.12 | 1.15 |
| 60° | 10 | 14 | 16.89 | 23.14 | 1.00 | 1.05 | 20 | 27 | 33.85 | 45.31 | 1.00 | 0.98 |
| 70° | 10 | 11 | 11.67 | 12.71 | 0.85 | 0.84 | 20 | 22 | 23.40 | 25.36 | 0.84 | 0.79 |
| 80° | 10 | 10 | 6.09 | 5.78 | 0.67 | 0.63 | 20 | 19 | 12.23 | 11.67 | 0.66 | 0.60 |
| 90° | 10 | 9 | 0.31 | 0.25 | 0.45 | 0.41 | 20 | 18 | 0.67 | 0.56 | 0.44 | 0.39 |

Observation 2: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between UEs at the farthest and closest points from the satellite for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows:

|  | Set-1 LEO 600km |  |  |  | Set-1 LEO 1200km |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δp (km) |  | Δt (μs) |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 50 | 281 | 144.43 | 816.01 | 90 | 465 | 259.98 | 1350.95 |
| 30 GHz | 20 | 109 | 57.77 | 315.04 | 40 | 181 | 115.55 | 522.61 |

| Δfd (ppm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set-1 LEO 600km |  |  |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |  |  |
|  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 2.1 | 1.94 | 2.1 | 1.95 | 2.11 | 2.12 | 2.84 | 4.15 | 1.81 | 1.86 | 1.81 | 1.87 | 1.82 | 2.03 | 2.78 | 3.89 |
| 30 GHz | 0.84 | 0.78 | 0.87 | 0.78 | 0.84 | 0.95 | 2.55 | 2.98 | 0.81 | 0.75 | 0.81 | 0.75 | 0.81 | 0.91 | 2.57 | 2.77 |

Observation 3: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between the farthest UE and the reference location for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows:

|  | Set-1 LEO 600km |  |  |  | Set-1 LEO 1200km |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δp (km) |  | Δt (μs) |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 25 | 164 | 72.8 | 487.17 | 45 | 270 | 131.5 | 801.15 |
| 30 GHz | 10 | 58 | 29 | 169.67 | 20 | 96 | 58.07 | 280.59 |

| Δfd (ppm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set-1 LEO 600km |  |  |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |  |  |
|  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 1.05 | 0.97 | 1.05 | 0.97 | 1.05 | 1.06 | 1.44 | 2.13 | 0.91 | 0.93 | 0.91 | 0.93 | 0.91 | 1.02 | 1.4 | 2 |
| 30 GHz | 0.42 | 0.39 | 0.42 | 0.39 | 0.42 | 0.48 | 1.28 | 1.51 | 0.4 | 0.37 | 0.4 | 0.37 | 0.41 | 0.46 | 1.29 | 1.41 |

Proposal 6: For the location uncertainty area defined as a circle of radius X km,

X = 164 km for Set-1 LEO 600km

X = 270 km for Set-1 LEO 1200km

Proposal 7: RAN1 to consider the following values of maximum differential delay (Δt),

Δt = 487 μs for Set-1 LEO 600km

Δt = 800 μs for Set-1 LEO 1200km

Proposal 8: RAN1 to consider the following values of maximum differential Doppler (Δfd),

Δfd = 1.51 ppm for Set-1 LEO 600km

Δfd = 1.41 ppm for Set-1 LEO 1200km

Scenario

For the study on GNSS resilient operation, the following agreement was made in RAN1#122 [2].

Agreement For the study on GNSS resilient operation, scenarios should be considered where: - Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link. - Scenario 2: There is a previously acquired GNSS based position - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded - FFS: value of T - FFS: GNSS based UE location validity duration - FFS: GNSS accuracy - Note: Two scenarios above belong to description as in SID.

As discussed in the above sections, the following two aspects are observed:

- In real world, GNSS unavailability may last more than hours

The maximum differential position (Δp), delay (Δt), and the differential Doppler (Δfd) values can be very large, e.g., the differential position can be multiple times of the radius of Nadir beam

In this regard, we think Scenario 1 should be the baseline and Scenario 2 can be optionally considered during this SI.

Observation 4: Regarding the GNSS unavailability in real world,

- GNSS unavailability may last more than hours

- The maximum differential position (Δp), delay (Δt), and the differential Doppler (Δfd) values can be very large, e.g., the differential position can be multiple times of the radius of Nadir beam

Proposal 9: RAN1 to consider the Scenario 1 as baseline

- Scenario 2 can be optionally considered during this SI

Potential solutions

For the study on GNSS resilient operation, the following solutions were observed in RAN1#122bis [3].

Observation: For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis: - Solution 1: To improve robustness: - Solutions based on modified random access procedures, using existing PRACH formats: - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats - Solution 1B: New PRACH restricted sets. - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) - Solution 1E: Selection of a set of existing PRACH root - Solution 1F: Using TA margin and RO masking. - Other solutions are not precluded - Solution 2: Solutions based on reducing time-frequency uncertainty - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications. - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Solution 2C: Solutions based on broadcasting DL timestamp(s) - Solution 2D: UE side time/frequency pre-compensation based on reference location. - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position - Solution 2F: UE side time/frequency pre-compensation based on reference signal. - Other solutions are not precluded - Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Other Solutions are not precluded

In this section, we provide a set of potential solutions for GNSS resilient operations. We think full pictures on major concepts of different solutions need to be drawn by the November meeting for comprehensive assessments.

Proposal 10: RAN1 to start discussion on solutions for GNSS resilient operation including:

- Define baseline schemes and evaluation methods

- Impacts on specification

From our first glance, the following options can be considered as implementation-based solutions or non-3GPP-based solutions to address GNSS resilient operations for NTN:

- NTN UL transmission is not allowed when GNSS is unavailable: This is not a new solution, but should be mentioned (and needs to be compared somehow for performance gain evaluation)

- Dual-(multi-)GNSSs: The case that a UE adopts more than two GNSSs

- Dual-(multi-)frequencies: The case that a UE utilizes a GNSS relying on more than two different frequency bands

- OS-NMA (Open Service - Navigation Message Authentication)

- UL time/frequency compensation via DL synchronization: See section 4.1 for details

- LPP-based NTN TA: See section 4.2 for details

Proposal 11: RAN1 to define a baseline scheme for GNSS resilient operation among the following implementation-/non-3GPP-based solutions:

- Option 0: NTN UL transmission is not allowed when GNSS is unavailable (Rel-19 UE behaviour)

- Option 1: Dual-(multi-)GNSSs

- Option 2: Dual-(multi-)frequencies

- Option 3: OS-NMA

- Option 4: UL time/frequency compensation via DL synchronization and signals

- Option 5: LPP-based approach (note: no enhancements on LPP itself)

- FFS, other options and/or a combination above

On top of the implementation-based solutions or non-3GPP-based solutions, solutions with potential specification impacts should be carefully assessed as well. Based on the studies from Rel-15/-16 and the specification supports so far, we think the following solutions can be further studied/evaluated.

- (New) timer (like IoT-NTN): For instance, a new timer to determine GNSS validity can be introduced for NR NTN

- Introduce new timestamp(s) in SIB19 and/or SIB9: See section 4.3 for details

- Reference location-based TA: See section 4.4 for details

- GNSS quality status information: See section 4.5 for details

- Requirement relaxation (RAN4): Legacy RAN4 requirements may need to be relaxed by considering UE location error due to GNSS unavailability

- Gradual timing adjustment (RAN4): See section 4.6 for details

- Expansion of closed-loop compensation into frequency domain: Legacy time-domain closed-loop compensation can be extended to frequency domain as well [5]

- New procedures to detect and/or report GNSS availability: New procedures for GNSS availability detection (and corresponding report on the detected state) may need to be defined, e.g., to trigger GNSS resilient operation

- New procedures to request, and/or configure, indicate GNSS resilient operation(s): Similarly, explicit UE request or gNB configuration/indication on GNSS resilient operation can be required

Proposal 12: RAN1 to study the following specification impacts for GNSS resilient operations:

- Option 1: (New) timer (like IoT-NTN, RAN2)

- Option 2: Introduce new timestamp(s) in SIB19 and/or SIB9 (RAN2, RAN1)

- Option 3: Reference location-based UL synchronization (RAN1, RAN2)

- Option 4: GNSS quality status information (RAN1, RAN2)

- Option 5: Requirement relaxation (RAN4)

- Option 6: Gradual timing adjustment (RAN4)

- Option 7: Expansion of closed-loop compensation into frequency domain (RAN1, RAN2)

- Option 8: New procedures to detect and/or report GNSS availability (RAN1, RAN2)

- Option 9: New procedures to request, configure, and/or indicate GNSS resilient operation(s) (RAN1, RAN2)

- FFS, other options and/or a combination above

UL time/frequency compensation via DL synchronization

The UE implements downlink frequency synchronization. Based on the relationship between the uplink and downlink frequencies, uplink frequency synchronization can also be performed in the UE implementation. If necessary, the network may provide the equation defining the relationship between uplink and downlink frequency synchronization.

LPP-based approach

When GNSS is temporarily unavailable, the positioning protocols, NRPPa (NR Positioning Protocol A) or LPP (LTE Positioning Protocol) can be used for improvement of positioning and time synchronization. The NRPPa-based approach enables network-centric control (gNB or GW), whereas the LPP-based approach operates at the NAS (Non-Access Stratum) protocol layer controlled by the 5G Core (e.g., AMF, SMF) [6], [7].

An LPP-based approach is a closed-based approach when the UE has entered the RRC_CONNECTED state. When identification of GNSS quality prior to RRC_CONNECTED state is needed, it would be useful to measure GNSS quality status during the initial access procedures, and then the network to decide appropriate decision before the actual LPP-based approach is processed.

Figure 6 shows a possible example of LPP-based approaches for positioning and time synchronization in NTN scenarios. As shown in the figure, once a UE evaluates its GNSS reception quality, it is able to report to an LMF (Location Management Function) server. Then, a closed-loop process can be performed between the UE and LMF, as the UE sends capabilities of positioning and time synchronization techniques, and also provides related information of GNSS quality. Based on the information by the UE, the LMF performs needed correction in the closed-loop manner, including correction level, supplying required parameters to the UE, and deciding when to end the process, Finally, the UE performs the correction based on the information from LMF, then report the results to LMF.

Figure 6. An example of LPP-based approaches for GNSS resilient operation

Timestamp-based solution

In the current specification, time information is transmitted through system information such as SIB19 and SIB9. When UE’s GNSS information becomes temporarily unavailable but the outage duration is not long, the time information previously acquired from GNSS can remain valid. The validity period of GNSS time information may vary depending on UE capability. In this case, the UE can utilize a timestamp representing the reception timing of downlink (DL) signals to pre-compensate the uplink timing advance (TA); the propagation delay can be conveyed through the timestamp at the UE, as well as via time information delivered through SIB19, SIB9, or newly defined fields. The following additional information may be considered to improve accuracy:

- Feeder link propagation delay

- Satellite time synchronization errors

- Configuration of validity durations according to UE capabilities

- Reference location and the corresponding propagation delay

- Other relevant parameters

The UE may utilize multiple timestamps not only to improve accuracy but also to estimate its position and velocity. The main advantage of the timestamp-based solution is its limited specification impact, as it merely adds a few new parameters to SIB19.

Reference location-based solution

Uplink synchronization based on reference location (position) has been considered as a potential solution since Rel-16. By pairing of reference location and associated information, the UE can select the reference location closest to its location and perform uplink time and frequency pre-compensation based on that position. For example, if a specific reference location and its corresponding DL frequency correction information are paired and delivered via SIB19, the UE can determine its nearest reference position from the DL frequency correction it performs, and subsequently apply the corresponding pre-compensation. By applying interpolation or similar techniques, the UE can achieve more precise uplink synchronization. The main advantage of the reference position-based solution is its limited specification impact, as it merely adds a few new parameters to SIB19.

GNSS quality status information in initial access procedures

It would be beneficial to study GNSS quality measurement in initial access procedures in order to address situations when GNSS is temporarily unavailable. By reporting GNSS quality information in earlier stages, the network can obtain early awareness of the UE’s GNSS-based synchronization capability and positioning, and therefore, the network can apply differentiated strategies depending on the GNSS quality measures. This approach can be also used with the LPP-based approach stated in section 4.2.

The network may provide information that facilitates measurement and estimation of the GNSS quality at UE, including number of visible GNSS satellites, GNSS satellite IDs, GNSS RSRP/RSRQ, and other related parameters. This information can be applied to both the initial access (including handover) procedure and connected mode.

Gradual timing adjustment

RAN4 has defined the gradual timing adjustment for NTN in order to address double correction issue when closed-loop and open-loop TA are used. While this solution of gradual timing adjustment mitigates the cases under nominal conditions, further study is required to evaluate its robustness in scenarios where GNSS is temporarily unavailable.

Table 18. Aggregate adjustment rate for gradual timing alignment in clause 7.1C.2.1 of TS 38.133 [8].

3b) For a FR2-NTN mobile UE , the maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be 3 * Tq_NTN per 200 ms.

As shown in Table 18, the aggregate adjustment rate for gradual timing alignment is defined up to 3 * Tq_NTN per 200 ms, considering high speed mobility of UEs in NTN scenarios. However, in cases where GNSS is temporarily unavailable, the open-loop timing error may accumulate significantly, particularly for high-speed UEs. As a result, even with the maximum aggregate adjustment rate of 3 * Tq_NTN per 200 ms, the UE may fail to meet the timing error requirement Te_NTN specified in TS 38.133. This is illustrated in the simulation results of Figure 7.

Figure 7. Gradual timing adjustment when GNSS is temporarily unavailable for 30 seconds. The maximum aggregate adjustment rate was 3 * Tq_NTN per 200 ms and mobile UE speed was 195 m/s.

Therefore, a more detailed study is needed to evaluate the behavior and performance of the gradual timing adjustment mechanism under conditions where GNSS is temporarily unavailable. This study aims to assess the potential impact on uplink timing accuracy and identify any enhancements or clarifications required to ensure robust operation in such environments.

Conclusion

In this contribution, ETRI’s views on NR-NTN GNSS resilient operations were shown and the following observation and proposals were made:

Observation 1: The differential beam radius between the beam footprints projected onto the Earth’s surface and the circular beam in Table 6.1.1.1-1 TR 38.821 are non-trivial.Observation 1: The differential beam radius between the beam footprints projected onto the Earth’s surface and the circular beam in Table 6.1.1.1-1 TR 38.821 are non-trivial.

Observation 2: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between UEs at the farthest and closest points from the satellite for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows:

|  | Set-1 LEO 600km |  |  |  | Set-1 LEO 1200km |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δp (km) |  | Δt (μs) |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 50 | 281 | 144.43 | 816.01 | 90 | 465 | 259.98 | 1350.95 |
| 30 GHz | 20 | 109 | 57.77 | 315.04 | 40 | 181 | 115.55 | 522.61 |

| Δfd (ppm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set-1 LEO 600km |  |  |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |  |  |
|  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 2.1 | 1.94 | 2.1 | 1.95 | 2.11 | 2.12 | 2.84 | 4.15 | 1.81 | 1.86 | 1.81 | 1.87 | 1.82 | 2.03 | 2.78 | 3.89 |
| 30 GHz | 0.84 | 0.78 | 0.87 | 0.78 | 0.84 | 0.95 | 2.55 | 2.98 | 0.81 | 0.75 | 0.81 | 0.75 | 0.81 | 0.91 | 2.57 | 2.77 |

Observation 3: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between the farthest UE and the reference location for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows:

|  | Set-1 LEO 600km |  |  |  | Set-1 LEO 1200km |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δp (km) |  | Δt (μs) |  | Δp (km) |  | Δt (μs) |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 25 | 164 | 72.8 | 487.17 | 45 | 270 | 131.5 | 801.15 |
| 30 GHz | 10 | 58 | 29 | 169.67 | 20 | 96 | 58.07 | 280.59 |

| Δfd (ppm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set-1 LEO 600km |  |  |  |  |  |  |  | Set-1 LEO 1200km |  |  |  |  |  |  |  |
|  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  | 0 km/h |  | 3 km/h |  | 120 km/h |  | 1500 km/h |  |
|  | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 | BP 1 | BP 2 |
| 2 GHz | 1.05 | 0.97 | 1.05 | 0.97 | 1.05 | 1.06 | 1.44 | 2.13 | 0.91 | 0.93 | 0.91 | 0.93 | 0.91 | 1.02 | 1.4 | 2 |
| 30 GHz | 0.42 | 0.39 | 0.42 | 0.39 | 0.42 | 0.48 | 1.28 | 1.51 | 0.4 | 0.37 | 0.4 | 0.37 | 0.41 | 0.46 | 1.29 | 1.41 |

Observation 4: Regarding the GNSS unavailability in real world,

- GNSS unavailability may last more than hours

- The maximum differential position (Δp), delay (Δt), and the differential Doppler (Δfd) values can be very large, e.g., the differential position can be multiple times of the radius of Nadir beam

Proposal 1: RAN1 to consider intentional threats, such as jamming and spoofing, as well as environmental impairments for practical GNSS performance degradation modelling.

- Intentional threats on GNSS may last several hours.

Proposal 2: RAN1 to consider the following aspects from UE perspectives for GNSS performance degradation modelling:

- Detection mechanism to clarify GNSS status (and corresponding detection probability/latency on GNSS status)

- Time-To-Recover

- Max position drift / drift variation

- FFS, other aspects

Proposal 3: RAN1 to consider the following aspects from system perspectives for GNSS performance degradation modelling:

- (Timing) pattern of GNSS-unavailability occurrence

- Maximum duration of GNSS-unavailable state (i.e., max outage duration)

- FFS, other aspects

Proposal 4: RAN1 to select Alt 1 for future evaluations:

- The typos in Alt1 from the Agreement need to be corrected (the version from final FLS is correct) as follows:

  - ∆t=dfar-dnearc and ∆fd=fdfar-fdnear

  - fdnear=vssinβ1-vuecosγcfc

  - φ1=φ-2RcRE+hue (for non-nadir beam) and φ=RcRE+hue (for nadir beam)

- It is preferred to capture Alt 1 for the NTN channel model TR

Proposal 5: RAN1 to consider the beam footprints projected onto the Earth’s surface for evaluation of NR-NTN GNSS resilience.

- The values of X and T should be determined by considering the radius of edge beam, rather than by the radius of Nadir beam

Proposal 6: For the location uncertainty area defined as a circle of radius X km,

- X = 164 km for Set-1 LEO 600km

- X = 270 km for Set-1 LEO 1200km

Proposal 7: RAN1 to consider the following values of maximum differential delay (Δt),

- Δt = 487 μs for Set-1 LEO 600km

- Δt = 800 μs for Set-1 LEO 1200km

Proposal 8: RAN1 to consider the following values of maximum differential Doppler (Δfd),

- Δfd = 1.51 ppm for Set-1 LEO 600km

- Δfd = 1.41 ppm for Set-1 LEO 1200km

Proposal 9: RAN1 to consider the Scenario 1 as baseline

- Scenario 2 can be optionally considered during this SI

Proposal 10: RAN1 to start discussion on solutions for GNSS resilient operation including at least:

- Define baseline schemes and evaluation methods

- Impacts on specification

Proposal 11: RAN1 to define a baseline scheme for GNSS resilient operation among the following implementation-/non-3GPP-based solutions:

- Option 0: NTN UL transmission is not allowed when GNSS is unavailable (Rel-19 UE behaviour)

- Option 1: Dual-(multi-)GNSSs

- Option 2: Dual-(multi-)frequencies

- Option 3: OS-NMA

- Option 4: UL time/frequency compensation via DL synchronization and signals

- Option 5: LPP-based approach (note: no enhancements on LPP itself)

- FFS, other options and/or a combination above

Proposal 12: RAN1 to study the following specification impacts for GNSS resilient operations:

- Option 1: (New) timer (like IoT-NTN, RAN2)

- Option 2: Introduce new timestamp(s) in SIB19 and/or SIB9 (RAN2, RAN1)

- Option 3: Reference location-based UL synchronization (RAN1, RAN2)

- Option 4: GNSS quality status information (RAN1, RAN2)

- Option 5: Requirement relaxation (RAN4)

- Option 6: Gradual timing adjustment (RAN4)

- Option 7: Expansion of closed-loop compensation into frequency domain (RAN1, RAN2)

- Option 8: New procedures to detect and/or report GNSS availability (RAN1, RAN2)

- Option 9: New procedures to request, configure, and/or indicate GNSS resilient operation(s) (RAN1, RAN2)

- FFS, other options and/or a combination above

References

- 3GPP RP-251863, Study on GNSS resilient NR-NTN operation, Thales (Moderator)

- 3GPP Chair's notes RAN1#122, 10.6.1 NR-NTN GNSS resilience

- 3GPP Chair's notes RAN1#122bis, 10.6.1 NR-NTN GNSS resilience

- 3GPP TR 38.821, Solutions for NR to support Non-Terrestrial Networks (NTN), 2019.12

- RP-250574, Proposal on improved resilient to GNSS interference, iDirect

- 3GPP TS 38.455, V18.6.0: NG-RAN; NR Positioning Protocol A (NRPPa), 2025.06.

- 3GPP TS 37.355, V18.5.0: LTE Positioning Protocol (LPP), 2025.06.

- 3GPP TS 38.133 V18.10.0: Requirements for support of radio resource management, 2025.06