3GPP TSG RAN WG1 #123			R1-2509316

Dallas, TX, November 17th – 21st, 2025

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN4 LS on RRM agreement update on switching pattern application

Document for:	Discussion, Decision

# 1	Introduction

RAN WG1 received an LS from RAN WG4 with the following information:

RAN4 has further discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following update to RAN1.

Agreement: Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied.

RAN4 requests RAN1 to take this agreement into account in their further work.

# 2	Discussion

[38.213 Clause 24]

A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots.

Based on this agreement, RAN1 will need to update 38.213 Clause 24, since the current version of this specification states that the RRC configured switching pattern is applied only when the SCell is activated. It is silent on the applicability during the activation procedure.

As can be seen in the most recent version of TS 38.133 Clause 8.3 [2], RAN4 specifies the precise time duration (i.e., which slots) the RRC configured switching pattern is considered applicable taking into account the SCell activation status (activated, deactivated, being activated/deactivated). Hence, there is no need for RAN1 specs to repeat this. Instead a reference to the appropriate clause in TS 38.133 Clause 8.3 can be provided regarding the switching pattern applicability. Based on this we propose the following:

RAN1 agrees on the following text proposal to 38.213 Clause 24 which provides a reference to RAN4 specifications on the applicability of the RRC configured switching pattern.

*** Begin TP ***

# 24	Downlink carrier aggregation via switching

A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell. The provided switching pattern is applicable for the time duration for which it is considered active as per TS 38.133 Clause 8.3 . The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots.

*** End TP ***

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 agrees on the above text proposal to 38.213 Clause 24

# References

R1-2508315 (R4-2514847) “LS of RAN4 RRM agreement update on switching pattern application”, RAN4, RAN1#123, November 2025.

3GPP TS 38.133, “Requirements for support of radio resource management,” v19.2.0, September 2025.