3GPP TSG-RAN WG1 Meeting # 123	R1-2509551

# Dallas, Texas, USA, November 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | 0034 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UL and DL reciprocity modelling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, InterDigital, Intel, Nokia, vivo, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_6GHz_CH_model, TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reciprocity requires using the same channel impulse response if transmitter and receiver switch their roles. Reciprocity between UL and DL channels is now supported in 38.901 by swapping arrival and departure parameters. However, it may be wrongly interpreted as swapping arrival and departure angles, resulting in the situation that BS and UT use each other’s angular spread. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ideal channel reciprocity is defined as using the same DL channel impulse response between two transmit antenna elements for UL, if the two antenna elements switch their roles of transmitter and receiver. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, UL channel impulse reponse may be generated by swapping arrival and departure angles of DL channel impulse reponse, resulting in the situation that BS and UT use each other’s angular spread. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis: This CR has isolated impact as it corrects an erroneous reference. The CR has no direct impact on UE implementation or network implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.5	Fast fading model

The radio channel realizations are created using the parameters listed in Table 7.5-1. The channel realizations are obtained by a step-wise procedure illustrated in Figure 7.5-1 and described below. It has to be noted that the geometric description covers arrival angles from the last bounce scatterers and respectively departure angles to the first scatterers interacted from the transmitting side. The propagation between the first and the last interaction is not defined. Thus, this approach can model also multiple interactions with the scattering media. This indicates also that e.g., the delay of a multipath component cannot be determined by the geometry. In the following steps, downlink is assumed. For uplink,it is assumed that BS receives with antenna element s and UT transmits with antenna element u. If UT transmits and receives with same antenna element u and BS transmits and receives with same antenna element s, use the downlink channel impulse response between transmit antenna element s and receive antenna element u for uplink, under ideal reciprocity condition.

< Unchanged parts are omitted >