# 3GPP TSG RAN WG1 #123																	         R1-2509570

# Dallas, US, Nov 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0708 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SRS port(s) mapping on OFDM symbol(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Till Rel-18, all SRS port(s) of an SRS resource are mapped to each symbol of the resource. However, in Rel-18, TDMed SRS port mapping is supported for 8-port SRS resource. When the RRC parameter nrofSRS-Ports-n8 is set to ports8 for an SRS resource, all the 8 SRS ports are mapped to each (S = 1) symbol of the resource. While when RRC parameter nrofSRS-Ports-n8 is set to ports8tdm for an SRS resource, the 8 SRS port(s) are mapped to every S = 2 symbols of the resource in a TDM manner. In clause 6.4.1.4 of TS 38.211, the TDM SRS port mapping scheme is clearly specified. However, current description in clause 6.2.1 of TS 38.214 does NOT capture the case where 8 SRS ports in an SRS resource are mapped to every S = 2 symbols of the resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that SRS port(s) of an SRS resource are mapped to every S symbol(s) of the resource according to clause 6.4.1.4 of TS 38.211. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current description would cause wrong UE behavior when nrofSRS-Ports-n8 is set to ports8tdm for an SRS resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.1	UE sounding procedure

<Unchanged part omitted>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to every S symbol(s) of the resource according to clause 6.4.1.4 of [4, TS 38.211]. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot. NS is divisible by S∗R, where S = 2 when nrofSRS-Ports-n8 is set to ports8tdm is configured and S = 1 otherwise, and R is the repetition factor.

<Unchanged part omitted>