3GPP TSG-RAN WG1 Meeting #123	R1-2509573

Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0165 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on sequence generation for uplink DM-RS in RACH-less HO in TS 38.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno, vivo, MediaTek Inc., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The configured grant based PUSCH transmission in NTN, TN, and mobile-IAB RACH-less handover is supported, and the mapping between the SS/PBCH block(s) and a PUSCH occasion and DM-RS is specified in section 22.1 of TS 38.213. However, in RACH-less handover, the value of nSCID used for initializing the pseudo-random sequence generator (for DMRS for PUSCH) when transform precoding is disabled, is unspecified in TS 38.211. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For sequence generation for a configured-grant based PUSCH transmission in RACH-less handover, it is specified that the quantity nSCID∈0,1 is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DMRS resource in accordance with Clause 22.1 of TS 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value of nSCID for initializing the pseudo-random sequence generator (for DMRS for PUSCH) for the configured grant based PUSCH transmission in RACH-less handover is not aligned with the CG RRC configuration (CG-RRC-Configuration-r18). Multiple DM-RS sequences will not be supported for RACH-less. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***********************Start of Changes*************************

6.4.1.1.1.1	Sequence generation when transform precoding is disabled

If transform precoding for PUSCH is not enabled, the sequence  shall be generated according to

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231

where  is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1, 0_2, or 0_3, or by a PUSCH transmission with a configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NID0,NID1∈0,1,…,65535 are, for each msgA PUSCH configuration, given by the higher-layer parameters msgA-ScramblingID0 and msgA-ScramblingID1, respectively, in the msgA-DMRS-Config IE if provided and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213];

-	NIDnSCIDλ=NIDcell  otherwise;

-	nSCIDλ  and λ are given by

-	if the higher-layer parameter dmrs-Uplink in the DMRS-UplinkConfig IE is provided

nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1
λ=λ

where λ is the CDM group defined in clause 6.4.1.1.3.

-	otherwise

nSCIDλ=nSCID
λ=0

The quantity nSCID∈0,1 is

-	indicated by the DM-RS initialization field, if present, either in the DCI associated with the PUSCH transmission if DCI format 0_1, 0_2, or 0_3, in [4, TS 38.212] is used;

-	indicated by the higher layer parameter dmrs-SeqInitialization, if present, for a Type 1 PUSCH transmission with a configured grant;

-	determined by the mapping between preamble(s) and a PUSCH occasion and the associated DMRS resource for a PUSCH transmission of Type-2 random access process in [5, TS 38.213];

-	determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DMRS resource for a configured-grant based PUSCH transmission in RRC_INACTIVE state [5, TS 38.213];

-	determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DMRS resource for a configured-grant based PUSCH transmission in RACH-less handover [5, TS 38.213];

-	otherwise nSCID=0.

***********************End of Changes*************************