3GPP TSG RAN WG1 #123	R1-2509603

Dallas, USA, Nov. 17th-21st

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0737 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | CR for TS 38.213 on RACH-less handover |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  | NEC, vivo, Samsung, Ericsson |  |  |  |  |  |  |  |
| Source to TSG: |  | R1 |  |  |  |  |  |  |  |
| Work item code: |  | NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18 |  |  |  |  | Date: |  | 2025-11-17 |
| Category: |  | F |  |  |  |  | Release: |  | Rel-18 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: | Firstly, based on the LS R1-2505120, the RACH-less handover applies to NTN UEs and TN UEs (including UEs on mobile IAB). However, in current RAN1 specifications, the description of RACH-less handover is only applicable for the NTN. It cannot be applied to the RACH-less handover in mIAB and TN. In addition, the mIAB and TN can also be applicable in TDD bands, the PO validation behaviour in TDD band for RACH-less handover is needed. Secondly, for power control, the downlink pathloss estimate for RACH-less handover is determined only using a RS resource from the SSB index provided by RACH-lessHO, which cannot support the case where the pathloss is estimated based on a RS indicated via the TCI state for TN and mobile-IAB. |  |  |  |  |  |  |  |  |
| Summary of change: | - Add descriptions for the case that an RS configured by a TC state is provided for power control in clause 7.1.1. - Remove the “ NTN” in “22 PUSCH transmission in NTN RACH-less handover” to cover the PUSCH transmission procedure in both NTN and TN (including mobile IAB) RACH-less handover. - Add the description in case the tci-StateID is provided in RACH-LessHO in clause “22.2 Dynamic-grant PUSCH transmission” - Add the description of PO validation in TDD band for RACH-less handover, and add the case that an RS configured by a TC state is provided for power determination in clause 22.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: | RACH-less handover for mobile IAB or TN can not be supported by RAN1 specification. RAN1 and RAN2 specifications are not consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: | 7.1.1, 22, 21.1, 22.2 |  |  |  |  |  |  |  |  |
|  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

### 7.1.1	UE behaviour

============== unchanged parts are omitted ===============

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c

-	If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd)

-	using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB, or using the SS/PBCH block the UE acquired the time and frequency synchronization for a secondary cell.

-	if the UE is provided RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync, or using a RS resource from the pathlossReferenceRS-Id-r17 configured by a TCI state associated with tci-StateID.

============== unchanged parts are omitted ===============

# 22	PUSCH transmission in RACH-less handover

22.1	Configured-grant PUSCH transmission

A UE indicated to perform PUSCH transmission in RACH-less handover can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the active UL BWP [12, TS 38.331]. For the remaining of this clause, PUSCH transmissions refer to configured grant Type-1 PUSCH transmissions for a configuration provided by ConfiguredGrantConfig.

A UE can be provided by rrc-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided rrc-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. A UE can be provided a number of repetitions for a PUSCH transmission by repK or numberOfRepetitions. If the number of repetitions is provided and larger than 1, all the PUSCH occasions of the repetitions for the PUSCH transmission are mapped to the same SS/PBCH block index(es). For the initial transmission or autonomous retransmission of an initial transport block provided for PUSCH transmission in RACH-less handover, the UE encodes the transport block using redundancy version number 0 if the UE is not provided repK-RV.

An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to valid PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to valid PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by rrc-SSB-PerCG-PUSCH. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions.

Each N of NPUSCHSS/PBCH SS/PBCH block indexes in increasing order are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	second, in increasing order of PUSCH configuration period indexes

where N is provided by rrc-SSB-PerCG-PUSCH.

A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.

For unpaired spectrum and for SS/PBCH blocks with indexes provided by rrc-SSB-Subset in cg-RRC-Configuration, or ssb-PositionsInBurst in ServingCellConfigCommon if rrc-SSB-Subset is not provided

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols

-	starts at least Ngap symbols after a last downlink symbol, and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2.

A UE determines a power of a PUSCH transmission as described in clause 7.1.1, where the UE obtains PLb,f,c(qd) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission or the pathlossReferenceRS-Id-r17 configured by a TCI state associated with tci-StateID.

22.2	Dynamic-grant PUSCH transmission

If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS configured by a TCI state associated with tci-StateID are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.