3GPP TSG RAN WG1 Meeting #66	R1-112331

Athens, Greece, 22nd - 26th August, 2011

Agenda Item:	6.3.1

Source:	LG Electronics

Title:	Performance Evaluation in Heterogeneous Networks

Document for:	Discussion and Decision

Introduction

In RP-110824 (WID) [1], issues for identifying the scenarios of further enhanced non-CS-based ICIC for LTE were presented and the brief summary is the same as following.

- Based on system performance gains identify the scenarios for which the following UE performance requirements will be specified, in terms of, e.g., number of interferers and their relative levels with respect to the serving cell.

- UE performance requirements to enable significantly improved detection of PCI and system information (MIB/SIB-1/Paging) in the presence of dominant interferers for FDD and TDD systems, and different network configurations (e.g., subframe offset / no-subframe offset).

- UE performance requirements for significantly improved DL control and data detection and UE measurement/reporting in the presence of dominant interferers (including colliding and non-colliding RS, as well as, MBSFN used as ABS, as well as, ABS subframe configurations) for FDD and TDD systems.

  - Dominant interference applicable to both macro-pico and CSG scenarios and  with or without handover biasing

In this contribution, we evaluated the system performance between "MeNBs and CSG HeNBs" and between "MeNB and outdoor pico-nodes" under the condition of co-channel deployment. Also, we discussed the scenarios of further enhanced non-CS-based ICIC for LTE based on the results of these simulations.

Scenario of MeNBs and CSG HeNBs

Simulation Assumptions

System Model

The basic simulation assumptions are set according to 3GPP TR 36.814 [2], assuming the dense urban dual stripe model and path loss model 1 for the HeNBs. Some specific parameters are listed in Table 1.

Table 1: Dense Urban Dual Stripe Model parameters

| N (number of apartments per row) | 10 |
| --- | --- |
| M (number of blocks per cell) | 1 |
| L (maximum number of floors per block) | 6 |
| R (deployment ratio ) | 0.1 |
| P (activation ratio) | 100% |
| Probability of MUE being indoor | 35% |

We assumed in the simulation scenarios that maximum and minimum HeNB transmit powers were 20dBm and 0dBm, respectively. Both MeNBs and HeNBs operate in the same 10 MHz bandwidth and full-buffer traffic model is used. There are 30 UEs dropped within each macro geographical area and both MeNBs and HeNBs independently allocate radio resources to their UEs using a time and frequency selective Proportional Fair scheduler.

HeNB Power Setting

Downlink HeNB power was set according to the method in R4-113320 [3] as agreed in RAN4 #59 meeting. It is explained in the followings.

- CRS Êc, measured in dBm, is the reference signal received power (RSRP) per resource element present at the Home BS antenna connector received from the co-channel Wide Area BS.

- Ioh, measured in mW, is the total received DL power, including all interference but excluding the own Home BS signal, present at the Home BS antenna connector on the Home BS operating channel.

- is the number of downlink resource blocks in the own Home BS channel.

- is the number of subcarriers in a resource block, .

- The output power, Pout, is the sum of transmits power across all the antennas of the Home BS, with each transmit power measured at the respective antenna connectors.

Table 2: HeNB Output Power for Co-channel E-UTRA Channel Protection

| Input Conditions | Output power, Pout |
| --- | --- |
| Ioh (DL) > CRS Êc + 10log10() + [30] dB and [Option 1]: CRS Êc ≥ -127 dBm | ≤ [10] dBm |
| Ioh (DL) ≤ CRS Êc + 10log10() + [30] dB and [Option 1]: CRS Êc ≥ -127 dBm | ≤ max (Pmin, min (Pmax, CRS Êc + 10log10() + X )) [40] dB ≤ X ≤ [90] dB |

HeNB adjusts its maximum DL transmit power according to the output power described in Table 2. Here, the output power is the value determined after considering all of the strongest receiving power of MeNB and the total received DL power (, including all interference but excluding the own HeNB signal) at the HeNB. Also, parameter X that is set to control the output power of HeNB is a value determined from the network side.

In the simulation, X was separately set at 50dB, 60dB, or 70dB and each case was evaluated for its performance.

Coordination for DL Data Channel

In addition, we evaluated time domain cooperative silencing where HeNBs are mute in a fraction of subframes, while MeNBs can transmit in all subframes. In such cooperative silencing, MUEs have coordinated subframes in which significant interference from HeNBs is removed. Consequently, such MUEs experience high fluctuation of interference level subframe-by-subframe. In this case, the baseline is to schedule such MUEs at the coordinated subframes but it is also possible for MeNBs to serve some MUEs at the uncoordinated subframes with an adequately controlled MCS level if the interference from the HeNB is not fatally destructive to those MUEs.

There are three simulation scenarios according to adopted schemes:

- Macro-only deployment

- Uncoordinated HeNB deployment: Co-channel deployment where MeNBs and HeNBs transmit in all subframes without any interference coordination

- Uncoordinated HeNB deployment with and without HeNB Power Setting

- Coordinated HeNB deployment: HeNBs are mute in a fraction of subframes (for example, 5 subframes per frame), while MeNBs transmit in all subframes

- Coordinated HeNB deployment with and without HeNB Power Setting

Simulation Results

SINR Distribution

Figure 1: UE SINR Distribution in HeNB Deployment with and without HeNB Power Setting

Figure 2: MUE SINR Distribution in HeNB Deployment with and without HeNB Power Setting

Figure 1 shows that the tail MUEs in HeNB deployment experience severe interference problem caused by CSG HeNBs.

As shown in Figure 2, more significant SINR degradation occurs to indoor MUEs, especially when they are located in an apartment having a CSG HeNB in it. A large fraction of indoor MUEs experience the control channel decoding problem and they may declare radio link failure. In other words, such MUEs may fail to keep their connectivity without interference coordination.

In Figure 1 and 2, we also provide the performance with HeNB power setting. From the simulation results, we can observe that HeNB power setting can somewhat improve the SINR performance of the tail MUEs. However, as we expected, there occurred a degradation in the performance of SINR of HUEs by controlled the output power of HeNB.

The simulation results of edge (5-percentile) MUE geometry gains under several simulation scenarios are shown in Table 3.

Table 3: 5-percentile MUE Geometry Gain [dB]

| Scenario | 5-percentile MUE Geometry Gain [dB] |
| --- | --- |
| Macro-only deployment | -3 |
| HeNB deployment w/o HeNB Power Setting | -14 |
| HeNB deployment w/ HeNB Power Setting (‘X = 50dB’) | -5 |
| HeNB deployment w/ HeNB Power Setting (‘X = 60dB’) | -9 |
| HeNB deployment w/ HeNB Power Setting (‘X = 70dB’) | -14 |

When we look into the results described in Table 3, we can see that the higher value of X is set, the worse performance of 5-percentile MUE Geometry Gain becomes. That is, the higher value of X is set by the network, the higher possibility for HeNB to send a signal to HUE at maximum transmit power turns out to be and consequently MUE will receive a higher level of interference from HeNB.

Therefore, based on the results of 5-percentile MUE Geometry Gain described in Table 3, it is desirable to consider a scenario in which the interference level that MUE receives is about 10dB higher than the receiving signal power from its own serving cell (i.e., MeNB) giving consideration to the worst case, under the condition in which MeNBs and CSG HeNBs are deployed.

Observation 1: Under the condition in which MeNBs and CSG HeNBs are deployed, while we look into the simulation results of the 5-percentile MUE Geometry Gain, it is desirable to consider a scenario in which the interference level that MUE receives is about 10dB higher than the receiving signal power from its own serving cell (i.e., MeNB) relatively giving consideration to the worst case.

Throughput performance

The mean and edge (5-percentile) user throughput are listed in Table 4 and 5, respectively.

Note that in coordinated HeNB deployment, the number of coordinated subframes where HeNBs stop transmission is fixed as 5 in a heuristic manner and simulation results are expected to be suboptimal.

The results of uncoordinated HeNB deployment with and without HeNB power setting are listed as a reference even though there are large numbers of indoor MUEs experiencing radio link failure as shown in Figure 2.

Table 4: Mean User Throughput [kbps]

| Scenario | All UE | MUE | HUE |
| --- | --- | --- | --- |
| Macro-only deployment | 595 | 595 | - |
| Uncoordinated HeNB deployment w/o HeNB Power Setting | 24001 | 1100 | 48568 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 50dB’) | 18640 | 1113 | 37443 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 60dB’) | 22653 | 1105 | 45769 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 70dB’) | 23961 | 1102 | 48483 |
| Coordinated HeNB deployment w/o HeNB Power Setting (‘# of ABSs = 5’) | 12310 | 1146 | 24286 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 50dB’) | 9639 | 1148 | 18748 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 60dB’) | 11638 | 1145 | 22895 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 70dB’) | 12287 | 1146 | 24238 |

Table 5: Edge (5-percentile) User Throughput [kbps]

| Scenario | All UE | MUE | HUE |
| --- | --- | --- | --- |
| Macro-only deployment | 189 | 189 | - |
| Uncoordinated HeNB deployment w/o HeNB Power Setting | 282 | 19 | 23561 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 50dB’) | 344 | 247 | 18492 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 60dB’) | 302 | 129 | 22869 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 70dB’) | 280 | 20 | 23412 |
| Coordinated HeNB deployment w/o HeNB Power Setting (‘# of ABSs = 5’) | 392 | 328 | 11701 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 50dB’) | 391 | 322 | 9254 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 60dB’) | 394 | 323 | 11426 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 70dB’) | 392 | 322 | 11637 |

As shown in Table 4 and 5, introducing HeNBs into macro-only deployment generally brought a huge improvement in terms of mean user throughput due to the off-loading effect.

Also, as shown in Table 4 and 5, time domain cooperative silencing significantly improve the 5-pencentile MUE throughput. More detailed values are provided in Table 6 and 7.

Table 6: Tail MUE Throughput [kbps]

|  | X-percentile MUE Throughput [kbps] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | X=1% | 2% | 3% | 4% | 5% |
| Macro-only deployment | 123 | 147 | 163 | 178 | 189 |
| Uncoordinated HeNB deployment w/o HeNB Power Setting | 0 | 0 | 0 | 2 | 19 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 50dB’) | 0 | 9 | 125 | 208 | 247 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 60dB’) | 0 | 0 | 3 | 46 | 129 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 70dB’) | 0 | 0 | 0 | 3 | 20 |
| Coordinated HeNB deployment w/o HeNB Power Setting (‘# of ABSs = 5’) | 220 | 257 | 284 | 312 | 328 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 50dB’) | 217 | 254 | 281 | 301 | 322 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 60dB’) | 215 | 254 | 281 | 303 | 323 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 70dB’) | 221 | 260 | 283 | 307 | 322 |

Table 7: Tail Indoor MUE Throughput [kbps]

|  | X-percentile MUE Throughput [kbps] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | X=1% | 2% | 3% | 4% | 5% |
| Macro-only deployment | 115 | 138 | 149 | 164 | 178 |
| Uncoordinated HeNB deployment w/o HeNB Power Setting | 0 | 0 | 0 | 0 | 0 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 50dB’) | 0 | 0 | 0 | 2 | 5 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 60dB’) | 0 | 0 | 0 | 0 | 0 |
| Uncoordinated HeNB deployment w/ HeNB Power Setting (‘X = 70dB’) | 0 | 0 | 0 | 0 | 0 |
| Coordinated HeNB deployment w/o HeNB Power Setting (‘# of ABSs = 5’) | 217 | 265 | 306 | 320 | 341 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 50dB’) | 220 | 265 | 297 | 314 | 337 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 60dB’) | 215 | 264 | 293 | 309 | 330 |
| Coordinated HeNB deployment w/ HeNB Power Setting (‘# of ABSs = 5’, ‘X = 70dB’) | 219 | 262 | 295 | 315 | 331 |

Note that about 3% of MUEs cannot receive any packet when uncoordinated HeNBs are deployed with deployment ratio of 0.1 and a fraction of indoor MUE is 35%. Even with HeNB power setting, there are still some MUEs which cannot receive any packet, and significant throughput degradation occurs to indoor MUEs. Thus, the problem of MUEs experiencing the throughput outage will be worse if HeNB deployment ratio or indoor MUE ratio is increased in uncoordinated deployment scenarios.

Therefore, under the condition in which MeNBs and CSG HeNBs are deployed, if consideration is given to the simulation results of throughput outage probability of MUEs and edge user throughput, a conclusion could be made that time domain cooperative silencing method would generate performance improvement in the aspect of system performance.

The simulation result of UE throughput CDF according to each of the scenarios that mentioned above is provided in Appendix A.

Scenario of MeNBs and Outdoor Pico nodes

Simulation Assumptions

System Model

The basic simulation assumptions are set according to the latest 3GPP TR 36.814 [2] summarized in Appendix B.

Here we consider co-channel deployment where outdoor pico nodes for hotzone cells are placed throughout a macro-cell layout. The pico nodes and UEs are deployed according to configuration #1 (uniform placement) and configuration #4b (clustered placement). 4 pico nodes are randomly and uniformly dropped in each macro cell, and 30 UEs are dropped according to each configuration within each macro geographical area.

In particular, outdoor RRH/Hotzone path loss Model 2 described in [2] and fast fading with TU are used. All system simulations are based on 3GPP Case 1 scenario and the corresponding 3D antenna pattern (including vertical pattern) is assumed. Full-buffer traffic model is used and the macro and hotzone cells independently allocate radio resources to their UEs using a time and frequency selective Proportional Fair scheduler.

The serving cell selection is based on best reference signal received power (RSRP). In case of cell range expansion (CRE), a fixed bias value is added to RSRP of the pico nodes and then the cell with best RSRP is chosen as the serving cell.

Almost Blank Subframe (ABS)

We evaluated time domain cooperative silencing where MeNBs are mute in a fraction of subframes (i.e., ABSs), while pico nodes can transmit in all subframes. In such cooperative silencing, pico-UEs have “coordinated” subframes in which significant interference from MeNBs is removed. Consequently, such pico-UEs experience high fluctuation of interference level subframe-by-subframe. In this case, all subframes can be divided into two groups; one with low interference and the other with high interference.

Multiple CSI Reports (Subframe Grouping for CSI Measurements and Reports)

When ABS is adopted, pico-UEs’ CQI/PMI/RI are measured and reported separately depending on the group to which each subframe belongs. A pico-UE generates two CQI/PMI/RI values each of which targets the channel status at each subframe groups. By doing this, it becomes possible to serve a pico-UE at the uncoordinated subframes with an adequately controlled MCS level to cope with the interference from the MeNB (if it is not fatally destructive to the communication of the pico-UE).

There are three simulation scenarios according to adopted schemes:

- Macro-only deployment

- Reuse-1: Conventional co-channel deployment where macro and pico nodes transmit in all subframes.

- Reuse-1 with and without a Biased Serving Cell Selection

- ABS: ABS is adopted. The above-mentioned multiple CSI reports is used, i.e. two CSI reports – one for the coordinated subframes and the other for the uncoordinated subframes – are provided to the pico nodes.

- ABS with and without a Biased Serving Cell Selection

And the simulations were performed in order to evaluate system performance under each simulation scenario, assuming that {0, 6, 9, 12, 15, 25, 30} dB bias values were given to the pico nodes for serving cell selection.

Simulation Results

SINR Distribution

First we study the downlink wideband SINR (geometry) with and without a biased serving cell selection. Figure 3 and 4 show SINR distributions when pico nodes and UEs dropped according to the configuration #1 and #4b, respectively.

Figure 3: UE SINR Distribution in Pico node Deployment with and without Biased Serving Cell Selection (Configuration #1)

Figure 4: UE SINR Distribution in Pico node Deployment with and without Biased Serving Cell Selection (Configuration #4b)

In general, the SINR degradation is observed because of additional interferer (pico nodes). The limited SINR gain in high SINR region is come from a few pico-UEs located close enough to the serving pico node and these pico-UEs overcome harsh interference from MeNBs.

When a biased serving cell selection (i.e., CRE) is adopted, significant SINR degradation occurs in both configurations. This is because newly connected pico-UEs suffer from not only lower transmission power of pico nodes but also harsh interference from the original serving MeNBs. Even in configuration 4b where the most UEs are placed in proximity to the pico nodes and have lower path loss to the serving pico nodes than that to the MeNBs, if the cell selection bias is high, interference from macro cells causes serious performance degradation due to the large difference in transmission power.

Therefore, such coverage expansion of pico nodes should be accompanied with resource coordination (i.e., ABS) so that some resources are reserved for pico-UEs only.

Throughput Performance

Table 8 and 9 show each of the performance gains of both mean and 5% worst user throughput under configuration #1 and #4b according to the various scenarios mentioned above, respectively. In addition, Table 8 and 9 also showed the performance gain according to each scenario with the relative ratio of the performance gain under macro-only deployment scenario.

Especially when performance assessment was conducted under the scenario of ABS with and without a biased serving cell selection, we observed the mean and 5% worst user throughput for each setting, altering ABS value under certain bias values. Here, we assumed in the simulation that the bias values could be {0, 6, 9, 12, 15, 25, 30} dB and ABS values could range from 0 up to 5.

Table 8 and 9 show only the results of the simulation for ABS value settings that had the highest 5% worst user throughput under certain bias values in the scenario of ABS with and without a biased serving cell selection. The entire results of the simulation are provided in Appendix C, including all the possible combinations of bias values and ABS values in the scenario of ABS with and without a biased serving cell selection.

Table 8: User Throughput [kbps] (Configuration #1)

| Scenario | Bias [dB] | 0 | 6 | 9 | 12 | 15 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Macro-only deployment | Mean | 856 | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | (1.00) |  |  |  |  |  |  |
|  | 5% worst | 237 |  |  |  |  |  |  |
|  |  | (1.00) |  |  |  |  |  |  |
| Reuse-1 w/ and w/o a Biased Serving Cell Selection | Mean | 3293 | 3088 | 3010 | 2968 | 2953 | 2967 | 3004 |
|  |  | (3.85) | (3.61) | (3.52) | (3.47) | (3.45) | (3.47) | (3.51) |
|  | 5% worst | 421 (1.78) | 512 (2.16) | 354 (1.49) | 160 (0.68) | 50 (0.21) | 0 (0.00) | 0 (0.00) |
|  | Pico-UE Ratio | 0.44 | 0.55 | 0.6 | 0.65 | 0.7 | 0.83 | 0.88 |
| ABS w/ and w/o a Biased Serving Cell Selection | # of ABS | 0 | 1 | 1 | 2 | 4 | 5 | 5 |
|  | Mean | 3293 | 3473 | 3441 | 3684 | 4006 | 3995 | 3924 |
|  |  | (3.85) | (4.06) | (4.02) | (4.30) | (4.68) | (4.67) | (4.58) |
|  | 5% worst | 421 (1.78) | 529 (2.23) | 549 (2.32) | 577 (2.43) | 577 (2.43) | 539 (2.27) | 454 (1.92) |
|  | Pico-UE Ratio | 0.44 | 0.55 | 0.60 | 0.65 | 0.7 | 0.83 | 0.88 |

Table 9: User Throughput [kbps] (Configuration #4b)

| Scenario | Bias [dB] | 0 | 6 | 9 | 12 | 15 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Macro-only deployment | Mean | 868 | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | (1.00) |  |  |  |  |  |  |
|  | 5% worst | 238 |  |  |  |  |  |  |
|  |  | (1.00) |  |  |  |  |  |  |
| Reuse-1 w/ and w/o a Biased Serving Cell Selection | Mean | 3851 | 3698 | 3637 | 3613 | 3614 | 3677 | 3735 |
|  |  | (4.44) | (4.26) | (4.19) | (4.16) | (4.16) | (4.24) | (4.30) |
|  | 5% worst | 789 (3.32) | 738 (3.10) | 491 (2.06) | 232 (0.97) | 125 (0.53) | 5 (0.02) | 1 (0.00) |
|  | Pico-UE Ratio | 0.66 | 0.76 | 0.81 | 0.84 | 0.87 | 0.93 | 0.95 |
| ABS w/ and w/o a Biased Serving Cell Selection | # of ABS | 0 | 2 | 3 | 4 | 5 | 5 | 5 |
|  | Mean | 3851 | 4438 | 4635 | 4773 | 4881 | 4806 | 4774 |
|  |  | (4.44) | (5.11) | (5.34) | (5.50) | (5.62) | (5.54) | (5.50) |
|  | 5% worst | 789 (3.32) | 1026 (4.31) | 1063 (4.47) | 1055 (4.43) | 1054 (4.43) | 939 (3.95) | 850 (3.57) |
|  | Pico-UE Ratio | 0.66 | 0.76 | 0.81 | 0.84 | 0.87 | 0.93 | 0.95 |

In Table 8 and 9, we can see that the scenario of reuse-1 with and without biased serving cell selection has a considerable gain as compared with the scenario of macro-only deployment in terms of mean user throughput.

As shown in Table 8 and 9, the scenario of ABS with a biased serving cell selection shows the best output in terms of overall performance among other scenarios. The reason for this kind of simulation result is that, when macro cell sets ABS, the overall resource that allocated to MUEs decreases and the throughput loss of MUEs occurs, but throughput gain of pico-UEs due to ABS setting by macro cells is greater than the throughput loss of MUEs. So it could lead to a significant performance improvement from the perspective of mean user throughput.

And, if the higher bias value is set, the more UEs will be attached to the pico nodes and thus the remaining macro-UEs can have more resources. Regardless of the above situation, the interference level that pico-UEs receive from macro eNBs becomes higher relatively. However, because of ABS setting of macro cells, interference that pico-UEs receive at ABS from macro eNBs will be removed, and it will lead to the performance improvement from the perspective of edge user throughput.

The results described in Table 8 and 9 show us that the combinations of bias value and ABS value which have the highest edge user throughput performance under configuration #1 and configuration #4b are “15dB bias, # of ABSs = 4” and “9dB bias, # of ABSs = 3”, respectively.

Therefore, when we look into the simulation results of the various combinations of bias value and ABS value, especially in terms of bias value that has the highest edge user throughput, it is desirable to consider a scenario in which the interference level that pico-UEs receives is about 10 dB higher than the receiving signal power level from its own serving cell (i.e., pico eNB) relatively, under the condition that both MeNBs and outdoor pico nodes are deployed.

The simulation results of UE throughput CDF according to the simulation scenarios which mentioned above are provided in Appendix C.

Observation 2:  Looking into the simulation results for the combinations of bias value and ABS value, especially in terms of bias value that has the highest edge user throughput, it is desirable to consider a scenario in which the interference level that pico-UEs receive is about 10dB higher than the receiving signal power from its own serving cell (i.e., pico eNB) relatively, under the condition that both MeNBs and outdoor pico nodes are deployed.

Summary

In this contribution, we evaluated the system performance between "MeNBs and CSG HeNBs" and between "MeNB and outdoor pico-nodes" under the condition of co-channel deployment. And, we discussed the scenarios of further enhanced non-CS-based ICIC for LTE on the basis of the results from these simulations. The following proposals are made based on observation 1 and 2.

Proposal: Looking into the system performance gains according to the various HetNet deployment scenarios through simulation, it is desirable to consider a scenario of further enhanced non-CS-based ICIC for LTE in which the interference level that UEs receive is about 10dB higher than the receiving signal power from its own serving cell relatively.

References

[1] RP-110824, “Further Enhanced Non CA-based ICIC for LTE – Core part”

[2] 3GPP TR 36.814 v9.0.0 “Further Advancements for E-UTRA: Physical Layer Aspects”

[3] R4-113320, “Requirements for HeNB Power Setting for HeNB-eNB Scenario”, Ericsson, ST-Ericsson, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Nokia Siemens Networks, PicoChip, NEC, III

- Appendix A: Simulation results for the scenario of MeNBs and CSG HeNBs

Figure 5: UE Throughput CDF in Uncoordinated HeNB Deployment w/ and w/o HeNB Power Setting

Figure 6: UE Throughput CDF in Coordinated HeNB Deployment w/ and w/o HeNB Power Setting

Figure 7: UE Throughput CDF in HeNB Deployment w/ and w/o HeNB Power Setting (‘# of ABSs=5’, ‘X=50dB’)

- Appendix B: Simulation parameters for the Scenario of MeNBs and Outdoor Pico nodes

| Parameter |  | Assumption/Value |
| --- | --- | --- |
| Simulation scenario |  | 3GPP Case 1 |
| Cellular layout |  | Hexagonal grid, 19 macro cell sites, 3 cells per site, wrap-around |
| Hotzone deployment |  | 4 Hotzone nodes per cell, wrap-around |
| Inter-site distance (ISD) |  | 500 m |
| UE deployment |  | 30 UEs per cell (Configuration 1, 4b) |
| Distance-dependent path loss |  | TR 36.814 Outdoor RRH/Hotzone Model 2 |
| Shadowing standard deviation | Pico to UE | 10 dB |
|  | Macro to UE | 8 dB |
| Correlation distance of Shadowing |  | 50 m |
| Shadowing correlation Between cells |  | 0.5 |
| Channel model |  | TU |
| Penetration loss | Pico to UE | 20 dB |
|  | Macro to UE | 20 dB |
| Carrier frequency |  | 2 GHz |
| Bandwidth |  | 10 MHz (50RB) |
| Minimum distance between UE and Macro |  | 35 m |
| Minimum distance between UE and Pico |  | 10 m |
| Minimum distance between Pico eNBs |  | 40 m for Case 1 |
| Thermal noise density |  | -174 dBm/Hz |
| Antenna pattern for macro cell (horizontal) |  | = 70 degrees, Am = 25 dB (70 degree horizontal beamwidth) |
| Antenna pattern for macro cell (vertical) |  | = 10 degrees, SLAv = 20 dB |
| Antenna pattern for Hotzone cell |  | Omni-directional |
| Total macro TX power |  | 46 dBm |
| Total pico TX power |  | 30 dBm |
| Macro antenna gain |  | 14 dBi |
| Pico antenna gain for Pico-UE link |  | 5 dBi |
| UE antenna gain |  | 0 dBi |
| Macro and Pico transmitter to UE |  | 2 antennas, 0.5 wavelengths separation |
| UE receiver |  | 2 antennas, 0.5 wavelengths separation |
| UE speed |  | 3 km/h |
| UE noise figure |  | 9 dB |
| Pico noise figure |  | 5 dB |
| Traffic type |  | Full buffer for BS |
| Scheduler |  | Time and frequency selective Proportional Fair scheduler |
| Downlink link adaptation |  | Wideband PMI CQI sub-band size: 6 RB CQI reports: 5 ms CQI delay: 6 ms CQI measurement error: N(0,1) per PRB CQI quantization: 5 bit CQI, 1.2 dB granularity ( -7 ~ 29 dB) MCSs based on LTE transport formats [36.213] |
| Hybrid ARQ |  | Incremental Redundancy (IR), Maximum four transmissions, Initial transmission target FER: 10% |
| Hybrid ARQ round trip delay for UE |  | 8 subframes (8 ms) |
| Downlink receiver type |  | MMSE |
| Channel Estimation |  | Non Ideal |
| Feedback and control channel errors |  | Ideal |
| Simulation drops |  | 3 |
| Link to System Mapping |  | MIESM |

- Appendix C: Simulation results for the Scenario of MeNBs and Outdoor Pico nodes

Table 10: User Throughput [kbps] (Configuration #1)

| Scenario | Bias [dB] | 0 | 6 | 9 | 12 | 15 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ABS w/ and w/o a Biased Serving Cell Selection | # of ABS | 0 |  |  |  |  |  |  |
|  | Mean | 3293 | 3088 | 3010 | 2968 | 2953 | 2967 | 3004 |
|  | 5% worst | 421 | 512 | 354 | 160 | 50 | 0 | 0 |
|  | # of ABS | 1 |  |  |  |  |  |  |
|  | Mean | 3566 | 3473 | 3441 | 3424 | 3402 | 3365 | 3375 |
|  | 5% worst | 377 | 529 | 549 | 517 | 401 | 163 | 121 |
|  | # of ABS | 2 |  |  |  |  |  |  |
|  | Mean | 3798 | 3741 | 3708 | 3684 | 3649 | 3577 | 3564 |
|  | 5% worst | 335 | 486 | 547 | 577 | 535 | 298 | 219 |
|  | # of ABS | 3 |  |  |  |  |  |  |
|  | Mean | 3984 | 3952 | 3914 | 3890 | 3844 | 3738 | 3706 |
|  | 5% worst | 295 | 425 | 502 | 561 | 575 | 400 | 303 |
|  | # of ABS | 4 |  |  |  |  |  |  |
|  | Mean | 4139 | 4127 | 4085 | 4060 | 4006 | 3876 | 3822 |
|  | 5% worst | 253 | 366 | 435 | 512 | 577 | 481 | 388 |
|  | # of ABS | 5 |  |  |  |  |  |  |
|  | Mean | 4281 | 4278 | 4237 | 4208 | 4145 | 3995 | 3924 |
|  | 5% worst | 211 | 303 | 362 | 438 | 522 | 539 | 454 |
|  | Pico-UE Ratio | 0.44 | 0.55 | 0.60 | 0.65 | 0.7 | 0.83 | 0.88 |

Table 11: User Throughput [kbps] (Configuration #4b)

| Scenario | Bias [dB] | 0 | 6 | 9 | 12 | 15 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ABS w/ and w/o a Biased Serving Cell Selection | # of ABS | 0 |  |  |  |  |  |  |
|  | Mean | 3851 | 3698 | 3637 | 3613 | 3614 | 3677 | 3735 |
|  | 5% worst | 789 | 738 | 491 | 232 | 125 | 5 | 1 |
|  | # of ABS | 1 |  |  |  |  |  |  |
|  | Mean | 4180 | 4157 | 4154 | 4144 | 4146 | 4183 | 4236 |
|  | 5% worst | 730 | 996 | 914 | 802 | 696 | 409 | 332 |
|  | # of ABS | 2 |  |  |  |  |  |  |
|  | Mean | 4442 | 4438 | 4431 | 4414 | 4411 | 4413 | 4438 |
|  | 5% worst | 652 | 1026 | 1035 | 979 | 863 | 629 | 521 |
|  | # of ABS | 3 |  |  |  |  |  |  |
|  | Mean | 4641 | 4645 | 4635 | 4614 | 4603 | 4578 | 4579 |
|  | 5% worst | 571 | 998 | 1063 | 1038 | 977 | 772 | 654 |
|  | # of ABS | 4 |  |  |  |  |  |  |
|  | Mean | 4800 | 4809 | 4797 | 4773 | 4757 | 4705 | 4688 |
|  | 5% worst | 497 | 920 | 1056 | 1055 | 1028 | 878 | 773 |
|  | # of ABS | 5 |  |  |  |  |  |  |
|  | Mean | 4943 | 4949 | 4934 | 4904 | 4881 | 4806 | 4774 |
|  | 5% worst | 407 | 803 | 1008 | 1053 | 1054 | 939 | 850 |
|  | Pico-UE Ratio | 0.66 | 0.76 | 0.81 | 0.84 | 0.87 | 0.93 | 0.95 |

Figure 8: UE Throughput CDF for Configuration #1

Figure 9: UE Throughput CDF for Configuration #4b