3GPP TSG-RAN WG1 #68	R1-120635

Dresden, Germany, 6 – 10 February 2012

Source:	MediaTek Inc.

Title:	Analysis of half duplex operation

Agenda Item: 7.7.5

Document for: 	Discussion

# Introduction

Motivated by the goal of avoiding, in the future, the need to maintain a separate GSM/GPRS network just for MTC devices, the study item of provisioning of low-cost MTC UEs based on LTE was proposed and approved [1]. The most important factor for the business success of LTE based MTC is obviously cost if satisfactory coverage & power consumption can be ensured as well.

In the last RAN1 #67 meeting, it was agreed that the reference for the cost comparison for the low cost MTC device will be a single band, single RAT, Cat-1 UE, and operating on a 20 MHz carrier. Also, the following techniques have been identified for further analysis:

- Reduction of maximum bandwidth

- Single receive RF chain

- Reduction of peak rate

- Reduction of transmit power

- Half duplex operation

A tabulated analysis of most of the proposed techniques can be found in [4]. In this contribution, we present more detailed analysis on half duplex MTC UE. We follow the structure of the TR section 6 so that most of the text in all the sections can be considered as text proposal.

---------------------------Text Proposal Start (other than the text in brackets) ------------------------------------

# Description

HDD is a technique aimed to lower the cost by allowing UE not to transmit and receive simultaneously. It is expected that this technique will have much of the cost saving from replacing duplexer with a switch.

# Analysis/evaluation of performance against requirements

An analysis of the technique against system requirements is provided in this section. The analysis is to be used in combination with the cost analysis:

Table 1. Impact of half duplex operation

| Metric | Impact (Yes/No) |
| --- | --- |
| Coverage same as GSM/EGPRS [and legacy LTE] | Yes |
| Minimum Data rate | No |
| Power consumption | No |
| Impact to non-MTC UE | No |
| eNB Hardware impact | No |
| Impact on specification | Yes (potentially) |
| Cell spectral efficiency | Yes |
| ….. |  |
| …… |  |

[Editor’s Note: Whilst the Low cost MTC UE based on LTE is required to meet all the requirements, a particular requirement may not be applicable to an identified technique. Evaluation/analysis of impact (positive/negative) to be provided below for only for the requirement’s that has an impact (indicated by “Yes” above in the table). Below shown are example placeholders for some analysis/evaluation of some of the requirements]

# Coverage Analysis

It is expected that replacing duplexer with a switch will remove the duplexer insertion loss by 1.5-2dB, which will help to improve the UL coverage

# Specification impact

Half-duplex FDD is technically supported in Rel-8, where the burden is on eNB to observe the constraint of HD-FDD operation. Additional spec support to explicitly define/allow Tx/Rx switch time, or to make the system more robust under HDD operation, is FFS.

# Analysis/evaluation of cost reduction

Given the duplexer cost is in the range of 15-20% of RF [3] (which is 40% of the total modem cost), and also given that a switch is about 20-30% of the cost of a duplexer, the total cost saving for HDD is 4-6%. There may also be a need to investigate the potential change of RF design with respect to whether additional filtering is needed given the duplexer filters are removed.

---------------------------Text Proposal Ends ------------------------------------

# Conclusion

In this contribution, we present more detailed analysis on HDD.

We propose:

- Consider the text in all the sections as text proposal

- Note that half-duplex FDD is technically supported in Rel-8, where the burden is on eNB to observe the constraint of HD-FDD operation. Additional spec support to explicitly define/allow Tx/Rx switch time, or to make the system more robust under HDD operation, is FFS.

- Note that the total maximal cost saving for HDD is 4-6%. There may also be a need to investigate the potential change of RF design with respect to whether additional filtering is needed given the duplexer filters are removed.

# References

- R1-111112, Study Item “Provision of low-cost MTC UEs based on LTE”

- R1-113616, “TR36.888 - Study on provision of low-cost MTC UEs based on LTE”

- R1-120636, “Cost Drivers of LTE modem and Text Proposal”, MediaTek, RAN1#68

- R1-113867, “LTE MTC Device Cost Reduction Measures with Cost Analysis”, MediaTek, RAN1#67