TSG-RAN WG1 #72	R1-130658

St. Julian’s, Malta, 28th January– 1st February 2013

Source:	Orange, AT&T, DISH Network, Qualcomm Incorporated

Title:	eMBMS support in New Carrier Type

Agenda Item:	7.3.1.4

Document for:	Discussion and decision

# Introduction

eMBMS has been specified in Release 8 and 9 to allow mobile network operators to provide broadcast and multicast services over LTE. Apart from mobile TV, eMBMS can provide services such as live streaming in a stadium, pushed content via user equipment caching or even some machine-to-machine services. The potential savings in radio resources and backhaul resources, along with the guarantee that content is delivered with secured quality of service in a defined area justifies to further enhance eMBMS transmissions in release 12. In particular, it would be valuable to relieve the current constraint that only 60% of the subframes can be used for MBSFN transmissions within a radio frame. This feature would allow semi-static allocation of up to nearly 100% of resources for eMBMS transmissions in a carrier shared between unicast and eMBMS transmission, giving higher operator flexibility to boost eMBMS capacity when required. The NCT discussed in Release 12 provides a unique opportunity to address this task  with limited specification and development effort [1].

# Increased MBSFN subframes ratio

Currently, subframes 0, 4, 5, 9 of a radio frame cannot be MBSFN subframes in a FDD system, which means that the maximum utilization ratio for MBSFN is 60%.

In legacy carriers, this is to avoid disturbing synchronization signals (in subframes 0 and 5) and paging occurrences (in subframes 0, 4, 5 and 9), and to ensure that sufficient CRS are transmitted to correctly decode the broadcast system information.

MBSFN transmission in subframes 4 and 9:

In legacy carriers, subframes 4 and 9 can potentially be used for paging, depending on the cell-specific paging cycle, which is broadcasted by the network and the UE-specific paging cycle, configured by upper layers. UEs use these two parameters to calculate the radio frames and subframes where paging could occur.

Allowing MBSFN transmission in subframes 4 and 9 would require to change the allowed values for the cell-specific paging cycle and the UE-specific paging cycle to accommodate every paging occurrence in subframes 0 and 5, which feasibility requires further study. This would allow for a MBSFN ratio up to 80%.

MBSFN transmission in subframes 0 and 5:

Whether MBSFN transmission in NCT could be envisioned in subframe 0 or in subframe 5 depends on the decisions on synchronization signals that will be taken as a part of this WI.

# Conclusions

This paper has emphasized the interest from an operator point of view to improve the number of subframes in which eMBMS services can be provided within a radio frame. This requirement can be addressed with limited effort for NCT, during the TM related work planned to be undertaken in the New Carrier Type WI.

Proposal: RAN1 defines PMCH operation in the NCT design, with increased MBSFN subframes ratio compared to the legacy carrier type.

# References

[1] RP-121415, New Carrier Type for LTE, Chicago, USA, September 4 - 7, 2012